3GPP TSG-RAN WG5 Meeting # 104	R5-245660

# Maastricht, NL,  19 – 23 Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4619 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update UE pre-test configurations for test case 6.5.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE pre-test condition needs update |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the UE configuration to make the NR Cell1 has higher priority than NR Cell3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case may not run normally |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-245079. |  |  |  |  |  |  |  |  |

#### 6.5.3.5	SNPN Selection in Automatic Mode / Using credentials from a credentials holder

6.5.3.5.1	Test Purpose (TP)

(1)

with { UE in Automatic network selection mode and supports access to an SNPN using credentials from a credentials holder, using the SNPN selection parameters in the selected entry of the "list of subscriber data" }

ensure that {

when { SNPNs are available which broadcast the indication that access using credentials from a credentials holder is supported }

then { UE selects a cell of the highest priority SNPN and UE attempts a registration on the selected cell }

}

START OF CHANGE

(2)

with { UE in Automatic network selection mode and supports access to an SNPN using credentials from a credentials holder, using the SNPN selection parameters in the selected entry of the "list of subscriber data" }

ensure that {

when { A SNPN is available which broadcasts the indication that access using credentials from a credentials holder is supported, and the highest priority SNPN is not available}

then { UE selects a cell of the next priority available SNPN and UE attempts a registration on the selected cell }

}

<unchanged contents omitted>

6.5.3.5.3	Test description

6.5.3.5.3.1	Pre-test conditions

System Simulator:

-	2 SNPN cells NR Cell 1, NR cell 3 are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-27 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	UE is configured with subscriber Data list that allows access to SNPN IDs of NR Cell 1 and NR Cell 3

-	SNPN ID of NR Cell 1 has a higher priority than SNPN ID of NR Cell 3

-	The SNPN ID of NR Cell 1 is in credentials holder controlled prioritized list of preferred SNPNs

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A).

<unchanged contents omitted>

END OF CHANGE