3GPP TSG-RAN5 Meeting #104	R5-245982

# Maastricht , Netherlands, 19-23, August, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5517 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test case 6.2.4A.1_3 A-MPR for CA HPUE (CA_NS_04) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test, LTE_CA_C_B41_PC2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In many of the test points of this test case, the RB allocation number do not satisfy the multiples of 2^a*3^b*5^c , which is required for LTE UL. It was found that RB numbers are not configurable during test, thus make the test points unable to proceed. In addition, some the RB start is also incorrect since exceeding the maximum RB in a carrier. Some requirements are also not correctly calculated based on RAN4 requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the RB number allocation to let the test points satisify the RB number rules, and correct the requirements accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Many of the test points are untestable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4A.1_3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.905 CR 0274 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Change from R5-244659. |  |  |  |  |  |  |  |  |

6.2.4A.1_3	Additional Maximum Power Reduction (A-MPR) for CA and HPUE (intra-band contiguous DL CA and UL CA)

6.2.4A.1_3.1	Test purpose

Same test purpose as in clause 6.2.4A.1.1.

6.2.4A.1_3.2	Test applicability

The requirements of this test apply in test case 6.6.2.2A.1_3 Additional Spectrum Emission Mask for CA and HPUE (intra-band contiguous DL CA and UL CA) for network signalled value CA_NS_04 to all types of E-UTRA Power Class 2 UE release 10 and forward that support intra-band contiguous DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

The requirements of this test apply in test case 6.6.3.3A.1_3 Additional spurious emissions for CA and HPUE (intra-band contiguous DL CA and UL CA) for network signalled values CA_NS_04 to all types of E-UTRA Power Class 2 UE release 10 and forward that support intra-band contiguous DL CA and UL CA but do not support UL 64QAM or UL 256QAM.

NOTE:	As a result TC 6.2.4A.1_3 has not been included in the test case applicability table 4.1-1, TS 36.521-2 [11]. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.4A.1_3.3	Minimum conformance requirements

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the UE Maximum Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) as specified in subclause 6.2.2A.1_3.3.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A.1_3.3-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A.1_3.3 does not apply, i.e. the carrier aggregation MPR = 0 dB.

For the UE maximum output power modified by A-MPR specified in table 6.2.4A.1_3.3-1, the power limits specified in subclause 6.2.5A.1_3.3 apply.

Table 6.2.4A.1_3.3-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA for HPUE

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_04 | 6.6.2.2A.1_3.3.1, 6.6.3.3A.1_3.3.8 | CA_41C | 6.2.4A.1_3.3.4 |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.4.2A.1-1 for which none of the additional requirements in subclauses 6.6.3.3A or 6.6.2.2A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For intra-band carrier aggregation, the A-MPR is evaluated per slot and given by the maximum value taken over the transmission(s) on all component carriers within the slot; the maximum A-MPR over the two slots is then applied for the entire subframe.

6.2.4A.1_3.3.4	A-MPR for CA_NS_04 for CA_41C

If the UE is configured to CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.1_3.3.4-1.

Table 6.2.4A.1_3.3.4-1: Contiguous Allocation A-MPR for CA_NS_04 (power class 2)

| CA Bandwidth Class C | Lower edge cutoff frequency [MHz]5 | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR per modulation [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 25 RB / 100 RB | 2513.5 | 0 – 42 | >0 | N/A | ≤5 | ≤5 | ≤5 | FFS |
|  |  | 43 – 81 | N/A | >82 | ≤1 | ≤1.5 | ≤1.5 | FFS |
|  |  | 82 – 124 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | FFS |
| 50 RB / 100 RB | 2518.4 | 0 – 52 | >0 | N/A | ≤5 | ≤5 | ≤5 | FFS |
|  |  | 53 – 94 | N/A | >95 | ≤1 | ≤1.5 | ≤1.5 | FFS |
|  |  | 95 – 149 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | FFS |
| 75 RB / 75 RB | 2519.0 | 0 – 54 | >0 | N/A | ≤5 | ≤5 | ≤5 | FFS |
|  |  | 55 – 94 | N/A | >95 | ≤2 | ≤2.5 | ≤2.5 | FFS |
|  |  | 95 – 149 | >0 | N/A | ≤1.5 | ≤2 | ≤2 | FFS |
| 75 RB / 100 RB | 2523.4 | 0 – 64 | >0 | N/A | ≤5 | ≤5 | ≤5 | FFS |
|  |  | 65 – 114 | N/A | >115 | ≤2 | ≤2.5 | ≤2.5 | FFS |
|  |  | 115 – 174 | >0 | N/A | ≤1 | ≤1.5 | ≤2 | FFS |
| 100 RB / 100 RB | 2528.3 | 0 – 69 | >0 | N/A | ≤5 | ≤5 | ≤5 | FFS |
|  |  | 70 – 129 | N/A | >130 | ≤2 | ≤2.5 | ≤2.5 | FFS |
|  |  | 130 – 199 | >0 | N/A | ≤1.5 | ≤1.5 | ≤2 | FFS |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. NOTE 5: The A-MPR values in this table shall apply when the lower edge of the aggregated channel bandwidth (Figure 5.4.2A-1) is less than or equal to the lower edge cutoff frequency specified in this table for the corresponding CA bandwidth combination. When the lower edge of the aggregated channel bandwidth exceeds the lower edge cutoff frequency, then the A-MPR shall be equal to the MPR specified in Table 6.2.3A.1_3.3-1a. |  |  |  |  |  |  |  |  |

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 2

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	11.5,	 				0≤ A < 0.05

= -55.0A + 14.25,		0.05≤ A < 0.15

= -4.0A + 6.60, 			0.15≤ A < 0.40

= -0.833A + 5.333,		0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

Where MA is defined as follows when the lower edge of the aggregated channel bandwidth (Table 5.4.2A-1) is less than or equal to the lower edge cutoff frequency specified in Table 6.2.4A.1_3.3.4-1 for the corresponding CA bandwidth combination

MA 	= 13.0,		 				0 ≤ A < 0.05

= 15.33 – 46.67A,		0.05 ≤ A < 0.20

= -7.0 – 5.0A, 			0.20 ≤ A < 0.50

= 4.5,						0.50 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

The normative reference for requirement is in TS 36.101 [2] clause 6.2.4A.4.

6.2.4A.1_3.4	Test description

Same test description as in clause 6.2.4A.1.4 excepting replace Table 6.2.4A.1.4.1-4 with Table 6.2.4A.1_3.4.1-1.

Table 6.2.4A.1_3.4.1-1: Test Configuration Table (network signalled value "CA_NS_04")

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | C: Low range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  | As in Table 6.2.4A.1_3.3.4-1 |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  | PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 5 | 20 | N/A | QPSK | 1 |  | 0@0 | 1@17 |  |  |
| 2 | 5 | 20 |  | QPSK | 1 |  | 0@0 | 1@55 |  |  |
| 3 | 5 | 20 |  | QPSK | 40 |  | 0@0 | 40@17 |  |  |
| 4 | 5 | 20 |  | QPSK | 81 |  | 0@0 | 81@17 |  |  |
| 5 | 5 | 20 |  | QPSK | 45 |  | 0@0 | 45@55 |  |  |
| 6 | 5 | 20 |  | QPSK | 1 |  | 1@0 | 0@0 |  |  |
| 7 | 5 | 20 |  | QPSK | 1 |  | 0@0 | 1@99 |  |  |
| 8 | 5 | 20 |  | QPSK | 125 |  | 25@0 | 100@0 |  |  |
| 9 | 5 | 20 |  | 16QAM | 1 |  | 0@0 | 1@17 |  |  |
| 10 | 5 | 20 |  | 16QAM | 1 |  | 0@0 | 1@55 |  |  |
| 11 | 5 | 20 |  | 16QAM | 40 |  | 0@0 | 40@17 |  |  |
| 12 | 5 | 20 |  | 16QAM | 81 |  | 0@0 | 81@17 |  |  |
| 13 | 5 | 20 |  | 16QAM | 45 |  | 0@0 | 45@55 |  |  |
| 14 | 5 | 20 |  | 16QAM | 1 |  | 1@0 | 0@0 |  |  |
| 15 | 5 | 20 |  | 16QAM | 1 |  | 0@0 | 1@99 |  |  |
| 16 | 5 | 20 |  | 16QAM | 125 |  | 25@0 | 100@0 |  |  |
| 17 | 5 | 20 |  | 64QAM | 1 |  | 0@0 | 1@17 |  |  |
| 18 | 5 | 20 |  | 64QAM | 1 |  | 0@0 | 1@55 |  |  |
| 19 | 5 | 20 |  | 64QAM | 40 |  | 0@0 | 40@17 |  |  |
| 20 | 5 | 20 |  | 64QAM | 81 |  | 0@0 | 81@17 |  |  |
| 21 | 5 | 20 |  | 64QAM | 45 |  | 0@0 | 45@55 |  |  |
| 22 | 5 | 20 |  | 64QAM | 1 |  | 1@0 | 0@0 |  |  |
| 23 | 5 | 20 |  | 64QAM | 1 |  | 0@0 | 1@99 |  |  |
| 24 | 5 | 20 |  | 64QAM | 125 |  | 25@0 | 100@0 |  |  |
| 25 | 10 | 20 |  | QPSK | 1 |  | 0@0 | 1@2 |  |  |
| 26 | 10 | 20 |  | QPSK | 1 |  | 0@0 | 1@43 |  |  |
| 27 | 10 | 20 |  | QPSK | 40 |  | 0@0 | 40@2 |  |  |
| 28 | 10 | 20 |  | QPSK | 48 |  | 0@0 | 48@2 |  |  |
| 29 | 10 | 20 |  | QPSK | 54 |  | 0@0 | 54@46 |  |  |
| 30 | 10 | 20 |  | QPSK | 1 |  | 1@0 | 0@0 |  |  |
| 31 | 10 | 20 |  | QPSK | 1 |  | 0@0 | 1@99 |  |  |
| 32 | 10 | 20 |  | QPSK | 150 |  | 50@0 | 100@0 |  |  |
| 33 | 10 | 20 |  | 16QAM | 1 |  | 0@0 | 1@2 |  |  |
| 34 | 10 | 20 |  | 16QAM | 1 |  | 0@0 | 1@43 |  |  |
| 35 | 10 | 20 |  | 16QAM | 40 |  | 0@0 | 40@2 |  |  |
| 36 | 10 | 20 |  | 16QAM | 48 |  | 0@0 | 48@2 |  |  |
| 37 | 10 | 20 |  | 16QAM | 54 |  | 0@0 | 54@46 |  |  |
| 38 | 10 | 20 |  | 16QAM | 1 |  | 1@0 | 0@0 |  |  |
| 39 | 10 | 20 |  | 16QAM | 1 |  | 0@0 | 1@99 |  |  |
| 40 | 10 | 20 |  | 16QAM | 150 |  | 50@0 | 100@0 |  |  |
| 41 | 10 | 20 |  | 64QAM | 1 |  | 0@0 | 1@2 |  |  |
| 42 | 10 | 20 |  | 64QAM | 1 |  | 0@0 | 1@43 |  |  |
| 43 | 10 | 20 |  | 64QAM | 40 |  | 0@0 | 40@2 |  |  |
| 44 | 10 | 20 |  | 64QAM | 48 |  | 0@0 | 48@2 |  |  |
| 45 | 10 | 20 |  | 64QAM | 54 |  | 0@0 | 54@46 |  |  |
| 46 | 10 | 20 |  | 64QAM | 1 |  | 1@0 | 0@0 |  |  |
| 47 | 10 | 20 |  | 64QAM | 1 |  | 0@0 | 1@99 |  |  |
| 48 | 10 | 20 |  | 64QAM | 150 |  | 50@0 | 100@0 |  |  |
| 49 | 15 | 15 |  | QPSK | 1 |  | 1@54 | 0@0 |  |  |
| 50 | 15 | 15 |  | QPSK | 1 |  | 0@0 | 1@18 |  |  |
| 51 | 15 | 15 |  | QPSK | 40 |  | 20@55 | 20@0 |  |  |
| 52 | 15 | 15 |  | QPSK | 95 |  | 20@55 | 75@0 |  |  |
| 53 | 15 | 15 |  | QPSK | 54 |  | 0@0 | 54@21 |  |  |
| 54 | 15 | 15 |  | QPSK | 1 |  | 1@0 | 0@0 |  |  |
| 55 | 15 | 15 |  | QPSK | 1 |  | 0@0 | 1@74 |  |  |
| 56 | 15 | 15 |  | QPSK | 150 |  | 75@0 | 75@0 |  |  |
| 57 | 15 | 15 |  | 16QAM | 1 |  | 1@54 | 0@0 |  |  |
| 58 | 15 | 15 |  | 16QAM | 1 |  | 0@0 | 1@18 |  |  |
| 59 | 15 | 15 |  | 16QAM | 40 |  | 20@55 | 20@0 |  |  |
| 60 | 15 | 15 |  | 16QAM | 95 |  | 20@55 | 75@0 |  |  |
| 61 | 15 | 15 |  | 16QAM | 54 |  | 0@0 | 54@21 |  |  |
| 62 | 15 | 15 |  | 16QAM | 1 |  | 1@0 | 0@0 |  |  |
| 63 | 15 | 15 |  | 16QAM | 1 |  | 0@0 | 1@74 |  |  |
| 64 | 15 | 15 |  | 16QAM | 150 |  | 75@0 | 75@0 |  |  |
| 65 | 15 | 15 |  | 64QAM | 1 |  | 1@54 | 0@0 |  |  |
| 66 | 15 | 15 |  | 64QAM | 1 |  | 0@0 | 1@18 |  |  |
| 67 | 15 | 15 |  | 64QAM | 40 |  | 20@55 | 20@0 |  |  |
| 68 | 15 | 15 |  | 64QAM | 95 |  | 20@55 | 75@0 |  |  |
| 69 | 15 | 15 |  | 64QAM | 54 |  | 0@0 | 54@21 |  |  |
| 70 | 15 | 15 |  | 64QAM | 1 |  | 1@0 | 0@0 |  |  |
| 71 | 15 | 15 |  | 64QAM | 1 |  | 0@0 | 1@74 |  |  |
| 72 | 15 | 15 |  | 64QAM | 150 |  | 75@0 | 75@0 |  |  |
| 73 | 15 | 20 |  | QPSK | 1 |  | 1@64 | 0@0 |  |  |
| 74 | 15 | 20 |  | QPSK | 1 |  | 0@0 | 1@38 |  |  |
| 75 | 15 | 20 |  | QPSK | 50 |  | 10@65 | 40@0 |  |  |
| 76 | 15 | 20 |  | QPSK | 110 |  | 10@65 | 100@0 |  |  |
| 77 | 15 | 20 |  | QPSK | 60 |  | 0@0 | 60@40 |  |  |
| 78 | 15 | 20 |  | QPSK | 1 |  | 1@0 | 0@0 |  |  |
| 79 | 15 | 20 |  | QPSK | 1 |  | 0@0 | 1@99 |  |  |
| 80 | 15 | 20 |  | QPSK | 175 |  | 75@0 | 100@0 |  |  |
| 81 | 15 | 20 |  | 16QAM | 1 |  | 1@64 | 0@0 |  |  |
| 82 | 15 | 20 |  | 16QAM | 1 |  | 0@0 | 1@38 |  |  |
| 83 | 15 | 20 |  | 16QAM | 50 |  | 50@64 | 40@0 |  |  |
| 84 | 15 | 20 |  | 16QAM | 110 |  | 9@64 | 100@0 |  |  |
| 85 | 15 | 20 |  | 16QAM | 60 |  | 0@0 | 60@40 |  |  |
| 86 | 15 | 20 |  | 16QAM | 1 |  | 1@0 | 0@0 |  |  |
| 87 | 15 | 20 |  | 16QAM | 1 |  | 0@0 | 1@99 |  |  |
| 88 | 15 | 20 |  | 16QAM | 175 |  | 75@0 | 100@0 |  |  |
| 89 | 15 | 20 |  | 64QAM | 1 |  | 1@64 | 0@0 |  |  |
| 90 | 15 | 20 |  | 64QAM | 1 |  | 0@0 | 1@38 |  |  |
| 91 | 15 | 20 |  | 64QAM | 50 |  | 10@65 | 40@0 |  |  |
| 92 | 15 | 20 |  | 64QAM | 110 |  | 10@65 | 100@0 |  |  |
| 93 | 15 | 20 |  | 64QAM | 60 |  | 0@0 | 60@40 |  |  |
| 94 | 15 | 20 |  | 64QAM | 1 |  | 1@0 | 0@0 |  |  |
| 95 | 15 | 20 |  | 64QAM | 1 |  | 0@0 | 1@99 |  |  |
| 96 | 15 | 20 |  | 64QAM | 175 |  | 75@0 | 100@0 |  |  |
| 97 | 20 | 20 |  | QPSK | 1 |  | 1@69 | 0@0 |  |  |
| 98 | 20 | 20 |  | QPSK | 1 |  | 0@0 | 1@28 |  |  |
| 99 | 20 | 20 |  | QPSK | 60 |  | 30@70 | 30@0 |  |  |
| 100 | 20 | 20 |  | QPSK | 130 |  | 30@70 | 100@0 |  |  |
| 101 | 20 | 20 |  | QPSK | 72 |  | 0@0 | 72@28 |  |  |
| 102 | 20 | 20 |  | QPSK | 1 |  | 1@0 | 0@0 |  |  |
| 103 | 20 | 20 |  | QPSK | 1 |  | 0@0 | 1@99 |  |  |
| 104 | 20 | 20 |  | QPSK | 200 |  | 100@0 | 100@0 |  |  |
| 105 | 20 | 20 |  | 16QAM | 1 |  | 1@69 | 0@0 |  |  |
| 106 | 20 | 20 |  | 16QAM | 1 |  | 0@0 | 1@28 |  |  |
| 107 | 20 | 20 |  | 16QAM | 60 |  | 30@70 | 30@0 |  |  |
| 108 | 20 | 20 |  | 16QAM | 130 |  | 30@70 | 100@0 |  |  |
| 109 | 20 | 20 |  | 16QAM | 72 |  | 0@0 | 72@28 |  |  |
| 110 | 20 | 20 |  | 16QAM | 1 |  | 1@0 | 0@0 |  |  |
| 111 | 20 | 20 |  | 16QAM | 1 |  | 0@0 | 1@99 |  |  |
| 112 | 20 | 20 |  | 16QAM | 200 |  | 100@0 | 100@0 |  |  |
| 113 | 20 | 20 |  | 64QAM | 1 |  | 1@69 | 0@0 |  |  |
| 114 | 20 | 20 |  | 64QAM | 1 |  | 0@0 | 1@28 |  |  |
| 115 | 20 | 20 |  | 64QAM | 60 |  | 30@70 | 30@0 |  |  |
| 116 | 20 | 20 |  | 64QAM | 130 |  | 30@70 | 100@0 |  |  |
| 117 | 20 | 20 |  | 64QAM | 72 |  | 0@0 | 72@28 |  |  |
| 118 | 20 | 20 |  | 64QAM | 1 |  | 1@0 | 0@0 |  |  |
| 119 | 20 | 20 |  | 64QAM | 1 |  | 0@0 | 1@99 |  |  |
| 120 | 20 | 20 |  | 64QAM | 200 |  | 100@0 | 100@0 |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. |  |  |  |  |  |  |  |  |  |  |

6.2.4A.1_3.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.4A.1_3.5-1. For QPSK and 16QAM the allowed maximum output power reduction is specified in Table 6.2.4A.1_3.3.4-1 and clause 6.2.3A.1_3 does not apply, i.e. carrier aggregation MPR = 0. For 64QAM the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A-1_3.3-1 and A-MPR requirements specified in Table 6.2.4A.1_3.3.4-1.

Table 6.2.4A.1_3.5-1: Test requirement (network signalled value "CA_NS_04")

| Configuration ID | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 2 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 3 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 4 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 5 |  |  | 26 | + 2.7 / -3.7 |  |  |
| 6 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 7 |  |  | 26 | + 2.7 / -3.7 |  |  |
| 8 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 9 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 10 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 11 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 12 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 13 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 14 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 15 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 16 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 17 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 18 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 19 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 20 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 21 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 22 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 23 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 24 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 25 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 26 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 27 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 28 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 29 |  |  | 26 | + 2.7 / -3.7 |  |  |
| 30 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 31 |  |  | 26 | + 2.7 / -3.7 |  |  |
| 32 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 33 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 34 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 35 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 36 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 37 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 38 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 39 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 40 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 41 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 42 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 43 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 44 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 45 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 46 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 47 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 48 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 49 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 50 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 51 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 52 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 53 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 54 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 55 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 56 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 57 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 58 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 59 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 60 |  |  | 26 | + 2.7 / -5.2 |  |  |
| 61 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 62 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 63 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 64 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 65 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 66 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 67 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 68 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 69 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 70 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 71 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 72 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 73 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 74 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 75 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 76 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 77 |  |  | 26 | + 2.7 / -3.7 |  |  |
| 78 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 79 |  |  | 26 | + 2.7 / -3.7 |  |  |
| 80 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 81 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 82 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 83 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 84 |  |  | 26 | + 2.7 / -5.2 |  |  |
| 85 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 86 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 87 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 88 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 89 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 90 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 91 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 92 |  |  | 26 | + 2.7 / -5.2 |  |  |
| 93 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 94 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 95 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 96 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 97 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 98 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 99 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 100 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 101 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 102 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 103 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 104 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 105 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 106 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 107 |  |  | 26 | + 2.7 / -2.7 |  |  |
| 108 |  |  | 26 | + 2.7 / -5.2 |  |  |
| 109 |  |  | 26 | + 2.7 / -5.2 |  |  |
| 110 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 111 |  |  | 26 | + 2.7 / -4.2 |  |  |
| 112 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 113 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 114 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 115 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 116 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 117 |  |  | 26 | + 2.7 / -5.7 |  |  |
| 118 |  |  | 26 | + 2.7 / -7.7 |  |  |
| 119 |  |  | 26 | + 2.7 / -4.7 |  |  |
| 120 |  |  | 26 | + 2.7 / -7.7 |  |  |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5A.1_3.3-1. |  |  |  |  |  |  |