3GPP TSG-RAN5 Meeting #104	R5-245480

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5336 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS P-CSCF restoration test case 10.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 10.4.3 (“EPS bearer context deactivation / reactivation requested / new P-CSCF address / Initial IMS registration”) needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 10.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage does not exist for P-CSCF restoration support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1461 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-245480 is the final version of R5-244845 with below changes: 1. Added text in Table 10.4.3.3.2-2 indicating IMS registration procedure shall be sent to newly allocated P-CSCF address. 2. Removed Additional_P-CSCF_IPv4, Additional_P-CSCF_IPv6 in Table 10.4.3.3.3-2. |  |  |  |  |  |  |  |  |

<Start of modified section>

### 10.4.3	EPS bearer context deactivation / reactivation requested / new P-CSCF address / Initial IMS registration

10.4.3.1	Test Purpose (TP)

(1)

with { the UE in BEARER CONTEXT ACTIVE STATE and in EMM-CONNECTED mode and having performed IMS registration }

ensure that {

when { the UE receives a DEACTIVATE EPS BEARER CONTEXT REQUEST message indicating the EPS bearer identity of the PDN used for IMS and ESM cause set to ‘reactivation requested’ }

then { UE transmits a PDN CONNECTIVITY REQUEST and performs initial IMS registration using the first P-CSCF address received }

}

10.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.229, clauses L.2.2.1, L.2.2.1C and TS 24.301, clauses 6.4.4.2, 6.4.4.3. Unless otherwise stated these are Rel-9 requirements.

[TS 24.301, clause 6.4.4.2]

If a NAS signalling connection exists when the MME initiates the EPS bearer context deactivation procedure, the MME shall initiate the EPS bearer context deactivation procedure by sending a DEACTIVATE EPS BEARER CONTEXT REQUEST message to the UE, start the timer T3495, and enter the state BEARER CONTEXT INACTIVE PENDING (see example in figure 6.4.4.2.1). The DEACTIVATE EPS BEARER CONTEXT REQUEST message contains an ESM cause typically indicating one of the following:

#8:		operator determined barring;

#26:	insufficient resources;

#36:	regular deactivation;

#38:	network failure;

#39:	reactivation requested;

#112:	APN restriction value incompatible with active EPS bearer context; or

#113:	Multiple accesses to a PDN connection not allowed.

If the deactivation is triggered by a UE initiated bearer resource modification procedure or UE requested PDN disconnect procedure, the DEACTIVATE EPS BEARER CONTEXT REQUEST message shall contain the procedure transaction identity (PTI) value received by the MME in the BEARER RESOURCE MODIFICATION REQUEST or PDN DISCONNECT REQUEST respectively.

When the MME wants to deactivate all EPS bearer contexts of a PDN connection and thus disconnect the UE from the PDN, the MME shall include the EPS bearer identity of the default bearer associated to the PDN in the DEACTIVATE EPS BEARER CONTEXT REQUEST message. In this case, the MME shall not include the WLAN offload indication in the DEACTIVATE EPS BEARER CONTEXT REQUEST message, and if the UE receives the WLAN offload indication, the UE shall ignore the indication.

If no NAS signalling connection exists when the MME initiates the EPS bearer context deactivation, the ESM entity in the MME shall locally deactivate the EPS bearer context towards the UE without any peer-to-peer ESM signalling between the MME and the UE.

NOTE:	The EPS bearer context state(s) can be synchronized between the UE and the MME at the next EMM-IDLE to EMM-CONNECTED transition, e.g. during a service request or tracking area updating procedure.

[TS 24.301, clause 6.4.4.3]

Upon receipt of the DEACTIVATE EPS BEARER CONTEXT REQUEST message, the UE shall delete the EPS bearer context identified by the EPS bearer identity. After deactivating the identified EPS bearer context, the UE shall respond to the MME with the DEACTIVATE EPS BEARER CONTEXT ACCEPT.

If the EPS bearer identity indicated in the DEACTIVATE EPS BEARER CONTEXT REQUEST is that of the default bearer to a PDN, the UE shall delete all EPS bearer contexts associated to the PDN. After deactivating all EPS bearer contexts, the UE shall respond to the MME with the DEACTIVATE EPS BEARER CONTEXT ACCEPT.

Upon sending the DEACTIVATE EPS BEARER CONTEXT ACCEPT message, the UE shall enter the state BEARER CONTEXT INACTIVE. If due to the EPS bearer context deactivation only the PDN connection for emergency bearer services remains established, the UE shall consider itself attached for emergency bearer services only.

If the DEACTIVATE EPS BEARER CONTEXT REQUEST includes ESM cause #39 "reactivation requested" and the EPS bearer context is a default EPS bearer context, and the UE provided an APN for the establishment of the PDN connection, the UE shall stop timer T3396 if it is running for the APN provided by the UE. The UE should then re-initiate the UE requested PDN connectivity procedure for the same APN as the deactivated default EPS bearer context to reactivate the EPS bearer context. If the UE did not provide an APN for the establishment of the PDN connection and the request type was different from "emergency", the UE shall stop the timer T3396 associated with no APN if it is running, and should re-initiate the UE requested PDN connectivity procedure without including an APN. Additionally, the UE should re-initiate the request(s) for dedicated bearer resources that have been activated on request of the UE and released as a result of this EPS bearer context deactivation procedure. If the DEACTIVATE EPS BEARER CONTEXT REQUEST message was received for an emergency PDN connection, the UE shall not stop the timer T3396 associated with no APN if it is running. The UE should then re-initiate the UE requested PDN connectivity procedure for the emergency PDN connection.

NOTE 1:	User interaction is necessary in some cases when the UE cannot re-activate the EPS bearer context(s) automatically.

…

[TS 24.229, clause L.2.2.1]

Prior to communication with the IM CN subsystem, the UE shall:

…

c)	acquire a P-CSCF address(es).

The methods for P-CSCF discovery are:

I.	When using IPv4, employ the Dynamic Host Configuration Protocol (DHCP) RFC 2132 [20F], the DHCPv4 options for SIP servers RFC 3361 [35A], and RFC 3263 [27A] as described in subclause 9.2.1. When using IPv6, employ Dynamic Host Configuration Protocol for IPv6 (DHCPv6) RFC 3315 [40], the DHCPv6 options for SIP servers RFC 3319 [41] and DHCPv6 options for Domain Name Servers (DNS) RFC 3646 [56C] as described in subclause 9.2.1.

II.	Transfer P-CSCF address(es) within the EPS bearer context activation procedure.

The UE shall indicate the request for a P-CSCF address to the network within the Protocol Configuration Options information element of the PDN CONNECTIVITY REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message.

If the network provides the UE with a list of P-CSCF IPv4 or IPv6 addresses in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message or ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message, the UE shall assume that the list is prioritised with the first address within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

…

The UE can freely select method I or II for P-CSCF discovery, if:

-	the UE is in the home network; or

-	the UE is roaming and the P-CSCF is to be discovered in the visited network.

…

[TS 24.229, clause L.2.2.1C]

A UE supporting the P-CSCF restoration procedure performs one of the following procedures:

A	if the UE used method II for P-CSCF discovery and if the UE receives one or more P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message the one or more P-CSCF addresse(s) do not include the address of the currently used P-CSCF, then the UE shall acquire a different P-CSCF address from the one or more P-CSCF addresse(s) in the Modify EPS Bearer Context Request message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority.

If the UE used method II for P-CSCF discovery and if the UE has previously sent the "P-CSCF Re-selection support" PCO indicator at PDN creation and if the UE receives one or more P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message, then the UE shall acquire a P-CSCF address from the one or more P-CSCF addresse(s) in the Modify EPS Bearer Context Request message. If more than one P-CSCF address with the same container identifier (i.e. "P-CSCF IPv6 Address" or "P-CSCF IPv4 Address") are included, then the UE shall assume that the more than one P-CSCF addresses with the same container identifier are prioritised with the first P-CSCF address with the same container identifier within the Protocol Configuration Options information element as the P-CSCF address with the highest priority;

B	if the UE uses RFC 6223 [143] as part of P-CSCF restoration procedures, and if the P-CSCF fails to respond to a keep-alive request, then the UE shall acquire a different P-CSCF address using one of the methods I, III and IV for P-CSCF discovery described in the subclause L.2.2.1.

When a the UE has acquired the P-CSCF address, the UE shall perform an initial registration as specified in subclause 5.1.

NOTE:	For UEs using procedure A described above, the network ensures that P-CSCF address(es) in the Protocol Configuration Options information element of a Modify EPS Bearer Context Request message is sent only during P-CSCF restoration procedures as defined in subclause 5 of 3GPP TS 23.380 [7D].

10.4.3.3	Test description

10.4.3.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18].

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

10.4.3.3.2	Test procedure sequence

Table 10.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-17 | Steps 1 to 16c1 of the generic test procedure for UE registration (TS 36.508 4.5.2.3-1) take place. | - | - | - | - |
| 18 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST including the EPS bearer identity of the default EPS bearer associated with IMS PDN with ESM cause #39. | <-- | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 19 | UE transmits DEACTIVATE EPS BEARER CONTEXT ACCEPT message | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |
| 20 | Check: Does the UE transmit a PDN CONNECTIVITY REQUEST? | --> | PDN CONNECTIVITY REQUEST | 1 | P |
| 21 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | <-- | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: In parallel to the event described in step 22 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 22 below the generic procedure for IMS signalling in the U-plane specified in table 10.4.3.3.2-2 takes place. | - | - | - | - |
| 22 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 23 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST including the EPS bearer identity of the internet PDN. | <-- | DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 24 | Check: Does the UE transmit a DEACTIVATE EPS BEARER CONTEXT ACCEPT? | --> | DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | - |

Table 10.4.3.3.2-2: Procedure for IMS signalling in the U-plane

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| - | The subsequent messages are sent to the first P-CSCF address received at step 21. | - | - | - | - |
| 1-9 | Registration procedure according TS 34.229-1 [43] subclause C.2 (steps 3-11). | - | - | 1 | P |

10.4.3.3.3	Specific message contents

Table 10.4.3.3.3-1: Message DEACTIVATE EPS BEARER CONTEXT REQUEST (step 18, Table 10.4.3.3.2-1)

| Derivation path: 36.508 table 4.7.3-12 and table 4.6.1-8 with condition DRB-REL(1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS bearer identity | EPS bearer identity | Same value as in ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST for the IMS PDN |  |
| Procedure transaction identity | 0 | No procedure transaction identity assigned |  |
| ESM cause | 00100111 | Reactivation requested |  |
| Protocol configuration options | Not present |  |  |

Table 10.4.3.3.3-2: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Step 21, Table 10.4.3.3.2-1)

| Derivation path: TS 36.508, Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol configuration options |  |  |  |
| Container ID 1 | ‘0001’H |  | P-CSCF IPv6 |
| Length of container ID 1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 1 contents | IPv6 address | P-CSCF IPv6 Address different to that allocated in the previous ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |
| Container ID 2 | ‘000C’H |  | P-CSCF IPv4 |
| Length of container ID 2 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 2 contents | IPv4 address | P-CSCF IPv4 Address different to that allocated in the previous ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |
| Container ID 3 | ‘0001’H |  | P-CSCF_IPv6 |
| Length of container ID 3 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 3 contents | IPv6 address | Additional P-CSCF IPv6 Address different to that allocated in the previous ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |
| Container ID 4 | ‘000C’H |  | P-CSCF_IPv4 |
| Length of container ID 4 contents |  | Length value determined by the TTCN implementation |  |
| Container ID 4 contents | IPv4 address | Additional P-CSCF IPv4 Address different to that allocated in the previous ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message |  |

| Condition | Explanation |
| --- | --- |
| P-CSCF_IPv6 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message for IMS, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv6 Address Request”. |
| P-CSCF_IPv4 | If in the last PDN CONNECTIVITY REQUEST or ESM INFORMATION RESPONSE sent prior to this message for IMS, the Protocol configuration options and the additional parameter list was included with a “P-CSCF IPv4 Address Request”. |

<End of modified section>