3GPP TSG RAN meeting #105										       RP-242344

Melbourne, Australia, September 9-12, 2024

## Status Report to TSG

Agenda item:			9.4.1.6

| WI / SI Name | Introduction of Ku Bands for NR NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ku_bands |  |  |  |  |
| Unique ID | 1040135 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241690 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 15% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Manook Soghomonian |
|  | Company | Intelsat |
|  | Email | Manook.soghomonian@intelsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

#### 2.1.1	Agreements

RAN WG4 Meeting #112, Maastricht, Netherlands, 19th - 23rd Aug, 2024:

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_General in [35] were approved:

#### Core part:

#### Issue 1-1: Work plan

#### The work plan in R4-2411506 is approved

#### Issue 1-2: Band definitions

Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S).

Additional band definitions are expected once the applicable regulations have been identified and studied.

#### Issue 2-2: Coexistence scenarios

Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies

#### Issue 2-4: Coexistence frequencies

#### Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band.

#### Issue 2-8: Coexistence TN channel model

For TN channel model, reuse existing version of TR 38.901.

#### Issue 2-9: Coexistence NTN channel model

For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information:

- Shadow fading and clutter loss tables for the reference frequencies

- Tropospheric scintillation.

#### Issue 2-10: TN Parameters

As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from:

- The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz

- The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

#### Issue 3-1: Frequency range definition

Following the scope of the WID, definition of a new frequency range e.g. FR3, is not in scope.

#### Issue 3-2: Numerology - Decision criteria

The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

#### Issue 3-3: Numerology – timing aspects

The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-4: Numerology – phase noise

The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-5: Numerology – efficient channel utilization

The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied.

#### Issue 3-6: Numerology– beam hopping granularity

An analysis of the impact of different SCS on performance, although not critical, can influence the decision on FR1 vs FR2 numerology.

The following agreements were made and documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [37]:

#### Issue 1-1-1: Supported VSAT Types in the NR NTN Ku band work

- The current NTN VSAT types in 38.101-5 are considered in this Ku band work item

  - Both mechanical and electrically steered antennas can be considered

  - The discussion on down selecting between FR1/FR2 approaches for the NTN Ku band will not be impacted by this WF.

#### Perforamance part:

No progress.

#### 2.1.2	Remaining Open issues

#### Core part:

#### Define additional bands for Ku

#### Completion of co-existence study

#### Issue 2-1: Coexistence scope

#### Issue 2-3: Coexistence assumptions

#### Issue 2-5: Coexistence SAN parameters

#### Issue 2-6: Coexistence UE parameters

#### Issue 2-7: Coexistence channel bandwidth

System parameters

Choose FR1 or FR2 numerology by RAN #106

Specify RF requirements

SAN RF requirements

UE RF Requirements

Specify RRM requirements

#### From [37] Issue 1-1-2: Open issues to be discussed

- Companies are encouraged to provide the following information for the NTN Ku band in the next meeting:

  - Equivalent antenna aperture for the Ku band VSAT

  - Aspects on whether the common Tx/Rx antenna or separate Tx/Rx antenna can be assumed for VSAT in Ku band

  - Parameterized array antenna model for electronic steering antenna (the table 8.1.1-1 in TR 38.921 can be referred as an example format)

  - The range of TRP/EIRP for the Ku band

  - The UL/DL budget

  - Regulatory requirements

#### Perforamance part:

- RRM performance requirements and test cases

- UE demodulation and CSI reporting requirements

- Satellite access node demodulation requirements

- Satellite access node conformance tests

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

#### Not applicable.

## 4.	References

| Ref | Tdoc | Subject | Source |
| --- | --- | --- | --- |
| [1] | R4-2411120 | Discussion on coexistence evaluations for Ku-band for NR NTN | CATT |
| [2] | R4-2411121 | Discussion on system parameters for Ku-band for NR NTN | CATT |
| [3] | R4-2411122 | Discussion on SAN RF core requirements for Ku-band for NR NTN | CATT |
| [4] | R4-2411188 | NTN Ku-band - Coexistence | Ericsson |
| [5] | R4-2411189 | NTN Ku-band - FR1-NTN vs FR2-NTN | Ericsson |
| [6] | R4-2411190 | NTN Ku-band - Regulatory aspects and bands definition | Ericsson |
| [7] | R4-2411191 | NTN Ku-band - SAN RF requirements | Ericsson |
| [8] | R4-2411192 | NTN Ku-band - UE RF requirements | Ericsson |
| [9] | R4-2411481 | Analysis of numerology options for Ku band | Eutelsat Group |
| [10] | R4-2411506 | Ku Band Work Plan | Intelsat |
| [11] | R4-2411507 | Ku Band Co-existence | Intelsat |
| [12] | R4-2411508 | Ku Band Parameters and evaluation of FR1-NTN and FR2-NTN numerologies | Intelsat |
| [13] | R4-2411509 | Channel Bandwidth Mapping | Intelsat |
| [14] | R4-2411658 | Discussion on Ku band for NR NTN | Nokia |
| [15] | R4-2411777 | Discussions on system parameters of Ku Band for NR NTN | MediaTek inc. |
| [16] | R4-2411858 | Discussion on coexistence study based on ITU regulations for NTN Ku band | ZTE Corporation, Sanechips |
| [17] | R4-2411859 | Discussion on system parameters for NTN Ku band | ZTE Corporation, Sanechips |
| [18] | R4-2411860 | Discussion on UE RF requirements for NTN Ku band | ZTE Corporation, Sanechips |
| [19] | R4-2411861 | Discussion on SAN RF requirements for NTN Ku band | ZTE Corporation, Sanechips |
| [20] | R4-2411877 | UE RF requirements for Ku band NTN | LG Electronics |
| [21] | R4-2411950 | Discussion on NR NTN system parameters for Ku band | Nokia |
| [22] | R4-2412079 | Discussion on system parameters for Ku band supporting NTN | vivo |
| [23] | R4-2412265 | Discussion on NR NTN Ku band plan & system parameters | CHTTL |
| [24] | R4-2412560 | Discussion on Ku-band co-ex studies | Samsung |
| [25] | R4-2412561 | Discussion on Ku-band UE RF | Samsung |
| [26] | R4-2412960 | Initial discussion on NR NTN Ku band | Huawei, HiSilicon |
| [27] | R4-2412961 | Initial discussion on VSAT requirements for NR NTN Ku band | Huawei, HiSilicon |
| [28] | R4-2412962 | Discussion on coexistence study for NR NTN Ku Band | Huawei, HiSilicon |
| [29] | R4-2412993 | Regulatory status of NTN in bands above 10 GHz post WRC-23 | Eutelsat Group |
| [30] | R4-2413217 | Consideration of Linear Polarization in Ku Band Coexistence Studies | SKY Perfect JSAT Corporation |
| [31] | R4-2413247 | Initial discussion on for Ku operating bands | Huawei, HiSilicon |
| [32] | R4-2413414 | Topic summary for [112][314] NR_NTN_Ku_Band_General | Moderator (Eutelsat) |
| [33] | R4-2413415 | Topic summary for [112][315] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL) |
| [34] | R4-2413458 | Ku Band Channel Allocation for Legacy Satellite System | SKY Perfect JSAT Corporation |
| [35] | R4-2413519 | Way Forward for [112][314] NR_NTN_Ku_Band_General | Eutelsat |
| [36] | R4-2413520 | Ad-hoc meeting minutes for [112][314] NR_NTN_Ku_Band_General | Eutelsat |
| [37] | R4-2413525 | Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template