3GPP TSG RAN meeting #105										RP-242398

Melbourne, Australia, September 9-12, 2024						(Revision of RP-241910)

## Status Report to TSG

Agenda item:			9.3.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air |  |  |  |  |
| Unique ID | 1020093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240774 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: Sept ‘25 | Performance part: March ‘26 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 33% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2/RAN3) |
|  | Company | Qualcomm; CAICT; Ericsson |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Additional RAN2 TUs per RAN2 Chair request: 2.5 TUs/meeting for the rest of the Release.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

It is suggested to approve the CR(s) for TR 38.843 submitted by the rapporteurs in RAN#105 incorporating the endorsed agreements / TPs at the WGs.

## 2.1	RAN1

#### 2.1.1	Agreements

// Specification support for beam management

Agreement

For UE sided model in beam management, support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Agreement

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement

For UE-sided model at least for BM Case-1, for inference results report

- Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

  - FFS whether support only resource set for Set B is configured

- UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

- The beam information in the inference report refers to the resource set for Set A

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including:

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - Note: resources for Set B for monitoring are not precluded and can be study.

  - Note: this is only applicable when the model can predict RSRP

- Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam

  - Note: this is only applicable when the model can generate probability information

- FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including

  - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference.

- For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Agreement

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

// Specification support for positioning accuracy enhancement

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Conclusion

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring,

- Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

Agreement

For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement,

- For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective

Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

Agreement

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples

- The Nt samples have timing granularity T.

- FFS: the starting time of the list of Nt samples

- FFS: the value range of Nt

For the sample-based measurement (if accepted in Rel-19),

- For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power.

- Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

Agreement

For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

Conclusion

From RAN1 perspective, for Case 3a measurements,

- The existing procedures can be reused in terms of SRS configuration.

  - Note: parameter values for SRS configuration can be further discussed

- These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures.

- Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

Agreement

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

// Additional study on AI/ML for NR air interface: CSI prediction

Observation

For the generalization verification of CSI prediction using UE sided model over various UE speeds, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h and

    - UE speed#A is 30 km/h or 120km/h, 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -5.5% degradation, and 1 source [LG Electronics] observes -16.2% degradation.

    - UE speed#A is 60 km/h, 2 sources [CMCC, LG Electronics] observe -26.79%~-13.3% degradation.

  - If UE speed#B is 30 km/h and

    - UE speed#A is 60km/h, 4 sources [Huawei, CATT, Ericsson, vivo] observe a generalized performance of -11.4%~-2.7% degradation and 4 sources [CMCC, Nokia, LG Electronics, ZTE] observe a generalized performance of -33.6%~-19% degradation.

    - UE speed#A is 10km/h or 120km/h, 3 sources [CMCC, MediaTek, Fujitsu] observe a generalized performance of -72.37%~-51.5% degradation.

  - If UE speed#B is 60 km/h and

    - UE speed#A is 10km/h, 1 source [LG Electronics] observes a generalized performance of -16.4 degradation

    - UE speed#A is 30km/h, 1 source [ZTE] observes a generalized performance of -5.4% degradation and 6 sources [Huawei, Fujitsu, MediaTek, Ericsson, vivo, LG Electronics, CATT] observe a generalized performance of -56.3%~-13.8% degradation

  - If UE speed#B is 120 km/h and

    - UE speed#A is 30km/h, 1 source [MediaTek] observes a generalized performance of -56.3% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-3.8% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 8 sources [Fujitsu, Huawei, CATT, vivo, Apple, Ericsson, ZTE, LG Electronics]

  - If UE speed#B is 10 km/h

    - 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -0.2% degradation, 1 source [LG Electronics] observe -5.1% degradation, and 1 source [CMCC] observes -16.87% degradation.

  - If UE speed#B is 30 km/h

    - 9 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson, MediaTek, ZTE, LG Electronics] observe a generalized performance of less than -3.8% degradation, 1 source [CMCC] observes a generalized performance of -15.44% degradation.

  - If UE speed#B is 60 km/h and

    - 8 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson, ZTE, LG Electronics] observe a generalized performance of less than -3.63% degradation, 3 sources [CMCC, MediaTek] observe a generalized performance of -13.5%~-6.77% degradation.

  - If UE speed#B is 120 km/h and

    - 1 source [MediaTek] observes a generalized performance of -43.6% degradation

  - Note: For generalization Case 3, 2 sources [CMCC, MediaTek] observe performance degradations (-43.6%~-13.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, 120 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 4 sources [vivo, Ericsson, MediaTek, ZTE] consider spatial consistency. Other sources do not consider spatial consistency.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - Note: Results refer to Table 3-1 of R1-2407338

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from perspective of phase discontinuity modelling

- If phase discontinuity modelling is not adopted

  - For 30km/h UE speed,

    - 2 sources [vivo, Fujitsu] observe 4.24%~4.3% gain with N4=1

    - 1 source [vivo] observes 3.1% ~ 40.5% with N4=4

    - 1 source [Qualcomm] observes -1.2%~6.3% with N4=1 depending on filter updates between 10 and 70 learning window size

  - For 60km/h UE speed,

    - 1 source [vivo] observe 13.5% gain with N4=1, and 1 source [vivo] observe 8.2%~34.6% gain with N4=4

    - 1 source [Qualcomm] observes 0%~8% with N4=1 depending on filter updates between 10 and 70 learning window size

- If phase discontinuity modelling is adopted,

  - For 30km/h UE speed,

    - Without phase adjustment,

        - 2 sources [vivo, Fujitsu] observe 7.26%~7.8% gain with N4=1

        - 1 source [vivo] observes 6.5%~23.5% gain with N4=4

        - 1 source [Qualcomm] observes -0.8%~8% with N4=1 depending on filter updates between 10 and 70 learning window size

    - With phase adjustment,

      - 1 source [Qualcomm] observes 1.2%~7.7% gain with N4=1 depending on filter updates between 10 and 70 learning window size

      - 1 source [Fujitsu] observes 25.6% gain

  - For 60km/h UE speed,

  - Without phase adjustment,

  - 1 source [vivo] observe 6.6% gain with N4=1 and 6.4%~55% with N4=4

  - 1 source [Qualcomm] observes 5.7%~17.5% gain with N4=1 depending on filter updates between 10 and 70 learning window size

  - With phase adjustment, 1 source [Qualcomm] observes 4.2%~11.9% gain with N4=1 depending on filter updates between 10 and 70 learning window size

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix as model input is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Note: Results refer to Table 2-12 of R1-2407338

- Note: N4 refers to the number of predicted CSI instances

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from channel estimation perspective

- If ideal channel estimation is adopted

  - For N4=1

    - 9 sources [vivo, Nokia, Intel, Fujitsu, InterDigital, Qualcomm, OPPO, Spreadtrum, Lenovo] observe -1.2%~6.3% gain

    - 1 source [Apple] observes -2% ~17% gain depending on filter complexity and filter update

  - For N4=4

    - 2 sources [OPPO, Intel] observe -1.6%~5.56%

    - 1 source [InterDigital] observes -4.5%~6.93%

    - 1 source [vivo] observes 3.1%~40.5%

- If realistic channel estimation is adopted,

  - For N4=1

    - 5 sources [Intel, Fujitsu, Ericsson, MediaTek, CATT] observe 0.43%~7.26% gain

    - 2 sources [vivo, Nokia] observe 15.2%~19.5 gain

    - 1 source [InterDigital] observes 68% gain

    - 1 source [ZTE] observes 5.8%~16.4% gain depending on traffic load

  - For N4=4

    - 2 sources [Intel, MediaTek] observe 0.1%~1.4% gain

    - 1 source [Ericsson] observes 5%~29% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix as model input and UE speed of 30km/h is assumed.

  - 3 sources [vivo, ZTE, Ericsson] consider spatial consistency, and other sources do not consider spatial consistency.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-3 of R1-2407338

- Note: N4 refers to the number of predicted CSI instances

Conclusion

For computational complexity of both AI/ML and non-AI/ML based CSI prediction, to report the number of FLOPs assuming whole bandwidth and one prediction sample.

Observation (Update of the observation made in RAN1#117)

- For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective,

  - If spatial consistency is not adopted, and if N4=1

    - For 10km/h UE speed, 1 source [InterDigital] observes 6% gain

    - For 30km/h UE speed, 9 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, Qualcomm, NTT Docomo] observe 5.9%~20.6% gain and 4 sources [Intel, LG Electronics, ZTE, Fujitsu] observe 23.2%~35.4% gain. 2 sources [Nokia, InterDigital] observe 54%~106% gain

    - For 60km/h UE speed, 4 sources [Qualcomm, China Telecom, Huawei, ZTE] observe 10.5%~26.53% gain

  - If spatial consistency is adopted, and if N4=1

    - For 30km/h UE speed, 2 sources [Ericsson, ZTE] observes 22.93%~23% gain, 1 source [vivo] observe 68.5% gain

    - For 60km/h UE speed, 3 sources [Ericsson, vivo, ZTE] observe 16.2~24.3% gain

  - If spatial consistency is not adopted, and if N4=4

      - For 10km/h UE speed, 1 source [LG Electronics] observes 9.1%~29.7% gain

      - For 30km/h UE speed, 1 source [NTT Docomo] observes 4.3%~12.3% gain, and 2 sources [OPPO, Intel] observe 17.6%~35.4%gain

  - If spatial consistency is adopted, and if N4=4

      - For 10km/h UE speed, 1 source [Samsung] observes -1.61%~62.9% gain

      - For 30km/h UE speed, 1 source [Ericsson] observes 23%~34% gain, 1 source [MediaTek] observe 20.9%~76.4% gain

      - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 5.96%~-22% gain,

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 20ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 7 sources [ZTE, Ericsson, Intel, vivo, Fujitsu, Samsung, MediaTek] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [NTT Docomo] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, 1 source [Samsung] considers per layer raw channel matrix after pre-processing, and other sources do not consider pre/post processing.

  - The performance metric is SGCS in linear value for layer 1.

- Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-1 of R1-2407338

Observation (Update of the observation made in RAN1#117)

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#1 of the nearest historical CSI:

- For FTP traffic with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1

    - 2 sources [Intel, NTT Docomo] observe -0.2%~1% gain

    - 4 sources [Ericsson, Fujitsu, Apple, Nokia] observe 4.4%~9% gain.

  - For 60km/h UE speed, and N4=1

    - 2 sources [Huawei, Ericsson] observe 1.2%~5% gain;

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 0.3% gain.

    - 1 source [Ericsson] observes 7% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 5% gain.

- For FTP traffic with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1

    - 2 sources [Intel, NTT Docomo] observe 0.9%~2.3% gain

    - 1 source [Apple] observe 8.5% gain

    - 2 sources [Ericsson, Fujitsu] observe 22.3%~37% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observe 1.8%~3.5% gain;

    - 1 source [Ericsson] observes 21% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 2.3% gain.

    - 1 source [Ericsson] observes 29% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 21% gain.

- For FTP traffic with high RU (RU>=70%)

  - For 30km/h UE speed and N4=1

    - 4 sources [Intel, NTT Docomo, OPPO] observe 2%~3.6% gain

    - 2 sources [Fujitsu, Apple] observe 8.5%~31.7% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 7.2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 4.2% gain;

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 3 sources [Lenovo, Fujitsu, vivo] observe 27%~51% gain.

    - 2 sources [MediaTek, Intel] observe 1.2%~8.7% gain.

  - For 60km/h UE speed and N4=1

    - 1 source [vivo] observe 12.1% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [Fujitsu] observes 11.6% gain.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, OPPO, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-5/2-7 of R1-2407338

Observation (Update of the observation made in RAN1#117)

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of 5% UE UPT, gains are observed compared to Benchmark#1 of the nearest historical CSI:

- For FTP traffic with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1

    - 1 source [NTT Docomo] observes 1.5% gain.

    - 4 sources [Intel, Fujitsu, Apple, Nokia] observe 6.7%~15.7% gain.

    - 1 source [Ericsson] observes 27% gain.

  - For 60km/h UE speed, and N4=1

    - 2 sources [Huawei, Ericsson] observe 9%~18.3% gain;

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 3.6% gain.

    - 1 source [Ericsson] observes 14% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 4% gain.

- For FTP traffic with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1

    - 1 source [NTT Docomo] observes 4.1% gain.

    - 2 sources [Intel, Fujitsu, Apple] observe 14.3%~38.9% gain.

    - 1 source [Ericsson] observes 100% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 11.3% gain;

    - 1 source [Ericsson] observes 45% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 10% gain.

    - 1 source [Ericsson] observes 77% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 26% gain.

- For FTP traffic with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 1 source [Fujitsu] observes 51.7% gain.

    - 3 sources [Intel, NTT Docomo, Apple] observe 7.8%~15.8% gain

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observe 17.8% gain;

  - For 30km/h UE speed, and N4=4

    - 1 source [NTT Docomo] observes 6.2% gain

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 4 sources [Fujitsu, MediaTek, Lenovo, Intel] observe 7.7%~10.5% gain.

    - 1 source [vivo] observes 39.7% gain.

  - For 60km/h UE speed and N4=1

    - 1 source [vivo] observes 20.6% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [Fujitsu] observes 2.1% gain.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, OPPO, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-5/2-7 of R1-2407338

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from UE speed perspective

- If spatial consistency is not adopted, if N4=1

  - For 10km/h UE speed, 1 source [InterDigital] observes 0% gain

  - For 30km/h UE speed,

    - 7 sources [Lenovo, Spreadtrum, Intel, CATT, Fujitsu, ZTE, Nokia] observe 1%~19.5% gain,

    - 1 source [InterDigital] observes 68% gain

    - 2 sources [Apple, Qualcomm] observe -2% ~17% gain depending on filter complexity and filter update

  - For 60km/h UE speed,

    - 3 sources [Huawei, CATT, Qualcomm, ZTE] observe 3.6%~26.35% gain,

    - 1 source [InterDigital] observes 35% gain

    - 1 source [Qualcomm] observe 0% ~8.7% gain depending on filter complexity and filter update

- If spatial consistency is adopted, if N4=1

  - For 30km/h UE speed,

    - 1 source [Ericsson] observes 5% gain,

    - 2 sources [vivo, ZTE] observe 15.2%~16.4%

  - For 60km/h UE speed,

    - 3 sources [Ericsson, vivo, ZTE] observes 3%~13.5% gain,

- If spatial consistency is not adopted, if N4=4

  - For 10km/h UE speed, 1 source [InterDigital] observes 0%~1.4% gain

  - For 30km/h UE speed, 3 sources [InterDigital, Intel, OPPO] observe -1.6%~6.93% gain,

  - For 60km/h UE speed, 1 source [InterDigital] observes -4%~7.3% gain

- If spatial consistency is adopted, if N4=4

  - For 30km/h UE speed,

    - 1 source [MediaTek] observes 0.43%~0.97% gain

    - 1 source [Ericsson] observes 5%~29% gain

    - 1 source [vivo] observes 28.9%~43.6% gain

  - For 60km/h UE speed,

    - 1 source [MediaTek] observes -1.01%~1.52% gain

    - 1 source [Ericsson] observes 3%~16% gain

    - 1 source [vivo] observes 10.0%~32.3% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 6 sources [ZTE, Ericsson, Intel, vivo, Fujitsu, CATT] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - All sources consider Raw channel matrix as model input.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - The performance metric is SGCS in linear value for layer 1.

- Note: N4 refers to the number of predicted CSI

- Note: Results refer to Table 2-3 of R1-2407338

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction approach:

- For FTP traffic, with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1,

    - 2 sources [Ericsson, InterDigital] observe 7.6%~9% gain.

    - 4 sources [CATT, Intel, Fujitsu, Nokia] observe -2.41%~1.1% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [InterDigital] observe -3.4% gain

    - 2 sources [Huawei, CATT] observes 0.9%~1.2% gain;

    - 1 source [Ericsson] observes 11% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 13% gain.

    - 2 sources [InterDigital, MediaTek] observe 0%~0.3% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson,] observes 13%

    - 1 source [MediaTek] observes 0.14% gain

- For FTP traffic, with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1,

    - 1 source [Ericsson] observes 24% gain.

    - 3 sources [InterDigital, Intel, Fujitsu] observe 0.2%~5.1% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [InterDigital] observes -29.4% gain;

    - 1 source [Huawei] observes 3.1% gain;

    - 1 source [Ericsson] observes 31% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 35% gain.

    - 2 sources [MediaTek, InterDigital] observe -0.25%~2% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [InterDigital] observes -16.2% gain.

    - 1 source [MediaTek] observes 0.25% gain.

    - 1 source [Ericsson] observes 32% gain.

- For FTP traffic, with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 4 sources [InterDigital, Intel, Fujitsu, OPPO] observes -0.2%~9.2% gain;

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 2.5% gain;

  - For 30km/h UE speed and N4=4,

    - 2 sources [InterDigital, MediaTek] observes -0.8%~0.11% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [InterDigital] observes -19.1% gain.

    - 1 source [MediaTek] observes 0.92% gain.

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 2 sources [Lenovo, ZTE] observe 10.6%~24% gain.

    - 2 sources [Fujitsu, vivo] observe 7.8%~9.7% gain.

    - 3 sources [CATT, MediaTek, Intel] observe -0.6%~1.2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [CATT] observes 0.2% gain

    - 1 source [vivo] observes 8.4% gain

  - For 30km/h UE speed and N4= 4

    - 1 source [Fujitsu] observes 7% gain.

    - 1 source [vivo] observes 6.8% gain

  - For 60km/h UE speed and N4=4

    - 1 source [vivo] observes 11.6% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-247339

- Note: The following boxchart shows the median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of 5% UE UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction approach:

- For FTP traffic, with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1,

    - 2 sources [Ericsson, InterDigital] observe 18%~26% gain.

    - 4 sources [CATT, Fujitsu, Intel, Nokia] observe -1.1%~7.6% gain.

  - For 60km/h UE speed, and N4=1

    - 3 sources [Huawei, InterDigital, CATT] observe 1.9%~5.2% gain;

    - 1 source [Ericsson] observes 17% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 23% gain.

    - 1 source [InterDigital] observes -3.7% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observe 19%gain.

    - 1 source [InterDigital] observes 4%gain.

- For FTP traffic, with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1,

    - 1 source [Ericsson] observes 46% gain.

    - 1 source [InterDigital] observes 18.7% gain.

    - 2 sources [Intel, Fujitsu] observe 4%~6.6% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 8.6% gain;

    - 2 sources [Ericsson, InterDigital] observe 23%~66% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 73% gain.

    - 1 source [InterDigital] observes -9% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 56% gain.

    - 1 source [InterDigital] observes 11.5% gain.

- For FTP traffic, with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 1 source [Intel] observes 1.9% gain;

    - 2 sources [InterDigital, Fujitsu] observe 20.7%~26.3% gain;

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei, InterDigital] observes 14.8%~16% gain;

  - For 30km/h UE speed and N4=4,

    - 1 source [InterDigital] observes 0.9% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [InterDigital] observes 22.8% gain.

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 2 sources [vivo, ZTE] observe 15.7%~16.1% gain.

    - 4 sources [Lenovo, Fujitsu, CATT, Intel] observe 0.2%~6% gain.

    - 1 source [MediaTek] observes -2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [CATT] observes 0.4% gain

    - 1 source [vivo] observes 11.6% gain

  - For 30km/h UE speed and N4= 4

    - 1 source [Fujitsu] observes 6.3% gain.

    - 1 source [vivo] observes 21% gain

  - For 60km/h UE speed and N4=4

    - 1 source [vivo] observes 26.7% gain

- Note: the above results are based on the following assumptions

- The observation window considers to start as early as 15ms~50ms.

- A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

- Raw channel matrix is considered as model input

- 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

- 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

- 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- 5 sources [Ericsson, Fujitsu, CATT, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

- 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-2407339

- Note: The following boxchart shows the median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

Observation

- From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. The actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations.

- From a perspective of complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss. For example, 7 sources adopt the algorithm subject to the computational complexity of filter updates and inference in units of FLOPs from 0.47M to 106M and 0.067M to 3M, respectively.

- Results refer to Figure 2-1, Table 2-9, and Table 2-10 in R1-2407339.

Observation

For the generalization verification of CSI prediction using UE sided model over various deployment scenarios, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B applied for inference with a same deployment scenario#B

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - For deployment scenario#B is UMa

    - 2 sources [Ericsson, MediaTek] observe -1.88%~0% degradation

    - 1 source [MediaTek] observe -6.8% degradation

  - For deployment scenario#B is UMi

    - 1 source [MediaTek] observe 0% degradation

    - 3 sources [Ericsson, vivo, ZTE] observe -4.85%~-3.03% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (-1.95%~0% loss) for deployment scenario#B subject to any of UMa and UMi, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B as observed by 3 sources.

  - Minor loss (0%~-1.95%) are observed by 3 sources [vivo, ZTE, MediaTek].

  - Note: Moderate degradations of -5.2% are observed by 1 source [vivo] for deployment scenario#B subject to Uma

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - Note: Results refer to Table 3-2 of R1-2407339

Observation

For the generalization verification of CSI prediction using UE sided model over various carrier frequency, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B applied for inference with a same carrier frequency#B

- For generalization Case 2, significant degradations are suffered in general from the perspective of the layouts of antenna ports, as observed by 3 sources:

  - For carrier frequency#B is 2GHz

    - 1 source [MediaTek] observe -11.4% degradation

    - 1 source [vivo] observe -80.53% degradation

  - For carrier frequency#B is 3GHz or 4GHz

    - 2 sources [MediaTek, vivo] observe -34.23%~-80.53% degradation

    - 1 source [ZTE] observe -4.21% degradation

- For generalization Case 3, generalized performance may be achieved for some certain combinations of carrier frequency#A and carrier frequency#B but not for others, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B

  - For carrier frequency#B is 2GHz

    - 1 source [MediaTek] observe -0.5% degradation

    - 1 source [vivo] observe -9.27% degradation

  - For carrier frequency#B is 3GHz or 4GHz

    - 2 sources [MediaTek, ZTE] observe -1.93%~-5.1% degradation

    - 1 source [vivo] observe -14.94% degradation

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 2 sources [vivo, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - Note: Results refer to Table 3-3 of R1-2407339

Observation

For the generalization verification of CSI prediction using UE sided model over multiple aspects, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to certain aspects #B applied for inference with the same aspects #B,

- For generalization Case 2,

  - 1 source [NTT DOCOMO] observes -9.8% ~ -1.5% degradation when the aspects #A is (2 GHz carrier frequency, 100% outdoor UE) and the aspects #B is (4GHz carrier frequency, 20% outdoor UE+80% indoor UE)

  - 1 source [NTT DOCOMO] observes -10.7%~-1.8% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 100 outdoor UE)

  - 1 source [NTT DOCOMO] observes -21.2%~-2.4% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

  - 1 source [Nokia] observe -3% degradation when aspects#A is (30km/h UE speed, 100% UE in a car, 2GHz carrier frequency) and the aspects#B is (3km/h UE speed, 20% outdoor UE, 4GHz carrier frequency)

- For generalization Case 3,

  - 1 source [NTT DOCOMO] observes -1.1%~0% when the aspects#A is (2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects #B is (4GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

  - 1 source [NTT DOCOMO] observes -2.4%~-0.8% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 100 outdoor UE)

  - 1 source [NTT DOCOMO] observes -3.4%~-0.3% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 1 source [NTT Docomo] considers eigenvector as model input, and 1 source [Nokia]s considers Raw channel matrix as model input.

  - Note: Results refer to Table 3-4 of R1-2407339

Observation

For the CSI prediction using CSI-RS with 20ms periodicity, till the RAN1#118 meeting, in terms of mean and 5% UE UPT, gains are observed compared to Benchmark #1 of the nearest historical CSI and Benchmark #2 of non-AI/ML based CSI prediction,

- Compared to Benchmark 1

- In terms of UPT gain

  - For N4=1 and UE speed of 3km/h, 1 source [Ericsson] observes -18%~-7% and -26%~-8% degradation in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 3km/h, 1 source [Ericsson] observes 1%~6% and 6%~17% gain in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 30km/h, 1 source [Ericsson] observes 2%~17% and 13%~19% gain in terms of mean and 5% UE UPT, respectively, by using one 20ms periodic CSI-RS and three aperiodic CSI-RS.

- Compared to Benchmark 2

- For N4=1 and UE speed of 3km/h,

  - 1 source [Ericsson] observes 10%~19% and 23%~31% gain in terms of mean and 5% UE UPT, respectively.

  - 1 source [InterDigital] observes -2.7%~-1% and -10.4%~-4.9% degradation in terms of mean and 5% UE UPT, respectively.

- For N4=1 and UE speed of 10km/h, 1 source [InterDigital] observes 0.6%~1% gain and -14.2%~-8% gain in terms of mean and 5% UE UPT, respectively.

- For N4=4 and UE speed of 3km/h, 1 source [Ericsson] observes 14%~30% and 19%~56% gain in terms of mean and 5% UE UPT, respectively.

- For N4=4 and UE speed of 30km/h,

  - 1 source [ZTE] observes 17.3% and 16.1 gain in terms of mean and 5% UE UPT, receptively.

  - 1 source [Ericsson] observes 5%~19% and 13%~40% gain in terms of mean and 5% UE UPT, respectively, by using one 20ms periodic CSI-RS and three aperiodic CSI-RS.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

- 1 source [ZTE] considers full buffer model, other sources consider FTP traffic model

- Raw channel matrix is used as the model input.

- 5 sources [Ericsson, ZTE] consider realistic channel estimation, and other sources consider ideal channel estimation.

- The performance metric is SGCS in linear value for layer 1/2/3/4 and mean UPT, 5% UPT.

- 2 sources [Ericsson] consider spatial consistency. Other sources do not consider spatial consistency.

- Note: Results refer to Table 2-13, 2-14, 2-15, 2-16 of R1-2407339

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction, from channel estimation perspective:

- For FTP traffic, with low RU (RU<=39%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Nokia, InterDigital] observe -2.41%~1.8% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [InterDigital, Huawei] observe -3.4%~0.9% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 0.3% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Ericsson, InterDigital] observe 7.6%~9% gain.

      - 3 sources [CATT, Intel, Fujitsu] observe 0%~1.1% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [InterDigital, CATT] observe -3.4%~1.2% gain

      - 1 source [Ericsson] observes 11% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 13% gain.

      - 1 source [MediaTek] observes 0% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 13%

      - 1 source [MediaTek] observes 0.14% gain

- For FTP traffic, with mid RU (40<=RU<=69%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes -4.5% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -7.1% gain

      - 1 source [Huaweil] observes 3.1% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -2% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Ericsson] observe 24% gain.

      - 3 sources [InterDigital, Intel, Fujitsu] observe 0.2%~5.1% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Ericsson] observes 31% gain

      - 1 source [InterDigital] observes -29.4% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 35% gain.

      - 2 sources [MediaTek, InterDigital] observe -0.25%~1.1% gain

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 32%

      - 1 source [MediaTek] observes 0.25% gain

- For FTP traffic, with high RU (RU>=70%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes -4.8% gain

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -9% gain

      - 1 source [Huaweil] observes 2.5% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -0.8% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 3 sources [InterDigital, Intel, OPPO] observe 0%~0.8%

      - 1 source [Fujitsu] observes 9.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -9% gain

    - For 30km/h UE speed and N4=4,

      - 2 sources [MediaTek, InterDigital] observe 0%~0.1% gain

    - For 60km/h UE speed, and N4=4

      - 1 source [MediaTek] observes 0.92% gain

- For full buffer model,

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [Lenovo] observes 24%

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1

      - 3 sources [Fujitsu, vivo, ZTE] observe 7.8%~10.6% gain.

      - 3 sources [CATT, MediaTek, Intel] observe -0.6%~1.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 0.2% gain

      - 1 source [vivo] observes 8.4% gain

    - For 30km/h UE speed and N4= 4

      - 1 source [Fujitsu] observes 7% gain.

      - 1 source [vivo] observes 6.8% gain

    - For 60km/h UE speed and N4=4

      - 1 source [vivo] observes 11.6% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-2407340

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of 5% UE UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction, from channel estimation perspective:

- For FTP traffic, with low RU (RU<=39%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observe -5.5% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [InterDigital, Huawei] observe 4%~4.3% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -3.7% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Ericsson] observe 17% gain.

      - 2 sources [CATT, InterDigital] observe 0%~4% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 1.9% gain

      - 1 source [Ericsson] observes 17% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 23% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 19%

- For FTP traffic, with mid RU (40<=RU<=69%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes -12.9% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes 2.6% gain

      - 1 source [Huaweil] observes 8.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -9% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Intel, Fujitsu] observe 4%~6.6% gain.

      - 1 source [InterDigital] observes 18.7% gain.

      - 1 source [Ericsson] observes 46% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Ericsson] observes 66% gain

      - 1 source [InterDigital] observes 2.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 73% gain.

      - 1 source [InterDigital] observes 18.7% gain

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 56%

- For FTP traffic, with high RU (RU>=70%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes 3.6% gain

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -10.7% gain

      - 1 source [Huaweil] observes 14.8% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 0.9% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [InterDigital, Fujitsu] observes 20.7%~26.3%

      - 1 source [Intel] observes 1.9% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes 3.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 0.9% gain

- For full buffer model,

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [Lenovo] observes 0.2%

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1

      - 2 sources [vivo, ZTE] observe 15.7%~16.1% gain.

      - 3 sources [Fujitsu, CATT, Intel] observe 2.6%~7.7% gain.

      - 1 source [MediaTek] observes -2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 0.4% gain

      - 1 source [vivo] observes 11.6% gain

    - For 30km/h UE speed and N4= 4

      - 1 source [Fujitsu] observes 6.3% gain.

      - 1 source [vivo] observes 21% gain

    - For 60km/h UE speed and N4=4

      - 1 source [vivo] observes 26.7% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-2407340

Observation

The following aspects have been studied for CSI prediction using UE-sided model:

- From the perspective of basic performance gain over non-AI/ML benchmark (without considering generalization),

  - It has been studied with corresponding observations on:

    - the metrics of SGCS, mean UPT, 5% UPT;

    - the benchmarks of nearest historical CSI and non-AI/ML based CSI prediction.

    - The impact of channel estimation error, phase discontinuity, spatial consistency, UE Speed, observation window, prediction window, CSI-RS periodicity.

- It has been studied with corresponding observations on complexity for both AI/ML based CSI prediction and non-AI/ML based CSI prediction.

- It has been studied on localized model including evaluation methodology, but is lack of observations.

- From the perspective of generalization over various scenarios,

  - It has been studied with corresponding observations on (with the metric of SGCS):

    - the scenario including various UE speeds, deployment scenarios, carrier frequency

- From the perspective of performance monitoring, it has been studied on boundary between Type 1 and 3 performance monitoring, and potential specification impact for each performance monitoring type 1,2 and 3.

Observation

Based on the evaluation for CSI prediction, the following high-level observations are provided:

- From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources.

- From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss.

- From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity.

- From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others.

  - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

Agreement

From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

// Additional study on AI/ML for NR air interface: CSI compression

Agreement

For temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, among Cases 1, 2, 3, 4, and 5, prioritize further discussion on Case 2 and Case 3.

Agreement

For studying the standardization of model structure RAN1, RAN1 assumes at least the following:

- Precoding matrix as an input (as opposed to raw channel matrix)

  - Per-layer processing, with common structure across ranks and layers (corresponding to Option 2-1, 2-2, 3-1, or 3-2 for handling rank ≥ 1)

- For temporal domain aspects Case 2 and Case 3, strive to reuse the model structure of Case 0 where appropriate, with additional layers or operations either at the input/output domain or at the latent domain.

- For Case 0, use precoding matrix, e.g., eigen vector in spatial-frequency domain, and/or angular, delay domain representation such as eType-II W2.

- For Case 2 and Case 3, use precoding matrix, e.g., spatial-frequency domain for each CSI observation instance, and/or angular, delay, and Doppler domain representation such as eType-II W2.

- Consider scalability approaches over numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

Observation

Option 5 alone doesn’t fully address inter-vendor collaboration complexity, but Option 5 has potential performance benefit and flexibility compared to Option 3.

Agreement

Continue the study of following directions - on-device operation and UE side offline engineering

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4)

  - Potential down-selection into one or more among sub-options 3a/5a-1, 3a/5a-2, 3a/5a-3, 4-1, 4-2, and 4-3 considering their feasibility and performance, including at least the following issues

    - [Issue 1] What additional information should be shared from NW-side to UE-side to enable UE-side encoder training, validation, and testing?

    - [Issue 2] Is there concern for NW’s proprietary information disclosure, and if so, how to address it?

    - [Issue 3] Is there an overhead concern, and if so, how to address it?

    - [Issue 4] Is there performance impact due to mismatch between NW side data distribution and UE side data distribution, and if so, how to address it?

- Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues

  - [Issue 3] Is there an overhead concern, and if so, how to address it?

  - [Issue 5] Whether it is feasible to use a common encoder across UEs, and whether it is feasible for NW-side to train multiple encoders for different UEs?

  - [Issue 6] Is there performance impact due to mismatch between NW side data distribution and UE side inference data distribution, and if so, how to address it?

  - [Issue 7] Is there concern for NW’s and UE’s proprietary information disclosure, and if so, how to address it?

- Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues

  - [Issue 8] Whether to consider 3GPP’s statistical channel model or field data for reference model(s) training. In case of the latter, how does RAN1 collect field data and agree on them?

  - [Issue 9] Is there performance impact due to mismatch between the distribution of the dataset used for reference model(s) training, UE-side data distribution, and NW-side data distribution, and if so, how to address it?

  - [Issue 10] What additional information should be specified to enable UE-side encoder training, validation, and testing, and NW-side decoder training, validation, and testing?

  - Note:

    - 1-1: Only reference encoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference encoder.

    - 1-2: Only reference decoder is specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately compatible to the reference decoder.

    - 1-3: Both reference encoder and reference decoder are specified, and NW-side and/or UE-side may train their actual CSI generation part and actual CSI-reconstruction part separately that are compatible to the reference decoder/encoder.

- Note: UE-side data and NW-side data in “UE-side data distribution” and “NW-side data distribution” are field data.

- Note: Some issues identified in one direction may/may not be applicable for other Direction.

- Note: potential down selection among the 3 directions is not precluded

- Study of data distribution mismatch to consider the use of synthetic data and/or field data.

Agreement

Adopt all the observations in Section 2 of R1-2407499.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, study the performance impact resulting from non-ideal UCI feedback.

- Scenario A: no UCI loss

  - Note: Corresponds to an upper bound or re-aligning missing historical CSI information

- Scenario B: UCI loss, known at NW and unknown at UE, with mitigation at NW

  - Note: Corresponds to implementation-based mitigation at NW but no signaling to UE.

- Scenario C: UCI loss, known at NW and UE, with mitigation at NW and UE

  - Note: Corresponds to reset of historical CSI information at both UE and NW or any other mitigation approach enabled by signaling.

- UCI loss modeling

  - 10% UCI loss probability on all UCI reports

  - Other options are not precluded, e.g., No UCI loss for the first UCI report of each observation window, and 10% UCI loss probability for the subsequent reports of each observation window, as a shortcut to simplify the evaluation work.

  - Other values for UCI loss probability are not precluded.

- Note: The same UCI loss modeling shall be applied to the benchmark for fair comparison.

- FFS: partial PMI-related UCI loss

Agreement

According to the Rel-19 study objective,

- For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

RAN1 is recommending extending the study of the AI/ML CSI compression based on RAN1 understanding the study on CSI compression is not completed.

// Additional study on AI/ML for NR air interface: Other aspects of AI/ML model and data

Conclusion

- From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

Conclusion

The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work.

Conclusion

The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work.

Agreement

Confirm the following Working assumption.

Working Assumption

Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell

- FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Agreement

From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects

- Model type/backbone (e.g., Transformer, CNN and so on)

- In case model type is a neural network

  - Number of layers

  - Layer types/structure (e.g., full connected, activation layer and so on)

  - Layer size (e.g., the number of parameters of a layer)

  - Connection between different layers

- model input/output related information

Conclusion

From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

Agreement

RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

Agreement

Adopt the TP1, TP2, TP3 and TP4 in Section 2 of R1-2407520 in principle.

#### 2.1.2	Remaining Open issues

- Finalize specification support for beam management

- Finalize specification support for positioning accuracy enhancements

- Decide how to proceed with the CSI prediction study: continue the study, start normative work or stop.

- Extend study on CSI compression

- Extend study on other aspects of AI/ML model and data with RAN1 impact

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#127 discussed LCM for UE-sided model for Beam Management use case and made the following agreements:

- Agreements on definitions:

  - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

  - Applicable functionalities refers to functionalities that the UE is ready to apply for inference

  - Activated functionalities refers to functionalities already enabled for performing inference

- Agreements on procedures:

  - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

  - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

  - “Step 3”: Following configurations are provided from NW to UE:

    - 1) UE is allowed to do UAI reporting via OtherConfig.

    - 2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

    - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

  - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

  - “Step 4”: UE reports applicable functionality in the following scenarios:

    - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

    - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

  - Step 5:

    - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

    - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

  - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality

  - RAN2 has also agreed to send an LS to RAN1 to provide the above list of agreements and ask specific questions to facilitate RAN2 progress. An email discussion was assigned to discuss the detailed content of such LS.

RAN2#127 discussed NW-side data collection and made the following agreements:

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported.

- FFS on event based data collection/logging.

- On-demand request from the network is supported. FFS details on signalling.

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

RAN2#127 discussed UE-side data collection and made the following agreements:

- Clarify in the TR 38.843 that controllability can be achieved via SLA (Service Level Agreement)

- Clarify in the TR 38.843 that full controllability implies that the MNO has the capability to manage data transfer to the server for UE-side data collection without the need of SLA.

- Clarify in the TR 38.843 that visibility of data content signifies that the MNO can at least, be aware of, access, and comprehend the data without the need of SLA.

On the topic of UE-side data collection, RAN2 has also endorsed a TP for TR 38.843 in R2-2407807, including the agreements from RAN2#125-bis, RAN2#126, RAN#127. Further, an LS was sent to RAN1 to inform RAN1 about such endorsed TP (LS available in R2-2407808).
No recommendations and no requirements for the UE-side data collection were agreed in RAN2.

The following topics were not discussed in RAN2#127:

- LCM for NW-sided model for Beam Management use case

- LCM for Positioning use case

#### 2.2.2	Remaining Open issues

Related to the topic of LCM for UE-sided model for Beam Management use case, RAN2 can continue analysing the details of LCM-related signalling procedures, focusing on the FFS agreed during RAN2#127. RAN1 inputs are also needed to enable RAN2 progress.

Related to the topic of LCM for NW-sided model for Beam Management use case, RAN2 can monitor the progress in RAN1 in order to assess impacts, if any, in RAN2 specifications.

Related to the topic of LCM for Positioning use case, the discussion on how to handle dynamic capabilities (FFS from RAN2#126) depends on further RAN1 progress and understanding of the functionality. The focus should be on the 1st priority cases, and, based on RAN1 progress, on the 2nd priority cases.

Related to the topic of NW-side data collection, RAN2 can continue the discussion on configuration and reporting mechanisms to enable NW-side data collection, focusing on the FFS agreed during RAN2#127, e.g. mechanisms to start/stop the data collection, mechanisms to start/stop the reporting of collected data, etc.

Related to the topic of UE-side data collection, RAN2 has finalized the study and the analysis is captured in TP for TR 38.843 in R2-2407807. It is up to RAN#105 the decision on whether to continue the study in RAN, whether to start or not a normative work in RAN, whether to involve SA WGs in the discussion.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Issue 1-2: Channel modelling

Agreement:

- Use synthetic channels as the default assumption for conformance requirements/test.

  - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

    - The field data can be considered for the analysis.

Issue 1-3: Testing goals

Agreement:

- The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

  - FFS on whether and how many different test configurations/parameters are used for tests.

  - LCM would be tested

Issue 1-6: CR in R4-2412023

Terminology update in R4-2414308 is endorsed.

Issue 2-3: Channel models

Agreement:

Further discuss and down-select the following options considering TE feasibility

- Option 2: TDL

- Option 5: simplified CDL

  - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Issue 2-4: Test setup needs

Issues listed below to be further investigated in order to determine the capabilities of the test equipment:

- set B Tx beams; set A Tx beams

      - number of beams transmitted simultaneously

      - overall number of beams transmitted during the test

- AoA

- AoD

- UE rotation during the test

Other aspects can also be considered

Issue 2-6:	Measurement error impact

Agreement:

RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study.

impact on different prediction accuracy metrics should be taken into account

studies already performance by other groups(RAN1) could also be taken into account

Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Issue 2-7:	UE reporting for network side models

Agreement:

postpone discussion until other WGs define any reporting mechanisms

Issue 3-2: Requirements for case 2a

Agreement:

RAN4 should not define any positioning accuracy requirements because positioning is LMF based

Issue 3-3: Reported metrics for case 2a

Agreement:

- RAN4 will take existing requirements for existing metrics as baseline

  - FFS whether to revisit these requirements

- RAN4 will study how to define requirement for any new reported metrics

  - FFS how to define requirements for LOS/NLOS

Issue 3-5: Reported metrics for case 2b

Agreement:

postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported.

Issue 3-6: Reported metrics for case 3a/3b

Agreement:

postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

Issue 4-1: Option 3 next steps

Agreement:

- Check on performance alignment -> see simulation results from contributing companies

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Select one or more decoder for further analysis

  - selection criteria to be discussed

- Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

Note: some steps could be reused for Option 4, FFS on what can be reused

Issue 4-2: Simulation results and alignment

Agreement:

- perform a round of alignment until RAN4#112-Bis

- refined simulation parameters can be found in R4-2414447

Issue 4-4: Options for Option 4

Agreement:

Option 4 sub-options:

- 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

- 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

- 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

Companies are invited to bring further analysis on how to perform a feasibility study for these options.

Issue 4-5: CSI prediction accuracy metric

Agreement:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

#### 2.4.2	Remaining Open issues

- General aspects

  - Post deployment handling

  - AI/ML model performance

  - Generalization

  - LCM handling

- Testability and interoperability issues for beam management

  - Metrics/KPIs for BM

  - RSRP Prediction accuracy

  - Reported measurements and ground truth related issues

  - Channel models

  - Test setup needs

  - OTA Test setup for beam management

  - Measurement error impact

  - UE reporting for network side models

  - Beam prediction and TCI state related issues

  - UE side beam prediction requirements impact

- Testability and interoperability issues for positioning accuracy enhancement

  - Requirements for case 1/2a/2b/3a/3b

  - Requirements for measurements and reported metrics/values

  - KPIs

- Testability and interoperability issues for CSI compression and CSI prediction

  - CSI Prediction Accuracy metrics

  - Testing options for 2-sided model

  - Reference/test encoder/decoder

  - Option 3 for 2-sided model

  - Option 4 for 2-sided model

  - Comparison table

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

### 4.1	RAN1#118

// Specification support for beam management

R1-2407324 	FL summary #4 for AI/ML in beam management	Samsung (Moderator)

R1-2407323 	FL summary #3 for AI/ML in beam management	Samsung (Moderator)

R1-2407322 	FL summary #2 for AI/ML in beam management	Samsung (Moderator)

R1-2407321 	FL summary #1 for AI/ML in beam management	Samsung (Moderator)

R1-2407320	FL summary #0 for AI/ML in beam management	Samsung (Moderator)

R1-2405808	Discussion on specification support for AI/ML-based beam management	FUTUREWEI

R1-2405899	Discussion on AIML for beam management	Spreadtrum Communications

R1-2405944	Specification Support for AI/ML for Beam Management	Kyocera

R1-2405950	AI/ML based Beam Management	Google

R1-2405963	AI/ML for Beam Management	Tejas Networks Limited

R1-2405975	Discussion on specification support for beam management	CMCC

R1-2406014	Specification support for AI/ML for beam management	Intel Corporation

R1-2406054	Discussion on AI/ML-based beam management	ZTE Corporation, Sanechips

R1-2406141	AI/ML for beam management	Ericsson

R1-2406172	Specification support for beam management	vivo

R1-2406254	On specification for AI/ML-based beam management	OPPO

R1-2406269	Specification support for beam management	Xiaomi

R1-2406305	Discussion on specification support on AI/ML for beam management	Fujitsu

R1-2406353	Specification support for AI/ML-based beam management	CATT

R1-2406395	Discussion on AIML Beam Management	TCL

R1-2406416	Discussions on AI/ML for beam management	LG Electronics

R1-2406440	AI/ML specification support for beam management	Lenovo

R1-2406463	Discussions on AI/ML for beam management	Sony

R1-2406492	Specification support for AI-enabled beam management	NVIDIA

R1-2406497	Discussion on AI/ML for beam management	InterDigital, Inc.

R1-2406526	Discussion on specification support for beam management	Panasonic

R1-2406541	Discussion on specification support for beam management	NEC

R1-2406571	Discussion on AI/ML based beam management	Hyundai Motor Company

R1-2406586	AI/ML for Beam Management	Nokia

R1-2406593	Discussions on specification support for beam management	Ruijie Networks Co. Ltd

R1-2406637	Discussion for supporting AI/ML based beam management	Samsung

R1-2406699	Discussion on specification support for AI/ML beam management	Transsion Holdings

R1-2406718	Discussion on specification support for beam management	ETRI

R1-2406765	Discussion on specification support for AIML-based beam management	MediaTek Inc.

R1-2406826	Discussion on AI/ML beam management	Apple

R1-2406884	Discussion on AI/ML based beam management	KT Corp.

R1-2406888	AI/ML for Beam Management	Meta Ireland

R1-2406894	Discussions on AI/ML for beam management	CAICT

R1-2406920	Discussion on AI/ML for beam management	NTT DOCOMO, INC.

R1-2406969	Discussion on specification support for beam management	Sharp

R1-2406977	Discussion on beam management for AI/ML	Huawei, HiSilicon

R1-2407019	Specification support for AI-ML-based beam management	Qualcomm Incorporated

R1-2407064	Discussions on Specification Support of AI/ML for Beam Management	Indian Institute of Tech (M), IIT Kanpur

R1-2407109	Specification support for beam management	Fraunhofer HHI, Fraunhofer IIS

R1-2407116	A Novel Model-ID Free Approach for Interoperability in AI/ML Beam Management Use Cases			NTU

R1-2407120	Specification support for AI/ML beam management	ITL

R1-2407142	Specification support for beam management	KDDI Corporation

// Specification support for positioning accuracy enhancement

R1-2407271 	Summary #5 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2407270 	Summary #4 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2407269 	Summary #3 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2407268 	Summary #2 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2407267	Summary #1 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2405945	AI/ML for Positioning Accuracy Enhancement	Ericsson Inc.

R1-2405951	AI/ML based Positioning	Google

R1-2405976	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2406015	Specification support for AI/ML for positioning accuracy enhancement	Intel Corporation

R1-2406055	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2406173	Specification support for positioning accuracy enhancement	vivo

R1-2406206	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2406255	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2406270	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2406306	Discussion on specification support for AI/ML positioning accuracy enhancement	Fujitsu

R1-2406354	Specification support for AI/ML-based positioning	CATT, CICTCI

R1-2406394	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2406441	Specification impacts for AI/ML Positioning	Lenovo

R1-2406464	Discussion on AI/ML for positioning accuracy enhancement	Sony

R1-2406493	Specification support for AI-enabled positioning	NVIDIA

R1-2406536	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2406587	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2406594	Discussions on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2406638	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2406711	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2406719	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2406827	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2406921	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2406970	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2406978	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2407020	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2407063	Discussion on Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2407076	Discussion on specification support for AI/ML positioning accuracy enhancement	CEWiT

R1-2407105	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2407166	Discussions on positioning accuracy enhancement for AI/ML	ITL

// Additional study on AI/ML for NR air interface: CSI prediction

R1-2407340 	Summary #3 of CSI prediction	Moderator (LG Electronics)

R1-2407339 	Summary #2 of CSI prediction	Moderator (LG Electronics)

R1-2407338	Summary #1 of CSI prediction	Moderator (LG Electronics)

R1-2405900	Discussion on AIML for CSI prediction	Spreadtrum Communications, BUPT

R1-2405952	AI/ML based CSI Prediction	Google

R1-2405961	Discussion on AI/ML for CSI prediction	Tejas Networks Limited

R1-2405977	Discussion on AI/ML for CSI prediction	CMCC

R1-2406016	AI/ML for CSI prediction	Intel Corporation

R1-2406056	Discussion on study for AI/ML CSI prediction	ZTE Corporation, Sanechips

R1-2406068	Discussion on AI/ML for CSI prediction	BJTU

R1-2406070	AI/ML for CSI prediction	Ericsson

R1-2406174	Discussion on CSI prediction	vivo

R1-2406256	Additional study on AI/ML-based CSI prediction	OPPO

R1-2406271	Views on UE-side AI/ML model based CSI prediction	Xiaomi

R1-2406307	Discussion on CSI prediction with AI/ML	Fujitsu

R1-2406355	Further study on AI/ML for CSI prediction	CATT

R1-2406389	Discussion on AI/ML for CSI prediction	China Telecom

R1-2406391	Discussion on AI/ML for CSI prediction	Panasonic

R1-2406417	Study on CSI prediction	LG Electronics

R1-2406442	On AI/ML for CSI prediction	Lenovo

R1-2406465	Discussion on model monitoring of AI/ML CSI Prediction	Sony

R1-2406494	Additional study on AI-enabled CSI prediction	NVIDIA

R1-2406501	Discussion on AI/ML-based CSI prediction	InterDigital, Inc.

R1-2406533	Discussion on CSI prediction	NEC

R1-2406588	AI/ML for CSI Prediction	Nokia

R1-2406639	Discussion for further study on AI/ML-based CSI prediction	Samsung

R1-2406683	Discussion on AI/ML for CSI prediction	SK Telecom

R1-2406788	Additional Study on AI/ML - CSI Prediction	MediaTek Korea Inc.

R1-2406828	Discussion on AI based CSI prediction	Apple

R1-2406870	Discussion on AI/ML for CSI prediction	AT&T

R1-2406904	AI/ML for CSI prediction	Mavenir

R1-2406922	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

R1-2406979	Discussion on CSI prediction for AI/ML	Huawei, HiSilicon

R1-2407021	Additional study on CSI prediction	Qualcomm Incorporated

R1-2407183	Further study on AI/ML for CSI prediction	CATT

// Additional study on AI/ML for NR air interface: CSI compression

R1-2407495 	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407346	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407499 	Summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407433	Summary of Evaluation Results for Additional study on AI/ML for NR air interface: CSI compression		Moderator (Qualcomm)

R1-2407345 	Summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407344 	Summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407343 	Summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2407342	Draft summary of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

R1-2405809	Discussion on additional study on AI/ML for NR air interface for CSI compression				FUTUREWEI

R1-2405863	Discussion on AI/ML for CSI compression	Huawei, HiSilicon

R1-2405901	Discussion on AIML for CSI compression	Spreadtrum Communications, BUPT

R1-2405953	AI/ML based CSI Compression	Google

R1-2405960	Discussion on AI/ML for CSI Compression	Tejas Networks Limited

R1-2405978	Discussion on AI/ML for CSI compression	CMCC

R1-2406017	AI/ML for CSI compression	Intel Corporation

R1-2406057	Discussion on study for AI/ML CSI compression	ZTE Corporation, Sanechips

R1-2406069	AI/ML for CSI compression	Ericsson

R1-2406175	Discussion on CSI compression	vivo

R1-2406257	Additional study on AI/ML-based CSI compression	OPPO

R1-2406272	Views on two-sided AI/ML model based CSI compression	Xiaomi

R1-2406308	Discussion on CSI compression with AI/ML	Fujitsu

R1-2406356	Further study on AI/ML for CSI compression	CATT

R1-2406392	Discussion on AI/ML for CSI compression	Panasonic

R1-2406396	Discussions on AIML CSI compression	TCL

R1-2406418	Study on CSI compression	LG Electronics

R1-2406443	On AI/ML for CSI compression	Lenovo

R1-2406466	Discussion on CSI compression	Sony

R1-2406495	Addtional study on AI-enabled CSI compression	NVIDIA

R1-2406502	Discussion on AI/ML-based CSI compression	InterDigital, Inc.

R1-2406534	Discussion on CSI compression	NEC

R1-2406589	AI/ML for CSI Compression	Nokia

R1-2406640	Discussion for further study on AI/ML-based CSI compression	Samsung

R1-2406720	Discussion on AI/ML for CSI compression	ETRI

R1-2406789	Additional study on AI/ML - CSI compression	MediaTek Korea Inc.

R1-2406829	Discussion on AI based CSI compression	Apple

R1-2406871	Discussion on AI/ML for CSI compression	AT&T

R1-2406895	Discussions on AI/ML for CSI feedback	CAICT

R1-2406923	Discussion on AI/ML for CSI compression	NTT DOCOMO, INC.

R1-2407022	Additional study on CSI compression	Qualcomm Incorporated

R1-2407065	Discussion on AI/ML for CSI Compression	Indian Institute of Tech (M), IIT Kanpur

R1-2407077	Discussion on AI/ML for CSI Compression	CEWiT

R1-2407121	Discussion on AI/ML based CSI compression	ITL

R1-2407186	AI/ML for CSI compression	Intel Corporation

Revision of R1-2406017

// Additional study on AI/ML for NR air interface: Other aspects of AI/ML model and data

R1-2407520	TPs to capture the outputs of 9.1.3.3	Moderator (OPPO)

R1-2407304 	Summary#1 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2407303 	Summary#1 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2407302	Summary#1 for other aspects of AI/ML model and data	Moderator (OPPO)

R1-2405810	Discussion on other aspects of AI/ML model and data on AI/ML for NR air-interface				FUTUREWEI

R1-2405902	Discussion on other aspects of AI/ML model and data	Spreadtrum Communications

R1-2405954	AI/ML Model and Data	Google

R1-2405962	Other aspects of AI/ML Model and Data	Tejas Networks Limited

R1-2405979	Discussion on other aspects of AI/ML model and data	CMCC

R1-2406018	Other study aspects of AI/ML for air interface	Intel Corporation

R1-2406058	Discussion on other aspects of AI/ML model and data	ZTE Corporation, Sanechips

R1-2406064	Discussion on other aspects of AI/ML model and data	Continental Automotive

R1-2406142	Discussion on other aspects of AI/ML	Ericsson

R1-2406176	Other aspects of AI/ML model and data	vivo

R1-2406258	Additional study on other aspects of AI/ML model and data	OPPO

R1-2406273	Further study on AI/ML model and data	Xiaomi

R1-2406309	Discussion on other aspects of AI/ML model and data	Fujitsu

R1-2406357	Further study on AI/ML for other aspects	CATT, CICTCI

R1-2406397	Discussions on Other Aspects of AIML In NR Airinterface	TCL

R1-2406419	Discussion on other aspects of AI/ML model and data	LG Electronics

R1-2406444	Discussion on other aspects of AI/ML model and data	Lenovo

R1-2406459	Discussion on other aspects of AI/ML model and data	IIT Kanpur

R1-2406496	Additional study on other aspects of AI model and data	NVIDIA

R1-2406503	Discussion on other aspects of AI/ML model and data	InterDigital, Inc.

R1-2406542	Discussion on other aspects of AI/ML model and data	NEC

R1-2406590	Other Aspects of AI/ML Model and Data	Nokia

R1-2406641	Discussion for further study on other aspects of AI/ML model and data	Samsung

R1-2406674	Discussion on other aspects for AI/ML for air interface	Panasonic

R1-2406721	Discussion on other aspects of AI/ML model and data	ETRI

R1-2406830	Discussion on other aspects of AI/ML model and data	Apple

R1-2406872	Other Aspects of AI/ML framework	AT&T

R1-2406889	Other Aspects of AI/ML Model and Data	Meta Ireland

R1-2406924	Discussion on other aspects of AI/ML model and data	NTT DOCOMO, INC.

R1-2406964	Discussion on other aspects of AI/ML model and data	Sharp

R1-2406976	Discussion on other aspects of the additional study for AI/ML	Huawei, HiSilicon

R1-2407023	Other aspects of AI/ML model and data	Qualcomm Incorporated

### 4.2	RAN2#127

RAN2#127

78 contributions (for details see agenda item 8.1 in Tdoc list)

### 4.3	RAN4#112

// General aspects

R4-2411258	Discussion on general aspects of AIML for NR air interface

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2411340	Discussion on general aspects for AIML for NR air

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2411408	General aspects on AI/ML for NR Air Interface

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2411625	Discussion on general aspects of AI/ML testability and interoperability

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2411980	Discussion on generalization

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2412021	On General Aspects of AI/ML for Air Interface

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2412249	Discussion on general aspects for AIML

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2412331	Views on general aspects of AI/ML testability and interoperability

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2412765	Discussion on general aspects for AIML

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2413040	Discussion on the AI/ML general aspects

Type: discussion		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2413391	On general AI/ML aspects

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On general AI/ML aspects

Decision:		Noted.

Draft CR

R4-2412023	Draft CR on Correction in AIML Term Definitions

Type: draftCR		For: Endorsement
					38.843 v18.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Decision:		Revised to R4-2414308 (from R4-2412023).

R4-2414308	Draft CR on Correction in AIML Term Definitions

Type: draftCR		For: Endorsement
					38.843 v18.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Decision:		Endorsed.

// Testability and interoperability issues for beam management

R4-2411177	Testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision:		Noted.

R4-2411254	OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Decision:		Noted.

R4-2411259	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2411279	Study on CDL channel model for AI/ML beam management in FR2

Type: other		For: Approval
					Source: Anritsu Corporation

Abstract:

In this contribution we share our findings on the conditions to apply the CDL channel model to the beam management verification.

Decision:		Noted.

R4-2411292	Testability and interoperability issues  for beam management

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2411341	Discussion on testability and interoperability issues for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2411409	Discussion on Testability and Interoperability issues for Beam Management

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2411587	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision:		Noted.

R4-2411626	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2411635	Further discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2411706	Discussion on AI/ML RAN4 BM use case

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2411978	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2412231	Testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discuss AIML for beam management relevant issuses.

Decision:		Noted.

R4-2412250	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2412766	Discussion on testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2412994	Testability and interoperability issues for beam management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2413038	Discussion on the Interoperability and testability aspects of AI/ML Beam management

Type: discussion		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2413168	On AI/ML RAN4 Use Cases: Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Testability and interoperability issues for positioning accuracy enhancement

R4-2411293	Testability and interoperability issues for positioning accuracy enhancement

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2411342	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2411410	Discussion on Testability and Interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2411627	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2411682	Discussions on testability and interoperability for positioning

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2411981	Discussion on testability and interoperability issues for positioning

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2412251	Discussion on testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2412695	On AI/ML for positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion paper on AI/ML for positioning. Issues related to use case 2a and 2b are primarily discussed.

Decision:		Noted.

R4-2412767	Discussion on Testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2413039	Discussion on the Interoperability and testability aspects of AI/ML positioning

Type: discussion		For: Discussion
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2413291	Testability and interoperability issues for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

//Testability and interoperability issues for CSI compression and CSI prediction

R4-2411178	Testability and interoperability issues for CSI compression and CSI prediction

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision:		Noted.

R4-2411294	Testability and interoperability issues for CSI compression and CSI prediction

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2411343	Discussion on testability and interoperability issues for CSI enh

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2411411	Discussion on Testability and Interoperability issues for CSI Compression and Prediction

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2411534	Discussion on reference decoder for CSI compression

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision:		Noted.

R4-2411628	Discussion on testability and interoperability issues for CSI compression

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2411636	Further study on testability and interoperability issues for AI-CSI

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2411711	Discussion on AIML RAN4 CSI Interoperability and testability

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2411786	Summary of e-mail discussion on AI/ML Model for CSI testing Option 3 feasibility study

Type: other		For: Information
					Source: Moderator (Qualcomm Incorporated)

Decision:		Noted.

R4-2411979	Discussion on testability and interoperability issues for  CSI compression and CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2412022	On AI/ML-based CSI Feedback

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2412130	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case

Type: discussion		For: Discussion
					Source: NTU

Decision:		Noted.

R4-2412252	Discussion on testability and interoperability issues for CSI compression and CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2412332	Views on AI/ML testability for CSI compression and prediction

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision:		Noted.

R4-2412609	CSI compression option 3 and option 4 considerations

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution considers issues for CSI compression and presents results for the reference model.

Decision:		Noted.

R4-2412768	Discussion on Testability and interoperability issues for CSI compression and CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2412792	Discussion on testability and interoperability issues for AI-CSI

Type: discussion		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2413169	On AI/ML RAN4 Use Cases: CSI Interoperability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Moderator summary and conclusions

R4-2412831	Topic summary for [112][129] NR_AIML_air

Type: other		For: Information
					Source: Moderator(Qualcomm)

Abstract:

Summary for AI 8.17

Decision:		Noted.

Minutes and conclusions in the first round

Please refer to the following hyperlinks for the detailed minutes

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_112/Inbox/Drafts/%5B112%5D%5B100%5D%20Main%20Session/3.Wednesday/1.%5B129%5D_AIML%5B112%5D%5B129%5DAIMLDraftSummary_before_v9_Ericsson2_Nokia.docx

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_112/Inbox/Drafts/%5B112%5D%5B100%5D%20Main%20Session/3.Wednesday/1.%5B129%5D_R4-2412831.docx

R4-2414446	Ad hoc minutes for AI/ML

Type: other		For: Approval
					Source: Qualcomm

Decision:		Noted.

R4-2414323	WF on testability and requirements for AIML air interface

Type: other		For: Approval
					Source: Qulacomm

Decision:		Approved.

R4-2414447	Alignement of model training and data set parameters for AI/ML based CSI compression

Type: other		For: Approval
					Source: Nokia

Decision:		Approved.

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template