3GPP TSG RAN Meeting #106										RP-242588

Madrid, Spain, December 09-12th, 2024

Title:	Ku band NTN proposed way forward

Source: 	THALES

Type:	discussion

Document for:	Approval

Agenda Item:	9.4.1.6

# Introduction

- According to RAN4 WID [1], there is a checkpoint at RAN #106 as follows:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106.

RAN4#113 has concluded that both FR1 and FR2 numerologies are suitable for the Ku band. However, it has been challenging to select one due to the diverse use cases that favor either numerology. To address this, RAN4 has decided to recommend the specification of two separate sets of bands for the same frequency range: one for FR1 numerology and another for FR2 numerology. These bands will function independently, with coordination managed through existing inter-operator co-existence mechanisms, which are defined outside of 3GPP.

In this contribution, we share our view on the WF on 3GPP bands definition for Ku band.

# Discussion

Issue related to NTN Ku band System parameters and FR1/FR2 numerology down selection was extensively discussed in RAN4#113. Two distinct 3GPP sets of bands within Ku were further recommended by RAN4 as result of very intensive discussions:

1/ one band set with FR1 numerology and the other for

2/ one band set with FR2 numerology.

Further, coordination with RAN1 and RAN2 and further analysis are deemed essential for a conclusive decision.

The following observations [2] were made in RAN4#113:

- Overall observation is that both FR1 numerology and FR2 numerology are feasible. - There are use cases that favour either FR1 numerology or FR2 numerology - Some of the disadvantages of numerology combinations can be mitigated with implementation choices

According to the discussion from different companies, the following conclusions were reached at RAN4#113:

### Sub-topic 3-1: UL timing FR1 numerology is feasible for NGSO. FR1 numerology is feasible for GSO by adopting a new error model where the correlated part of Tp,san is discounted Assuming a new error model where the correlated part of Tp,san is discounted: - FR2 numerology for Fixed and mobile VSAT served by NGSO and GSO with 60 kHz UL SCS is feasible - FR2 numerology for Fixed VSAT served by NGSO and GSO with 120 kHz UL SCS is feasible if the installation has a wide sky view down to 30 degrees, or a manual map-based installation. - FR2 numerology for Mobile VSAT served by NGSO and GSO with 120 kHz UL SCS is feasible for platform types that can achieve a 31.6 m one-way path length uncertainty ### Sub-topic 3-2: Phase noise (and residual frequency error impact) FR2 numerology is feasible with regards to phase noise FR1 numerology is feasible with regards to phase noise but may require a higher quality oscillator, particularly for 15 kHz SCS to avoid performance degradation. ### Sub-topic 3-3: Efficient channel utilization (allocation of RBs) FR1 numerology is feasible with regards to efficient allocation of RBs to the required bandwidths FR2 numerology is feasible with regards to efficient allocation of RBs to the required bandwidths but is less efficient than FR1 numerology for legacy narrower bandwidths but more efficient than FR1 for 250 MHz ### Sub-topic 3-4: Beam hopping granularity Both FR1 numerology and FR2 numerology are feasible with regards to beam hopping granularity. FR2 numerology provides more flexibility than FR1 numerology for systems employing analogue beamforming. ### Sub-topic 3-5: Link budget Both FR1 numerology and FR2 numerology are feasible with regards to link budget. ### Sub-topic 3-6: Doppler Both FR1 numerology and FR2 numerology are feasible with regards to Doppler. ### Sub-topic 3-7: Resilience to frequency Errors/ UE Doppler pre-compensation Both FR1 numerology and FR2 numerology are feasible with regards to residual frequency error FR1 numerology with 15 kHz SCS may experience reduced performance due to residual frequency error (a 1.4 kHz offset at 14 GHz) ### Sub-topic 3-8: Spectral efficiency & maximum throughput per channel configuration (currently CA not supported) Both FR1 numerology and FR2 numerology are feasible with regards to spectral efficiency Based on impairments related to residual frequency error, FR1 numerology with 15 kHz SCS may provide a lower maximum throughput than 30 Hz SCS or FR2 numerology.

As a reasonable way forward, the following was agreed in RAN4#113 [2]:

Agree to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. RAN4 will further discuss SCS options.

From our perspective, two sets of 3GPP bands would simplify the approach and align with existing specifications, reducing the workload. We propose reusing requirements to minimize effort.

It is worth noting that during RAN #104, integrating FR1 and FR2 numerologies within a single band definition was deemed outside the scope of a spectrum-only Work Item and potentially problematic. In our view, the approach consisting of defining two distinct 3GPP bands within Ku is still aligned with the WID [1] objectives.

In this regards, it was recommended in RAN4#113 to consult RAN1 and RAN2 for potential impacts when adopting the approach with two sets of bands (one with FR1 and one with FR2 numerologies). Thereby, RAN4 send an LS to RAN1 and RAN2 in [3].  As agreed in previous RAN4 meeting, to avoid delays in their work RAN4 will proceed with two sets of bands approach as per the above agreement. Although some companies doubt RAN plenary will decide, asserting RAN4's decision-making role, we believe that RAN should make final decision at the upcoming check point otherwise Ku band would not be defined in Rel-19.

Observation 1: For the Ku-band, defining two sets of frequency bands - one frequency band set with FR1 numerology and another frequency band set with FR2 numerology - is a reasonable working assumption in RAN4 to prevent delays in their tasks.

Proposal 1: At RAN#106, RAN should endorse the proposal to support both FR1 and FR2 numerologies for the Ku band.

Regarding the impact on supporting two distinct 3GPP set of bands overlapping with different numerologies within Ku, we believe that such approach is feasible. Any potential impacts may be only on cell search/Initial cell selection procedure, the way the UE derives SSB Subcarrier Spacing and the interpretation of the subCarrierSpacingCommon:

- For SSB Subcarrier Spacing: The majority of operating bands within FR1 have a single permitted subcarrier spacing (SCS) which simplifies the cell search procedure. The same approach should be adopted for both defined bands within Ku. That is, the 3GPP band within Ku with FR1 numerology can use SCS of 15 or 30 kHz, and the other 3GGP band within Ku with FR2 numerology can use SCS of 120 or 240 kHz.

- UE may need to perform two 3GPP bands scan.

- Regarding subCarrierSpacingCommon which is used in MIB to indicate (in one bit) subcarrier spacing for SIB1, Msg.2/4 and MsgB for initial access, paging and broadcast SI-messages: according to existing specs if the UE acquires this MIB on an FR1 carrier frequency, the value scs15or60 corresponds to 15 kHz and the value scs30or120 corresponds to 30 kHz. If the UE acquires this MIB on an FR2 carrier frequency, the value scs15or60 corresponds to 60 kHz and the value scs30or120 corresponds to 120 kHz. with two sets of bands approach (one for FR1 numerology and the other for FR2 numerology) in Ku band, the same interpretation of subCarrierSpacingCommon can be adopted. The UE derive the configured numerology based on SSB Subcarrier Spacing in use (as discussed in (1))

Few companies favor a single band that accommodates multiple numerologies. In our opinion, defining a single band that supports a mix of FR1 and FR2 numerologies could complicate the specifications and delay their availability. Indeed, many features are specified differently for FR1 and FR2, such as SS/PBCH block and CORESET multiplexing patterns. Therefore, even if a single band is defined accommodating multiple numerologies, only a set of parameters belonging to either FR1 or FR2 could be used in practice. Moreover, since the working assumption in RAN4 is to move forward with the recommendation to define two separate sets of bands - one set with FR1 numerology and another set with FR2 numerology - we believe there is no need to continue discussing a single band with mixed numerologies at this time.

Observation 2: To minimize the impact on existing specifications, both FR1 and FR2 numerologies should not be used simultaneously within the same 3GPP NTN band.

Observation 3: The definition of a single 3GPP NTN band that supports simultaneously both FR1 and FR2 numerologies would result in significant specification impacts and create unnecessary workload across various RAN working groups.

Observation 4: The effort required to define two bands with respectively FR1 and FR2 is minor compared to the effort necessary to define one band, since both FR1 and FR2 numerologies require OTA requirements, and coexistence studies are applicable to both FR1 and FR2 numerologies if 50 or 100MHz channelization is selected. In addition, the definition of FR2 numerology for Ku band can reuse most requirements defined for Ka band.

Based on the above discussion we propose to define two 3GPP bands within Ku bands, one for FR1 and the other for FR2:

Proposal 2: Define two 3GPP NTN bands covering the same frequencies - one with FR1 numerology and the other with FR2 numerology.

De facto, a revision of the WID [RP-241690] might be needed to explicitly state that "a single 3GPP band supporting both FR1 and FR2 is not considered”.

Proposal 3: Revise the “clause objective” of WID [RP-241690] as follows

- “Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following but not limited to:

  - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

  - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing

  - For both FR1 and FR2 numerologies, the choice of the subcarrier spacing will be discussed and decided in RAN4”

As a matter of fact, the current understanding from the WID [RP-241690] was not to define a single band with mixed numerologies, which might be complicated to handle from the specification impact point of view. As discussed per RAN4#113, the impact of mixed numerology might be too heavy since the following modifications might be required:

- The FR1-NTN sync raster would need to be added to FR2-NTN.

- All FR2-NTN SCS would become optional to accommodate different use cases.

- Half as many bands would be required.

- Is RAN1 input not required?

- Any other unintended consequences?

In the following, we discuss how the definition of 3GPP overlapping set of frequency bands (one frequency band set for FR1-NTN and other frequency band set for FR2-NTN) within Ku could be envisioned by minimizing as much as possible the work load.

One of the potential approaches is to extend the FR1-NTN range up to 14.5 GHz and to extend FR2-NTN range down to 10.7 GHz according to the current Ku-band 3GPP band proposals.

Then, 3GPP would define new Ku-bands with n2xx numbering for FR1-NTN bands and n5xx numbering for FR2-NTN bands.

FR2-NTN Ku-bands can follow the current definitions for Ka-bands with 50, 100, 200, 400 MHz, which is straightforward.

FR1-NTN Ku-bands will have to consider new channel bandwidths such as 25 MHz, 35 MHz, 50 MHz, 70 MHz, 100 MHz (for the current FR1-NTN definition in S/L-bands the requirements have been defined only for CBWs up to 20 MHz) combined with CA to reach 125MHz and 250 MHz of bandwidth as per some current satellite configurations.

The Ku-band definition will then follow OTA requirements approach for both FR1-NTN and FR2-NTN ranges, for both SAN and UE.

As a matter of fact, as per contribution R4-2419727 (THALES), the following proposals were already made at RAN4#113 and RAN4#112-bis:

Proposal 4: RAN4 to consider, as part of the Ku-band WI, the definition of two NTN frequency set of bands:

- One set of bands as FR1-NTN with at least SCS = 30 kHz configuration (specially adapted to narrowband satellite transponders such as 36 and 72 MHz) to address narrowband satellite transponders

- One set of bands as FR2-NTN with at least SCS = 120 kHz configuration (specially adapted to wideband satellite transponders with more than 100 MHz) to address wideband satellite transponders

Proposal 5: Try not to mix FR1-NTN and FR2-NTN numerologies on the same band definition, as per current Ku-band WID description.

The following set of proposals could be also considered:

Proposal 6: Extend the FR1-NTN range up to 14.5 GHz and extend FR2-NTN range down to 10.7 GHz according to the current Ku-band 3GPP band proposals.

Proposal 7: Define FR2-NTN Ku-bands with CBWs of 50 MHz, 100 MHz, 200 MHz, 400 MHz (following Ka-band definition approach).

Proposal 8: Include the following FR1-NTN CBWs for Ku-band: 25 MHz, 35 MHz, 50 MHz, 70 MHz, 100 MHz (together with 10, 15, 20 MHz already considered for FR1-NTN).

Proposal 9: The Ku-band definition will follow OTA requirements approach for both FR1-NTN and FR2-NTN ranges, for both SAN and UE.

# Conclusion

In this contribution. we made the following observations and proposals:

Observation 1: For the Ku-band, defining two sets of frequency bands - one frequency band set with FR1 numerology and another frequency band set with FR2 numerology - is a reasonable working assumption in RAN4 to prevent delays in their tasks.

Observation 2: To minimize the impact on existing specifications, both FR1 and FR2 numerologies should not be used simultaneously within the same 3GPP NTN band.

Observation 3: The definition of a single 3GPP NTN band that supports simultaneously both FR1 and FR2 numerologies would result in significant specification impacts and create unnecessary workload across various RAN working groups.

Observation 4: The effort required to define two bands with respectively FR1 and FR2 is minor compared to the effort necessary to define one band, since both FR1 and FR2 numerologies require OTA requirements, and coexistence studies are applicable to both FR1 and FR2 numerologies if 50 or 100MHz channelization is selected. In addition, the definition of FR2 numerology for Ku band can reuse most requirements defined for Ka band.

Proposal 1: At RAN#106, RAN should endorse the proposal to support both FR1 and FR2 numerologies for the Ku band.

Proposal 2: Define two 3GPP NTN bands covering the same frequencies– one with FR1 numerology and the other with FR2 numerology.

Proposal 3: Revise the “clause objective” of WID [RP-241690] as follows

“Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following but not limited:

  - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths

  - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths

  - For both FR1 and FR2 numerologies, the choice of the subcarrier spacing will be discussed and decided in RAN4”

Proposal 4: RAN4 to consider, as part of the Ku-band WI, the definition of two NTN frequency set of bands:

- One set of bands as FR1-NTN with at least SCS = 30 kHz configuration (specially adapted to narrowband satellite transponders such as 36 and 72 MHz) to address narrowband satellite transponders

- One set of bands as FR2-NTN with at least SCS = 120 kHz configuration (specially adapted to wideband satellite transponders with more than 100 MHz) to address wideband satellite transponders

Proposal 5: Try not to mix FR1-NTN and FR2-NTN numerologies on the same band definition, as per current Ku-band WID description.

Proposal 6: Extend the FR1-NTN range up to 14.5 GHz and extend FR2-NTN range down to 10.7 GHz according to the current Ku-band 3GPP band proposals.

Proposal 7: Define FR2-NTN Ku-bands with CBWs of 50 MHz, 100 MHz, 200 MHz, 400 MHz (following Ka-band definition approach).

Proposal 8: Include the following FR1-NTN CBWs for Ku-band: 25 MHz, 35 MHz, 50 MHz, 70 MHz, 100 MHz (together with 10, 15, 20 MHz already considered for FR1-NTN).

Proposal 9: The Ku-band definition will follow OTA requirements approach for both FR1-NTN and FR2-NTN ranges, for both SAN and UE.

# References

- RP-241690, Revised WID: New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, June, 2024

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), Nov, 2024

- R4-2419902, LS on Ku band numerology, RAN WG4, Nov 2024

# Annex: collected company’s views and related discussions at RAN4#113

Option 1: Propose to define two sets of bands, one for FR1 numerology and the other for FR2 numerology.

Option 2: Add 15 kHz and 30 kHz SCS to FR2-NTN

Option 3: Down select to FR1 numerology or FR2 numerology as per the WID

Option 1 has broad support

Option 2 requires further study by companies during this week.

- The FR1-NTN sync raster would need to be added to FR2-NTN.

- All FR2-NTN SCS would become optional to accommodate different use cases.

- Half as many bands would be required.

- Is RAN1 input not required?

- Any other unintended consequences?

Option 3 would be very difficult to agree

[Intelsat] Preference is Option 1 with 15 kHz and 30 kHz SCS for FR1. Additionally, it is preferred to have 60 kHz SCS option for various services for FR1.

[MediaTek} On the numerology issue, MTK is fine to go with Option 1. Some down-selection on the SCS is fine, if the workload is the concern. In our view, Option 2 can be removed, because it is very confusing and it definitely requires additional works in RAN1 and RAN2.

[Lockheed Martin] Supports - Option 1: which Proposes to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. As These bands would operate independently and offer flexibility !

[European Space Agency] ESA is supporting Option 1. If SCS priority might help the RAN4 workload, I am fine with 15 / 30 / 120.

[Eutelsat] Prefer option 1 with 15 & 30 kHz and 60 kHz as a possible option.

[Qualcomm] QC also support option 1.

1)	Regarding the feasibly of numerology, we don’t think RAN4 has the official agreements on those numbers. For example, in the UL timing analysis, there was no consensus that 31.6 m is only way or round trip path length uncertainty. We would not keep this wording in the WF but if companies think it is helpful, we can put it in the Annex and keep it for information. It would be preferable for us if you can remove the details from WF and capture them in the TR.

2) Regarding the down-selection of SCS from FR1 and FR2, we slightly prefer to choose one from FR1 (e.g., 30kHz) and one from FR2 (e.g., 120khz) to control the workload.

[JSAT] Regarding FR1/FR2 down selection, SKY Perfect JSAT supports Option 1: Propose to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. FR1's small channel bandwidth is necessary not only for legacy GEO satellites but also for upcoming SDS/HTS GEO satellites. In particular, the transponder channel bandwidth of legacy GEO satellite, such as 27MHz, 36MHz or 72Mhz, makes it difficult to provide 5G NTN service without the smaller channels offered by FR1.

[Sharp] We support Option 1(Propose to define two sets of bands, one for FR1 numerology and the other for FR2 numerology) given the estimated work scope is less than other options.

[Thales] Supports option 1.

[Airbus] Supports option 1.

[SES] Supports option 1.

[ESA] Supports option 1.

[CHTTL] Supports option 1

END