3GPP TSG RAN Meeting #106							RP-242599

Madrid, Spain, December 9-12th, 2024

Agenda Item:	15.2

Source:	Ericsson

Title:	On MIMO Phase 6 in Rel.20

Document for:	Discussion, Decision

# 1	Introduction

This contribution proposes a set of potential MIMO Phase 6 enhancements for Rel.20 work item. Our guiding principles are to focus on enhancements based on inefficiencies and shortcomings seen in MIMO product implementation and deployments where the 3GPP specifications is limiting.

We have excluded proposals that are more speculative and requires studies, as we believe those should better be part of the 6G study item for MIMO.

# 2  MIMO Phase 6 proposals

We have identified 4 items to be considered for Rel.20 MIMO and we discuss the motivations here.

## 2.1 Uplink DFT-s-OFDM with rank>1

It is well known that every app in a UE has its own coverage map. So the term “coverage” is a multi-faceted term, as coverage doesn’t simply mean the reach of the signals at the lowest bitrate. For the network deployment, the traditional coverage and cell edge throughput may be important but for the end smart phone user, the App coverage is more relevant.

This proposal which introduces multi-layer UL MIMO for DFT-s-OFDM waveform, targets “App coverage” (and not the traditional cell edge coverage) and is expected to improve the end user uplink throughput and experience.

Figure 1 Every App has its own coverage map

Rel-18 supports dynamic DFT-S/CP-OFDM switching, which allows low PA backoff for single layer DFT-S-OFDM transmission in rank 1 channel conditions, and multi-layer CP-OFDM transmission when channel conditions improve.

A first observation from our system level simulations (Figure 2) is that that rank 2 uplink transmission is very common in a cell, and that multi-layer transmission can be a mechanism to deliver higher power especially for non-coherent UL MIMO UEs. The details of the simulation setup and further discussion can be found in [6].

Therefore, the increased available power from DFT-S-OFDM compared to CP-OFDM can be reaped over the vast majority of the cell since channel conditions are suited to multi-layer transmission. Current specifications prevent access to these benefits since rank is restricted to one for DFT-S-OFDM. Switching to CP-OFDM gives rank 2 possibility but this implies that the power boosting benefit is lost.

Figure 2	UL MIMO rank histograms for 4 and 32 Rx gNB in UMa 500m

To quantify the relative gains of fast DFT-S/CP-OFDM switching and multi-layer DFT-S-OFDM, we considered 3 scenarios where the two first ones are possible in today’s specifications:

- Rel-15 codebook-based UL MIMO using CP-OFDM,

- Rel-18 fast switching between CP-OFDM and DFT-S-OFDM for rank 1, but CP-OFDM for rank 2, and

- Proposed Rel-19 DFT-S-OFDM for both rank 1 and rank 2.

And the comparison is given in these tables. The “App coverage” would be some value between then mean and the cell edge throughput gains.

Table 1: Mean and cell edge throughput gains over Rel.16 for fast switching and multi-layer DFT-S-OFDM: 2Tx, xpol.

|  | Mean Throughput Gain |  |  | Cell Edge Throughput Gain |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20% | 50% | 70% | 20% | 50% | 70% |
| Rel.15 CP-OFDM | 0% | 0% | 0% | 0% | 0% | 0% |
| Rel.18 Fast-Switching | 4% | 2% | 1% | 17% | 27% | 34% |
| Rel.19 DFT-S-OFDM | 11% | 14% | 17% | 31% | 33% | 34% |

Table 2: Mean and cell edge gains over legacy for fast switching and multi-layer DFT-S-OFDM: 4Tx, directional.

|  | Mean Throughput Gain |  |  | Cell Edge Throughput Gain |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20% | 50% | 70% | 20% | 50% | 70% |
| Rel.15 CP-OFDM | 0% | 0% | 0% | 0% | 0% | 0% |
| Rel.18 Fast-Switching | 5% | 8% | 9% | 68% | 63% | 59% |
| Rel.19 DFT-S-OFDM | 12% | 16% | 19% | 70% | 78% | 81% |

The non-coherent codebook is used in all simulated cases with Rel-16 UL full power ‘mode 0’ (where each PA is capable of full power).  We note that this full power mode will reduce the gains from rank 2 DFT-S-OFDM relative to rank 1 DFT-S-OFDM, since the full power will be available for rank 1 non-coherent operation.  The 500m UMa scenario is used, given its relevance to coverage.

Observing that there is 1 and 3 dB MPR in 38.101 in Section 6.2.2 for QPSK with DFT-S-OFDM and CP-OFDM, respectively, in the edge and outer bands, and a similar difference in power (of 1.5 dB) for the inner bands, in these simulations we model DFT-S-OFDM transmission maximum power as 22 dBm, while CP-OFDM is 20 dBm.  We expect these differences are conservative given that 38.101 captures minimum performance requirements and given the higher amounts of cubic metric observed as discussed above. Furthermore, techniques to further reduce MPR for DFT-S-OFDM were studied in Rel-18, and transparent MPR reduction of 1 dB via power boosting was specified in RAN4. If boosting is used or further MPR reduction techniques are specified in later releases, there may be additional coverage benefits of DFT-S-OFDM, and therefore multi-layer DFT-S-OFDM.

The system level simulations further use FTP model 1 traffic, and link adaptation selection between rank 1 and 2. Results from both 4 Tx and 2 Tx are given Error! Reference source not found. and Error! Reference source not found. below, respectively, for 20, 50, and 70% cell load, as gains over the CP-OFDM baseline. Detailed simulation assumptions are given in [6].

Examining the results, we see that the main gains from fast switching are for cell-edge throughput, especially at low loads.  The 2 Tx gains range from 17—34%, while the 4 Tx gains are 59—68%.  However, the mean throughput gains from fast switching are more modest: up to 4% and 9% with 2 and 4 Tx, respectively.

By contrast, while multi-layer DFT-S-OFDM further improves cell edge gains, it additionally provides strong mean throughput gains, especially at high load and with 2 Tx.  Mean throughput gains of 11—17% and 12—19% are found for 2 and 4 Tx, respectively, which are up to 16% and 10% better than fast switch for 2 and 4 Tx (at 70% resource utilization). Cell edge gains for 2 Tx with multilayer DFT-S-OFDM range from 31% to 34% and up to 14% better than fast switching (at 20% resource utilization), while for 4 Tx they are 70—81%, and up to 12% better than fast switching (at 70% resource utilization.)

These higher mean throughputs from rank 2 DFT-S-OFDM as compared to fast switching are natural since the higher throughput from rank 2 is available over more of the cell, i.e., improved “App coverage”.  Similarly, since rank 2 is commonly used in the cell, multi-layer DFT-S-OFDM can even improve cell edge performance.

- The Rel.18 fast DFT-S/CP-OFDM switching has strong cell edge gains over configured DFT-S- & CP-OFDM waveforms, but less benefit to mean cell throughput.

- The proposed Rel.20 multi-layer DFT-S-OFDM has substantial mean throughput gains (e.g., ~15% or ~20% for 2 or 4 Tx) at the system level, as well as for cell edge (e.g., up to ~35 or ~80% for 2 or 4 Tx).

- Because it improves coverage of rank 2+ UL MIMO, multilayer DFT-S-OFDM improves mean throughput substantially over fast DFT-S/CP-OFDM switching, by up to, e.g., 16% and 10% for 2 Tx and 4 Tx, respectively.  Multi-layer DFT-S-OFDM also improves cell edge throughput over fast switching by up to, e.g., ~14% or ~12% for 2 or 4 Tx.

Based on these observations, we propose

For MIMO Phase 6, introduce multi-layer DFT-S-OFDM for PUSCH.

## 2.2 MAC CE based QCL switching of TRS

The QCL source for the PDCCH/PDSCH reception is the TRS, and since a TRS always is associated with an SSB via QCL association (‘QCL-Type-C’ and ‘QCL-TypeD’ where applicable), it is preferred to configure a TRP at FR2 with a one-to-one mapping between an SSB and a TRS (i.e., for each transmitted SSB, a TRS is transmitted in the same beam as the SSB).

However, for TRPs at mmWave frequencies with many beams and carriers, it is not possible to configure the UE with one TRS per SSB per carrier due to that UEs support a limited number of TRSs across all configured carriers (indicated by maxConfiguredResourceSetsAllCC in 38.306).

This means that when a UE moves to a new SSB beam which is not associated with an TRS (for one or more of the DL carriers), the TRP needs to update the QCL of a TRSs such that it is associated with the new SSB. The QCL update of the TRS requires RRC signaling, which introduces additional latency and overhead signaling in the system.

- For TRPs in FR2 with many beams and DL carriers, the QCL source of TRS need to be updated when a UE moves around in the cell. This requires RRC signalling, which introduces additional latency, overhead and NW side processing complexity.

To mitigate this issue, it would be beneficial to introduce a more dynamic QCL source indication for TRSs, hence we propose:

For MIMO Phase 6, introduce dynamic switching for QCL source of TRS.

In practice, this would imply that we allow that a semi-persistent TRS is used as QCL source for PDCCH/PDSCH DMRS. The UE would use this semi-persistent TRS also for radio link monitoring and beam failure detection. As the QCL source for semi-persistent CSI-RS can be updated using MAC CE, RRC reconfigurations are avoided.

## 2.3 Interference plus noise report from UE

Reciprocity based DL MIMO precoding, when conditions allow it to be used, has advantage over CSI feedback-based precoding at least in TDD systems and is widely used in current 5G systems.  In this case, the NW has full knowledge of the channels to all the MU-MIMO-scheduled UEs and can perform cross-UE interference suppression on the NW side. However, setting the correct transmission rank and MCS for these UEs requires additional efforts and the use of the non-PMI based CSI feedback can be  used to transmit precoded CSI-RS with multiple rank hypotheses and let the UE to determine and report a preferred rank and an associated CQI.

However, that is a two-step approach (SRS first, then CSI-RS based report using precoded CSI-RS based on the SRS measurements) which introduce additional delays between the measurement of the uplink channel and to the point in time when CQI has been obtained. This delay is causing a CQI inaccuracy due to channel aging. In addition, since the precoding is UE specific, large feedback overhead and scheduling complexity can occur with this approach when there are many UEs.

It is noted that what is missing for the NW to autonomously perform the rank selection and MCS determination is the interference plus noise experienced by each UE. Hence, a possible enhancement in MIMO Phase 6  is to introduce an interference plus noise report from the UE.  This report is not linked to a certain precoder or SRS measurement, it can be a stand-alone report triggered when needed by the network. This would help the gNB to have a more accurate DL CQI estimation without the need to relying on the two-step non-PMI based reporting procedure.

It should be noted that the interference plus noise experienced by the UE may vary quite dynamically.  Hence, how to acquire interference plus noise related information at the network node when the interference level varies dynamically is an issue that may need to be further discussed. In addition, the dynamic range and resolution of the IpN report needs to be addressed as well as the pros/cons of having a subband report vs. a wideband report.

For MIMO Phase 6, consider interference plus noise reporting from the UE to improve link adaptation for reciprocity-based DL MIMO transmission.

## 2.4 Early CSI reporting

Link adaptation in NR and LTE is rather slow and inaccurate. Often a conservative approach is needed (start with lowest MCS) and an open loop link adaptation (OLLA) algorithm is used in the gNB to adjust the CSI reported from the UE to make a better MCS selection for PDSCH.  In NR, a CSI report is only available to the gNB after the UE is RRC configured with the corresponding CSI reporting configuration and when the UE has had sufficient time to measure and report the CSI.  Hence, efficient transmission (including multi-layer transmission) is delayed until CSI reporting configuration is completed and CSI feedback is available.

To activate an SCell, there has been enhancements in NR to reduce the activation delay, but it still takes 10s of milliseconds before MIMO can be used on that SCell. Also, the CSI reported on recently activated SCells have turned out to be inaccurate, leading to a large number of HARQ retransmissions. Typically, the NW starts to transmit the large packet on already activated cells, and when MIMO is enabled on the additional SCell, the full packet is already delivered.

This motivates the need for specifying early CSI reporting where the UE can report CSI during initial access so that the gNB can acquire a CSI report for a new cell even before completion of CSI reporting configuration via RRC.  We provide below simulation results to highlight the benefits of supporting early CSI.  Simulation assumptions corresponding to the results are shown in Table 4.

Figure 3 shows the reduction in number of slots needed to receive an FTP packet of size of 25 kilobytes with FTP model 1 for the following schemes:

- Early CSI Feedback:  case where single port CSI feedback with early CQI reporting which is taken into account by gNB for scheduling PDSCH

- Initial MCS = 0:  case where there is no early CSI and the gNB uses the most conservative MCS initially for scheduling PDSCH

- Initial MCS = 26:  case where there is no early CSI and the gNB uses the most aggressive MCS initially for scheduling PDSCH

Among the 2 last schemes, ‘Initial MCS=0’ is the most practical as the gNB will likely schedule PDSCH with an initial MCS that is most conservative in the absence of early CSI.  As can be seen from Figure 3, early CSI feedback can significantly reduce the number of slots needed to receive a packet when compared to other schemes where early CSI is not available.

Figure 3. Reduction in slots needed to receive a packet with early CSI

Figure 4 shows performance comparison between early CSI with different number of ports.  Here, PDSCH scheduling is according to RI/PMI/CQI reported as part of the respective early CSI reports.  As can be seen from the figure, notable performance improvement can be achieved when increasing the number of ports in the early CSI.

Figure 4. Performance comparison of early CSI with different number of ports

Based on the above early results, we propose the following:

For MIMO Phase 6, introduce early CSI reporting for both single port and multiple ports.

# 2	Discussion on MIMO Phase 6 proposals from other companies

## 2.1 Uplink Frequency Selective Precoding and Precoder Cycling

As can be seen from proposals for the up scoping of MIMO Phase 5 discussed in RAN#105, there is continued interest in open loop based UL precoder cycling.  OL precoder cycling is the simplest form of uplink frequency selective precoding, and so it may have less performance benefit than more complex forms.  Therefore, it is worth considering UL FS precoding in the most general sense, including closed loop FS precoding.  In this section, we first consider the performance of OL precoder cycling, and then aspects such as implementation complexity and its relation to alternative solutions.  Next, we consider potential schemes for CL FS precoding and what aspects should be studied.

It should be noted that current uplink channel and interference estimation algorithms operate on the full PUSCH bandwidth. If UL FS precoding is introduced, then gNB needs to cope with this in their implementation and additionally, there is likely performance drop due to smaller averaging bandwidth.

Introducing uplink frequency selective precoding requires new and narrowband channel and interference covariance matrix estimation algorithms in gNB, compared to today’s wideband estimation. The expected performance benefits needs to motivate the additional implementation efforts and cost as well as the drawback of lacking a wide signal for estimation.

In addition, the PAPR is severely affected, especially if UL FS is applied to DFT-s-OFDM based precoding. Furthermore, there will be an impact to radio interfaces and processing, if subband precoding is used, this needs to be better understood as well to assess the feasibility to introduce such feature late in a “G-cycle”.

### 2.1.1  Open Loop Precoder Cycling

Open loop precoder cycling provides spatial diversity by changing an effective antenna pattern as a function of frequency.  The cycling is usually done over some number of consecutive PRBs in a precoding resource block group (PRG).  Small PRG sizes will have worse channel estimation and interference covariance estimation  performance, since frequency domain averaging is not possible, while large PRG sizes have less diversity gain.  Therefore, this tradeoff of channel and interference estimation vs. diversity gain is a key aspect of OL precoding cycling.

Another consideration is that closed loop precoding will outperform OL precoder cycling when there is sufficient uplink CSI for accurate precoder selection.  This often limits the benefit of OL precoder cycling to where the UE is traveling at high enough speeds that CSI from SRS is stale.

It should be noted that OL precoding for coherent DL is specified for 2TX and 4TX in LTE. The 4TX version use 16 precoders and is not useful and is to our knowledge not used in any deployments. The reason is that with 4 TX, only a few precoders out of the 16 provides an effective channel with gain, the others just waste energy into the channel null subspace.

OL precoding will generally not allow coherent precoding to apply, in the sense that there is no TPMI selection by the network that would allow the phase of the Tx chains to align such that they combine coherently.  On the other hand, it is possible to apply a random relative phase among the Tx chains such that their average power combines, which will avoid loss of power according to Rel-15 UL power scaling for non-coherent precoders.

Factors such as channel estimation losses, performance over closed loop precoding as a function of UE speed, and the need for non-coherent transmission should be considered when evaluating the performance of OL precoder cycling.

Figure 5 and Figure 6 present the link-level UL throughput comparison between non-coherent Rel.15 closed-loop (CL) wideband precoding and open-loop (OL) precoding cycling. The analysis considers scenarios where the UE moves at speeds of 3 km/h and 60 km/h, transmitting with a maximum rank of 1. The UE is equipped with 2 full-power non-coherent transmit chains, while the receiver at the network has 4 and 32 receive chains for the evaluation in Figure 5 and Figure 6, respectively. Accordingly,

- When the UE moves at a pedestrian speed of 3 km/h, OL precoding cycling experiences a performance loss over a wide SNR range compared to CL wideband precoding (i.e. Rel.15 specification).

  - Specifically, at SNRs of -5, 0,10 dB, losses are observed for OL precoding cycling as -26, -19,-12% and -14, -13,-7%  for 4 and 32 receive chains, respectively. This performance gap arises because, in relatively static channels, non-coherent CL precoding can select the optimal transmit chain, whereas OL precoding randomly selects transmit chains per subband.

- In contrast, at a higher UE speed of 60 km/h, OL precoding cycling performs comparably to CL wideband precoding, regardless of the number of receive chains at the network.

  - The rapid channel variations at this speed provide a diversity gain for OL precoding cycling, enhancing its performance. However, despite this improvement, OL precoding cycling still fails in outperforming CL wideband precoding (i.e., Rel.15 precoding).

Figure 5	UL throughout versus SNR, comparing non-coherent Rel.15 CL wideband precoding and OL precoding cycling for 4 Rx (left figure) and 32 Rx (right figure) with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 1 and is moving with pedestrian speed, i.e., 3 km/h. The remaining simulation parameters are collected in Table 3 in the Appendix.

Figure 6	UL throughout versus SNR, comparing non-coherent Rel.15 CL wideband precoding and OL precoding cycling for 4 Rx (left figure) and 32 Rx (right figure) with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UE transmits with max rank 1 and is moving with 60 km/h. The remaining simulation parameters are collected in Table 3 in the Appendix.

Similar to the above, Figure 7 and Figure 8 present the link-level UL throughput comparison between non-coherent CL wideband precoding and OL precoding cycling, where the UE moves at speeds of 3 km/h and 60 km/h, transmitting with a maximum rank of 2. The receiver at the network has 4 and 32 receive chains for the evaluation in Figure 7 and Figure 8, respectively Accordingly,

- At a pedestrian speed of 3 km/h, OL precoding cycling suffers noticeable performance losses at low SNRs compared to CL precoding but achieves similar performance at higher SNRs.

  - For instance, at an SNR of -5 dB, OL precoding cycling shows losses of -26% and -8% for 4 and 32 receive chains, respectively. This loss at low SNRs is attributed to the higher frequency of rank-1 transmissions, where OL precoding cycling in a relatively static channel is less effective, as discussed above. At higher SNRs, the probability of rank-2 transmission increases, and since only one rank-2 non-coherent precoder is available, both precoding schemes perform equally well.

- At a UE speed of 60 km/h, OL precoding cycling achieves comparable performance to CL wideband precoding, irrespective of the number of receive chains in the network.

  - The rapid channel variations at this speed provide a diversity gain for OL precoding cycling in rank-1 transmission at low SNR, narrowing the performance gap with CL wideband precoding. On the other hand, at high SNR, the absence of a performance gap is attributed to the dominance of rank-2 transmission, where only a single non-coherent precoder is available. As a result, OL precoding cycling does not surpass CL wideband precoding under these conditions.

Figure 7	UL throughout versus SNR, comparing non-coherent CL wideband precoding and OL precoding cycling for 4 Rx (left figure) and 32 Rx (right figure) with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 2 and is moving with pedestrian speed, i.e., 3 km/h. The remaining simulation parameters are collected in Table 3 in the Appendix.

Figure 8	UL throughout versus SNR, comparing non-coherent CL wideband precoding and OL precoding cycling for 4 Rx (left figure) and 32 Rx (right figure) with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 2 and is moving with 60 km/h. The remaining simulation parameters are collected in Table 3 in the Appendix.

We thus have a set of observations:

Compared to non-coherent closed loop wideband precoding (Rel.15), and when the maximum rank is fixed to 1, precoder cycling with two active Tx chains has a loss at pedestrian speeds over a wide range of SNRs and the same performance at 60 kmph.

Compared to non-coherent closed loop wideband precoding (Rel.15), and when the maximum rank is 2, there is less loss from precoder cycling with two active Tx chains at pedestrian speeds, but this is likely because there is no gain from precoding at rank 2.

The use of 32 vs 4 gNB Rx antennas does not seem to affect the relative gains of (frequency selective) precoder cycling and (wideband) closed loop precoding.

Mechanisms that can improve uplink robustness for UL MIMO are important for NR, and enhancements can and should be considered. However, at this stage of NR, there is a wide variety of mechanisms already specified as well as possible in implementation. Therefore, it is crucial that careful study is made of enhancements, comparing to the best relevant schemes already in NR using system level simulation studies with realistic assumptions.  Results in this section compare to wideband closed loop precoding, but other schemes may have as good or better performance at higher UE speeds.

Complexity is also (as always) a key factor, and frequency selective precoding schemes such as OL UL precoder cycling can impact both UE and gNB complexity. UE PAPR can be increased by FS precoding, which can increase UE Tx chain complexity and/or power requirements, possibly reducing any coverage gains from FS precoding. FS precoding can also complicate gNB implementation by requiring more complex channel estimation, as well as greater storage and/or bus bandwidths given the increased number of channel coefficients, especially in massive MIMO reception.  For a given level of gNB complexity, there may be reduced UL performance from degraded channel and interference estimation.

Moreover, UL FS precoding schemes are a strong deviation from Rel-15, since uplink PRGs (and therefore frequency selective precoding) would be defined, which should be considered in a wider context than a small part of a Rel-20 WI or a TEI.

We therefore think if UL precoder cycling is to be considered, further study is needed, including the performance gains over frequency hopping based schemes, possibly combined with CDD, etc. Other uplink robustness schemes should also be considered, including enhancements to frequency hopping, etc.

In addition, system level simulations that take into account the reduced interference covariance matrix estimation bandwidth need to be performed.

MIMO robustness can go beyond diversity schemes. As discussed in Section  2.1, DFT-S-OFDM allows higher power transmission since PA backoff is reduced, and this higher power can improve app coverage. NR lacks support for multi-layer DFT-S-OFDM, even though it is supported in LTE. Having higher power allows more frequent use of multi-layer transmission, and so improves UL throughput. The LTE UL MIMO codebooks supporting DFT-S-OFDM are simple and closely related to NR UL MIMO codebooks.

Enhancements to UL MIMO robustness should be compared to the best relevant schemes already available for NR in specification and by implementation (e.g. frequency hopping and/or transparent diversity). Alternatives such as enhanced frequency hopping schemes and multi-layer DFT-S-OFDM should also be considered. Gains of these enhancements should be shown for the system level.

Potential study of frequency selective precoding schemes should not be limited to open loop UL precoder cycling, and substantial time should be allocated to ensure the most beneficial schemes are investigated.

So we propose

Study of open loop frequency selective precoding in 6G SI to explore whether and how it can provide sufficient benefits.

### 2.1.2 Closed Loop Frequency Selective Precoding

Closed loop frequency selective precoding is natural to consider for the uplink, given its well known gains for MU-MIMO on the downlink.  There has been interest in FS precoding for the NR uplink since Rel-15 (see e.g. R1-1721399), and this has revived in more recent discussions.  However, to our knowledge, there has not been an in-depth study of frequency selective precoding that assumes a number of important features of NR.

A first important feature is the wide bandwidths ultimately used for NR.  These wide bandwidths may require very high TPMI overhead for UL precoding that can’t be carried in an NR PDCCH.

Furthermore, we are not aware of studies that take into account the impact of UE RF limitations e.g. on maximum Tx chain power, coherence, or PAPR.  UL MIMO UEs in present deployments use non-coherent transmission, which can lead to a need for full power Tx chains in order to maintain UL performance, and which would require constraints on frequency selective transmission.  If fully coherent UEs are used, schemes such as DFT-S-OFDM to reduce PAPR may not be supported.  RAN4 requirements for fully coherent operation will need to be revised for frequency selective transmission, and related deviations from ideal behaviour may need to be taken into account in order to determine the potential gain of coherent frequency selection precoding.

It may be observed that frequency selective precoding is already supported in NR, in a multi-user sense.  If UEs are frequency multiplexed, a precoder corresponds to each UE and is applied over a portion of the band.  In order for frequency selective precoding transmissions from each UE to be beneficial, this may imply that UEs should be scheduled in a wideband mode, and which may limit scheduling flexibility.  Therefore, if scheduling strategies for frequency selective precoding require wideband transmission for gain may need study.

The gains from adaptive precoding decrease with increasing rank, and so performance studies of FS precoding should take into account rank adaptation.  Similarly, appropriate gNB antenna configurations should be considered, since multi-antenna reception in gNB improves performance and can allow higher rank UE transmission.

Lastly, since interference limits the capacity of the uplink, and again given the gains on the downlink, FS precoding schemes targeting the suppression of UL interference should be studied.

Frequency selective precoding is known to significantly improve throughput on the downlink, but its benefit on the uplink is not as well studied.

Uplink frequency selective precoding overhead can be large, as it grows with bandwidth and the number of UEs served in a slot. Such large overheads may not fit in Rel-19 NR PDCCH.

NR supports frequency multiplexed PUSCHs, and in that sense already supports frequency selective precoding.  If scheduling with FS precoding requires wideband transmissions may therefore need study.

If fully coherent UEs are assumed for FS precoding, UE RF implementation aspects such as maximum Tx chain power,  PAPR, and relative phase error requirements should be taken into account in the schemes’ designs.

Given the downlink gains, potential of FS precoding targeting uplink interference suppression should be studied.

Study the use of closed loop frequency selective precoding in 6G SI, considering impact on control channel designs and taking into account UE RF implementation aspects such as maximum Tx chain power, PAPR, and relative phase error requirements.

# Conclusion

Given the discussion and observations, we propose the following:

Proposal 1	For MIMO Phase 6, introduce multi-layer DFT-S-OFDM for PUSCH.

Proposal 2	For MIMO Phase 6, introduce dynamic switching for QCL source of TRS.

Proposal 3	For MIMO Phase 6, consider interference plus noise reporting from the UE to improve link adaptation for reciprocity-based DL MIMO transmission.

Proposal 4	For MIMO Phase 6, introduce early CSI reporting for both single port and multiple ports.

Proposal 5	Study of open loop frequency selective precoding in 6G SI to explore whether and how it can provide sufficient benefits.

Proposal 6	Study the use of closed loop frequency selective precoding in 6G SI, considering impact on control channel designs and taking into account UE RF implementation aspects such as maximum Tx chain power, PAPR, and relative phase error requirements.

# References

RWS-230154, “Discussion on Release 19 MIMO enhancements”, Ericsson, RAN Rel-19 WS, Taipei, June 2023

# Appendix

Table 3	Parameters for UL link level simulations for open-loop precoding cycling

| Link level simulation parameters |  |
| --- | --- |
| Metric | UL throughput |
| Carrier frequency | 3.5 GHz |
| Subcarrier spacing | 30 kHz |
| Bandwidth | 52 PRBs (~ 20 MHz) |
| Channel model | CDL-C with 300 ns delay spread |
| Power mode | Rel-16 mode 0 |
| SRS periodicity | 10 ms |
| Modulation | Up to 256 QAM |
| Channel estimation | Practical, including optimizations for frequency selective transmission |
| Link adaptation | Practical |
| Receiver | MMSE-IRC |
| BS antenna configuration | - (M,N,P,Mg,Ng,Mp,Np) = (4,8,2,1,1,2,8) with (dH, dV) = (0.5, 0.8)λ, tilt: 10° - (M,N,P,Mg,Ng,Mp,Np) = (8,2,2,1,1,1,2) with (dH, dV) = (4.0, 0.5)λ, tilt: 10° |
| BS antenna pattern | Directional (8 dBi, 65° BW) |
| BS noise figure | 5 dB |
| UE antenna configuration | 1 dual pol with isotropic antenna element |
| UE coherence | Non-coherent |
| Max rank | 1 or 2 |
| UE speed | 3 km/h or 60km/h |
| Subband granularity | 4 PRBs per subband (for precoding cycling) |

Table 4	Parameters for DL system level simulations for early CSI reporting

| Metric | DL throughput |
| --- | --- |
| Carrier frequency | 3.5 GHz |
| Subcarrier spacing | 30 KHz |
| Bandwidth | 20 PRBs |
| Scenario | UMa |
| Traffic Model | FTP Model 1, 25kbytes packet size |
| Max rank | 4 |
| UE speed | 3 km/h |