3GPP TSG RAN Meeting #106	RP-242765

Madrid, Spain, 9th – 12th December 2024

# Source:	ViaSat Satellite Holdings Ltd, Inmarsat, Terrestar Solutions, Ligado Networks, Omnispace, ESA, Novamint

# Title:	New WI Proposal: Intra-SAN Carrier Aggregation (CA) for NR-NTN

# Document for:	Approval

Agenda Item:	9.1.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Intra-SAN Carrier Aggregation (CA) for NR-NTN

Acronym: NR_NTN_CA

Unique identifier:	9xxxxx

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Target Release:	Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
|  |  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

Carrier Aggregation (CA) is an available feature in Terrestrial Networks (TN) but is not part of Non-Terrestrial Networks (NTN).  This feature disparity causes significant problems for satellite networks that are being planned as it impacts user speeds and experiences.

There are numerous practical reasons which justify the introduction of CA

- NTN channel allocations are constrained (especially in FR1). Typically operators have access to non-contiguous bands of spectrum

- NR NTN smartphones are typically UL link budget limited and thus a practical way to increase the number of PRBs assigned to a UE is by assigning different PRB groups to different Power Amplifiers (PAs) available in the UE and this necessitates CA.

- Non-smartphone devices (e.g. CPE/FWA, automotive/vehicular, UAV/UAS, directional/high-gain UE) with higher power class/higher gain antennas will especially benefit from CA and this is essential for opening up new markets for NTN operators.

- User experience/speeds necessitate CA in both uplink and downlink.  Without this the user experience may not enable a successful NTN deployment from the point of view of user uptake.

- The recently introduced approved WI on the introduction of the Ku band may also require the introduction of Carrier Aggregation in order to support the higher channel bandwidths required by satellite operators in this bands (e.g. such as 250MHz), in case the FR1 numerology structure is adopted.

- Current satellite constellations being designed are typically using dual bands in the same payload

# 4	Objective

### 4.1	Objective of Core part WI

The Core Part objectives are as follows:

- Specify the introduction of Carrier Aggregation (CA) for NR NTN for intra-SAN scenarios where component carriers are co-located in the same Satellite Access Node (SAN) and supported by the same payload.

- Support for intra-band and inter-band non-contiguous CA

- Support for inter-band support for DL CA,

- UL CA

  - Simultaneous intra-band non-contiguous and inter-band UL CA in low-low combinations is limited to non-handheld UE

- UL CA with TX band switching

- Support for CA shall be agnostic to NR NTN band(s)

- Specify the necessary RF and RRM requirements

- Reuse the same assumptions from existing NR NTN specs (e.g. frequency error, timing reference, timing error, etc)

- Maximize reuse of existing requirements from terrestrial specifications where applicable

- Assume PC3 per each TX as a starting point

- Additional UL/DL RF requirements for RX MSD, TX MPR/A-MPR can be further specified if necessary.

- Reuse the existing signalling from the terrestrial specifications

- Consider n256, n255, n254 as reference bands as a starting point

- Support for other bands, including those currently being defined in Rel-19 may be added later as part of a Basket WI

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

The Performance Part objectives of the work item are the following:

- Specify RRM performance requirements [RAN4]

- Specify UE demodulation requirements [RAN4]

- Specify SAN demodulation requirements [RAN4]

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | NR; Physical channels and modulation | RAN#109 | Core part |
| 38.212 | NR; Multiplexing and channel coding | RAN#109 | Core part |
| 38.213 | NR; Physical layer procedures for control | RAN#109 | Core part |
| 38.214 | NR; Physical layer procedures for data | RAN#109 | Core part |
| 38.215 | NR; Physical layer measurements | RAN#109 | Core part |
| 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#109 | Core part |
| 38.304 | NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state | RAN#109 | Core part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#109 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification: | RAN#109 | Core part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#109 | Core part |
| 38.101-5 | NR; User Equipment (UE) radio transmission and reception, part 5: Satellite Access Radio Frequency (RF) and performance requirements | RAN#109 | Core part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#109 | Core part |
| 38.108 | NR; Satellite Access Node radio transmission and reception | RAN#109 | Core part |
| 38.101-5 | NR; User Equipment (UE) radio transmission and reception, part 5: Satellite Access Radio Frequency (RF) and performance requirements | RAN#111 | Perf part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#111 | Perf part |
| 38.181 | NR; Satellite Access Node conformance testing | RAN#111 | Perf part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

TBD

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN WG4

Potential secondary responsible Working Group(s) (TO BE CONFIRMED): [RAN1], RAN2

# 8	Aspects that involve other WGs

None so far.

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Inmarsat

Viasat

Terrestar Solutions

Omnispace

ESA

Novamint

Ligado Networks