3GPP TSG RAN meeting #106										RP-242877

Madrid, Spain, December 9 – 12, 2024

## Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Evolution of NR duplex operation: Sub-band full duplex (SBFD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_duplex_evo |  |  |  |  |
| Unique ID | 1020090 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241614 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 55% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xinghua Song, He (Jackson) Wang |
|  | Company | Huawei, Samsung |
|  | Email | songxinghua@huawei.com, h0809.wang@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#118bis, the following agreements were made.

SBFD TX/RX/measurement procedures

Conclusion

Nominal RBG size for PDSCH/PUSCH in SBFD symbols is determined based on the size of DL/UL BWP as legacy. Nominal CSI reporting subband size in SBFD symbols is determined based on the size of DL BWP as legacy.

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs.

- UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception.

- For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal.

- For collision between PRACH triggered by higher layer and DL channel/signal, leave it to UE implementation whether to receive the DL or transmit PRACH.

Agreement

For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands,

- Option 3: It is up to UE capability to indicate one of the following:

  - CSI processing timeline and CPU counting are the same as legacy.

  - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Agreement

The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols.

- The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy.

The number of bits used to indicate FH offset in DCI is determined as legacy.

Agreement

Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options.

- Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

  - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols.

    - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped.

    - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission.

    - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols.

  - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols.

    - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped.

    - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission.

    - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols.

  - FFS applicable usages

- Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource.

  - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured.

    - FFS separate configurations of other parameters

  - FFS applicable usages

Agreement

For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Agreement

For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability

- UE capability reports the maximum number of supported partial PRGs in SBFD symbols.

Agreement

- For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs.

Agreement:

For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by:

RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1

For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by:

RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1

Where

- RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP.

- NUL SBsize is the number of UL usable PRBs.

- RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP.

- RB offset is the frequency hopping offset for PUSCH in SBFD symbols

- Note: Definition of nsμ is unchanged from existing specifications

Decide in RAN1#119, one of the following options:

- Alt1: Remove -RBUL SB start in the above equations

- Alt2: Remove square brackets from [-RBUL SB start] in the above equations

Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options

- Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis

- Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis

  - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded.

- Option 3: Configuration 1 or Configuration 2 is on a per cell basis.

  - No separate UE capabilities for Configuration 1 and 2.

Agreement

At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots,

- The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy.

- The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy.

- Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols:

  - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation:

    - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD

      - Negative values of RBoffsetSBFD are not precluded.

    - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize

    - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

    - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize

  - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation:

    - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize)

    - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize )

    - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) +  RBstartUL SB

    - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD

  - The variables are defined as follows:

    - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP

    - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP

    - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP

    - NBWPsize is the number of PRBs of UL BWP

    - NUL SBsize is the number of PRBs of UL usable PRBs

    - NPUSCHsize is the number of PRBs for PUSCH transmissions

  - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols.

    - FFS: Condition to enable/disable

  - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs.

  - It applies at least to RA type 1. FFS for RA type 0.

  - Note: Other equations are not precluded.

Working Assumption

For SSB symbols configured with SBFD subbands,

- Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

  - Note: The SSB block is assumed to be within DL subband

Agreement

If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs.

- Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.

Agreement

For a PDSCH/PUSCH with assigned PRBs overlapping with SBFD subband boundary and only the PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH reception/transmission, legacy DMRS sequence mapping is applied to assigned PRBs within DL/UL usable PRBs only (effectively, this is same as the case when the DMRS sequence mapped to the RBs outside the DL/UL usable PRBs are punctured).

Agreement

For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols,

- When an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

- SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

- FFS: The above applies to SP-CSI on PUSCH when applicable

For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, then

- Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH.

- Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

- Existing RB indexing and VRB-to-PRB mapping are reused.

- The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

SBFD random access operation

Agreement

For RACH configuration Option 1 with Alt 1-1, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs.

- The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs.

Agreement

For RACH configuration Option 2, an additional-RO is valid if:

- It is within SBFD symbols, or network configures that it can be valid if it starts from SBFD symbol and ends in non-SBFD symbol either in the same slot or across different slots.

- It starts at least Ngap symbols after a last downlink non-SBFD symbol.

- It starts at least Ngap symbols after the latest SSB

- It does not overlap with SSB in time domain

Note1: The Ngap here is the same as the Ngap in the current specification.

Note2: It is up to network to ensure an additional-RO is configured within the bandwidth of the UL usable PRBs.

Agreement

Confirm the following Working Assumption:

Working Assumption

For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

- FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

Agreement

Confirm the following working assumption.

Agreement

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

Agreement

For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols

- FFS: Details of the separate power control parameters

Conclusion

There is no consensus in RAN1 to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs. It’s up to RAN2 to decide whether to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs.

Working Assumption

For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs.

- the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Conclusion

PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported.

Agreement

For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission, reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop, with the update that the frequency offset for 2nd hop is based on the size of UL usable PRBs.

- FFS the case when the value of NUL,hop= 11

- FFS the definition of UL usable PRBs for Msg3 PUSCH transmission

Agreement

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives.

- Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs).

- FFS: Possible modifications

Conclusion

There is no consensus in RAN1 to support the following in Rel-19:

Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

CLI handling

Agreement

Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement.

- The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic

- Note: No new usage needs to be defined for SRS resource sets

Agreement

For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter.

- FFS: Whether to reuse the legacy SRS resource set

- FFS: Whether available slot offset list is also reused

For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

Agreement

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, one of the following options is selected for DG PUSCH and Type 2 CG PUSCH

- Option 1: The time location of each of one or two UL muting symbols is semi-statically configured and cannot be dynamically indicated

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

- Option 3: The time location of each of one or more UL muting symbols is semi-statically configured, and none, one or two the time location of UL muting symbol(s) is dynamically indicated by TDRA field in DCI

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH.

FFS: details of semi-static configuration and dynamic indication (if supported)

FFS: how to handle UL resource muting for PUSCH repetition and for multi-PUSCH scheduled by a single DCI

Agreement

For UCI rate matching, to determine the number of coded modulation symbols per layer for UCI transmission on PUSCH, the following option is adopted

- Option 2: The number of coded modulation symbols per layer for UCI on PUSCH is calculated based on MscUCIl after excluding the UL muting resource.

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

Agreement

For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs.

- A single wideband RSSI measurement report is supported

  - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Agreement

For aperiodic CLI reporting on PUSCH

- Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig

- Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report

- Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig

Agreement

- For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

Agreement

In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList:

- In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo

  - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic

- If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource:

  - If unifiedTCI-StateType is not configured

    - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET”

  - If unifiedTCI-StateType is configured

    - “Indicated” DL only/joint TCI state as specified in TS 38.214

  - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources

FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

Agreement

Update previous agreement as follows (update in red).

Agreement

Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement.

- The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic

- The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications.

- Note: No new usage needs to be defined for SRS resource sets

Agreement

Update the following agreement (update in red).

Agreement

For aperiodic CLI reporting on PUSCH

- Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig

- Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report

- Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig

- Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources.

Agreement

Alt 2: To determine the value of k in Prii,CSIy,k,c,s for CSI reports carrying L1 UE-to-UE CLI report, the following options is adopted

- Option 1-2: Reusing existing k value, k = 1

Agreement

If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol.

In RAN1#119, the following agreements were made.

SBFD TX/RX/measurement procedures

Agreement

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot, except

- PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2.

- For PUSCH repetition type B, down-select from the following options.

  - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

  - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.

Agreement

Confirm the following working assumption made in RAN1#118.

Working Assumption

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Agreement

For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SP-CSI on PUSCH is SBFD symbols,

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

Agreement

Update the following agreement made in RAN1#118 meeting:

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) with Configuration 1,

- For PUSCH repetition type A with available slot counting, A-SRS with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type.

- For CG PUSCH with neither TBoMS nor PUSCH repetition type A with available slot counting,and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

Agreement

Update the following agreement made in RAN1#118bis meeting in cyan:

Agreement

For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by:

RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1

For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by:

RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1

Where

- RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP.

- NUL SBsize is the number of UL usable PRBs.

- RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols.

- RB offset is the frequency hopping offset for PUSCH in SBFD symbols

- Note: Definition of nsμ is unchanged from existing specifications

Decide in RAN1#119, one of the following options:

- Alt1: Remove -RBUL SB start in the above equations

- Alt2: Remove square brackets from [-RBUL SB start] in the above equations

Agreement

For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination.

- Option 1:

  - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

  - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

Agreement

Red text is working assumption.

For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1.

Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

- SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols.

  - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped.

  - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission.

    - FFS impact on availableSlotOffsetList

  - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols.

  - FFS: whether there is impact on frequency hopping counter

- SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols.

  - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped.

  - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission.

    - FFS impact on availableSlotOffsetList

  - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols.

  - FFS: whether there is impact on frequency hopping counter

- The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a  SRS-ResourceSet for non-SBFD symbols for the same usage.

- For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy.

- For SRS-ResourceSets with usage set to 'Codebook',

  - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets

  - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2'

- Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant

  - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant.

  - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant.

  - Second SRS resource indicator (SRI) field is absent from DCI

    - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI.

      - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission

    - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI.

      - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource  identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission

    - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI.

      - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols.

    - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set.

  - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant.

    - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission.

      - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource  identified by the SRI in the SRS-ResourceSet corresponding to  the same symbol type as the valid symbol type of PUSCH transmission.

    - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator  is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator   is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols.

      - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in  the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in  the SRS-ResourceSet corresponding to non-SBFD symbols.

  - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS.

- Applicable to usage set to 'codebook' or 'nonCodebook'

- FFS usage set to  'beamManagement'

- Not applicable to usage set to 'antennaSwitching’

Conclusion

There is no RAN1 consensus to support mTRP scenario for SBFD in Rel-19

Agreement

The maximum number of power control loops is the same as legacy (i.e. no more than 2) for SBFD operation in Rel-19.

Agreement

For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols

- Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only,

- For type 2 CG PUSCH, SPS PDSCH, down-select from the following options:

  - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

  - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

Agreement

At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols,

- Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

  - If RBoffsetSBFD is not configured, it is zero

Agreement

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework,

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE,

- Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability.

- SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis.

  - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP.

  - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP.

    - For SRS, only Configuration 1 is applicable.

Agreement

It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI.

- It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

<Unrelated part omitted>

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

<Unrelated part omitted>

SBFD random access operation

Agreement

Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

Agreement

For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs.

- FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

Agreement

For RACH configuration Option 2, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs.

- The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs.

Agreement

Confirm the following working assumption

Working Assumption

For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs.

- the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Agreement

It is up to RAN2 to determine the detailed specified/configured conditions/prioritizations for RO selection for initial PRACH transmission attempt in one random access procedure.

Agreement

For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

Agreement

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options:

- Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2

  - FFS: details of the subframe/slot offset

- Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers.

  - Details are FFS.

- Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols.

  - Details are FFS.

- Alt-4: no enhancement

Agreement

For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs.

Agreement

For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission,

- the number of NUL,hop hopping bits is based on the number of PRBs in initial UL BWP as in legacy specification.

- the frequency offset for the 2nd hop is reserved for the case when the value of NUL,hop= 11 as in legacy specification.

Agreement

For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition

Agreement

For determining PUCCH resource sets in SBFD symbols before dedicated PUCCH resource configuration, reuse the Table 9.2.1-1 in TS 38.213, with the following modifications:

- use the size of UL usable PRBs to replace the size of initial UL BWP to determine the PRB offset in row index 15.

Agreement

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, Alt 2-1 is supported.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- ROs that are outside of UL usable PRBs are invalid

Agreement

For SBFD aware UEs, update the following equations to determine the lowest PRB indices of the PUCCH transmission in the first hop and second hop in SBFD symbols before dedicated PUCCH resource configuration:

- if rPUCCH/8=0

  - RBUL SB start+RBBWPoffset∙NRB+rPUCCH/NCS∙NRB for the first hop

  - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+rPUCCH/NCS) ∙NRB for the second hop

- if rPUCCH/8=1

  - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+(rPUCCH-8)/NCS) ∙NRB for the first hop

  - RBUL SB start+RBBWPoffset∙NRB+(rPUCCH-8)/NCS∙NRB for the second hop

Where

- RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP.

- NUL SBsize is the number of UL usable PRBs.

- RBBWPoffset is the RB offset with reference respective to the lowest PRB of UL usable PRBs.

Note: Definitions of NRB, rPUCCH and NCS are unchanged from existing specifications.

Agreement

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives:

- Alt-1: do not optimize this case

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

Agreement

For Msg3 PUSCH repetition Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission occasion.

- UE does not expect the first transmission occasion includes different symbol types (i.e., SBFD symbols and non-SBFD symbols)

- FFS: Available slot counting on SBFD symbols

Agreement

Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters.

preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR

CLI handling

Agreement

Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement

- Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis)

- FFS: Other configurations

Define SRS-RSRP-MeasurementResource with the following parameters:

- Legacy SRS Resource IE

- FFS: Other parameters

Agreement

Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement.

- Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis)

Define CLI-RSSI-MeasurementResource with the following parameters:

- CLI-RSSI measurement resource ID

- Starting PRB index

- Number of PRBs

- Starting symbol of the CLI-RSSI resource within a slot

- Number of symbols of the CLI-RSSI resource within a slot

- Periodicity and slot offset for the CLI-RSSI resource

- FFS: Other parameters

Agreement

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

Agreement

For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Agreement

If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Agreement

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

Agreement

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter

- For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD.

- If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

Agreement

For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported

- Method #3: UE measures RSSI within UL subband

- A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Conclusion

There is no RAN1 consensus on the following:

- Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

Agreement

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

Agreement

Send an LS to RAN4 with the following contents:

On UL resource muting, RAN1 has made the following agreements

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Note that RAN1 is still discussing the boosting in symbols containing PTRS. RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR, if any.

Final LS is in R1-2410889.

Agreement

Send reply LS to RAN4 with following information:

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios.

For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations.

However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Final LS is in R1-2410890.

Conclusion

For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

Conclusion

There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19

Agreement

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported

- The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4}

- The M CLI measurement resource ID(s) are reported

- The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported

- When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference

- FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

- FFS: whether/how to report differential value in case the largest measurement is out of range

Agreement

For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions

For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

#### 2.1.2	Remaining Open issues

SBFD TX/RX/measurement procedures

  - Remaining details on semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode

  - Remaining details on UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE

  - Remaining details on transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

  - Remaining details on enhancement on resource allocation in frequency domain in SBFD symbols, including

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

  - Remaining details on enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

  - Remaining details on configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

  - Remaining details on collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

SBFD random access operation

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode

CLI handling

  - Remaining details on UL resource muting for PUSCH, including

      - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

      - PUSCH resource mapping, i.e., rate-matching around the muted REs

      - UCI resource determination in symbols with muted REs

  - Remaining details on L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including

      - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

      - At least aperiodic reporting

      - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - UCI bits generation

      - Priority rules for multiple CSI reporting

## 2.2	RAN2

#### 2.2.1	Agreements

In RAN2#127bis, the following agreements were made.

Random access in SBFD

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

Other aspects

For UL resource muting for PUSCH, the configuration of time and frequency location for UL resource muting should be introduced based on R1 agreement.

For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement.

For L1 based UE-to-UE CLI reporting mechanism, the configuration of report quantities should be introduced based on R1 agreement.

In RAN2#128, the following agreements were made.

Random access in SBFD

CFRA

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

CBRA

FFS on the following options

Option 1

Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO.

If there is no such indication from the NW, FFS on the following mechanism

- UE select legacy RO or SBFD RO based on SSB RSRP, or

- UE select the legacy RO, or

- UE select the SBFD RO, or

- Other metrics than SSB RSRP.

Option 2

UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

RACH configuration

Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

Other aspects

Prioritization of SBFD cells / frequencies during cell reselection is not considered.

RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

#### 2.2.2	Remaining Open issues

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode

  - Other remaining details that have RAN2 impact such as SBFD time/frequency subbands configuration

## 2.3	RAN3

#### 2.3.1	Agreements

In RAN3#125bis, the following agreements were made.

[Agreement]

RAN3 agrees to Exchange the following information over Xn interface and F1 interface:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

- Measurement resource configuration

- Strongest DL beam information (FFS on the details)

- CLI-mitigation request (FFS on the details)

In RAN3#126, the following agreements were made.

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

#### Remaining Open issues

  - Remaining details on information exchange among gNBs, including

      - Semi-static cell-specific SBFD time and frequency location configuration

      - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

      - Strongest DL beam information

      - CLI-mitigation request

## 2.4	RAN4

#### 2.4.1	Agreements

In RAN4#112bis, the following agreements were made.

The agreements were captured in following t-docs:

| Tdoc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2416584 | Way Forward for [112bis][306] NR_duplex_evo_General | Samsung | Approved |
| R4-2416585 | Way Forward for [112bis][307] NR_duplex_evo_BSRF | Huawei | Approved |
| R4-2416919 | LS on the scenario of UE to UE CLI measurement report in SBFD | Apple | Approved |
| R4-2416858 | WF on RRM requirements for NR_duplex_evo | Huawei | Approved |
| R4-2416910 | Simulation assumptions for L1 SRS-RSRP measurement | Ericsson | Approved |

General aspects (including RAN4 aspects for SBFD system parameters)

Sub-topic 1-1: SBFD frequency domain issues

Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz)

Agreement:

- Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

  - The supported channel bandwidth(s) is declared by gNB vendors

  - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

  - The supported channel bandwidth shall be no less than X MHz

    - X = 40 as the default value

      - For band n39, X = 20MHz is used instead.

Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD

Agreement:

- To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD.

  - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD

  - FFS the band for NR-U shall be supported

Issue 1-1-4: Transmission bandwidth configuration for SBFD

Way Forward:

- Illustrative figures from Ericsson will be used as the baseline for further study:

  - FFS the restriction of SCS for DL and UL subbands

  - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL

  - FFS other potential issues.

Issue 1-1-5: DL/UL subband configurations support for SBFD

Way Forward:

- RAN4 to further study Option 2b by considering the below additional restrictions such as:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth

  - FFS the number of DL subband sizes to be declared and tested

    - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed.

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

    - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested

      - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS.

  - Restriction on UL subband size based on the specific channel bandwidth

    - FFS to specify a range of ratio for UL subband size over channel bandwidth.

    - FFS the restriction is different for DU and DUD

    - FFS the restriction is applied for FR2 case.

Sub-topic 1-2: Other general issues for SBFD requirements

Issue 1-2-1: BS RF Specification structure for SBFD requirements

Way Forward:

- FFS options of how to introduce BS RF new requirements for SBFD-capable BS:

  - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

  - Option-3: A new specification for SBFD BS RF requirements

Potentially new requirements for SBFD

Sub-topic 2-1: Requirements for Transient period between SBFD and non-SBFD

Issue 2-1-1: Transient period requirement applicability on frequency/antenna

Way Forward:

- FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498.

Issue 2-1-2: Location of transient period between SBFD and non-SBFD

Way Forward:

- FFS location of transient period between SBFD and non-SBFD:

  - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not.

  - Option 2: Place the transient period outside of both SBFD and non-SBFD slots.

  - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

Sub-topic 2-2: In-channel adjacent subband leakage ratio requirements

Issue 2-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

Way Forward:

- FFS based on the way forward approved in RAN4#112.

Sub-topic 2-3: In-channel adjacent subband selectivity/blocking requirements

Issue 2-3-1/2-3-2: Necessity of in-channel adjacent subband selectivity/blocking requirements

Way Forward:

- FFS based on the way forward approved in RAN4#112.

  - Consider the possibility of specifying RX requirement to cover both in-channel adjacent subband selectivity/blocking requirements

Modification of existing Tx requirements for FR1 and FR2-1

1.1. ACLR and OBUE

WF:

- FFS on the followings:

  - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly?

    - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence.

    - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirement.

  - How to define the additional co-existence requirements:

    - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level.

      - [1dB DESENS in the victim SBFD receiver]

      - [typical CL values based on typical SBFD-capable gNB implementation]

      - [frequency offset for the additional OBUE requirement is declared]

    - Option 2: by improving ACLR1 and ACLR2 requirements

    - Option 3: others

1.2. Joint measurement for normal DL symbols/slots and SBFD symbols/slots

Agreement:

- Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

- Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

1.3. OTA sensitivity degradation

WF:

- FFS on the followings:

  - Option 1: 1.0 dB degradation

  - Option 2: 0.5 dB degradation

1.4. Dynamic range

WF:

- FFS on the followings:

  - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work.

  - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

  - Option 3: Dynamic range of FR1/FR2-1 indoor scenario can reuse the existing requirement.

1.5. ACS requirement

Agreement:

- The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

WF:

- FFS on the followings:

  - If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

  - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

1.6. In-band blocking

Agreement:

- For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements.

WF:

- FFS on the followings:

  - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined?

    - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel.

    - Option 2: separate requirements are defined,

      - One set of requirements for coexisting with legacy TDD system in adjacent channel

      - One set of requirements for coexisting with new SBFD system in adjacent channel

  - For FR1 WA BS co-existence study, FFS grid-shift values should be considered.

    - Option 1: 10%

    - Option 2: 20%

    - Option 3: 50%

    - Option 4: 100%

  - In-band blocking requirement for FR1 MR SBFD BS

  - In-band blocking requirement for FR2-1 WA and MR SBFD BS

  - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

1.7. Necessity of New RX intermodulation requirement with 1 interfering signal

WF:

- FFS on the followings:

  - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: RAN4 to study further RX intermodulation scenario with 1 interfering signal.

1.8. Necessity of New RX intermodulation requirement with 2 interfering signals

WF:

- FFS on the following:

  - Option 1:

    - For the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

    - For receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver.

  - Option 2: New IMD requirement with 2 interfering signal scenario is not needed.

  - Option 3: RAN4 to study further

RRM impacts of UE-to-UE CLI handling

Issue 1-1-1: Measurement quantities

- Agreements (from online session)

  - RAN4 to define requirements for L1-SRS-RSRP and L1-CLI-RSSI.

  - Requirements for other measurement quantities can be considered based on further RAN1 agreement.

Issue 1-1-6: Side condition

- Agreements (from online session)

  - Send LS to RAN1: in addition to intra-cell scenario, whether inter-cell scenario is considered in this WI. Further discuss the details in the LS.

  - For SRS configuration, re-use Rel-16 side condition as baseline.

Issue 1-1-14: Measurement timing for L1-SRS-RSRP

- Agreements (from online session)

  - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

  - This can be revisited if it proves to be conflicting with further RAN1 agreements.

RRM impacts of SBFD operation

Issue 1-2-1: Requirements for SSB based measurement

- Agreements (from online session)

  - Down-select one of the two options:

    - Option 1:

      - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

      - This can be revisited if it proves to be conflicting with further RAN1 agreements.

    - Option 2:

      - For inter-frequency measurement without gap, UL transmission is prioritized.

      - Follow option 1 in other cases.

Issue 1-2-2: Requirements for CSI-RS based measurement

- Agreements (from online session)

  - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations.

    - CSI-RS colliding with dynamically scheduled UL transmission

    - CSI-RS puncturing in frequency domain in SBFD symbols

    - CSI-RS distributed over SBFD and non-SBFD symbols

  - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI.

  - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

Issue 1-2-9: Limits on the maximum number of DL/UL switching

- Agreements (from online session)

  - RAN4 not to discuss limits on the maximum number of DL/UL switching unless requested by other WGs.

In RAN4#113, the following agreements were made

The agreements were captured in following t-docs:

| Tdoc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2419907 | Way Forward for [113][306] NR_duplex_evo_General | Samsung | Approved |
| R4-2419908 | Way Forward for [113][307] NR_duplex_evo_BSRF | Huawei | Approved |
| R4-2420097 | WF on RRM requirements for NR_duplex_evo | Huawei | Approved |
| R4-2420165 | LS on CSI-RS measurement with SBFD operation | MTK | Approved |

General aspects (including RAN4 aspects for SBFD system parameters)

Sub-topic 1-1: SBFD applicability in NR band list

Issue 1-1-1: SBFD operation support in NR-U bands

Agreement:

- SBFD operation shall not be applied in NR-U bands, i.e., n46, n96 and n102.

Issue 1-1-3: Specification implementation of operating bands support for SBFD

Agreement:

- The following TP is adopted for clause 5.2 in TS 38.104

  - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2.

- FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

Issue 1-1-4: Specification implementation of BS channel bandwidth support for SBFD

Agreement:

- The following note is introduced in Table 5.3.2-1 and Table 5.3.5-1 in TS 38.104:

  - In the bands applicable to SBFD operation, the channel bandwidth for SBFD operation shall be no less than 20MHz for band n39 and no less than 40MHz otherwise.

Sub-topic 1-2: Transmission bandwidth configuration for SBFD

Issue 1-2-2: Restriction to use the same SCS for DL and UL subbands

Conclusion:

- The restriction of SCS values for UL and DL subbands:

  - It is RAN4 common understanding the same SCS value should be used for UL and DL subbands

  - The restriction if any should be given in RAN2 spec, rather than in RAN4 spec.

Issue 1-2-3: Symbols for SBFD DUD configuration

Agreement:

- Update the symbols as follows:

  - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

  - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

  - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU, UD and DUD case, expressed in resource blocks.

Issue 1-2-4: Illustrative figures for transmission bandwidth configuration for SBFD

Agreement:

- The following figures are adopted in RAN4 requirement to illustrate transmission bandwidth configuration for SBFD:

Sub-topic 1-3: DL/UL subband configurations support for SBFD

Issue 1-3-1: Clarification on declaration and flexibility

Agreement:

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested.

  - FFS on how to capture this in the specification.

  - This restriction shall at least be applied to FR1 WA BS

  - FFS other BS classes

    - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations.

      - FFS how to capture this in the specifications

Issue 1-3-2: DL/UL subband configurations support for SBFD

Agreement:

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration:

  - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD

    - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB

  - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

    - FFS different sizes for specific UL subband for DUD and DU/UD.

    - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth.

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

  - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case

    - This restriction shall be applied to FR1 WA BS only, as the starting point

      - FFS the restriction is also applied to FR1 MR BS

Issue 1-3-6: Other restriction on DL subband size

Way Forward:

- FFS the following restriction on DL subband size:

  - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size).

  - FR2 minimum CBW to support SBFD is 100MHz.

Sub-topic 1-4: Other general issues for SBFD requirements

Issue 1-4-1: BS RF Specification structure for SBFD requirements

Agreement:

- Draft CR’s endorsed against 38.104 does not imply that a new specification could not be agreed.

RF requirement impact for PUSCH with resource muting

Issue 2-1-1: RF requirement impact for PUSCH with resource muting

Way Forward:

- FFS the RF requirement impact, if any, due to UL resource muting introduced in RAN1.

Potentially new requirements for SBFD

Sub-topic 3-1: Requirements for Transient period between SBFD and non-SBFD

Issue 3-1-1: Transient period requirement applicability on frequency/antenna

Agreement:

- RAN4 shall not define an SBFD-architecture dependent requirement.

- Transient period between SBFD and non-SBFD is applicable to all four cases.

Issue 3-1-2: Location of transient period between SBFD and non-SBFD

Way Forward:

- FFS whether the transmitter transient period shall be located within the SBFD symbol or not:

  - FFS it is an implementation issue and no RAN4 standard impact

Sub-topic 3-2: In-channel adjacent subband leakage ratio requirements

Issue 3-2-1: Necessity of in-channel adjacent subband leakage ratio requirements

Agreement:

- New requirement for in-channel adjacent subband leakage ratio should be defined

  - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

Sub-topic 3-3: In-channel adjacent subband selectivity/blocking requirements

Issue 3-3-2: Necessity of in-channel adjacent subband blocking requirement

Agreement:

- New requirement for in-channel adjacent subband blocking should be defined

  - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation.

  - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement.

  - The requirement level shall be driven by feasibility consideration.

Issue 3-3-1: Necessity of in-channel adjacent subband selectivity requirements

Agreement:

- No necessity to introduce the new requirement for in-channel adjacent subband selectivity.

Sub-topic 3-4: Inter-sector and inter-site interference

Issue 3-4-1: Necessity of new requirement for inter-sector and inter-site interference

Agreement:

- No necessity to introduce the new requirement for inter-sector interference.

Modification of existing Tx requirements for FR1 and FR2-1

Issue 1: Total dynamic range

Agreement:

- The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

Issue 2: Unwanted emissions

Agreement:

- FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

Issue 3: Joint measurement for normal DL symbols/slots and SBFD symbols/slots

Agreement:

- FFS on the Proposal 3 from R4-2419221.

Issue 4: BS type for SBFD

Agreement:

- For SBFD, BS type 1-C is precluded.

Issue 5: Text Proposals for implementing SBFD RF requirements

Agreement:

- The BS specification structure conclusion captured in General thread shall be applied here.

- FFS on how to implement corresponding SBFD RF requirements by using the following proposals as baseline.

- Text Proposal in R4-2418099 on BS output power requirement (taking the new clause 6.2B, titled as “Base station output power for SBFD” as example): =================== Start of Text Proposal =================== 6.2B Base station output power for SBFD 6.2.1B General The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1. 6.2.2B Minimum requirement for BS type 1-C The same requirement as clause 6.2.2 is applicable to BS output power requirement for SBFD operation. 6.2.3B Minimum requirement for BS type 1-H The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation. =================== End of Text Proposal =================== - Text Proposal in R4-2418099 on BS output power dynamics requirement (taking the new clause 6.3.3B, titled as “Total power dynamic range for SBFD” as example): =================== Start of Text Proposal =================== 6.3.3B Total power dynamic range for SBFD 6.3.3.1B General The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. NOTE 1: The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB. NOTE 2: The requirement does not apply to operation with shared spectrum channel access. 6.3.3.2 Minimum requirement for BS type 1-C and BS type 1-H The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured. =================== End of Text Proposal =================== - Text Proposal in R4-2418099 on transmitter intermodulation requirement (taking the new clause 6.7B, titled as “Transmitter intermodulation for SBFD” as example): =================== Start of Text Proposal =================== 6.7B Transmitter intermodulation for SBFD The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied. =================== End of Text Proposal ===================

Issue 6: OTA sensitivity degradation

Agreement:

- 0.7 dB degradation on top of existing minimum requirements for OTA sensitivity.

Issue 7: Dynamic range requirement

Agreement:

- FFS on the followings:

  - The dynamic range requirements of FR1 can reuse the existing IoT level.

  - The dynamic range requirements of FR2-1 can reuse existing IoT level.

Issue 8: ACS requirement

Agreement:

- FFS on the following:

  - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

Issue 9: In-band blocking

Agreement:

- For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

- For FR1 WA BS, FFS following options.

  - Option 1: a unified blocking requirement to consider both scenarios

  - Option 2: separate requirements are defined

    - One set of requirements for coexisting with legacy TDD system in adjacent channel

    - One set of requirements for coexisting with new SBFD system in adjacent channel

  - Option 3: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting).

- For FR1 WA BS co-existence study, FFS what grid-shift value(s) should be considered.

  - Option 1: 10%

  - Option 2: 20%

  - Option 3: 50%

  - Option 4: 100%

  - Option 5: 5%

  - Option 6: 20% ~ 50%

- For FR2-1 WA SBFD BS, FFS following two options.

  - Option 1: The current FR2-1 WA BS in-band blocking requirements can be re-used

  - Option 2: Define in-band blocking interferer signal power to -23 dBm

Issue 10: Necessity of New RX intermodulation requirement with 1 interfering signal

Agreement:

- FFS on the following options:

  - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

Issue 11: Impact on existing RX intermodulation requirement with 2 interfering signals

Agreement:

- FFS on the following options:

  - Option 1:

    - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

    - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver.

  - Option 2: new IMD requirement with 2 interfering signal scenario is not needed.

  - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined.

  - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

RRM impacts of UE-to-UE CLI handling

Issue 1-1-2: Measurement methods

- Agreements (from Thu AH session)

  - RAN4 shall specify L1 CLI measurement requirements at least for below methods:

    - Method#1: UE measures RSSI within DL subband

    - Method#2: UE measures RSRP of aggressor UE within UL subband

  - Additional methods can be considered based on further RAN1 agreements.

  - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

Issue 1-1-3: Rx beam

- Agreements (from Thu AH session)

  - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec.

  - Rx beam sweeping is not needed for L1 CLI measurement.

Issue 1-1-6: Measurement period

- Agreements (from Wed AH session)

  - For L1-SRS-RSRP, further discuss the following options based on simulation results:

    - Option 1: requirement specified with [1] sample measurement assumption

    - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

  - For L1-CLI-RSSI,

    - Confirm measurement period requirements are defined based on 1 sample, and

    - Clarify in the requirements that the measurement is based on DL timing.

Issue 1-1-9: Scheduling and measurement restriction

- Agreements (from Thu AH session)

  - For scheduling restriction,

    - For L1-SRS-RSRP,

      - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption

      - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption

    - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing.

  - For measurement restriction,

    - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

      - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

RRM impacts of SBFD operation

Issue 1-2-1: Requirements for SSB based measurement

- Agreements (from online session)

  - For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

  - This can be revisited if it proves to be conflicting with further RAN1 agreements.

Issue 1-2-2: Requirements for CSI-RS based L1 measurement

- Agreements (from online session)

  - For CSI-RS colliding with UL transmission

    - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission.

    - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed.

  - For CSI-RS puncturing in frequency domain in SBFD symbols,

    - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

    - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold.

      - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs.

      - FFS for the DUD configuration.

    - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions.

    - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold.

    - FFS the measurement behavior for DUD configuration.

    - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement.

  - For CSI-RS distributed over SBFD and non-SBFD symbols,

    - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS.

Issue 1-2-3: Requirements for CSI-RS based L3 measurement

- Agreements (from online session)

  - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

Issue 1-2-4: Requirements for scheduling restriction

- Agreements (from Wed AH session)

  - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction.

  - Further discuss the impact to inter-frequency SSB measurement without gap case.

#### 2.4.2	Remaining Open issues

  - Remaining details on RAN4 aspects for SBFD system parameters

  - Remaining details on BS RF requirements for SBFD operation at gNB:

    - Potentially new requirements for SBFD

    - Modification of existing Tx/RX requirements for FR1 and FR2-1

  - Remaining details on applicable RRM core requirements for UE-to-UE CLI handling mechanisms

  - Remaining details on other RRM core requirements for SBFD operation, if identified

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#118bis

R1-2407624	Discussion for SBFD TX/RX/ measurement procedures	New H3C Technologies Co., Ltd.

R1-2407625	Discussion for SBFD random access operation	New H3C Technologies Co., Ltd.

R1-2407626	Discussion on CLI handling	New H3C Technologies Co., Ltd.

R1-2407631	Discussion on SBFD TX/RX/measurement procedures	TCL

R1-2407632	Discussion on SBFD random access operation	TCL

R1-2407675	On subband full duplex design	Huawei, HiSilicon

R1-2407676	On subband full duplex random access operation	Huawei, HiSilicon

R1-2407677	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2407702	Discussion on SBFD TX/RX/measurement procedures	Spreadtrum Communications

R1-2407703	Discussion on SBFD random access operation	Spreadtrum Communications

R1-2407704	Discussion on CLI handling	Spreadtrum Communications

R1-2407730	Discussion on SBFD TX/RX/measurement procedures	China Telecom

R1-2407731	Discussion on SBFD random access operation	China Telecom

R1-2407752	Discussion for SBFD TX/RX/ measurement procedures	Tejas Network Limited

R1-2407753	Discussion on SBFD Random Access Operation	Tejas Network Limited

R1-2407754	Discussion on gNB-gNB CLI handling	Tejas Network Limited

R1-2407807	Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE Corporation, Sanechips

R1-2407808	Discussion on SBFD random access operation	ZTE Corporation, Sanechips

R1-2407809	Discussion on CLI handling for Rel-19 duplex operation	ZTE Corporation, Sanechips

R1-2407857	Discussion on Rel-19 SBFD operation	vivo

R1-2407858	Discussion on random access for Rel-19 SBFD	vivo

R1-2407859	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2407901	Discussion on SBFD TX/RX/measurement procedures	CMCC

R1-2407902	Discussion on SBFD random access operation	CMCC

R1-2407903	Discussion on CLI handling	CMCC

R1-2407926	Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc

R1-2407927	Discussion on SBFD Random Access Operation	MediaTek Inc

R1-2407928	Discussion on CLI Handling in SBFD System	MediaTek Inc

R1-2407965	Discussion on reception and transmission procedure for SBFD operation	Xiaomi

R1-2407966	Discussion on SBFD random access operation	Xiaomi

R1-2407967	Discussion on CLI handling for SBFD operation	Xiaomi

R1-2408043	Discussion on SBFD TX/RX/measurement procedures	CATT

R1-2408044	Discussion on SBFD random access operation	CATT

R1-2408045	Discussion on CLI handling for NR duplex evolution	CATT

R1-2408086	Discussion on SBFD TX/RX/measurement procedures	LG Electronics

R1-2408087	Discussion on SBFD random access operation	LG Electronics

R1-2408088	Discussion on CLI handling	LG Electronics

R1-2408112	Discussion on SBFD Tx/Rx/measurement procedures	Fujitsu

R1-2408115	Discussion on SBFD random access operation	Korea Testing Laboratory

R1-2408119	Discussion on SBFD operation	Transsion Holdings

R1-2408120	Discussion on SBFD random access operation	Transsion Holdings

R1-2408142	Discussion on SBFD Tx/Rx/measurement procedures	OPPO

R1-2408143	Discussion on SBFD random access operation	OPPO

R1-2408144	CLI handling in NR duplex operation	OPPO

R1-2408215	Discussion on random access for SBFD	NEC

R1-2408216	CLI handling for NR duplex operations	NEC

R1-2408232	Discussion on SBFD TX-RX-measurement procedures	HONOR

R1-2408276	On TX RX and measurement procedures for SBFD operation	InterDigital, Inc.

R1-2408277	Discussion on SBFD random access operation	InterDigital, Inc.

R1-2408278	Discussion on CLI handling for SBFD operation	InterDigital, Inc.

R1-2408325	SBFD random access operation	Lenovo

R1-2408328	SBFD Tx/Rx/measurement aspects	Sharp

R1-2408329	SBFD random access aspects	Sharp

R1-2408373	Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

R1-2408383	Discussion on SBFD TX/RX/measurement procedures	Ruijie Networks Co. Ltd

R1-2408384	Discussion on SBFD random access operation	Ruijie Networks Co. Ltd

R1-2408407	SBFD Tx/Rx/Measurement Procedures	Sony

R1-2408408	SBFD RACH Operations	Sony

R1-2408409	CLI handling for SBFD	Sony

R1-2408461	Discussion on SBFD TX/RX/measurement procedures	Apple

R1-2408462	Views on SBFD random access operation	Apple

R1-2408463	Discussion on CLI handling	Apple

R1-2408498	Discussion on CLI handling	Panasonic

R1-2408504	Discussion on SBFD random access operation	Fujitsu

R1-2408525	SBFD Tx/Rx/measurement procedures	Ericsson

R1-2408526	SBFD random access operation	Ericsson

R1-2408527	SBFD CLI Handling	Ericsson

R1-2408530	Discussion on SBFD TX/RX/measurement procedures	Panasonic

R1-2408531	Discussion on SBFD random access operation	Panasonic

R1-2408551	Views on SBFD TX/RX Measurement Procedures	Lenovo

R1-2408565	Discussion on SBFD TX/RX/measurement procedures	ETRI

R1-2408566	SBFD random access operation	ETRI

R1-2408567	Discussion on CLI handling for SBFD operation	ETRI

R1-2408587	On SBFD random access operation	Google Ireland Limited

R1-2408588	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Ireland Limited

R1-2408593	On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

R1-2408594	SBFD random access operation	Nokia, Nokia Shanghai Bell

R1-2408595	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2408600	Discussion on SBFD Tx/Rx/measurement procedures	Fraunhofer HHI, Fraunhofer IIS

R1-2408642	SBFD operation and procedures	Samsung

R1-2408643	Random access on SBFD resources	Samsung

R1-2408644	Cross-link interference handling for SBFD	Samsung

R1-2408752	Discussion on RO validation for SBFD aware UE	ASUSTeK

R1-2408754	Discussion on SBFD Random Access operation	KT Corp.

R1-2408782	Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

R1-2408783	Discussion on SBFD random access operation	NTT DOCOMO, INC.

R1-2408784	Discussion on CLI handling	NTT DOCOMO, INC.

R1-2408825	Discussion on SBFD TX/RX/measurement procedures	Kookmin University

R1-2408826	Discussion on SBFD random access operation	Kookmin University

R1-2408828	Discussion on SBFD TX/RX/measurement procedures	ITRI

R1-2408829	Discussion on SBFD random access operation for SBFD aware UEs	ITRI

R1-2408846	SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

R1-2408847	SBFD Random Access Operation	Qualcomm Incorporated

R1-2408848	Enhancements for CLI handling	Qualcomm Incorporated

R1-2408879	Discussion on SBFD TX/RX/measurement procedures	Google Ireland Limited

R1-2408884	Cross-link interference management for duplexing evolution	Lenovo

R1-2408898	Discussion on SBFD TX/RX/measurement procedures	WILUS Inc.

R1-2408899	Discussion on SBFD random access operation	WILUS Inc.

R1-2408907	PRG allocation for SBFD	ASUSTeK

R1-2408927	Discussion on SBFD TX/RX/ measurement procedures	CEWiT

R1-2408928	Discussion on SBFD random access operation	CEWiT

R1-2408929	Discussion on CLI handling	CEWiT

R1-2409046	Summary#1 on SBFD random access operation	Moderator (CMCC)

R1-2409047	Summary#2 on SBFD random access operation	Moderator (CMCC)

R1-2409086	Summary #1 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2409087	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2409088	Summary #3 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2409175	Summary #1 of CLI handling	Moderator (Huawei)

R1-2409176	Summary #2 of CLI handling	Moderator (Huawei)

R1-2409177	Summary #3 of CLI handling	Moderator (Huawei)

RAN1#119

R1-2409374	Discussion for SBFD TX/RX/ measurement procedures	New H3C Technologies Co., Ltd.

R1-2409375	Discussion for SBFD random access operation	New H3C Technologies Co., Ltd.

R1-2409376	Discussion on CLI handling	New H3C Technologies Co., Ltd.

R1-2409410	On subband full duplex design	Huawei, HiSilicon

R1-2409411	On subband full duplex random access operation	Huawei, HiSilicon

R1-2409412	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2409452	Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

R1-2409453	Discussion on SBFD Random Access Operation	MediaTek Inc.

R1-2409454	Discussion on CLI Handling in SBFD System	MediaTek Inc.

R1-2409474	Discussion on SBFD TX/RX/measurement procedures	TCL

R1-2409475	Discussion on SBFD random access operation	TCL

R1-2409488	Discussion on SBFD TX/RX/measurement procedures	LG Electronics

R1-2409489	Discussion on SBFD random access operation	LG Electronics

R1-2409490	Discussion on CLI handling	LG Electronics

R1-2409508	Discussion on SBFD TX/RX/measurement procedures	CMCC

R1-2409509	Discussion on SBFD random access operation	CMCC

R1-2409510	Discussion on CLI handling	CMCC

R1-2409538	Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE Corporation, Sanechips

R1-2409539	Discussion on SBFD random access operation	ZTE Corporation, Sanechips

R1-2409540	Discussion on CLI handling for Rel-19 duplex operation	ZTE Corporation, Sanechips

R1-2409566	Discussion on SBFD TX_RX_measurement procedures	InterDigital, Inc.

R1-2409567	On SBFD random access operation	InterDigital, Inc.

R1-2409568	On CLI handling for SBFD operation	InterDigital, Inc.

R1-2409593	SBFD operation and procedures	Samsung

R1-2409594	Random access on SBFD resources	Samsung

R1-2409595	Cross-link interference handling for SBFD	Samsung

R1-2409632	Discussion on SBFD TX/RX/measurement procedures	Spreadtrum, UNISOC

R1-2409633	Discussion on SBFD random access operation	Spreadtrum, UNISOC

R1-2409634	Discussion on CLI handling	Spreadtrum, UNISOC

R1-2409677	Discussion on Rel-19 SBFD operation	vivo

R1-2409678	Discussion on random access for Rel-19 SBFD	vivo

R1-2409679	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2409763	Discussion for SBFD TX/RX/ measurement procedures	Tejas Networks Limited

R1-2409764	Discussion on SBFD Random Access Operation	Tejas Networks Limited

R1-2409765	Discussion on gNB-gNB CLI handling	Tejas Networks Limited

R1-2409796	Discussion on SBFD TX/RX/measurement procedures	Apple

R1-2409797	Views on SBFD random access operation	Apple

R1-2409798	Discussion on CLI handling	Apple

R1-2409843	Discussion on SBFD TX/RX/measurement procedures	Ruijie Networks Co. Ltd

R1-2409844	Discussion on SBFD random access operation	Ruijie Networks Co. Ltd

R1-2409873	Discussion on random access for SBFD	NEC

R1-2409874	CLI handling for NR duplex operations	NEC

R1-2409892	Discussion on reception and transmission procedure for SBFD operation	Xiaomi

R1-2409893	Discussion on SBFD random access operation	Xiaomi

R1-2409894	Discussion on CLI handling for SBFD operation	Xiaomi

R1-2409910	Discussion on SBFD random access operation	Korea Testing Laboratory

R1-2409937	Discussion on SBFD TX/RX/measurement procedures	CATT

R1-2409938	Discussion on SBFD random access operation	CATT

R1-2409939	Discussion on CLI handling for NR duplex evolution	CATT

R1-2409963	Discussion on SBFD operation	Transsion Holdings

R1-2409964	Discussion on SBFD random access operation	Transsion Holdings

R1-2409998	Discussion on SBFD TX/RX/measurement procedures	China Telecom

R1-2409999	Discussion on SBFD random access operation	China Telecom

R1-2410045	Discussion on SBFD TX/RX/measurement procedures	Panasonic

R1-2410046	Discussion on SBFD random access operation	Panasonic

R1-2410047	Discussion on CLI handling	Panasonic

R1-2410057	Discussion on SBFD Tx/Rx/measurement procedures	Fujitsu

R1-2410058	Discussion on SBFD random access operation	Fujitsu

R1-2410086	Discussion on SBFD Tx/Rx/measurement procedures	OPPO

R1-2410087	Discussion on SBFD random access operation	OPPO

R1-2410088	CLI handling in NR duplex operation	OPPO

R1-2410137	On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

R1-2410138	SBFD random access operation	Nokia, Nokia Shanghai Bell

R1-2410139	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2410140	SBFD TX/RX/measurement procedures	Ericsson

R1-2410141	SBFD Random access operation	Ericsson

R1-2410142	CLI handling	Ericsson

R1-2410172	Discussion on SBFD random access operation	Hyundai Motor Company

R1-2410177	Discussion on SBFD TX/RX/measurement procedures	HONOR

R1-2410191	Views on SBFD TX/RX Measurement Procedures	Lenovo

R1-2410206	Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

R1-2410222	SBFD Procedures	Sony

R1-2410223	SBFD RACH Operations	Sony

R1-2410224	SBFD CLI Handling	Sony

R1-2410241	Discussion on SBFD Tx/Rx/measurement procedures	Fraunhofer HHI, Fraunhofer IIS

R1-2410251	SBFD random access operation	Lenovo

R1-2410264	Discussion on SBFD TX/RX/measurement procedures	ETRI

R1-2410265	SBFD random access operation	ETRI

R1-2410266	Discussion on CLI handling for SBFD operation	ETRI

R1-2410324	On SBFD random access operation	Google Ireland Limited

R1-2410325	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Ireland Limited

R1-2410345	Discussion on SBFD Random Access operation	KT Corp.

R1-2410385	Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

R1-2410386	Discussion on SBFD random access operation	NTT DOCOMO, INC.

R1-2410387	Discussion on CLI handling	NTT DOCOMO, INC.

R1-2410410	SBFD Tx/Rx/measurement aspects	Sharp

R1-2410412	SBFD random access aspects	Sharp

R1-2410428	Discussion on SBFD TX/RX/measurement procedures	ITRI

R1-2410429	Discussion on SBFD random access operation for SBFD aware UEs	ITRI

R1-2410433	Views on uplink resource muting	SHARP Corporation

R1-2410474	SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

R1-2410475	SBFD Random Access Operation	Qualcomm Incorporated

R1-2410476	Enhancements for CLI handling	Qualcomm Incorporated

R1-2410544	Discussion on SBFD random access operation	ASUSTEK COMPUTER (SHANGHAI)

R1-2410545	Discussion on SBFD TX/RX/measurement procedures	Google Ireland Limited

R1-2410548	Partial PRG handling for SBFD	ASUSTeK

R1-2410558	Discussion on SBFD TX/RX/measurement procedures	WILUS Inc.

R1-2410559	Discussion on SBFD random access operation	WILUS Inc.

R1-2410574	Discussion on SBFD TX/RX/ measurement procedures	CEWiT

R1-2410575	Discussion on SBFD random access operation	CEWiT

R1-2410576	Discussion on CLI handling	CEWiT

R1-2410674	Summary#1 on SBFD random access operation	Moderator (CMCC)

R1-2410675	Summary#2 on SBFD random access operation	Moderator (CMCC)

R1-2410676	Summary#3 on SBFD random access operation	Moderator (CMCC)

R1-2410692	Summary #1 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2410693	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2410694	Summary #3 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2410788	Summary #1 of CLI handling	Moderator (Huawei)

R1-2410789	Summary #2 of CLI handling	Moderator (Huawei)

R1-2410790	Summary #3 of CLI handling	Moderator (Huawei)

R1-2410889	LS on impact on transmit signal quality/MPR requirement	RAN1, Huawei

R1-2410890	Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	RAN1, Huawei

RAN2#127bis

R2-2407917	LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation (R1-2407533; contact: Ericsson)	RAN1

R2-2407950	Random Access in SBFD symbols	CATT

R2-2407951	Discussion on other aspects for SBFD	CATT

R2-2407955	Discussion on RACH in SBFD	Xiaomi

R2-2408035	Other aspects of SBFD	Xiaomi

R2-2408067	Discussion on random access in SBFD	CMCC

R2-2408089	Discussion on SBFD related issues	CMCC

R2-2408103	Discussion on random access procedure in SBFD	vivo

R2-2408104	Discussion on other aspects in SBFD	vivo

R2-2408219	Discussion on random access procedure in SBFD	ZTE Corporation

R2-2408220	Discussion on CLI measurement in SBFD	ZTE Corporation

R2-2408364	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon

R2-2408365	Discussion on the SBFD configuration and CLI measurement	Huawei, HiSilicon

R2-2408420	Random Access Procedures for SBFD	Sharp

R2-2408508	SBFD RACH configuration for initial random access	Charter Communications, Inc

R2-2408550	Random Access for SBFD Operation	NEC

R2-2408594	Framework to support RACH in SBFD	Apple

R2-2408647	Random Access Operation of SBFD	Nokia Corporation

R2-2408690	Random access in Sub-Band Full Duplex	Google Ireland Limited

R2-2408717	Random access for SBFD Operation	Sony

R2-2408799	Views on random access for SBFD	Qualcomm Incorporated

R2-2408800	Other aspects of SBFD	Qualcomm Incorporated

R2-2408855	SBFD RA aspects	Ericsson

R2-2408856	Non-RA aspects for subband full duplex (SBFD) operation	Ericsson

R2-2409008	Random access in SBFD	Samsung

R2-2409089	Other aspects of SBFD	Nokia

R2-2409098	Support of Cross Link Interference in SBFD	Samsung

R2-2409152	Discussion on Random Access procedure for SBFD	LG Electronics Inc.

RAN2#128

R2-2409571	Discussion on random access procedure in SBFD	ZTE Corporation

R2-2409572	Discussion on CLI measurement in SBFD	ZTE Corporation

R2-2409584	Discussion on RACH in SBFD	Xiaomi

R2-2409625	Consideration on Random Access in SBFD symbols	CATT

R2-2409626	Discussion on other aspects for SBFD	CATT

R2-2409638	Other aspects of SBFD	Xiaomi

R2-2409680	SBFD RACH configuration for initial random access	Charter Communications, Inc

R2-2409681	SBFD-aware UE capability indication	Charter Communications, Inc

R2-2409745	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon

R2-2409746	Other impacts by the evolution of duplex operation	Huawei, HiSilicon

R2-2409793	Selection of SBFD Cell	NEC

R2-2409794	Random Access for SBFD Operation	NEC

R2-2409913	Discussion on Random Access in SBFD	LG Electronics Inc.

R2-2409974	Detailed design for RACH in SBFD	Apple

R2-2409995	SBFD RA aspects	Ericsson

R2-2409996	Non-RA aspects for subband full duplex (SBFD) operation	Ericsson

R2-2410088	RA Aspects for SBFD	Sharp

R2-2410241	Random Access Operation of SBFD	Nokia Corporation

R2-2410258	Other aspects of SBFD	Nokia

R2-2410336	Discussion on random access in SBFD	CMCC

R2-2410385	Random access for SBFD Operation	Sony

R2-2410478	Views on random access for SBFD	Qualcomm Incorporated

R2-2410479	Other aspects of SBFD	Qualcomm Incorporated

R2-2410574	Random Access in Sub-Band Full Duplex	Google Ireland Limited

R2-2410609	Random access in SBFD	Samsung

R2-2410623	Support of Cross Link Interference in SBFD	Samsung

R2-2410791	Discussion on random access procedure in SBFD	vivo

R2-2410792	Discussion on other aspects in SBFD	vivo

R2-2410794	Discussion on random access in SBFD	Fujitsu Limited

RAN3#125bis

R3-245006	LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation	RAN1(Ericsson)

R3-245068	Signaling design to enable NR SBFD operation	Qualcomm Incorporated

R3-245133	Discussion on information exchange among gNBs for CLI handling	Samsung

R3-245134	TPs to BLCRs of TS38.473 and TS38.423 on CLI handling	Samsung

R3-245154	TPs to BLCRs of TS38.473 and TS38.423 on CLI handling	Samsung

R3-245187	Discussion on Evolution of Duplex Operation	CATT

R3-245188	(TP for 38.423 and 38.473) Support SBFD Operation	CATT

R3-245198	Discussion on supporting CLI handling in SBFD	CMCC

R3-245216	General discussion on RAN3 impacts by the RAN1 agreements	Huawei

R3-245217	BL CR on the introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Huawei

R3-245283	Discussion on CLI handling support	NEC

R3-245496	Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD	Ericsson

R3-245497	Enhancements for CLI handling in SBFD	Ericsson

R3-245537	Discussion on Information exchange for SBFD	ZTE corporation

R3-245538	(TP for BLCR 38.300&420&423&473-36.423-37.340) SBFD	ZTE corporation

R3-245562	Initial discussion on RAN3 work on Evolution of NR duplex operation: Sub-band full duplex (SBFD)	Nokia

R3-245583	RAN3 issues related to SBFD	Charter Communications, Inc

RAN3#126

R3-247224	Discussion on CLI handling support	NEC

R3-247281	Signaling design to enable NR SBFD operation	Qualcomm Incorporated

R3-247347	Discussion on information exchanges among gNBs for CLI handling	Samsung

R3-247348	TPs to BLCRs of TS38.423 and TS38.473 on CLI handling	Samsung

R3-247519	Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD	Ericsson

R3-247520	TP to TS38.423 on enhancements for CLI handling for SBFD	Ericsson

R3-247567	Further discussion on inter-gNB CLI mitigation for SBFD operation	Nokia

R3-247598	SBFD related CLI measurement configuration, CLI mitigation and CLI testing	Charter Communications

R3-247639	Further discussions on SBFD related information exchange	Huawei

R3-247640	BL CR on the introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)	Huawei

R3-247664	Discussion on supporting CLI handling in SBFD	CMCC

R3-247665	(TP for TS 38.423) Support CLI handling	CMCC

R3-247698	Open issues on CLI handling in SBFD	LG Electronics Inc.

R3-247699	(TP for NR_duplex_evo-Core for TS 38.423 and TS 38.473) Open issues on CLI handling in SBFD	LG Electronics Inc.

R3-247716	Discussion on Evolution of Duplex Operation	CATT

R3-247717	(TP for 38.420, 38.423, 37.340 and 38.473) Support SBFD Operation	CATT

R3-247732	Discussion on Information exchange for SBFD	ZTE corporation

R3-247733	(TP for BLCR 38.300&401&420&423&473-36.423-37.340) SBFD	ZTE corporation

RAN4#112bis

R4-2415021	Discussion on SBFD general part	CMCC

R4-2415022	Discussion on existing Rx requirements for SBFD	CMCC

R4-2415023	Discussion on existing Tx requirements for SBFD	CMCC

R4-2415024	Discussion on new RF requirement for SBFD	CMCC

R4-2415042	Discussion on SBFD General aspects	Tejas Network Limited

R4-2415043	Discussion on Potentially new requirements for SBFD operation	Tejas Network Limited

R4-2415045	Discussion on existing Tx requirements for SBFD operation	Tejas Network Limited

R4-2415046	Views on modification of existing Rx requirements for SBFD	Tejas Network Limited

R4-2415100	Discussion on SBFD system parameters	CATT

R4-2415106	SBFD in-band blocking and dynamic range simulation results	CATT

R4-2415107	Discussion on modification of existing Rx requirements for SBFD operation	CATT

R4-2415108	Discussion on modification of existing Tx requirements for SBFD operation	CATT

R4-2415109	Discussion on potentially new requirements for SBFD operation	CATT

R4-2415145	Consideration on RRM for SBFD	vivo

R4-2415178	Discussions on SBFD RRM requirements	CATT

R4-2415243	Discussion on RRM requirements for SBFD	CMCC

R4-2415272	Consideration of band/channel bandwidth support for SBFD	CableLabs, Charter Communications

R4-2415362	Discussion on RRM requirements for NR duplex operation	LG Electronics Inc.

R4-2415467	Views on general aspects for SBFD	Qualcomm Germany

R4-2415468	Views on new requirements for SBFD	Qualcomm Germany

R4-2415469	Views on existing Tx BS RF requirements for SBFD	Qualcomm Germany

R4-2415470	Views on existing Rx BS RF requirements for SBFD	Qualcomm Germany

R4-2415512	On UE RRM requirement for SBFD	Apple

R4-2415573	RRM requirements for evolution of NR duplex operation	Nokia, Nokia Shanghai Bell

R4-2415587	SBFD minimum channel bandwidth and SBFD as a band-specific feature	Charter, CableLabs

R4-2415661	Topic summary for [112bis][214] NR_duplex_evo	Moderator (Huawei)

R4-2415675	On existing BS RF RX requirements for SBFD operation	Nokia

R4-2415676	On existing BS RF TX requirements for SBFD operation	Nokia

R4-2415677	On potentially new BS RF requirements for SBFD operation	Nokia

R4-2415678	On SBFD system parameters	Nokia

R4-2415690	Views on RRM requirements for Rel-19 SBFD WI	SAMSUNG R&D INSTITUTE JAPAN

R4-2415703	Discussion on RRM aspects of R19 SBFD	ZTE Corporation, Sanechips

R4-2415735	Discussion on RRM core requirements for evolution of NR duplex operation	China Telecom

R4-2415748	Discussion on the subband configurations and guardbands for gNB SBFD	vivo

R4-2415866	Discussion on RRM requirements for SBFD	Huawei, HiSilicon

R4-2416134	SBFD general aspects	Ericsson

R4-2416135	Potentially new SBFD BS RF requirements	Ericsson

R4-2416136	Impact on SBFD BS RF TX requirements	Ericsson

R4-2416137	Impact on SBFD BS RF RX requirements	Ericsson

R4-2416167	Disucssion on SBFD RRM core requirements and simulation assumptions	Ericsson

R4-2416185	On general aspects for SBFD	Huawei, HiSilicon

R4-2416186	On potentially new requirements for SBFD	Huawei, HiSilicon

R4-2416248	Discussion on system parameters for SBFD BS	ZTE Corporation, Sanechips

R4-2416249	Discussion on potentially new requirements for SBFD operation	ZTE Corporation, Sanechips

R4-2416250	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2416251	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2416284	On modification of existing TX RF requirements for SBFD	Huawei, HiSilicon

R4-2416285	On modification of existing RX RF requirements for SBFD	Huawei, HiSilicon

R4-2416453	Discussion on RRM requirements for SBFD	MediaTek inc.

R4-2416471	RRM requirements for SBFD	Qualcomm Incorporated

R4-2416497	Discussion on RAN4 aspects for SBFD system parameters	Samsung

R4-2416498	On the potentially new requirements for SBFD operation for FR1 and FR2-1	Samsung

R4-2416499	On the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS	Samsung

R4-2416500	On the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS	Samsung

R4-2416527	Topic summary for [112bis][306] NR_duplex_evo_General	Moderator (Samsung)

R4-2416528	Topic summary for [112bis][307] NR_duplex_evo_BSRF	Moderator (Huawei)

R4-2416552	Ad-hoc meeting minutes for [112bis][306] NR_duplex_evo_General	Samsung

R4-2416553	Ad-hoc meeting minutes for [112bis][307] NR_duplex_evo_BSRF	Huawei

R4-2416583	Ad-hoc meeting minutes for [112bis][306] NR_duplex_evo_General	Samsung

R4-2416584	Way Forward for [112bis][306] NR_duplex_evo_General	Samsung

R4-2416585	Way Forward for [112bis][307] NR_duplex_evo_BSRF	Huawei

R4-2416858	WF on RRM requirements for NR_duplex_evo	Huawei

R4-2416907	Coffee break discussion minutes for NR_duplex_evo	Samsung

R4-2416909	LS on the scenario of UE to UE CLI measurement report in SBFD operation	Apple

R4-2416910	Simulation assumptions for L1 SRS-RSRP measurement	Ericsson

R4-2416919	LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	Apple

RAN4#113

R4-2417696	Discussion on UE to UE CLI handling	CATT

R4-2417697	Discussion on impacts of SBFD on RRM requirements	CATT

R4-2417786	Discussion on SBFD General aspects	Tejas Network Limited

R4-2417787	Discussion on Potentially new requirements for SBFD operation	Tejas Network Limited

R4-2417788	Discussion on existing Tx requirements for SBFD operation	Tejas Network Limited

R4-2417789	Views on modification of existing Rx requirements for SBFD	Tejas Network Limited

R4-2417845	Discussion on SBFD system parameters	CATT

R4-2417852	Discussion on modification of existing Tx requirements for SBFD operation	CATT

R4-2417853	Discussion on modification of existing Rx requirements for SBFD operation	CATT

R4-2417854	Discussion on potentially new requirements for SBFD operation	CATT

R4-2418020	On existing BS RF RX requirements for SBFD operation	Nokia

R4-2418021	SBFD RX in-band blocking simulation results	Nokia

R4-2418022	On existing BS RF TX requirements for SBFD operation	Nokia

R4-2418023	On potentially new BS RF requirements for SBFD operation	Nokia

R4-2418024	On SBFD system parameters	Nokia

R4-2418052	Cross Link Interference (CLI) handling impact on MPR for SBFD operation	Qualcomm Incorporated

R4-2418068	RRM requirements for UE-to-UE CLI handling	Nokia, Nokia Shanghai Bell

R4-2418069	RRM requirements for SBFD operation	Nokia, Nokia Shanghai Bell

R4-2418097	Discussion on RAN4 aspects for SBFD system parameters	Samsung

R4-2418098	On the potentially new requirements for SBFD operation for FR1 and FR2-1	Samsung

R4-2418099	On the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS	Samsung

R4-2418100	On the modification of existing RX requirements for FR1 and FR2-1 SBFD-capable BS	Samsung

R4-2418105	Discussion on RRM impacts of UE-to-UE CLI handling	LG Electronics Inc.

R4-2418106	Discussion on RRM impacts of SBFD operation	LG Electronics Inc.

R4-2418159	SBFD minimum channel bandwidth	Charter Communications, Inc

R4-2418160	Dradt CR for SBFD minimum channel bandwidth	Charter Communications, Inc

R4-2418192	Discussion on the subband configurations and guardbands for gNB SBFD	vivo

R4-2418281	Topic summary for [113][223] NR_duplex_evo	Moderator (Huawei)

R4-2418447	Discussion on RRM impacts for SBFD operation	CMCC

R4-2418448	Discussion on RRM requirements for UE-to-UE CLI handling	CMCC

R4-2418469	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation	China Telecom

R4-2418470	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation	China Telecom

R4-2418535	On UE RRM requirement for SBFD	Apple

R4-2418602	Views on new requirements for SBFD	Qualcomm Germany

R4-2418603	Views on existing Tx BS RF requirements for SBFD	Qualcomm Germany

R4-2418604	Views on existing Rx BS RF requirements for SBFD	Qualcomm Germany

R4-2418738	Discussion on SBFD general part	CMCC

R4-2418739	Discussion on existing Rx requirements for SBFD	CMCC

R4-2418740	Discussion on existing Tx requirements for SBFD	CMCC

R4-2418741	Discussion on new RF requirement for SBFD	CMCC

R4-2418790	SBFD general aspects	Ericsson

R4-2418791	Potentially new SBFD BS RF requirements	Ericsson

R4-2418792	Impact on SBFD BS RF TX requirements	Ericsson

R4-2418793	Impact on SBFD BS RF RX requirements	Ericsson

R4-2418894	Consideration on RRM requirements for UE-to-UE CLI handling	vivo

R4-2418895	Consideration on RRM impacts for SBFD operation	vivo

R4-2419033	On general aspects for SBFD	Huawei, HiSilicon

R4-2419034	On potentially new requirements for SBFD	Huawei, HiSilicon

R4-2419147	Discussion on UE CLI measurement for SBFD	Huawei, HiSilicon

R4-2419148	Initial simulation results for UE CLI measurement	Huawei, HiSilicon

R4-2419149	Discussion on RRM impacts for SBFD operation	Huawei, HiSilicon

R4-2419219	Discussion on system parameters for SBFD BS	ZTE Corporation, Sanechips

R4-2419220	Discussion on potentially new requirements for SBFD operation	ZTE Corporation, Sanechips

R4-2419221	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2419222	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2419244	On modification of existing TX RF requirements for SBFD	Huawei, HiSilicon

R4-2419245	On modification of existing RX RF requirements for SBFD	Huawei, HiSilicon

R4-2419269	Discussion on UE-to-UE CLI of R19 SBFD	ZTE Corporation, Sanechips

R4-2419270	Discussion on other RRM impacts of R19 SBFD	ZTE Corporation, Sanechips

R4-2419442	Simulation results for SBFD UE UE CLI measurement L1-SRS-RSRP value	Ericsson

R4-2419443	Discussion on RRM requirements for UE-to-UE CLI handling	Ericsson

R4-2419444	Discussion on RRM impacts for SBFD operations	Ericsson

R4-2419512	Discussion on SBFD UE-to-UE CLI	MediaTek inc.

R4-2419513	Discussion on SBFD Operation	MediaTek inc.

R4-2419514	Discussion on SBFD simulation performance	MediaTek inc.

R4-2419593	Topic summary for [113][306] NR_duplex_evo_General	Moderator (Samsung)

R4-2419594	Topic summary for [113][307] NR_duplex_evo_BSRF	Moderator (Huawei)

R4-2419619	Views on RRM requirements for Rel-19 SBFD operation	SAMSUNG R&D INSTITUTE JAPAN

R4-2419620	Initial L1 CLI measurement simulation results	SAMSUNG R&D INSTITUTE JAPAN

R4-2419621	Views on L1 CLI measurement requirements	SAMSUNG R&D INSTITUTE JAPAN

R4-2419766	RRM requirements for SBFD	Qualcomm Incorporated

R4-2419815	Ad-hoc meeting minutes for [113][306] NR_duplex_evo_General	Samsung

R4-2419816	Ad-hoc meeting minutes for [113][307] NR_duplex_evo_BSRF	Huawei

R4-2420097	WF on RRM requirements for NR_duplex_evo	Huawei

R4-2420165	LS on CSI-RS measurement with SBFD operation	MediaTek

R4-2420166	Summary of simulation results for L1-SRS-RSRP	Huawei, HiSilicon

R4-2420167	Coffee break discussion minutes for NR_duplex_evo	Samsung

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template