3GPP TSG RAN #106	RP-243112

Madrid, Spain, December 9th – 12th, 2024

Agenda Item:	9.1.5

Source:	CATT

Title:	Views on support of irregular channel bandwidth in Rel-19

Document for:	Discussion and decision

# Introduction

To optimize the use of spectrum already owned by operators, there is a significant demand for supporting irregular channel bandwidths. As a result, 3GPP initiated a new study item in Release 17 to explore potential solutions for this support [1]. The table below highlights these demands across some specific frequency bands.

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11, 12.5 MHz |
| n12 | 12 MHz |
| n26 | 7 MHz |
| n28 | 13, 33 MHz |
| n29 | 6, 11 MHz |

When the study item was finalized, four solutions were identified and thoroughly compared, each with its own set of pros and cons [2].

After the completion of the study item, no follow-up work item was initiated for these solutions. However, in Rel-18, the enhanced channel raster was introduced to partially address the issues identified during the study [3].

In this contribution, we present our views on the support of irregular channel bandwidth in Rel-19.

# Discussion

- On one hand, supporting irregular channel bandwidth enables operators to make the most of their spectrum assets, optimize network performance, reduce costs, and better meet the demands of modern mobile services, which is critical in the context of 5G deployments, and operators must find ways to efficiently manage and utilize a wide range of frequency bands.

- On the other hand, 3GPP RAN4 workload is always a persistent challenge in meeting the diverse needs related to specifying bands, band combinations, and other features. Therefore, RAN plenary should also consider the available RAN4 TU budget when addressing the support for irregular channel bandwidths. Finding a balanced approach is crucial in order to minimize the RAN4 workload while still meeting the demands for irregular spectrum usage. The key goal is to ensure that the support for irregular channel bandwidths is effectively specified in Rel-19 without overburdening RAN4.

- Proposal 1:   RAN plenary to ensure that the support for irregular channel bandwidths is effectively specified in Rel-19 without overburdening RAN4.

- Actually, there are possibly two tracks on how to specify such support in Rel-19:

- Track #1: specifying a generic solution instead of some specific irregular channel bandwidths with minimized specs impacts.

- Track #2: specifying the support of some specific irregular channel bandwidths with most urgent demands.

- Observation 1: There are possibly two tracks on how to specify such support in Rel-19, one for targeting for a generic solution, the other for some specific irregular channel bandwidth.

- Obviously, if there are demands for other irregular channel bandwidths requested in the future, the works on track #2 cannot directly be reused, hence track #2 is not a future-proof way.

- Observation 2: A future-proof way is preferred if RAN decides to specify the support of irregular channel bandwidth in Rel-19.

- In order to reduce the workload of RAN4, the generic solution to be specified should be based on the outcome of the study item, which also possibly minimize the specs impacts.

- Observation 3: A generic solution is preferred with minimized specs impacts among the identified solutions by the Rel-18 study item.

- Below are four solutions identified in the study item [2]:

- Two-cell approach:

  - Overlapping CA

- One-cell approach:

- Next wider channel bandwidth with PRB blanking

- Overlapping CBWs from the network perspective

- Combined UE CBW

- The overlapping CBWs solution has the least impact on specifications among the four options. Therefore, if the RAN plenary decides to support irregular channel bandwidths, this solution would be the optimal choice.

- Proposal 2: RAN to specify the approach of overlapping CBWs from the network perspective if deciding to support irregular channel bandwidths in Rel-19.

# Conclusions

In this contribution, we share our views on the support of irregular channel bandwidth in Rel-19 with the following observations and proposals:

- Observation 1: There are possibly two tracks on how to specify such support in Rel-19, one for targeting for a generic solution, the other for some specific irregular channel bandwidth.

- Observation 2: A future-proof way is preferred if RAN decides to specify the support of irregular channel bandwidth in Rel-19.

- Observation 3: A generic solution is preferred with minimized specs impacts among the identified solutions by the Rel-18 study item.

- Proposal 1:   RAN plenary to ensure that the support for irregular channel bandwidths is effectively specified in Rel-19 without overburdening RAN4.

- Proposal 2: RAN to specify the approach of overlapping CBWs from the network perspective if deciding to support irregular channel bandwidths in Rel-19.

# References

RP-202103, “New SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths”, T-Mobile USA, Ericsson

3GPP TR 38.844, “Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidth”

RP-230813, “New WID on Channel raster enhancement”, Ericsson