3GPP TSG RAN WG meeting #106	                                          RP-243166

Madrid, Spain, Dec 9-12, 2024

# Title: 	Discussion on XR enhancements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	9.3.2.3

# Document for:	Discussion and decision

# Introduction

In this paper, we discuss the conclusions made in RAN2/RAN for the multi-modality related objectives and propose the updated objective for the WI. Further, during the discussion in RAN2 for RAN controlled rate adaptation for XR, it was agreed that the DL rate control needs further discussion in RAN plenary from the WI scope perspective. This is also discussed in this contribution.

# Multi-modality

The WI for XR enhancements in Rel-19 has a check-point in RAN#106 to check the progress of multi-modality related objective in the WID:

RP-241771: - Specify support for multimodality in RAN for UL and DL [RAN3]: NOTE: This is subject to alignment with SA2, e.g., if MMSID is not available from CN, then UE assistance information-based approach as an alternative. RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE. NOTE: Review in RAN#106 the conclusions in SA2 side (including MMSID delivery to RAN) to allocation for further TUs in RAN3 from Q1/25 onwards.

With regards to multi-modality, RAN2 has made a number of agreements:

RAN2#126 Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. Ask SA2 whether/what information could be provided to RAN Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows. RAN2#127 Working assumption: Regardless of SA2 decision, RAN2 can extend the UAI for multi-modal awareness at least for uplink QoS flows in Rel-19 XR, by having the UE report existence of multi-modality application and association information among QFIs to gNB. FFS whether this can be applied to DL. RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. - The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

Based on the above agreements, it can be observed that RAN2 made significant progress for multi-modality study and identified a number of promising options for normative work. There were agreements both on which items to pursue further, but also regarding which items are not further considered. Further RAN2 has sent LSs asking SA2 to provide the MMSID from CN to RAN (R2-2409272) and RAN3 (R3-245682) confirmed the feasibility of the signalling of MMSID in the LS to SA2.  However, there still seems to be no consensus in SA2 for the CN to deliver the existing MMSID to RAN. Based on this, as per the agreement at RAN#105, it is proposed hence to proceed with UE Assistance Information (UAI) based approach for provisioning multi-modality related assistance information in RAN.

Thus, the following objectives are proposed for multi-modality support in Rel-19:

Proposal 1: Add the following objectives for normative work on multi-modality in Rel-19:

- Specify enhancements to extend the UE Assistance Information framework to enable UE to provide multi-modality related assistance information [RAN2]

# RAN exposure/RAN involved rate control for XR

At RAN2#128, The following agreements were reached for XR rate control:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

As noted above, there was some discussion in RAN2 regarding DL rate control. However, it was not possible to agree this because the WID objective mentions only UL. It should be noted the existing mechanism for rate control supports both UL and DL. The support for DL rate control comes without any further enhancements in RAN level since the DL rate control is negotiated directly between the application server and the UE using existing mechanisms (which are used for DL rate control for the voice applications). Thus, not supporting DL rate control for XR when the existing mechanisms already support this is a bit restrictive from the feature perspective and would also result in unnecessary restrictions in the specifications since the gNB can always use the existing recommended bit rate MAC CE for both UL and DL anyway.

Proposal 2: Modify the existing objective for RAN based XR rate control to support both UL and DL.

# SA2 progress

Based on the current SA2 progress, normalization of the following aspects may need further discussion in RAN WGs.

- Key issue#1

The following content has been agreed in SA2 and captured in the updated TR 23.700-70.

1.	In the case of no DL PDU Set QoS parameters provided by the AF, according to the Protocol Description and PSA UPF supports of PDU Set Information marking, the SMF sends NG-RAN a new indicator indicating the DL PDU Set Information Marking is supported. The SMF activates the PDU Set Information marking based on information from NG-RAN.

It is clear that SMF may send the DL PDU Set Information information to NG-RAN node to show this function is supported by SMF.

In addition, based on the received LS(R3-247113) from SA2, RAN3 further discussed how to transmit the PDU Set based Handling information between SMF and NG-RAN. No consensus has been made in RAN3 on how does SMF and NG-RAN send the PDU Set handling information. Hence, it is clear that at least there is stage 2 impact in RAN3 to capture the SA2 decision in the other PDU Set Handling. Whether any stage 3 enhancement on this aspect depends on SA2 further progress.

- Key issue#5

Time to Next Burst information has been defined by SA2 which is used to provide the time interval between the current burst and the next burst in the 1st packet of the burst via N6. SA2 also sent LS(S2-2407351) to SA4 and asked whether it is feasible for the server to provide such info. Besides, the Burst Size has also been defined by SA2 and introduced in SA2 TR. If this is to be supported in RAN, then signalling needs to be enabled by RAN3. Detailed impact at RAN interfaces may be further discussed in related WGs (e.g., RAN3, SA2, SA4).

- Key issue#9

In the reply LS(S2-2412720), SA2 answers the RAN3 question on the Available Data Rate and further clarify SA2 understanding on how to use this information for XR data transmission. In addition, the following content has been captured in the updated TR 23.700-70.

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], and the available data rate is reported to the AF directly or via the local NEF.

-	Available data rate refers to the data rate that RAN is able to provide within a duration. Such duration is based on RAN implementation.

NOTE 1:	It is recommended that the available data rate exposure is not reported very frequently.

NOTE 2:	How the RAN measures the available UL and/or DL data rate provided by the RAN node is to be refined based on the potential feedback from RAN WG3.

RAN3 will evaluate and further discuss whether/how to introduce the Available Data Rate in next RAN3 meeting. Whether there is any specification impact depends on the RAN3 decisions.

Proposal 3: Add the following objective for normative work on XR in Rel-19 [RAN3]:

- Support PDU Set Information Marking without QoS parameters;

- Identify signalling impacts towards Key issue#5 (support of time to next burst/burst size information) and #9 (exposure of available data rate to CN), if any.

# Conclusion

Proposal 1: Add the following objectives for normative work on multi-modality in Rel-19:

- Specify enhancements to extend the UE Assistance Information framework to enable UE to provide multi-modality related assistance information [RAN2]

Proposal 2: Modify the existing objective for RAN based XR rate control to support both UL and DL.

Proposal 3: Add the following objective for normative work on XR in Rel-19[RAN3]:

- Support PDU Set Information Marking without QoS parameters;

- Identify signalling impacts towards Key issue#5 (support of time to next burst/burst size information) and #9 (exposure of available data rate to CN), if any.

The following updates to the objectives are proposed as a result:

The Rel-19 XR Phase 3 objectives are as follows: - Specify support for multimodality in RAN for UL and DL using UE Assistance Information [RAN2]: - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2]: - Specify in MAC layer, XR rate control signalling on a per QoS flow basis to enable faster source rate adaption to uplink/downlink congestion. - Specify Core requirements related to the above objectives as necessary [RAN4]. - Support PDU Set Information Marking without QoS parameters [RAN3] - Identify signalling impacts towards Key issue#5 (support of time to next burst/burst size information) and #9 (exposure of available data rate to CN), if any [RAN3]

# References

- RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#103