3GPP TSG RAN meeting #106										       RP-243310

Madrid, Spain, December 9-12, 2024

## Status Report to TSG

Agenda item:			9.3.1.6

| WI / SI Name | Multi-carrier enhancements for NR Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_MC_enh2 |  |  |  |  |
| Unique ID | 1050120 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242408 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 40% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Haipeng Lei |
|  | Company | Lenovo |
|  | Email | leihp1@lenovo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#118bis:

Agreement

- For multiple PUSCHs/PDSCHs scheduled on a cell by a DCI format 0_3/1_3,

- Common FDRA is applied to the PUSCHs/PDSCHs on the cell as Rel-16/17 multi-PUSCH/PDSCH scheduling.

- Common MCS is applied to the PUSCHs/PDSCHs on the cell as Rel-16/17 multi-PUSCH/PDSCH scheduling.

- HARQ process number indicated for the cell is applied to the first scheduled PUSCH/PDSCH and then incremented by 1 for subsequent PUSCHs/PDSCHs on the cell (with modulo operation if needed) as Rel-16/17 multi-PUSCH/PDSCH scheduling.

Agreement

- In DCI format 0_3/1_3, for each block of NDI field, consider the following options:

  - Option 1: the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

  - Option 2: the number of bits is equal to the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

  - Option 3: if the number of scheduled PUSCH/PDSCH is 1, then one bit NDI is applied; otherwise, option 1 is applied.

Agreement

- In DCI format 0_3/1_3, for each block of RV field, consider the following options:

  - Option 1: the number of bits is determined based on the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell.

  - Option 2: the number of bits is determined based on the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell.

  - Option 3: if the number of scheduled PUSCH/PDSCH is 1, then option 2 is applied; otherwise, option 1 is applied.

Agreement

- A single TDRA field in DCI format 0_3/1_3 indicates one row from a joint TDRA table.

- Each row in the table contains only one TDRA index for each BWP of each cell within the set of cells. Each TDRA index points to one or multiple time domain resource allocations in the TDRA table applicable for multi-PUSCH/PDSCH scheduling by DCI format 0_3/1_3 for the corresponding cell.

Agreement

- Time domain HARQ-ACK bundling is supported.

Agreement

- Consider at least the case that up to two different SCS can be scheduled by a DCI format 0_3/1_3 in Rel-19.

- Consider at least the following cases for scheduled cells in Rel-19:

- Case 1: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR1 licensed TDD cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 2: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 3: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with SCS1 and FR2-1 cell(s) with SCS2.

  - SCS1 can be same or different to SCS2.

- Case 4: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed FDD cell(s) with different SCS.

- Case 5: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR1 licensed TDD cell(s) with different SCS.

- Case 6: A DCI format 0_3/1_3 scheduling PUSCHs/PDSCHs on FR2-1 cell(s) with different SCS.

RAN1#119:

Agreement

- Time-domain HARQ-ACK bundling is configured per cell as Rel-17.

Agreement

- Specification supports the maximum number of PUSCHs/PDSCHs for a scheduled cell by a DCI format 0_3/1_3 is 8.

- Payload size of a DCI format 0_3/1_3 exceeding 140 is not supported in Rel-19.

Agreement

- Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook.

- The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception.

- The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1.

- Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18.

- Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18.

Agreement

- For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, to select one of the following options.

  - Option 1: M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration.

  - Option 2: M is explicitly configured by RRC parameter for the PUCCH group.

  - Other options are not precluded.

Agreement

- For NDI indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18.

Agreement

- For RV indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18.

#### 2.1.2	Remaining Open issues

Remaining details on NDI indication

Remaining details on RV indication

Remaining details on HARQ-ACK enhancement,

- the number of HARQ-ACK information bits for each DCI format 1_3 associated with the second sub-codebook.

- HARQ-ACK information bit ordering for a DCI format 1_3 associated with the second sub-codebook.

- the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#118bis

- R1-2409166	Feature lead summary #5 on multi-cell scheduling with a single DCI 	Moderator (Lenovo)

- R1-2409165	Feature lead summary #4 on multi-cell scheduling with a single DCI 	Moderator (Lenovo)

- R1-2409164	Feature lead summary #3 on multi-cell scheduling with a single DCI 	Moderator (Lenovo)

- R1-2409163	Feature lead summary #2 on multi-cell scheduling with a single DCI 	Moderator (Lenovo)

- R1-2409162	Feature lead summary #1 on multi-cell scheduling with a single DCI 	Moderator (Lenovo)

- R1-2408261	Work plan for Rel-19 Multi-carrier enhancements	 Lenovo

- R1-2407688	Discussion on Rel-19 Multi-carrier enhancements	 Huawei, HiSilicon

- R1-2407726	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI Spreadtrum Communications

- R1-2407810	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI ZTE Corporation, Sanechips

- R1-2407883	Discussion on enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI	vivo

- R1-2407925	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CMCC

- R1-2408038	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CATT

- R1-2408157	Discussion of multi-cell scheduling with a single DCI	OPPO

- R1-2408260	On Rel-19 Multi-carrier enhancements for NR Phase 2	Nokia

- R1-2408262	Discussion on multi-cell scheduling with a single DCI	Lenovo

- R1-2408364	Discussion on multi-carrier enhancements for NR Phase 2	Panasonic

- R1-2408492	On multi-cell PUSCH/PDSCH scheduling with single DCI	Apple

- R1-2408668	Enhancements for multi-cell PUSCH/PDSCH scheduling	Samsung

- R1-2408669	Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI	TCL

- R1-2408683	Discussion on single DCI based multi-cell scheduling for Rel-19	LG Electronics

- R1-2408713	Multi-Carrier enhancements initial views	MediaTek Inc.

- R1-2408805	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

- R1-2408872	Multi-cell PUSCH/PDSCH scheduling with a single DCI	Qualcomm Incorporated

- R1-2408893	Multi-cell PxSCH scheduling with a single DCI	Ericsson

RAN1#119

- R1-2410905	Feature lead summary #5 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

- R1-2410732	Feature lead summary #4 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

- R1-2410731	Feature lead summary #3 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

- R1-2410730	Feature lead summary #2 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

- R1-2410729	Feature lead summary #1 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

- R1-2409484	Discussion on multi-cell scheduling with a single DCI	Lenovo

- R1-2409532	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CMCC

- R1-2409541	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	ZTE Corporation, Sanechips

- R1-2409619	Enhancements for multi-cell PUSCH/PDSCH scheduling	Samsung

- R1-2409655	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	Spreadtrum, UNISOC

- R1-2409703	Discussion on enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI	vivo

- R1-2409716	On Rel-19 Multi-carrier enhancements for NR Phase 2	Nokia

- R1-2409828	On multi-cell PUSCH/PDSCH scheduling with single DCI	Apple

- R1-2409868	Discussion on multi-cell scheduling with a single DCI	NEC

- R1-2409931	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CATT

- R1-2410010	Discussion on multi-carrier enhancements for NR phase 2	China Telecom

- R1-2410066	Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI	TCL

- R1-2410100	Discussion of multi-cell scheduling with a single DCI	OPPO

- R1-2410250	Discussion on multi-carrier enhancements for NR Phase 2	Panasonic

- R1-2410281	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	ETRI

- R1-2410298	Discussion on single DCI based multi-cell scheduling for Rel-19	LG Electronics

- R1-2410408	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

- R1-2410500	Multi-cell PUSCH/PDSCH scheduling with a single DCI	Qualcomm Incorporated

- R1-2410509	Multi-cell PUSCH/PDSCH scheduling with a single DCI	MediaTek Inc.

- R1-2410536	Multi-cell PxSCH scheduling with a single DCI	Ericsson

- R1-2409404	Discussion on Rel-19 Multi-carrier enhancements	Huawei, HiSilicon

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template