3GPP TSG RAN Meeting #105						                                           RP-242346

Melbourne, Australia, Sept 9-12, 2024                                                                          (revision of RP-242061)

Status Report to TSG

Agenda item:			10.4.2

| WI / SI Name | # Introduction of another IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_FDD_S_band |  |  |  |  |
| Unique ID | 1040130 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241662 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 3/2025 | Performance part: 6/2025 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 20% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Hsieh, Daniel |
|  | Company | MediaTek Inc. |
|  | Email | daniel.hsieh@mediatek.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.1.1	RAN4 #112, Aug’24

All the following agreements are based on R4-2414287 “WF on Rel-19 FR1 NTN bands” [8].

Band Numbering

Agreement: Adopt 252 as the band number for the new IoT NTN S-band.

Band Plan

Agreement: Adopt the proposed band plan as follows

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – |  | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz |  | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - |  | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - |  | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - |  | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. |  |  |  |  |  |  |  |  |

Intended Regions and Countries

Agreement: Clarify that the new NTN S-band is only targeting north America.

Regulatory Background

Agreement:

- Capture all the applicable regulations in a new section of TR 38.863

- ATC is not applicable, hence out of scope for this work.

Applicable TN Bands for Coexistence

Agreement:

- Focus on coexistence of S-band UL with B2/n2 and B25/n25 DL.

- Capture the clarifications on the co-existence issues with B70/n70 and B66/n66 and that there is no 3GPP solution for them in the TR

TN-NTN UE-to-UE Coexistence

Agreement:

Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering:

- Realistic usage scenarios

- consider UE antenna gain for smartphone of -5.5dBi

- UE-to-UE separations distance

- probability of the UE-to-UE coexistence scenarios occurring

Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work.

Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710).

Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted.

Channel Bandwidths

Agreement:

-	No impact to existing Cat M1 and Cat NB1, NB2 UE channel bandwidths

EARFCN for CAT M1

Agreement:

-	Adopt the following table as a starting point:

- Table 2.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [252] | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |

Default TX-RX Separation

Agreement:

-	Adopt default TX-RX frequency separation as fixed value as a starting point. Further study whether flexible TX-RX frequency separation can be supported, following conclusions from UE RF maintenance.

General views on Impact to UE Requirements

Agreement:

-	Use the following merged table with common views on identified specification impact as a starting point.

-	Companies to further assess focusing on the missing or TBC sections.

| Clause/Section | Requirement | COMMON VIEWS |
| --- | --- | --- |
| Clause 6 | Transmitter Characteristics |  |
| 6.2A.1 | UE maximum output power | Some impact |
| 6.2B.1 |  |  |
| 6.2A.1 | UE maximum output power tolerance | Some impact |
| 6.2B.1 |  |  |
| 6.2A.2 | UE maximum output power reduction (MPR) | No Impact |
| 6.2B.2 |  |  |
| 6.2A.3 | UE additional maximum output power reduction | Some impact |
| 6.2B.3 |  |  |
| 6.2A.4 | Configured transmitted Power for category M1 | No Impact |
| 6.2B.4 | Configured transmitted Power for category NB1 and NB2 | No Impact |
| 6.3 | Output power dynamics | No Impact |
| 6.4 | Transmit signal quality | No Impact |
| 6.4.2 | Transmit modulation quality | No Impact |
| 6.5A.3.2 | Spectrum emission mask | No Impact |
| 6.5B.3.2 |  |  |
| 6.5A.2 | Occupied bandwidth for category M1 | No Impact |
| 6.5B.2 | Occupied bandwidth for category NB1 and NB2 | No Impact |
| 6.5A.3.3 | Additional Spectrum Emission Mask for category M1 | TBC |
| 6.5B.3.3 | Additional Spectrum Emission Mask for category NB1 and NB2 |  |
| 6.5A.3.4 | Adjacent Channel Leakage Ratio | No Impact |
| 6.5B.3.4 |  |  |
| 6.5A.4.3 | Spurious emissions for UE co-existence | Some impact |
| 6.5B.4.3 |  |  |
| 6.5A.4.4 | Additional spurious emissions | Some impact |
| 6.5B.4.4 |  |  |
| 6.6A | Transmit intermodulation for category M1 | Not applicable |
| 6.6B | Transmit intermodulation for category NB1 and NB2 | No impact |
| Clause 7 | Receiver characteristics |  |
| 7.2 | Diversity characteristics | No impact |
| 7.3A | Reference sensitivity power level for UE category M1 | Some Impact |
| 7.3 | Reference sensitivity power level for UE category NB1 and NB2 | Some Impact |
| 7.4 | Maximum input level | No impact |
| 7.5 | Adjacent Channel Selectivity | No impact |
| 7.6A.2 | In-band blocking requirements for category M1 | TBC |
| 7.6B.2 | In-band blocking requirements for category NB1 and NB2 |  |
| 7.6A.3 | Out-of-band blocking requirements for category M1 | Some impact |
| 7.6B.3 | Out-of-band blocking requirements for category NB1 and NB2 |  |
| 7.6A.4 | Narrow band blocking for category M1 | TBC |
| 7.6B.4 | Narrow band blocking for category NB1 and NB2 |  |
| 7.7 | Spurious response | No impact |
| 7.8 | Intermodulation characteristics | No impact |
| 7.9 | RX spurious emission | TBC |

Other Sub-topics/Issues

Agreement:

-	Other topics and issues are postponed to future meetings

Work Split/Interest

Agreement:

-	No formal CR/Spec Work Split agreed in this meeting

-	Companies to report areas/topics of interest for contribution or future CR work split

-	Following table shall be used

| TS/TR (if applicable) | Clause/Section (if applicable | Topic/Requirement | Interested contributing companies |
| --- | --- | --- | --- |

2.4.2	Remaining Open issues

- System parameters including channel raster and flexible TX-RX frequency separation,

- UE RF requirements including MOP, A-MPR, UE-coexistence requirements, reference sensitivity

- Detailed specification updates

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#112, Aug’24:

- R4-2411304	NTN MSS S-band band plan, regulations, and deployment scenarios 	EchoStar, DISH Network, TerreStar, Thales, Gatehouse, Novamint

- R4-2411547	Work plan for an IoT-NTN S-band for North America (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) 	Mediatek Inc.

- R4-2412460	Discussion on NTN-TN UE-coexistence issue for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)	Mediatek Inc.

- R4-2411548	Discussion on IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements	Mediatek Inc.

- R4-2411845	Discussion on system parameters and UE RF requirements for IoT-NTN S band	ZTE Corporation, Sanechips

- R4-2411846	Discussion on SAN RF requirements for IoT-NTN S band	ZTE Corporation, Sanechips

- R4-2411847	draftCR to TS36.108 Introduction of IoT-NTN S band	ZTE Corporation, Sanechips

- R4-2414287	WF on Rel-19 FR1 NTN bands	Inmarsat