3GPP TSG RAN Meeting #105							RP-242354

Melbourne, Australia, Sep 9-12 2024					   (revision of RP-241650)

# Source:	Ericsson

# Title:	Revised WID: Enhancements of network energy savings for NR

# Document for:	Approval

Agenda Item:	9.3.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Enhancements of network energy savings for NR

Acronym: Netw_Energy_NR_enh

Unique identifier:	1020095

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-19

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
|  | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

### 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_Netw_Energy_NR | RAN1 | 940080 | Study on network energy savings for NR |
| Netw_Energy_NR | RAN1 | 981037 | Network energy savings for NR (Release 18) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| EE_5G | 810023 | Energy Efficiency of 5G | Definition of energy efficiency (EE) KPI for NG-RAN, use cases, requirements and procedures for collecting performance measurements (namely data volume measurements and PEE (Power, Energy and Environment) parameters from 5G base stations (TS28.310) |
| FS_EE_5G | 760064 | Study on system and functional aspects of Energy Efficiency in 5G networks | Study Item that lead to WI on Energy Efficiency of 5G |
| FS_ENEFF | 710049 | Study on Energy Efficiency Aspects of 3GPP Standards | SA study item |
| FS_NG_SReq | 700058 | Study on Scenarios and Requirements for Next Generation Access Technologies | RAN study item |
| SON_5G | 850030 | Self-Organizing Networks (SON) for 5G networks | Energy Saving NRM based on SON NRM |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

# 3	Justification

Network energy saving is of great importance for environmental sustainability, to reduce environmental impact (greenhouse gas emissions), and for operational cost savings. As 5G is becoming pervasive across industries and geographical areas, handling more advanced services and applications requiring very high data rates (e.g. XR), networks are being denser, use more antennas, larger bandwidths and more frequency bands. The environmental impact of 5G needs to stay under control, and novel solutions to improve network energy savings need to be developed.

Energy consumption has become a key part of the operators’ OPEX. According to the report from GSMA [1], the energy cost on mobile networks accounts for ~23% of the total operator cost. Most of the energy consumption comes from the radio access network and in particular from the Active Antenna Unit (AAU), with data centres and fibre transport accounting for a smaller share. The power consumption of a radio access can be split into two parts: the dynamic part which is only consumed when data transmission/reception is ongoing, and the static part which is consumed all the time to maintain the necessary operation of the radio access devices, even when the data transmission/reception is not on-going.

During the study in the SI phase [2], the network energy consumption model for the base station (BS) was defined including the reference configurations for FR1 TDD/FDD and FR2, the deep/light/micro sleep power states with corresponding relative power, transition time and energy consumption among different power states based on two types of BS categories, and the scaling rules for the active DL/UL power states considering BS power split by a static part of power and a dynamic part of power with the latter part reflecting the dynamic power consumption with respect to transmission/reception resource configurations in time, frequency, spatial and power domains. In addition, evaluation methodology and assumptions were achieved to study and evaluate the network energy saving gains for potential techniques with respect to other KPI including UPT, access delay, UE power consumption, etc.

Based on the agreed BS energy consumption model, and the evaluation methodology and assumptions, potential network energy saving techniques in various domains were evaluated with respect to the energy saving gains and the corresponding performance impact considering the above KPIs. The studied techniques are classified into time, frequency, spatial and power domains, and the technical descriptions as well as the legacy UE and specification impacts are summarized in the technical report [2]. The techniques in time and frequency domains mainly aim to reduce the power consumption for dynamic part by trying to shutdown more symbols on one or more carriers to achieve BS micro sleep, and even the static power part by enlarging the interval between the contiguous active transmission/reception occasions to achieve BS light/deep sleep. The techniques in spatial and power domains mainly aim to reduce the power consumption of the TRX chains and PAs by trying to shutdown more spatial elements and/or reduce transmission power/power spectrum density, or increase the PA efficiency. As shown in Section 7 in TR 38.864 [2], some of the studied techniques are beneficial for network energy savings.

The Rel-18 work item on network energy savings for NR led to the specification of some of the techniques that were found beneficial in the study, primarily for RRC Connected, user specific signals and channels, and low load scenarios. The techniques specified in Rel-18 include SSB-less SCell operation for inter-band CA for FR1 and co-located cells, enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, inter-node information exchange on cell DTX/DRX, techniques in spatial and power domains to enable efficient adaptation of spatial elements as well as efficient adaptation of power offset values between PDSCH and CSI-RS, as well as mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, CHO procedure enhancement(s), and inter-node beam activation and enhancements on restricting paging in a limited area, and the corresponding RRM/RF core requirements.

Some other techniques also found to be beneficial in the study were not yet specified in Rel-18. This Rel-19 work item aims to specify further network energy savings targeting the beneficial techniques studied in Rel-18, but yet unspecified, including on-demand SSB and on-demand SIB1 transmissions, as well as adaptation of common signal/channel transmissions.

- GSMA, 5G energy efficiencies: Green is the new black, https://data.gsmaintelligence.com/api-web/v2/research-file-download?id=54165956&file=241120-5G-energy.pdf

- 3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

# 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

    - Note: No modification of SSB will be discussed under this objective

  - Specify inter NG-RAN node signalling (Xn/F1) at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Specify the corresponding core requirements, for the above features [RAN4].

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

- Specify corresponding performance requirements and test cases for the network energy saving techniques [RAN4].

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | NR; Physical channels and modulation | RAN#108 | Core Part |
| 38.212 | NR; Multiplexing and channel coding | RAN#108 | Core Part |
| 38.213 | NR; Physical layer procedures for control | RAN#108 | Core Part |
| 38.214 | NR; Physical layer procedures for data | RAN#108 | Core Part |
| 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#109 | Core part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#109 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#109 | Core Part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#109 | Core Part |
| 38.420 | NG-RAN; Xn general aspects and principles | RAN#109 | Core part |
| 38.423 | NG-RAN; Xn Application Protocol (XnAP) | RAN#109 | Core part |
| 38.470 | NG-RAN; F1 general aspects and principles | RAN#109 | Core part |
| 38.473 | NG-RAN; F1 Application Protocol (F1AP) | RAN#109 | Core part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#109 | Core part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#111 | Performance part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#111 | Performance part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#111 | Performance part |
| 38.101-4 | NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements | RAN#111 | Performance part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#111 | Performance part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

# 6	Work item Rapporteur(s)

Nimbalker, Ajit, Ericsson, ajit.nimbalker@ericsson.com

Cheng, Peng, Apple, pcheng24@apple.com, RAN2

NOTE:	The first listed Rapporteur has the overall responsibility for this WI (incl all secondary tasks).

# 7	Work item leadership

RAN1

Secondary responsible Working Group(s): RAN2, RAN3, RAN4.

# 8	Aspects that involve other WGs

None identified yet.

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

# 9	Supporting Individual Members

Supporting IM name

Ericsson

AT&T

Apple

CATT

CHTTL

China Unicom

China Mobile

China Telecom

ETRI

Fujitsu

Futurewei

Huawei

HiSilicon

III

InterDigital

ITRI

Intel Corporation

KDDI

KT Corp.

Lenovo

LG Electronics

MediaTek

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

NTT DOCOMO

OPPO

Orange

Ovidio

Qualcomm

Samsung

Sharp

Sony

Spreadtrum

WILUS

vivo

ZTE Corporation

Sanechips

FGI

Rakuten Mobile Inc.

Dish Network

Fraunhofer IIS

Fraunhofer HHI

Xiaomi

s