3GPP TSG RAN WG1 #118          					          R1-2405942

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on gNB-gNB CLI handling

Agenda Item:       9.3.3

Document for:     Discussion and decision

# Introduction

The RAN#104 meeting discussed and revised the Work Item Description on the Evolution of NR Duplex Operation in RP-241614[1]. Updated objectives of WID are given below:

Agreement

Followings are assumed based on TR 38.858

  - SBFD at the gNB side

  - Half duplex operation at the UE side

  - FR1 and FR2-1

  - SBFD operation Option 4, i.e., both time and frequency locations of sub-bands for SBFD operation are known to SBFD aware UEs

  - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side

  - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies

  - One UL sub-band for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier

- Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

For enhancements for CLI handling, the following are recommended from RAN1's perspective:

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3]

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3]

- Information exchange of strongest DL beam information [RAN3]

  - Information exchange of CLI-mitigation request [RAN3]

  - Note: No normative behaviour is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signalling can be discussed in RAN3.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - Note: No new DCI field / MAC CE is introduced

  - PUSCH resource mapping, i.e., rate-matching around the muted REs

  - UCI resource determination in symbols with muted REs

    - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212.

  - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH.

  - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode.

  - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol.

  - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols.

  - Note: No changes on TBS determination for PUSCH.

  - Note: Check impact on transmit signal quality/MPR requirement, if any, with RAN4

  - Note: This feature is subject to UE capability

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

  - Measurement resources

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource

      - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs.

    - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource

  - Measurement reporting

    - At least aperiodic reporting

      - Note: Periodic and semi-persistent reporting can be considered

    - New report quantities: e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - Note: At least wideband reporting is supported

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

  - CLI measurement accuracy requirement [RAN4]

- Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

In this contribution, we provide our views on the indication/determination of UL resource muting for PUSCH, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in the time domain. We also provide a scheme for handling the timing offset between UL received signal and CSI-RS due to NTA-Offset.

# Indication/Determination of UL resource muting for PUSCH based on semi-static SBFD configuration

In the case of SBFD UL reception, the gNB receives the signal from its serving UEs (desired signal), and it also receives CLI interference from the neighbouring gNBs of the same network and/or different network gNBs, due to which there is a significant drop in UL throughput performance. To measure the interference for its mitigation, UE needs to mute some of the symbols such that the UL receiver at gNB can measure the interference due to neighbouring gNBs accurately and use it for equalizer weight calculation to suppress interference. The UL Resource muting for PUSCH is done assuming comb-2 for DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in the time domain per slot. As precoding/beamforming applied for PDCCH symbols and PDSCH symbols can be different. This would lead to uplink symbols in a slot experiencing different gNB-gNB CLI. To measure the gNB-gNB CLI, in uplink, we can mute one symbol out of the first three symbols of the slot to capture the interference due to PDCCH channel leakage of the interfering gNB and one OFDM symbol out of the remaining symbols of the slot to measure the gNB-gNB interference due to PDSCH channel leakage. The information pertaining to the symbol(s) to be muted needs to be present at both UE and gNB. The UE needs to know muting symbol information while generating the PUSCH signal, and the gNB needs to know muting symbol information to measure the interference on those muted symbols of the received PUSCH signal. Hence, gNB configures and indicate to serving UEs the symbol to be muted in uplink PUSCH.

In this section, we define two schemes for the indication/determination of the PUSCH muting symbol.

- Scheme –1: PUSCH muting symbol configuration and indication using available padding bits in DCI by gNB.

- Scheme –2: Determination of PUSCH muting symbol and activation using existing DCI field or RRC Message.

- Scheme-1: PUSCH muting symbol configuration and indication using available padding bits in DCI format 0_0

This scheme for PUSCH muting symbol indication for SBFD slots involves optimization of bits for indication without incurring additional bits in DCI, especially in format 0_0. The parameters are identified to define the flexibility of muting symbols and identify the number of bits to transmit the configured muting index. The UE extracts the bits to identify muting symbol index from received DCI to mute PUSCH muting symbols which enable CLI interference computation for its suppression.

The following steps are involved in this resource muting scheme,

Step 1: Provisioning of muting indication bits in DCI formats to transmit the muting index to its serving UEs

As specified in 38.212, the minimum bit size of DCI format 1_0 is 28 excluding frequency domain resource allocation (FDRA) bits, and the maximum bit size of DCI format 0_0 is 23 bits in the worst case, excluding FDRA bits. Hence DCI format 0_0 will have at least 5 padding bits to make the size of DCI format 0_0 and DCI format 1_0 the same (as per 38.212) when FDRA bits of DCI format 0_0 less than or equal to FDRA bits of DCI format 1_0 (which occurs when UL bandwidth part (BWP) size is less than or equals to the DL BWP size). These padding bits can be utilized to indicate the PUSCH resource muting.

As explained in the above paragraph, we observed the availability of at least 5 padding bits in the case of DCI format 0_0, which can be utilized by the gNB for transmitting the muting information.

The below steps describe a scheme to efficiently use these bits to indicate PUSCH muting symbols.

Observation 1: In the case of SBFD, The DCI format 0_0 will have at least 5 padding bits when FDRA bits of DCI format 0_0 is less than or equal to FDRA bits of DCI format 1_0.

Step 2: Defining and configuring parameters required for PUSCH muting symbol indication and to decide the flexibility of PUSCH muting symbol position.

In this scheme, we have defined 3 configuration parameters, K11, K21, and K22, which decide the flexibility of PUSCH resource muting symbol position within a slot. The value of these parameters depends on the maximum bits available for PUSCH resource muting indication using DCI. These parameters also decide the available set of symbols for muting with the exclusion of reference symbols. K11 decides the flexibility of the position of the symbol muting when only one symbol is muted in a slot, K21 decides the flexibility of the position of the first symbol muting out of the first three symbols of a slot, and K22 decides the flexibility of position of second symbol muting out of last 11 symbols of the slot for UL resource muting when two symbols are muted per slot.

We propose to configure these parameters, K11, K21, and K22 to UEs using two alternative ways: 1) the values of the parameters are configured to UEs by gNB using RRC message or 2) the values of the parameters are fixed in the network to certain values.

The configuration of these parameters to UE by gNB is done as follows,

Alt-1: The value of parameter K11 as SBFDOneSymbolMutingParm_r19, K21 as SBFDTwoSymbolMutingParm1_r19, and K22 as SBFDTwoSymbolMutingParm2_r19 are RRC Configured.

Alt-2: The values of parameter K11, K21, and K22 are fixed and preconfigured at UEs and gNBs.

The number of bits required to transmit the muting index is calculated as follows,

mutingbits=ceil(log2 K22*K21+K11+1).

Table 1 represents the example of the number of muting bits required for PUSCH Resource Muting Indication for different values of K11, K21, and K22.

| K11 | K21 | K22 | Number of muting bits required for PUSCH Resource Muting Indication |
| --- | --- | --- | --- |
| 10 | 3 | 7 | 5 |
| 6 | 3 | 3 | 4 |
| 7 | 2 | 4 | 4 |
| 6 | 1 | 1 | 3 |
| 3 | 2 | 2 | 3 |
| 2 | 1 | 1 | 2 |

Observation 2: The PUSCH resource muting parameter values are decided based on the available padding bits in DCI format 0_0.

Observation 3: The PUSCH muting parameters K11, K21, and K22 decide the available set of symbols for muting with the exclusion of reference symbols.

Observation 4: The following table captures examples of the possible values of muting parameters considering the maximum available padding bits of 5 in DCI format 0_0.

| K11 | K21 | K22 | Number of muting bits required for PUSCH Resource Muting Indication |
| --- | --- | --- | --- |
| 10 | 3 | 7 | 5 |
| 6 | 3 | 3 | 4 |
| 7 | 2 | 4 | 4 |
| 6 | 1 | 1 | 3 |
| 3 | 2 | 2 | 3 |
| 4 | 1 | 1 | 3 |
| 2 | 1 | 1 | 2 |

Step3: Configuration of muting symbol index  lmute,1 and  lmute,2 from the available set of symbols in a slot at gNB

The gNB configures the muting symbol index  lmute,1 and  lmute,2 based on the PUSCH time domain resource allocation for each serving UE as follows. We need to consider 2 cases here: the first one is when only one symbol is muted per slot, and the second case is when two symbols are muted per slot.

Case-1: When only one symbol is muted per slot

- List down the first K11 available symbols of the slot for muting in ascending order of symbol number {L0< L1 < L2 < L3 <… < LK11-1}, where Lj is the symbol number. This set does not include the symbols allocated to PUSCH DMRS.

- Assume gNB configures symbol number Lj to be muted, then the muting symbol index  lmute,1 = j.

Example 1: - Let us assume one PUSCH configuration with start symbol (S) = 0 and the number of symbols (L) = 6, and with single symbol DMRS and DMRS position is symbol number 3. The gNB configuration for muting one symbol is K11 = 4. Hence available symbols for muting are {0, 1, 2, 4}. The muting symbol index  lmute,1 calculated from muting index i is equal to 2. Hence, the symbol index in slot corresponds to lmute,1  = 2 is L2 = 2 as shown in the Figure 1.

Figure 1 Muted symbols for PUSCH mapping type A with PUSCH Resource allocation start symbol (S) = 0 and number of symbols (L) = 14.

Example 2: - Let us assume one PUSCH configuration with S = 4 and L = 6, and with single symbol DMRS and DMRS position is symbol number 4. The gNB configuration for muting one symbol is K11 = 4. Hence available symbols for muting are {5, 6, 7, 8}. The muting symbol index  lmute,1 calculated from muting index i is equal to 3. Hence the symbol number in the slot corresponds to  lmute,1  = 3 is L3 = 8 as shown in Figure 2.

Figure 2 Muted symbols for PUSCH mapping type B with PUSCH Resource allocation S = 4 and L = 6.

Case 2: When two symbols are muted per slot, we need to compute.

First symbol muting configuration and muting index  lmute,1  calculation

- List down the first K21 symbols of the slot for muting in ascending order of symbol number {L0< L1 < L2 <… < LK21-1 }, where LK21-1 < K21, after excluding symbols that are not allocated to PUSCH data transmission from the set.

- Assume gNB configures symbol Lj to be muted, then the muting index  lmute,1 = j.

Second symbol muting configuration and muting index  lmute,2  calculation

- List down the first K22 available symbols of the slot for muting in ascending order of symbol number {L0< L1 < L2 <… < LK22-1} where L0 > 2 and LK22-1 ≤ 13, after excluding the symbols that are not allocated to PUSCH data transmission from the set.

- Assume gNB configures symbol number Lj of the slot to be muted, then the muting index  lmute,2 = j.

Note: OFDM symbols that are allocated to that UE for PUSCH data transmission are considered for muting.

Example 1: - Let us assume one PUSCH type-A configuration with S = 0 and L = 14, and with single symbol DMRS and DMRS position is symbol number 2. The gNB configuration for muting one symbol are K11 = 4, K21 = 3 and K22 = 4. Hence available symbols for muting are {0, 1, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14}. The muting symbol index  lmute,1  and  lmute,2 are calculated from muting index i = 12, which is equal to 1 and 3, respectively. The available symbols for muting the first symbol are {0, 1}, and the available symbols for muting the second symbol are {3, 4, 5, 6}. Hence the symbol number in the slot corresponds to lmute,1  = 1 is L0 = 1, and the symbol number in the slot corresponds to lmute,2  = 3 is L3 = 6, as shown in Figure 3.

Figure 3 Muted symbols for PUSCH mapping type A with PUSCH Resource allocation S = 0 and L = 14.

Example 2: - Let us assume one PUSCH type-B configuration with S = 1 and L = 8, and with single symbol DMRS and DMRS position is symbol number 1. The gNB configuration for muting one symbol are K11 = 4, K21 = 3 and K22 = 4. Hence available symbols for muting are {2, 3, 4, 5, 6, 7, 8}. The muting symbol index  lmute,1  and  lmute,2 are calculated from muting index i = 7 are equal to 0 and 2, respectively. The available symbols for muting the first symbol are {2}, and the available symbols for muting the second symbol are {3, 4, 5, 6}. Hence the first symbol number in the slot corresponds to lmute,1  = 0 is L0 = 2, and the second symbol number in the slot corresponds to lmute,2  = 3 is L3 = 6, as shown in the Figure 4.

Figure 4 Muted symbols for PUSCH mapping type B with PUSCH Resource allocation S = 1 and L = 8.

Table 3 Muting index i corresponding to the muting symbol index  lmute,1and  lmute,2 for K11 = 6, K21 = 3 and K22 = 3.

| lmute,1 | lmute,2 | Muting Index (i) |
| --- | --- | --- |
| No Muting |  | 0 |
| 0 | - | 1 |
| 1 | - | 2 |
| 2 | - | 3 |
| 3 | - | 4 |
| 4 | - | 5 |
| 5 | - | 6 |
| 0 | 0 | 7 |
| 1 | 0 | 8 |
| 2 | 0 | 9 |
| 0 | 1 | 10 |
| 1 | 1 | 11 |
| 2 | 1 | 12 |
| 0 | 2 | 13 |
| 1 | 2 | 14 |
| 2 | 2 | 15 |

Table 4 Muting index i corresponding to the muting symbol index  lmute,1and  lmute,2 for K11 = 2, K21 = 1 and K22 = 1.

| lmute,1 | lmute,2 | Muting Index (i) |
| --- | --- | --- |
| No Muting |  | 0 |
| 0 | - | 1 |
| 1 | - | 2 |
| 0 | 0 | 3 |

Table 5 Muting index i corresponding to the muting symbol index  lmute,1and  lmute,2 for K11 = 1, K21 = 1 and K22 = 1.

| lmute,1 | lmute,2 | Muting Index (i) |
| --- | --- | --- |
| No Muting |  | 0 |
| 0 | - | 1 |
| 0 | 0 | 2 |
| Reserved |  | 3 |

Step4: UL resource muting index i calculation for SBFD slot at gNB, which is sent over DCI

PUSCH resource muting index is calculated using K11, K21, K22 and, muting symbol index  lmute,1 and  lmute,2 as follows,

Equation 1

i=0,  if mutng is disabledlmute,1+1,      when one symbol is muted in slot (0≤ lmute,1≤K11-1)lmute,2+ K22*(lmute,1+1)+1+K11-K22, when two symbols are muted  per slot (0≤ lmute,2≤K22-1, 0≤ lmute,1≤K21-1)

Observation 5: The PUSCH resource muting index i is calculated using parameters K11, K21, K22 and configured muting symbol index  lmute,1 and  lmute,2.

Step5: Transmission of muting index i using DCI format 0_0

The muting index i is transmitted using the available padding bits available in the DCI format 0_0, as explained in Step 1.

Step6: Determination of muting symbol indexes  lmute,1 and  lmute,2 using the muting index i received on the control channel at UE

As there is a unique one-to-one mapping between the muting index i and, the muting symbol index  lmute,1 and  lmute,2. Hence UEs form a lookup table for all the possible values of lmute,1 and  lmute,2 to calculate the muting index i using Equation 1 for configured values of K11, K21, and K22. This lookup table is used by UE to get the muting symbol index  lmute,1 and  lmute,2 from received muting index i.

Table 6 Muting index i corresponding to the muting symbol index  lmute,1and  lmute,2 for K11 = 10, K21 = 3 and K22 = 7.

| lmute,1 | lmute,2 | Muting Index (i) | lmute,1 | lmute,2 | Muting Index (i) |
| --- | --- | --- | --- | --- | --- |
| No Muting |  | 0 | 5 | 0 | 16 |
| 0 | - | 1 | 6 | 0 | 17 |
| 1 | - | 2 | 0 | 1 | 18 |
| 2 | - | 3 | 1 | 1 | 19 |
| 3 | - | 4 | 2 | 1 | 20 |
| 4 | - | 5 | 3 | 1 | 21 |
| 5 | - | 6 | 4 | 1 | 22 |
| 6 | - | 7 | 5 | 1 | 23 |
| 7 | - | 8 | 6 | 1 | 24 |
| 8 | - | 9 | 0 | 2 | 25 |
| 9 | - | 10 | 1 | 2 | 26 |
| 0 | 0 | 11 | 2 | 2 | 27 |
| 1 | 0 | 12 | 3 | 2 | 28 |
| 2 | 0 | 13 | 4 | 2 | 29 |
| 3 | 0 | 14 | 5 | 2 | 30 |
| 4 | 0 | 15 | 6 | 2 | 31 |

Step7: Mapping the muting symbol index  lmute,1 and  lmute,2  to PUSCH muting symbol number of a slot at UE

Case-1 When only one symbol is muted per slot

UE constructs a list of the first K11 available symbols within a slot in ascending order based on the time domain resource configuration received using the control channel for PUSCH, as explained in step 2. The muting symbol index  lmute,1 specifies the index of the symbol number in the obtained list of available symbols for muting.

Case-2 When two symbols are muted per slot

Mapping of first muting symbol index to symbol number in the slot which needs to be muted

UE constructs a list of the first available K21 symbols within a slot in ascending order based on the time domain configuration received using the control channel for PUSCH, as explained in step 2. The mapping of muting symbol index  lmute,1 specifies the index of the symbol number in the obtained list of available symbols for muting.

Mapping of second muting symbol index to symbol number in the slot which needs to be muted

UE constructs a list of the first available K22 symbol of slot in their increasing symbol number for muting based on the time domain configuration received using the control channel for PUSCH configuration as explained in step 2. The mapping of muting symbol index  lmute,2 specifies the index of the symbol number in the obtained list of available symbols for muting.

Proposal 1: For sending the resource muting indication bits to UE over DCI, we propose to use the available padding bits (≥5) in DCI format 0_0 when FDRA bits of DCI format 0_0 less than or equal to FDRA bits of DCI format 1_0.

Proposal 2: For PUSCH muting symbol configuration and indication, we propose the following steps,

- Step1: Provisioning of muting indication bits in DCI formats for transmitting muting index to its serving UEs

For sending the resource muting indication bits to UE over DCI, we propose to use the available padding bits (≥5) in DCI format 0_0 when FDRA bits of DCI format 0_0 less than or equals to FDRA bits of DCI format 1_0.

- Step 2: Defining and configuring parameters required for PUSCH muting symbol indication and deciding the flexibility of PUSCH muting symbol position.

We propose to configure these parameters, K11, K21, and K22 to UE using two alternative ways: 1) the values of the parameters are configured in a UEs by gNB using RRC message or 2) the values of the parameters are fixed and preconfigured at UEs and gNBs.

Configuration of parameters K11, K21, and K22 to UE by gNB is done as follows

Alt-1: The value of parameter K11 as SBFDOneSymbolMutingParm_r19, K21 as SBFDTwoSymbolMutingParm1_r19, and K22 as SBFDTwoSymbolMutingParm2_r19 are RRC Configured.

Alt-2: The values of parameter K11, K21, and K22 are fixed for network.

The number of bits required to transmit the muting index is calculated as follows,

mutingbits=ceil(log2 K22*K21+K11+1).

- Step3: Configuration of muting symbol index  lmute,1 and  lmute,2 from the available set of symbols in a slot at gNB

- Step4: UL resource muting index i calculation for SBFD slot at gNB, which is sent over DCI

i=0,  if mutng is disabledlmute,1+1,      when one symbol is muted in slot (0≤ lmute,1≤K11-1)lmute,2+ K22*(lmute,1+1)+1+K11-K22, when two symbols are muted  per slot (0≤ lmute,2≤K22-1, 0≤ lmute,1≤K21-1)

- Step5: Transmission of muting index i using DCI format 0_0

- Step6: Determination of muting symbol indexes  lmute,1 and  lmute,2 using the muting index i received on the control channel at UE

As there is a unique one-to-one mapping between the muting index i and, the muting symbol index  lmute,1 and  lmute,2. Hence UEs form a lookup table for all the possible values of lmute,1 and  lmute,2 and calculate the muting index i using Equation 1 for configured values of K11, K21, and K22. This lookup table is used by UE to get the muting symbol index  lmute,1 and  lmute,2 from received muting index i.

The table below contains all the possible combinations of  lmute,1 and  lmute,2 and corresponding muting index i for K11= 10, K21=3, and K22=7.

| lmute,1 | lmute,2 | Muting Index (i) | lmute,1 | lmute,2 | Muting Index (i) |
| --- | --- | --- | --- | --- | --- |
| No Muting |  | 0 | 5 | 0 | 16 |
| 0 | - | 1 | 6 | 0 | 17 |
| 1 | - | 2 | 0 | 1 | 18 |
| 2 | - | 3 | 1 | 1 | 19 |
| 3 | - | 4 | 2 | 1 | 20 |
| 4 | - | 5 | 3 | 1 | 21 |
| 5 | - | 6 | 4 | 1 | 22 |
| 6 | - | 7 | 5 | 1 | 23 |
| 7 | - | 8 | 6 | 1 | 24 |
| 8 | - | 9 | 0 | 2 | 25 |
| 9 | - | 10 | 1 | 2 | 26 |
| 0 | 0 | 11 | 2 | 2 | 27 |
| 1 | 0 | 12 | 3 | 2 | 28 |
| 2 | 0 | 13 | 4 | 2 | 29 |
| 3 | 0 | 14 | 5 | 2 | 30 |
| 4 | 0 | 15 | 6 | 2 | 31 |

- Step7: Mapping of muting symbol index  lmute,1 and  lmute,2  back to PUSCH muting symbol number of a slot at UE

FFS: - Values of K11, K21 and K22 for resource indication.

- Scheme-2: Determination of PUSCH resource muting and enable/disable using existing DCI field dynamically or using RRC message semi-statically

To measure the gNB-gNB CLI due to the PDCCH channel, we need to mute one OFDM symbol from the first three OFDM symbols of the slot and the second symbol from the last 11 OFDM symbols of the slot to measure the gNB-gNB CLI due to the PDSCH channel.

In this scheme, we propose to enable/disable the PUSCH resource muting in two ways: 1) Dynamic indication using DCI format 0_1 and 0_2 and 2) semi-static indication using RRC message.

- Dynamic indication of muting flag

The dynamic indication of PUSCH resource muting is done by sending the muting flag to UEs over downlink control channel. The provisioning of one bit to transmit a muting flag by reusing existing DCI field. For example, UL/SUL indicator bit can be reused for PUSCH resource muting enable/disable as follows.

For SBFD carrier

- 1 bit for PUSCH resource muting enable/disable in SBFD slots, where 0 indicates muting disabled and 1 indicates muting enabled.

Otherwise

- 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission.

1 bit for UL/SUL indication.

- Semi-static indication of muting flag

In this case, the Muting flag is configured using the RRC messaging.

The UE and gNB determine the symbol in the slot that needs to be muted based on the PUSCH resource configuration using a defined set of rules. When muting is enabled, the muted symbol number of a slot is determined as follows,

First muting symbol number calculation

- The first available symbol out of the first 3 symbols of the slot is muted to measure the gNB-gNB CLI due to the PDCCH channel. If a symbol is not available for muting in the first three symbols of a slot, then no symbol is muted to measure the gNB-gNB CLI due to the PDCCH channel.

Second muting symbol number calculation

- To measure the gNB-gNB CLI due to the PDSCH channel, the first available symbol out of the last 11 symbols of the slot is muted. If a symbol is not available for muting in the last 11 symbols of the slot, then no symbol is muted to measure the gNB-gNB CLI due to the PDSCH channel. This second muted PUSCH symbol is always muted close to the first PUSCH DMRS symbol, which will help in the fast UL receiver processing and save the UL receiver buffer.

Note: “Available symbols for muting” are the symbols that are allocated to PUSCH data transmission.

Figure 5 Muted symbols for PUSCH mapping type A, where PUSCH Resource allocation for example (a) are S = 0 and L = 14 and for example (b) are S = 1 and L = 13 with DMRS length two

.

Figure 6 Muted symbols for PUSCH mapping type B, The PUSCH Resource allocation for example (a) are S = 3 and L = 11 and for example (b) are S = 6 and L = 8.

Proposal 3: Consider the following scheme for the determination of UL PUSCH resource muting and to enable/disable the muting,

Consider the following methods to enable/disable the UL resource muting,

- Dynamic indication of muting flag by reusing a field of DCI to enable/disable the PUSCH resource muting.

FFS: Provisioning of one bit to transmit a muting flag by reusing existing DCI field.

- Semi-static indication of the muting flag using RRC message.

When muting is enabled using the control channel, the index of the muted symbol number in the slot is determined at gNB and UE as follows

First muting symbol number calculation

- The first available symbol out of the first 3 symbols of the slot is muted to measure the gNB-gNB CLI due to the PDCCH channel. If a symbol is not available for muting in the first three symbols of the slot, then no symbol is muted to measure the gNB-gNB CLI due to the PDCCH channel.

Second muting symbol number calculation

- To measure the gNB-gNB CLI due to the PDSCH channel, the first available symbol out of the last 11 symbols of the slot is muted. If a symbol is not available for muting in the last 11 symbols of the slot, then no symbol is muted to measure the gNB-gNB CLI due to the PDSCH channel. This second muted PUSCH symbol is always muted close to the first PUSCH DMRS symbol, which will help in the fast UL receiver processing and save the UL receiver buffer.

Note: “Available symbols for muting” are the symbols that are allocated to PUSCH data transmission.

Proposal 4: For the scenarios where DCI format 0_0 does not have enough padding bits and for DCI format 0_1 and 0_2, we propose to use the Scheme-2 for determination of PUSCH resource muting.

# gNB-gNB channel measurement timing offset handling

In the SBFD system, when gNB receives the UE uplink signal along with that, gNB receives CLI from the neighbouring gNB downlink leakage, which is called gNB-gNB CLI. It has been agreed in earlier RAN1 meetings to use channel measurement-based beam nulling scheme for CLI interference suppression as one of the options. For this Channel measurement-based beam nulling scheme, aggressor gNBs need channel information between aggressor gNB and victim gNB. The channel information between the gNBs is acquired by using CSI-RS transmission by aggressor gNB or victim gNB in DL SBFD slot, and victim gNB or aggressor gNB receives in uplink reception.

When gNB-gNB channel measurement needs to be done, the gNB A (aggressor/victim) transmits a reference signal (CSI-RS) in the SBFD slot, and gNB B (victim/aggressor) receives the signal and measures the channel for CLI suppression. As gNB A transmits this reference signal at the downlink symbol boundary and gNB B receives it after a propagation delay depending on the distance between gNB A and gNB B. But as it is an SBFD slot, gNB B also receives the UL signal from its serving UEs in the UL band, which is received after the TTA (Tc.NTA-Offset + Tprop) time configured by the serving gNB B. Hence, there is a time difference between the signal received by gNB B from gNB A for channel measurement and the UL signal received from the serving UEs, as shown in Figure below.

Figure 7  Signal transmitted by the UE with time advanced by TTA (in sec).

Figure 8 Reference signal transmitted by gNB A with zero time advance.

Figure 9  Signal received by gNB B from serving UE and reference signal of gNB A

As per TS 38.133, the value of Tc.NTA-Offset is 13 us (microseconds) for FR1 with no coexistence with the LTE scenario. The propagation delay for serving UEs is always greater than 0. The propagation delay for gNB-gNB reference signal transmission (Tprop, gnb) is ~1.67 us (for ISD = 500m). Hence the delay between serving UE received signal and gNB reference signal received signal is ~13 + 1.67 us, which is more than the Cyclic prefix of the OFDM symbol. Hence, it is not possible to receive the signal from serving UEs and other gNBs reference signals with a single FFT window.

To receive signal from serving UEs accurately and measure the channel accurately of other gNBs using reference signal. We can use two separate FFT windows at the required time instances. As we can see in Figure 9, FFT window-1(default FFT window for UL signal reception) is used to receive the signal of serving UEs. FFT window-2 (additional FFT window for accurate channel measurement considering the start of other gNBs received signal at “n+1 OFDM symbol start boundary + Tprop, gnb”) is used to receive the signal of other gNBs reference signal for channel measurement.

Observation 6: The time difference between UL signal received from serving UEs and signal received from other gNBs for channel measurement can be handled using two FFT operations at desired time instances.

Proposal 5: The time difference between the UL signal received from serving UEs and the signal received from other gNBs for channel measurement can be handled by implementation. Hence, no discussion is required for timing offset handling.

# Discussion on Inter/Intra operator Adjacent Channel CLI

As per Rel-18 SI, the coexistence evaluation from RAN-4 was conducted considering the different scenarios as in the below table for the FR-1 band.

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro |
| 2 | FR1 | Urban Hotspot | Urban Hotspot |
| 3 | FR1 | Indoor | Indoor |
| 4 | FR1 | Urban Macro | Micro |
| 5 | FR1 | Micro | Micro |

For the performance evaluation, RAN4 had only considered the case of {D, U} as an SBFD configuration as it is comparable to the {D, U, D} SBFD configuration. Throughput loss due to the adjacent channel CLI was studied, and Fig-1 through Fig-3 are box plots that represent the summary of a median throughput loss from performance reports of adjacent channel coexistence study [4]. Plots include the minimum, first (lower) quartile, median, third (upper) quartile, and maximum of the median throughput losses published in the coexistence study.

Figure 10 Adjacent Channel CLI median throughput loss (%) summary for Scenario-1(FR-1) concluded in Rel-18 CLI

Figure 11 SBFD UL median throughput loss summary at cell edge and cell centre for Scenario-1

Figure 12 SBFD UL median throughput loss summary for Cell centre (Left) & Cell edge (Right) users for varying ACIR for Scenario-1

The following observations were made from the performance results with respect to both legacy TDD and SBFD as victims.

- The cell edge throughput degradation is worse than the average throughput degradation.

- The throughput degradation is due to the inter-BS ACI introduced by TDD-UL over SBFD, which increases as grid-shift (BS-to-BS distance) decreases in scenario 1.

- SBFD BS antenna configuration slightly impacts the throughput degradation.

- Major SBFD UL throughput degradation is observed cell edge uplink.

- For higher BS Tx power and lower grid-shifts the degradation is increased for cell edge throughput and average throughput.

- No throughput degradation is observed on the SBFD DL for both average throughput and cell edge throughput, respectively, for different BS Tx powers (ranging from 46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and SBFD BS antenna configurations.

Observation 7: RAN4 Rel-18 SI concluded that the Adjacent Channel CLI significantly impacts the Uplink performance for both SBFD and Legacy TDD UL.

Observation 8: The Uplink throughput performance loss becomes a strong function of grid-shift of adjacent channel gNB, especially for cell-centre UE’s, severely constraining the deployment capability of multiple operators in macro gNB deployments.

RAN4 had considered the case of {D, U} as an SBFD configuration instead of {D, U, D} SBFD configuration.

But when you consider {D, U, D} with {40MHz,20MHz,40MHz} bandwidth, there is an additional protection of 40MHz for the leakage from the adjacent channel CLI.

The study done as part of TR 38.858 [2] discussed the feasibility of analog filtering options for self-interference and co-channel interference due to co-site/co-sector in sections 9.3.1 and 9.3.2, respectively. It was dropped due to design complexity, other insertion loss due to a narrow roll-off from a few kHz to a few MHz, and limitations in supporting multi-carrier. However, this feasibility was not studied from an adjacent channel blocker perspective, considering the benefit of 40MHz filter roll-off due to {D, U, D} configuration for SBFD.

For the SBFD symbol represented by {D, U, D}, Adjacent channel gNB-gNB CLI is seen in the UL sub-band of the SBFD symbol due to DL slots of legacy TDD from neighbouring gNBs overlapping with SBFD symbols.

For the Legacy TDD symbols, the Adjacent channel gNB-gNB CLI is seen in UL symbols in the Flexible slot overlapping with the SBFD slot of neighbouring gNB.

Since the configurations considered for the adjacent channel CLI for throughput analysis are different from what has been studied in TR 38.858 [2]  there is a need for further study on network throughput impact due to adjacent channel co-existence.

Proposal 6: Further study the impact of adjacent channels on network throughput with the {D, U, D} SBFD configurations.

## Applicability of co-channel gNB-gNB CLI Schemes for intra-operator adjacent channel CLI

The following schemes are discussed for the co-channel CLI handling.

- Spatial domain coordination scheme

  - scheme#1: Beam nulling based on steering vector between gNB-gNB.

  - scheme#2: Beam nulling based on gNB-gNB channel measurement.

- UL resource muting-based-scheme

  - Option 1: gNB-gNB covariance matrix estimation based on transparent UL resource muting.

  - Option 2: gNB-gNB covariance matrix estimation based on non-transparent UL resource muting.

Channel measurement based gNB-gNB increases the complexity since this requires the victim gNB to measure the channel for the CSI-RS/NCD-SSB beams from the adjacent channel. Instead, the steering vector-based method allows the victim gNB to measure the leakage from adjacent channel gNB in the ZP-CSI-RS or CSI-IM. The measured interference in the ZP-CSI-RS or CSI-IM is used to calculate steering vector weights for the aggressor gNB, which is causing interference. The residual leakage from the sidelobes can be measured with non-transparent muting for interference cancellation.

For adjacent channel intra-operator gNB’s can co-ordinate over the Xn interface to share the information on orthogonal resource sets and reference signals for CLI measurements, DL/UL muting patterns and their signalling for rate-matching at UE or gNB’s, and Interference measurements on muted symbols.

Proposal 7: Co-channel gNB-gNB CLI handling schemes are applicable for intra-operator adjacent channel CLI.

## Applicability of co-channel gNB-gNB CLI Schemes for inter-operator adjacent channel CLI

In the context of TDD configurations, when one operator switches from a specific TDD configuration (e.g., DDDSU) to a different supported SBFD configuration (e.g., DXXXU), the uplink portion of the SBFD symbols will experience adjacent channel interference from the TDD ‘D’ slots and the DL Symbols from Special Slot ‘S’ of another TDD operator, which overlaps with the SBFD symbols ‘X’. This interference includes the adjacent channel blocker as well as co-channel interference caused by leakage due to adjacent channel leakage of the TDD downlink slots and symbols.

To address this interference, some of the schemes discussed under co-channel gNB-gNB CLI schemes require inter-gNB coordination and information exchange. However, based on the new WID [5] RAN3 (“RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling”), there will be no specified enhancements to network signalling to support inter-operator coordination for CLI handling. As a result, co-channel gNB-gNB CLI handling schemes as discussed in section 2.1 cannot be applied for inter-operator CLI handling.

Proposal 8: Schemes for inter-operator adjacent channel CLI handling requires further study due to absence of signalling between inter-operator gNB’s.

# Conclusion

In this contribution, we discuss about CLI handling with the following proposals and observations.

Observation 1: In the case of SBFD, The DCI format 0_0 will have at least 5 padding bits when FDRA bits of DCI format 0_0 is less than or equal to FDRA bits of DCI format 1_0.

Observation 2: The PUSCH resource muting parameter values are decided based on the available padding bits in DCI format 0_0.

Observation 3: The PUSCH muting parameters K11, K21, and K22 decide the available set of symbols for muting with the exclusion of reference symbols.

Observation 4: The following table captures examples of the possible values of muting parameters considering the maximum available padding bits of 5 in DCI format 0_0.

| K11 | K21 | K22 | Number of muting bits required for PUSCH Resource Muting Indication |
| --- | --- | --- | --- |
| 10 | 3 | 7 | 5 |
| 6 | 3 | 3 | 4 |
| 7 | 2 | 4 | 4 |
| 6 | 1 | 1 | 3 |
| 3 | 2 | 2 | 3 |
| 4 | 1 | 1 | 3 |
| 2 | 1 | 1 | 2 |

Observation 5: The PUSCH resource muting index i is calculated using parameters K11, K21, K22 and configured muting symbol index  lmute,1 and  lmute,2.

Proposal 1: For sending the resource muting indication bits to UE over DCI, we propose to use the available padding bits (≥5) in DCI format 0_0 when FDRA bits of DCI format 0_0 less than or equal to FDRA bits of DCI format 1_0.

Proposal 2: For PUSCH muting symbol configuration and indication, we propose the following steps,

- Step1: Provisioning of muting indication bits in DCI formats for transmitting muting index to its serving UEs

For sending the resource muting indication bits to UE over DCI, we propose to use the available padding bits (≥5) in DCI format 0_0 when FDRA bits of DCI format 0_0 less than or equals to FDRA bits of DCI format 1_0.

- Step 2: Defining and configuring parameters required for PUSCH muting symbol indication and deciding the flexibility of PUSCH muting symbol position.

We propose to configure these parameters, K11, K21, and K22 to UE using two alternative ways: 1) the values of the parameters are configured in a UEs by gNB using RRC message or 2) the values of the parameters are fixed and preconfigured at UEs and gNBs.

Configuration of parameters K11, K21, and K22 to UE by gNB is done as follows

Alt-1: The value of parameter K11 as SBFDOneSymbolMutingParm_r19, K21 as SBFDTwoSymbolMutingParm1_r19, and K22 as SBFDTwoSymbolMutingParm2_r19 are RRC Configured.

Alt-2: The values of parameter K11, K21, and K22 are fixed for network.

The number of bits required to transmit the muting index is calculated as follows,

mutingbits=ceil(log2 K22*K21+K11+1).

- Step3: Configuration of muting symbol index  lmute,1 and  lmute,2 from the available set of symbols in a slot at gNB

- Step4: UL resource muting index i calculation for SBFD slot at gNB, which is sent over DCI

i=0,  if mutng is disabledlmute,1+1,      when one symbol is muted in slot (0≤ lmute,1≤K11-1)lmute,2+ K22*(lmute,1+1)+1+K11-K22, when two symbols are muted  per slot (0≤ lmute,2≤K22-1, 0≤ lmute,1≤K21-1)

- Step5: Transmission of muting index i using DCI format 0_0

- Step6: Determination of muting symbol indexes  lmute,1 and  lmute,2 using the muting index i received on the control channel at UE

As there is a unique one-to-one mapping between the muting index i and, the muting symbol index  lmute,1 and  lmute,2. Hence UEs form a lookup table for all the possible values of lmute,1 and  lmute,2 and calculate the muting index i using Equation 1 for configured values of K11, K21, and K22. This lookup table is used by UE to get the muting symbol index  lmute,1 and  lmute,2 from received muting index i.

The table below contains all the possible combinations of  lmute,1 and  lmute,2 and corresponding muting index i for K11= 10, K21=3, and K22=7.

| lmute,1 | lmute,2 | Muting Index (i) | lmute,1 | lmute,2 | Muting Index (i) |
| --- | --- | --- | --- | --- | --- |
| No Muting |  | 0 | 5 | 0 | 16 |
| 0 | - | 1 | 6 | 0 | 17 |
| 1 | - | 2 | 0 | 1 | 18 |
| 2 | - | 3 | 1 | 1 | 19 |
| 3 | - | 4 | 2 | 1 | 20 |
| 4 | - | 5 | 3 | 1 | 21 |
| 5 | - | 6 | 4 | 1 | 22 |
| 6 | - | 7 | 5 | 1 | 23 |
| 7 | - | 8 | 6 | 1 | 24 |
| 8 | - | 9 | 0 | 2 | 25 |
| 9 | - | 10 | 1 | 2 | 26 |
| 0 | 0 | 11 | 2 | 2 | 27 |
| 1 | 0 | 12 | 3 | 2 | 28 |
| 2 | 0 | 13 | 4 | 2 | 29 |
| 3 | 0 | 14 | 5 | 2 | 30 |
| 4 | 0 | 15 | 6 | 2 | 31 |

- Step7: Mapping of muting symbol index  lmute,1 and  lmute,2  back to PUSCH muting symbol number of a slot at UE

FFS: - Values of K11, K21 and K22 for resource indication.

Proposal 3: Consider the following scheme for the determination of UL PUSCH resource muting and to enable/disable the muting,

Consider the following methods to enable/disable the UL resource muting,

- Dynamic indication of muting flag by reusing a field of DCI to enable/disable the PUSCH resource muting.

FFS: Provisioning of one bit to transmit a muting flag by reusing existing DCI field.

- Semi-static indication of the muting flag using RRC message.

When muting is enabled using the control channel, the index of the muted symbol number in the slot is determined at gNB and UE as follows

First muting symbol number calculation

- The first available symbol out of the first 3 symbols of the slot is muted to measure the gNB-gNB CLI due to the PDCCH channel. If a symbol is not available for muting in the first three symbols of the slot, then no symbol is muted to measure the gNB-gNB CLI due to the PDCCH channel.

Second muting symbol number calculation

- To measure the gNB-gNB CLI due to the PDSCH channel, the first available symbol out of the last 11 symbols of the slot is muted. If a symbol is not available for muting in the last 11 symbols of the slot, then no symbol is muted to measure the gNB-gNB CLI due to the PDSCH channel. This second muted PUSCH symbol is always muted close to the first PUSCH DMRS symbol, which will help in the fast UL receiver processing and save the UL receiver buffer.

Note: “Available symbols for muting” are the symbols that are allocated to PUSCH data transmission.

Proposal 4: For the scenarios where DCI format 0_0 does not have enough padding bits and for DCI format 0_1 and 0_2, we propose to use the Scheme-2 for determination of PUSCH resource muting.

Observation 6: The time difference between UL signal received from serving UEs and signal received from other gNBs for channel measurement can be handled using two FFT operations at desired time instances.

Proposal 5: The time difference between the UL signal received from serving UEs and the signal received from other gNBs for channel measurement can be handled by an implementation. Hence, no discussions are required for timing offset handling.

Observation 7: RAN4 Rel-18 SI concluded that the Adjacent Channel CLI significantly impacts the Uplink performance for both SBFD and Legacy TDD UL.

Observation 8: The Uplink throughput performance loss becomes a strong function of grid-shift of adjacent channel gNB, especially for cell-centre UE’s, severely constraining the deployment capability of multiple operators in macro gNB deployments.

Proposal 6: Further study the impact of adjacent channels on network throughput with the {D, U, D} SBFD configurations.

Proposal 7: Co-channel gNB-gNB CLI handling schemes are applicable for intra-operator adjacent channel CLI.

Proposal 8: Schemes for inter-operator adjacent channel CLI handling requires further study due to absence of signalling between inter-operator gNB’s.

# Reference

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- RP-240172, Evolution of NR duplex operation: Sub-band full duplex (SBFD).

- 3GPP TR 38.858: “Study on Evolution of NR Duplex Evolution”.

- R1-2403894, Discussion on gNB-gNB CLI handling, Tejas Networks Ltd.

- RP-240789, Evolution of NR duplex operation: Sub-band full duplex (SBFD).