3GPP TSG RAN WG1 #118	R1-2406005

Maastricht, NL, August 19th -August 23rd, 2024

Source: 	CMCC

Title:	Discussion on the NR-NTN uplink capacity/throughput enhancements

Agenda item:	9.11.3

Document for:	Discussion & Decision

Introduction

Rel-19 WI on Non-Terrestrial Networks (NTN) for NR Phase 3 was updated in RP-241667 with the following objectives.

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we discuss potential enhancements on OCC based uplink capacity/throughput enhancements and provide preliminary LLS simulation results.

Potential OCC schemes to enhance uplink capacity/throughput

In RAN1#117 meeting, it was concluded that the at least one of the OCC techniques will be specified for the normative phase. Three candidate techniques had been agreed to be down selected. One is the inter-slot time domain OCC with PUSCH repetition Type A with OCC length 2 or 4. The 2nd is the inter-symbol(s) time domain OCC with OCC length 2 or 4. The 3rd one is intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4.

Agreement[RAN1#117] For the normative phase, at least one of the OCC techniques will be specified: - Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4 - Inter-symbol(s) time domain OCC with OCC length 2 or 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4 - FFS Combination of OCC techniques including multiplexing of 8 UEs - FFS Use of OCC techniques with TBoMS - FFS Backward compatibility with non-Rel-19 UEs

Scheme 1: Inter-slot time-domain OCC with PUSCH repetition Type A

Since it is in the NTN scenario, due to the long propagation distance and the large pathloss fading, for even LEO-600km scenario, the uplink transmission is still coverage limited. For the small data rate transmission with payload 96bits, at least 8 repetitions should be used to fulfil the required SINR. And for the VoIP traffic, 20 repetitions would be required for the uplink link transmission with the payload of 184bits. PUSCH repetition type A will be definitely used for any data traffic transmission in uplink. It was also agreed in the RAN1#116bis meeting that, the PUSCH repetition type A would be used combined with the OCC enhancements.

Observation 1

PUSCH repetition type A is required for all data traffic transmission in uplink for NR-NTN, including both small data rate and VoIP traffic.

Then it is more straightforward to use the inter-slot time domain OCC with PUSCH repetition type A (i.e., inter-slot time-domain OCC) with different OCC lengths. Based on the PUSCH repetition Type A, the inter-slot time-domain OCC has the least specification impacts and the easiest one for implementation based on current specifications, which only requires mapping the orthogonal cover code over the REs for PUSCH within each slot.

Observation 2

The inter-slot time-domain OCC has the least specification impacts for PUSCH RE mapping and is easiest one for realization, compared with the other two OCC solutions.

Nevertheless, realistic impairments, e.g. Doppler, time variation, phase distortion, etc, may impact the orthogonality of the OCC sequence and reduce the performance, especially when the OCC operation spans over a long period. As shown in Figure 1, in the current spec, the RV of PUSCH repetition scheduled by DCI would change according to the pattern of [0,2,3,1]. It will increase the time span of OCC operation, since one OCC code should cover at least 4 slots with different RV. TBoMS repetition has even longer time span of OCC operation, since RV is updated every N slots in the pattern of [0, 2, ,3, 1], where N is number of slots used for TBS determination. CG repetition can support shorter time span of OCC operation when RV pattern is configured as [0, 3, 0, 3] or [0, 0, 0, 0]. Thus, slot-level OCC scheme needs to pay more attention realistic impairments, e.g. Doppler, time variation, phase distortion, etc.

Figure 1  Example of inter-slot time-domain OCC considering legacy RV pattern.

Observation 3

Inter-slot time-domain OCC schemes need to pay more attention on realistic impairments.

To alleviate the impact of realistic impairment, potential enhancement on RV cycling over repetitions can be further studied, as shown in Figure 2. The detailed LLS evaluation method can be found in section 3.

Figure 2  Slot-level OCC with RV cycling over repetitions.

Scheme 2: Inter-symbol(s) time domain OCC with OCC

For the inter-symbol time domain OCC, there are two ways for realization. One is to apply the OCC codes over the consecutive repeated symbols, as illustrated in Case 1 and 2 in Figure 3. It is obvious that the RE mapping would be changed according to the number of repeated symbols compared with the legacy. The other way is to realize it through the PUSCH repetition Type B as illustrated in Case 3 in Figure 3, in which the OCC would be mapped over each PUSCH Type B repetition with 7 symbols. But if the PUSCH repetition Type B is repeated with other value of symbol numbers, e.g. 6 or 8 symbols, the operation for across slot repetitions or the actual repetitions should be studied further, in which the specification impact is also not trivial. In addition, the PUSCH repetition type A is usually used for the coverage enhancement scenarios instead of repetition PUSCH type B. Then, either way of inter-symbol time domain OCC has a large specification impact.

Observation 4

The inter-symbol time domain OCC would have a large specification impact for either the realization of consecutive symbol repetitions within one slot or PUSCH repetition Type B-like operations.

Figure 3 Examples for inter-symbol time-domain OCC

Scheme 3: Intra-symbol pre-DFT-s OCC

As shown in Figure 4, intra-symbol pre-DFT-s OCC (i.e., pre-DFT OCC scheme in frequency domain, or OCC spreading before transform precoding), similar as PUCCH format 4, generates a comb-like structure in the frequency domain.

Figure 4  Illustration of physical layer processing of intra-symbol pre-DFT-s OCC.

It is obvious that if the pre-DFT-s OCC is introduced, the operation of PUSCH RE mapping should be changed. On the other hand, as a frequency domain enhancement, it is less sensitive to the channel variation as the enhancements of time-domain OCC.

But it should be further clarified the benefits when the occupied PRBs is larger than one, e.g. two PRBs, over the single PRB transmission without intra-symbol pre-DFT-s OCC. For example, as shown in Figure 5, assuming the same RE number allocated to each UE with different allocation pattern, when the two PRBs are allocated for PUSCH transmission with intra-symbol pre-DFT-s OCC with OCC length 2 (Figure 5 (a)), the performance is almost same with or even worse than single UE with single PRB PUSCH transmission (Figure 5 (b)), since intra-symbol pre-DFT-s OCC may have additional performance loss due to the inference between combs from different UEs.

Figure 5  Intra-symbol pre-DFT-s OCC with two PRBs.

Considering that two PRBs are required for VoIP traffic, most traffic with similar or even higher data rate would require larger than two PRBs’ resource allocations. Then it is questionable to support the intra-symbol pre-DFT-s OCC instead of using less frequency resources schemes without any specification impact.

Observation 5

The performance of PUSCH transmission adopting two PRBs with intra-symbol pre-DFT-s OCC with OCC length 2 has almost the similar or even worse performance than PUSCH transmission with single UE and single PRB. The latter scheme will not suffer from the inter-comb interference and can be realized without any specification efforts.

Inter-symbol OCC scheme would introduce time domain repetitions within the slot. And the intra-symbol pre-DFT-s OCC would introduce the un-occupied resources in frequence domain. Both schemes will reduce the effective resources within one PUSCH transmission slot. If no additional resources are allocated for the PUSCH, to maintain the same traffic, higher modulation levels and higher required SINR are needed, which also degrades the link level performances. Then the TBoMS, which is single TB transmission over multiple slots, should be introduced to reduce impacts of less effective resources within one slot.

Observation 6

TBoMS should be introduced to maintain the same data rate and lower the code rate for PUSCH transmission if the inter-symbol OCC or the intra-symbol pre-DFT-s OCC is introduced for multiple UE multiplexing.

Preliminary evaluation results of OCC enhancements

In the previous meeting, the evaluation methodologies and evaluation assumptions have been agreed. Based on the evaluation assumptions, the performance between single UE and MU with OCC are compared. In Table 1, different schemes of redundancy versions are compared trying to reuse the legacy RV cycling mechanisms. And in Table 2 and 3, a fixed redundancy version is assumed with an updated receiver mechanism focusing on the performance difference between slot level OCC and SU with same repetitions.

Table 1: Preliminary evaluation results of the SINR losses of OCC enhancements

| Cases | MU/SU | Repetitions | Redundancy Version #1 | SNR(dB) 10% iBLER | SINR losses (dB) of OCC multiplexing compared with SU PUSCH repetitions | Redundancy Version #2 | SNR(dB) 10% iBLER | Redundancy Version #3 | SNR(dB) 10% iBLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | RV = {0, 0, 0, 0, 0, 0, 0, 0} | -7.3 |  |  |  |  |  |
| 2 | 2 | 8 | RV = {0, 0, 0, 0, 0, 0, 0, 0} | -6.84 | 0.46 | Case 10: RV = {0,0,2,2,3,3,1,1} | -6.45 | Case 12: RV = {0,2,3,1}*2 | -2.15 |
| 3 | 4 | 8 | RV = {0, 0, 0, 0, 0, 0, 0, 0} | -5.85 | 1.45 |  |  |  |  |
| 4 | 1 | 16 | RV = {0}*16 | -9.18 |  |  |  |  |  |
| 5 | 2 | 16 | RV = {0}*16 | -8.89 | 0.29 |  |  |  |  |
| 6 | 4 | 16 | RV = {0}*16 | -8.33 | 0.85 | Case 11: RV = {0}*4, {2}*4, {3}*4, {1}*4, | -6.85 | Case 13 RV = {0,2,3,1}*4 | / |
| 7 | 1 | 32 | RV = {0}*32 | -11.12 |  |  |  |  |  |
| 8 | 2 | 32 | RV = {0}*32 | -11.01 | 0.11 |  |  |  |  |
| 9 | 4 | 32 | RV = {0}*32 | -10.72 | 0.4 |  |  |  |  |

In Table 1, three types of redundancy versions are considered in the link level evaluations. The examples are illustrated in the figure below.

Figure 6 An example of three types of redundance versions for the OCC performance evaluations

Redundancy version of all zeros, as Redundancy version #1, are considered in the Case 1~9 as baseline for the simplicity. RV#3 is aligned with the current specification for the RV cycling between different repetitions for the dynamic grant PUSCH repetitions. And the RV#2 is an updated version based on RV#3, in which the same RV are located closely. And the redundancy versions are cycled between multiple repetitions, e.g. 4 repetitions as in Figure 6.

Compared between Case 1 and 2, the performance loss on required SINR due to MU=2 OCC is 0.46dB. In Case 10, where the RV#2 is used, the performance loss on the required SINR is similar as Case 2. But in the Case 12, in which the legacy RV cycling is reused, the performance loss is beyond 5dB.

Compared the Case 6, 11 and 12, the performance loss of the required SINR is similar between Case 6 and Case 11, in which RV#1 and RV#2 are used. But in the Case 13 with RV#3, the legacy RV cycling mechanism, it cannot support the demodulation of MU=4 with PUSCH OCC.

Observation 7

The legacy redundancy version cycling as defined in current specification for DG PUSCH repetition may not be workable as the repetition number increases. The enhancements to the RV cycling for PUSCH repetition when slot level OCC multiplexing is adopted.

Proposal 1

The enhancement to the redundancy version cycling of the PUSCH repetition should be considered, when the slot-level PUSCH OCC multiplexing is adopted.

In Table 2 and 3, only redundancy version #1 is considered. In Table 2, the Low data rate is simulated for the comparison between the required SINR of SU with repetitions and the MU with OCC multiplexing. The repetition number is considered from 1 to 32. And the maximum supported OCC number is 4.

Table 2: Preliminary evaluation results of the SINR losses of OCC enhancements – Low data rate

| Cases | MU/SU | Repetitions | Redundancy Version #1 | SNR(dB) 10% iBLER | SINR loss (dB) of OCC multiplexing compared with SU PUSCH repetitions |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | RV = {0, 0, 0, 0,0,0,0,0} | -7.3 |  |
| 2 | 2 | 8 | RV = {0, 0, 0, 0,0,0,0,0} | -6.84 | 0.46 |
| 3 | 4 | 8 | RV = {0, 0, 0, 0,0,0,0,0} | -5.85 | 1.45 |
| 4 | 1 | 16 | RV = {0}*16 | -9.18 |  |
| 5 | 2 | 16 | RV = {0}*16 | -8.89 | 0.29 |
| 6 | 4 | 16 | RV = {0}*16 | -8.33 | 0.85 |
| 7 | 1 | 32 | RV = {0}*32 | -11.12 |  |
| 8 | 2 | 32 | RV = {0}*32 | -11.01 | 0.11 |
| 9 | 4 | 32 | RV = {0}*32 | -10.72 | 0.4 |

For the case of 8 PUSCH repetitions, the performance loss of MU=2 and 4 are limited below 2dB, both of which can also be supported considering the CNR limitation for uplink in LEO-600km scenarios. For the case with 16 and 32 repetitions, the required SINR can all be supported by the CNR of LEO-600km in uplink. And with the increase of the repetition, the performance loss of the SINR decrease.

Observation 8

For the traffic of small data rate, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below,

- With PUSCH repetition =8

  - The performance loss of OCC =2 compared with SU equals to 0.46.

  - The performance loss of OCC =4 compared with SU equals to 1.45.

- With PUSCH repetition =16

  - The performance loss of OCC =2 compared with SU equals to 0.29.

  - The performance loss of OCC =4 compared with SU equals to 0.85.

- With PUSCH repetition =32

  - The performance loss of OCC =2 compared with SU equals to 0.11.

  - The performance loss of OCC =4 compared with SU equals to 0.4.

In Table 3, the VoIP traffic is simulated for the comparison between the required SINR of SU with repetitions and the MU with OCC multiplexing.

Table 3: Preliminary evaluation results of the SINR losses of OCC enhancements – VoIP

| Cases | MU/SU | Repetitions | Redundancy Version #1 | SNR(dB) 2% iBLER | SINR losses (dB) of OCC multiplexing compared with SU PUSCH repetitions |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 8 | RV = {0, 0, 0, 0, 0, 0, 0, 0} | -2.56 |  |
| 2 | 2 | 8 | RV = {0, 0, 0, 0, 0, 0, 0, 0} | -1.74 | 0.82 |
| 3 | 4 | 8 | RV = {0, 0, 0, 0, 0, 0, 0, 0} | -0.1 | 2.46 |
| 4 | 1 | 16 | RV = {0}*16 | -4.81 |  |
| 5 | 2 | 16 | RV = {0}*16 | -4.34 | 0.47 |
| 6 | 4 | 16 | RV = {0}*16 | -3.55 | 1.26 |
| 7 | 1 | 20 | RV = {0}*20 | -5.45 |  |
| 8 | 2 | 20 | RV = {0}*20 | -5.18 | 0.27 |
| 9 | 4 | 20 | RV = {0}*20 | -4.13 | 1.32 |

In the case of VoIP, it can be observed that with 8 repetitions, the performance loss of MU with OCC 2 and 4 is about 0.82dB and 2.46dB. For the case of repetition 16, the performance loss of OCC 2, 4 is 0.47dB, 1.26 dB. In the case of repetition of 20, the performance loss of OCC 2, 4 is 0.27dB, 1.32dB.

Observation 9

For the traffic of VoIP, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below,

- With PUSCH repetition =16

  - The performance loss of OCC =2 compared with SU equals to 0.47.

  - The performance loss of OCC =4 compared with SU equals to 1.26.

- With PUSCH repetition =32

  - The performance loss of OCC =2 compared with SU equals to 0.27.

  - The performance loss of OCC =4 compared with SU equals to 1.32.

In Table 4 and 5, only redundancy version #1 is considered. In Table 4 and Table 5, the required SINR under each OCC enhancement for VoIP and small data rate are listed below.

Table 4 The required SINR for VoIP traffic

|  | With/without UE multiplexing | Repetitions /occupied slots | TBoMS | repetitions of TBoMS | SNR(dB)(2% iBLER) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Inter-slot without TBOMS | Inter-symbol with TBOMS | Intra-symbol with TBOMS |
| Case1 | Single UE without multiplexing | 16 | 1 | 16 | -4.81 |  |  |
| Case2 |  | 20 | 1 | 20 | -5.45 |  |  |
| Case3 | 2 UEs /OCC length 2 | 16 | 2 | 8 | -4.34 | -4.78 | -4.79 |
| Case4 |  | 20 | 2 | 10 | -5.18 | -5.27 | -5.39 |
| Case5 | 4 UEs /OCC length 4 | 16 | 4 | 4 | -3.55 | -4.21 | -4.52 |
| Case6 |  | 20 | 4 | 5 | -4.13 | -4.74 | -5.2 |

* MCS level = 11

* RV 0 are used for all repetitions

Table 5 The required SINR for small data rate

|  |  |  |  |  | SNR(dB)(10% iBLER) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | With/without UE multiplexing | Repetitions /occupied slots | TBoMS | repetitions of TBoMS | Inter-slot without TBOMS | Inter-symbol with TBOMS | Intra-symbol with TBOMS |
| Case1 | Single UE without multiplexing | 8 | 1 | 8 | -7.3 |  |  |
| Case2 |  | 16 | 1 | 16 | -9.18 |  |  |
| Case3 |  | 32 | 1 | 32 | -11.12 |  |  |
| Case4 | 2 UEs /OCC length 2 | 8 | 2 | 4 | -6.84 | -6.97 | -6.99 |
| Case5 |  | 16 | 2 | 8 | -8.89 | -9.12 | -9.14 |
| Case6 |  | 32 | 2 | 16 | -11.01 | -11.05 | -11.09 |
| Case7 | 4 UEs /OCC length 4 | 8 | 4 | 2 | -5.85 | -6.58 | -6.85 |
| Case8 |  | 16 | 4 | 4 | -8.33 | -8.61 | -8.89 |
| Case9 |  | 32 | 4 | 8 | -10.72 | -10.7 | -10.95 |

* MCS level = 8

* RV 0 are used for all repetitions

Based on the evaluation results of VoIP traffic, the performance loss due to introducing multiple UE multiplexing through OCC is marginal between single UE and multiple UEs with OCC length 2. For the case of inter-slot OCC mechanism, the performance loss is less than 0.5dB for 16 repetitions, and less than 0.3dB for 20 repetitions. When OCC length is two, the differences between inter-slot OCC, inter-symbol OCC with TBoMS and the intra-symbol OCC with TBoMS are less than 0.45dB for 16 repetitions, 0.21dB for 20 repetitions.

For the case of small data rate with payload of 96bit, the performance loss compared with single UE transmission and performance margin between different solutions are even smaller. The performance loss of inter-slot OCC with length 2 relative to single UE repetition with repetition number {8,16,32} is {0.46, 0.29, 0.11}dB. Along with the increase of the repetition number, the performance loss decreases. The performance margin between inter-slot OCC with length 2 relative to intra-symbol with TBoMS with repetition {8,16,32} are {0.15, 0.25, 0.08}. The performance margin between inter-slot OCC and intra-symbol OCC is even smaller along with the increase of the repetition number. In the case of 4 UE multiplexing with OCC length 4, the performance loss is larger than that of 2 UE multiplexing with OCC length 2. But the trend is same for both performance loss compared with single UE transmission and the performance margin between different solutions.

Observation 10

The performance difference between different solutions, including inter-slot OCC, inter-symbol OCC and intra-symbol OCC is marginal when a relative high repetition number is applied.

Though the scheme of inter-slot OCC does not have the best performance between the 3 candidate solutions, but it has less impact to the current PUSCH RE mapping which is more compatible with current implementation. And the inter-slot OCC does not have the dependence on the application of TBoMS as inter-symbol OCC and intra-symbol OCC. Then considering both the performance and the specification impact, the inter-slot OCC scheme is preferred for the normative phase.

Proposal 2

Considering both the performance and the specification impact, the inter-slot OCC scheme is preferred for the normative phase.

Other issues

OCC indication/configuration

In our view, OCC length and pattern can be configured via RRC signalling. Nevertheless, DMRS port indication in DCI field can be reused to additional indicate OCC index to reduce signalling overhead, since the functionality of both DMRS port indication and OOC indication are similar to support UE multiplex.

Proposal 3

Support reusing DMRS port indication in DCI field to additional indicate OCC index to reduce signalling overhead.

UCI multiplexing rules for PUSCH with OCC

As captured in TS 38.213, if a PUSCH repetition occasion is overlapped with a PUCCH with UCI, UE may multiplex the UCI in the overlapped PUSCH repetition is some cases. And in other cases (e.g., if overlapped with a PUCCH repetition), UE may transmit the PUCCH and does not transmit the overlapped PUSCH repetition occasion. The detailed description in the specification is captured in the Annex II.

As shown inFigure 7, if the PUCCH repetitions are overlapped with the PUSCH repetitions with OCC, the PUSCH repetition, e.g. slot n+1 as in UE 1, may be dropped. And if a UCI would be reported in the slot n+2 in UE 2, the UCI should be multiplexed in the PUSCH repetition in that slot. Both the dropping of the slot of UE1 and the UCI multiplexing of UE2 will destroy the orthogonality of OCC enhanced PUSCH repetition. Since the content of PUSCH repetition would be different from other PUSCH repetitions.

Figure 7. UCI multiplexing issues for PUSCH with OCC.

Observation 11

Based on current specifications, the UCI multiplexing with PUSCH repetition and the dropping of PUSCH repetition happens, which will break the orthogonality of the OCC enhanced PUSCH repetitions.

Based on the discussion above, potential enhancement for the case of PUSCH repetition dropping and the UCI multiplexing can be considered when the time domain-based OCC enhancements are applied.

Proposal 4

Potential enhancements for the cases of PUSCH repetition dropping and the UCI multiplexing can be considered when the time domain-based OCC enhancements are applied.

Frequency hopping

The intention of frequency hopping is to use the frequency diversity to counter the deep fading in the frequency domain. But considering the NTN working scenario is almost the LOS scenario. And based on the simulation assumption of very low delay spreads, the diversity gain in the frequency domain would be very low. On the other side, the performance of OCC multiplexing is very sensitive to the change of channel response and the channel estimation to keep the orthogonality between multiplexed UEs. But the frequency hopping would change the location of PRBs, which induce different channel response in the frequency domain. And the change of frequency location would also break the phase continuity if DMRS bundling or the joint channel estimation is used, which also reduce the performance of channel estimations. Then it should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain.

Proposal 5

It should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain.

Combination of multiple OCC schemes

It was also discussed whether the combination of candidate solutions should be supported for OCC length 8. From our understanding, introducing either inter-symbol OCC or intra-symbol OCC schemes will have impact to the legacy PUSCH RE mapping. The specification impact will not be reduced when considering any combination of inter-slot OCC with either inter-symbol or intra-symbol OCC.

On the other side, considering there is no need to introduce multiple solutions for single enhancements, only one solution should be selected. RAN plenary can ask RAN1 to focusing on down selection to only one solution from the proposed 3 solutions. And if one combination of the candidate solutions is selected, only one UE capability to support the combined solutions should be supported. Namely, the OCC enhancement though which may be a combination of, for example, inter-slot OCC and intra-symbol OCC solution, should be supported a unified capability, without further differentiation of the sub-capabilities supporting either inter-slot OCC or intra-symbol OCC.

Proposal 6

Only one solution should be down-selected from the current candidate solutions.

Proposal 7

If a combined solution with multiple candidate solutions is supported, only one UE capability is defined or supported without differentiation of any sub-capability or sub-solutions.

Conclusions

In this contribution, we discuss potential enhancements on OCC based uplink capacity/throughput enhancement and provide preliminary LLS simulation results. The observations and proposals are listed below.

Observation 1

PUSCH repetition type A is required for all data traffic transmission in uplink for NR-NTN, including both small data rate and VoIP traffic.

Observation 2

The inter-slot time-domain OCC has the least specification impacts for PUSCH RE mapping and is easiest one for realization, compared with the other two OCC solutions.

Observation 3

Inter-slot time-domain OCC schemes need to pay more attention on realistic impairments.

Observation 4

The inter-symbol time domain OCC would have a large specification impact for either the realization of consecutive symbol repetitions within one slot or PUSCH repetition Type B-like operations.

Observation 5

The performance of PUSCH transmission adopting two PRBs with intra-symbol pre-DFT-s OCC with OCC length 2 has almost the similar or even worse performance than PUSCH transmission with single UE and single PRB. The latter scheme will not suffer from the inter-comb interference and can be realized without any specification efforts.

Observation 6

TBoMS should be introduced to maintain the same data rate and lower the code rate for PUSCH transmission if the inter-symbol OCC or the intra-symbol pre-DFT-s OCC is introduced for multiple UE multiplexing.

Observation 7

The legacy redundancy version cycling as defined in current specification for DG PUSCH repetition may not be workable as the repetition number increases. The enhancements to the RV cycling for PUSCH repetition when slot level OCC multiplexing is adopted.

Observation 8

For the traffic of small data rate, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below,

- With PUSCH repetition =8

  - The performance loss of OCC =2 compared with SU equals to 0.46.

  - The performance loss of OCC =4 compared with SU equals to 1.45.

- With PUSCH repetition =16

  - The performance loss of OCC =2 compared with SU equals to 0.29.

  - The performance loss of OCC =4 compared with SU equals to 0.85.

- With PUSCH repetition =32

  - The performance loss of OCC =2 compared with SU equals to 0.11.

  - The performance loss of OCC =4 compared with SU equals to 0.4.

Observation 9

For the traffic of VoIP, the performance comparison between PUSCH OCC and single UE without multiplexing are listed below,

- With PUSCH repetition =16

  - The performance loss of OCC =2 compared with SU equals to 0.47.

  - The performance loss of OCC =4 compared with SU equals to 1.26.

- With PUSCH repetition =32

  - The performance loss of OCC =2 compared with SU equals to 0.27.

  - The performance loss of OCC =4 compared with SU equals to 1.32.

Observation 10

The performance difference between different solutions, including inter-slot OCC, inter-symbol OCC and intra-symbol OCC is marginal when a relative high repetition number is applied.

Observation 11

Based on current specifications, the UCI multiplexing with PUSCH repetition and the dropping of PUSCH repetition happens, which will break the orthogonality of the OCC enhanced PUSCH repetitions.

Proposal 1

The enhancement to the redundancy version cycling of the PUSCH repetition should be considered, when the slot-level PUSCH OCC multiplexing is adopted.

Proposal 2

Considering both the performance and the specification impact, the inter-slot OCC scheme is preferred for the normative phase.

Proposal 3

Support reusing DMRS port indication in DCI field to additional indicate OCC index to reduce signalling overhead.

Proposal 4

Potential enhancements for the cases of PUSCH repetition dropping and the UCI multiplexing can be considered when the time domain-based OCC enhancements are applied.

Proposal 5

It should be further studied whether to support the frequency hopping in the NTN scenario and with the OCC multiplexing in the time domain.

Proposal 6

Only one solution should be down-selected from the current candidate solutions.

Proposal 7

If a combined solution with multiple candidate solutions is supported, only one UE capability is defined or supported without differentiation of any sub-capability or sub-solutions.

References

- Chairman's Notes RAN1#117, Fukuoka City, Fukuoka, JP, May 20th – May 24th, 2024

- R1-2403422, Feature lead summary #1 of AI 9.11.3 on NR-NTN uplink capacity and throughput	Moderator (MediaTek)

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#104, Shanghai, China, June 17-20, 2024

Annex I: Link level simulation assumptions for the slot-level OCC

Table 1 Parameters for the OCC evaluations

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Channel model | NTN-TDL-C Rural, 30° elevation angle |
| Frequency offset | [-0.1 ppm,0.1ppm] |
| Timing offset | without TO |
| SCS | 15kHz |
| BW | For single UE 2PRB |
| MCS | Low data rate: QPSK MCS=8 for OCC UE <= 4 QPSK MCS=9 for OCC UE = 8 VoIP: 16QAM MCS=11 for OCC UE <= 4 16QAM MCS=12 for OCC UE = 8 |
| TBS | 96bits for low data rate 184bits for VoIP |
| DMRS configuration | Type I, single-symbol, 2 DMRS symbol, no multiplexing with data, [2 11] When OCC enhancements with 8 UEs multiplexing (8 port DMRS), two symbol front loaded DMRSs are used. DMRS bundling is enabled. |
| Symbol allocation | 14 symbols |
| Waveform | DFT-S-OFDM |
| Channel estimation | Realistic |
| BLER | 2% rBLER for VoIP 10% iBLER for low data rate |
| Evaluation angle | 30O LEO |
| Repetition | PUSCH repetition Type-A 1, 2, 4, 8, 16, 32 |
| OCC scheme | OCC multiplexing across slots |

Annex II: Quotations of section 9 from TS 38.213

[TS 38.213] - 9 UE procedure for reporting control information If a UE transmits a PUSCH over multiple slots or multiple PUSCHs over multiple slots that are scheduled by a DCI format 0_1, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent. If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol. If the PUSCH transmission over the multiple slots is scheduled by a DCI format that includes a DAI field, the value of the DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information. When a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-UCI-Multiplexing; otherwise, if the HARQ-ACK information and the PUSCH have same priority index, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission; if the HARQ-ACK information and the PUSCH have different priority indexes, the UE does not transmit the channel with the smaller priority index. - 9.2.6 PUCCH repetition procedure A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where - if the PUCCH resource is indicated by a DCI format and includes pucch-RepetitionNrofSlots, NPUCCHrepeat is provided by pucch-RepetitionNrofSlots - otherwise, NPUCCHrepeat is provided by nrofSlots If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type A or with TB processing over multiple slots over a second number of slots, and the PUCCH transmission would overlap with the PUSCH transmission in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied in the overlapping slots, the UE transmits the PUCCH and does not transmit the PUSCH in the overlapping slots. If a UE would transmit a PUCCH over a first number NPUCCHrepeat>1 of slots and the UE would transmit a PUSCH with repetition Type B over a second number of slots, and the PUCCH transmission would overlap with actual PUSCH repetitions in one or more slots, and the conditions in clause 9.2.5 for multiplexing the UCI in the PUSCH are satisfied for the overlapping actual PUSCH repetitions, the UE transmits the PUCCH and does not transmit the overlapping actual PUSCH repetitions.