3GPP TSG RAN WG1#118                                                 R1-2406131

Maastricht, NL, August 19th – 23rd, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on support of RedCap/eRedCap UEs for NR NTN

Agenda Item:	9.11.2

Document for: Discussion

- Introduction

              - In RAN1#117 meeting, the support of Redcap/eRedcap UEs for NR-NTN was further discussed and the following conclusions /observations are achieved [1].

Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded. Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. Observation TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective. - Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item

              - Based on the progress in previous meetings, an updated WID for Rel-19 NR NTN was approved in RAN#104 plenary meeting [2]. In the updated WID, only HD-FDD collisions cases 3 and 4 have been identified to be enhanced in RAN1. New objects for support of RedCap/eRedCap UEs for NR NTN are as follows.

5. Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

              - In this contribution, the analysis on HD-FDD collision cases 3 and 4 for (e)RedCap HD-FDD UEs in NR NTN are provided.

- Discussion on HD-FDD collision cases 3 and 4

For HD collision cases 3 and 4, it was observed in RAN1#116bis meeting that occurrence of cases 3 and 4 can be avoided by network scheduling, but at cost of less resources available for the scheduled UE in NTN than TN as shown in Figure 1. According to the analysis and calculation in [3], the ratio of unavailable resource for the scheduled UE due to avoiding cases 3 and 4 is limited, especially considering that repetition transmission can be performed in NTN to handle the large path loss.

Figure 1 Avoid UL-DL collision based on TA

At least for solving collision case 4 (dynamically scheduled DL reception collides with dynamic scheduled UL transmission), if TA report is further enhanced, e.g., by introducing finer TA report granularity and smaller triggering offset threshold, the ratio of unavailable resource can be further reduced.

The legacy TA report granularity for NR NTN is one slot with 15 kHz SCS. Finer TA report granularity can be one symbol with 15 kHz SCS or one slot with 30 kHz SCS, etc. If finer TA report granularity is introduced, the determination of TA report granularity can include the following methods:

- Opt 1: The gNB indicates whether TA report granularity is the legacy granularity or the new finer granularity;

- Opt 2: The gNB and/or the UE determines TA report granularity according to predefined rules, such as using the same method as TA report granularity determination for ATG in FR1;

- Opt 3: The UE determines TA report granularity relying on UE’s implementation and indicates TA report granularity to the gNB via MAC CE.

New candidate triggering offset threshold can be less than 0.5 ms, e.g., equal to 0.25 ms, 0.1 ms, or expressed by symbols. Introducing smaller triggering offset threshold will promote more frequent TA report. That can reduce the impacts on resources available for a scheduled UE when avoiding the occurrence of case 4 through network scheduling though it may increase signaling overhead. By introducing smaller offset threshold values, the gNB can strike a balance between resource utilization, collision avoidance and signaling overhead.

If both finer TA report granularity and smaller offset threshold are introduced, in addition to the above three options, the gNB can implicitly indicate TA report granularity by configuring an offset threshold. For example, if the configured offset threshold is a legacy value, TA report granularity is the legacy granularity. If the configured offset threshold is a newly introduced value, TA report granularity is the new finer granularity.

The above enhancement of TA report can also be used as a general approach to alleviate the impacts due to the other HD collision cases to some extent, including cases 1, 2, 3, 5 and 6.

Observation 1: At least for case 4, with finer TA report granularity and triggering offset threshold, the ratio of unavailable resource for the scheduled UE due to avoiding collisions can be further reduced due to smaller TA mismatch.

Proposal 1: As a general approach, the TA report can be enhanced by introducing finer granularity and triggering offset threshold to further reduce the ratio of unavailable resource due to avoiding collision cases.

Due to the impracticality of frequent reconfiguration for semi-statically configured transmission/reception, the benefits brought by TA report enhancement for solving case 3 (semi-statically configured DL reception collides with semi-statically configured UL transmission) may not be as significant as for solving case 4. Therefore, additional enhancements for solving case 3 can also be considered.

For example, some priority rule of semi-statically configured DL reception vs. UL transmission can be defined case by case. It should be note that semi-statically configured DL reception and UL transmission defined in case 3 do not include SSB/PRACH. Collisions with SSB reception and PRACH transmission belong to cases 5 and 6. They can be solved based on existing mechanisms and have been determined not to be enhanced in the updated WID.

- The priority of PDCCH reception configured by a CSS set configuration (e.g., Type-0/0A/0B/1/2-PDCCH) is higher than all semi-statically configured UL transmissions in a same time resource. The reason is that such PDCCH reception is generally related to the scheduling of system information and paging, so it is necessary to prioritize the reception of this type of DL channels.

- For other semi-statically configured DL receptions and UL transmissions in case 3, the UE can also make decisions based on its implementation. However, considering that in most use cases of (e)RedCap UE, the importance of UL transmission may be greater than DL reception, it is also acceptable for RAN1 to define that the priority of UL transmission is greater than that of DL reception.

Proposal 2: For case 3, RAN1 can additionally define following rules to address the collision between semi-statically configured DL reception vs. UL transmission case by case:

- For the collision with PDCCH reception configured by a CSS set configuration vs semi-statically configured UL transmissions: prioritize the reception of this type of DL channels

- For the other collision, it’s up to UE’s implementation.

- Conclusions

      - According to the analysis given above, we have the following observations and proposals:

Observation 1: At least for case 4, with finer TA report granularity and triggering offset threshold, the ratio of unavailable resource for the scheduled UE due to avoiding collisions can be further reduced due to smaller TA mismatch.

Proposal 1: As a general approach, the TA report can be enhanced by introducing finer granularity and triggering offset threshold to further reduce the ratio of unavailable resource due to avoiding collision cases.

Proposal 2: For case 3, RAN1 can additionally define following rules to address the collision between semi-statically configured DL reception vs. UL transmission case by case:

- For the collision with PDCCH reception configured by a CSS set configuration vs semi-statically configured UL transmissions: prioritize the reception of this type of DL channels

- For the other collision, it’s up to UE’s implementation.

- Reference

- Draft_Minutes_report_RAN1#117_v020

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R1-2404215, Discussion on support of RedCap/eRedCap UEs for NR NTN, ZTE

- R1-2405741, Summary #3 for 9.11.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands.