3GPP TSG RAN WG1 #118                                                           R1-2406172

Maastricht, NL, August 19th – 23rd, 2024

Source:	vivo

Title:	Specification support for beam management

Agenda Item:	9.1.1

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, WID [1] for NR AI/ML air interface was approved which includes enhancement on beam management, which is copied below for reference.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signaling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we focus on the normative work for the use case of beam management.

- Consistency issue for UE-side model

In RAN#116bis meeting, the following agreement was achieved,

Agreement

Further study, for the consistency of NW-side additional condition across training and inference for UE-side model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B:

- Opt1: Based on associated ID (Referring to AI 9.1.3.3)

  - FFS on what can be assumed by UE with the same associated ID across training and inference

  - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

- Opt2: Performance monitoring based

  - FFS details

- Other options are not precluded.

In Rel-18 study, the consistency of the beam information (including aspects on beam codebook, beam width, etc.) between Set B and Set A was studied. Mismatched beam information of Set B and Set A can indeed lead to severe performance degradation.

Figure 2-1: Top-1 performance comparison for mis-matched beam width/beam pointing angle of Set A and Set B across training and inference

Figure 2-2: Top-2/1 performance comparison for mis-matched beam width/beam pointing angle of Set A and Set B across training and inference

The comparisons presented in Figures 2-1 and 2-2 were chosen from a total of 13 or 11 results obtained from 6 different companies submitted in TR 38.843[2]. These results show that the DL Tx beam prediction performance with mis-matched codebook consistency of Set A and Set B, as well as the performance when Set A and Set B codebook is consistent, across both training and model inference. This includes codebooks where there are mis-matched beam pointing angles and mismatched beam widths across training and inference. The results clearly demonstrate that significant performance improvements can be achieved with ensured consistency of Set A and Set B codebook on beam width/beam pointing angle across training and inference.

It is also expected that mismatched beam indexing mechanisms (mapping between beam index and actual beams) between training and inference would lead to severe performance degradation.

- Huge performance degradation can be observed without ensured consistency of Set A and Set B on beam width/beam pointing angle across training and inference.

- It is also expected that mismatched beam indexing mechanisms (mapping between beam index and actual beams) between training and inference would lead to severe performance degradation.

With above observations, for the “FFS on what can be assumed by UE with the same associated ID across training and inference” in the agreement, our understanding is that the same associated ID would imply the same network-side additional conditions including beam codebook corresponding to Tx beam boresight direction (azimuth and elevation), 3dB beam-width and mapping order of actual beams to beam indices which may corresponding to same transmission filter, for same resource IDs or same ordering in a list in Set A/Set B.

Option2 in previous agreement is based on performance monitoring. This performance monitoring-based approach consumes huge efforts at both UE side and NW side. For example, if a UE stores 20 models for beam prediction, each tailored to address different generalization issues, it would require a significant amount of time to monitor each model after the UE accesses one cell in order to determine model applicability. This inefficient process not only reduces efficiency but also adds unnecessary computational burden on the UE. In other words, it is infeasible for opt2 (performance monitoring based) to address the consistency issue for UE side model without the help of associated ID, due to large complexity/power consumption/overhead at both NW and UE side. Conversely, by using an associated ID, the UE can quickly determine the suitability of a model without incurring any unnecessary computational overhead. We, thus, propose,

For inference, for UE-side model, it can be assumed that same transmission filter for same resource IDs or through other rules (e.g. same ordering in a list) in Set A/Set B is consistent from training to inference if UE receives the same associated ID.

For inference, for UE-side model, support that associated ID representing NW-side additional conditions is provided to UE to ensure consistency between training and inference, as well as to address NW-side proprietary information disclosing issue.

Further, in order to address the issue of how to configure associated ID, it is necessary to first discuss how associated IDs are determined or allocated as global ID or local ID. The details of global associated ID and local associated ID are provided in the following,

- Global associated ID would refer to NW side additional conditions of different NW-side implementation/configurations. Different global associated ID would refer to different Tx beam boresight direction (azimuth and elevation), 3dB beam-width and mapping of actual beams to beam indices, etc.

- Local associated ID is cell/site/region specific. NW may use different beam codebook in the daytime and night, since the UE distribution and UE requirement may be different in the daytime and night. Then NW could use different local associated ID to indicate different beam codebook.

The local associated ID requires large efforts at UE side to categorize the collected data to maintain consistency between training and inference. Depending on how local the associated ID is applied, the trained model may only be applicable to a relatively small region thus may potentially require cell/site/region specific model development. Conversely, global associated ID is useful for maintaining consistency between training and inference and help UE to train a model with good generalization performance globally and thus would be friendlier to UE side management.

However, global associated ID may potentially expose deployment choices of NW side, which results in the working assumption agreed in AI 9.1.3.3 for local associated ID to address consistency issues.

Along the line of the spirit of the working assumption in AI 9.1.3.3, local associated ID can be prioritized. On the other hand, ways to resolve UE side complexity on managing local associated IDs should be further studied. We, thus, propose,

For inference, for UE-side model, along the line of the agreed working assumption on the consistency issue in AI 9.1.3.3, support to prioritize discussion on how UE-side AI model can work with local associated ID and study how to reduce UE side complexity on managing local associated IDs.

Figure 2-3: cell specific AI model and multi-cell AI model with Local associated ID

If the UE 1 is connected to a cell that was used for data collection and reports the associated ID for inference, the network has the capability to assess the consistency of the UE sided AI model. By contrast, if an AI model for UE 1 is trained by a dataset collected from cell 1 to cell N, and UE 1 reports only the supported local associated ID for inference to cell M, which has the same transmission filters as cell 1 to cell N, the network maybe unable to determine the consistency of the UE sided AI model in cell M due to the limited application area of local associated ID. From UE’s perspective, cell specific AI model without model transfer is not an efficient way to address consistency issue for UE-sides model, while multi-cell model assumption at UE sided needs to be further studied for local associated ID, including both the case that the model is developed for multi-cell and the case when UE moves to a cell that UE has not collected data before.

For inference, for UE-side model, suggest to further study on multi-cell AI model with local associated ID, including both the case that the model is developed for multi-cell and the case when UE moves to a cell that UE has not collected data before.

For the case when UE moves to a cell that UE has not collected data before, we can consider the following procedures to make UE side mode more efficient.

Figure 2-4: associated ID confirmation for multi-cell AI model

For data collection and model training:

- Step 1: Beam related data is collected with associated ID and corresponding cell ID(s), e.g. associated ID + {cell 1, cell 2, cell 3, …, cell N}, or associated ID + cell 1

- Step 2: AI model is trained by collected data, and mapping to the associated ID + cell ID(s)

For inference phase:

- Step 1: UE reports supported associated IDs with corresponding cell IDs to the accessed cell, e.g. associated ID + cell 1, another associated ID + another cell,

- Step 2: Supported associated ID information is processed at network.

- Step 3: The selected associated ID among supported associated IDs can be configured with RS/report configuration to UE, if the ID is also supported by the accessed cell.

The aforementioned process ensures the practical functionality of the multi-cell AI model with local associated ID. However, additional discussion is required to determine how to efficiently implement the cell-specific AI model with local associated ID.

For inference, for UE-side model, further study how to determine consistency for a UE sided AI model for multi-cell AI model, e.g. local associated ID can be mapped to an assistant information (cell ID) during data collection.

- Performance monitoring

In TR 38.843[2], 4 options of performance metrics are captured,

For the performance monitoring of BM-Case1 and BM-Case2:

-	Performance metric(s) with the following alternatives:

-	Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

-	Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

-	Alt.3: Performance metric based on input/output data distribution of AI/ML

-	Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

-	Benchmark/reference for the performance comparison, including:

-	Alt.1: The best beam(s) obtained by measuring beams of a set indicated by gNB (e.g., Beams from Set A)

-	Alt.4: Measurements of the predicted best beam(s) corresponding to model output (e.g., Comparison between actual L1-RSRP and predicted RSRP of predicted Top-1/K Beams)

-	Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

The KPIs provided by Alt 2, including throughput, L1-RSRP, L1-SINR, and hypothetical BLER, are limited to indicating communication quality and are not capable of evaluating the accuracy of AI model predictions. As a result, Alt 2 is not suitable as performance monitoring metrics.

Regarding Alt 3, its effectiveness in beam prediction is uncertain, and its necessity in scenarios where labels can be obtained is also in doubt. Furthermore, the latest FL summary suggests alternative methods to Alt 3, such as

- Alt 3-1: Probability information of the predicted beam to be the Top 1.

- Alt 3-2: A confidence interval or prediction interval associated with predicted L1-RSRPs at a specific confidence level (e.g., 95%).

The method of obtaining probability or confidence in beam prediction usage is unclear. From our perspective, the only method is to determine probability or confidence based on model output. First of all, there is uncertainty about the reliability of probability or confidence determined by the model output. Besides, using probability or confidence which are obtained based on an AI model output results does not allow for the determination of prediction performance of the AI model. Therefore, Alt 3-1/Alt 3-2 as performance metrics are also not suitable.

Based on above analysis, our preferred metrics include the beam prediction accuracy related KPIs and L1-RSRP difference, i.e. Alt.1 and Alt.4, which have been evaluated across companies and are directly related with the model's performance. Thus, we propose,

For performance monitoring, at least support beam prediction accuracy related KPI and L1-RSRP difference as potential metrics, while probability and confidence information should be deferred after related agreement is achieved in inference phase.

Further, in RAN1# 117 meeting, the following agreement was achieved for monitoring procedure,

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

For type 1 option 1:

The UE is required to report measurement results for a specific resource set (e.g. Set A) to enable the network (NW) to calculate performance metrics. However, it is inefficient in terms of resource utilization to report all measurement results of Set A, through either L1 signaling or higher-layer signaling. Additionally, since the process of model monitoring on the NW-side AI model and NW side performance monitoring of UE-side AI model is similar, it is crucial to reduce report overhead for both. Moreover, considering the issues of data validity and real-time model monitoring, it is advisable to at least support the use of L1 signaling to feedback measurement results.

For performance monitoring, at least support L1 beam reporting for both UE-side model and NW-side model.

For monitoring report, support report overhead reduction for NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1), as well as for NW-side AI model, e.g., report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

Furthermore, it is important to note that the UE has already conducted measurements on the resources corresponding to Set A during performance monitoring, and the current beam indicated by DCI should also be encompassed within the Set A. If the report only includes the top-k measured results of Set A, there is a risk that the measured results of the current beam may not be included in the selected top-k beams, thus hindering the network side from acquiring model monitoring information for further beam indication. This scenario leads to a significant waste of UE energy consumption and resources. Consequently, it may be prudent to consider incorporating the measured results of the specified resources of Set A with the top-k measured resources in performance monitoring report.

For monitoring report, support to report measured L1-RSRP of indicated beam(s), e.g. current beams indicated by DCI, with measured L1-RSRP of top-k beams for both NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1) and NW-side AI model.

For type 1 option 2:

In FL summary, potential contents are discussed in RAN1 #117:

- Option B: Report the beam prediction accuracy related information based on measurements [of a window]

- Option C: Report the RSRP difference information between the measured and predicted

- Option D: Report probability information of the predicted beam of Top 1 or Top K beams

  - FFS on probability information and the quantization, including

    - #1: The probability information of predicted Top 1

      - Note: this can be treated as report of inference result as well

    - #2: The probability information of each or sum of predicted Top Top-K beams.

      - Note: this can be treated as report of inference result as well

    - #3: Beam information that the probability information comparing to a threshold.

- Option E: Report confidence information of the predicted RSRP error

  - FFS on confidence information and the quantization, including

    - #1: The confidence information of predicted RSRP of the predicted Top 1

      - Note: this can be treated as report of inference result as well

    - #1: The averaged confidence information of predicted RSRP of the predicted Top Top-K beams.

      - Note: this can be treated as report of inference result as well

    - #3: The averaged confidence information of predicted RSRP of all beams in Set A.

- Option F: Report hypothetical BLER-like metrics

As we explained that probability and confidence information should be deferred after related agreement is achieved in inference phase, Option D and Option E should be deprioritized and support Option B and Option C for further study detail.

For performance monitoring, for UE-side model, support Option B and Option C, i.e. report the beam prediction accuracy related information and report the RSRP difference information between the measured and predicted, for further study, while Option D and Option E, i.e. probability and confidence information, should be deferred after related agreement is achieved in inference phase.

Due to a brief mention of event-based reporting but lacked in-depth discussion in previous meeting, we discuss the difference between metric-based reporting and event-based reporting. Before delving into event-based reporting, it is imperative to establish a clear definition of metric-based reporting and event-based reporting, as well as the corresponding reporting content. As per the understanding of specification, a metric pertains to a calculated result of corresponding KPIs, whereas an event pertains to the occurrence of a specific event based on the results of corresponding metric. Therefore, based on this comprehension, the reporting content can be expanded as follows.

- Opt1: report metric

- Opt2: report event

- Opt3: based on event, report event and metric(s)

In Option 1, the UE consistently reports metrics based on a pre-configured periodicity, whereas in Option 2, the UE reports an event only when the event is triggered. Additionally, considering the UE's capability to predict comprehensive results, it can furnish more pertinent information including triggered event and associated event or metrics. For instance, when an event linked to the Top-1 beam accuracy smaller than a threshold is triggered, the UE can also report the top-1 RSRP difference to aid the network in determining whether to de-activate the model. Upon the triggering of this event, if the actual top-1 RSRP difference falls within an acceptable range, it indicates that the predicted top-1 beam and beam qualities remain favorable. Consequently, Option 3 has the potential to offer more valuable information with limited report overhead increase.

For performance monitoring, for UE-side model, support following options for further study on what/how to report,

- Opt1: report metric

- Opt2: report event

- Opt3: based on event, report event and metric(s)

- Other options cannot be precluded

TR 38.843[2] outlines two options of Type 1 performance monitoring for UE-side models, namely NW-side performance monitoring and UE-assisted performance monitoring, where the UE sends reports to the NW for NW-side performance monitoring, or calculates performance metric(s) and reports them or an event to the NW for UE-assisted performance monitoring. The use of UE-assisted performance monitoring for the second operation can significantly reduce the report overhead for type 1 performance monitoring of UE-side model. On the other hand, performance monitoring for NW-side model can also benefit from such overhead reduction given by UE assisted performance monitoring.

Figure 3-1: UE assisted model monitoring for NW-side model

Figure 3-1 illustrates the performance monitoring procedure for UE-assisted model monitoring of NW-side models, which uses BM case 2 as an example. The procedure involves the following steps:

Step 1: The UE reports measurement results of Set B to the NW with a NW-side model for temporal domain beam prediction at occasion T.

Step 2: Temporal domain predicted labels can be obtained from the prediction results of top-N beam ID and/or corresponding L1-RSRP of future 8 occasions from occasion T + 3 to occasion T + 10. The predicted labels at occasion T +3 to occasion T + 10 are then transmitted to the UE at occasion T + 1.

Per occasion report:

Step 3: The UE receives monitoring RS resources at occasion T + 3 to obtain real measurement labels as performance monitoring reference. At the same time, the UE calculates KPI of performance metrics based on measurement labels of occasion T + 3 and predicted labels of occasion T + 3 received at step 2.

Step 4: The UE reports performance metrics or events of occasion T +3 obtained from step 3.

Step 5: The above two steps, i.e. step 3 and step 4, are cyclically executed from occasion T + 4 to occasion T + 10.

Multi-occasion results report:

Step 3: The UE receives monitoring RS resources from occasion T + 3 to occasion T + 10 to obtain real measurement labels of 8 occasions. Then, the UE calculates KPI of performance metrics based on 8 measurement labels and 8 predicted labels of occasion T + 3 ~ occasion T + 10.

Step 4: The UE reports performance metrics or events of occasion T +3 ~ occasion T + 10 obtained from step 3.

The above steps demonstrate the feasibility of UE-assisted model monitoring for NW-side models, and also can provide significant improvement on UCI overhead reduction. For example, assuming a NW-side model has 32 beams in Set A and predicts beams of 8 future occasions based on per occasion report, the following assumptions can be made to calculate UCI report overhead,

Assumption 1: Set A = 32 beams, predict 8 future occasions, report Top-4 beams information

Assumption 2: per beam ID report overhead = 5 bits per beam

Assumption 3: combination 4 beam ID report overhead = 16 bits

Assumption 4: KPI-1 = Top 4/1 beam accuracy, report overhead = 5 bits

Assumption 5: KPI-2 = Top-1 L1-RSRP difference, report overhead = 4 bits

Table 2: UCI overhead reduction of UE assisted performance monitoring for NW-side model

|  | UCI Report overhead per occasion | Total UCI report overhead | Overhead reduction |
| --- | --- | --- | --- |
| Method 1: beam ID report | Per beam ID report = 5 * 4= 20 bits Combination beam ID report = 16 bits | 16 * 8 ~ 20 * 8 = 128 bits ~ 160 bits | 68% ~ 75% |
| Method 2: beam ID of UE assisted report | Per Top-4/1 beam accuracy report = 5 bits | 5 * 8 = 40 bits |  |
| Method 3: beam ID + L1- RSRP report | Per beam ID report = 5 * 4= 20 bits Combination beam ID report = 16 bits L1-RSRP report = 7 + 4 * 3 = 19 bits | (128 bits ~ 160 bits) + (19 * 8) = 280 bits ~ 312 bits | 74%~77% |
| Method 4: beam ID + L1- RSRP of UE assisted report | Per Top-4/1 beam accuracy report = 5 bits Per Top-1 L1-RSRP accuracy report = 4 bits | 5 * 8 + 4 * 8 = 72 bits |  |

Table 2 shows the UCI overhead reduction achieved by various reporting methods. Method 1 reports only the Top-4 beam ID of 8 occasions to the NW, while method 2 reports the performance metric of KPI-1. Method 3 reports the Top-4 beam ID and corresponding L1-RSRP, whereas method 4 reports both KPI-1 and KPI-2 to the NW. It can be observed that method 2 achieves a UCI overhead reduction of 68% to 75% compared to method 1 with per beam ID report or combination beam ID report. Similarly, method 4 achieves a UCI overhead reduction of approximately 74%~77% compared to method 3, which reports the Top-4 beam ID and corresponding L1-RSRP. Please note that along with the need of having 64 or 128 set A beams, more overhead can be reduced based on method 2 or 4.

Significant UCI overhead reduction can be achieved by UE-assisted model monitoring compared to direct measurement results report for NW-side AI/ML model.

For performance monitoring, for NW-side model, support to report metrics for UCI reporting overhead reduction.

- Configuration of RS for UE-side model

  - Set B configuration

In TR 38.843[2], following options are studied on the selection of Set B of beams (pairs):

-	Option 1: Set B is fixed across training and inference

-	Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference)

-	Opt 2A: Set B is changed following a set of pre-configured patterns

-	Opt 2B: Set B is randomly changed among pre-configured patterns

-	Opt 2C: Set B is randomly changed among Set A beams (pairs)

-	Opt 2D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

-	The number of beams(pairs) in Set B can be fixed or variable

-	Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

-	Note: BM-Case1 and BM-Case2 may be considered for different option.

-	Note: This does not preclude the alternative that Set B is different from Set A.

It has been proved that while a single pre-defined beam subset with a fixed pattern in Set B may theoretically exhibit good performance, it lacks the necessary flexibility for practical implementation. In real-world scenarios, unexpected channel variations such as blockages can cause a particular beam or beam pair to experience performance loss or significant interference. Evaluation results from various companies in TR 38.843 [2] have highlighted the significant performance degradation that can occur when using mismatched fixed patterns in Set B during model training and inference. Therefore, it is crucial to consider the impact of channel variations and to incorporate flexibility into Set B design to ensure optimal performance in practical implementations.

Figure 4-1: Top-1 performance degradation for mismatched fixed pattern for DL Tx beam prediction

Figure 4-2: Top-2/1 performance degradation for mismatched fixed pattern for DL Tx beam prediction

Figure 4-3: Top-4/1 performance degradation for mismatched fixed pattern for DL Tx beam prediction

The simulation results presented in the three figures above were obtained from 7 companies that submitted their results in TR 38.843[2], which demonstrates beam accuracy degradation of DL Tx beam prediction performance on mis-matched fixed patterns in Set B between model training and model inference. Further, the beam accuracy degradation shown in the figures represents the difference between the beam accuracy with mis-matched fixed patterns in Set B and the beam accuracy with matched fixed patterns in Set B.

Based on these simulation results, it is observed that there is more than 65% performance degradation among different companies in the KPI of Top-1 beam prediction accuracy for DL Tx beam prediction with mis-matched fixed patterns in Set B. Similarly, a performance degradation of about 55-60% is observed in the KPI of Top-2/1 and Top4/1 beam prediction accuracy. These results highlight the importance of considering the impact of mis-matched fixed patterns in Set B and the need for flexible Set B design to ensure performance.

- The simulation results from TR 38.843 show that using a wrong fixed pattern causes at least 65/60/55 percent performance degradation for the KPI of Top1, Top2/1 and Top 4/1 beam prediction accuracy, respectively.

The simulation results submitted by 10 companies in TR 38.843[2] provide a comparison between fixed patterns and pre-configured patterns in Set B. Based on these results, it is observed that there is a slightly performance degradation when using pre-configured patterns compared to fixed pattern in Set B.

Figure 4-4: Top-1 performance degradation for pre-configured patterns compared to fixed pattern

Figure 4-5: Top-2/1 performance degradation for pre-configured patterns compared to fixed pattern

Based on the simulation results presented in the figures above, it can be observed that there is an average performance decrease of less than 3% among 10 companies for the KPI of Top-1 and Top-2/1 beam prediction accuracy when using pre-configured patterns with Tx beam ID information as input, compared to fixed patterns in Set B. Further, it is important to note that approximately less than 2% performance degradation of Top-1 beam prediction accuracy and almost same Top-2/1 beam prediction accuracy of pre-configured pattern in Set B can be achieved for the majority of companies in comparison with fixed pattern in Set B. Thus, we have below observations,

- Less than 3% average performance decrease among 10 companies submitted in TR 38.843 [2] for the KPI of Top1 and Top2/1 beam prediction accuracy of pre-configured patterns with Tx beam ID information compared to fixed pattern in Set B.

- It can be observed from majority companies in TR38.843 [2] that only 2% performance degradation of Top-1 beam prediction accuracy and almost same performance of Top-2/1 beam prediction accuracy achieved for pre-configured patterns in Set B with Tx beam ID information as input compared to fixed pattern in Set B.

During the model training phase, the network only sends all the beams corresponding to Set A or Set B. The specific Set B patterns used for training, as well as which Set B patterns may provide better performance, are determined based on UE implementation and real channel. Consequently, during model inference phase, the network also needs to configure same Set B patterns. Therefore, UE needs to recommend some Set B patterns to prevent performance degradation due to inconsistencies in Set B patterns between the training and inference phases, and gNB to configure a resource list including multiple Set B patterns. Further, due to the time varying of channel conditions such as movement, blockage and UE rotation, gNB needs to further activate resources of one Set B pattern from the list by signaling. We, thus, propose,

For inference, for UE-side model, support UE to recommend some preferred Set B patterns which were trained during the UE-side model training phase.

For inference, for UE-side model, support gNB to configure a resource list including multiple Set B patterns and activate resources of one Set B pattern from the list by signaling during the inference phase.

  - Set A configuration

For data collection, Set A should be configured for measurement as label for model training. For inference phase，although no measurement is needed for Set A, when considering beam interference management, it is important to address the following scenario: the base station decides not to use Tx beam 1 due to potential interference to other cells. Despite the gNB not utilizing Tx beam 1 for the subsequent data transmission, the beam may still be selected by the UE to report as a top-k predicted beam. Therefore, it is necessary to provide support for configuring beam subset restrictions of Set A associated with predicted beam reports to mitigate interference issues. This will help ensure that potential interference concerns are addressed and that the reporting of predicted beams takes into account the restrictions set by the base station to manage interference effectively. Thus, it is necessary to configure full and/or subset of Set A for UE-side model.

Additionally, when considering the different usage of Set A for data collection and model inference, it may be beneficial to indicate Set A or resources within Set A as a virtual set or virtual resource for inference configuration. In this context, "virtual set" or "virtual resource" refers to a legacy configuration that does not require measurements on these set A resources for the associated report. This approach can help streamline the configuration process and ensure that resources within Set A are optimized for model inference without the need for additional measurements.

For UE-side model, support to configure full and/or subset of Set A associated with predicted beam report to address interference issue.

For inference, for UE-side model, Set A or resources in Set A should be indicated as virtual Set or virtual resource, which does not require measurement on these resources for the report.

  - Set B and Set A association

In RAN 1#116bis meeting, following agreement was achieved for Set B and Set A association,

Agreement

For UE-side model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting

- FFS on the details in the CSI-ReportConfig, at least considering:

  - Alt 1: one CSI-ResourceConfigId is configured for Set B

    - FFS: how UE can determine the information about set A

  - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

    - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

  - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

    - FFS: how to configure/indicate separate resource set(s) for Set A

  - Note: separate CSI-ReportConfig for Set A and Set B are not precluded.

  - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig

  - FFS on the association between Set A and Set B with or without additional IE

Other necessary configuration are not precluded.

Based on the agreement and the current reporting configuration method, different alternatives for Set A and Set B association can be expanded into different potential options, as shown in the figure below,

Figure 4-6: potential options of different alternatives for the configuration of inference results reporting

Potential options for CSI-ReportConfig

- Alt1-Opt1: one CSI-ReportConfig, only includes Set B, Set A without any configuration can be assumed by associated ID.

- Alt1-Opt2: one CSI-ReportConfig, only includes Set B, Set A is configured out of the CSI-ReportConfig and associated to Set B

- Alt1-Opt3: two CSI-ReportConfig, one CSI-ReportConfig includes Set B and another CSI-ReportConfig includes Set A.

- Alt2: one CSI-ReportConfig and one CSI-ResourceConfig, the CSI-ResourceConfig includes Set B and Set A

- Alt3-Opt1: one CSI-ReportConfig with two CMR, one CMR includes Set B and another CMR includes Set A

- Alt3-Opt2: one CSI-ReportConfig with one CMR, the CMR includes Set B and Set A

It is advisable to initially focus on clearly explaining the various alternatives before proceeding with any down selection. Furthermore, as the definition of Alt4 is unclear and may related to Set A configuration, it is suggested to postpone any consideration of Alt4 until the discussion on the configuration of Set A is fully completed. This approach will ensure a comprehensive understanding of all options before making any decisions on potential selections. Thus,

For inference, for UE-side model, support below potential options of different alternatives for the configuration of inference results reporting,

- Alt1-Opt1: one CSI-ReportConfig, only includes Set B, Set A without any configuration can be assumed by associated ID.

- Alt1-Opt2: one CSI-ReportConfig, only includes Set B, Set A is configured out of the CSI-ReportConfig and associated to Set B

- Alt1-Opt3: two CSI-ReportConfig, one CSI-ReportConfig includes Set B and another CSI-ReportConfig includes Set A.

- Alt2: one CSI-ReportConfig and one CSI-ResourceConfig, the CSI-ResourceConfig includes Set B and Set A

- Alt3-Opt1: one CSI-ReportConfig with two CMR, one CMR includes Set B and another CMR includes Set A

- Alt3-Opt2: one CSI-ReportConfig with one CMR, the CMR includes Set B and Set A

For inference, for UE-side model, suggest to postpone any consideration of Alt4 until the discussion on the configuration of Set A is fully completed.

  - How to configure RS for Set B measurement of multiple time instances for BM-case 2

Figure 4-7: same beams of Set B and different beams of Set B corresponding to multiple historical periods

For BM-case 2, the need to utilize measurement information from Set B across multiple historical periods creates the challenge of RS configuration of Set B across different historical periods. As illustrated in above figure, in method a, multiple historical periods use the same beams of Set B for measurement, corresponding to each historical period using beam 1 to beam 8. Conversely, in method b, multiple historical periods use different beams of Set B for measurement, providing more diverse beam information to assist UE for inference. This issue has been discussed in the SI, where it was noted that, under the same overhead, method b) significantly improves predictive performance compared to method a). Therefore, it is recommended to support the configuration of different historical periods corresponding to different beams of Set B for measurement. This approach can enhance the predictive performance and provide more diverse beam information for inference across multiple historical periods.

For inference, suggest to configure different beams of Set B for measurement corresponding to different historical periods for BM-Case 2.

Figure 4-8: unnecessary measurements for Set B occasions existed in the prediction window

As depicted in the above figure, the gray occasions represent the periodic RS resources, transmitted from T1, by legacy configuration method, and UE performs measurements at corresponding occasions to obtain Set B information for inference. During the 1st inference, UE uses beams from T1 to T4 as historical measurement information to infer the future prediction results from T5 to T10. However, at the next inference moment, there are two different behaviors. For option 1, UE measures T5-T8 to predict the future results from T9 to T14, without considering that T9 and T10 have already been predicted in previous prediction moment. On the other hand, for option 2, UE can measure from T7 to T10 to predict the future occasions from T11 to T16, and the Set B resources at T5 and T6 do not need to be measured. We believe that both options exist, and the benefit from Option 2 is clear as RS overhead for a number of CSI-RS occasions can be saved. gNB can even use these RS REs for scheduling other channels or UEs. Therefore, for the second configuration, further study is needed on how to avoid unnecessary measurements and RS overhead for CSI-RS occasions existed in the prediction window. We, thus, propose,

For inference, further study how to avoid unnecessary measurements and RS overhead for CSI-RS occasions existed in the prediction window for BM-Case 2.

  - Rx beam assumption for Set B

In TR 38.843[2], following observation was approved for the performance study on Quasi-Rx beam in DL Tx beam prediction,

Performance with different Rx beam assumption for DL Tx beam prediction

At least for BM-Case1 when Set B is a subset of Set A, and for DL Tx beam prediction, with the measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample, AI/ML provides the better performance than with measurements of random Rx beam(s).

-	Evaluation results from 12 sources show 20%~50% degradation with random Rx beam(s) comparing with the "best" Rx beam in terms of Top-1 prediction accuracy.

-	Evaluation results from 1 source shows 12% degradation with measurement of random Rx compared with measurement of best Rx in term of Top-1 beam prediction accuracy.

It demonstrates quasi-best Rx beam has considerable performance improvement for DL Tx beam prediction in comparison with random Rx beam. Although current specification provides implementation method to obtain Quasi-best Rx beam, it depends heavily on gNB implementation and UE coordination.

For example, a beam resource set with multiple beam resources is transmitted to UE for P3 procedure, and beam resources in a beam resource set for P2 procedure are QCLed to a beam resource specified in the P3 procedure. Thus, it can be assumed that UE receives P2 resource set by the best Rx beam searched by P3 resource set. In the current specification, a P3 procedure can only let UE acquire a best Rx beam corresponding to one Tx beam, while this best Rx beam may be different from the overall best Rx beam, which is the best Rx beam from all Tx beams. However, to acquire the best Rx beam corresponding to one Tx beam may not be sufficient for the quasi-best Rx beam acquisition for AI-based beam prediction, which is sensitive to Rx beam used in P2 resource set.

Based on evaluation in the SI, such best Rx beam corresponding to one Tx beam will lead to certain performance loss [2]. One the other hand, to search the best Rx beam for all Tx beams based on the current specification will lead to large RS overhead and large latency.  Due to time interval between P3 resource set and P2 resource set is also performed by gNB implementation, the quasi-best Rx beam obtained from P3 resource set may expire for the following P2 resource set receiving. Therefore, to trade off prediction performance, RS overhead and latency, it is desirable to perform P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for set B measurement result acquisition. Typically, to have a limited number of Tx beams can increase the prediction performance a lot. Meanwhile, it is also desirable to maintain the principle that UE does not need to report anything for P3 procedure, and the detailed Rx beam is up to UE implementation.

Figure 4-9: Quasi-best Rx beam acquisition

The figure above illustrates how to obtain the quasi-best Rx beam with two Tx beams from a Set B pattern. The first Tx beam, marked as blue, and the third beam, marked as yellow, of the Set B pattern are transmitted with 4 times repetition at T+1 and T+2, respectively. This enables the UE to obtain the quasi-best Rx beam by performing P3 based on the two Tx beams. Therefore, it is recommended to support the performance of P3 UE Rx beam search based on multiple Tx beams, which can be configured within a time interval limitation for P2 resource set receiving for Set B measurement result acquisition.

To sum up, it is essential to support the assumption of a quasi-best Rx beam for the measurement of Set B for model inference with UE-side model. This is crucial for ensuring accurate and effective model inference, particularly in scenarios where the UE-side model is utilized to predict L1-RSRP. By supporting the quasi-best Rx beam assumption, the inference process can be optimized, leading to more reliable and precise model outcomes.

For data collection and inference, for UE-side model, support to use quasi-best Rx beam for Set A and/or Set B measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A/B.

- Configuration of RS for NW-side model

  - Set B configuration

In 4.1, we elaborate the importance of having multiple set B patterns for UE side model. The importance also holds for NW side model. If only one set B pattern is supported in NW side, and some beams in this set are blocked in fields, large performance loss is expected for AI based prediction. Hence NW side model also need to support multiple set B patterns.

In order to maintain consistency and improve flexibility in the Set B patterns used for mode inference with NW-side model, a dedicated set of beam measurement resources is established. This set supports multiple pre-configured Set B patterns and is made accessible to the UEs during the model inference procedure. This set may include relatively large number of beams, and it is not feasible to report all L1-RSRPs in UCI considering the large overhead. For example, if 4 set B patterns each with 8 beams are supported by gNB, and the union set contains 20 beams for measurement, to let UE reports all the 20 L1-RSRPs causes too much overhead. However, if UE freely selects the top N (e.g., top 8) beams from the 20 beams, the reported beam may form a pattern which hasn’t been trained by gNB, which makes the AI inference not possible.

Therefore, one way to address this issue is that the measurement results of a single Set B pattern, chosen from all the measured Set B patterns, are reported during the accessed UE report procedure. We, thus, propose,

For inference, for NW-side model, support gNB to configure a resource list including multiple Set B patterns and active resources of one Set B pattern from the list by signaling during the inference phase.

  - Set B and Set A association

During the inference phase of the NW-side model, only the measurements and feedback from Set B are taken into consideration. However, during model training and performance monitoring procedure, UE can report measurements of Set B and Set A separately. In this method, UE does not need to perceive the association between Set B and Set A. Consequently, there is no motivation to configure Set B and Set A simultaneously in one report.

For NW-side model, there is no motivation to configure Set B and Set A simultaneously in one report.

  - Rx beam assumption for Set B

For the same reasons as outlined in sub-section 4.5, it is important to support the assumption of quasi-best Rx beam for the measurement of Set A and/or Set B. Due to the inability of the UE to perceive Set B and Set A during data collection and model inference with NW-side model, specific enhancement should be focused on the configuration of beam sweeping resources to achieve quasi-best Rx beam acquisition.

For data collection and inference, for NW-side model, support to use quasi-best Rx beam for Set A measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A.

- Inference report for UE-side model

  - Report content for BM-Case 1, for inference

In RAN1#116 meeting, following agreement was approved for model inference with UE-side model,

Agreement

For UE-side model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - FFS on the quantization method of probability information

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

- Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

  - FFS on definition of reported RSRP

  - FFS on the definition and quantization method of confidence information

- Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

For the report content of UE-side model inference, supports one remaining issue is whether extra overhead reduction is needed. We think overhead reduction can be considered at least for L1-RSRP prediction due to the maximum number of K can be large. As the beams with RSRP lower than a certain threshold for a gNB will anyway not facilitate NW scheduling, it is not efficient to always report a fixed number of beams. Hence, we support to report M beams within X dB gap to the largest predicted L1-RSRP.

For inference, for UE-side model, support to report predicted L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest predicted value of L1-RSRP, as well as the number of reported beams.

In RAN 1#116bis meeting, following agreement and working assumption were approved for the definition of the RSRP of precited Top-k beam(s),

Agreement

For report content of inference results for UE-side model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

In the scenario where Set B is a subset of Set A for model inference with UE-side model, the UE's predicted results may pertain to the measurements from Set B. Therefore, when the UE reports L1-RSRP of Top-k beams predicted from Set A, and the beam information of the Top-k predicted beams belongs to Set B, the UE reports the actual measured RSRP corresponding to the beam, which provides more useful and reliable information. Thus, we propose,

For inference, for UE-side model, when UE reports L1-RSRP of Top-k beams predicted from Set A, and for those that belongs to Set B, the UE reports the actually measured RSRP corresponding to the beam.

  - Report content for BM-Case 2, for inference

In RAN1#116bis meeting, following agreement was approved for BM-Case 2,

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

Due to predicted beam information of multiple future occasions should be contained in one beam report, time stamp information is naturally contained in this report.

For inference, for UE-side model, support time stamp information as beam content for BM-Case2.

For inference, for UE-side model, support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions.

Furthermore, at least beam content in BM-Case 1 should be supported for BM-Case 2. We, thus, propose,

For inference, for UE-side model, support beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams per predicted instance for content in the report for BM-Case 2.

  - Report configuration for BM-Case 1 and BM-Case 2

Figure 6-1: legacy report configuration and predicted report configuration

In above figure, reportConfig-1 represents the conventional report configuration utilized for measurement report, while reportConfig-2 is employed for UE feedback of predictive results and is linked to Set B for measurement and Set A for prediction. Both types of report coexist for a Rel-19 UE supporting AI based beam report. Consequently, in the inference stage, it becomes imperative to contemplate how to differentiate between these two types of reports.

Since UE needs to feed back predicted results in a beam report for model inference with UE-side model, the report configuration can associate with a new report quantity to determine predicted results report without the need to define report purpose. This would minimize the impact on the existing specification.

For inference, for UE-side model, support new report quantity to differentiate between AI-based predicted report with legacy measured report.

As per the current report method, the UE reports top-k predicted L1-RSRPs and corresponding beam information, and the network may directly utilize this information for subsequent beam management. However, since the current beam, e.g. beam indicated by DCI, may not be included in the top-k predicted beams, the network cannot compare predicted beams with current beam. This situation could potentially lead to unnecessary frequent beam switching. Furthermore, if the network obtains measured L1-RSRP of the current beam through configuring additional RS resources to avoid frequent beam switching, additional measurement resources and reports for the current beam will be required, decreasing overall resource utilization of the system.

To address this issue, network can configure a beam resource indicator to force reporting the predicted L1-RSRP of an indicated beam. This will enable the network to obtain the predicted L1-RSRP of the top-k beams as well as the indicated beam, e.g. current beam indicated in TCI by gNB. By doing so, the network can avoid unnecessary beam switching or increasing resource utilization.

For inference, for UE-side model, support to report predicted L1-RSRP of indicated beam(s), e.g. current beam indicated by TCI, with predicted L1-RSRP of top-k beams in one prediction report.

  - Report overhead reduction for BM-Case 2

In RAN1#116bis meeting, following agreement was approved for BM-Case 2,

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

For a UE side model, UE needs to report multiple Top-N predicted beams in multiple future occasions to NW during model inference in BM-Case2. Due to beam correlation across multiple occasions, temporal domain compression of beam resource indication shall be considered to further reduce report overhead in a beam report with multiple time occasions for UE side model.

Figure 6-2: direct predicted beam resource indication

For example, a beam report includes 4 occasions where each occasion feeds back Top-8 predicted beam resource indicators from 32 total beams. If the direct predicted beam resource indication method is used in Figure 3-9, total 32 of CRI or SSBRI including in 4 occasions need to be reported in one beam report, and the corresponding report costs of CRI or SSBRI is 160 bits.

Figure 3-10: Time resource indication

Considering time correlation of beams across multiple occasions, the Top-N beams does not change too fast within a certain period. In other words, beam resource indicators may overlap between adjacent occasions. For example, unique beam resource indicators of 4 occasions in Figure 3-10 is far less than 32. Since the number of total time-domain occasions is much less than the total number of beam resources, the feedback cost of beam reports can be reduced by using TRI (time resource indicator). More specifically, first step is to determine unique predicted beam resources across multiple occasions, and then feed back corresponding time-domain indications for each unique predicted beam resource indicator. Thus, a TRI method can be considered to further reduce report overhead when multiple occasion results are included in one beam report.

Table 1: UCI overhead reduction of time resource utilization for BM-Case2

|  | Beam resource indicator | Total UCI report overhead | Overhead reduction of method 3 |
| --- | --- | --- | --- |
| Method 1: Implicit time stamp with CRI | 1st occasion: 8 * 5 = 40 bits 2nd occasion: 8 * 5 = 40 bits 3rd occasion: 8 * 5 = 40 bits 4th occasion: 8 * 5 = 40 bits Note: report 8 beams for each occasion, and beam resource indicator needs 5 bits for each reported beam indicated from total 32 beams. | 160 bits | 31.25%~45% |
| Method 2: Implicit time stamp with bitmap | 1st occasion: 32 bits 2nd occasion: 32 bits 3rd occasion: 32 bits 4th occasion: 32 bits Note: 32 bits corresponding to the total 32 beams to indicator reported beams per occasion | 128 bits |  |
| Method 3: TRI assisted report | Beam indicator: 32 bits TRI indicator: 4 * 14 = 56 bits Note: Beam indicator here is used to indicate 14 of unique predicted beam resources from 32 of total beams across multiple occasions, and each TRI indicator is used to indicated time resource for each unique beam (4-bit bitmap). | 88 bits |  |

We evaluate three types of UCI indication methods in Table 1 based on the case in Figure 3-10, where only 14 out of 32 predicted beam resources need to be indicated in one beam report for 4 occasions.

- Method 1 - Implicit time stamp with CRI: 8 CRIs each with 5 bits are reported for one occasion. 4 occasions are mapped to UCI sequentially, where time stamps are indicated implicitly based on the mapping order. Total 160 bits are required for this case.

- Method 2 - Implicit time stamp with bitmap: Similar mapping approach is used as Method 1. The difference is a fixed 32 bits bitmap is used to indicate 8 CRIs in each occasion. Total 128 bits are required.

- Method 3 - TRI assisted report: A 32-bit bitmap is firstly used to indicate the select 14 unique beams for 4 occasions. Further, 14 bitmaps are used to indicate TRI, where each bitmap contains 4 bits, used to indicate which ones from the four occasions the 14 beams locate. Total 88 bits are needed, and it achieves significant report overhead reduction ratio, i.e. 31%~45%, compared with Method 1 or Method 2.

For inference, for UE-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

- Measurement report for NW-side model

  - Report content for data collection

During data collection procedure, network may receive UE logged measurement results from higher layer signaling if timeline of data collection is loose, while L1-layer signaling can be used for data reporting with restricted timeline. Thus, in order to enhance the flexibility of data collection for NW, we prefer to support both L1-layer signaling and high-layer signaling to contain the report of measurement data.

For data collection, for NW-side model, support to report UE measurement results via L1-layer signaling and higher-layer signaling.

In order to provide the NW with sufficient measurement results for model training, especially when a large number of beams are included in Set A and/or Set B (e.g., 64 beams in Set A), the UE needs to be able to provide feedback on these 64 L1-RSRPs or feedback on a subset of the L1-RSRP and corresponding resource indicators. It has been agreed to support the reporting of more than 4 beams in a beam report. Additionally, considering the excessive information contained in one report, it is essential to explore methods to reduce the overhead of the report.

For data collection, for NW-side model, confirming the agreement from SI phase that more than 4 beams can be reported in a beam report.

- The maximum number of reported beam related information in one report is related to UE’s capability.

For data collection, for NW-side model, it is crucial to investigate approaches to minimize the overhead of the report transmitted through L1-layer or higher-layer signaling.

In situations where the number of beams in Set A configured by the NW exceeds the maximum reported beam quantity allowed by the UE's capability, both L1-RSRPs and corresponding resource indicators should be indicated in a beam report to select a subset of beams. Furthermore, as the reporting of L1-RSRP and relative resource indicators are already supported in the current specification, the focus should be on confirming that the same report content can be utilized for data collection with the NW-side AI model.  Further, considering large size of Set A, enhancement on overhead reduction should be supported.

For data collection, for NW-side model, report content supported in current specification can be re-used for BM-Case 1 and BM-Case 2, i.e. L1-RSRPs and corresponding beam information.

For data collection, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 1.

Furthermore, due to beam correlation across multiple measurement occasions, similar as UE side model inference report in section 6.4, temporal domain compression of beam resource indication shall be considered to further reduce report overhead in a beam report with multiple time occasions for NW-side model. We, thus, propose,

For data collection and inference, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 2, e.g. consider to use time domain data compression to reduce overhead.

  - Report content for BM-Case 1, for inference

In RAN1#117 meeting, following agreement was achieved for NW-side model,

Agreement

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specification

It has been determined that if M represents the size of the measurement resource set, the content includes all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set. However, solely defining it in this manner may result in inconsistency understanding in the mapping order of reported L1-RSRPs to resources in the measurement resource set between UE side and NW side. This inconsistency could potentially lead to the network being unable to correctly interpret the corresponding measurement report. Therefore, further definition of the mapping order in the feedback report is necessary for the inference report for the NW-side model.

For inference, for NW-side model, if M = the size of the measurement resource set, report the L1-RSRP in a predetermined order corresponding to the measurement resource set.

Considering overhead reduction for large size of Set B, the enhancement of the quantization range directly results in numerous unnecessary resource indicators and/or corresponding L1-RSRPs being exempt from reporting. Consequently, support for an adaptive number of beams in a beam report should be considered.

For inference, for NW-side model, support to report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, as well as the number of reported beams

Furthermore, the current specification uses resource indicators to indicate which beams are included in the beam report and need to be reported. However, this design is based on the fact that only the top-1 or top-4 measured beams are reported. During model inference with a NW-side model, the UE may need to report a significantly larger number of beams than just the top-1 or top-4 beams. Therefore, it may be worth considering introducing a beam index type indicator to specify the meaning of the beam resource indicators included in a beam report.

For example, if the majority of beams are selected for reporting, the UE could report partial measured L1-RSRPs and the index of lowest-quality beams based on a beam set to the gNB, with the beam indices in the report used to indicate the lowest-quality beams. Similarly, if the minority of beams are selected for reporting, the UE could report partial measured L1-RSRPs and the index of highest-quality beams based on a beam set to the gNB, with the beam indices in the report used to indicate the highest-quality beams as legacy behavior.

For inference, for NW-side model, support to introduce a beam index type indicator to specify the meaning of resource indicator, which indicates the reported beams are the beams with highest quality or lowest quality.

  - Report content for BM-Case 2, for inference

To account for data correlation across multiple measurement periods for BM-Case 2, the report may include time stamp information if one report contains data from multiple periods. Specifically, if UE can’t report data from multiple occasions in one report, NW has to configure RS periodicity to be same as report periodicity to ensure NW has the knowledge of time information for the reported beams for training. This is restrictive. Hence the configuration of RS and report periodicities is more flexible if UE can report multiple measurement occasions in one report. Further, it can also facilitate the utilization of aperiodic report and aperiodic RS measurement.

For inference, for NW-side model, support to report of measurement results of multiple past time instances in one report for BM-Case 2.

  - Report enhancement for BM-Case 1 and BM-Case 2

The quantization overhead cost can be affected by two factors, which are quantization granularity and quantization range. The current NR specification employs 1 dB quantization granularity with a 7-bit quantization range from -140 dBm to -44 dBm for the best measurement beam, while a 2dB quantization granularity with a 4-bit quantization range is used to other beams differentially to the best beam.

It has been demonstrated in [2] that enlarging quantization steps for differential beams has minimal impact on beam prediction performance. Therefore, it may be beneficial to consider reducing report overhead from quantization perspective. Due to immoderately enlarging quantization steps may have a significant impact on prediction performance, it is important to balance report overhead reduction and beam predicted performance. In addition, considering that beams closer to the strongest beam are more important than others with lower L1-RSRP, for example, if the L1-RSRP of the first, second and third beam is -82dBm, -93dBm and -109dBm, respectively, the L1-RSRP of the first and second beams can be quantized more finely, while the L1-RSRP of the third beam can be quantized more coarsely. Thus, using multi-resolution L1-RSRP quantization, such as high-resolution quantization for a group of best RSRPs and low-resolution quantization for others, can help reduce feedback overhead as well as minimizing performance deterioration.

Additionally, existing quantization methods lead to significant performance degradation if quantization range from -140dBm to -44dBm is considered. For cases involving a quantization range, input L1-RSRPs lower than -140dBm are quantized as the lowest L1-RSRP level. However, in terms of UE’s measurement accuracy, -140dBm is far below than its’ noise floor. This implies that any values measured by the UE below the noise floor are actually invalid data. Therefore, it may be worth considering raising the lower limit of the quantization range.

For data collection and inference, for NW-side model, support enhancements on quantization range and quantization step to reduce overhead for measurement results report.

Section 6.4 offers a comprehensive explanation of how predicting multiple future beams can substantially decrease feedback overhead by leveraging on the correlation across multiple time instances. Similarly, in the context procedure with a NW-side model supporting the reporting of buffered measurement results across multiple measurement occasions, the correlation between measurement results from multiple time instances can also be leveraged to reduce feedback overhead. This approach can help streamline the feedback process and optimize the utilization of resources.

For data collection and inference, for NW-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

- AI processing capability

The current specification defines maximum number of CSI processing units and CPU processing timeline for CSI report. Correspondingly, for AI processing units, it is also necessary to consider how to define its timeline and quantity of AI processing units. We, thus, propose,

For UE-side model, further study whether to define AI process capability including re-use or modified the existing CSI computation time and CSI processing units.

- How to initiate data collection procedure

Figure 9-1: data collection procedure for UE-side model

For data collection procedure, where the AI model is deployed at UE side, the following steps are needed:

- In T-1: UE reports its capability information related to data collection purpose.

  - The specific contents of the capability information need to be considered with how the data collection procedure is triggered. For example, when the UE-side data collection procedure is initiated by the UE request signaling, extra information corresponding to model input/output is needed and can be carried through the request signaling. When the UE-side data collection procedure is initiated by NW, the extra information can be determined through the capability report, or through communication between UE-side OTT server and NW.

- In T-2: data collection procedure is initiated by UE request signaling or NW configuration.

- In T-3: gNB configures specific RS for data collection purpose with associated ID.

- In T-4: UE measures resources of Set A and/or Set B for model training based on a pre-measured Rx beam.

The content of UE capability reporting may be linked to the data collection process for the UE-side model, as outlined above. The specifics of data collection procedure initiation are illustrated in the following figures.

Figure 9-2: initiated data collection procedure through UE request

The figure above illustrates that the UE's OTT server sends a data collection request to the UE, and UE initiates data collection procedure via sending base station request signaling. Further, additional information pertaining to the model's input and/or output can also be signaled to NW along with the data collection request. Subsequently, the network can determine when or whether to configure/trigger the corresponding data collection resources based on the request signaling.

Figure 9-3: initiated data collection procedure through NW configuration

In Figure 9-3, the data collection request is sent by the OTT server through higher layer signaling or user plane, and then the NW can initiate the data collection procedure. Both of the aforementioned data collection methods for the UE-side model are feasible. Therefore, for the sake of flexibility, it is recommended to support both initiated behaviors for the data collection procedure of the UE-side model.

For UE-side model, data collection procedure can be initiated by gNB configuration or UE request signaling.

- Beam failure detection/report for model inference

Figure 10-1: BFD RS monitoring procedure

Currently, the UE monitors beam failure detection RS on all serving beams to evaluate whether the beam failure trigger condition is met. If all serving beam metrics meet the pre-defined conditions, the UE reports a beam failure instance indication to the UE higher layer. Conversely, if the UE physical layer determines that no beam failure instance has occurred, no indication is sent to the UE higher layer. For example, in above figure, UE-2 suffers beam failure instance and sends BFI indication to UE higher layer for BFI counter calculation.

In AI based beam prediction, an AI model are specified for model inference to predict Top-N L1-RSRPs or even all of the L1-RSPRs. Further, AI model has ability of temporal domain beam prediction for the prediction of L1-RSPRs at future time occasions. In the aforementioned scenarios, the inference side of the AI model is capable of obtaining all or Top-N predictive L1-RSRPs at multiple future occasions. As such, this can assist the detection of beam failure and no dedicated BFD RS may be needed, if AI model of L1-RSPR prediction enabled.

Figure 10-2: early cell switching

Figure 10-3: suspend transmission

More specifically, if model inference results of all predictive L1-RSPRs or Top-N predictive L1-RSPRs at one future occasion are less than or equal to pre-defined L1-RSRP threshold, UE can assume a hypothetical beam failure instance and reports to the higher layer. Consequently, an early beam failure event can be identified from UE side. Further, UE may perform early beam recovery procedure or even cell switching procedure based on time duration of hypothetical beam failure in Figure 10-2, or suspend transmission can be achieved in NW side within time duration of hypothetical beam failure from UE side in Figure 10-3. Overall, incorporating AI in the beam failure detection procedure has the potential to significantly enhance the reliability and efficiency of beam management.

Support to using AI beam prediction for beam failure detection/report enhancement.

- Conclusions

In this contribution, we discuss some issues on AI/ML for beam management and have the following observations:

- Huge performance degradation can be observed without ensured consistency of Set A and Set B on beam width/beam pointing angle across training and inference.

- It is also expected that mismatched beam indexing mechanisms (mapping between beam index and actual beams) between training and inference would lead to severe performance degradation.

Significant UCI overhead reduction can be achieved by UE-assisted model monitoring compared to direct measurement results report for NW-side AI/ML model.

- The simulation results from TR 38.843 show that using a wrong fixed pattern causes at least 65/60/55 percent performance degradation for the KPI of Top1, Top2/1 and Top 4/1 beam prediction accuracy, respectively.

- Less than 3% average performance decrease among 10 companies submitted in TR 38.843 [2] for the KPI of Top1 and Top2/1 beam prediction accuracy of pre-configured patterns with Tx beam ID information compared to fixed pattern in Set B.

- It can be observed from majority companies in TR38.843 [2] that only 2% performance degradation of Top-1 beam prediction accuracy and almost same performance of Top-2/1 beam prediction accuracy achieved for pre-configured patterns in Set B with Tx beam ID information as input compared to fixed pattern in Set B.

- and proposals:

- For inference, for UE-side model, it can be assumed that same transmission filter for same resource IDs or through other rules (e.g. same ordering in a list) in Set A/Set B is consistent from training to inference if UE receives the same associated ID.

For inference, for UE-side model, support that associated ID representing NW-side additional conditions is provided to UE to ensure consistency between training and inference, as well as to address NW-side proprietary information disclosing issue.

For inference, for UE-side model, along the line of the agreed working assumption on the consistency issue in AI 9.1.3.3, support to prioritize discussion on how UE-side AI model can work with local associated ID and study how to reduce UE side complexity on managing local associated IDs.

For inference, for UE-side model, suggest to further study on multi-cell AI model with local associated ID, including both the case that the model is developed for multi-cell and the case when UE moves to a cell that UE has not collected data before.

For inference, for UE-side model, further study how to determine consistency for a UE sided AI model for multi-cell AI model, e.g. local associated ID can be mapped to an assistant information (cell ID) during data collection.

For performance monitoring, at least support beam prediction accuracy related KPI and L1-RSRP difference as potential metrics, while probability and confidence information should be deferred after related agreement is achieved in inference phase.

For performance monitoring, at least support L1 beam reporting for both UE-side model and NW-side model.

For monitoring report, support report overhead reduction for NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1), as well as for NW-side AI model, e.g., report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

For monitoring report, support to report measured L1-RSRP of indicated beam(s), e.g. current beams indicated by DCI, with measured L1-RSRP of top-k beams for both NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1) and NW-side AI model.

For performance monitoring, for UE-side model, support Option B and Option C, i.e. report the beam prediction accuracy related information and report the RSRP difference information between the measured and predicted, for further study, while Option D and Option E, i.e. probability and confidence information, should be deferred after related agreement is achieved in inference phase.

For performance monitoring, for UE-side model, support following options for further study on what/how to report,

- Opt1: report metric

- Opt2: report event

- Opt3: based on event, report event and metric(s)

- Other options cannot be precluded

For performance monitoring, for NW-side model, support to report metrics for UCI reporting overhead reduction.

For inference, for UE-side model, support UE to recommend some preferred Set B patterns which were trained during the UE-side model training phase.

For inference, for UE-side model, support gNB to configure a resource list including multiple Set B patterns and activate resources of one Set B pattern from the list by signaling during the inference phase.

For UE-side model, support to configure full and/or subset of Set A associated with predicted beam report to address interference issue.

For inference, for UE-side model, Set A or resources in Set A should be indicated as virtual Set or virtual resource, which does not require measurement on these resources for the report.

For inference, for UE-side model, support below potential options of different alternatives for the configuration of inference results reporting,

- Alt1-Opt1: one CSI-ReportConfig, only includes Set B, Set A without any configuration can be assumed by associated ID.

- Alt1-Opt2: one CSI-ReportConfig, only includes Set B, Set A is configured out of the CSI-ReportConfig and associated to Set B

- Alt1-Opt3: two CSI-ReportConfig, one CSI-ReportConfig includes Set B and another CSI-ReportConfig includes Set A.

- Alt2: one CSI-ReportConfig and one CSI-ResourceConfig, the CSI-ResourceConfig includes Set B and Set A

- Alt3-Opt1: one CSI-ReportConfig with two CMR, one CMR includes Set B and another CMR includes Set A

- Alt3-Opt2: one CSI-ReportConfig with one CMR, the CMR includes Set B and Set A

For inference, for UE-side model, suggest to postpone any consideration of Alt4 until the discussion on the configuration of Set A is fully completed.

For inference, suggest to configure different beams of Set B for measurement corresponding to different historical periods for BM-Case 2.

For inference, further study how to avoid unnecessary measurements and RS overhead for CSI-RS occasions existed in the prediction window for BM-Case 2.

For data collection and inference, for UE-side model, support to use quasi-best Rx beam for Set A and/or Set B measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A/B.

For inference, for NW-side model, support gNB to configure a resource list including multiple Set B patterns and active resources of one Set B pattern from the list by signaling during the inference phase.

For NW-side model, there is no motivation to configure Set B and Set A simultaneously in one report.

For data collection and inference, for NW-side model, support to use quasi-best Rx beam for Set A measurement, where quasi-best Rx beam is derived from P3 measurement on a small number of Tx beams from Set A.

For inference, for UE-side model, support to report predicted L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest predicted value of L1-RSRP, as well as the number of reported beams.

For inference, for UE-side model, when UE reports L1-RSRP of Top-k beams predicted from Set A, and for those that belongs to Set B, the UE reports the actually measured RSRP corresponding to the beam.

For inference, for UE-side model, support time stamp information as beam content for BM-Case2.

For inference, for UE-side model, support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions.

For inference, for UE-side model, support beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams per predicted instance for content in the report for BM-Case 2.

For inference, for UE-side model, support new report quantity to differentiate between AI-based predicted report with legacy measured report.

For inference, for UE-side model, support to report predicted L1-RSRP of indicated beam(s), e.g. current beam indicated by TCI, with predicted L1-RSRP of top-k beams in one prediction report.

For inference, for UE-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

For data collection, for NW-side model, support to report UE measurement results via L1-layer signaling and higher-layer signaling.

For data collection, for NW-side model, confirming the agreement from SI phase that more than 4 beams can be reported in a beam report.

- The maximum number of reported beam related information in one report is related to UE’s capability.

For data collection, for NW-side model, it is crucial to investigate approaches to minimize the overhead of the report transmitted through L1-layer or higher-layer signaling.

For data collection, for NW-side model, report content supported in current specification can be re-used for BM-Case 1 and BM-Case 2, i.e. L1-RSRPs and corresponding beam information.

For data collection, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 1.

For data collection and inference, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 2, e.g. consider to use time domain data compression to reduce overhead.

For inference, for NW-side model, if M = the size of the measurement resource set, report the L1-RSRP in a predetermined order corresponding to the measurement resource set.

For inference, for NW-side model, support to report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, as well as the number of reported beams

For inference, for NW-side model, support to introduce a beam index type indicator to specify the meaning of resource indicator, which indicates the reported beams are the beams with highest quality or lowest quality.

For inference, for NW-side model, support to report of measurement results of multiple past time instances in one report for BM-Case 2.

For data collection and inference, for NW-side model, support enhancements on quantization range and quantization step to reduce overhead for measurement results report.

For data collection and inference, for NW-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions.

For UE-side model, further study whether to define AI process capability including re-use or modified the existing CSI computation time and CSI processing units.

For UE-side model, data collection procedure can be initiated by gNB configuration or UE request signaling.

Support to using AI beam prediction for beam failure detection/report enhancement.

References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

3GPP TR 38.843