3GPP TSG RAN WG1 #118	              	     R1- 2406262

Maastricht, NL, August 19th – 23th, 2024

Source:	OPPO

Title:	Discussion on 3-antenna-port codebook-based transmissions

Agenda Item:	9.2.3

Document for:	Discussion and Decision

Introduction

In 3GPP RAN #102 meeting, a new work item on further enhancement of MIMO was approved [1], which includes 3-antenna-port codebook-based transmissions:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

In this contribution, we further discuss remaining issues and some other issues related to 3-antenna-port uplink transmissions.

Discussion

2.1 RRC parameter for 3Tx transmission

Following agreements are achieved for 3Tx transmission in previous meetings:

Agreement For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port

For codebook-based uplink transmission, a 3Tx UE can report a maximum three SRS ports per SRS resource. When the gNB configures a 4-port SRS resource, the UE can mute the 4th port to use the resource as a 3-port SRS resource. The UE behavior is clear without ambiguity. Therefore, there is no need to introduce RRC parameter for 3Tx operation.

Proposal 1: No need to introduce RRC parameter for 3Tx operation.

2.2 SRS antenna switching

On SRS antenna switching, the situation from RAN #104 is as following:

- RAN4 is tasked to specify RF requirements for 6 Rx UE. RAN4 has revised the WID to include 3T6R antenna switching according to the Rel-19 revised WID RP-241656 [2].

- Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

Considering the RAN4 discussion on RF requirements for 6 Rx UE and addressing the issue of SRS antenna switching for 6 Rx UE, 3T6R SRS antenna switching can be considered in Rel-19. This would be similar to 1T2R, with the main distinction of each SRS resource in a given set for 3T6R consisting of three SRS ports, thus minimalizing the impact on specifications. If 3T6R is not supported, then needs to fallback to 2T6R. This, however, increases SRS sounding overhead and may lead to a loss in the DL throughput performance, e.g., more sounding OFDM symbols and gaps.

Proposal 2: If 3T6R SRS antenna switching is supported in Rel-19,

- Support all resource types for SRS resource set used for antenna switching.

- Use two 4-port SRS resources, with one SRS port muted on each resource.

2.3 Non-codebook transmission

Although non-codebook transmission falls outside the scope of the WID, the SRI indication for non-codebook-based operation with up to 3 SRS resources and 1 to 3 layers is already specified in 38.212. Therefore, in our understanding, this seems quite easy to specify.

In addition, we perform system-level simulation to evaluate the performance of 2Tx non-codebook transmission and 3Tx non-codebook transmission. Through system-level simulation, it has been observed that 3Tx non-codebook transmission can achieve a cell average spectral efficiency gain of 14% over 2Tx non-codebook transmission. Furthermore, it has also been noted that the 5% spectral efficiency gain of 3Tx non-codebook transmission over 2Tx non-codebook transmission is approximately 14%.

Table 1 Performance comparison between 2Tx non-codebook and 3Tx non-codebook

|  | Cell average | 5% |
| --- | --- | --- |
| 2Tx and non-codebook transmission | 7.58 | 0.43 |
| 3Tx and non-codebook transmission | 8.64 | 0.49 |

Observation 1: 3Tx non-codebook transmission can achieve a cell average spectral efficiency gain of 14% over 2Tx non-codebook transmission.

Observation 2: For 3Tx non-codebook transmission, 5% spectral efficiency gain is around 14% when compared to 2Tx non-codebook transmission.

2.4 Partial-coherent transmission

The agreement has been reached on 3Tx non-coherent codebooks. The question remains whether partial coherent codebooks can be supported, provided that it is out of the WID’s scope. In regards to partial-coherent codebook transmission, we have conducted a system-level simulation to assess the performance of 3Tx partial coherent codebook transmission utilizing full power mode 0. Through system-level simulation, it has been observed that 3Tx partial coherent codebook transmission with full power mode 0 can achieve a cell average spectral efficiency gain of 8.5% over 2Tx non-coherent transmission with full power mode 0. Moreover, it has been observed that 3Tx partial coherent codebook transmission with full power mode 0 achieves a cell average spectral efficiency gain of 8.2% over 2Tx full coherent transmission. Regarding the 5% spectral efficiency gain, the performance is almost identical between 3Tx partial coherent transmission and 2Tx non-coherent/full coherent transmission.

Table 2 Performance comparison for 3Tx partial coherent codebook and 2Tx codebooks

|  | Cell average | 5% |
| --- | --- | --- |
| 2Tx, non-coherent, full power mode 0 | 7.54 | 0.41 |
| 2Tx, full coherent | 7.56 | 0.42 |
| 3Tx, partial coherent, full power mode 0 | 8.18 | 0.42 |

Observation 3: 3Tx partial coherent codebook transmission with full power mode 0 can achieve a cell average spectral efficiency gain of 8.5% over 2Tx non-coherent transmission with full power mode 0.

Observation 4: 3Tx partial coherent codebook transmission with full power mode 0 can achieve a cell average spectral efficiency gain of 8.2% over 2Tx full coherent transmission.

Observation 5: For 5% spectral efficiency gain, the performance is almost identical between 3Tx partial coherent transmission and 2Tx non-coherent/full coherent transmission.

Conclusions

In this contribution, we further discuss remaining issues and some other issues related to 3-antenna-port uplink transmissions. Based on the discussion, the following proposals and observations are made:

Observation 1: 3Tx non-codebook transmission can achieve a cell average spectral efficiency gain of 14% over 2Tx non-codebook transmission.

Observation 2: For 3Tx non-codebook transmission, 5% spectral efficiency gain is around 14% when compared to 2Tx non-codebook transmission.

Observation 3: 3Tx partial coherent codebook transmission with full power mode 0 can achieve a cell average spectral efficiency gain of 8.5% over 2Tx non-coherent transmission with full power mode 0.

Observation 4: 3Tx partial coherent codebook transmission with full power mode 0 can achieve a cell average spectral efficiency gain of 8.2% over 2Tx full coherent transmission.

Observation 5: For 5% spectral efficiency gain, the performance is almost identical between 3Tx partial coherent transmission and 2Tx non-coherent/full coherent transmission.

Proposal 1: No need to introduce RRC parameter for 3Tx operation.

Proposal 2: If 3T6R SRS antenna switching is supported in Rel-19,

- Support all resource types for SRS resource set used for antenna switching.

- Use two 4-port SRS resources, with one SRS port muted on each resource.

References

- RAN Meeting#102, RP-234007, New WID: NR MIMO Phase 5

- RAN Meeting#104, RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

Simulation parameters

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreements

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of

- Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

- Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set,

The value for X is FFS, and it will be determined according to the selected alternative.

Agreement

For a 3TX UE, down-select one of the following options for the number of PTRS ports,

- Option-1: A single PTRS port is supported.

- Option- 2: Up to 2 PTRS port may be configured.

Agreement

For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association.

Agreement

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

Agreement

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Conclusion

There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 3Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% Outdoor FWA: 100%, 0% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°] |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | Rel-15 2Tx non-coherent |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreement

For performance evaluation of 3TX UE, consider following reference configurations,

- A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ,

  - For example:		 |←0.5λ→|←0.5λ→|

- A configuration of a cross-polarized and a single-polarized antennas,

  - For example:		 ×←0.5λ → ⁄

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

Agreement

For codebook-based transmission by a 3TX UE,

- Only PUSCH antenna ports 1000, 1001, 1002 are used

- Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig,

  - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication.

Above is only for single panel transmission.

Agreement

To support codebook-based UL transmission by a 3TX UE, the agreed rank1 precoders in RAN1#116 can also be used when transform precoding is enabled (DFT-s-OFDM ).

Agreement

To indicate precoding information for codebook-based UL transmission by a 3TX UE,

- Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated

- TPMI field is 2 or 3bits for 3-antenna-port transmission

  - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM

  - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1,

- Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

  - FFS muting mechanism

where X can be up to 2, subject to UE capability.

Agreement

- For codebook-based UL transmission by a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Note: PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT_RS port 0, and PUSCH antenna port 1001 is associated with PT_RS port 1

- Number of bits used for the indication

  - 1 bit

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource,

- Option 3: Always a same port is muted, e.g., the 4th port

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

Agreement

For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0).

- FFS: Whether specification needs to be updated to reflect the above

Agreement

For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

- Alt2: 2-bit indication

PTRS-DMRS association when 1 PT-RS port is configured

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port Reserved |

Agreement

For a 3TX UE, support Rel-17 M-TRP PUSCH repetition where,

- Two SRS resource sets, each with up to 2 of 4-port SRS resources are configured,

Note: The configured 4 port SRS resources are used to enable 3-port SRS transmission

Agreement

For codebook-based UL transmission by a 3TX UE, subject to its capability,

- A 3TX UE may report a maximum number of 3 layers

- A 3TX UE may report a maximum number of SRS ports of up to 3

Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.