3GPP TSG RAN WG1 #118		R1-2406294

Maastricht, NL, August 19th – 23rd, 2024

Agenda item: 	9.5.3

Source: 	Xiaomi

Title:	Discussion on adaptation of common signal and channel transmissions

Document for:	Discussion

1 Introduction

In RAN1#117 meeting, the following agreements on adaptation of common signal and channel transmissions were achieved:[1]

Conclusion There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

In this contribution, we provide our views on the potential mechanisms for adapting common signal/channel transmissions, focusing on SSB, PRACH and paging.

2 Discussion

2.1 Generic issue for adaptation of common signal/channel transmissions

2.1.1 Clarification on target scenario and UE type

In the WID of Rel-19 NES, the applicable scenario for on-demand SSB and on-demand SIB1 is clearly announced, which is shown as below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

However, the target scenario for adaptation of common signal/channel transmissions is not crystal clear as on-demand SSB and on-demand SIB1. There is a risk that different companies may have different understanding. On the other hand, the target scenario impacts the detail design for adaptation. Therefore, it is necessary to first clarify common understanding on target scenarios for adaptation of common signal/channel transmissions.

Taking SSB as an example. SSB is constantly and periodically transmitted with characteristic of beam sweeping. Despite of UE status, i.e., RRC_CONNECTED UE or RRC_IDLE/INACTIVE UE, SSB is essential for the following procedure:

- RRM measurement including cell selection and re-selection, mobility, SCell activation/De-activation, etc.

- RLM measurement, i.e., whether the current cell is qualified for service.

- Beam management and beam failure recovery.

- RACH procedure.

Though it may introduce different issues for RRC_CONNECTED UE and RRC_INACTIVE/IDLE UE respectively, adaptation of SSB should be applied to both of UE in order to maximize energy saving gain. Otherwise, it is impossible to target specific UE considering there are always different UE in different RRC status at the same time.

In RAN1#116 meeting, the applicable scenarios for the adaptation mechanisms of SSB in time-domain were discussed. The following scenarios were identified for further study:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

In order to obtain NES gain as much as possible, we don’t see rationales to put any restriction on the applicable scenarios for the adaptation of SSB transmission. Different from on-demand SSB, which is impossible to applied to PCell, the adaptation of SSB transmission doesn’t target to turn off SSB. In the other words, SSB is always available on a cell with an adjustable characteristic. Therefore, we don’t see any obstacles for supporting SSB adaptation on PCell.

Proposal 1: For the adaptation mechanisms of SSB in time-domain, it can be applied to the following scenarios:

- UE in idle/inactive and connected mode

- PCell and SCell(s)

In RAN1#116bis meeting, we further discussed what kind of SSB can be supported for the purpose of time domain adaptation. Three options were raised for further discussion:

- Option 1: adaptation for CD-SSB

- Option 2: adaptation for SSB that is not CD-SSB

- Option 3: adaptation for SSB not on sync raster

As analyzed in our companion contribution, we think whether SSB on SCell is CD-SSB or NCD-SSB depends on gNB implementation.[2] If SSB adaptation in time domain is supported for PCell/SCell, typically SSB has to be cell defined as UE needs to complete cell search procedure and access to this cell. On the other hand, if NES UE is RedCap capable UE, it is also possible that SSB is not on sync raster. However, we need to first discuss whether to apply NES techniques to RedCap UE. For SCell, we don’t see the necessity to put any restriction on SSB.

Proposal 2: For the adaptation of SSB in time-domain,

- For PCell, adaptation can be applied to CD-SSB.

- For SCell, adaptation can be applied to CD SSB, NCD-SSB and SSB not on sync raster.

2.1.2 Consciousness on adaptation of common signal/channel

It is well known that there are many physical procedures ties to SSB transmission, for example:

- UE needs to know the RE occupied by SSB so that it can correctly understand how to decoding downlink packet.

- UE needs to know the OFDM symbols occupied by SSB so that it can correctly cancel uplink transmission once uplink transmission partially overlaps with SSB symbol.

Furthermore, if SSB is adapted without informing UE, UE has to try to receive SSB with assuming it is transmitted via default pattern. It leads to power wastage at UE side which is unfriendly for UE power saving. More importantly, UE may conduct unexpected measurement on the symbols which SSB is actually not transmitted.

Based on the above analyses, we think it is important that UE have the full knowledge of SSB adaptation in time domain.

For CFRA, UE transmits preamble following PDCCH order from gNB. If gNB prefers to receive preamble with a larger periodicity, it can accordingly transmit PDCCH order in a suitable PDCCH monitoring occasion. On the other hand, CFRA can also be triggered by higher layer without gNB indication. In this case, CFRA failure is inevitable if UE tries to transmit on a RO before PRACH adaptation. For CBRA, UE transmits preamble on one of the configured RO in an opportunistic manner. gNB may or may not reply the UE with expected preamble index and resolve the contention. From this point of view, it may be fine to keep adaptation of PRACH transparent to UE. However, a very large PRACH periodicity may wrongly leads to unexpected failure of random access. For example, there are some restrictions for PRACH transmission and the overall random access procedure:

- preambleTransMax defines the maximum number of preamble UE can transmit before random access procedure is completed.

Obviously, if adaptation of PRACH is transparent to UE, it is highly possible that UE misunderstands the situation and treat random access failure happen.

Another issue is that transparent mechanism makes UE to transmit PRACH in vain on the unavailable RO which UE cannot recognize. It is a huge waste of UE power which is definitely unexpected. UE may also wrongly drop other uplink transmission colliding with a valid RO on which it intends to transmit PRACH.

For paging, it is used for paging a user to pick up a call or indicating all users within a cell the SI will be updated. UE determines its PO within a DRX circle depending on the parameters provided by SIB1 and its own UE_ID. It only tries to receive paging on a particular PO. If paging transmission is adapted, the PO distribution will be totally changed within DRX circle. UE may no longer to receive any paging as normal. It means the overall paging functionality is disabled. The system performance will be significantly degraded. Furthermore, UE has to monitor paging as normal on the pre-adapted PO and receive nothing. In the end, UE power is wasted.

Proposal 3: UE needs to have the knowledge on adaptation of common signal/channel transmissions, including SSB, PRACH and paging.

2.2 Adaptation of SSB in time domain

SSB is one of the most fundamental signal for 5G NR system, which is the basis for many procedures, including RRM measurement, RLM measurement, beam management, etc.  Accordingly, there is non-negligible impact on UE behavior and RAN1/RAN4 procedure if the time pattern of SSB is changed. Therefore, minimum or zero influence on UE behavior coming along with adaptation of SSB in time domain should be pursued. In current RAN1 specification, the following restriction and rule are captured:

- For DL RE mapping, UE assumes any RE allocated for DL transmission overlaps with SSB in time and frequency domain is set to zero.

- For UL transmission, UE assumes uplink transmission cannot happen on any symbol overlaps with SSB reception.

- For initial access, UE assumes the periodicity of SSB transmission equals 20ms.

- After initial access, UE assumes SSB is transmitted with predefined SSB pattern or with SSB pattern indicated via SIB1.

On the other hand, the importance of SSB and its periodic character makes SSB an obvious reason of network energy consumption, which lead to significant increase on OPEX. Therefore, adaptation of SSB in time domain, e.g., enlarge transmission periodicity when traffic load is very low, is very helpful to reduce unnecessary power consumption and provide more opportunity for NES gNB converts into sleep mode.

Proposal 4: For adaptation of SSB in time domain, the impacts on UE behavior need to be carefully studied, following aspects can be considered as starting point:

- The impact on DL RE mapping

- The impact on UL transmission

- The impact on initial access

- UE’s assumption on SSB pattern

2.2.1 Physical structure of SSB

Changing SSB physical structure means the time pattern for a particular SSB block, e.g., the starting symbol within a SSB burst and the GAP between two adjacent SSB, can be adapted to a brand new structure. It means non-NES UE cannot receive the new SSB as it is non-backward compatible. In Figure 1, SSB burst case A to case E are illustrated. It can be observed that the default SSB pattern is determined by SCS and frequency range, which define the number of SSB within 5ms and the starting OFDM symbol index of SSB within a burst.

Per the discussion happened during Rel-18 study item, there are two potential mechanisms to change the physical structure of SSB burst, i.e., remove part of channel/signal within current SSB and remove the gap between two adjacent SSB within a burst.

Figure 1 Physical structure of current SSB burst

From our understanding, the motivation of defining such kind of new physical structure for SSB can be summarized as below:

- Simplify the functionality of SSB, e.g., remove PBCH while keep PSS/SSS, so as to sustain DL synchronization during SSB is off.

- Compress SSB burst, e.g., remove the GAP between two adjacent SSB, so as to make sure gNB can finish SSB transmission as soon as possible. Accordingly, gNB may have more opportunities to enter sleep mode.

For simplifying the functionality of SSB, it comes from the assumption that SSB is turned off so that network has to provide another downlink signal for DL synchronization. Therefore, we think it should be discussed under the umbrella of on-demand SSB. Rather than introducing a simplified SSB, it seems more suitable to define/introduce a new DL reference signal for DL synchronization during SSB off if necessity.

Proposal 5: Simplified SSB should be discussed under the umbrella of on-demand SSB, if needed.

For compact SSB, it can reduce the time required for a SSB burst transmission. In theory, it helps gNB to convert to sleep mode so that additional energy saving gain can be obtained. In Table 1, we provide the number of slots occupied by SSB within a SSB burst periodicity for legacy SSB pattern and compact SSB pattern, respectively. Taking SSB pattern case A as example, the following can be observed:

- For legacy SSB pattern with 5ms periodicity, SSB occupies 4 slots out of 5 slots, i.e., the ratio between SSB-slot and SSB periodicity is 4/5.

- For legacy SSB pattern with 160ms periodicity, SSB occupies 4 slots out of 160 slots, i.e., the ratio between SSB-slot and SSB periodicity is 4/160.

- For compact SSB pattern, i.e., there is no gap for two adjacent SSB, SSB occupies 16/7  slots out of 5 slots, i.e., the ratio between SSB-slot and SSB periodicity slot is 3/5.

For this case, compact SSB pattern can reduce 1 slot for SSB transmission compared to legacy SSB pattern with 5ms periodicity. However, the benefit for large SSB periodicity is pretty small and can be neglected.

The periodicity of SSB circle can be 20ms to 160ms in the current specification. Even if periodicity extension is not introduced in Rel-19, the duration reduction coming along with compact SSB is trivial compared with the transmission periodicity of SSB. From this point of view, the benefit from compact SSB is negligible. Furthermore, there are many restrictions related to SSB, including:

- SSB symbol cannot be converted into UL symbol.

- Uplink transmission is cancelled if any symbol conflicts with SSB symbol.

- UE assumes zero value on any RE of DL transmission overlaps with SSB RE.

- Continuous SSB may have significant impacts on the resource allocation for SIB and paging, e.g., we have to carefully take look into the tables for CORESET#0 determination especially for SSB and CORESET#0 multiplexing pattern1.

If compact SSB does need to be specified, the above issues have to be resolved one by one. Considering the jumbo specification impacts and trivial energy saving gain, we don’t think compact SSB should be on the table in Rel-19 NES.

Table 1 Reduction on transmission time coming along with compact SSB

| SSB case | Legacy SSB(5ms) | Legacy SSB (160ms) | Compacted SSB |
| --- | --- | --- | --- |
| A | 4 slot / 5 slot | 4 slot/ 160 slot | 16/7 slot/ 5slot |
| B | 4 slot /10 slot | 4 slot/ 320 slot | 16/7 slot / 10 slot |
| C | 4 slot /10 slot | 4 slot/ 320 slot | 16/7 slot / 10 slot |
| D | 32 slot / 40 slot | 32 slot / 1280 slot | 48/7 slot / 40 slot |
| E | 64 slot / 80 slot | 64 slot / 2560 slot | 96/7 slot / 80 slot |
| F | 64 slot / 160 slot | 64 slot / 5120 slot | 96/7 slot / 160 slot |
| G | 64 slot / 320 slot | 64 slot / 10240 slot | 96/7 slot / 320 slot |

Proposal 6: Compact SSB is not pursued in Rel-19 as it only brings very limited NES gain but jumbo specification impacts.

2.2.2 Potential adaptation of SSB transmission

Regarding to adaptation of SSB in time domain, it is straightforward that we should at least consider the transmission periodicity and the content of SSB burst. Hence, we focus on the adaptation of SSB in terms of periodicity and available SSB within a SSB burst.

Adaptation of SSB periodicity

In order to harvest energy saving gain as much as possible, it is beneficial to adjust the periodicity of SSB depending on the traffic load or the number of UE camping on the serving cell. For example, gNB can transmit SSB with a larger periodicity than a default periodicity for energy saving purpose.

There are different assumptions on SSB periodicity for different UEs within the same cell. For UE in initial access procedure, it assumes SSB periodicity equals to 20ms with one of SSB case defined in TS38.213. For UE already obtains SIB1, it follows the configuration from parameter named as ssb-PeriodicityServingCell, which provides a periodicity from {ms5, ms10, ms20, ms40, ms80, ms160}. In this sense, adaptation of SSB periodicity has already been partially available in the current specification. Furthermore, it is straightforward that the larger SSB periodicity, the better energy saving performance. For example, a SSB periodicity as large as 1280ms can be introduced. Therefore, SSB periodicity may be enlarged in order to obtain more energy saving gain.

On the other hand, a smaller SSB periodicity can also be considered to expedite RAN1/RAN4 procedure, e.g., SCell activation for unknown cell. It should be noticed that the SSB transmission with small periodicity should be followed by that of larger periodicity. In the other hand, we need to consider dynamically transition between different SSB periodicity.

Proposal 7: The SSB transmission periodicity can be adapted to larger or smaller than a default SSB transmission periodicity.

- Default SSB transmission periodicity is the one predefined by SSB pattern or indicated by SIB1.

Adaptation of SSB number within a burst

According to the current specification, network can configure the contents of a SSB burst, i.e., the actual SSBs transmitted within a SSB burst which are selected from the default SSB pattern defined in TS38.213. To be specific, actual SSBs contained in a SSB burst are provided by the following IE:

ssb-PositionsInBurst SEQUENCE { inOneGroup BIT STRING (SIZE (8)), groupPresence BIT STRING (SIZE (8)) OPTIONAL -- Cond FR2-Only },

Theoretically, the less SSB to be transmitted, the more NES gain will be harvested. On the other hand, the benefit from SSB reduction within a SSB burst only is certainly small, if not negligible, compared to adaptation of SSB burst periodicity. In order to investigate the deference between SSB reduction within a SSB burst only and adaptation of SSB periodicity, we provide quantitative analysis assuming the reference SSB burst pattern is Case A, which is defined as below:

- Case A - 15 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes of . For carrier frequencies smaller than or equal to 3 GHz, . For carrier frequencies within FR1 larger than 3 GHz, .

In this example, the periodicity of reference SSB burst is assumed to be 5ms. The following three alternatives are brought up:

- Alternative#1: each SSB burst consisting of 8 SSB as defined by case A, and transmission periodicity is 5ms.

- Alternative#2: only the first 4 SSBs within SSB burst are transmitted which is configured by ssb-PositionsInBurst, and transmission periodicity is 5ms.

- Alternative#3: each SSB burst consisting of 8 SSB as defined by case A, and transmission periodicity is 40ms.

Figure 2 Examples for SSB reduction only and SSB periodicity adaptation

Based on the aforementioned assumption, the number of occupied OFDM symbols by SSB every 40ms are shown in Table 2. It can be observed that both alt.2 and alt.3 can significantly reduce the number of OFDM symbols occupied by SSB. It is obvious that alt.3 can provide more NES gain because the ratio of SSB symbols within a 40ms periodicity can be as low as 5.71%. The larger periodicity is adapted, the more significant NES gain. Hence, SSB reduction within a SSB burst only is not as convincing as periodicity adaptation. More importantly, a larger periodicity provides more opportunity for gNB to transform into sleep mode, which means power consumption can be further reduced. On the other hand, gNB still need to wake up and transmit SSB every 5ms with alt.2. Surely gNB doesn’t have as many chances as alt.3 if alt.2 is applied.

Table 2: Number of OFDM symbols occupied by SSB within 40ms

| SSB transmission assumption | Number of occupied OS within 40ms | Ratio of SSB symbol |
| --- | --- | --- |
| Alternative#1 | 256 | 256/560=45.7% |
| Alternative#2 | 128 | 125/560=22.85% |
| Alternative#3 | 32 | 32/560=5.71% |

Proposal 8: SSB reduction/adaptation within a SSB burst, if needed, should work together with periodicity adaptation for NES purpose.

2.2.3 Mechanisms for adaptation of SSB in time domain

First of all, explicit indication informing UE SSB periodicity is already available, which is carried by SIB1. gNB shall transmit SSB aligning with the periodicity and SSB index configuration. For UE in initial access that has not received SIB1, it assumes the SSB periodicity equals 20ms. No matter what periodicity and which SSBs are transmitted, default operation such as initial access cannot be impacted.

It is possible that gNB adjust SSB periodicity with a transparent mechanism, i.e., gNB can transmit SSB with a larger periodicity for energy saving purpose without informing UE. The default UE behavior and procedure should be guaranteed, e.g., network should make sure a UE initially accesses to the cell can still receive SSB with the assumption of 20ms SSB periodicity. Considering it is totally transparent to UE, it will impact RRM, RLM and BM which needs to be seriously studied.

Although it is possible gNB adjusts SSB periodicity via SIB1, it is a kind of one-shot configuration which is lack of flexibility. Frequently updating SIB1 will bring unnecessary SIB1 overhead, increase UE’s effort on receiving and processing SIB1, and also consume extra resources for the transmission of DCI format 1_0 carrying short message indicating system information modification.

To address the drawbacks coming along with the aforementioned mechanism, a dynamically indication on adaptation of SSB periodicity can be considered.

In RAN1#117 meeting, it was agreed that option 1 is supported, i.e., adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s). Considering SSB is a cell-specific channel/signal which needs to be received/monitored by different UEs, the SSB burst periodicity value need to accommodate UE assumption and implementation. From this perspective, the existing values for SSB burst periodicity should be reused, i.e., {20ms, 40ms, 80ms, 160ms}.

In order to fully exploit NES gain from dynamic indication, the adaptation indication can be carried by common DCI. It is unnecessary to define new DCI format for SSB periodicity adaptation. Considering there are sufficient spare bits in DCI format 1_0 scrambled by SI-RNTI, P-RNTI or RA-RNTI, gNB can indicate UE which SSB burst periodicity is applied via DCI format 1_0 transmitted in CSS.

Proposal 9: Adaptation of SSB burst periodicity is indicated by DCI format 1_0 transmitted in CSS.

- The DCI format 1_0 is scrambled by SI-RNTI, P-RNTI or RA-RNTI.

- The spare bits in DCI format 1_0 is used to indicate SSB periodicity.

- Existing SSB periodicity values are reused as starting point, i.e., {20ms, 40ms, 80ms, 160ms}.

2.3 Adaptation of PRACH in time domain

Basically, there are two kinds of PRACH, which can be configured by cell-specific signaling and UE-dedicated signaling respectively:

- PRACH associated with CBRA, including 4-step RACH and 2-step RACH.

- PRACH associated with CFRA, including 4-step RACH and 2-step RACH.

Despite of RACH type, the resource for PRACH, e.g., time domain resource, frequency-domain resources, spatial domain resource, has to be a subset of those provided by SIB1. To be specific, RACH-ConfigCommon and RACH-ConfigCommonforTwoStepRA provide the super set of PRACH resources for all the UEs belonging to the serving cell. No matter what kind of PRACH adaptation is adopted, it will impact the association between valid RO and SSB. Valid RO and SSB association are common to any kind of PRACH transmission.

Proposal 10: The adaptation of PRACH can be applied to CBRA and CFRA, including 4-step RACH and 2-step RACH.

2.3.1 Configuration of additional PRACH resource for adaptation

UE derives time domain resources and frequency domain resources for PRACH according to RACH configuration index and msg1-FDM/msg1-FrequencyStart, respectively. Regarding how to configure additional PRACH resource, it was agreed that a PRACH configuration index is needed. Regarding whether a same RACH configuration index as legacy PRACH resource is provided for additional PRACH resource, there are two directions:

- Option 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource. Additional parameters are needed for determining the additional PRACH resources. In the other words, legacy PRACH resources play a role as reference resource for determining additional PRACH resources.

- Option 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource.

From technical perspective, we believe either way works. Option 2 implies that the additional PRACH resources has to be determined by one of the configurations defined in TS38.211. The time domain resources for additional PRACH may be lack of flexibility to some extent. Option 1 treat legacy PRACH resources as reference and determine additional PRACH resources via adjusting relevant parameters, e.g., periodicity, time offset, (x,y) value and slot number. In theory, option 1 is more flexible as it doesn’t need to follow the restriction of current RACH configuration. On the other hand, option 2 is simpler.

Regarding whether muting/masking ROs is needed, we don’t see the necessity. If a subset of additional PRACH resources fully overlapped with legacy PRACH resources, they should be treated as legacy PRACH resources instead of additional ones.

Proposal 11: For adaptation of PRACH in time domain, the additional PRACH resources are configured based on a same PRACH or a different configuration index from the PRACH configuration index for the legacy PRACH resources.

2.3.2 Mechanisms for adaptation of PRACH in time domain

Network can inform UE the detail adaptation for PRACH either via explicit mechanisms or implicit mechanisms. Currently, the RACH configuration is provided by SIB1 and UE-dedicated RRC signaling. gNB can adjust available RACH resources based on current specification. Take common configuration as an example, gNB can send short message to inform UE that SI is updated. UE will try to receive SIB1 in the next adjacent SIB transmission period and obtain the new bunch of configurations for PRACH. To this point, PRACH adaptation is done with the penalty of increasing latency and more power consumption from more frequent paging transmission. To address these problems, gNB can also adapt PRACH transmission in time domain via new mechanisms. At least the following two directions can be further studied:

- Inform PRACH adaptation in time domain via dynamic signaling, e.g., PDCCH. gNB can directly indicate the intended RACH configuration or indicate a scaling factor for the periodicity of RACH configuration. Meanwhile, we need to keep in mind that we should also consider the potential impacts on non-NES UE. Furthermore, standardization efforts should also be carefully assessed.

- The PRACH adaptation in time domain is determined or triggered by certain conditions, e.g., if SSB is turned off or the SSB periodicity is adapted to very long, it may be unnecessary to receive PRACH in regular way as it is not helpful to reduce power consumption at gNB side.

- The PRACH adaptation in time domain is indicated via reusing existing mechanism, i.e., gNB indicate UE an intended RACH configuration via updating SI. In this mechanism, gNB first has to transmit short message, which bring more power consumption and more resource consumption on paging transmission. The overall delay of PRACH adaptation should also be considered.

In RAN1#117 meeting, the following options are identified for further down selection:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

- Option 5: Enhanced cell DRX

For option 1, the overall delay for a UE to successfully decode higher layer signaling jeopardize the performance of PRACH adaptation. It may not be able to match the changing conditions in the network. The overhead is also a concern if UE-dedicated RRC signaling is employed. Furthermore, the advantage of higher layer signaling based PRACH resource adaptation compared to the existing mechanism of RACH configuration is unclear.

The basic idea of option 2 and option 4-rev1 is to reuse existing DCI format to indicate PRACH resource adaptation. Thanks to the in-time nature of L1 signaling, option 2 and option 4-rev1 can achieve the best NES gain. The key difference between option 2 and option 4-rev1 is the granularity of adaptation. Option 2 can indicate which predefined RACH configuration is available, i.e., legacy RACH configuration or additional RACH configuration. Option 4-rev is able to adapt PRACH resource in a much smaller granularity, e.g., a subset of PRACH resources determined by additional RACH configuration can be deactivated. Considering the benefit comes from enlarging the time domain gap between adjacent ROs so that gNB has more opportunity to go to sleep mode, it may be not promising to consider a mechanism of turn on/off a subset of PRACH resources from a RACH configuration. On the other hand, option 4-rev1 is much more complex than option 1. Furthermore, if several RACH configurations are provided for dynamic indication purpose, option 2 is also able to achieve very good NES gain.

Proposal 12: For the adaptation mechanism for additional PRACH resources, further study option 2 and option 4-rev.

Considering DCI format 1_0 scrambled by SI-RNTI, P-RNTI and RA-RNTI has lots of spare bits, they can be used for PRACH resource adaptation. The detail DCI content design can be determined after the down-selection between option 2 and option 4-rev.

2.3.3 The impacts on relevant UE behavior

PRACH is essential channel in NR system, which means it is the basis for lots of procedures. In Table 3, we summarize the relevant events triggering PRACH according to section 9.2.6 in TS 38.300.

Table 5 Use case of RACH procedure

| Event index | Descriptions |
| --- | --- |
| 1 | Initial access from RRC_IDLE |
| 2 | RRC Connection Re-establishment procedure |
| 3 | DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" |
| 4 | UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available |
| 5 | Handover |
| 6 | SR failure |
| 7 | Explicit request by RRC upon synchronous reconfiguration |
| 8 | RRC Connection Resume procedure from RRC_INACTIVE |
| 9 | To establish time alignment for a primary or a secondary TAG |
| 10 | Request for Other SI |
| 11 | Beam failure recovery |
| 12 | Consistent UL LBT failure on SpCell |
| 13 | SDT in RRC_INACTIVE |
| 14 | Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning |
| 15 | Early UL synchronization with an LTM candidate cell |
| 16 | RACH-based LTM cell switch |

As summarized in Table 3, RACH procedure is very important for a UE to sustain basic functionality in NR system. Regardless what adaptation of PRACH is adopted, it may bring significant influence on those procedures.

Observation 1: Adaptation of PRACH may bring significant influence on UE behavior associated with RACH procedure.

2.4 Adaptation of Paging occasions

Generally speaking, paging is a way for gNB to find a target IDLE/INACTIVE UE and to indicate all UEs the update of SI as soon as possible.

The configuration of Paging occasions is actually pretty flexible. Paging frames are distributed evenly within a window, i.e., DRX period for paging. Within a paging frame, there is one or more paging occasions constituted of several paging monitoring occasions. In order to guarantee coverage, each PMO is associated with an SSB. From network perspective, it can initiate a paging for data transmission purpose or SI update purpose on any paging occasion if necessary. From UE perspective, it only needs to monitor a specific PO within a specific paging frame which is special for it. No matter UE receives paging or not, there is no impact on UE behavior. To be specific, PF/PO/PMO are determined by the following formulae:

The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns The following parameters are used for the calculation of PF and i_s above: T: DRX cycle of the UE (T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied). N: number of total paging frames in T Ns: number of paging occasions for a PF PF_offset: offset used for PF determination UE_ID: 5G-S-TMSI mod 1024

In Figure 3, it shows the distribution of PF, PO and PMO with assuming T=32 radio frame, PF offset=0, Number of PO within a PF=4 and number of PMO within a PO=4.

Figure 3 Example for paging occasion determination from gNB perspective

It may be helpful to reducing power consumption at gNB side if the paging frame or paging occasion can be sparser. However, gNB still needs to wake up to transmit paging and the latency may increase. It can be seen that the current mechanism of determining paging occasion is to make sure POs are distributed evenly within DRX circle from network perspective. Therefore, paging transmission is rarely blocked due to simultaneously requirement for paging different UE. At the same time, it guarantees each UE has a chance to be paged within a certain period. Paging latency from UE perspective can be controllable as well.

However, the even distribution of PO makes gNB rarely have chance to convert into deep sleep mode, which is unfriendly to energy saving. In order to confront the issue caused by evenly distributed PO in time domain, the following two options can be considered as starting point:

- Option 1: PO are transmitted on several consecutive paging frame within a DRX circle, i.e., the PO are transmitted in a centralized manner instead of a distributed manner.

- Option 2: gNB adjust parameters related to PO determination so that PO can be transmitted in a sparser manner.

Proposal 13: For adaptation of paging occasion, the following options can be considered as starting point.

- Option 1: Paging can be transmitted with a centralized manner within DRX circle.

- Option 2: gNB adjust parameters related to PO determination so that paging transmission is sparser.

Similar as SSB and PRACH, the parameters related to PO determination are provided by SIB1. From this perspective, gNB can adjust paging occasions with reusing current mechanism, i.e., adjust paging occasions with updating SI. Considering the updated SI is applicable in the next SI transmission period, there is no ambiguity issue. However, the penalty is large latency and complexity. Another potential mechanism is that the adaptation of paging occasions is associated with certain conditions. For example, if SSB is not transmitted, paging transmission is adjusted accordingly. It may be challenging for UE to recognize such kind of adaptation in a timely manner. A straightforward mechanism is to introduce new mechanism to indicate adaptation of paging, e.g., dynamic indication. However, the standardization efforts may be huge. Considering paging is used to paging a UE once a call is coming or indicate UE that SI will be updated, it is network’s freedom on whether to send paging on a PO or how to send paging. It is also possible to leave it to gNB implementation. Accordingly, we have the following proposal:

Proposal 14: The following mechanisms for adapting paging in time domain can be further studied:

- Option 1: Reuse current mechanism to indicate UE the adaptation of paging.

- Option 2: Network adjust paging transmission by itself.

- Option 3: Design dynamic mechanism to indicate UE the adaptation of paging.

- Option 4: Adjust paging according to certain condition.

2.5 Potential impacts on legacy UE

The adaptation of common signal and channel transmissions may or may not be known by non-NES UEs, depending how to indicate/adjust common signal and channel transmissions. For example, if existing mechanism is fully reused, i.e., adjust common signal and channel transmissions via updating SI, there will be no impact on legacy UE. Otherwise, non-NES UE may be impacted as it cannot recognize the adaptation.

Observation 2: The impacts on legacy UE due to adaptation of common signal and channel transmissions depend on mechanism of realizing those adaptation.

If common signal and channel are adapted via a scheme transparent to non-NES UE, it is inevitable that non-NES UE is impacted as it cannot recognize the adaptation of common signal and channel transmissions. Generally speaking, the functionality of different common signal/channel is diverse.

- SSB is very basic DL channel/signal, which is used for cell search, RLM, RRM, beam-specific operation, etc. No matter SSB transmission is adapted or not, non-NES UE still tries to receive or measure SSB according to default SSB pattern, i.e., the one predefined by frequency range and SCS or the one indicated by SIB1. Power consumption at UE side goes up unnecessarily. More importantly, non-NES UE cannot distinguish different cases of SSB reception failure, i.e., channel condition is too bad to receive SSB and SSB is not transmitted on the default SSB occasion. Accordingly, unexpected measurement result may be reported to gNB. However, as NES gNB have full knowledge on the adaptation of SSB transmission, it may be capable to compensate or amend the received reporting from non-NES UE.

- PRACH is very basic UL signal, which is used for random access, UL synchronization, BFR, hand over, etc. Once the PRACH transmission is adapted, non-NES UE cannot receive any response after it transmit preamble on an RO invalid for gNB. Delay related to aforementioned procedure increases. On the other hand, power wastage is inevitable. However, the functionality may be not impacted with the penalty of large latency.

- Paging is used for paging UE once call is coming and indicating SI will be updated. If paging transmission is adapted, non-NES UE may not be able to receive paging as the realistic PO distribution is totally different from the one it assumes. Considering VoIP technology can at least partially bear voice traffic and RRC reconfiguration/re-establishment can at least update network configuration, it may be still affordable for non-NES UE.

The detail impacts on non-NES UE can be further studied once the big picture for adaptation of common signal/channel transmission is clearer. It should be considered together with other NES techniques, i.e., on-demand SSB and on-demand SIB1. On the other hand, network can guarantee that non-NES UE can only access to a gNB not supporting adaptation of common signal/channels.

Proposal 15: The impacts on legacy UE due to adaptation of common signal and channel transmissions can be further studied after RAN1 achieves big picture on NES technique.

2.6 Inter-action among different NES technology

As captured in the WID of Rel-19 NES, the following technology will be specified:

- On-demand SSB SCell operation for UEs in connected mode

- On-demand SIB1 for UEs in idle/inactive mode, if justified

- Adaptation of SSB in time domain, e.g., adapting periodicity

- Adaptation of PRACH in time domain

- Adaptation of PRACH in spatial domain, if justified

- Adaptation of paging occasions in the time domain

The basic idea of the above NES technologies is to reduce unnecessary power consumption and increase opportunity of converting into sleep mode. Although each technology is able to bring NES gain, it is reasonable to clarify the inter-action among different mechanisms. For example, if gNB determines to turn off SSB, it means this gNB determines to go to NES mode so that power consumption can be reduced. Considering SSB is turned off, it may be not necessary to transmit other common DL channel/signals and receive UL signal as normal. Otherwise, gNB can rarely reduce power consumption. Therefore, it is important to study joint operation among different NES technologies and achieve common understanding at such point.

Proposal 16: Further study the inter-action among different NES technologies.

3 Conclusion

Based on the discussion and analyses in section 2, we have the following observations and proposals:

Observations:

Observation 1: Adaptation of PRACH may bring significant influence on UE behavior associated with RACH procedure.

Observation 2: The impacts on legacy UE due to adaptation of common signal and channel transmissions depend on mechanism of realizing those adaptation.

Proposals:

Proposal 1: For the adaptation mechanisms of SSB in time-domain, it can be applied to the following scenarios:

- UE in idle/inactive and connected mode

- PCell and SCell(s)

Proposal 2: For the adaptation of SSB in time-domain,

- For PCell, adaptation can be applied to CD-SSB.

- For SCell, adaptation can be applied to CD SSB, NCD-SSB and SSB not on sync raster.

Proposal 3: UE needs to have the knowledge on adaptation of common signal/channel transmissions, including SSB, PRACH and paging.

Proposal 4: For adaptation of SSB in time domain, the impacts on UE behavior need to be carefully studied, following aspects can be considered as starting point:

- The impact on DL RE mapping

- The impact on UL transmission

- The impact on initial access

- UE’s assumption on SSB pattern

Proposal 5: Simplified SSB should be discussed under the umbrella of on-demand SSB, if needed.

Proposal 6: Compact SSB is not pursued in Rel-19 as it only brings very limited NES gain but jumbo specification impacts.

Proposal 7: The SSB transmission periodicity can be adapted to larger or smaller than a default SSB transmission periodicity.

- Default SSB transmission periodicity is the one predefined by SSB pattern or indicated by SIB1.

Proposal 8: SSB reduction/adaptation within a SSB burst, if needed, should work together with periodicity adaptation for NES purpose.

Proposal 9: Adaptation of SSB burst periodicity is indicated by DCI format 1_0 transmitted in CSS.

- The DCI format 1_0 is scrambled by SI-RNTI, P-RNTI or RA-RNTI.

- The spare bits in DCI format 1_0 is used to indicate SSB periodicity.

- Existing SSB periodicity values are reused as starting point, i.e., {20ms, 40ms, 80ms, 160ms}.

Proposal 10: The adaptation of PRACH can be applied to CBRA and CFRA, including 4-step RACH and 2-step RACH.

Proposal 11: For adaptation of PRACH in time domain, the additional PRACH resources are configured based on a same PRACH or a different configuration index from the PRACH configuration index for the legacy PRACH resources.

Proposal 12: For the adaptation mechanism for additional PRACH resources, further study option 2 and option 4-rev.

Proposal 13: For adaptation of paging occasion, the following options can be considered as starting point.

- Option 1: Paging can be transmitted with a centralized manner within DRX circle.

- Option 2: gNB adjust parameters related to PO determination so that paging transmission is sparser.

Proposal 14: The following mechanisms for adapting paging in time domain can be further studied:

- Option 1: Reuse current mechanism to indicate UE the adaptation of paging.

- Option 2: Network adjust paging transmission by itself.

- Option 3: Design dynamic mechanism to indicate UE the adaptation of paging.

- Option 4: Adjust paging according to certain condition.

Proposal 15: The impacts on legacy UE due to adaptation of common signal and channel transmissions can be further studied after RAN1 achieves big picture on NES technique.

Proposal 16: Further study the inter-action among different NES technologies.

4 Reference

  - RAN1#117 meeting chairman notes.

  - R1-2406294, Discussion on on-demand SSB SCell operation, xiaomi

  - TR38.864, Study on network energy savings for NR (Release 18)