3GPP TSG RAN WG1 #118	R1-2406321

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	5

Source:	CATT

Title:	         Discussion on LS reply on UL synchronization for contention based Msg3 transmission

Document for:	Discussion and Decision

Introduction

RAN1 has received one LS from RAN2 related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN [1]. Within this LS, RAN2 asked the RAN1 with the following two questions:

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

In this contribution, we analyzed UL synchroniztion issue for msg3 transmission in IoT NTN.

Discussion

In the initial access, UE should acquire the ephemeris information and UE location, and calculate the pre-compensated TA and send the PRACH to initiate UL access. Hence, the initial TA offset is up to PRACH signal tolerance. However, if without Msg1 and Msg2 transmission, the tolerable TA offset of Msg3 at the eNB will be limited. In this context, the msg3 transmission is risky, even not workable. In normal PUSCH transmission, TA offset should be shorter than CP, only taking into account limited TA error. But in the Msg3 transmission without Msg1 and Msg2, tolerable TA offset should be comparable with the length of PRACH CP.

Proposal 1:  Due to no indicated timing adjustment at Msg2 RAR, the required timing accuracy of Msg3 transmission without Msg1/Msg2 should be defined separately with that of normal PUSCH transmission.

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

Reply to Q1:

Normally, the requirement of timing accuracy for Msg3 transmission should be defined separately. Existing requirement of PUSCH transmission may not satisfy the timing accuracy for Msg3 transmission without Msg1/Msg2.

Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Reply to Q2:

It is up to RAN4 evaluation. If it is really not workable for Msg3 transmission without Msg1 and Msg2, the WID can be modified.

Conclusion

In this contribution, we analzyed the potential issue of UL timing accuracy if Msg3 transmission with Msg1 and Msg2, and provide relevant reply to RAN2 questions.

Due to no indicated timing adjustment at Msg2 RAR, the required timing accuracy of Msg3 transmission without Msg1/Msg2 should be defined separately with that of normal PUSCH transmission.

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

Reply to Q1:

Normally, the requirement of timing accuracy for Msg3 transmission should be defined separately. Existing requirement of PUSCH transmission may not satisfy the timing accuracy for Msg3 transmission without Msg1/Msg2.

Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Reply to Q2:

It is up to RAN4 evaluation. If it is really not workable for Msg3 transmission without Msg1 and Msg2, the WID can be modified.

References

[1] R2-2405769, RAN2, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2