3GPP TSG RAN WG1 #118                                             R1-2406376

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on on-demand SSB SCell operation

Agenda item:	9.5.1

Document for:	Discussion and Decision

Introduction

In RAN1#117 meeting, several agreements were achieved related to on-demand SSB SCell operation [1].

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement - For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE. - Sub-carrier spacing of the on-demand SSB - Physical Cell ID of the on-demand SSB - Location of on-demand SSB burst - Downlink transmit power of on-demand SSB - FFS: Other parameters - FFS: Whether each of above parameters is configured/indicated explicitly or not

This contribution discusses on-demand SSB SCell operation. Section 2 discusses the scenarios and cases of on-demand SSB SCell operation. Section 3 discusses signalling of on-demand SSB operation. Section 4 discusses TX behaviour of on-demand SSB burst. Section 5 discusses L1/L3 measurement based on on-demand SSB. Section 6 discusses further details of on-demand SSB operation. Section 7 discusses UE-triggered on-demand SSB SCell operation. Finally, section 8 summarizes the observation and proposals with conclusions.

Scenarios and cases of on-demand SSB SCell operation

In RAN1#116-bis meeting, one agreement was achieved related to the scenarios and cases of on-demand SSB SCell operation [2].

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Before going to the discussion of use cases and scenarios, we need to first investigate the role of SSB during legacy SCell activation. Figure 1 shows the procedure of current SCell activation for both known SCell case and unknown SCell case. According to [3], SCell activation completes after UE reports a valid CSI. If L3-RSRP has been reported before SCell activation command within a certain period, gNB would acquire UE’s measurement result in advance. When UE receives SCell activation command, it is feasible for gNB to quickly determine the TCI state used for further data transmission. Such SCell is defined as known SCell, which is shown in Figure 1 (a). In this case, after receiving SCell activation command, UE would use SSB for fine time tracking. On the other hand, if there is no L3-RSRP report before SCell activation, the SCell is regarded as unknown SCell, as shown in Figure 1 (b). After SCell activation command, UE has to perform AGC, synchronization and L1 measurement/report based on SSB. Then gNB actives TCI state for data transmission according to UE’s report. Therefore, larger activation delay is required for the unknown SCell.

(a) Known SCell

(b) Unknown SCell

Figure 1: Procedure of current SCell activation

Observation 1: In the current system, after UE receives SCell activation command, for a known SCell, UE acquires SSB for fine time tracking. For an unknown SCell, UE acquires SSB to perform AGC, synchronization and L1 measurement report.

According to the above agreement, on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

And the following combinations of scenarios/cases should be further studied:

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

According to the agreement above, the definitions of scenario #3A and scenario #3B are listed as follows:

- Scenario #3A refers to

  - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

- Scenario #3B refers to

  - “When SCell activation is completed and SCell is activated” or

  - “After SCell activation is completed and SCell is activated”

According to the agreement in RAN1#116, the definitions of case #1 and case #2 are listed as follows:

- Case #1: There is no always-on SSB on the cell.

- Case #2: Always-on SSB is periodically transmitted on the cell.

The combinations of scenario #3A/3B and case #1/2 are shown in the following Figure 2:

- Scenario #3A and Case #1:

(a) Scenario #3A and Case #1

- Scenario #3A and Case #2:

(b) Scenario #3A and Case #2

- Scenario #3B and Case #1:

(c) Scenario #3B and Case #1

- Scenario #3B and Case #2:

(d) Scenario #3B and Case #2

Figure 2: Combinations of scenario #3A/3B and case #1/2

For case#1, as mentioned above, on-demand SSB is naturally required after SCell activation command for AGC, synchronization and L1 measurement. Further, when SCell activation is completed, on-demand SSB is still needed to execute beam recovery in case of beam failure and maintain synchronization for data transmission unless the SCell is deactivated. Hence, the combinations of Scenario #3A/3B and Case #1 should be supported, which are shown in Figure 2 (a) and (c).

For case#2, UE is feasible to report L3-RSRP based on the always-on SSB before SCell activation command. If so, the SCell is a known SCell. But the always-on SSB may be too sparse to perform SCell activation and corresponding procedures. For less activation delay, on-demand SSB is needed after SCell activation command for quick fine time tracking. When SCell activation is completed, always-on SSB is sufficient to maintain synchronization for data transmission. Hence, the combination of Scenario #3A and Case #2 should be supported, which is shown in Figure 2 (b).

Proposal 1: For the identified scenarios and cases (as per RAN1#116 and RAN1#116-bis agreements), on-demand SSB can be triggered by gNB for the following scenarios/cases:

- Scenario #3A and Case #1

- Scenario #3A and Case #2

- Scenario #3B and Case #1

In RAN1#116-bis meeting, one agreement was achieved related to the cells supporting on-demand SSB SCell operation [2].

Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB

According to the above agreement, for a cell supporting on-demand SSB SCell operation, It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. Regarding the issue of whether on-demand SSB is cell-defining SSB or not, when an SSB is not associated with an RMSI, the SSB is referred to as a non-Cell Defining SSB (NCD-SSB), which can be used to perform RLM, BFD, and RRM measurements and measurements for RA resource selection inside the active DL BWP when the active BWP does not contain the CD-SSB. The SCell of a UE would be the PCell of another UE. Since on-demand SSB is temporarily transmitted, it should not be used for initial cell search by other UEs.

If the value in IE ‘cellBarrd’ in MIB is set to ‘barred’, the UE does not perform the initial cell search in this SCell. Hence, on-demand SSB can be cell-defining SSB or non-cell-defining SSB if the value in IE ‘cellBarrd’ in MIB is set to ‘barred’for legacy UE in RRC IDLE/INACTIVE states. If the value in IE ‘cellBarrd’ in MIB is not set to ‘barred’ for legacy UE in RRC IDLE/INACTIVE states, on-demand SSB should be limited to non-cell-defining SSB.

Regarding the above solution of ‘cellBarrd’ in MIB, an issue was raised in RAN1#117. It is argued that even if IE ‘cellBarrd’ in MIB is set to ‘barred’, some types of legacy UEs (e.g., UEs supporting NTN, RedCap, NES) are required to obtain barring information in SIB1. Thus, it would be safer to limit non-cell-defining SSB for on-demand SSB, in order to avoid any potential impacts to idle/inactive UEs. However, this issue can be easily solved. Taking RedCap UE as an example, for on-demand CD-SSB, its associated SIB1 contains barring information. In this case, the fields cellBarredRedCap and cellBarredeRedCap in SIB1 can be set to ‘barred’. In this way, after receiving SIB1, since the RedCap UE is barred by the SCell, it will not attempt to access the SCell. Since the RedCap UE will not attempt to access the SCell, it will not continue to search for the SSB of the SCell and instead search for the SSB on the next sync raster. If the UE cannot successfully decode SSB/MIB, the maximum duration the UE spends searching for SSB before considering a frequency change is specified by the reselection timer, so the UE will also consider the next location in the sync raster.

Proposal 2: Regarding whether cell-defining SSB or non-cell-defining SSB applies for on-demand SSB

- If the value in IE ‘cellBarrd’ in MIB is set to ‘barred’ for legacy UE in RRC IDLE/INACTIVE states, on-demand SSB can be cell-defining SSB or non-cell-defining SSB; Otherwise, on-demand SSB should be non-cell-defining SSB

Signalling of on-demand SSB operation

In RAN1#117 meeting, RRC based signalling and MAC-CE based signalling have been both supported to indicate on-demand SSB transmission [1].

In the current spec, besides using legacy SCell activation/deactivation MAC-CE, SCell could also be activated by RRC signaling. The RRC parameter sCellState in SCellConfig indicates whether the SCell shall be considered to be in activated state upon SCell configuration. In Rel-19, a new RRC parameter, e.g. ODSSBState, could be employed for each SCell to indicate whether on-demand SSB is transmitted. It could also be configured in SCellConfig as shown in the following. With the combination of these two parameters, RRC based signaling would be applied to either Scenario#2 or Scenario#2A.

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]]

[[

ODSSBState                       ENUMERATED {activated}                                          OPTIONAL,

]]

}

Proposal 3: For RRC based signalling, a new RRC parameter is employed for each SCell to indicate whether on-demand SSB is transmitted.

Proposal 4: RRC based signalling could be applied to both Scenario#2 and Scenario#2A.

For MAC-CE based signalling, some enhancements on legacy SCell activation/deactivation MAC-CE may be considered. For instance, to support both Scenario#2 and Scenario#2A, information on whether to activate/deactivate SCell simultaneously is needed in the signalling. If not simultaneously, this MAC-CE is only used to indicate on-demand SSB transmission, For SCell activation, another MAC-CE is needed. In addition, multiple sets of on-demand SSB configuration are preferred to be available for a cell for different deployment scenarios (e.g. L1/L3 measurements, SCell activation). If so, on-demand SSB configuration ID shall also be included in the signaling. In Figure 3, one example of MAC-CE based signalling is illustrated. Ci indicates the activation/deactivation status of the SCell with SCellIndex i. The reserved bit of legacy MAC-CE, denoted by field 2/2A, is used to indicate this MAC-CE is applied to Scenario#2 or Scenario#2A. SSB Config IDi indicates on-demand SSB configuration ID for a SCell.

Figure 3: Example of MAC-CE based signalling

Proposal 5: MAC-CE based signalling should be applied to both Scenario#2 and Scenario#2A. Information on whether to activate/deactivate SCell simultaneously is needed in the signalling.

If on-demand SSB is activated for a SCell, it should be informed as soon as possible to all the UEs sharing this SCell. UEs may use this SSB for RRM measurement, SCell activation and so on. Compared with the UE-specific RRC or MAC-CE based signaling, group-common DCI based signalling seems more suitable and more efficient to indicate on-demand SSB transmission. Moreover, as discussed in AI 9.5.3, to support SSB adaptation for SCells, a relative dynamic signaling is also preferred to adjust the periodicity values of the SSB. Intuitively, a unified group-common DCI could be designed to indicate either on-demand SSB transmission or SSB adaptation.

Proposal 6: A unified group-common DCI could be designed to indicate either on-demand SSB transmission or SSB adaptation.

TX behaviour of on-demand SSB burst

Five options for SSB burst transmission have been proposed in the last several meetings. From the perspective of energy saving, when SCell is deactivated, on-demand SSB should be terminated correspondingly. Correspondingly, periodical-like SSB transmission without termination (e.g. Option 1 and Option 4) should be avoided.

Proposal 7: For SSB burst(s) indicated by on-demand SSB SCell operation, the following options are preferred for further study:

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

- Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

In the last meeting, time instance A was agreed to be dependent on the time instance of the MAC-CE based on-demand SSB triggering signalling. The advantage is to trigger on-demand SSB more quickly, where gNB doesn’t have to wait HARQ-ACK for the MAC-CE. Figure 4 shows the relationship among MAC-CE, timing offset T and time instance A. For details, since the SSB symbols within a slot are predefined in the spec, time instance A, which indicates the first actually transmitted on-demand SSB burst, could be defined in a slot level. Correspondingly, T could also be defined in a slot level. Therefore, we have the following proposal:

Figure 4: Time instance A for on-demand SSB transmission

Proposal 8: For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is the slot boundary of the first SSB time domain position of actually transmitted on-demand SSB burst which is T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

Compared with intra-frequency SCell, additional processing time would be needed for UE to measure on-demand SSB transmitted on an inter-frequency SCell. Therefore, the timing offset T should be different for intra-frequency SCell and inter-frequency SCell.

Proposal 9: The value of T should be different for intra-frequency SCell and inter-frequency SCell.

The termination of on-demand SSB should also be known by the UE to avoid blind SSB detection. Instead of explicit indication of Option 1A and Option 3, Option 2 provides a way of implicit indication, which obviously saves the signalling overhead. For Option 2, the time instance B could be associated with predefined SSB termination event(s). This means, on-demand SSB would be terminated when the associated event happens. Referring to the usage of SSB during the current SCell activation procedure analyzed above, several candidate termination events may be considered. For example, when valid CSI is reported for SCell activation, which means the SCell activation is completed, on–demand SSB could then be terminated (shown in Figure 5(a)). Or when legacy SCell deactivation MAC-CE is received, on-demand SSB is simultaneously turned off (shown in Figure 5(b)).

(a)

(b)

Figure 5: Time instance B for on-demand SSB transmission

Proposal 10: For SSB burst transmission, the time instance B could be associated with predefined SSB termination events.

L1/L3 measurement based on on-demand SSB

In RAN1#117 meeting, one agreement was achieved related to L1 measurement based on on-demand SSB [1].

Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

In RAN1#116bis meeting meeting, L1 and/or L3 measurement based on on-demand SSB was supported for the cell [2], which is helpful to ensure the quality of the SCell to be activated. In our opinion, L3 measurement is preferred to be discussed by RAN2/4, since L3 report is sent to CU instead of DU, while RAN1 should focus on L1 measurement only.

Proposal 11: For L1 and/or L3 measurement based on on-demand SSB, RAN1 should focus on L1 measurement while let RAN2/RAN4 take the lead on the requirement for L3 measurement.

SS-RSRP, SS-RSRQ and SS-SINR measurement have been defined for SSB based measurement. SS-RSRP, SS-RSRQ measurements are used for cell selection, cell reselection, power control calculations, mobility procedures and beam management. SS-SINR measurements are be used for connected mode mobility procedures and beam management. These measurements can be generated and reported at both Layer 1 and Layer 3. For example, a UE can provide SS-RSRP measurements at Layer I when sending Channel State Information (CSI) to the gNB. Alternatively, the UE can provide SS-RSRP measurements at Layer 3 when sending an RRC. A UE is configured to report Layer l SS-RSRP if reportQuantity within CSI-ReportConfig is set to ssb-Index-RSRP. For SS-RSRQ, SS-SINR and layer 3 SS-RSRP measurements, SS/PBCH Block Measurement Timing Configuration (SMTC) configuration is used. Measurement and report based on on-demand SSB can reuse these legacy procedures.

For L1 measurement, according to the above agreement, the existing legacy CSI framework can be used to configure additional measurements. According to current CSI framework, each Reporting Setting (i.e., CSI-ReportConfig) is associated with a CSI Resource Setting (i.e., CSI-ResourceConfig) for channel measurement. If the CSI-RS Resources within the CSI Resource Setting are semi-persistent, the time domain behavior of the Reporting Setting can be aperiodic (using PUSCH) or semi-persistent (using PUCCH, and DCI activated PUSCH). Since the TX behaviour of on-demand SSB burst is similar to that of semi-persistent CSI-RS, in addition to the agreed semi-persistent report, aperiodic report based on existing CSI framework should also be supported for on-demand SSB.

In legacy system, semi-persistent CSI-RS is activated by SP CSI-RS activation MAC-CE. When semi-persistent CSI on PUCCH is required, SP CSI reporting on PUCCH activation MAC-CE is used to activate the CSI report. When semi-persistent CSI on PUSCH is required, DCI scrambled by SP-CSI-RNTI would be used to trigger the CSI report. In addition, if aperiodic CSI is expected, the report is triggered by DCI with CSI request field. Such mechanism could be applied to L1 measurement based on on-demand SSB. As shown in Figure 6, on-demand SSB transmission on SCell is activated by on-demand SSB triggering signaling mentioned in section 3, and its associated L1 reporting setting configured in PCell is triggered by another L1 report triggering signalling. The L1 report triggering signaling could be MAC-CE or DCI depending on whether the reported is carried by PUCCH or PUSCH.

Figure 6: L1 measurement/report based on on-demand SSB

Proposal 12: Aperiodic report based on existing CSI framework should be supported for on-demand SSB.

Enhancements on resource setting configuration

In this section, the enhancements on CSI resource configuration for on-demand SSB measurement are discussed. The issue in the existing CSI framework and related solution are provided.

Issue-1: How to add on-demand SSB resource configuration to existing CSI resource configuration?

Based on the existing CSI framework, the IE CSI-ResourceConfig in TS 38.331 gives the CSI resource configuration for CSI-RS resources and SSB resources [6]. The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet. The csi-SSB-ResourceSetList is the list of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set.

However, the SSB resource in CSI-SSB-ResourceSet is legacy periodic SSB and the IE CSI-ResourceConfig does not support resource configuration of on-demand SSB. The issue of how to add on-demand SSB resource configuration to existing CSI resource configuration should be resolved.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

Solutions to issue-1:

In order to solve the issue-1 above, the existing CSI resource configuration should be enhanced. There are two solutions to add on-demand SSB resource configuration to existing CSI resource configuration.

The Alt-1 solution is that the existing IE CSI-ResourceConfig should include the on-demand SSB resource configuration information. The Alt-1 solution has less specification impact but it will change the contents of the existing IE CSI-ResourceConfig.

The Alt-2 solution is that a new dedicated resource configuration IE for on-demand SSB resource configuration should be introduced, e.g. CSI-ResourceConfig-NES. The Alt-2 solution will not change the contents of the existing IE CSI-ResourceConfig but it has more specification impact since a new IE will be introduced.

Proposal 13: Consider two candidate solutions to add on-demand SSB resource configuration to existing CSI resource configuration.

- Alt-1: The existing IE CSI-ResourceConfig should include the on-demand SSB resource configuration information.

- Alt-2: A new dedicated resource configuration IE for on-demand SSB resource configuration should be introduced, e.g. CSI-ResourceConfig-NES.

Enhancements on CSI reporting configuration

In this section, the enhancements on CSI reporting configuration for on-demand SSB measurement are discussed. The issue in the existing CSI framework and related solution are provided.

Issue-2: How to add on-demand SSB reporting configuration to existing CSI reporting configuration?

Based on the existing CSI framework, the IE CSI-ReportConfig in TS 38.331 gives the CSI reporting configuration for the periodic, semi-persistent and aperiodic L1 measurement reports [6]. The IE CSI-ReportConfig is used to configure a periodic or semi-persistent report sent on PUCCH on the cell in which the CSI-ReportConfig is included, or to configure a semi-persistent or aperiodic report sent on PUSCH triggered by DCI received on the cell in which the CSI-ReportConfig is included.

However, the reporting configuration in CSI-ReportConfig is designed for legacy periodic SSB and the IE CSI-ReportConfig does not support reporting configuration of on-demand SSB. The issue of how to add on-demand SSB reporting configuration to existing CSI reporting configuration should be resolved.

CSI-ReportConfig information element

-- ASN1START

-- TAG-CSI-REPORTCONFIG-START

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

[………]

Solutions to issue-2:

In order to solve the issue-2 above, the existing CSI reporting configuration should be enhanced. There are two solutions to add on-demand SSB reporting configuration to existing CSI reporting configuration.

The Alt-1 solution is that the existing IE CSI-ReportConfig should include the on-demand SSB reporting configuration information. The Alt-1 solution has less specification impact but it will change the contents of the existing IE CSI-ReportConfig.

The Alt-2 solution is that a new dedicated reporting configuration IE for on-demand SSB reporting configuration should be introduced, e.g. CSI-ReportConfig-NES. The Alt-2 solution will not change the contents of the existing IE CSI-ReportConfig but it has more specification impact since a new IE will be introduced.

Proposal 14: Consider two candidate solutions to add on-demand SSB reporting configuration to existing CSI reporting configuration.

- Alt-1: The existing IE CSI-ReportConfig should include the on-demand SSB reporting configuration information.

- Alt-2: A new dedicated reporting configuration IE for on-demand SSB reporting configuration should be introduced, e.g. CSI-ReportConfig-NES.

Enhancements on MAC CE activation for PUCCH-based semi-persistent reporting

In this section, the enhancements on MAC CE activation for PUCCH-based semi-persistent reporting for on-demand SSB measurement are discussed. The issue in the existing CSI framework and related solution are provided.

Issue-3: How to support the activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB?

Based on the existing CSI framework as shown in the following Figure 7, the MAC CE SP CSI reporting on PUCCH Activation/Deactivation in TS 38.321 is used to activate and deactivate the semi-persistent L1 measurement reporting on PUCCH [7]. This field L indciates whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for LTM or not.: The field Si indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList if L field is set to 1, or csi-ReportConfigToAddModList if L field is set to 0, S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId and so on.

However, the existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE is only used to activate and deactivate CSI reporting for MIMO or CSI reporting for LTM. This MAC CE does not support activation and deactivation of CSI reporting for on-demand SSB. The issue of how to support the activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB should be resolved.

Figure 7: Existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Solutions to issue-3:

In order to solve the issue-3 above, the existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE should be enhanced. There are two solutions to support the activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB.

The Alt-1 solution is that the existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE should include the activation and deactivation of SP CSI reporting on PUCCH for on-demand SSB, e.g., one of the reserved bits can be used to indciate whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for on-demand SSB or not. If on-demand SSB is not configured, R field is present instead (i.e. set to 0). The Alt-1 solution has less specification impact but it will change the contents of the existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

The Alt-2 solution is that a new dedicated MAC CE should be introduced for activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB. The Alt-2 solution will not change the contents of the existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE but it has more specification impact since a new MAC CE will be introduced.

If the report configuration refered by Si is related to on-demand SSB measurement reporting, the enhanced MAC CE or the new introduced MAC CE can also be used to activate and deactivate semi-persistent L1 measurement reporting on PUCCH for on-demand SSB.

Proposal 15: Consider two candidate solutions to activate and deactivate semi-persistent L1 measurement reporting on PUCCH for on-demand SSB.

- Alt-1: The existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE should include the activation and deactivation of SP CSI reporting on PUCCH for on-demand SSB, e.g., one of the reserved bits can be used to indicate whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for on-demand SSB or not.

- Alt-2: A new dedicated MAC CE should be introduced for activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB.

Enhancements on DCI triggering for PUSCH-based semi-persistent reporting

In this section, the enhancements on DCI triggering for PUSCH-based semi-persistent reporting for on-demand SSB measurement are discussed. The issues in the existing CSI framework and related solutions are provided.

Issue-4: How to support the triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB?

Based on the existing CSI framework, the DCI format 0_1 and 0_2 are used to trigger the semi-persistent L1 measurement reporting on PUSCH. Each codepoint of the DCI field CSI request is associated with one trigger state, and one trigger state is associated with one CSI report configuration, so the field CSI request is used to indciates the CSI Report Configuration for the semi-persistent L1 measurement reporting on PUSCH.

However, the existing DCI field CSI request is used only to trigger semi-persistent CSI reporting for MIMO or CSI reporting for LTM. This DCI does not support the triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB. The issue of how to support the triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB should be resolved.

Solutions to issue-4:

In order to solve the issue-4 above, the existing DCI format 0_1 and 0_2 should be enhanced. There are two solutions to support the triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB.

The Alt-1 solution is that the existing DCI field CSI request is reused to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, and the existing DCI field Transform precoding indicator is used to indicate the DCI is used to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, or for legacy MIMO/LTM. For a DCI format 0_1 or 0_2 with CRC scrambled by SP-CSI-RNTI, the bit Transform precoder indicator in DCI format 0_1 or 0_2 is reserved, so this reserved bit can be used to indicate whether the DCI triggers semi-persistent L1 measurement reporting on PUSCH for on-demand SSB or not. If the bit Transform precoder indicator in DCI format 0_1 or 0_2 with CRC scrambled by SP-CSI-RNTI is set to 1, the DCI field CSI request is used to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB. If the bit Transform precoder indicator is set to 0, the DCI field CSI request is used to trigger semi-persistent L1 measurement reporting on PUSCH for legacy MIMO/LTM. The Alt-1 solution has less specification impact but it will use the field Transform precoding indicator.

The Alt-2 solution is that a new dedicated RNTI (e.g., NES-RNTI) for DCI format 0_1 and 0_2 should be introduced for triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB. If the CRC is scrambled by NES-RNTI, the DCI field CSI request is used to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB. If the CRC is scrambled by SP-CSI-RNTI, the DCI field CSI request is used to trigger semi-persistent L1 measurement reporting on PUSCH for legacy MIMO/LTM. The Alt-2 solution will not use the field Transform precoding indicator but it has more specification impact since a new dedicated RNTI will be introduced.

Proposal 16: Consider two candidate solutions to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB.

- Alt-1: The existing DCI field CSI request is reused to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, and the existing DCI field Transform precoding indicator is used to indicate the DCI is used to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, or for legacy MIMO/LTM.

- Alt-2: A new dedicated RNTI (e.g., NES-RNTI) for DCI format 0_1 and 0_2 should be introduced for triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB.

Issue-5: How to support the semi-persistent L1 measurement reporting on PUSCH for multiple on-demand SSBs from multiple SCells?

Based on the existing CSI framework, the DCI format 0_1 and 0_2 are used to trigger the semi-persistent L1 measurement reporting on PUSCH. Each codepoint of the DCI field CSI request is associated with one trigger state, and one trigger state is associated with one CSI report configuration, so this field CSI request is used to indciates the CSI Report Configuration for the semi-persistent L1 measurement reporting on PUSCH.

However, the existing each trigger state is associated with only one CSI report configuration, which means each DCI can trigger the semi-persistent L1 measurement reporting on PUSCH for on-demand SSB from only one SCell. Hence the UE has to report the on-demand SSB measurement results of multiple SCells to gNB by multiple PUSCH transmission. When there are a large number of SCells that need to be reported (the maximumn number is 32 SCells), it will result in significant reporting latency and signalling overhead. Therefore, the issue of how to support the semi-persistent L1 measurement reporting on PUSCH for multiple on-demand SSBs from multiple SCells should be resolved.

CSI-SemiPersistentOnPUSCH-TriggerStateList information element

-- ASN1START

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-START

CSI-SemiPersistentOnPUSCH-TriggerStateList ::= SEQUENCE(SIZE (1..maxNrOfSemiPersistentPUSCH-Triggers)) OF CSI-SemiPersistentOnPUSCH-TriggerState

CSI-SemiPersistentOnPUSCH-TriggerState ::=     SEQUENCE {

associatedReportConfigInfo                     CSI-ReportConfigId,

...,

[[

sp-CSI-MultiplexingMode-r17                    ENUMERATED {enabled}                                           OPTIONAL   -- Need R

]],

[[

csi-ReportSubConfigTriggerList-r18             CSI-ReportSubConfigTriggerList-r18                             OPTIONAL,  -- Need R

ltm-AssociatedReportConfigInfo-r18             LTM-CSI-ReportConfigId-r18                                     OPTIONAL   -- Need R

]]

}

-- TAG-CSI-SEMIPERSISTENTONPUSCHTRIGGERSTATELIST-STOP

-- ASN1STOP

Solutions to issue-5:

In order to solve the issue-5 above, the existing triggering state should be enhanced. There are two solutions to support the semi-persistent L1 measurement reporting on PUSCH for multiple on-demand SSBs from multiple SCells.

The Alt-1 solution is that the existing IE CSI-SemiPersistentOnPUSCH-TriggerState should include multiple CSI-ReportConfigIds. Each CSI-ReportConfigId is associated with one on-demand SSB resource configuration information. Hence, the enhanced CSI-SemiPersistentOnPUSCH-TriggerState will be associated with multiple on-demand SSBs from multiple SCells. The Alt-1 solution has less specification impact but it will change the contents of the existing IE CSI-SemiPersistentOnPUSCH-TriggerState.

The Alt-2 solution is that a new dedicated trigger state IE for on-demand SSB should be introduced, e.g. CSI-SemiPersistentOnPUSCH-TriggerState-NES. The Alt-2 solution will not change the contents of the existing IE CSI-SemiPersistentOnPUSCH-TriggerState but it has more specification impact since a new IE will be introduced.

Proposal 17: Consider two candidate solutions to support the semi-persistent L1 measurement reporting on PUSCH for multiple on-demand SSBs from multiple SCells.

- Alt-1: The existing IE CSI-SemiPersistentOnPUSCH-TriggerState should include multiple CSI-ReportConfigIds. Each CSI-ReportConfigId is associated with one on-demand SSB resource configuration information.

- Alt-2: A new dedicated trigger state IE for on-demand SSB should be introduced, e.g. CSI-SemiPersistentOnPUSCH-TriggerState-NES.

Further details on on-demand SSB operation

Taking the above MAC-CE based signaling as an example, assume the Ci field is set to 1 to indicate that on-demand SSB shall be transmitted for the SCell with SCellIndex i. When two MAC-CE based signallings with Ci field set to 1 are received successively, UE has to differentiate whether a new on-demand SSB transmission is activated or this is still for the previous on-demand SSB transmission. The solution depends on whether to allow multiple activated on-demand SSBs being effective at the same time for a cell. Considering the purpose of on-demand SSB, from UE’s perspective, only a single on-demand SSB is allowed to be activated at a period of time.

Proposal 18: From UE’s perspective, multiple configuration of on-demand SSBs activated at the same time for a cell should not be allowed.

According to the TX behavior of on-demand SSB mentioned in section 4, for each MAC-CE based signaling, there would be a timing offset T before on-demand SSB transmission. In the case of multiple successive MAC-CE based signallings to indicate on-demand SSB transmission for a cell, there would be an ambiguity on SSB transmission, which is shown in Figure 8(a). To avoid such ambiguity, a rule should be specified for this case (as shown in Figure 8 (b)) to make sure only one time instance A is followed, for example, depending on the first MAC-CE signalling.

(a)

(b)

Figure 8: On-demand SSB transmission with two successive MAC-CE based signallings

Proposal 19: When multiple MAC-CE based signallings indicating on-demand SSB transmission are received successively, time instance A is decided only by the first MAC-CE signalling.

UE-triggered on-demand SSB SCell operation

In this section, we will discuss the on-demand SSB SCell operation triggered by UE and provide the views on the design of UL WUS from existing signal/channel.

In RAN1#117 meeting, the following proposal was discussed without reaching the consensus on supporting on-demand SSB SCell operation triggered by UE [8].

### Proposal #6-1 (UE-triggering): - Support on-demand SSB SCell operation triggered by UE. - It is up to gNB whether to transmit on-demand SSB after receiving UE’s request. - FFS on details of UL signaling to trigger on-demand SSB

The following cases were identified where UE has more knowledge than the gNB or UE-triggered on-demand SSB could benefit in [4]:

- UE’s mobility may result in SCell’s low quality (e.g., when its coverage is smaller than PCell’s coverage)

- UE’s UL data arrival which can make SR procedure efficient by utilizing UE-triggered on-demand SSB

- For fast beam recovery in case of beam failure or UE-triggering beam management (as discussed in Rel-19)

- For Rel-18 SSB-less SCell scenario where gNB may not know whether some conditions (e.g., RTD ≤ 3 us) are satisfied at UE side

Therefore, at least for the above cases, on-demand SSB SCell operation triggered by UE should be supported. It is up to gNB whether to transmit on-demand SSB after receiving UE’s request.

Proposal 20: Support on-demand SSB SCell operation triggered by UE.

- If gNB decides to transmit on-demand SSB upon UE’s request, UE will be notified by on-demand SSB transmission indication signaling.

Candidate UL WUS from existing signal/channel

According to the WID [5], in order to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA, the triggering method(s) should be specified. UE uplink wake-up signal (UL WUS) using an existing signal/channel is one of the optional triggering methods.

As mentioned in the WID, the UE UL WUS should use an existing signal/channel, so the new UL WUS should not be introduced. In our view, the candidate existing channels can be PRACH, PUCCH, and PUSCH.

Firstly, PRACH can be used as the UL WUS to trigger on-demand SSB transmission of a SCell. When a UE wants to send PRACH to a cell, it should first obtain downlink synchronization with the cell, but it does not need uplink synchronization with the cell, since the purpose of PRACH is to establish uplink synchronization. The transmission of PUCCH or PUSCH requires both downlink synchronization and uplink synchronization between the UE and the cell to which the UL WUS is sent. In this sense, the requirements in synchronization with the cell which the WUS is sent to are lower for PRACH than that for PUCCH or PUSCH. Hence, PRACH can be sent to either PCell or SCell to be activated as UL WUS to trigger the on-demand SSB transmission of the SCell to be activated.

Secondly, PUCCH or PUSCH can also be used as the UL WUS to trigger on-demand SSB transmission of an SCell. Since the UE is in the RRC_CONNECTED state, it can send the PUCCH or PUSCH to the PCell as UL WUS to trigger the on-demand SSB transmission of the SCell to be activated. The merit of using PUCCH or PUSCH as UL WUS is that it saves the scarce PRACH resources.

Regarding the three candidate existing uplink channels for UL WUS, the transmission time domain and frequency domain resources of these channels should be (pre-)configured by the network, before the UE sends the UL WUS.

Therefore, the candidate existing channels for UL WUS including:

- PRACH on PCell/SCell

- PUCCH on PCell

- PUSCH on PCell

Proposal 21: The following existing channels should be considered as the candidate UE UL WUS to support on-demand SSB SCell operation for UE in connected mode configured with CA.

- PRACH on PCell/SCell

- PUCCH on PCell

- PUSCH on PCell

Conditions for UE to send UL WUS

Due to the propagation characteristics of wireless signals, the communication link between a UE in connected mode configured with CA and its serving cells (including PCell and connected SCell) may be blocked by buildings, or the UE may move to the edge of its serving cells during the data communication procedure. Consequently, the channel quality between the UE and its serving cells may significantly degrade. In this case, the BLER of packets will increase significantly and the data transmission rate will decrease significantly. The reliable and effective data transmission services may not be provided to the UE and its data communication requirement cannot be meet. In order to improve the experience of the data communication, the UE can send UL WUS to the PCell or a potential SCell to be activated to trigger on-demand SSB transmission of the SCell to be activated.

As shown in Figure 9, the UE moves to the edge of its serving cells(PCell & SCell 1), and the RSRP/RSRQ/SINR of the communication link between the UE and its serving cells is below a (pre)-configured threshold. Additionally, there is one potential SCell 2 near the UE which can be activated. Then, the UE will send UL WUS to PCell or SCell 2 to trigger on- demand SSB transmission of the SCell 2.

In addition, another possible condition for a UE to send UL WUS to a Cell for triggering on-demand SSB transmission can be that there is uplink data that needs to be transmitted for the UE. When a UE has the requirement of uplink data transmission and cannot be satisfied by current serving cells, it can send UL WUS to trigger a SCell in energy-saving state to send the on-demand SSB and transmit the uplink data in the activated SCell.

Therefore, the conditions for UE to send UL WUS to a Cell for triggering on-demand SSB transmission at least include:

- The channel quality of the communication link between the UE and its serving cells (including PCell and activated SCell(s)) is below a (pre)-configured threshold.

  - The metrics of the channel quality can be RSRP, RSRQ and SINR.

- There is uplink data that needs to be transmitted for the UE.

Figure 9: Conditions for UE to send UL WUS

Proposal 22: The conditions for a UE to send UL WUS to a Cell for triggering on-demand SSB transmission at least include:

- The channel quality of the communication link between the UE and its serving cells (including PCell and activated SCell(s)) is below a (pre)-configured threshold.

  - The metrics of the channel quality can be RSRP, RSRQ and SINR.

- There is uplink data that needs to be transmitted for the UE.

Which cell the UE sends UL WUS to

When a UE in connected mode configured with CA tries to send UL WUS to trigger a SCell in an energy-saving state to transmit on-demand SSB, it can send the UL WUS to either PCell (Option-1) or SCell to be activated (Option-2), as shown in the following Figure 10.

Figure 10: Two options for the cells UE can send UL WUS to

Regarding Option-1, it is natural for the UE to send the UL WUS to PCell, since the UE is connected to PCell and has obtained the synchronization with the PCell.

The UE can send PRACH, PUCCH, or PUSCH as the UL WUS to the PCell without the synchronization issues. However, as mentioned in the above section, the condition for UE to send UL WUS may be that the communication link between the UE and its PCell is blocked by buildings or the UE moves to the edge of its PCell, the channel quality between the UE and its PCell may significantly degrade. In this case, the UL WUS sent to PCell may not be received correctly.

Regarding Option-2, the UE can measure and select a SCell in energy-saving state with satisfactory chancel quality and send the UL WUS to SCell to be activated.

As the UE can obtain downlink synchronization with the SCell to be activated by measuring the sparse SSB sent by the SCell, PRACH can be used as the UL WUS by the UE to wake up the SCell in an energy-saving state. However, since the uplink synchronization between the UE and the SCell to be activated has not yet been achieved, it seems that the UE cannot use PUCCH or PUSCH as UL WUS. By measuring the sparse SSB sent by the SCell, the UE can select the SCell with better channel quality between them, and then send WUS to that SCell for triggering the SCell to send on-demand SSB. In this way, the issue of UL WUS transmission failure due to poor channel quality between PCell and the UE can be avoided. Then the success rate of UL WUS transmission can be ensured by Option-2.

The comparison between Option-1 and Option-2 is shown in the following Table 1, focusing on the aspect of Synchronization conditions, available candidate uplink channels for UL WUS and the quality of communication link.

Table 1: Comparison between Option-1 and Option-2 on which cell does UE sends UL WUS to

| Comparison items | Option-1: PCell | Option-2: SCell to be activated |
| --- | --- | --- |
| Synchronization conditions | Both DL and UL synchronization between PCell and UE has been obtained. | Only DL synchronization between SCell and UE has been obtained |
| Available candidate uplink channels for UL WUS | PRACH, PUCCH, PUSCH | PRACH |
| The quality of communication link | Maybe poor | good |

Proposal 23: Both of the following options should be supported for the cell UE may send UL WUS to:

- Option-1: PCell (PCell needs to further trigger the on-demand SSB transmission of potential SCell to be activated).

- Option-2: Potential SCell to be activated (SCell may start to send the on-demand SSB after receiving the UL WUS).

Conclusions

In this contribution, we discuss on-demand SSB SCell operation, and give the following observation and proposals:

Observation 1: In the current system, after UE receives SCell activation command, for a known SCell, UE acquires SSB for fine time tracking. For an unknown SCell, UE acquires SSB to perform AGC, synchronization and L1 measurement report.

Proposal 1: For the identified scenarios and cases (as per RAN1#116 and RAN1#116-bis agreements), on-demand SSB can be triggered by gNB for the following scenarios/cases:

- Scenario #3A and Case #1

- Scenario #3A and Case #2

- Scenario #3B and Case #1

Proposal 2: Regarding whether cell-defining SSB or non-cell-defining SSB applies for on-demand SSB

- If the value in IE ‘cellBarrd’ in MIB is set to ‘barred’ for legacy UE in RRC IDLE/INACTIVE states, on-demand SSB can be cell-defining SSB or non-cell-defining SSB; Otherwise, on-demand SSB should be non-cell-defining SSB

Proposal 3: For RRC based signalling, a new RRC parameter is employed for each SCell to indicate whether on-demand SSB is transmitted.

Proposal 4: RRC based signalling could be applied to both Scenario#2 and Scenario#2A.

Proposal 5: MAC-CE based signalling should be applied to both Scenario#2 and Scenario#2A. Information on whether to activate/deactivate SCell simultaneously is needed in the signalling.

Proposal 6: A unified group-common DCI could be designed to indicate either on-demand SSB transmission or SSB adaptation.

Proposal 7: For SSB burst(s) indicated by on-demand SSB SCell operation, the following options are preferred for further study:

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

- Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

Proposal 8: For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is the slot boundary of the first SSB time domain position of actually transmitted on-demand SSB burst which is T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

Proposal 9: The value of T should be different for intra-frequency SCell and inter-frequency SCell.

Proposal 10: For SSB burst transmission, the time instance B could be associated with predefined SSB termination events.

Proposal 11: For L1 and/or L3 measurement based on on-demand SSB, RAN1 should focus on L1 measurement while let RAN2/RAN4 take the lead on the requirement for L3 measurement.

Proposal 12: Aperiodic report based on existing CSI framework should be supported for on-demand SSB.

Proposal 13: Consider two candidate solutions to add on-demand SSB resource configuration to existing CSI resource configuration.

- Alt-1: The existing IE CSI-ResourceConfig should include the on-demand SSB resource configuration information.

- Alt-2: A new dedicated resource configuration IE for on-demand SSB resource configuration should be introduced, e.g. CSI-ResourceConfig-NES.

Proposal 14: Consider two candidate solutions to add on-demand SSB reporting configuration to existing CSI reporting configuration.

- Alt-1: The existing IE CSI-ReportConfig should include the on-demand SSB reporting configuration information.

- Alt-2: A new dedicated reporting configuration IE for on-demand SSB reporting configuration should be introduced, e.g. CSI-ReportConfig-NES.

Proposal 15: Consider two candidate solutions to activate and deactivate semi-persistent L1 measurement reporting on PUCCH for on-demand SSB.

- Alt-1: The existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE should include the activation and deactivation of SP CSI reporting on PUCCH for on-demand SSB, e.g., one of the reserved bits can be used to indicate whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for on-demand SSB or not.

- Alt-2: A new dedicated MAC CE should be introduced for activation and deactivation of semi-persistent L1 measurement reporting on PUCCH for on-demand SSB.

Proposal 16: Consider two candidate solutions to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB.

- Alt-1: The existing DCI field CSI request is reused to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, and the existing DCI field Transform precoding indicator is used to indicate the DCI is used to trigger semi-persistent L1 measurement reporting on PUSCH for on-demand SSB, or for legacy MIMO/LTM.

- Alt-2: A new dedicated RNTI (e.g., NES-RNTI) for DCI format 0_1 and 0_2 should be introduced for triggering of semi-persistent L1 measurement reporting on PUSCH for on-demand SSB.

Proposal 17: Consider two candidate solutions to support the semi-persistent L1 measurement reporting on PUSCH for multiple on-demand SSBs from multiple SCells.

- Alt-1: The existing IE CSI-SemiPersistentOnPUSCH-TriggerState should include multiple CSI-ReportConfigIds. Each CSI-ReportConfigId is associated with one on-demand SSB resource configuration information.

- Alt-2: A new dedicated trigger state IE for on-demand SSB should be introduced, e.g. CSI-SemiPersistentOnPUSCH-TriggerState-NES.

Proposal 18: From UE’s perspective, multiple configuration of on-demand SSBs activated at the same time for a cell should not be allowed.

Proposal 19: When multiple MAC-CE based signallings indicating on-demand SSB transmission are received successively, time instance A is decided only by the first MAC-CE signalling.

Proposal 20: Support on-demand SSB SCell operation triggered by UE.

- If gNB decides to transmit on-demand SSB upon UE’s request, UE will be notified by on-demand SSB transmission indication signaling.

Proposal 21: The following existing channels should be considered as the candidate UE UL WUS to support on-demand SSB SCell operation for UE in connected mode configured with CA.

- PRACH on PCell/SCell

- PUCCH on PCell

- PUSCH on PCell

Proposal 22: The conditions for a UE to send UL WUS to a Cell for triggering on-demand SSB transmission at least include:

- The channel quality of the communication link between the UE and its serving cells (including PCell and activated SCell(s)) is below a (pre)-configured threshold.

  - The metrics of the channel quality can be RSRP, RSRQ and SINR.

- There is uplink data that needs to be transmitted for the UE.

Proposal 23: Both of the following options should be supported for the cell UE may send UL WUS to:

- Option-1: PCell (PCell needs to further trigger the on-demand SSB transmission of potential SCell to be activated).

- Option-2: Potential SCell to be activated (SCell may start to send the on-demand SSB after receiving the UL WUS).

References

- Chair notes RAN1#117, 3GPP TSG RAN WG1 Meeting #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- Chair notes RAN1#116-bis, 3GPP TSG RAN WG1 Meeting #116-bis, Changsha, Hunan Province, China, April 15th –19th, 2024.

- 3GPP TS 38.133, Requirements for support of radio resource management (Release 18), V18.4.0 (2023-12).

- R1-2403765, Summary #5 of on-demand SSB for NES, 3GPP TSG RAN WG1 Meeting #116-bis, Changsha, Hunan Province, China, April 15th –19th, 2024.

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN#102 Meeting, December 11th-15th, 2023.

- 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification (Release 18), V18.2.0 (2024-06).

- 3GPP TS 38.321, Medium Access Control (MAC) protocol specification (Release 18), V18.2.0 (2024-06).

- R1-2405723, Summary #5 of on-demand SSB for NES, 3GPP TSG RAN WG1 Meeting #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.