3GPP TSG RAN WG1 #118		                                                    R1-2406552

Maastricht, The Netherlands, 19th – 23rd August, 2024

Agenda Item:	5

Source:	NEC

Title:	Discussion on RAN2 LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Document for:	Discussion

Introduction

RAN1 received an LS from RAN2 on DL coverage enhancements [1].

1. Overall Description: In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved: Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.. Based on the above agreements, RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? 2. Actions: To RAN4 and RAN1: # ACTION: RAN2 respectfully requests RAN4 and RAN1 to take above RAN2 progress into account and provide feedback.

In this contribution, we provide our views on this.

Discussion

The timing relationship of IoT NTN is illustrated in Fig. 1.

Fig. 1 Illustration of timing relationship (for collocated eNB and NTN Gateway)

For the serving cell, the network broadcast ephemeris information and common Timing Advance (common TA) parameters. The UE shall have valid GNSS position as well as the ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before and during connection to a cell, the UE shall pre-compensate the Timing Advance, by considering the common TA, UE position and the NTN payload position through the ephemeris[2].

Transmission of the uplink radio frame number i from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs seconds before the start of the corresponding downlink radio frame at the UE [3], which is illustrated in Fig. 2.

Fig. 2 Uplink-downlink timing relation

For frame structure type 1 NTA offset=0 and for frame structure type 2 NTA offset=624 unless stated otherwise in [4].

The quantity NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0.

The quantity NTA,adjUE is computed by the UE based on UE position and serving satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

The determining procedure of NTA is specified in [4].

- In case of random access response, an 11-bit timing advance command [5], TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16. NTA is defined in [3].

- In other cases, a 6-bit timing advance command [5] or the Timing advance adjustment field in DCI format N0 if present [6], TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

Observation 1: For pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN), there is no NTA provided by the network.

Observation 2: For Msg3 transmission, an 11-bit timing advance command is required to meet the timing accuracy.

Observation 3: For other uplink transmissions after Msg3, a 6-bit timing advance command or the Timing advance adjustment field in DCI format N0 is required to meet the timing accuracy.

Observation 4: Without any valid NTA, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can not satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Observation 5: If an RRC Idle UE is configured a NTA which is still valid for the timing of the contention-based Msg3 transmission,  a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

After RRC Release,

- If an RRC Idle UE remains in the same cell before sending contention based Msg3 and the NTA for contention based Msg3 transmission is still valid within a specified time (similar to the pur-TimeAlignmentTimer in [5]), an RRC Idle UE with a pre-compensated TA and the valid NTA can meet the required timing accuracy for Msg3 transmission.

- If an RRC Idle UE is in a new cell under the same satellite when sending contention based Msg3 and the network could preconfigure a NTA with a valid timer for the target cell, an RRC Idle UE with a pre-compensated TA and the valid NTA can meet the required timing accuracy for Msg3 transmission.

- If an RRC Idle UE is in a new cell under a different satellite, which is connected to the original satellite by ISL, when sending contention based Msg3 and the network could preconfigure a NTA with a valid timer for the target cell, an RRC Idle UE with a pre-compensated TA and the valid NTA can satisfy the required timing accuracy for Msg3 transmission.

The determination of the NTA and the estimation of the valid timer length can be handled by the network implementation.

Observation 6: There are potential ways to provide a valid NTA for an RRC Idle UE in the same /a different cell to calculate the TTA to satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Proposal 1: Respond to RAN2 with the following information and a question:

1) An RRC Idle legacy UE with a pre-compensated TA (used for Msg1 transmission during random access for IoT NTN) cannot meet the required timing accuracy for Msg3 transmission without Msg1/Msg2;

2) To meet the timing accuracy for Msg3 transmission,  a valid NTA is necessary at the time of sending Msg3. Further investigation in RAN1 is needed for enhancements on NTA for contention based Msg3;

3) RAN1 would like to inquire whether RAN2 is considering an RRC Idle UE sending contention-based msg3 via a different cell in the Rel-19 scope.

Conclusion

From the discussion, we have the following observations and proposals.

Observation 1: For pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN), there is no NTA provided by the network.

Observation 2: For Msg3 transmission, an 11-bit timing advance command is required to meet the timing accuracy.

Observation 3: For other uplink transmissions after Msg3, a 6-bit timing advance command or the Timing advance adjustment field in DCI format N0 is required to meet the timing accuracy.

Observation 4: Without any valid NTA, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can not satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Observation 5: If an RRC Idle UE is configured a NTA which is still valid for the timing of the contention-based Msg3 transmission,  a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2

Observation 6: There are potential ways to provide a valid NTA for an RRC Idle UE in the same /a different cell to calculate the TTA to satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Proposal 1: Respond to RAN2 with the following information and a question:

1) An RRC Idle legacy UE with a pre-compensated TA (used for Msg1 transmission during random access for IoT NTN) cannot meet the required timing accuracy for Msg3 transmission without Msg1/Msg2;

2) To meet the timing accuracy for Msg3 transmission,  a valid NTA is necessary at the time of sending Msg3. Further investigation in RAN1 is needed for enhancements on NTA for contention based Msg3;

3) RAN1 would like to inquire whether RAN2 is considering an RRC Idle UE sending contention-based msg3 via a different cell in the Rel-19 scope.

- References

- R1-2405786(R2-2405769), LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, RAN1#118

- TS 36.300

- TS 36.211

- TS 36.213

- TS 36.321

- TS 36.212