3GPP TSG RAN WG1 #118			                                            R1-2406883

Maastricht, NL, August 19th – 23rd, 2024

Source:	Sharp

Title:	Discussion on LP-WUS operation in CONNECTED modes

Agenda Item:	9.6.3

Document for:	Discussion and decision

Introduction

In RAN#102, WI on Low-power wake-up signal and receiver for NR was approved for UE power saving[1]. The objectives of the WI include specifying procedures to allow UE MR PDCCH monitoring triggered by LP-WUS and activation and deactivation procedures of LP-WUS monitoring. In RAN1#117, some issues on LP-WUS monitoring are discussed and agreed for further study. This contribution discusses further considerations on LP-WUS monitoring for UE in CONNECTED mode.

Discussion

LP-WUS monitoring procedure

Unicast LP-WUS for special UE or group cast LP-WUS for a group of UE can be two candidates for connected UE. Considering the lower spectrum efficiency of LP-WUS waveform, group cast LP-WUS should be considered with higher priority, In the options discussed in 9.6.1 for LP-WUS information structure in connected mode, code point may be more match with unicast LP-WUS, and bitmap or multiple code point/bit blocks can be considered for group cast LP-WUS with further study to how to set bitmap/bit blocks for UEs. In addition, as all configuration of LP-WUS for UE via dedicated RRC message, The gNB can also configure LP-WUS for only one UE depending its configuration .

Proposal 1: Support group cast LP-WUS for UE in connected mode.

In RAN1#116b[2], the LP-WUS procedures on trigger PDCCH monitoring were discussed, and options for combining with C-DRX were selected for further study. In RAN2#126, RAN2 assumes that legacy DCP and Option 1-1 are not configured simultaneously for a UE.

Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

In these options, Option 1-1 can provide the waking-up indication in LP-WUS to replace the similar function in DCP. Since LP-WUS monitoring can tolerate more synchronization shifts and lower power consumption, UE can get more sleep time for WUS signal monitoring using LP-WUS, resulting in more power-saving gain than DCP. In addition, option 1-1 may require less specification effort to adapt the C-DRX procedure, it can be considered as a baseline for LP-WUS monitoring for UE in connected mode.

For option 1-2-1, it is assumed that it will always be used in combination with option 1-1, and therefore some of the details of the combination need to be clarified. For example, does the  “outside of activetime” include the duration for OnDurationTimer of C-DRX, and the OnDurationTimer is not triggered to run by the indication in LP-WUS with option 1-1.

for Option 1-2-2 , if it is not combined with Option 1-1 or DCP, UE still needs to follow the legacy DRX procedure, and the power-saving benefit may be minimal.

Option 1-3 is useful for some situations, such as traffic arrival jitter case. In addition, it can be used in conjunction with options 1-1 to cover more cases. For example, if only UE monitors an active indication in LP-WUS for option 1-1, then UE monitors LP-WUS for option 1-3 in a related DRX cycle, otherwise, UE can keep sleep to save power.

Proposal 2:  LP-WUS procedures Option 1-1 ,which replaces DCP functionality, can be used as a baseline for UE in connected mode.

Proposal 3: Further consider the combination of Option 1-1 and Option 1-2-1 and the combination of Option 1-1 and Option 1-3 for UE in connected mode.

CA

When CA is supported by UE, it needs further discussion on whether and how to support LP-WUS for the case[3].

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded

In general, we cannot see the reason to limit the LP-WUS  application for UE with CA configuration.  to save system overhead for LP_WUS resource, the LP-WUS for connected UE can also be provided only in Pcell to carry indication for all SCell.

To enable fast SCell activation when CA is configured, dormancy indication in DCI is used to trigger dormant BWP switching. Doing the similar function of DCP, it can be discussed whether dormancy indication should also be included in LP-WUS. we can notice that the dormancy indication for UE is included in the DCI format 2_6 for DCP. Since RAN2 assumes the DCP and LP-WUS for option 1 will not be configured at the same time, the function of dormancy indication should also be considered in the LP-WUS design. Besides DCI format 2_6, within active time, the dormant BWP switching can also be triggered by indication in DCI format 0_1/1_1 for data scheduling. so including the dormancy indication in LP-WUS may be not an obligation.

Furthermore, LP-WUS can use a more flexible method to trigger PDCCH monitoring when serving cells applicable for UE,e.g. trigger PDCCH monitoring according to traffic model in these Scells respectively. Option 1 of one or more serving cells based on gNB indication/configuration in LP-WUS can be considered. Typically, option 2 can be implemented as a case of option 1.

Observation 1: DCI format 0_1/1_1 can also provide the function of dormancy indication. whether including dormancy indication in LP-WUS needs further study.

Proposal 4: Support LP-WUS for UE configured with CA in connected mode.

Proposal 5: Support Option 1, i.e. including indication for one or more serving cells in a LP-WUS.

Activation and deactivation procedure

For connected UEs, the UE can directly monitor LP-WUS or not according to the configuration of the gNB, just like the process of monitoring ps-PDCCH. On the other hand, considering that the target coverage of LP-WUS and LP-SS may coincide with the PUSCH coverage of msg3, which is usually smaller than that of DL PDCCH, the UE may fall back to traditional PO monitoring when it leaves the coverage of LP-WUS and cell reselection is not triggered. In this case, it can be considered to send additional L1/L2 commands in WUS monitoring mode to obtain synchronization between the gNB and the UE. For example, the gNB can send a reset indication in the L1/L2 command to maintain the LP-WUS monitoring timers running in the gNB and the UE to maintain synchronization between them.

Meanwhile, for a cell with full LP-SS/LP-WUS coverage, no special configuration would be required. That is, LP-WUS could be configured/activated for a UE in the cell, and not used when it is released/de-activated by gNB in the cell.

Proposal 6: Consider both RRC signaling and L1/L2 command for activation and deactivation of LP-WUS monitoring.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: DCI format 0_1/1_1 can also provide the function of dormancy indication. whether including dormancy indication in LP-WUS needs further study.

Proposal 1: Support group cast LP-WUS for UE in connected mode.

Proposal 2:  LP-WUS procedures Option 1-1 ,which replaces DCP functionality, can be used as a baseline for UE in connected mode.

Proposal 3: Further consider the combination of Option 1-1 and Option 1-2-1 and the combination of Option 1-1 and Option 1-3 for UE in connected mode.

Proposal 4: Support LP-WUS for UE configured with CA in connected mode.

Proposal 5: Support Option 1, i.e. including indication for one or more serving cells in a LP-WUS .

Proposal 6: Consider both RRC signaling and L1/L2 command for activation and deactivation of LP-WUS monitoring.

References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Chair notes RAN1#116b

[3] Chair notes RAN1#117