3GPP TSG RAN WG1 #118                                                                                                   R1-2406901

Maastricht, NL, August 19th – 23rd, 2024

Source:               TCL

Title:                    Discussion on HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN

Agenda item:      9.11.2

Document for:    Discussion and Decision

- Introduction

In the RAN #104 meeting, a revised WID was approved for Rel-19 NR NTN[1], one of the objectives of the WID is related to Reduced Capability (RedCap) UEs with NR NTN operating, the details are shown in below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In RAN1#117 meeting [2], the following conclusion and observation have been achieved.

Conclusion

For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4.

- Note: further discussion on other cases is not precluded.

Conclusion

For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN.

Observation

TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective.

- Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item

In this contribution, further enhancements for HD-FDD RedCap UEs and eRedCap UEs to support operating in FR1-NTN are discussed.

- Discussion

The handling rules for DL and UL overlapping and back-to-back non-overlapping cases were defined in TS 38.213 under TN scenario, further enhancements of case 3 and case 4 for HD-FDD RedCap UEs and eRedCap UEs are needed.

- Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission

Handle rules: DL reception except UL transmission within processing time.

- Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission

Handle rules: UL transmission.

- Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission

Handle rule: Error case.

- Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

Handle rule: Error case

- Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission

Handle rule: Prioritize SSB reception.

- Case 6: Dynamic or semi-static DL collides with valid RO

Handle rule: Up to UE implementation.

- Case 7: Collision due to direction switching

Handle rule: Prioritize SSB reception or up to UE implementation based on sub-case

TA misalignment between gNB and UE

For TN scenario, the distance between gNB and UE is small, accordingly, the RTT between gNB and UE is also small, thus, weak impact of time advance on TN for data scheduling and potential TA misalignment to cause data collision can be avoid based on gNB implementation. However, for NTN scenario, due to non-negligible misalignment TA between gNB and UE, based on scheduling mechanism at gNB side to avoid collision between DL reception and UL transmission may not be suitable any more.

In current specification, the granularity of TA reporting is based on slot with 15Khz SCS, the Timing Advance field indicates the least integer number of slots for NTN or symbols for ATG, using sub-carrier spacing of 15 kHz for NTN and either 15 kHz or 30 kHz for ATG, greater than or equal to the Timing Advance value, which mean a TA reported by a UE need to align with the granularity of 1ms, however, there is an uncertainty of 1 ms about actual TA between gNB and UE. In other words, gNB may not know the actual location of a TA within a slot/ms, thus, based on gNB scheduling mechanism to avoid collision between DL reception and UL transmission will be caused resource waste due to gNB need to adapt a very conservative scheduling way.

Observation1: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, if DL reception and UL transmission collision avoiding based on gNB scheduling mechanism, resources waste will be caused due to TA reporting granularity.

In addition, for NTN, a Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

However, for timing advance report based on the variation between the current estimate of the timing advance value and the last reported timing advance value is equal to or larger than a offsetThresholdTA, with this way, there is a gap between the last reported timing advance and the current estimate of the timing advance value and the gap value is equal to or large than the configured offsetThresholdTA,  if  DL reception and UL transmission collision need to avoid based on gNB scheduling mechanism, thus, for a data transmission/reception, gNB need to reserved large of resources, for instance, as show in figure 1, due to TA mismatching between gNB and UE, then gNB cannot judge the actual overlapping area of DL reception and UL transmission. If gNB need to avoid collision between DL reception and UL transmission, then much resources need to be reserved to guarantee one of data transmission, this will cause large resource wasting.

Figure1, scheduling limitation due to TA mis-matching

Observation 2: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, if DL reception and UL transmission collision avoiding based on gNB scheduling mechanism, resources waste will be caused due to misalignment TA between gNB and UE.

Enhancements for collision of case 3 and case 4

Based on analysis of above, collision between DL reception and UL transmission overlapping and/or back-to-back non-overlapping symbols without sufficient gap caused by TA misalignment between gNB and UE for redcap UE under NTN scenario need to study.

One potential solution is to enhance the TA report, finer granularity for TA reporting by UE can be considered. Reserved bits within a TA reported MAC-CE in current spec can be used to indicate a re-fined TA granularity, e.g., 2 bits can be used, the symbols within a slot can be split into 4 groups. Then a symbol group based granularity of TA can be reported, it’s beneficial for gNB scheduling and potential scheduling collision between DL reception and UL transmission caused by TA granularity misalignment can be relieved. In addition, since TA value will be change due to UE and satellite movement, however, due to configured value of OffsetThresholdTA, UE does not need to report TA until a trigger condition is satisfied, which means, even through TA is changed, if the changed value is not large than configured OffsetThresholdTA value, the actual TA can not know by gNB in time. To handle of this, the most straightforward way is enabled UE to update TA frequently, as a result, a gNB can receive the actual TA timely.

Proposal 1: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, TA reporting with finer granularity can be considered.

Proposal 2: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, TA update frequently to let gNB know the actual TA in time can be considered.

Another potential solution is to define new rules to instead of relying on gNB to avoid the collision through scheduling or regarding it as an error case when the collision happened, to improve the resources efficient, just only one of a transmission of DL reception and UL transmission need to drop, by adopting this approach, the principle of maximizing system capacity can be taken into consideration

Proposal 3: For HD collision of case 3 and case 4, define a new rule to handle of collision between DL reception and UL transmission for HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN to achieve maximum system capacity can be considered.

  - SIB19 reception under NTN scenario

In current specification, other system information (OSI) are configured/indicated by SIB1, a periodicity and SI window can be configured to UE, when more than one SIs have the same periodicity, then the SIs can be transmitted within a SI window, the order of the SIs are configured via SIB1, as show in figure 2. However, with this case, the collision between SIB19 reception and UL transmission will be happened.

Figure 2, SIs pattern in time domain

For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, the UE need to receive SIB19 to acquire information of satellite (e.g., ephemeris) timely, due to collision between SIB19 reception and UL transmission, some opportunity for SIB19 receiving will be missed, especially for HD-FDD redcap UE, thus, the current rules for SIB19 reception and uplink transmission is not suitable any more, the opportunity of SIB 19 reception for HD-FDD Redcap UEs and eRedCap UEs need to guarantee.

Proposal 4: Opportunity of SIB 19 reception for HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN need to guarantee.

- Conclusion

In this contribution, the following observations and proposals have been made:

Observation1: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, if DL reception and UL transmission collision avoiding based on gNB scheduling mechanism, resources waste will be caused due to TA reporting granularity.

Observation 2: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, if DL reception and UL transmission collision avoiding based on gNB scheduling mechanism, resources waste will be caused due to misalignment TA between gNB and UE.

Proposal 1: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, TA reporting with finer granularity can be considered.

Proposal 2: For HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN, TA update frequently to let gNB know the actual TA in time can be considered.

Proposal 3: For HD collision of case 3 and case 4, define a new rule to handle of collision between DL reception and UL transmission for HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN to achieve maximum system capacity can be considered.

Proposal 4: Opportunity of SIB 19 reception for HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN need to guarantee.

4. References

[1] RP-240667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

[2] 3GPP RAN1 #117 meeting, Chairman notes.