3GPP TSG-RAN Meeting #118	R1-2407094

# Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on QCL assumption for periodic CSI-RS in unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the UE is configured with dl-OrJointTCI-StateList, UE assumes that the indicated Rel-17 TCI state is never applied, i.e. the corresponding TCI state must always be indicated/updated via RRC which creates additional signaling overhead and delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Periodic CSI-RS can implicitly follow indicated TCI state in uTCI framework |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Periodic CSI-RS TCI state is undefined unless qcl-InfoPeriodicCSI-RS in NZP-CSI-RS-Resource is provided. TCI state must always be updated with RRC configuration/reconfiguration NZP-CSI-RS-Resource |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR impacts TCI framework - If a network is implemented according to the CR and the UE is not, the gNB may expect the UE to follow the indicated TCI state for P-CSI-RS, while the UE would expect the network to signal it via higher layers. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.5	Antenna ports quasi co-location

< Unchanged text is omitted >

For semi-persistent CSI-RS, if the UE is configured with dl-OrJointTCI-StateList, the UE can assume that the indicated TCI-State is not applied.

For periodic CSI-RS, if the UE is configured with dl-OrJointTCI-StateList and qcl-InfoPeriodicCSI-RS is not configured, the UE can assume that the indicated TCI-State is applied.

< Unchanged text is omitted >

##### 5.2.2.3.1	NZP CSI-RS

-	qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS configured with qcl-Type set to 'typeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP. The reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell. When this field is absent for target periodic CSI-RS, the UE can use QCL information included in the indicated TCI-State.

< Unchanged text is omitted >