3GPP TSG RAN WG1 #118								R1-2407124

Maastricht, Netherlands. 19 – 23 August 2024

Agenda Item 	:	9.4.2.4

Source 	:	Sony

Title 	:	Controllable Carrier Wave Characteristics

Document for 	:	Discussion and decision

# Introduction

In RANP#103 Maastricht, the study item on Ambient IoT was updated [1], including the following objective:

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

The following proposals [2] that are relevant to this agenda item were endorsed at RANP#103 Maastricht:

Proposal 3v2 - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary

While RAN1 has been mainly concerned with the waveform characteristics of the carrier wave, there are other aspects / characteristics of the carrier wave that also require consideration in order to allow work to progress in RAN2 and RAN4. The actual work on the higher layer control signalling for control of the carrier wave emitter can be left to RAN2 and / or RAN3 working groups. This control does not necessarily happen over a Uu or 3GPP-backhaul interface.

This document discusses the following RAN1-related aspects:

- Control of on / off status of WC node for energising tag, network energy saving and spatial multiplexing.

- Control of CW type – whether single unmodulated tone or multiple unmodulated tones.

- Control of frequency of CW.

- Control of power of CW.

Aspects of carrier wave node control that do not impact RAN1 are considered in our companion RAN2 [2] and RAN4 [3] Tdocs.

# RAN1 aspects of CW Node Control

2.1 Control of CW node when responding to DL trigger message

The network needs to be able to control the CW node for the D2R response after a DL trigger message. If the CW node is not controlled, too many devices may respond, resulting in high collision and interference. Continuous transmission from the CW node would also be undesirable from the perspective of network energy saving / NES. It should be possible to control the ON/OFF state of the CW node within a timescale of the time between the R2D command and the associate D2R response, TR2D_min.

Observation 1: It should be possible to turn the CW node on and off.

2.2 Spatial multiplexing

When there are multiple AIoT devices in an area, a degree of spatial multiplexing can be achieved by activating different CW nodes in the coverage area at different times. This can allow different tags within the coverage area to respond at different times. In order to support this spatial multiplexing functionality, it should be possible to turn CW nodes on and off.

2.3 Control of unmodulated single-tone vs two unmodulated single tone CW waveform

The following observation was made at RAN1#117 Fukuoka:

Observation For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Based on this observation, it is apparent that some infrastructure (reader) vendors consider that there is a complexity impact from support of two unmodulated CW tones. It is assumed that this complexity is surmountable and some infrastructure vendors will be able to solve any D2R decoding complexity issue.

There could potentially be issues on the type of CW that a device can successfully backscatter. While the backscattering circuit merely switches load impedances (and is hence mainly agnostic to the CW itself), there could be complexity issues if different D2R waveforms are required for different CW types.

It is hence currently unclear whether the number of tones for the CW waveform will be fixed (by specification) on configurable. A configurable number of tones could be useful in case either (1) the tag has a capability on the types of CW that it can successfully backscatter (e.g. if different waveforms / load impedance switching patterns are required depending on the number of CW tones. The ability to create that waveform / switching pattern might be beyond the capability of the device)  or (2) the reader has a capability on the types of backscattered CW waveform that it can demodulate (as a function of complexity etc). In the case that different tags have different capabilities, it would be necessary to switch the waveform type of the CW within a timescale of TR2D_min. In the case that the reader has a capability on the CW waveform type, the CW node should be configurable semi-statically.

Observation 2: It should be possible to control the number of tones transmitted by the CW node.

2.4 Control of CW waveform frequency

Controlling the frequency of the CW would allow for frequency division multiplexing on the D2R link: tags operating in the coverage areas of different CW nodes would backscatter at different frequencies, allowing the reader to separate tag transmissions at the different frequencies.

The benefit of a two-tone CW waveform is to provide resilience to frequency selective fading on the D2R link. If it were found that one of the tones were consistently faded for a tag, or for tags under the control of a CW node, it would be beneficial to change the frequency of one (or both) of the CW tones.

Observation 3: It should be possible to control the frequency of the tones(s) transmitted by the CW node.

2.5 Power control

An ultra-low complexity Ambient-IoT tag is unlikely to have the ability to control its reflection power. This would require precise control of the load impedance for device type 1 or 2a, or would require control of the gain of a reflection amplifier for device type 2a [3]. In order to power control receptions at the reader and to control interference, the transmit power of the CW node could be controlled.

Observation 4: It should be possible to control the transmit power of the CW node.

2.6 Summary: Controllable characteristics of carrier wave

Based on the observations above, we make the following proposal on the characteristics of the carrier wave that should be controllable by the network.

Proposal 1: The following characteristics of the carrier wave should be controllable:

  - ON / OFF status

  - Number of tones of the CW

  - Frequency of the CW tones

  - Transmit power of the CW

# Conclusion

This document has considered the characteristics of the CW that should be controllable through control of the CW node. The following observations and proposal are made:

Observation 1: It should be possible to turn the CW node on and off.

Observation 2: It should be possible to control the number of tones transmitted by the CW node.

Observation 3: It should be possible to control the frequency of the tones(s) transmitted by the CW node.

Observation 4: It should be possible to control the transmit power of the CW node.

Proposal 1: The following characteristics of the carrier wave should be controllable:

  - ON / OFF status

  - Number of tones of the CW

  - Frequency of the CW tones

  - Transmit power of the CW

References

[1]	RP-240826. “Study on solutions for Ambient IoT (Internet of Things) in NR”. RANP#103. Maastricht, Netherlands. March 2024.

[2]	R2-2406614. “Considerations on functionality aspects for Ambient IoT”. Sony. RAN2#127. Maastricht, Netherlands. March 2024.

[3]	R4-2411537. “Further considerations on the ambient IoT device implementation and RF aspect”. Sony. RAN4#112. Maastricht, Netherlands. August 2024.