3GPP TSG RAN WG1 #119			R1- 2409489

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.3.2

Source: 	LG Electronics

Title: 	Discussion on SBFD random access operation

Document for:	Discussion and decision

# Introduction

Subband non-overlapping full duplex (SBFD) was included as one of Rel-19 WIs to be studied and specified in the Evolution of NR duplex operation approved in RAN #104 [1]:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, the followings are discussed for SBFD random access operation.

- RACH framework

- RACH configuration

- RO validity

- PRACH repetition

- PRACH power control

- Msg2/Msg3/Msg4 related transmission/reception in SBFD symbols

# RACH framework

Comparing with semi-static TDD HD, SBFD would reduce UL latency for SBFD aware UE to allow transmitting in SBFD UL subband on consecutive SBFD slots on top of UL slots. With enhanced configuration flexibility providing more UL opportunities, SBFD could contribute to improve system capacity as well. 3GPP has specified random access operation relevant features to be fit in different type of circumstances. The 4-step RACH was introduced as part of the first NR release, 3GPP Rel.15 while 2-step RACH was adopted in Rel.16 to simplify the random access procedure by combining step 1/3 into a single step A and step 2/4 into a single step B that benefits by reducing the latency of the RACH procedure. In SBFD operation using the UL subband on consecutive SBFD symbols, both 4-step RACH and 2-step RACH could secure more opportunities for PRACH transmission which has the advantage of reducing random access latency and decreasing the RACH collision probability. It continues on the journey to develop and maintain RACH for RedCap in Rel.17 and PRACH repetition in Rel.18, respectively. Enabling PRACH transmission on SBFD UL subband in the SBFD symbols can not only enhance coverage but also reduce random access latency.

With the agreements made in the last RAN1 meetings, SBFD aware UE could utilize SBFD UL subband by enabling a single PRACH transmission and PRACH transmission with preamble repetitions to reduce PRACH collision probability and random access latency.

Agreement For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols. Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-ROs and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases. Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported.

It was also agreed to support Type-1 random access procedure (4-step RACH) in SBFD symbols for SBFD aware UEs while Type-2 random access procedure is FFS in RAN1 #116. In Rel.19, enabling those existing random access operation in SBFD UL subband could leverage to facilitate SBFD aware UE to select ROs suitably configured during SBFD operation.

Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

Proposal 1: RAN1 to allow 2-step RACH to be supported in SBFD symbols for SBFD aware UEs to fit into the latency limited conditions.

For semi-static SBFD operation with explicit configuration of SBFD, it was discussed and agreed in RAN1 #116 and #117 that for RRC connected mode UEs, at least cell-specific configuration on time period and frequency location of SBFD subbands is supported within a TDD carrier in AI 9.3.1. Furthermore, as in the agreement made in RAN1 #118 since it is decided that SBFD operation supports random access in SBFD symbols by SBFD aware UEs in RRC CONNECTED mode and RRC_IDLE/INACTIVE mode, the previous agreements for UEs in RRC_CONNECTED mode are extended to SBFD aware UEs in RRC_IDLE/INACTIVE mode.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode. Agreement Extend the previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode.

Having consensus on the feasibility of initial random access in SBFD symbols in RRC_IDLE/INACTIVE mode, semi-static cell-specific configuration to signal frequency and time location of the SBFD UL subband should be broadcasted via SIB1 for SBFD aware UE  in RRC_IDLE/INACTIVE mode to allow transmissions of PRACH, Msg3 PUSCH and PUCCH for Msg4 in SBFD symbols, and relevant enhancements for CBRA should be considered to have common operation for SBFD aware UE in both RRC_CONNECTED mode and RRC_IDLE/INACTVIE mode.

Observation 1: Referring to the agreement made in RAN2 on cell-specific SBFD time/frequency configuration provided by SIB1, semi-static configuration to signal frequency and time location of the SBFD UL subband are broadcasted via SIB1 for SBFD aware UE  in RRC_IDLE/INACTIVE.

- RAN2 Agreement in [R2-2407572]: Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

Agreement A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE: - The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH. - UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols - There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH. - Other assumptions are not precluded. NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB. Agreement Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. - The RO includes at least one DL symbol configured by tdd-UL-DL-ConfigurationCommon

While RAN1 had exhausted to reach the agreement on the conditions for a valid RO across SBFD symbols and non-SBFD symbols in the same slot or across slots, it is necessary to make certain whether RAN1 has the common understanding on how the network configures such RO as valid since there might have different alternatives to implement. For Alt. 1, a new parameter could be introduced to configure explicitly whether the RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is valid or invalid when such ROs are configured. For Alt. 2, the network configuration as in the agreement may not necessarily meant to be the new parameter. For example, given the specific conditions assumed by gNB and SBFD aware UE, it is up to gNB to ensure that it configures PRACH occasions with the PRACH configuration index in additional PRACH configuration of RACH configuration Option 2 such that the RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is regarded valid implicitly once configured. Otherwise, network does not configure such ROs since it is invalid by default based on the agreement. On the other hand, according to the agreement in RAN1 #118, for RACH configuration Option 1, an RO across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. And it was also mentioned, but not formally agreed, for RACH configuration Option 1, a RO across SBFD symbols and non-SBFD symbols is identified and determined as valid since the legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. We may need further discussion whether the network configuration defined in RACH configuration Option 2 could be applicable to RACH configuration Option 1 as well.

Observation 2: For network configuration in the agreement on a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots, consider two alternatives to support.

- Alt. 1: Network explicitly indicates whether such RO is valid.

- Alt. 2: Network configuration is not necessarily meant to be a new parameter to show whether a RO across SBFD symbols and non-SBFD symbol is valid or not. For example, such ROs are regarded as valid implicitly once configured by PRACH configuration index in additional RACH configuration.

Observation 3: During the discussion in RAN1 #118, it was mentioned, but not formally agreed, for RACH configuration Option 1, a RO across SBFD symbols and non-SBFD symbols is identified and determined as valid since the legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

Proposal 2: RAN1 to consider a network configuration agreed for RACH configuration Option 2 is applicable to RACH configuration Option 1.

With the agreement made in RAN1 #116 and RAN1 #116bis, SBFD aware UE can transmit PRACH transmission on SBFD symbols for both CFRA and CBRA procedures in RRC_CONNECTED mode. Following the current specification, the indexing of the PRACH occasion indicated by the mask index value is reset per mapping cycle of consecutive PRACH occasions per SS/PBCH block index. The UE selects for a PRACH transmission the PRACH occasion indicated by PRACH mask index value for the indicated SS/PBCH block index in the first available mapping cycle.

Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used

Given separate SSB-RO mapping cycles for legacy ROs and additional ROs, RAN1 may need to discuss whether clarification and specification effort if any for CFRA operation in SBFD random access are needed.  For example, there might be ambiguity for SBFD aware UE to figure out the first valid mapping cycle when there are both the first valid mapping cycle for legacy ROs and the first valid mapping cycle for additional ROs, respectively. Several alternatives could be considered as below.

- Alt 1: the network explicitly indicates whether CFRA operation is based on legacy ROs or additional ROs.  Existing parameters such as SS/PBCH block index and mask index could be reused since the network is supposed to configure the values with the indicated RO type.

- Alt 2: SBFD aware UE is allowed to select RO type for CFRA operation in SBFD random access based on certain specified/configured conditions. In accordance with the RO type to be selected, corresponding SS/PBCH block index and/or mask index values could be separately indicated by the network. The separate SS/PBCH block index parameter and/or mask index parameter could be introduced for legacy RO and additional RO, respectively.

- Alt 3: SBFD aware UE selects the first available mapping cycle for additional ROs. CFRA operation in SBFD random access is based on prioritizations for additional ROs. Existing parameters such as SS/PBCH block index and mask index could be reused for additional RO.

As for the first valid mapping cycle, there may be one or more mapping cycles depending on the association period in accordance with the increased number of configured ROs for PRACH transmission. In that sense, it seems not to be necessary to have a restriction to the only first valid mapping cycle for configuration flexibility for CFRA operation in SBFD random access.

Observation 4: For PRACH mask indication for CFRA triggered by PDCCH order for SBFD aware UE, RAN1 to discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

- The ambiguity for SBFD aware UE to figure out the first valid mapping cycle when there are the first valid mapping cycle for legacy ROs and the first valid mapping cycle for additional ROs, respectively.

- The restriction release on only first valid mapping cycle.

Proposal 3: To handle the ambiguity on the first valid mapping cycles, consider the following alternatives to indicate/determine additional-RO only, legacy-RO only, or both at least for RACH configuration Option 1.

- Alt 1: the network explicitly indicates whether CFRA operation is based on legacy ROs or additional ROs.

- Alt 2: SBFD aware UE is allowed to select RO type for CFRA operation in SBFD random access based on certain specified/configured conditions.

- Alt 3: SBFD aware UE selects the first available mapping cycle based on the prioritization for additional ROs.

# RACH configuration

## 3.1 RACH configuration options

Further discussion and decision should be made for RACH configuration options. The RACH configuration Option 1 is a one single legacy RACH configuration with possible enhancement by reinterpreting only based on the existing parameters for SBFD aware UE from RO configuration aspect. The ROs in SBFD symbols indicated as DL for legacy UE are considered invalid while those are determined as valid for SBFD aware UE.

Agreement For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols. - Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon). - FFS the details - FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s). Agreement For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, no separate prach-ConfigurationIndex to be configured in this option. Conclusion For SBFD aware UE s in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs Agreement For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size. - Note: Other alternatives are not precluded Agreement For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives. - Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation. - Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs. - Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs). - FFS: Possible modifications

When it comes to msg1-FrequencyStart in rach-ConfigCommon, we support to reinterpret msg1-FrequencyStart as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs in SBFD symbol. It is not to cause UL resource fragmentation in non-SBFD symbol by allowing same value in msg1-FrequencyStart for legacy ROs and additional ROs while additional ROs could be configured within the UL usable PRBs location to support SBFD random access in Figure 1. It turns out to be the simple and practical approach considering the commercial NR deployments since the ROs are usually configured in the lowest part of the BWP, e.g. starting in the first few PRBs within the initial UL-BWP to allow for PUCCH allocation at lower edge of the UL BWP as discussed in RAN1 #118bis.

Figure 1

Meanwhile, some companies share concerns when some ROs are configured in SBFD symbols regarded as flexible by tdd-UL-DL-ConfigurationCommon, the frequency location of all configured ROs is expected to be located within UL usable PRB and reinterpretation on msg1-FrequencyStart may not be needed. The conclusion was made in RAN1 #117 that if legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs. With the conclusion to be honored, if necessary the frequency location in UL usable PRB could be configured at the bottom side in frequency domain such that the lowest PRB of UL usable PRBs in SBFD symbols becomes the lowest PRB of wideband in non-SBFD symbols in Figure 2. Another concern is the configured ROs could be located out of UL useable PRBs in SBFD symbols when msg1-FrequencyStart value is larger than the UL useable PRBs. It should be regarded corner case in real deployment. Furthermore, the network would rather choose RACH configuration Option 2 for SBFD aware UEs by introducing additional features or specific description to consider and resolve those ROs outside UL usable PRBs in SBFD symbols. The fundamental purpose to introduce RACH configuration Option 1 is to implement simple RACH configuration to live with less configuration flexibility. Accumulative features or specific description to cover each concern would even increase both the UE implementation complexity and specification efforts.

Figure 2

Observation 5: Given the conclusion made in RAN1 #117 and fundamental purpose to introduce RACH configuration Option 1, the network should ensure that configured ROs are within the UL usable PRBs in SBFD symbols. Allowing accumulative features or specific description could increase both the UE implementation complexity and specification efforts

Proposal 4: RAN1 to support Alt 2-1 in RACH configuration Option 1.

- The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

The RACH configuration Option 2 is to use two separate RACH configurations using multiple PRACH configurations (e.g. one legacy RACH configuration for legacy ROs and one additional RACH configuration for additional ROs). The additional RACH configuration could provide the parameters corresponding to additional ROs for SBFD aware UE, and the additional ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD aware UEs in Figure 3. With two RACH configurations providing each preamble format by each PRACH configuration index, it looks natural to allow different preamble format configured by gNB for legacy RO and additional RO if needed.

Figure 3

Proposal 5: With two separate RACH configurations in RACH configuration Option 2, same or different preamble formats by each PRACH configuration index can be configured by gNB. (i.e., one for legacy RACH configuration, one for additional RACH configuration).

Considering the parameters in additional RACH configuration are meant to be configured only for additional ROs, it seems natural that the parameters are described by means of UL usable PRBs. For example, the parameter msg1-FrequencyStart indicates the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs as shown in Figure 4.

Figure 4

Proposal 6: The parameter msg1-FrequencyStart in additional RACH configuration of RACH configuration Option 2 indicates the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

## 3.2 RO selection and switching

Given two different types of RO (legacy-ROs and additional-ROs), RAN1 made agreements to consider options that SBFD aware UE may select one set of random access resources to start initial PRACH transmission and PRACH transmission re-attempt in one random access procedure in RAN1 #118.

Agreement For SBFD aware UE s and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UE s and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure RAN2 agreement in R2-2409212 Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In RAN2#127bis, RAN2 agreed UE selects one type of ROs between legacy-ROs and additional-ROs for initial PRACH transmission, and UE is allowed to switch to the other type of ROs for PRACH transmission re-attempt. While RAN2 is under discussion to decide conditions and relevant parameters for RO type selection and switching, RAN1 could still work on those conditions and mechanisms with potential parameters to get aligned with RAN2’s decision.

Potentially, two conditions could be considered. Firstly, a threshold can be used as condition of RO selection/switching. In Rel.18, there are two thresholds related to select RO, msgA-rsrp-threshold and rsrp-thresholdSSB. msgA-rsrp-threshold is used to select whether to use 2-step RA or 4-step RA. rsrp-thresholdSSB is used to select one of SSBs which exceed the rsrp-thresholdSSB. Considering the purpose to introduce RO type selection/switching, a parameter similar with msgA-rsrp-threshold should be agreed, and at least a new parameter (e.g. rsrp-thresholdSBFD) as certain specified/configured condition should be defined. Meanwhile, measuring RSRP in every single PRACH transmission and comparing it to the threshold may result in significant complexity for the SBFD aware UE. So, it is suitable to use the threshold only for initial PRACH transmission. Secondly, transmission number of PRACH can be used as condition of RO type switching. In RAN2#127bis, RAN2 already decided to switch RO types after certain number of times of RACH attempt in SBFD RACH occasions without details. In Rel.18, RRC parameter preambleTransMax is used to configure max number of RA preamble transmission performed before declaring a failure. If a UE variable PREAMBLE_TRANSMISSION_COUNTER is over the RRC parameter preambleTransMax, UE considers the RACH procedure is completed unsuccessfully. Especially in 2-step RA, if PREAMBLE_TRANSMISSION_COUNTER is over the RRC parameter preambleTransMax, UE sets the RA_TYPE to 4-step RA which means UE switches RA type from 2-step RA to 4-step RA. Sharing conventional RRC parameter preambleTransMax for condition of RO type switching may cause unnecessary latency to complete the entire RACH procedure since preambleTransMax is meant to be used two times for one RO type as initial PRACH transmission attempt and re-attempts and switched RO type for PRACH transmission re-attempt for the rest RACH procedure. So, it is suggested to configure a new parameter (e.g. maxtrans_SBFD) for certain configured number of times of RACH attempt before UE is allowed to switch RO type.

Furthermore, RAN2 at least agreed the switching direction from additional-RO to legacy-RO when UE selects additional-RO based on certain specified/configured conditions/prioritizations for initial PRACH transmission attempt since it is a simple and clear approach considering additional-RO may experience severe interference, self-interference and/or gNB-to-gNB CLI. But in vice versa, it seems not to have enough advantage in switching from legacy-RO to additional-RO except for load balancing at the price of increased complexity such as new conditions to switch at least once or potentially more times back and forth and complete the RACH procedure. It also requires the enhanced RAN 2 RACH framework to support bidirectional switching and/or fallback mechanisms to be introduced in Rel.19.

Observation 6: While RAN2 is under discussion to decide conditions and relevant parameters for RO type selection and switching, RAN1 could still work on those conditions and mechanisms with potential parameters to get aligned with RAN2’s decision.

Proposal 7: For initial PRACH transmission of RACH configuration Option 1 and Option 2, at least threshold should be used for condition of RO type selection.

Proposal 8: For both of RACH configuration Option 1 and 2, for certain specified/configured conditions/prioritizations of initial PRACH transmission, a new parameter corresponding to rsrp-threshold (e.g. rsrp-thresholdSBFD) should be introduced.

Observation 7: During PRACH transmission re-attempt, measuring RSRP for rsrp-threshold comparison in every single RACH transmission will increase UE complexity.

Proposal 9: For PRACH transmission re-attempt of RACH configuration Option 1 and Option 2, at least transmission number of PRACH should be used for condition of RO type switching.

Proposal 10: For PRACH transmission re-attempt of RACH configuration Option 1 and Option 2, for a certain number of times to switch into other type of ROs, either a new parameter corresponding to the specific/configured number of re-attempts or existing parameter (e.g. maxtrans_SBFD or preambleTransMax) should be considered.

- A new parameter corresponding to the specific/configured number of re-attempts is preferred.

## 3.3 Random access configurations table

For SBFD random access operation, reusing the random access configurations table for unpaired spectrum was agreed since it is the simple approach at the price of limited configuration flexibility in RACH configuration Option 1. During RAN1 #118bis there was the agreement that two tables for unpaired spectrum (i.e., Table 6.3.3.2.-3 for FR1 and unpaired spectrum and Table 6.3.3.2-4 for FR2 and unpaired spectrum in TS 38.211) are reused with FFS whether to introduce new parameter(s) to determine the slot/subframe number for ROs in SBFD symbols in RACH configuration Option 2. Some companies showed concerns on existing random access configuration tables for unpaired spectrum since configured RO on time domain resource were designed for UL symbols/slots as the target which locate at the last slots. In SBFD operation, even though ROs could be configured on a set of symbols of a slot that are indicated as DL by tdd-UL-DL-ConfigurationCommon, given inherent restrictions from current random access configuration tables, gNB may not fully configure ROs to be located in those SBFD symbols/slots on top of UL symbols/slots.

Agreement For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, use existing random access configurations tables for unpaired spectrum (i.e., Table 6.3.3.2-3 for FR1 and Table 6.3.3.2-4 for FR2 in TS38.211). Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. Agreement Confirm the following working assumption. Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

With the fundamental concept to provide configuration flexibility in RACH configuration Option 2, there were discussions on several alternatives which are related to how to modify the subframe number and/or the slot number in random access configurations tables and then eventually to configure more ROs for SBFD aware UE.

- Alt 1: Introduce a new parameter to configure an additional subframe/slot number. For example, the additional subframe/slot number could replace the configured subframe/slot number by the PRACH configuration index indicated in existing random access configurations table for SBFD aware UE. For another example, the additional subframe/slot number could be combined with the configured subframe/slot number by the PRACH configuration index indicated in existing random access configurations table for SBFD aware UE.

- Alt 2: Introduce a new parameter to indicate a subframe offset value or a slot offset value, and apply each offset value to the configured subframe numbers or slot numbers by the PRACH configuration index indicated in existing random access configurations table for SBFD aware UE.

- Alt 3: Introduce a new parameter to indicate whether a subframe or a slot to be valid or invalid for the configured ROs by the PRACH configuration index indicated in existing random access configurations table for SBFD aware UE.

Proposal 11: For RACH configuration Option 2, support to introduce a new parameter to modify the subframe number or the slot number configured by existing random access configurations tables and then to allow more ROs.

# RO validity

## 4.1 RO validation rule

Agreement For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, - no enhancements for the RO validation rule for the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any). - FFS: the ROs in non-SBFD symbols that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - FFS: It’s up to network configuration to ensure the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, which are valid for non-SBFD aware UEs based on legacy RO validation rule, are also valid for SBFD aware UEs (i.e., the configured ROs in SBFD symbols, if configured as flexible by tdd-UL-DL-ConfigurationCommon, are within the UL usable PRBs) - the RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon is valid if at least: - Time and frequency resource of the RO are fully within UL usable PRBs, and not overlapped with SSB - FFS: Other condition. Note: For the case that all the SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, there is no restriction that all the configured ROs in SBFD symbols should be within the UL usable PRBs. Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs Agreement Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon. - The RO includes at least one DL symbol configured by tdd-UL-DL-ConfigurationCommon Agreement For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation. - Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least Ngap symbols after the SSB. - Note: The Ngap here is the same as the Ngap in the current specification Agreement For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD aware UE s in RRC CONNECTED state, down-select (in RAN1#117) from the following alternatives: - Alt 2-3: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs. - FFS: The case where the additional-ROs partially overlap with non-SBFD symbols - Alt 2-4: - The additional-ROs in non-SBFD symbols configured by additional RACH configuration can be valid for SBFD-aware UEs. For the legacy-ROs configured by legacy RACH configuration, the legacy RO validation rules and the legacy SSB-RO mapping rules are followed for SBFD aware UEs. Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD aware UE s). Agreement For RACH configuration Option 2, an additional-RO is valid if: - It is within SBFD symbols, or network configures that it can be valid if it starts from SBFD symbol and ends in non-SBFD symbol either in the same slot or across different slots. - It starts at least Ngap symbols after a last downlink non-SBFD symbol. - It starts at least Ngap symbols after the latest SSB - It does not overlap with SSB in time domain Note1: The Ngap here is the same as the Ngap in the current specification. Note2: It is up to network to ensure an additional-RO is configured within the bandwidth of the UL usable PRBs.

For RACH configuration Option 1 and Option 2, the enhancement in RO validation rule has been mostly settled down. Considering the current specification deliberately describes to avoid conflicts leading to error cases or unexpected UE behaviors, we prefer reusing the existing collision handling principles for HD-FDD RedCap UE, i.e., leave to UE implementation for dynamic or semi-static DL vs. valid RO.

Proposal 12: For dynamic or semi-static DL vs. valid RO,

- Reuse the existing collision handling principles for HD-FDD RedCap UE, i.e., leave to UE implementation.

## 4.2 SSB-RO mapping

Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used Agreement For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs. - FFS the association period and association pattern period. Agreement For RACH configuration Option 1 with Alt 1-1, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs. - The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs. Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs Agreement Confirm the following Working Assumption: Working Assumption For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration. - FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

For RACH configuration Option 2, legacy SSB-RO mapping rule (e.g. SSB-RO mapping order such as (e.g., frequency resource, time resource, and PRACH slots) is applicable to legacy RACH configuration and additional RACH configuration, respectively. For FFS in the agreement in RAN1 #118bis, network may ensure that the legacy ROs configured by legacy RACH configuration and the additional ROs configured by additional RACH configuration does not overlap if necessary. Especially for FR2, network may also ensure that the additional RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB as in the agreement for a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots. Further enhancement for FFS is not required.

Proposal 13: For RACH configuration Option 2, ensuring that network can handle the concern on the legacy ROs configured by legacy RACH configuration overlapped with the additional ROs configured by additional RACH configuration in time domain, further enhancement for FFS is not required in the agreement.

- For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

# PRACH repetition

PRACH repetition was one of key features specified during Rel.18 which could be applicable to SBFD random access operation to support the coverage enhancement. In accordance with the agreements made in RAN1 #118 and RAN1 #118bis, a set of RO (e.g., RO group) for PRACH transmission with preamble repetition could consist of only legacy ROs (e.g., legacy RO group) and only additional ROs (e.g., additional RO group) for SBFD aware UE in SBFD random access.  For RACH configuration Option 1, SBFD aware UE may select either legacy RO group or additional RO group. For RACH configuration Option 2 with separate RACH configurations, once SBFD aware UE selects one of RACH configurations, RO group type for PRACH repetition could be determined accordingly.

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-ROs and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases. Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported. Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

The reason for Working Assumption in RAN1 #118bis was that the discussion results from Rel.18 CE CR could make an impact on PRACH repetition in SBFD random access procedure depending on the interpretation of highlighted description as below. This discussion is about whether rach-ConfigCommon is allowed to configure one or more time periods depending on UE’s capacities and featureCombinationPreambleList-r17 configurations. Considering Working Assumption is about whether to reuse the framework (including the time period concept) for Rel.18 PRACH repetition, there seem not to wait until corresponding clarification currently under discussion is completed. With this Working Assumption to be confirmed, more than one time period will be configured with one rach-ConfigCommon, and the further future agreements on PRACH repetition in SBFD random access would be implemented on top of modified specification after all.

8.1 Random access preamble For a PRACH transmission with preamble repetitions, a time period, starting from frame 0, is the smallest integer number of association pattern periods such that at least one set of valid PRACH occasions for each of the NTxSSB SS/PBCH block indexes can be determined within the time period for all configured number of preamble repetitions. The set(s) of valid PRACH occasions for each configured number of preamble repetitions repeats every time period.

Observation 8: Since the discussion in Rel.18 CE CR is about whether rach-ConfigCommon is allowed to configure one or more time periods depending on UE’s capacities and featureCombinationPreambleList-r17 configurations, there seem not to wait until corresponding clarification currently under discussion is completed.

Observation 9: With this Working Assumption to be confirmed, more than one time period will be configured with one rach-ConfigCommon.

Proposal 14: The Working Assumption should be confirmed

- Working Assumption

For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs.

- the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

While RAN1 and RAN2 started discussion on RO type selection for initial transmission and re-attempt for single PRACH transmission, whether/how to operate RO group type selection should be discussed for PRACH repetition in SBFD random access separately for SBFD aware UE as well.

Similar to initial RO type selection, SBFD aware UE selects one type between legacy RO group and additional RO group based on certain specified/configured conditions/prioritizations. At least the certain condition could be a threshold related to rsrp, and a new threshold parameter could be considered for purpose of RO group type selection and corresponding PRACH repetition number, respectively. For example, when it comes to msg1-RepetitionNum and/or rsrp-ThresholdMsg1-RepetitionNumN (e.g., N could be 2, 4, and 8) for legacy RO group and additional RO group, PRACH repetition number (e.g., msg1-RepetitionNum) and/or values in RSRP-threshold for each PRACH repetition number (e.g., rsrp-ThresholdMsg1-RepetitionNum2, rsrp-ThresholdMsg1-RepetitionNum4, rsrp-ThresholdMsg1-RepetitionNum8 if any) could be differently configured between legacy RO group and additional RO group.

Considering the starting point of RAR window is after the last symbol of the last valid RO in the RO group in Rel.18 coverage enhancement, SBFD aware UE is not aware of whether transmitted PRACH is correctly received at gNB before all PRACH transmissions over configured ROs in RO group are completed and RAR is received on time. Furthermore, if PRACH repetition with initial RO group type fails, whether to increase repetition number of the same RO group type or to switch into different RO group type should be considered. For example, increasing PRACH repetition number up to the max value (e.g. 8) among all configured number of preamble repetitions in additional RO group as initial RO group selection for the rest of the random access procedure does not provide any benefits, since there could be the presence of self-interference and/or inter-gNB interference in SBFD symbols. Similar to RO re-attempt for single PRACH transmission, SBFD aware UE first uses RO group of the same type as the earlier PRACH transmission for up to a certain PRACH repetition number, then use RO group of the other type for the rest of the random access procedure. For a certain PRACH repetition number, a new parameter could be considered. When switching into other type of RO group, further discussion may need whether SBFD aware UE starts PRACH repetition with the certain PRACH repetition number.

Observation 10: Whether/how to operate RO group type selection should be discussed for PRACH repetition in SBFD random access for SBFD aware UE in RACH configuration Option 1 and RACH configuration Option 2.

Proposal 15: For SBFD aware UE, consider the following options for initial RO group type selection:

- Alt. 1: Select one RO group type between legacy RO group and additional RO group based on certain specified/configured conditions/prioritizations.

    - Same concept in RO type selection for initial PRACH transmission.

- Alt. 2: It’s up to UE implementation to select one RO group type between legacy RO group and additional RO group.

Proposal 16: For SBFD aware UE, consider separate msg1-RepetitionNum and/or rsrp-ThresholdMsg1-RepetitionNumN (e.g., N could be 2, 4, and 8) for legacy RO group and additional RO group.

- Values in PRACH repetition number and/or corresponding RSRP threshold could be differently configured between legacy RO group and additional RO group.

Proposal 17: For SBFD aware UE, consider the following options for RO group type switching if the earlier RO group fails:

- Alt. 1: Always use the RO group of the same type (i.e., legacy RO group or additional RO group) with increased PRACH repetition number as the earlier RO group for the rest of the random access procedure.

- Alt. 2: First use RO group of the same type as the earlier PRACH transmission for up to a certain PRACH repetition number, then use RO group of the other type for the rest of the random access procedure.

    - Same concept in RO type switching for RACH re-attempt

- Alt. 3: Switch into RO group of the other type if certain conditions are met for the rest of the random access procedure.

Observation 11: When switching into other type of RO group, further discussion may need whether SBFD aware UE starts PRACH repetition with the certain PRACH repetition number if Alt.2 is determined.

# PRACH power control

Agreement For Option 1 (i.e., use one single RACH configuration with possible enhancement) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, consider the following alternatives to derive the time and frequency resources of the configured ROs in SBFD symbols. - Alt 1-1: only based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon). - FFS the details - FFS: Alt 1-2: based on the existing parameters of the single RACH configuration (e.g., prach-ConfigurationIndex, msg1-FDM and msg1-FrequencyStart in rach-ConfigCommon) and newly introduced parameter(s). Agreement For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols - FFS: Details of the separate power control parameters Updated proposal 1-1-12a (Open) in RAN1 #117: For SBFD-aware UEs in RRC CONNECTED state and RACH configuration Option 2, no new PRACH power control parameters will be introduced for the additional RACH configuration, except the existing PRACH power control parameters (i.e., preambleReceivedTargetPower, powerRampingStep, preambleTransMax).

For RACH configuration Option 1, it is agreed to use separated power control parameter in RAN1#118bis. However, it is not decided in details of the separate power control parameter. To minimize spec impact, additional parameter for additional-RO corresponding existing parameter can be considered. For example, preambleReceivedTargetPower_SBFD, powerRampingStep_SBFD and preambleTransMax_SBFD for additional–RO can be introduced separately in addition to existing parameters preambleReceivedTargetPower, powerRampingStep and preambleTransMax for legacy-RO in rach-ConfigGeneric.

For initial PRACH transmission attempt and re-attempt until reaching the certain (configured) number of times of RACH attempt in SBFD RACH occasions, separated power control operation for PRACH transmission can be applied based on legacy RACH power control mechanism with preambleReceivedTargetPower and preambleReceivedTargetPower_SBFD, respectively.

One thing should be considered is how to control power between additional-RO and legacy-RO when RO type switching occurs. With less self-interference and/or gNB-to-gNB CLI in non-SBFD symbol when additional-RO is switched to legacy-RO, it seems reasonable that UE suspends the ramping counter counting (e.g. PREAMBLE_POWER_RAMPING_COUNTER) of ramping step, and as the concept of switching operation from 2-step RA to 4-step RA, calculate the power offset value with the counted ramping step as below. For PRACH transmission re-attempt, the power offset could be introduced to keep separated power control for PRACH transmission by compensating the difference of powerRampingStep and powerRampingStep_SBFD when RO type switching occurs.

- POWER_OFFSET_SBFD = (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREAMBLE_POWER_RAMPING_STEP_ SBFD – PREAMBLE_POWER_RAMPING_STEP).

PREAMBLE_POWER_RAMPING_COUNTER is UE variable for the number of PRACH transmission counter, PREAMBLE_POWER_RAMPING_STEP_SBFD is a value configured in the RRC parameter powerRampingStep_SBFD if determined to be introduced and PREAMBLE_POWER_RAMPING_STEP is a value configured in the RRC parameter powerRampingStep. UE has less complexity to switch power from addition-RO to legacy-RO by the power offset as below.

- PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_SBFD

PREAMBLE_RECEIVED_TARGET_POWER is the transmit power of PRACH in legacy RO,  preambleReceivedTargetPower is RRC parameter configured for legacy-RO,  PREAMBLE_POWER_RAMPING_COUNTER is UE variable which is suspended when the switching has occurred, and PREAMBLE_POWER_RAMPING_STEP is a value configured for power ramping in legacy-RO.

For RACH configuration Option 2, it is not agreed to use separated power control parameter yet. However, because it is decided to use two separated RACH configurations for RACH configuration Option 2 in RAN1#116, it is natural to have additional rach-ConfigCommon, so that gNB independently configures to allow SBFD aware UE to use separated transmit power operation based on two separated power control parameters. In detail, if SBFD aware UE uses legacy-RO for PRACH transmission, it uses the parameters configured as preambleReceivedTargetPower, powerRampingStep and preambleTransMax in rach-ConfigCommon. If SBFD aware UE uses additional-RO for PRACH transmission, it uses the parameter configured as preambleReceivedTargetPower, powerRampingStep and preambleTransMax in RACH-ConfigCommonSBFD-r19. In this case, RACH-ConfigCommonSBFD-r19is newly defined for additional-RO in Rel.19.

Proposal 18: For PRACH power control of RACH configuration Option 1, use additional power control parameter with corresponding existing parameter.

- The additional power control parameter corresponding existing parameters are at least preambleReceivedTargetPower_SBFD, powerRampingStep_SBFD and preambleTransMax_SBFD in rach-ConfigGeneric.

Proposal 19: For PRACH power control of RACH configuration Option 1 when RO type switching has occurred,

- Suspend the power ramping counter.

- UE could calculate POWER_OFFSET_SBFD value.

  - POWER_OFFSET_SBFD = (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREAMBLE_POWER_RAMPING_STEP_ SBFD – PREAMBLE_POWER_RAMPING_STEP).

Observation 12: In PRACH power control of RACH configuration Option 2, it is decided to use two separated RACH configurations for legacy-ROs and additional-ROs in RAN1#116 so that it is natural to have in any form of additional rach-ConfigCommon.

Proposal 20: For PRACH power control of RACH configuration Option 2, use separate power control parameters configured with RACH-ConfigCommonSBFD-r19 which is newly defined for additional-RO in Rel.19.

# Msg2/Msg3/Msg4 related transmission/reception in SBFD symbols

The agreement was made to take into account Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols in RAN1 #116 and RAN1 #118bis.

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1. Agreement For SBFD aware UEs, for Msg3 PUSCH transmission, reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop, with the update that the frequency offset for 2nd hop is based on the size of UL usable PRBs. - FFS the case when the value of NUL,hop= 11 - FFS the definition of UL usable PRBs for Msg3 PUSCH transmission Conclusion There is no consensus in RAN1 to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs. It’s up to RAN2 to decide whether to support preamble partitioning on legacy-ROs for early indication of SBFD-aware UEs.

Considering the ongoing discussion on general Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols in AI 9.3.1, it seems reasonable to wait and reuse the agreements to be made in non-RA SBFD operation as much as possible to prioritize RA specific items such as RO configuration, 4-step/2-step RACH operation, RO validity, SSB-RO mapping, PRACH repetition for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE modes and so on. During RAN1 #118bis, the very first agreement was virtually made to enhance Msg3 PUSCH transmission with frequency offset for 2nd hop based on the size of UL usable PRBs. Since many RA specific items are getting settled down, we may need to get down to the discussion on RA specific enhancements on Msg2/Msg3/Msg4 related operation for SBFD aware UE to provide benefits after PRACH transmission since early detection of SBFD capability in SBFD aware UE by gNB could provide more opportunities to transmit UL signal/channel at UL subband in SBFD symbol during the rest SBFD random access procedure. Without a new feature, gNB would detect early whether UE is capable of SBFD operation by successfully receiving the PRACH in additional ROs configured by either RACH configuration Option 1 or additional RACH configuration in RACH configuration Option 2. Msg 3 PUSCH transmission could provide one more chance for early detection of SBFD capability in SBFD aware UE such that gNB could transmit UL signal/channel at UL subband in SBFD symbol for remaining SBFD random access procedure followed by Msg 3 PUSCH (re)transmission.

For Msg3 PUSCH on non-SBFD symbols, a UE uses the initial UL BWP size of NBWPsize RBs when it processes the frequency domain resource assignment field and performs a PUSCH transmission with frequency hopping scheduled by RAR UL grant or of Msg3 PUSCH retransmission for the second hop. Given the agreement made to allow frequency offset for the second hop based on UL usable PRBs in Table 8.3-1 as below, whether/how to introduce a new frequency offset with  the value of NUL,hop= 11 remains FFS. Given relatively narrow size in UL usable PRB, it seems reasonable to keep the current frequency offset values for the second hop unless there is evident measurement to show that extra frequency offset would achieve frequency diversity gain enough. If not, the value of NUL,hop= 11 would be reserved for future use.

Observation 13: Considering relatively narrow size in UL usable PRB, it seems reasonable to keep the current frequency offset values for the second hop unless there is evident measurement to show that extra frequency offset would achieve frequency diversity gain enough. If not, the value of NUL,hop = 11 would be reserved for future use.

Proposal 21: FFS the case when the value of NUL,hop= 11 is not necessary and keep the value as Reserved as it is.

There are some concerns on the potential restrictions on initial BWP in SBFD random access operation based on identical center frequency of initial UL BWP and initial DL BWP. The initial BWP includes both PRBs for SSB/CORESET0 in the DL subband and PRBs for PRACH/Msg3 PUSCH in the UL subband to support the initial access for SBFD UE in IDLE/INACTIVE mode in SBFD symbols where the UL subband and the DL subband are not overlapped with each other. Without specification impact having separate initial BWP for SBFD aware UEs for initial access, larger initial UL BWP could be configured to ensure to cover both the UL subband and the DL subband not to be overlapped.

Proposal 22: RAN1 not to consider supporting a separate initial BWP for SBFD aware UEs.

For Msg4 HARQ-ACK PUCCH on non-SBFD symbols, a UE utilizes the initial UL BWP size of NBWPsize RBs when it determines the lowest PRB indices of the PUCCH transmission in the first hop and second hop from the PUCCH resource set provided by pucch-ResourceCommon before dedicated PUCCH resource configuration. We should consider enhancements on the frequency hopping behaviors for Msg4 HARQ-ACK PUCCH to be configured within UL usable PRB in SBFD symbol.

9.2.1 in TS38.213 If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the lowest PRB index of the PUCCH transmission in the first hop as RBBWPoffset⋅NRB+rPUCCHNCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as NBWPsize-RBBWPoffset∙NRB-1+rPUCCHNCS∙NRB, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the lowest PRB index of the PUCCH transmission in the first hop as NBWPsize-RBBWPoffset∙NRB-1+rPUCCH-8NCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as RBBWPoffset∙NRB+rPUCCH-8NCS∙NRB - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as … Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration Index; PUCCH format; First symbol; Number of symbols; PRB offset RBBWPoffset; Set of initial CS indexes. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 10; 4; 4; {0, 3, 6, 9}. 7; 1; 4; 10; 0; {0, 6}. 8; 1; 4; 10; 0; {0, 3, 6, 9}. 9; 1; 4; 10; 2; {0, 3, 6, 9}. 10; 1; 4; 10; 4; {0, 3, 6, 9}. 11; 1; 0; 14; 0; {0, 6}. 12; 1; 0; 14; 0; {0, 3, 6, 9}. 13; 1; 0; 14; 2; {0, 3, 6, 9}. 14; 1; 0; 14; 4; {0, 3, 6, 9}. 15; 1; 0; 14; NBWPsize4; {0, 3, 6, 9}.

For example, as one of enhancements for SBFD aware UE transmitting Msg4 HARQ-ACK PUCCH, whether to support Msg4 HARQ-ACK PUCCH in the first hop and second hop from the PUCCH resource set should be considered. Once the agreement is made for frequency hopping to support in SBFD symbols, the discussion on how to determine the lowest PRB index of the PUCCH transmission in the first hop and/or the lowest PRB index of the PUCCH transmission in the second hop should be followed since reusing the NBWPsize RBs, initial UL BWP size, in Table 9.2.1-1 may result in Msg4 HARQ-ACK PUCCH to be overlapped out of UL SBFD subband as shown in Figure 5 where for operation in FR2-2, pucch-ResourceCommon provide a number of NRB RBs for the PUCCH resource set; otherwise NRB=1. If not supported, the discussion on whether/how to disable and which PUCCH resource from lower edge or higher edge to use should be followed.

Figure 5

Proposal 23: RAN1 to consider enhancement on whether/how to determine the lowest PRB index of the PUCCH transmission in the first hop and/or the lowest PRB index of the PUCCH transmission in the second hop with the update that the first hop and the second hop is based on the size of UL usable PRBs.

- Msg4 HARQ-ACK PUCCH is configured within UL usable PRB in SBFD symbol

Currently in Table 9.2.1-1(PUCCH resource sets before dedicated PUCCH resource configuration), depending on the PUCCH format, the values of the First symbol and Number of symbols are specified. Considering a SBFD slot consisting of non-SBFD symbols and SBFD symbols if allowed for PUCCH resource sets before dedicated PUCCH resource configuration, there might be some restrictions to reuse the PUCCH resource sets on SBFD symbol. For example, given a slot consisting of 12 SBFD symbols and 2 non-SBFD symbol (if no gap between non-SBFD symbols and SBFD symbols), current the value in First symbol may not provide configuration flexibility for PUCCH resource sets in SBFD symbols. PUCCH format 0 could be transmitted only on non-SBFD symbol starting 12th first symbol with 2 symbols. The value in First symbol could be enhanced to allow SBFD symbols and/or non-SBFD symbols for the PUCCH resource sets before dedicated PUCCH resource configuration.

Proposal 24: RAN1 to consider enhancement on Table 9.2.1-1 (PUCCH resource sets before dedicated PUCCH resource configuration) to secure configuration flexibility for in SBFD symbols.

# Summary

In this contribution, we discussed on the subband non-overlapping full duplex.

Observation 1: Referring to the agreement made in RAN2 on cell-specific SBFD time/frequency configuration provided by SIB1, semi-static configuration to signal frequency and time location of the SBFD UL subband are broadcasted via SIB1 for SBFD aware UE  in RRC_IDLE/INACTIVE.

- RAN2 Agreement in [R2-2407572]: Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

Observation 2: For network configuration in the agreement on a RO across SBFD symbols and non-SBFD symbols in the same slot or across slots, consider two alternatives to support.

- Alt. 1: Network explicitly indicates whether such RO is valid.

- Alt. 2: Network configuration is not necessarily meant to be a new parameter to show whether a RO across SBFD symbols and non-SBFD symbol is valid or not. For example, such ROs are regarded as valid implicitly once configured by PRACH configuration index in additional RACH configuration.

Observation 3: During the discussion in RAN1 #118, it was mentioned, but not formally agreed, for RACH configuration Option 1, a RO across SBFD symbols and non-SBFD symbols is identified and determined as valid since the legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs.

Observation 4: For PRACH mask indication for CFRA triggered by PDCCH order for SBFD aware UE, RAN1 to discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

- The ambiguity for SBFD aware UE to figure out the first valid mapping cycle when there are the first valid mapping cycle for legacy ROs and the first valid mapping cycle for additional ROs, respectively.

- The restriction release on only first valid mapping cycle.

Observation 5: Given the conclusion made in RAN1 #117 and fundamental purpose to introduce RACH configuration Option 1, the network should ensure that configured ROs are within the UL usable PRBs in SBFD symbols. Allowing accumulative features or specific description could increase both the UE implementation complexity and specification efforts

Observation 6: While RAN2 is under discussion to decide conditions and relevant parameters for RO type selection and switching, RAN1 could still work on those conditions and mechanisms with potential parameters to get aligned with RAN2’s decision.

Observation 7: During PRACH transmission re-attempt, measuring RSRP for rsrp-threshold comparison in every single RACH transmission will increase UE complexity.

Observation 8: Since the discussion in Rel.18 CE CR is about whether rach-ConfigCommon is allowed to configure one or more time periods depending on UE’s capacities and featureCombinationPreambleList-r17 configurations, there seem not to wait until corresponding clarification currently under discussion is completed.

Observation 9: With this Working Assumption to be confirmed, more than one time period will be configured with one rach-ConfigCommon.

Observation 10: Whether/how to operate RO group type selection should be discussed for PRACH repetition in SBFD random access for SBFD aware UE in RACH configuration Option 1 and RACH configuration Option 2.

Observation 11: When switching into other type of RO group, further discussion may need whether SBFD aware UE starts PRACH repetition with the certain PRACH repetition number if Alt.2 is determined.

Observation 12: In PRACH power control of RACH configuration Option 2, it is decided to use two separated RACH configurations for legacy-ROs and additional-ROs in RAN1#116 so that it is natural to have in any form of additional rach-ConfigCommon.

Observation 13: Considering relatively narrow size in UL usable PRB, it seems reasonable to keep the current frequency offset values for the second hop unless there is evident measurement to show that extra frequency offset would achieve frequency diversity gain enough. If not, the value of NUL,hop = 11 would be reserved for future use.

Proposal 1: RAN1 to allow 2-step RACH to be supported in SBFD symbols for SBFD aware UEs to fit into the latency limited conditions.

Proposal 2: RAN1 to consider a network configuration agreed for RACH configuration Option 2 is applicable to RACH configuration Option 1.

Proposal 3: To handle the ambiguity on the first valid mapping cycles, consider the following alternatives to indicate/determine additional-RO only, legacy-RO only, or both at least for RACH configuration Option 1.

- Alt 1: the network explicitly indicates whether CFRA operation is based on legacy ROs or additional ROs.

- Alt 2: SBFD aware UE is allowed to select RO type for CFRA operation in SBFD random access based on certain specified/configured conditions.

- Alt 3: SBFD aware UE selects the first available mapping cycle based on the prioritization for additional ROs.

Proposal 4: RAN1 to support Alt 2-1 in RACH configuration Option 1.

- The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

Proposal 5: With two separate RACH configurations in RACH configuration Option 2, same or different preamble formats by each PRACH configuration index can be configured by gNB. (i.e., one for legacy RACH configuration, one for additional RACH configuration).

Proposal 6: The parameter msg1-FrequencyStart in additional RACH configuration of RACH configuration Option 2 indicates the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

Proposal 7: For initial PRACH transmission of RACH configuration Option 1 and Option 2, at least threshold should be used for condition of RO type selection.

Proposal 8: For both of RACH configuration Option 1 and 2, for certain specified/configured conditions/prioritizations of initial PRACH transmission, a new parameter corresponding to rsrp-threshold (e.g. rsrp-thresholdSBFD) should be introduced.

Proposal 9: For PRACH transmission re-attempt of RACH configuration Option 1 and Option 2, at least transmission number of PRACH should be used for condition of RO type switching.

Proposal 10: For PRACH transmission re-attempt of RACH configuration Option 1 and Option 2, for a certain number of times to switch into other type of ROs, either a new parameter corresponding to the specific/configured number of re-attempts or existing parameter (e.g. maxtrans_SBFD or preambleTransMax) should be considered.

- A new parameter corresponding to the specific/configured number of re-attempts is preferred.

Proposal 11: For RACH configuration Option 2, support to introduce a new parameter to modify the subframe number or the slot number configured by existing random access configurations tables and then to allow more ROs.

Proposal 12: For dynamic or semi-static DL vs. valid RO,

- Reuse the existing collision handling principles for HD-FDD RedCap UE, i.e., leave to UE implementation.

Proposal 13: For RACH configuration Option 2, ensuring that network can handle the concern on the legacy ROs configured by legacy RACH configuration overlapped with the additional ROs configured by additional RACH configuration in time domain, further enhancement for FFS is not required in the agreement.

- For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

Proposal 14: The Working Assumption should be confirmed

- Working Assumption

For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs.

- the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

Proposal 15: For SBFD aware UE, consider the following options for initial RO group type selection:

- Alt. 1: Select one RO group type between legacy RO group and additional RO group based on certain specified/configured conditions/prioritizations.

    - Same concept in RO type selection for initial PRACH transmission.

- Alt. 2: It’s up to UE implementation to select one RO group type between legacy RO group and additional RO group.

Proposal 16: For SBFD aware UE, consider separate msg1-RepetitionNum and/or rsrp-ThresholdMsg1-RepetitionNumN (e.g., N could be 2, 4, and 8) for legacy RO group and additional RO group.

- Values in PRACH repetition number and corresponding RSRP threshold could be differently configured between legacy RO group and additional RO group.

Proposal 17: For SBFD aware UE, consider the following options for RO group type switching if the earlier RO group fails:

- Alt. 1: Always use the RO group of the same type (i.e., legacy RO group or additional RO group) with increased PRACH repetition number as the earlier RO group for the rest of the random access procedure.

- Alt. 2: First use RO group of the same type as the earlier PRACH transmission for up to a certain PRACH repetition number, then use RO group of the other type for the rest of the random access procedure.

    - Same concept in RO type switching for RACH re-attempt

- Alt. 3: Switch into RO group of the other type if certain conditions are met for the rest of the random access procedure.

Proposal 18: For PRACH power control of RACH configuration Option 1, use additional power control parameter with corresponding existing parameter.

- The additional power control parameter corresponding existing parameters are at least preambleReceivedTargetPower_SBFD, powerRampingStep_SBFD and preambleTransMax_SBFD in rach-ConfigGeneric.

Proposal 19: For PRACH power control of RACH configuration Option 1 when RO type switching has occurred,

- Suspend the power ramping counter.

- UE could calculate POWER_OFFSET_SBFD value.

  - POWER_OFFSET_SBFD = (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREAMBLE_POWER_RAMPING_STEP_ SBFD – PREAMBLE_POWER_RAMPING_STEP).

Proposal 20: For PRACH power control of RACH configuration Option 2, use separate power control parameters configured with RACH-ConfigCommonSBFD-r19 which is newly defined for additional-RO in Rel.19.

Proposal 21: FFS the case when the value of NUL,hop= 11 is not necessary and keep the value as Reserved as it is.

Proposal 22: RAN1 not to consider supporting a separate initial BWP for SBFD aware UEs.

Proposal 23: RAN1 to consider enhancement on whether/how to determine the lowest PRB index of the PUCCH transmission in the first hop and/or the lowest PRB index of the PUCCH transmission in the second hop with the update that the first hop and the second hop is based on the size of UL usable PRBs.

- Msg4 HARQ-ACK PUCCH is configured within UL usable PRB in SBFD symbol

Proposal 24: RAN1 to consider enhancement on Table 9.2.1-1 (PUCCH resource sets before dedicated PUCCH resource configuration) to secure configuration flexibility for in SBFD symbols.

# Reference

[1] Huawei, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RP-241614, RAN#104, June 17-20, 2024