3GPP TSG RAN WG1 #119	                                         	 R1-2409545

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.5.2

Source:	Panasonic

Title:	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion/Decision

# Introduction

In the previous meetings, the agreements listed in appendix on on-demand SIB1 were achieved [1] [2][3][4]. In RAN #105 meeting, it was agreed to specify procedures and signaling methods for Case 2 on-demand SIB1 operation in IDLE/INACTIVE mode [5].

In this contribution, we further share our views on procedures and signaling design on on-demand SIB1 for UEs in idle/inactive state for Case 2.

# On UL WUS configuration and related procedure design

UL-WUS configuration

In the previous meeting, below agreement was achieved for NES cell configuration.

Agreement For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

On the other hand, even from a certain NES cell perspective, it can also indicate neighboring cell’s UL WUS configuration. This is also supported by RAN2 #127bis meeting agreement as below.

A NES cell can include neighbouring NES cell’s WUS configuration.

Therefore, in practice, multiple NES cells would operate simultaneously and the NES cell configuration and UL-WUS configuration should support to include a list of NES-CellId and associated SIB1-RequestConfig.

Proposal 1: In the UL WUS configuration, a list of NES-CellId should be supported, with each NES cell associated with SIB1-RequestConfig.

NES cell state

Regarding the possible NES Cell state, our observation from the discussion so far is that:

- SSB-only state: NES cell only transmits SSB for UE to identify but does not transmit OD-SIB1 and is not active of transmitting paging and receiving PRACH.

- Active but SIB1-less state: After being triggered to transmit OD-SIB1, the NES cell transmits the OD-SIB1 for a certain duration. After this duration, no SIB1 is transmitted but NES cell capable UEs already acquire the configurations within the SIB1 and camp in this NES cell. Thus, in our assumption, paging and PRACH related function should be active but only appliable to UEs already camping in this cell. For the new UEs, as no SIB1 is being transmitted, these UEs need to go to Cell A and acquire the NES cell related configuration.

- Fully active state: After being triggered to transmit OD-SIB1, the NES cell is supposed to transmit the OD-SIB1 periodically or for a certain duration, where it acts like a normal cell for UE to camp and access. This assumes full activity of paging and PRACH in this cell within the duration. We think it is beneficial to support that OD-SIB1 is transmitted periodically without being limited to a duration, as the NES cell state can be clearer and has no major energy loss because all other common channels are also active.

Hence, for clearer UE behaviour design, some more discussion is needed to clarify which of the above NES cell states should be defined and which is the UE behaviour in each state and during state switching.

Proposal 2: Below possible NES cell states should be further discussed and clarified:

- SSB-only state

- Active but SIB1-less state

- Fully active state

Observation 1: After OD-SIB1 is triggered and transmitted, it is beneficial to support that the OD-SIB1 can be broadcasted periodically without being limited to a time window.

Proposal 3: After OD-SIB1 is transmitted for a duration and then stops being transmitted, UE assumption should be further clarified on whether and how long the paging and PRACH configurations are valid.

# On-demand SIB1 monitoring occasions

Regarding type 0 PDCCH monitoring occasions for on-demand SIB1, below agreements were achieved in previous meetings.

Agreement from RAN1#117 meeting At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window - FFS: How the search space zero configuration is provided (e.g. from searchSpaceZero in MIB or from a new search space that is indicated by UL-WUS configuration) - FFS: Details of the time window, including at least the starting time and duration - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS Agreement from RAN1#118bis meeting For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2. Agreement from RAN1#118 meeting At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement from RAN1#118 meeting At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

As agreed, UE would acquire the searchSpaceZero and controlResourceSetZero from the UL-WUS configuration. Then how the location of on demand SIB1 type 0 PDCCH is determined can have below possibilities:

- Alt 1: Kssb is indicated by UL-WUS configuration and used as the same manner as legacy approach to decide the frequency location of the controlResourceSetZero

- Alt 2: A more flexible reference point like reference DL BWP is indicated by UL-WUS configuration to determine the frequence offset. More flexible searchSpaceZero and controlResourceSetZero can also be considered without being limited by the legacy 16 patterns.

Our preference is Alt 2 due to better network configuration flexibility, which is beneficial for NES gain.

Proposal 4: More flexible frequency offset, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided in the UL WUS configuration, without being limited by legacy approach and patterns.

Figure.1 Illustration of the candidate reference time points to determine the OD-SIB1 starting time

For reference time point, as shown in Figure.1, 4 points (3 fixed points and 1 varying point) can potentially be used, as per the agreement and discussion from previous meetings. Our views are:

- We think fixed point is better than varying point. It allows UE to prepare the OD-SIB1 reception with more certain expectations and even allows blind detection in case RAR is not received. It is also beneficial for gNB operation to accommodate multiple UEs requesting OD-SIB1.

- As all fixed points A/B/C are by configuration, functionally they have not so much difference on the complexity from both gNB and UE implementation.

- Therefore, to reduce the latency of OD-SIB1 transmission, we prefer fixed point A of UL-WUS transmission time due to merit on the latency but can be shorter than fixed point B of start of RAR window.

Proposal 5: On reference time point to determine the window starting time of type 0 PDCCH monitoring occasions for on demand SIB1, it is defined based on the UL-WUS transmission time. Our secondary preference is that it is defined based on the start of RAR window.

For the starting time and duration of the time window, indication in RAR of the UL-WUS transmission (Option 1) may have clearer UE behaviour that UE will not monitor SIB1 in type 0 CSS until it receives confirmation and indication from RAR. This may avoid misalignment between gNB and UE in case UL-WUS is not detected by NES cell. Option 2 (starting time and duration indicated in the UL-WUS configuration) can be simpler design, which allows UE even try to detect SIB1 before sending UL-WUS and can potentially save efforts of sending UL-WUS, if the SIB1 has already been triggered. Therefore, we slightly prefer Option 1 but can accept Option 2.

Proposal 6: On the starting time and duration of the time window of type 0 PDCCH monitoring occasions for on-demand SIB1, we prefer Option 1 but can also accept Option 2:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: In the UL WUS configuration, a list of NES-CellId should be supported, with each NES cell associated with SIB1-RequestConfig.

Proposal 2: Below possible NES cell states should be further discussed and clarified:

- SSB-only state

- Active but SIB1-less state

- Fully active state

Observation 1: After OD-SIB1 is triggered and transmitted, it is beneficial to support that the OD-SIB1 can be broadcasted periodically without being limited to a time window.

Proposal 3: After OD-SIB1 is transmitted for a duration and then stops being transmitted, UE assumption should be further clarified on whether and how long the paging and PRACH configurations are valid.

Proposal 4: More flexible frequency offset, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided in the UL WUS configuration, without being limited by legacy approach and patterns.

Proposal 5: On reference time point to determine the window starting time of type 0 PDCCH monitoring occasions for on demand SIB1, it is defined based on the UL-WUS transmission time. Our secondary preference is that it is defined based on the start of RAR window.

Proposal 6: On the starting time and duration of the time window of type 0 PDCCH monitoring occasions for on-demand SIB1, we prefer Option 1 but can also accept Option 2:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

# Reference

[1] 3GPP RAN1#116-bis meeting Chairman’s notes

[2] 3GPP RAN1#117 meeting Chairman’s notes

[3] 3GPP RAN1#118 meeting Chairman’s notes

[4] 3GPP RAN1#118 meeting Chairman’s notes

[5] RP-242354, Revised WID: Enhancements of network energy savings for NR

# Appendix:

Agreements from RAN1#116bis meeting

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

Agreement

RAN1 to further study the following UE operation scenarios in the UL WUS design:

- Scenario 1: UE requests SIB1 to camp on NES cell

- Scenario 2: UE request SIB1 to perform random access procedure to make RRC connection to NES cell

Agreement

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

- Option 1: By WUS configuration

- Option 2: By PBCH payload of NES cell

Agreement

Companies to report at least the following key settings used in the evaluation/simulation of achievable NES gain with on-demand SIB1 in idle/inactive mode

- Setting A: SIB1 period (20ms/40ms/160ms)

- Setting B1: Cell load (Empty/low/medium)

- Setting B2: Traffic model

- Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

- Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

- Setting E: PRACH configurations (including PRACH configuration index in 38.211 Table 6.3.3.2-3) for WUS and initial/random access

- Setting F: Cat1/Cat2 BS

- Setting G: Number of SSB beams

- Setting H: NES gain/loss on Cell A

- Setting I: On-demand SIB1 transmission rate (how often UE requests on-demand SIB1)

Agreement

For further study of the NES gain/loss evaluation assumption on Cell A with on-demand SIB1 on NES cell for idle/inactive mode UE,

- Assume the following for network energy evaluation of Cell A in FR1:

  - Company to report among empty/low/medium cell load as defined in 38.864

  - Same Cat BS as the Non-NES cell

  - 30kHz SCS, DDDSU TDD pattern

    - Same SSB period as the Non-NES cell and company to report SIB1 period

  - Same number of SSBs in a SSB burst as the Non-NES cell with SSB pattern case C

  - 20ms PRACH configuration periodicity for WUS and/or initial access RACH and company to report RACH configuration index in 38.211 Table 6.3.3.2-3

  - Same SSB/CORESET0 multiplexing pattern and same SIB1 PDSCH time domain resource allocation as the Non-NES cell

  - Same traffic model as the Non-NES cell

  - Companies to report the assumption of WUS configuration provision or UL WUS monitoring or on-demand SIB1 transmission on Cell A if Case 2 (Option 1+B+X) or Case 3 (Option 2+B+Y) is considered

Agreement

For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1

- FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS

- FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

Agreement

Conditions for triggering UL WUS transmission is up to RAN2. Any related work in RAN1 to be triggered by RAN2 LS. Send an LS to RAN2. Final LS in R1-2403779.

In RAN1 #116-bis, the following are discussed during online session of AI 9.5.2, but not agreed yet. It may serve as a starting point for further discussions in future RAN1 meetings:

Companies to consider the following for future meetings

- Option 1: SIB1 monitoring occasions within a time window

  - FFS: The starting time and duration of the time window

  - FFS: Interval between two SIB1 monitoring occasions in the time window

  - FFS: How gNB informs UE the details related to the time window

- Option 2: Periodic SIB1 monitoring occasions until gNB turns off the SIB1 transmission

  - FFS: The staring time of the SIB1 monitoring occasions

  - FFS: How gNB informs UE the SIB1 transmission is turned off

  - FFS: How gNB informs the UE the details related to periodicity

- Other options are not precluded

- FFS: Further details on SIB1 monitoring occasions

Agreements from RAN1#117 meeting

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

- Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

  - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

  - FFS: Details of the time window, including at least the starting time and duration

  - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

Agreement

From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it.

At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Agreement

For further study of on-demand SIB1 in idle/inactive mode, on the spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB, and UL WUS, as UL WUS is using dedicated PRACH resource, it is assumed that spatial relationships among PDCCH/PDSCH of on-demand SIB1, SSB and UL WUS can follow legacy mechanism.

Agreement

For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration.

Table I.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | Duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 39.37% - 47.69% from the following 7 sources

    - R1-2404463, R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2405162) reports 25% NES gain with PRACH configuration index = 152

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 24.6% with one beam of SIB1 PDSCH transmitted in each slot

    - 17.4% with two beams of SIB1 PDSCH transmitted in each slot

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 22.1% - 42.6% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 30.43% - 45.79% from the following 4 sources

    - R1-2404463, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152

    - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam

    - 23% with Case 2 (Options 1+B+X)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 21.79% - 33.11% from the following 2 sources

    - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 20.5% - 39.34% from the following 8 sources

    - R1-2404463, R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404507, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 11.9% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 19.31 – 19.39% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 29.19% - 36.87% from the following 3 sources

    - R1-2404463, R1-2403942, R1-2404859

  - One source (R1-2404561) reports 15.87%~20.18% NES gain with PRACH configuration index = 148

- For Cat 1 BS, 4/8 beams, 4% < on-demand SIB1 transmission rate < 24%, the NES gain is 17.98%~24.48% from one source (R1-2404408).

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.21 – 10.6% from the following 10 sources

  - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405162, R1-2404122, R1-2404507, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.7% - 6.23% from the following 5 sources

    - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758

  - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.75% - 8.48% from the following 6 sources

  - R1-2405106, R1-2405162, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.01% - 5.8% from the following 3 sources

  - R1-2405106, R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.24% - 7.76% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.96% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.5% - 7.44% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 30.56% - 41.8% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 13.9% with one beam of SIB1 PDSCH transmitted in each slot

    - 7.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16% - 36.2% from the following 5 sources

    - R1-2405247, R1-2405341, R1-2404033, R1-2405363, R1-2404625

  - One source (R1-2404033) reports 10% NES gain with FDMed SSB and PDSCH of SIB1

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 5.2% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.9% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 22.44% - 37.64% from the following 3 sources

    - R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 13.17% - 36.2% from the following 3 sources

    - R1-2404625, R1-2405247, R1-2405363

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 16.8% - 28.77% from the following 6 sources

    - R1-2403942, R1-2405341, R1-2404224, R1-2404859, R1-2404561, R1-2405595

  - One source (R1-2404224) reports 10.5% NES gain with medium cell load on Cell A

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 12.71% – 12.88% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 14.43% - 27.47% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.2 – 8.3% from the following 7 sources

    - R1-2405595, R1-2403979, R1-2405341, R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.2% - 5.46% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404625

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.87% - 7.38% from the following 4 sources

  - R1-2404859, R1-2405363, R1-2404625, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.97% - 4.48% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 3.04% - 5.57% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 1.79% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.12% - 5.26% from the following 3 sources

  - R1-2404561, R1-2403942, R1-2404859

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.88% - 14.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 4.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 2.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 4.92% - 8.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 1.5% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.5% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 11.5% - 12.88% from one source (R1-2405363)

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 6%~7.53% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 8.08% - 15.47% from the following 4 sources

  - R1-2405341, R1-2404224, R1-2404561, R1-2403942

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.09% – 4.19% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 10.78% - 15.3% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.3 – 2.44% from the following 4 sources

  - R1-2403979, R1-2405341, R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.1% - 1.3% from the following 3 sources

  - R1-2403979, R1-2405341, R1-2405363

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.24% - 2.26% from the following 2 sources

  - R1-2405363, R1-2404561

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 0.67% - 1.6% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.95% - 4.13% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 0.48% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 1.44% - 4.02% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 24.51% - 31.3% from the following 6 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2404507, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 16.3% with one beam of SIB1 PDSCH transmitted in each slot

    - 10.6% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 12.4% - 35.5% from the following 5 sources

    - R1-2404408, R1-2405341, R1-2404758, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 10.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 6.1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 19.44% - 28.81% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 12.87% - 20.09% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 11.77% - 25.46% from the following 5 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404507, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.69 – 10.74% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 16.46% - 24.24% from the following 2 sources

    - R1-2403942, R1-2404859

  - One source (R1-2404561) reports 8.92%~10.37% NES gain with PRACH configuration index = 148

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, low load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 18.38% - 26.4% from the following 5 sources

    - R1-2405595, R1-2405341, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 8.1% with one beam of SIB1 PDSCH transmitted in each slot

    - 4.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 9.84% - 17.41% from the following 3 sources

    - R1-2405341, R1-2405363, R1-2404625

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.8% with one beam of SIB1 PDSCH transmitted in each slot

    - 1% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 13.36% - 23.68% from the following 3 sources

  - R1-2404859, R1-2405363, R1-2404625

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.27% - 15.59% from the following 2 sources

  - R1-2405363, R1-2404625

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 10.48% - 17.72% from the following 4 sources

  - R1-2403942, R1-2405341, R1-2404859, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 7.01% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 7.7% - 17.05% from the following 3 sources

  - R1-2403942, R1-2404859, R1-2404561

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, medium load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 5.07% - 7.8% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 2.4% with one beam of SIB1 PDSCH transmitted in each slot

    - 1.2% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 2.46% - 4.4% from the following 2 sources

    - R1-2405341, R1-2405363

  - One source (R1-2403979) reports the following NES gain with PRACH configuration index = 98

    - 0.7% with one beam of SIB1 PDSCH transmitted in each slot

    - 0.3% with two beams of SIB1 PDSCH transmitted in each slot

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.62~7.01% from one source (R1-2405363)

- For Cat 1 BS, 4 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 2.77% - 3.85% from one source (R1-2405363)

- For Cat 2 BS, 8 beams, 0% on-demand SIB1 transmission rate, the NES gain is 4.08%~10.1% from the following 3 sources

  - R1-2403942, R1-2405341, R1-2404561

- For Cat 2 BS, 4 beams, 0% on-demand SIB1 transmission rate, the NES gain is 2.02%~2.09% from one source (R1-2405341)

- For Cat 2 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%, the NES gain is 5.61% - 9.94% from the following 2 sources

  - R1-2404561, R1-2403942

Agreement

For the evaluation of NES loss on cell A, the following is observed

  - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading):

    - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2)

  - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource.

    - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load

    - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load

  - One source (R1-2404122) reports

    - 2.95%(medium load)~15.49%(empty load) NES loss with UL WUS configuration transmitted in separated SIB on Cell A

      - The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 1.28%(medium load)~3.62%(empty load) NES loss with UL WUS configuration transmitted in legacy SIB on Cell A

Conclusion

For further study of on-demand SIB1 in idle/inactive mode, enabling or disabling specific operations (e.g. paging, RACH receiving, OSI request …) of the NES cell with on-demand SIB1 is up to RAN2.

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 1 BS, empty load, the NES gain is 37.77% from one source (R1-2404463)

- For Cat 2 BS, empty load,

  - the NES gain is 8.9% - 28.71% from the following 3 sources

    - R1-2404224, R1-2404561, R1-2404463

  - The NES gain is 1.47% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

- For Cat 2 BS, low load,

  - the NES gain is 7.9% - 14.65% from the following 2 sources

    - R1-2404224, R1-2404561

  - The NES gain is 0.42% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

- For Cat 2 BS, medium load,

  - the NES gain is 6.2% - 10.4% from the following 2 sources

    - R1-2404224, R1-2404561

  - The NES gain is 0.12% from one source (R1-2404561) with 75% on-demand SIB1 transmission rate

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 40ms SIB1 period (Case D), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 2 BS, empty/low/medium load, the NES gain is 3.77% - 5.97% from one source (R1-2404561)

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, 8 beams, on-demand SIB1 transmission rate > 30%

- For Cat 1/Cat 2 BS, empty/low/medium load, the NES gain is 0.31%~3.33% from the following 2 sources

  - R1-2404561, R1-2404122

Agreement

For the evaluation of the energy consumption to transmit WUS configuration on NES Cell:

- One source (R1-2405595) reports that for broadcasting WUS configuration using DCI occupying 2 symbols per SSB beam) on NES cell every 80 ms on CORESET#0 (48 RBs):

  - For BS category 1, empty/low load, the energy consumption is increased by 6.26%~8.51% over SSB transmission only. The energy saving gain with Case 1 is 40.52% / 32.41% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).

  - For BS category 2, empty/low load, the energy consumption is increased by 1.55%~2.11% over SSB transmission only. The energy saving gain with Case 1 is 28.25% / 23.03% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).

For future meetings:

Companies are encouraged to check section 3 of R1-2405632 for discussion on RAN1 / RAN2 workscope.

Agreements from RAN1#118 meeting

Agreement

For further work on on-demand SIB1 in idle/inactive mode, as a baseline, it is assumed that the transmit power control of UL WUS transmission based on PRACH is applied in the same manner as the legacy PRACH transmission.

- FFS: Potential optimization of the power ramp-up procedure

Agreement

For further work on on-demand SIB1 in idle/inactive mode related to dedicated PRACH resource usage, RAN1 to study the following options:

- Option 1 (shared RO): The dedicated WUS resource shares the same PRACH resource pool with PRACH resource for other usages.

- Option 2 (separated RO): The dedicated WUS resource uses an independent RACH resource pool with PRACH resource for other usages.

Agreement

RAN1 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options:

- Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission

  - FFS: Definition of RAR reception time

- Option 2: The reference time point is defined based on the UL-WUS transmission time

- Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for Case 3

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

Agreement

For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options:

- Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell

- Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration

- Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS.

Combination of multiple options is not precluded.

Agreements from RAN1#118bis meeting

Conclusion

No further optimization in RAN1 on power control and power ramp-up procedure for UL WUS in R19.

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration:

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

Agreement

For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration.

| Purpose | Parameters |
| --- | --- |
| WUS transmission | frequencyBandList |
|  | absoluteFrequencyPointA |
|  | offsetToCarrier |
|  | p-Max |
|  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

Note:

- In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

Agreement

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

- SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

- UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Agreement

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

Agreement

For The repetition periodicity of SIB1 within the time window of on-demand SIB1,:

- Up to NW implementation (no change to existing specification)

Agreement

RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point

Agreement

Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

Agreement

It is up to gNB to configure whether RACH occasions for UL WUS are shared or separated from the RACH occasions for other usages.