3GPP TSG RAN WG1 Meeting #119	R1-2409565

Orlando, USA, November 18 - 22, 2024

Title:	Analysis - loT-NTN TDD mode DL synchronization and SIB scheduling

Source: 	Iridium, CCL

Type:	Discussion

Document for:	Decision

Agenda Item:	9.11.5

# Introduction

In RAN#105 new WID RP-242415 was approved [1] to introduce IoT-NTN TDD mode.

The Objectives of the WID, as per [1]:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements).

In RAN#118bis following relevant agreements were made [3]:

The target operating DL SNR of NB-IoT NTN TDD is obtained following the parameters in TR 36.763 and modifying the carrier frequency to 1.6GHz: - For LEO-600, the operating DL SNR is [4.91dB] - For LEO-1200, the operating DL SNR is [5.51dB] - When reporting link level simulation results, companies are encouraged to report the SNR margin with respect to the above operating DL SNRs. - Companies may report values for other scenarios (e.g. antenna gain, LEO-800) with the corresponding assumptions. RAN1 evaluates via link level simulations the following: - NPSS/NSSS detection - NPBCH decoding The impact of the TDD pattern at least on the following channels / signals are evaluated at least qualitatively (e.g. specification impact, number of available instances, etc.): - SIB1-NB - Other SIBs needed for basic NB-IoT NTN operation (e.g. SIB2, SIB31) - Paging - NPRACH / RAR - NPSS/NSSS/NPBCH - NPDCCH, NPUSCH, NPDSCH - Note: Study for the above channels / signals may continue during the normative phase. - It is up to companies to perform link level evaluations for some of the aspects above For link level simulations of NPSS / NSSS / NPBCH, RAN1 uses the following assumptions: Parameter; Value. Carrier frequency; 1.6GHz. Channel model; NTN TDL-C rural, 30 degrees. Frequency error / timing drift (including XO error); 34 ppm for NPSS / NSSS (24ppm Doppler + 10ppm XO error) 0.1ppm for NPBCH. Variation of frequency error / variation of timing drift; 0.27ppm/s for NPSS / NSSS 0 for NPBCH. UE velocity; 3km/h. Target performance; For NPSS/NSSS: - 99% detection probability with 0.1% false alarm rate - FFS: How to define correct detection, including successful timing / frequency estimation / cell ID detection. For NPBCH: - 1% BLER For the channels / signals above, acquisition delay shall be reported for the corresponding detection probability. Other assumptions (e.g. combining length, frequency error hypothesis, etc.); To be reported and justified by companies. For the study phase, RAN1 evaluates the following combination of N (in radio frames) and D (D = number of consecutive downlink subframes): - N = 9: - D = 8 - D = 20 - D = 30 - N = 8: - D = 20 - Companies to report which subframe indices are included in the set of consecutive downlink subframes. - Other combinations are not precluded to be reported by companies

This contribution presents an evaluation of downlink synchronization performance to an IoT-NTN (NB-IoT) based Satellite Access Node operating TDD mode with a half-duplex FDD frame structure. We also offer insights on configuration parameters for new transmission patterns and how these can be communicated to the UE.

# Discussion

For NB-IoT frame structure type-1 to be utilized on the same frequency in both uplink and downlink, separate spectrum time is allocated for each, separated by a time gap. The period during which uplink operations are allowed is referred here as the active uplink, and the period during which downlink operations are allowed is referred to as the active downlink in Figure 1. In TSG RAN meeting #105 it was agreed that the periodicity of the active uplink and downlink times are fixed for NGSO satellite system operating in L-band (1616 – 1626.5 MHz) at 90ms with N=9. There is no RF transmission or reception during inactive times, resulting in discontinuous downlink transmissions.

Figure 1: IoT-NTN TDD mode with HD-FDD frame structure

This paper focuses on evaluating the performance of downlink synchronization, covering only the relevant details that impact it. We have also presented our views on the impact of IoT-NTN TDD frame pattern on SIB-1 and SI scheduling.

The duration of the downlink active time is selected as 8ms (N = 9, D = 8 in [3]). The start of the downlink active time is selected to provide adequate opportunities for UEs to receive NPSS, NSSS, and NBPCH to achieve downlink synchronization. Every active downlink time instance includes subframes containing NPSS and NPBCH and every alternate active downlink instance includes subframe containing NSSS. Thus, we assumed that every active downlink instance is containing subframes #0 (NPBCH), sf#4 (SIB-1), sf #5 (NPSS) and sf#9 (NSSS). The scheduling of NPSS, NPSS, and NPBCH remains unchanged from the standard NB-IoT FDD system, except that transmissions occur only during active downlink periods and are disabled/punctured during inactive downlink periods.

One example of the NB-IoT FDD subframe alignment to the IoT-NTN TDD mode downlink active time duration of 8ms which contains sf#0, sf#5, sf#4, and sf#9 in every downlink active opportunity is illustrated in Figure 2. In this example sf#6 and sf#7 are never transmitted. We have used this subframe alignment to evaluate SI scheduling in section 3.2

Figure 2 Example of frame alignment in the IoT-NTN TDD mode

## Downlink Synchronization Evaluation

### NPSS Detection Performance

The downlink transmission is configured such that NPSS is available every 9th frame i.e. every 90ms. There is no NPSS transmitted in 8 out of 9 frames. The simulation assumes the knowledge of periodicity of NPSS subframes. The performance of NPSS detection is assessed by Monte-Carlo simulations.

The NPSS detection algorithm first uses the autocorrelation at 1.9Msps to detect the peak and then does the autocorrelation at 240ksps to detect the timing. Timing detection is considered successful when timing is detected within +/- 5 samples at 240ksps. Single NPSS subframe is used for timing detection.

The NTN TDL-C rural channel model with elevation angle 30 degrees is used for the simulation as per the agreement in RAN1#118bis [3]. The first set of simulations is performed with no Doppler effect and another set is run with total carrier offset of 54.4KHz.

At 1.6 GHz, with max 24 ppm doppler and 10 ppm UE crystal error, worst case carrier offset is 54.4 KHz.

Simulation has also considered an additional Doppler of 4 Hz due to UE movement at 3 km/h.

The doppler effect is simulated both in the frequency domain with carrier offset and in the time domain to represent effect due to timing drift.

Figure 3 NPSS detection performance using single NPSS subframe

For 1% of missed NPSS detection minimum SNR required is -3.5 dB with no doppler when single NPSS subframe is used for detection (Figure 3).

For 1% of missed NPSS detection minimum SNR required is -0.5dB with 54.4 KHz doppler when single NPSS subframe is used for detection (Figure 3).

The simulation results show that that NPSS can be detected with Doppler. The performance can be further enhanced by combining multiple NPSS subframes and using more advanced NPSS detection algorithms.

### NSSS Detection Performance

The NPSS detection stage should provide good estimation of the timing, frequency, and carrier phase offset. So, for NSSS, detection as per RAN1#118bis agreement [3], frequency drift/timing error of 0.1ppm (i.e. 160 Hz with carrier frequency of 1.6 GHz) is considered in the simulation.

The NSSS detection is considered successful when both the physical cell identity and System Frame Number (SFN) modulo 8 is correctly estimated.

Figure 4 NSSS detection performance with 1 NSSS subframe

At 1% missed NSSS detection rate, minimum SNR required is -3.4dB when single NSSS subframe is used (Figure 4).

### NPBCH Decoding Performance

Each NPBCH subframe contains all 34 data bits plus enough redundancy to decode the MIB in reasonable signal conditions (code rate including 24-bit CRC of 0.29), which can reduce network acquisition time. In poor signal conditions a UE will soft-combine multiple NPBCH occurrences within downlink active times to decode the MIB.

In the NPBCH detection simulation, it is assumed that only 0.1 ppm carrier offset is present as most of the timing and frequency errors can be pre-compensated at the NPSS and NSSS detection stage. In this simulation up to 7 NPBCH subframes are used are used for combining in 640ms MIB periodicity time window.

Figure 5 NPBCH decoding performance

For 1% missed NPBCH detection minimum required SNR required is 4 dB with one NPBCH subframes, -2.7 dB with four NPBCH subframes and -4.5 dB with 7 NPBCH subframes (Figure 5).

### Downlink Synchronization vs Operating Point

The downlink link budget analysis is done as per parameters from section 6.2 of [2] but for the L-band of 1.6 GHz instead of S-band (Table 1).

| Satellite Orbit | LEO-600 SET-1 | LEO 1200 SET-1 | LEO 800 |
| --- | --- | --- | --- |
| Satellite altitude | 600 km | 1200 km | 800 km |
| Frequency (GHz) | 1.6 | 1.6 | 1.6 |
| Elevation Angle (Central beam edge) | 27.00 | 26.30 | 8.20 |
| TX: DL EIRP (dBm/MHz) | 64 | 70 | 67.13 |
| RX: G/T (dB/K) | -31.62 | -31.62 | -31.62 |
| Free space path loss (dB) | 157.77 | 163.17 | 164.51 |
| Atmospheric loss (dB) | 0.10 | 0.10 | 0.1 |
| Shadow fading margin (dB) | 3.0 | 3.0 | 3.0 |
| Scintillation Loss (dB) | 2.2 | 2.2 | 2.2 |
| Polarization loss (dB) | 3.0 | 3.0 | 3.0 |
| Additional losses (dB) | 0.0 | 0.0 | 0.0 |
| CNR (dB) 180KHz | 4.91 | 5.51 | 1.30 |

Table 1 Downlink link budget for different orbits

Table 2 shows downlink synchronization link margin against agreed operating point SNR of 4.91 dB for LEO-600 and 5.51 dB for LEO-1200

| DL Synchronization | SNR (dB) for 99% Success Rate | SNR Margin from the Operating Point (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | LEO-600 SET-1 | LEO-1200 SET-1 | LEO-800 |
| NPSS Detection (1 subframe) | -0.5 | 5.41 | 6.01 | 1.8 |
| NSSS Detection (1 subframe) | -3.4 | 8.31 | 8.91 | 4.7 |
| NPBCH Decoding (1 subframe) | 4.0 | 0.91 | 1.51 | -2.7 |
| NPBCH Decoding (4 sf) | -2.7 | 7.61 | 8.21 | 4 |
| NPBCH Decoding (7 sf) | -4.5 | 9.41 | 10.01 | 5.8 |

Table 2 Downlink synchronization link budget margin against reference scenarios

Considering downlink link budget analysis and downlink synchronization simulation results we made the following observations.

Observation 1: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, successful 99% NPSS detection with single NPSS subframe can be achieved with SNR margin of 5.41 dB and 6.01 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, operating in L-band.

Observation 2: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 1.8 dB to decode NPSSS successfully using single NPSS subframe. NPSS decoding performance can be further enhanced by combining multiple NPSS subframes and using more advanced NPSS detection algorithms.

Observation 3: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, successful 99% NSSS detection can be achieved with SNR margin of 8.31dB and 8.91 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, operating in L-band.

Observation 4: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 4.7 dB to decode NSSS successfully using single NSSS subframe. NSSS decoding performance can be further enhanced by combining multiple NSSS subframes.

Observation 5: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, NPBCH can be successfully decoded by receiving up to 4 NPBCH subframes with SNR margin of 7.61 dB and 8.21 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, operating in L-band.

Observation 6: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 4.0 dB to decode NPBCH successfully using up to four NPBCH subframes.

Observation 7: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, NPBCH can be successfully decoded by receiving up to 7 NPBCH subframes with SNR margin of 9.41 dB and 10.01 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, operating in L-band.

Observation 8: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 5.8 dB to decode NPBCH successfully using up to seven NPBCH subframes.

Proposal 1: In NB-IoT NTN TDD mode, with value N preset to 9 indicating an active downlink time periodicity of 90ms, a UE can successfully complete downlink synchronization by detecting NPSS, NSSS and NPBCH with sufficient margin. Thus, N=9 to be defined for the L-band (1616 – 1626.5 MHz).

# System Information Scheduling

In this section we present our views on the SIB-1 and SI messages scheduling with IoT-NTN TDD frame pattern shown in Figure 1 with N =9 and the duration of active downlink time set to 8 subframes (N = 9, D = 8 per [3]).

## SIB-1 Scheduling

In NB-IoT FDD mode, SIB-1 uses fixed schedule with a periodicity of 2560ms. It is transmitted on subframe 4 of every other frame and 8 subframes makes up one transport block. The starting frame for the first transmission of the SIB-1 is derived from the cell PCID.

The transport block size and the repetition rate of SIB1 is configurable, ranging from 208 to 680 bits, and 4, 8 or 16 block repetitions within a 2560ms period. All SIB1 subframe instances derived from the same transport block can be soft combined by the UE for processing gain in poor signal conditions. At the end of the 2560ms period the SIB1 contents may change.

The IoT-NTN TDD frame pattern limits the number of SIB1 subframes a UE can see within a SIB1 periodicity window of 2560ms. Table 3 shows the maximum number of SIB1 subframes that are visible and can be soft combined within a SIB1 periodicity window of 2560ms.

| SIB-1 Repetitions | Maximum number of SIB-1 subframes |
| --- | --- |
| 4 | 3 |
| 8 | 7 |
| 16 | 15 |

Table 3 Maximum number of SIB-1 subframes visible within SIB-1 period of 2560ms

Observation 9: When operating in NB-IoT NTN TDD mode, SIB-1 repetition pattern should be configured as 16 to allow every alternate active downlink opportunity to carry SIB-1 subframes for optimal SIB-1 acquisition time.

Observation 10: When operating in NB-IoT NTN TDD mode, SIB-1 repetition of 4 and 8 may also be sufficient for successful decoding. Satellite network operators can configure SIB-1 scheduling parameters from the existing parameters set depending on the required SIB-1 acquisition latency time.

Proposal 2: When operating in NB-IoT NTN TDD mode, there is no need to update the SIB-1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

## SI Scheduling

The SI messages are transmitted within periodically occurring time domain windows called SI-windows using scheduling information provided in SIB-1. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap.

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IOT downlink subframes depending on TBS. Any TBS of 208 bits or higher uses 8 consecutive NB-IOT downlink subframes.

### SI Window Start Time

Every SI window starts at sf# 0 which gets postponed to sf#1 as sf#0 is reserved for NPBCH.

With IoT-NTN TDD mode frame pattern, it is possible that the SI window start time falls in the inactive downlink time. This can be resolved by postponing the SI window to the next active downlink opportunity as shown in Figure 6.

Figure 6 Start of SI transmission is postponed to the next DL active time

Observation 11: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Proposal 3: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

### Single SI Transport Block Duration

Some of the SI messages like SIB-2 and SIB-31 can exceed TBS size of 208 bits and requires total 8 subframes to transmit a single SI transport block. With active downlink duration of 8 subframes, some are reserved for NPSS/NSS/NPBCH/SIB-1 so only four or five subframes of a single SI message can be accommodated within a single downlink opportunity. Thus, a single SI transport block of size 8 subframes cannot fit within a single downlink active time opportunity. For adequate SI decoding performance in the UE, all 8 subframes of the SI transport block should be transmitted. This requires transmission of a single SI block to spread across two downlink active time opportunities, as illustrated in Figure 7.

Figure 7 Single SI transport block transmission of duration 8 subframe across two DL opportunities

Observation 12: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks when SI TBS is equal to or greater than 208 bits.

Proposal 4: In IoT-NTN TDD mode, SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

### Overlap within SI Window

SI transmission postponement to the next active downlink opportunity could result in overlap with the next repetitions within the SI window. This can be resolved by truncating SI transport block transmission if it overlaps with the next SI repetition. As illustrated in Figure 8, transmission of first repetition gets truncated at SFN 8 where start of second repetition is scheduled. Thus, next downlink active time carry subframes from the second repetitions instead of carrying the postponed last three subframes of the first repetition.

Figure 8 Truncation of SI repetition with repetition pattern every 8th radio frame

Observation 13: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 14: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Proposal 5: In IoT-NTN TDD mode, SI transmissions shall be truncated, if due to the proposed postponement they overlap with the transmission of the next SI repetition.

The SI transmission postponement to the next downlink active time could also result in the SI transmission in the subframes outside of the configured SI window. This could be resolved by proposing changes in (36.331, 18.1.0) to truncate any SI transmissions outside of the SI transmission window as shown in Figure 9.

Figure 9 SI window overflow shall not be allowed

Observation 15: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in the SI transmission in the subframes outside of the configured SI window.

Proposal 6: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 7: In NB-IoT NTN TDD mode, with value N preset to 9 indicating an active downlink time periodicity of 90ms, a UE can successfully complete downlink synchronization including SIB-1 acquisition. By making minor modifications to the SI scheduling mechanism, all SI messages can be scheduled with active downlink time duration of 8ms. Thus, D=8 can work for the IoT-NTN TDD mode for scheduling SI messages.

# Conclusion

In this contribution, we have evaluated the performance of downlink synchronization which includes detecting NPSS, NSSS and decoding NPBCH in NB-IoT NTN TDD mode with Half-Duplex frame structure and provided some guidance on configuration parameters for the active downlink and active uplink patterns. Our observations and proposals are listed below:

Observation 1: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, successful 99% NPSS detection with single NPSS subframe can be achieved with SNR margin of 5.41 dB and 6.01 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, but operating in L-band.

Observation 2: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 1.8 dB to decode NPSSS successfully using single NPSS subframe. NPSS decoding performance can be further enhanced by combining multiple NPSS subframes and using more advanced NPSS detection algorithms.

Observation 3: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, successful 99% NSSS detection can be achieved with SNR margin of 8.31dB and 8.91 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, operating in L-band.

Observation 4: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 4.7 dB to decode NSSS successfully using single NSSS subframe. NSSS decoding performance can be further enhanced by combining multiple NSSS subframes.

Observation 5: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, NPBCH can be successfully decoded by receiving up to 4 NPBCH subframes with SNR margin of 7.61 dB and 8.21 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, operating in L-band.

Observation 6: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 4.0 dB to decode NPBCH successfully using up to four NPBCH subframes.

Observation 7: When operating in NB-IoT NTN TDD mode with half duplex FDD frame structure, NPBCH can be successfully decoded by receiving up to 7 NPBCH subframes with SNR margin of 9.41 dB and 10.01 dB for satellite parameter Set1 and the LEO600 and LEO1200 orbits respectively, considered in 3GPP TR 36.763, s operating in L-band.

Observation 8: For LEO-800, even at the lower elevation angle of 8.20 downlink link budget has SNR margin of 5.8 dB to decode NPBCH successfully using up to seven NPBCH subframes.

Observation 9: When operating in NB-IoT NTN TDD mode, SIB-1 repetition pattern should be configured as 16 to allow every alternate active downlink opportunity to carry SIB-1 subframes for optimal SIB-1 acquisition time.

Observation 10: When operating in NB-IoT NTN TDD mode, SIB-1 repetition of 4 and 8 may also be sufficient for successful decoding. Satellite network operators can configure SIB-1 scheduling parameters from the existing parameters set depending on the required SIB-1 acquisition latency time.

Observation 11: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Observation 12: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks when SI TBS is equal to or greater than 208 bits.

Observation 13: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 14: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Observation 15: With IoT-NTN TDD mode frame pattern and postponement of transmission of SI transport block to the next downlink active time can result in in the SI transmission in the subframes outside of the configured SI window.

Proposal 1: In NB-IoT NTN TDD mode, with value N preset to 9 indicating an active downlink time periodicity of 90ms, a UE can successfully complete downlink synchronization by detecting NPSS, NSSS and NPBCH with sufficient margin. Thus, N=9 to be defined for the L-band (1616 – 1626.5 MHz).

Proposal 2: When operating in NB-IoT NTN TDD mode, there is no need to update the SIB-1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

Proposal 3: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Proposal 4: In IoT-NTN TDD mode, SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

Proposal 5: In IoT-NTN TDD mode, SI transmissions shall be truncated if due to the proposed postponement, it overlaps with the transmission of the next SI repetition.

Proposal 6: In IoT-NTN TDD mode, SI transmission shall be truncated if due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 7: In NB-IoT NTN TDD mode, with value N preset to 9 indicating an active downlink time periodicity of 90ms, a UE can successfully complete downlink synchronization including SIB-1 acquisition. By making minor modifications to the SI scheduling mechanism, all SI messages can be scheduled with active downlink time duration of 8ms. Thus, D=8 can work for the IoT-NTN TDD mode for scheduling SI messages.

# References

[1]	RP-242415, “New WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

[2] 	3GPP TR 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)

[3]	R1-2409039, “Feature lead summary #2 on IoT-NTN TDD mode”, 3GPP TSG RAN WG1 Meeting #118bis, Hefei, China, October 14-18, 2024

[4]    R1-2408400, “Analysis - loT-NTN TDD mode DL synchronization”, 3GPP TSG RAN WG1 Meeting #118bis, Hefei, China, October 14-18, 2024