3GPP TSG RAN WG1 #119		R1-2409626

Orlando, US, November 18th – 22th, 2024

Agenda Item:     9.1.3

Source:	Spreadtrum, UNISOC

Title:	             Discussion on AIML for CSI prediction

Document for:	Discussion and decision

Introduction

In RAN#105 meeting, revised WID on AI/ML for NR Air interface is approved [1]. In this work item, the normative support for CSI prediction (UE-sided model) for air interfaces will be provided from Q1 of 2025. In addition, further study will be conducted to address consistency of training/inference in the current phase.

Provide specification support for the following aspects: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In this section, we would focus on the further study for CSI prediction consistency of training/inference.

Further study on consistency of training and inference

In RAN1 #118bis meeting [2], it was agreed to identify which potential NW-side additional conditions may impact on UE assumption.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

If the simulation results show that there is NW-side additional condiction affecting CSI prediction performance, the existing method of the one-side model should be followed as far as possible.

In RAN1 #116bis meeting [3], the associated ID was introduced in other aspects (9.1.3.3) firstly. Then, the associated ID was discussed in AI-BM (9.1.1) to ensure the consistency of NW-side additional condition across training and inference.

Agreement (9.1.3.3) From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. Agreement (9.1.1) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded.

For option1, if the data used by the training and inference is under the same associated ID, then UE assumes that the consistency across training and inference is guaranteed. This scheme will occur before model inference. However, for option 2, it is obvious that performance monitoring will happen after model inference to verify prediction performance. Therefore, UE must have a trial and error process, which will cause great performance loss for UE. Moreover, the performance monitoring based approach cannot determine whether the performance degradation is due to a consistency problem. So option 2 should be excluded.

For CSI prediction, considering workload, existing conclusions of one-side models (e.g., AI-BM) can be reused as much as possible. Therefore, associated ID can also be used to solve the consistency problem of CSI prediction

Proposal 1: For CSI prediction, associated ID can be used to ensure consistency between training and inference.

In RAN1 #118 meeting [4], there was a discussion about how to configure the associated ID and how do UE assume the associated ID in AI-BM (9.1.1). Details are as follows:

Agreement For UE sided model in beam management, introduce support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

For associated ID configuration, because the reference resources for channel measurement and beam management are both configured within CSI framework. Thus, the associated ID for CSI prediction should also be configured within CSI framework.

Proposal 2: For CSI prediction, associated ID can be configured within CSI framework.

Conclusion

In this contribution, we provide our opinions on standard impacts of CSI prediction.

Proposal 1: For CSI prediction, associated ID can be used to ensure consistency between training and inference.

Proposal 2: For CSI prediction, associated ID can be configured within CSI framework.

Reference

- RP-242399 Revised WID for NR_AIML_air

- Chair notes RAN1#118bis eom0

- Chair notes RAN1#116bis eom0

- Chair notes RAN1#118 eom0