3GPP TSG RAN WG1 #119		R1-2409628

Orlando, US, November 18th – 22nd, 2024

Agenda Item:     9.1.4.2

Source:	Spreadtrum, UNISOC

Title:	             Discussion on other aspects of AI/ML model and data

Document for:	Discussion and decision

Introduction

In Rel-18, the application of AI/ML techniques to NR air interface has been studied. In RAN#102 meeting, it has been agreed to do the normative support for the general framework for AI/ML for air interface, as well as, enable the recommended use cases in the preceding study[1]. In addition, we also agree to further study some issues identified during the Rel-18 study in an attempt to deepen the understanding in view of future normative work. Further, the study of general part regarding data collection for UE-sided model training, model transfer/delivery and mode identification has been extended to the end of Rel-19 in RAN#105 meeting [2].

In this paper, we would share our views on the general part, including data collection for UE-sided model training, model transfer/delivery and model identification/procedure for two-sided model.

Discussion

Data collection for UE-sided model training

In Rel-18 study stage, several of UE data collection mechanisms have been studied and identified. They also have been captured in TR38.843 [3] shown below.

The following proposals were discussed in RAN2: - UE collects and directly transfers training data to the Over-The-Top (OTT) server; 1a) OTT (3GPP transparent) 1b) OTT (non-3GPP transparent) - UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server. - UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server. RAN2 did not study or analyse these proposals and did not agree to requirements or recommendations.

Further, in Rel-19, RAN2 has done some analysis on the listed candidates, but RAN2 can not make decision on the down-selection. Thus, in RAN#105 meeting [2], one LS is to be sent to TSG SA, WG SA2, WG SA3 and WG SA5 to achieve more information.

In our mind, since the purpose of UE’s data collection is for UE-sided model training, we have not seen any strong justification to let the privacy data exposure to Network. Thus, we prefer mechanism 1a. Given what we have said, the issue is more related to RAN2. We can further discuss about it if needed after some progress of RAN2.

Proposal 1: For data collection for UE-side model training, no discussion is needed in RAN1.

Model transfer/delivery

In R18, six model transfer/delivery cases (i.e., case y/z1/z2/z3/z4/z5) have been identified, and pros and cons for each case also have been studied in RAN1. With the assumption of model transfer/delivery, RAN2 has done studies from the perspective of signaling procedure.

In previous meetings [4][5][6], some cases of model transfer/delivery have been concluded to be deprioritized. One initial summary on current status is shown below.

Table 1 The current status of mode transfer/delivery cases

|  | UE-sided model | Two-sided model |
| --- | --- | --- |
| Case y |  |  |
| Case z1 |  |  |
| Case z2 | Deprioritized | Deprioritized |
| Case z3 | Deprioritized | Deprioritized |
| Case z4 |  |  |
| Case z5 | Deprioritized | Deprioritized |

For Case z1, it may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y which does not have such burden. It seems that Case z1 may alleviate the burden of model maintenance/storage at the device side, and would be one additional benefit compared to case y. However, in general, the model(s) would be stored/maintained at UE’s server where the model training would happen, and the storage is not one issue for case y.

Proposal 2: From RAN1 perspective, the model transfer/delivery Case z1 should be deprioritized in Rel-19.

For Case z4 with model transfer in open format of a known model structure at UE, it may be beneficial for two-sided model in terms of device storage requirement, and model paring. However, proprietary design disclosure concern would arise. In addition, for model pairing, there are other solutions other than model delivery/transfer. The training type for two-sided model has not been decided, and multi-vendor issue is being discussed in AI9.1.4.1. Whether to support Case Z4 depends on the progress of 9.1.4.1.

Observation 1: Whether to support Case Z4 depends on the progress of 9.1.4.1 multi-vendor issue achieved.

For case z4, in RAN1#118 meeting, we have made progress on the “known model structure(s)”. In our understanding, RAN1 should focus on the content of model structure, and the signaling part of model is up to other WG.

Observation 2: RAN1 should focus on the content of model structure. The model format and/or signaling part is up to other WG.

Model identification

In [1], it has stated that we would continue to study model identification, including necessity and details in Rel-19.

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

For model-based identification, in Rel-18 study stage and also in TR38.843 [3], we have agreed three types of model identification where Type A is for offline identification, and Type B1 and Type B2 are for online identification.

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signalling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signalling after model identification. - Type B: Model is identified via over-the-air signalling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Note: This study does not imply that model identification is necessary.

In RAN1#116 [4], after some discussions, the following agreements have been achieved to provide some guidance for future discussions.

Agreement - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Observation The other options are proposed for model identification type B by companies during the discussion: - MI-Option 4. Model identification via standardization of reference models. (for CSI compression) - MI-Option 5. Model identification via model monitoring Agreement - Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects: - Relationship between model ID and data collection related configuration(s) and/or indication(s) - Information transmitted from NW to UE (if any) - Information transmitted from UE to NW (if any) - The associated procedure - Usage/Applicable use case(s) of MI-Option 1 Note: whether MI-Option 1 is needed or not is a separate discussion

In RAN1#118 [7], down-selection among MI-Options is achieved.

Conclusion The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work.

In RAN#105 [2], the study of model identification has been extended to the end of Rel-19 with the scope limited only for two-sided models.

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

Next, we would discussion model identification under the assumption of only two-sided models considered.

For Type A, a large of air interface overhead can be saved while offline engineering is required. For offline coordination, in theory, anything can be exchanged/coordination among different entities and it is not needed to be specified. It can be applicable for two-sided model and one-sided model.

For type B, Type B1 and Type B2 can be interpreted as online identification. It is obvious that offline co-engineering can be avoided but it would bring in air interface overhead, spec effort and so on.

Regarding MI-Option 1 for Type B, we have discussed and have one example for UE-sided model. For two-sided model case, considering NW is responsible for the initial model training as we have discussed in CSI compression use case, one possible example for MI-Option1 for two-sided use cases is shown below:

- Step A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s)

  - Associated IDs for each sub use case in relation with NW-sided additional conditions

- Step B: UE(s) collects the data corresponding to the associated ID(s)

- Step C: UE(s) reports the collected data to the NW

- Step D: AI/ML models are developed (e.g., trained, updated) at NW side based on the collected data corresponding to the associated ID(s).

- Step E: AI/ML models or data are delivered to UE by NW with corresponding model ID to develop the model of UE part.

According to the above example, for MI-Option1, model identification can be completed along with dataset collection related configuration(s) and/or indication, e.g., step E in above example. Regarding dataset collection related configuration, it can be RS configuration, or #subband configuration, or configuration reflecting NW-side additional condition, or configuration reflecting UE-side additional condition, and so on. The dataset achieved by basing the related configurations can be used for model training.

Depending on model training methods, one physical model can be trained with dataset achieved by basing on one specific dataset collection configuration/associated ID, or multiple datasets achieved by basing on multiple specific dataset collection configuration/associated ID. Regarding how logical model ID is determined, it is preferred that it is up to NW to assign the model ID to UE, since NW is always responsible for the cell management, and it can alleviate NW’s burden if unified model ID management is considered for all UEs in the cell.

Observation 3: From RAN1 perspective, the following procedure is an example of MI-Option 1 for two-sided model:

- Step A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s)

  - Associated IDs for each sub use case in relation with NW-sided additional conditions

- Step B: UE(s) collects the data corresponding to the associated ID(s)

- Step C: UE(s) reports the collected data to the NW

- Step D: AI/ML models are developed (e.g., trained, updated) at NW side based on the collected data corresponding to the associated ID(s).

- Step E: AI/ML models or data are delivered to UE by NW with corresponding model ID

- Note: It is up to NW to assign the model ID.

However, when we see the above example, Step A/B/C seems not to be necessary as the procedure of model identification, since there are also other ways to achieve date for gNB, e.g., utilizing SRS to achieve the data. The key point for model identification for MI-Option1 is step E, while step E can be realized by other MI-Options, e.g., MI-Option 2/3.

Proposal 3: MI-Option 1 shall not be considered for two-sided use case.

For MI-Option 2, last meeting [6], it has been agreed that one ID can be transmitted from network/network-side to UE/UE-side for the dataset. From our side, the ID can be model ID, and MI-Option 2 can be applied for two-sided model. In addition, it may be beneficial to enable model alignment between two sides. But it depends on whether option 4 for inter-vendor issue would be supported.

For MI-Option 3, model identification is accomplished with model transfer, where model ID can be included into the transferred information. It can be applied for any use case. In addition, it may be beneficial to enable model alignment between two sides. But it depends on whether model transfer would be supported.

For MI-Option 4, since the reference model would be standardized, it is natural to realize model identification, and it can be applicable for any use case. In addition, it may be beneficial to enable model alignment between two sides. But it depends on whether to define the reference model in Rel-19.

Observation 4: MI-Option 2/3/4 can be considered for two-sided use cases.

Conclusion

In this contribution, we provide our opinions on general aspects of AI/ML:

Observation 1: Whether to support Case Z4 depends on the progress of 9.1.4.1 multi-vendor issue achieved.

Observation 2: RAN1 should focus on the content of model structure. The model format and/or signaling part is up to other WG.

Observation 3: From RAN1 perspective, the following procedure is an example of MI-Option 1 for two-sided model:

- Step A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s)

  - Associated IDs for each sub use case in relation with NW-sided additional conditions

- Step B: UE(s) collects the data corresponding to the associated ID(s)

- Step C: UE(s) reports the collected data to the NW

- Step D: AI/ML models are developed (e.g., trained, updated) at NW side based on the collected data corresponding to the associated ID(s).

- Step E: AI/ML models or data are delivered to UE by NW with corresponding model ID

- Note: It is up to NW to assign the model ID.

Observation 4: MI-Option 2/3/4 can be considered for two-sided use cases.

Proposal 1: For data collection for UE-side model training, support 1a or we could wait the progress of RAN2.

Proposal 2: From RAN1 perspective, the model transfer/delivery Case z1 should be deprioritized in Rel-19.

Proposal 3: MI-Option 1 shall not be considered for two-sided use case.

Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TR38.843 V2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- Draft Report of 3GPP TSG RAN WG1 #116

- Draft Report of 3GPP TSG RAN WG1 #116b

- Draft Report of 3GPP TSG RAN WG1 #118b

- Draft Report of 3GPP TSG RAN WG1 #118