3GPP TSG RAN WG1 #119	R1-2410057

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.3.1

Source: 	Fujitsu

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:		Discussion and decision

Introduction

The Rel-19 work item on evolution of NR duplex operation includes a list of objectives as in the WID[1], out of which the following objectives should be discussed under agenda item 9.3.1.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators

In this contribution, we will share our views on indication of frequency and time location of SBFD subbands, and transmissions and receptions in SBFD symbols and/or non-SBFD symbols for SBFD-aware UEs.

Discussion

Configuration of SBFD subbands

In RAN1#117 meeting, it was agreed to support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode. This relies on support of cell-specific configuration of time and frequency location of SBFD subband which is broadcast in system information.

In RAN2#127 meeting, it was agreed to provide the cell-specific configuration by SIB1. While in RAN1, some previous agreements (as below) were formulated only for RRC connected mode UEs. Technically, they are also applicable for RRC_IDLE/INACTIVE mode UEs. And this is better to be clarified.

Proposal 1: The previous agreements for RRC connected mode UEs, listed below, are also applied to RRC_IDLE/INACTIVE UEs.

- Confirm the working assumption to support cell-specific configuration on frequency location of SBFD subbands

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement: For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options. - Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - FFS: Further details FFS: Details when two TDD-UL-DL patterns are configured. Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Regarding UE-specific configuration, in previous discussions, the most controversial part is whether UE-specific GB is needed. In our understanding, this depends on RAN4 study. If it is needed, RAN4 can define the candidate sizes. Then the cell-specific configuration can guarantee that the cell-specific GB is not smaller than any UE-specific GB. Therefore, it is not necessary to support UE-specific configuration.

Proposal 2: Do not support UE-specific configuration of SBFD subbands.

Frequency location

Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275 For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

Since the UL/DL subbands are configured for each SCS configuration in SCS-SpecificCarrierList for UL/DL, SCS-SpecificCarrier IE can be extended to configure the corresponding DL/UL subbands for the SCS configuration inside. In this way, the association between DL/UL subbands and the SCS configuration is clear and does not require additional signaling overhead.

Proposal 3: Confirm the following Working Assumption.

Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275 For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

Proposal 4: Extend SCS-SpecificCarrier IE to configure the corresponding DL/UL subbands for the SCS configuration inside.

Time location

Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details

In the similar principle as DL/UL subband, TDD-UL-DL-Pattern IE can be extended to configure the corresponding SBFD symbols for the TDD-UL-DL-Pattern inside. In this way, the association between DL/UL subbands and the TDD-UL-DL-Pattern is clear and does not require additional signaling overhead.

Proposal 5: Extend TDD-UL-DL-Pattern IE to configure SBFD symbols within the corresponding TDD-UL-DL pattern.

SBFD symbols are configured based on the reference SCS same as TDD UL-DL configuration. For TDD UL-DL configuration, the UE needs to determine the DL/UL/F symbols for DL/UL BWPs based on the DL/UL/F symbols configured with respect to the reference SCS. Similarly, for SBFD configuration, the UE also needs to determine the SBFD/non-SBFD symbols for DL/UL BWPs based on the configuration with respect to the reference SCS. The corresponding rules should be discussed.

The reference SCS is the smaller or equal to the SCS of any DL/UL BWP. For the DL/UL BWP with NCP, a symbol of the DL/UL BWP would overlap with only an SBFD symbol or only a non-SBFD symbol of the reference SCS. So, it is obvious that a symbol overlaps with an SBFD/non-SBFD symbol of the reference SCS should also be an SBFD/non-SBFD symbol. However, For the DL/UL BWP with ECP, a symbol may simultaneously overlap with an SBFD symbol and a non-SBFD symbol of the reference SCS, as shown in Figure 1. In this case, whether the symbol should be an SBFD symbol or non-SBFD symbol needs to be clarified.

Figure 1. BWP symbol overlapping with both SBFD and a non-SBFD symbol of the reference SCS

Proposal 6: RAN1 to discuss how to determine SBFD/non-SBFD symbols for active DL/UL BWPs with NCP or ECP based on the SBFD/non-SBFD symbols configured with respect to referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon.

UL/DL usable PRBs

Agreement If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.

As mentioned in 2.1.1, cell-specific UL/DL subbands are configured	per SCS configuration. It is straightforward to determine the UL/DL usable PRBs for an UL/DL BWP based on the UL/DL subbands corresponding to the SCS configuration of the UL/DL BWP. To avoid misunderstanding, we propose the following clarification.

Proposal 7: If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, for a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband corresponding to the SCS configuration of the UL BWP and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) corresponding to the SCS configuration of the DL BWP and the DL BWP in SBFD symbols.

Based on the definition of UL/DL usable PRBs, since the locations of UL/DL subbands and UL/DL BWP are quite flexible from RRC configuration perspective, for an UL/DL BWP, it is possible that the intersection between UL/DL subbands and the UL/DL BWP is empty (i.e, UL/DL usable PRBs are absent). However, previous discussions focused on the case where UL/DL usable PRBs are present. It is worthy to clarify whether/how to handle the case where UL/DL usable PRBs are absent for an UL/DL BWP.

Proposal 8: RAN1 to clarify whether/how to handle the case where UL/DL usable PRBs are absent for an UL/DL BWP.

Link direction determination and collision handling

As mentioned in the WID, half duplex operation is assumed at the UE side. In a semi-statically configured SBFD symbol, the UE could only receive or transmit.

To determine whether to receive or transmit at the UE side, two options have been provided for further consideration.

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

In our view, at least for Case 3, link direction indication is beneficial. According to previous agreements, if link direction indication is not supported, it would rely on gNB to avoid the collision Case 3. This case (semi-static DL reception (PDCCH, SPS PDSCH, P/SP CSI-RS etc.) vs. semi-static UL transmission (CG PUSCH, P/SP PUCCH, P/SP SRS etc.) involves many DL/UL channels/signals. When gNB tends to provide multiple configurations for each channel/signal, it may be difficult to for gNB implementation to avoid the collision case. In this case, link direction indication can alleviate the burden on gNB.

Proposal 9: To determine link direction for SBFD symbols, at least for Case 3 (semi-static DL reception (PDCCH, SPS PDSCH, P/SP CSI-RS etc.) vs. semi-static UL transmission (CG PUSCH, P/SP PUCCH, P/SP SRS etc.), support link direction indication.

If link direction indication for SBFD symbols is supported, regarding the signalling, the discussion should be under the umbrella of the Note below from the WID.

Since the Note is about the SBFD subbands configured in the flexible symbols indicated by TDD-UL-DL-ConfigCommon, the “legacy Uplink symbol” includes a flexible symbol which is indicated as UL by tdd-UL-DL-ConfigurationDedicated or an SFI in DCI format 2_0. So, according to the Note, it is expected that any flexible symbol indicated as UL by tdd-UL-DL-ConfigurationDedicated or an SFI in DCI format 2_0 is a non-SBFD UL-only symbol as in the legacy. In other words, flexible symbols indicated as UL by tdd-UL-DL-ConfigurationDedicated or an SFI in DCI format 2_0 cannot be SBFD symbols. Consequently, tdd-UL-DL-ConfigurationDedicated and an SFI in DCI format 2_0 cannot indicate UL/DL for SBFD symbols.

Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols

Observation 1: According to the Note in the WID:

- tdd-UL-DL-ConfigurationDedicated or an SFI in DCI format 2_0 cannot indicate link direction (UL/DL) for SBFD symbols.

Therefore, if link direction indication for SBFD symbols is supported, a new signaling should be introduced.

Proposal 10: If link direction indication for SBFD symbols is supported, a new signaling should be introduced.

For SSB symbols configured with SBFD subbands, three options were discussed, and the following Working Assumption was made.

- Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Option 2: The SBFD configuration is not applicable to a slot consisting of SSB symbols. - Option 3: The SBFD configuration is not applicable to a TDD pattern period with slot(s) consisting of SSB symbols.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

In our understanding, Option 1 has somehow been implicitly agreed by other agreements which were reached by assuming Option 1. For example, regarding RO validation discussed in AI 9.3.2, it has been agreed that an additional RO of RACH configuration Option 1 is invalid if it overlaps with SSB in time domain. This condition is needed based on Option 1 but not needed based on Option 2 and Option 3. Given the situation, we propose to confirm the Working Assumption.

Proposal 11: Confirm the following Working Assumption.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

Separate power control/beams for SBFD symbols and non-SBFD symbols

According to the following agreement in the RAN1#116b meeting, it was agreed to study separate power control and separate beams.

Agreement Study the feasibility and enhancements to support separate power control and/or spatial relation for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots, including repetition and non-repetition, by considering existing schemes, e.g. multi-TRP PUCCH/PUSCH repetition schemes.

According to the following agreement in the RAN1#117 meeting, it was further agreed to support separate power control.

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

A remaining issue is whether to support separate beams for the transmissions/receptions in SBFD symbols and non-SBFD symbols. The motivation to support separate power control mainly comes from different antenna configurations and different interference environments across SBFD and non-SBFD symbols. Similarly, different antenna configurations and different interference environments may prefer different beams. Since separate power control has been supported to better suited for the case, separate beams should be also supported.

Proposal 12: Separate beams/spatial relations for the transmissions/receptions in SBFD symbols and non-SBFD symbols should be supported.

In the RAN1#118 meeting, it was agreed to down-select from the following options to support separate power control.

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state

For Option 1, separate TCI states are applied to SBFD symbols and non-SBFD symbols. For Option 2, the same TCI state is associated with separate power control parameters for SBFD and non-SBFD symbols. For Option 3, the same TCI state is associated with the same power control parameters for SBFD and non-SBFD symbols, but a power offset is additionally used for SBFD symbols. Since separate power control parameters for SBFD and non-SBFD symbols have been agreed, the down-selection mainly depends on whether separate TCI states should be supported for SBFD symbols and non-SBFD symbols. SBFD and non-SBFD symbols may have different antenna configurations and different interference environments. Since different antenna configurations and different interference environments may prefer different beams, supporting separate TCI states can be beneficial. To this end, Option 1 is a convenient way to support separate TCI states. Also, it provides a simple way to support separate power control parameters without changing the existing TCI sate IE structure. Therefore, Option 1 is preferred.

Proposal 13: To support separate power control, the following Option 1 is preferred.

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

Another issue is whether the UL spatial relation Info framework is supported for SBFD. Our first reference is that both the unified TCI state framework and the UL spatial relation Info framework are supported. For the unified TCI state framework, it is the future trend and thus should be supported. For the UL spatial relation Info framework, it has been supported in the existing networks. Therefore, supporting it could be beneficial for the fast development of SBFD. However, it is also acknowledged that more effort is needed to support both. To this end, we can begin with the unified TCI state framework and support the UL spatial relation Info framework if time allows.

Proposal 14: It can be considered to support both the unified TCI state framework and the UL spatial relation Info framework for SBFD (with the unified TCI state framework prioritized).

Configuration 1 vs. Configuration	2

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2.

To deal with transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), Configuration 1 and Configuration 2 were introduced. According to discussions so far, such transmissions include PUSCH repetition, TBoMS, multi-PUSCH scheduled by a single DCI, CG PUSCH, SP-CSI on PUSCH, and may or may not include P/SP SRS, P/SP PUCCH, PUCCH repetition. And such receptions include PDSCH repetition, multi-PDSCH scheduled by a single DCI and SPS PDSCH, and may or may not include P/SP CSI-RS. Note that it is common understanding that PDCCH enhancement is not needed and thus it is not included. In those transmissions/receptions, PUSCH repetition other than Msg.3 PUSCH repetition, P/SP SRS, P PUCCH, PUCCH repetition, and P/SP CSI-RS are mandatory for a legacy UE, and the others are optional. In addition, those transmissions/receptions are possible only after the UE is provided with dedicated configurations, e.g. by an RRCReconfiguration, except for PUCCH repetition and Msg.3 PUSCH repetition.

With those said, support of Configuration 1/Configuration 2 for PUSCH repetition other than Msg.3 PUSCH repetition is necessary. For other transmissions/receptions, Configuration 1/Configuration 2 is not needed if they are not supported by the UE or not under Configuration 1/Configuration 2 framework. Furthermore, according to discussions on PDSCH and PUSCH, the complexity of UE processing for PUSCH under Configuration 2 would be higher than PDSCH and PUSCH under Configuration 1 due to the support of RB offset. Meanwhile, the design principle of SRS/PUCCH is also quite different from CSI-RS, e.g. with respect to frequency resource configuration etc. In addition, for DL, note that only P/SP CSI-RS is mandatory while CSI-RS may not be in the scope of Configuration 1/Configuration 2 discussion.

Based on the above, it is preferred that support of Configuration 1/Configuration 2 is subject to UE capability. In addition, at least for UL and DL, separate capabilities on support of Configuration 1/Configuration 2 should be defined. Discussion on whether finer definition, e.g. per channel/signal/scheduling manner (static/dynamic etc.), is needed can be leaved to later stage after the more concrete design for each channel/signal.

Proposal 15: For SBFD-aware UEs, support of Configuration 1/Configuration 2 is subject to UE capability.

- Define separate capabilities for UL and DL.

  - FFS finer definition, e.g. separate capabilities for different channels/signals/ scheduling manner (static/dynamic etc.).

In the three options, Option 2 is preferred because it can match well with the granularity of UE capability mentioned above. On top of that, it is beneficial to support separate configurations for configured and dynamic transmission/receptions for a given channel/signal. Take PUSCH as an example. As aforementioned, in case of Configuration 2, the UE needs to determine frequency resources based on RB offset for PUSCH in SBFD symbols. Assuming common configuration of Configuration 2 for PUSCHs, the UE may need to check per PUSCH whether it is in SBFD/non-SBFD symbols and then determine frequency resources accordingly, even if it is guaranteed by scheduling that all of the CG PUSCHs for a Type-1 CG PUSCH configuration are within SBFD symbols only or non-SBFD symbols only. In this case, to simplify UE processing, instead, Configuration 1 and the valid symbol type can be indicated in the CG PUSCH configuration.

For Type 2 CG PUSCH or SPS PDSCH, similar method as Type-1 CG PUSCH can be used. On the other hand, considering that they are activated by DCI with TDRA, they can also follow the configuration for dynamic PUSCH/PDSCH. The latter is slightly preferred for saving signaling overhead.

In addition, if support of Configuration 1/Configuration 2 is subject to UE capabilities, it is preferred to use dedicated RRC signaling rather than SIB to configure Configuration 1 or Configuration 2.

Therefore, we have the following proposal to support Option 2 with some changes.

Proposal 16: The SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis by dedicated RRC signaling.

- For Type 1 CG PUSCH, whether configuration 1 or 2 is applied is configured per CG PUSCH configuration.

- For PUSCH scheduled/activated by DCI, whether configuration 1 or 2 is applied is configured per BWP in PUSCH-Config.

- For PDSCH, whether configuration 1 or 2 is applied is configured per BWP in PDSCH-Config.

- FFS CSI-RS, SRS, PUCCH.

If dedicated RRC signaling is used for the configuration, the UE would be provided with explicit Configuration 1/Configuration 2 only after initial access. During initial access, there may be Msg.3 PUSCH repetition and PUCCH repetition etc. If the UE notifies the gNB that it is an SBFD-aware UE in some way (e.g. by using additional ROs dedicated for SBFD-aware Ues), two options can be considered for the follow-up transmissions/receptions before explicit Configuration 1/Configuration 2.

- Option 1: No need to define Configuration 1/Configuration 2 for the time duration. It is up to gNB to schedule appropriately. For example, if PUSCH/PDSCH is scheduled in SBFD symbols, the corresponding assigned PRBs should be within UL/DL subbands.

- Option 2: Define a default configuration. And Configuration 1 is preferred for its lower complexity.

Option 2 is preferred for scheduling flexibility. And this can also be applied to the case where the UE is not configured with Configuration 2 after initial access.

Proposal 17: For each channel/signal for which Configuration 1/Configuration 2 is applicable, if the SBFD-aware UE is not configured with Configuration 2, Configuration 1 is applied.

PDSCH

PDSCH TBS

For PDSCH, the following working assumption was made for Configuration 2. TBS determination is FFS.

Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

For SPS PDSCH without repetition, each PDSCH carries an independent TB. Therefore, when the number of valid PRBs is different for SBFD symbols and non-SBFD symbols, it is preferred to determine the corresponding TBS separately. For PDSCH repetition, including SPS PDSCH with repetition, the TBS determination can be simply based on the number of valid PRBs of the first repetition.

Proposal 18:

- For SPS PDSCH without repetition with Configuration 2,

  - For a PDSCH in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs only.

  - For a PDSCH in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

- For PDSCH repetition, including SPS PDSCH with repetition (for each SPS PDSCH), with Configuration 2

  - If the first repetition locates in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs only.

  - If the first repetition locates in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

multiple SPS PDSCHs in a slot

In the legacy, more than one SPS configuration can be activated. However, subject to UE capabilities, the UE may only be able to receive up to one/two PDSCHs in a slot. If there is more than one SPS PDSCH in a slot, the UE needs to do selection. The procedure is defined as below.

If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE.

For SBFD operation, in case of Configuration 1, the rule should be enhanced to resolve collision/overlapping with invalid symbol type. Besides, the impacts of tdd-UL-DL-ConfigurationDedicated should be considered. For example, if tdd-UL-DL-ConfigurationDedicated is not applicable to SBFD symbols but applicable to non-SBFD symbols, for PDSCH in SBFD symbols,  tdd-UL-DL-ConfigurationDedicated should be ignored for determining whether there exists collision, while for PDSCH in non-SBFD symbols, it should be taken into account.

Proposal 19: For SPS PDSCH, RAN1 to discuss how to enhance the rules for PDSCH selection in case of multiple PDSCHs in a slot.

CSI-RS

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware Ues, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

Regarding the Working Assumption above, the main concern was about CSI processing timeline as highlighted. Since that has been resolved in the previous meeting, we have the proposal as follows.

Proposal 20: Confirm the following Working Assumption with changes.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware Ues, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

CSI report

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

In the legacy, the maximum number P/SP/AP CSI report setting per BWP is subject to UE capabilities. For example, the UE may support {1, 2, 3, 4} P/AP CSI report setting per BWP and {0, 1, 2, 3, 4} SP CSI report setting per BWP. When less than two are supported, Option A would be inapplicable. From this perspective, Option B is preferred. In addition, similar restrictions as Option A are needed

Proposal 21: For CSI report associated with periodic/semi-persistent CSI-RS, support Option B with similar restrictions as Option A.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols.

  - gNB configuration may not ensure that the CSI-RS associated with each sub-configuration is confined to either SBFD symbols or non-SBFD symbols only.

    - For the sub-configuration associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the sub-configuration associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Regarding configuration of the valid symbol type, two options were considered

- Option 1: the valid symbol type is configured in CSI-ReportConfig.

- Option 2: the valid symbol type is configured in CSI-ResourceConfig with resourceType set to ‘periodic’ or ‘semiPersistent’.

Option 1 is preferred over Option 2 for less CSI-RS resource configuration consumption. With Option 1, CSI reports for SBFD symbols and non-SBFD symbols can share the same CSI-RS resource configuration by deriving CSI only using CSI-RS in SBFD symbols and non-SBFD symbols respectively. While with Option 2, CSI reports for SBFD symbols and non-SBFD symbols require separate CSI-RS resource configurations.

Proposal 22: For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured in CSI-ReportConfig.

PUSCH

PUSCH repetition type A with available slot counting or TBoMS

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1, - For PUSCH repetition type A with available slot counting, TboMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

In the agreement, for PUSCH repetition type A with available slot counting, and TBoMS, UE would postpone transmissions in the invalid symbol type. For CG PUSCH, transmissions in the invalid symbol type are dropped. It is not clear how to deal with the case of CG PUSCH with PUSCH repetition type A with available slot counting or TBoMS. It is necessary to make a clarification on that. In our view, the rule for PUSCH repetition type A with available slot counting and TBoMS should be used in principle.

Proposal 23: For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, RAN1 to clarify,

- For CG PUSCH with repetition type A with available slot counting or TBoMS, UE postpones transmissions in the invalid symbol type.

For repetition type A with available slot counting and TBoMS, the rule in the legacy for determining the N∙K slots for a PUSCH transmission is as below for dynamic PUSCH and CG PUSCH respectively. In general, collision with legacy TDD UL-DL configurations and SSB are considered.

For unpaired spectrum: - When AvailableSlotCounting is enabled, and in case K>1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. - A slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1, 0_2 or 0_3 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. … - The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1, 0_2 or 0_3, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1, 0_2 or 0_3. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. For unpaired spectrum: - The UE determines N∙K slots for a PUSCH transmission of TB processing over multiple slots with a Type 2 configured grant activated by DCI format 0_1 or 0_2, based on tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst, and the TDRA information field value in the DCI format 0_1 or 0_2. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of TB processing over multiple slots with a Type 2 configured grant activated by DCI format 0_1 or 0_2 if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Since the UE would postpone transmissions in the invalid symbol type, to resolve the collision with the invalid symbol type, the rule for available slot counting should be enhanced. Besides, to make PUSCH transmissions overlapping with legacy DL symbols possible, the rule to resolve the collision with TDD UL-DL configurations should be revised.

Proposal 24: For repetition type A with available slot counting and TboMS, enhance the rules for available slot counting to resolve the collision with the invalid symbol type and to allow PUSCH transmissions overlapping with symbols indicated DL by tdd-UL-DL-ConfigurationCommon.

For dynamically scheduled PUSCH transmissions with Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission/reception. Therefore, the first slot in the N∙K slots should not be determined based on invalid symbol type. A separate rule is needed.

Proposal 25: For dynamically scheduled PUSCH transmissions in case of Configuration 1, define separate rules for determining the first slot and the other slots in the corresponding N∙K slots.

- Only the other slots are determined based on the invalid symbol type.

SP-CSI on PUSCH

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1, - For PUSCH repetition type A with available slot counting, TboMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped. Agreement For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TboMS is SBFD symbols, - When an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs. - FFS: The above applies to SP-CSI on PUSCH when applicable

As shown in the agreements above, SP-CSI on PUSCH is also considered to support Configuration 1. Regarding the FFS, from frequency resource allocation perspective, SP-CSI PUSCH is similar to Type 2 CG PUSCH. Therefore, it is straightforward to reuse the corresponding method to demine valid PRBs. However, TBS determination is not needed because SP-CSI on PUSCH is not expected to be multiplexed with uplink data on the PUSCH.

Proposal 26: For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SP-CSI on PUSCH is SBFD symbols,

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

RB offset

Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TboMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded.

In the previous discussion, there were different understandings on whether Configuration 2 applies to the case where all transmissions/receptions scheduled/activated by a single CG/DCI are within the same symbol type (SBFD symbols or non-SBFD symbols).

Given the situation, in the agreement, it is unclear how to determine the starting PRB for PUSCH in SBFD symbols when all PUSCH scheduled/activated by a single CG/DCI are within SBFD symbols. For example, if Configuration 2 is applicable to this PUSCH scheduling case, the RB offset should be applied, otherwise not.

In addition, if Configuration 2 would apply to the case where all transmissions/receptions scheduled/activated by a single CG/DCI are within the same symbol type, whether it also applies to single-slot PUSCH and whether RB offset is applied to the single-slot PUSCH in SBFD symbols may also need to be clarified.

Proposal 27: To clarify whether RB offset is applied when all PUSCHs are within SBFD symbols when the UE is provided with Configuration 2, RAN1 to discuss whether Configuration 2 applies to the case where all transmissions/receptions scheduled/activated by a single CG/DCI are within the same symbol type (SBFD symbols or non-SBFD symbols).

In the options for determining starting PRB for PUSCH transmissions in SBFD symbols, Option 1 is preferred for flexibility. Basically, in Option 2, the starting PRB for SBFD symbols would be highly bundled with the starting PRB for non-SBFD symbols. It may be too restrictive for scheduling when both PUSCHs in SBFD symbols and non-SBFD symbols are scheduled together.

Look into the listed equations. In Option 1, Equation 1-A is the simplest. The intention of Equation 1-B may be to guarantee that the starting PRB would be inside the BWP while Equation 1-C/1-D to guarantee that the starting PRB would be inside the UL usable PRBs and Equation 1-C seems simpler.

In Option 2, Equation 2-A may be to guarantee that the whole PUSCH in SBFD symbols would be inside UL usable PRBs. The intention is good, but it does not support PUSCH size equal to the size of UL usable PRBs. Equation 2-B is similar to Equation 1-C but without RB offset. Equation 2-C is respectively similar to Alt.2-4 discussed for lowest RO in SBFD symbols and Alt.2-4 has been precluded. Equation 2-D is similar to Alt 2-1 discussed for lowest RO in SBFD symbols, and it seems most restrictive for scheduling. For example, it does not support the case that starting PRB for non-SBFD symbols is higher than the end of UL usable PRBs.

As agreed, no matter which equation is adopted, UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RB offset to be overlapped with PRBs outside UL usable PRBs. Therefore, Equation 1-A is preferred for both flexibility and simplicity. If it is intended to guarantee this by equation, the following alternative can be considered.

RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD+RBoffsetSBFD, max⁡(NUL SBsize -NPUSCHsize, 1))

Proposal 28: When the UE is provided with Configuration 2 for PUSCH other than Msg3 PUSCH, one or multiple RB offsets are configured for determining starting PRB for PUSCH transmissions in SBFD symbols. Down select from the following equations.

- Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD

  - Negative values of RBoffsetSBFD are not precluded.

- Equation 1-E: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD+RBoffsetSBFD, max⁡(NUL SBsize -NPUSCHsize, 1))

Frequency hopping

Agreement Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

As agreed above, separate FH offset lists are supported for SBFD symbols and non-SBFD symbols. However, it is not clear whether this can be applied to PUSCH for SP-CSI, although it is straight-forward to use the same method as type 2 CG PUSCH. Therefore, we propose to make the following clarification.

Proposal 29: RAN1 to clarify,

- The new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols also apply to PUSCH for SP-CSI.

When FH offset/FH offset list is not provided for SBFD symbols, two options are considered.

- Option 1: FH is disabled for PUSCH transmissions in SBFD symbols.

- Option 2: FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

In different cases, different options may be preferred. For example, when the size of UL usable PRBs is much smaller than UL BWP, Option 1 may be preferred. When the size of UL usable PRBs is almost equal to UL BWP, Option 2 makes more sense. Therefore, both options should be supported.

Proposal 30: The following two options are supported when FH offset/FH offset list is not provided for SBFD symbols.

- Option 1: FH is disabled for PUSCH transmissions in SBFD symbols.

- Option 2: FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

If both options are supported, the UE needs to determine which one should be used. One simple method is to introduce new RRC parameters to explicitly enable/disable frequency hopping for SBFD symbols. Use Option 1 if it is disabled, use Option 2 otherwise.

Proposal 31: When FH offset/FH offset list for SBFD symbols is provided but FH offset/FH offset list for SBFD symbols is not provided, use new RRC parameters to explicitly enable/disable frequency hopping for SBFD symbols. When FH for SBFD symbols is enabled, the FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

Agreement: For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

Regarding frequency hopping for PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, there were parallel discussions in AI 9.3.1 and 9.3.2. To avoid inconsistence, it is better to discuss it under a single AI. Because this is related to Msg3 PUSCH which is a part of CBRA, AI 9.3.1 is preferred.

Regarding frequency hopping for other PUSCHs, Alt.1 is preferred for simplicity.

Proposal 32: For frequency hopping for PUSCH other than PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, remove -RBUL SB start in the equations (Alt.1).

- Note: discuss PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI in AI 9.3.2.

SRS

In the RAN1#118b meeting, the following agreement was achieved for separate SRS configurations for SBFD and non-SBFD symbols.

Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages

According to the agreement, Option 1 and Option 2 are subject to further down-selection. In Option 1, separate SRS resource sets are used for SBFD and non-SBFD symbols. In Option 2, separate configurations in the same SRS resource are used for SBFD and non-SBFD symbols. Between these two options, Option 1 is preferred. In our view, Option 1 is a cleaner solution and also more flexible when indicating SRI. For Option 1, UL DCI can include two SRI fields and two TPMI fields. The two SRI fields can independently indicate SRIs from two separate SRS resource sets. Therefore, Option 1 is more flexible to indicate appropriate SRIs for SBFD and non-SBFD symbols respectively. Aforementioned indication of SRI and TPMI is for the codebook and non-codebook based UL transmission. Therefore, Option 1 can be supported at least for SRS with the usages of ‘codebook’ and ‘nonCodebook’.

Proposal 33: To support separate SRS configurations, the following Option 1 is preferred, and it can be supported at least for the usages of “codebook” and “nonCodebook”.

- Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

Carrier aggregation

Until now, the discussions are focused on how to support SBFD operation within a cell. It is not clear whether to support carrier aggregation (CA) with a carrier configured with SBFD operation. Considering CA can significantly increase data rate and is widely deployed, it is preferred to support the combination.

Proposal 34: Support carrier aggregation with a component carrier configured with SBFD operation.

Conclusion

According to the discussions above, we have the following observations and proposals.

Configuration of SBFD subbands

Proposal 1: The previous agreements for RRC connected mode UEs, listed below, are also applied to RRC_IDLE/INACTIVE UEs.

- Confirm the working assumption to support cell-specific configuration on frequency location of SBFD subbands

Agreement For RRC connected mode Ues, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement: For RRC connected mode Ues, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options. - Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - FFS: Further details FFS: Details when two TDD-UL-DL patterns are configured. Agreement For RRC connected mode Ues, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Proposal 2: Do not support UE-specific configuration of SBFD subbands.

(Frequency location)

Proposal 3: Confirm the following Working Assumption.

Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275 For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

Proposal 4: Extend SCS-SpecificCarrier IE to configure the corresponding DL/UL subbands for the SCS configuration inside.

(Time location)

Proposal 5: Extend TDD-UL-DL-Pattern IE to configure SBFD symbols within the corresponding TDD-UL-DL pattern.

Proposal 6: RAN1 to discuss how to determine SBFD/non-SBFD symbols for active DL/UL BWPs with NCP or ECP based on the SBFD/non-SBFD symbols configured with respect to referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon.

UL/DL usable PRBs

Proposal 7: If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, for a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband corresponding to the SCS configuration of the UL BWP and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) corresponding to the SCS configuration of the DL BWP and the DL BWP in SBFD symbols.

Proposal 8: RAN1 to clarify whether/how to handle the case where UL/DL usable PRBs are absent for an UL/DL BWP.

Link direction determination and collision handling

Proposal 9: To determine link direction for SBFD symbols, at least for Case 3 (semi-static DL reception (PDCCH, SPS PDSCH, P/SP CSI-RS etc.) vs. semi-static UL transmission (CG PUSCH, P/SP PUCCH, P/SP SRS etc.), support link direction indication.

Observation 1: According to the Note in the WID:

- tdd-UL-DL-ConfigurationDedicated or an SFI in DCI format 2_0 cannot indicate link direction (UL/DL) for SBFD symbols.

Proposal 10: If link direction indication for SBFD symbols is supported, a new signaling should be introduced.

Proposal 11: Confirm the following Working Assumption.

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware Ues. - Note: The SSB block is assumed to be within DL subband

Separate power control/beams for SBFD symbols and non-SBFD symbols

Proposal 12: Separate beams/spatial relations for the transmissions/receptions in SBFD symbols and non-SBFD symbols should be supported.

Proposal 13: To support separate power control, the following Option 1 is preferred.

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

Proposal 14: It can be considered to support both the unified TCI state framework and the UL spatial relation Info framework for SBFD (with the unified TCI state framework prioritized).

Configuration 1 vs. Configuration 2

Proposal 15: For SBFD-aware UEs, support of Configuration 1/Configuration 2 is subject to UE capability.

- Define separate capabilities for UL and DL.

  - FFS finer definition, e.g. separate capabilities for different channels/signals/ scheduling manner (static/dynamic etc.).

Proposal 16: The SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis by dedicated RRC signaling.

- For Type 1 CG PUSCH, whether configuration 1 or 2 is applied is configured per CG PUSCH configuration.

- For PUSCH scheduled/activated by DCI, whether configuration 1 or 2 is applied is configured per BWP in PUSCH-Config.

- For PDSCH, whether configuration 1 or 2 is applied is configured per BWP in PDSCH-Config.

- FFS CSI-RS, SRS, PUCCH.

Proposal 17: For each channel/signal for which Configuration 1/Configuration 2 is applicable, if the SBFD-aware UE is not configured with Configuration 2, Configuration 1 is applied.

PDSCH

(PDSCH TBS)

Proposal 18:

- For SPS PDSCH without repetition with Configuration 2,

  - For a PDSCH in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs only.

  - For a PDSCH in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

- For PDSCH repetition, including SPS PDSCH with repetition (for each SPS PDSCH), with Configuration 2

  - If the first repetition locates in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs only.

  - If the first repetition locates in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

(Multiple SPS PDSCHs in a slot)

Proposal 19: For SPS PDSCH, RAN1 to discuss how to enhance the rules for PDSCH selection in case of multiple PDSCHs in a slot.

CSI-RS

Proposal 20: Confirm the following Working Assumption with changes.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware Ues, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

CSI report

Proposal 21: For CSI report associated with periodic/semi-persistent CSI-RS, support Option B with similar restrictions as Option A.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols.

  - gNB configuration may not ensure that the CSI-RS associated with each sub-configuration is confined to either SBFD symbols or non-SBFD symbols only.

    - For the sub-configuration associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the sub-configuration associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

Proposal 22: For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured in CSI-ReportConfig.

PUSCH

(PUSCH repetition type A with available slot counting or TBoMS)

Proposal 23: For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, RAN1 to clarify,

- For CG PUSCH with repetition type A with available slot counting or TBoMS, UE postpones transmissions in the invalid symbol type.

Proposal 24: For repetition type A with available slot counting and TboMS, enhance the rules for available slot counting to resolve the collision with the invalid symbol type and to allow PUSCH transmissions overlapping with symbols indicated DL by tdd-UL-DL-ConfigurationCommon.

Proposal 25: For dynamically scheduled PUSCH transmissions in case of Configuration 1, define separate rules for determining the first slot and the other slots in the corresponding N∙K slots.

- Only the other slots are determined based on the invalid symbol type.

(SP-CSI on PUSCH)

Proposal 26: For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SP-CSI on PUSCH is SBFD symbols,

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

(RB offset)

Proposal 27: To clarify whether RB offset is applied when all PUSCHs are within SBFD symbols when the UE is provided with Configuration 2, RAN1 to discuss whether Configuration 2 applies to the case where all transmissions/receptions scheduled/activated by a single CG/DCI are within the same symbol type (SBFD symbols or non-SBFD symbols).

Proposal 28: When the UE is provided with Configuration 2 for PUSCH, one or multiple RB offsets are configured for determining starting PRB for PUSCH transmissions in SBFD symbols. Down select from the following equations.

- Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD

  - Negative values of RBoffsetSBFD are not precluded.

- Equation 1-E: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD+RBoffsetSBFD, max⁡(NUL SBsize -NPUSCHsize, 1))

(Frequency hopping)

Proposal 29: RAN1 to clarify,

- The new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols also apply to PUSCH for SP-CSI.

Proposal 30: The following two options are supported when FH offset/FH offset list is not provided for SBFD symbols.

- Option 1: FH is disabled for PUSCH transmissions in SBFD symbols.

- Option 2: FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

Proposal 31: When FH offset/FH offset list for SBFD symbols is provided but FH offset/FH offset list for SBFD symbols is not provided, use new RRC parameters to explicitly enable/disable frequency hopping for SBFD symbols. When FH for SBFD symbols is enabled, the FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

Proposal 32: For frequency hopping for PUSCH other than PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, remove -RBUL SB start in the equations (Alt.1).

- Note: discuss PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI in AI 9.3.2.

SRS

Proposal 33: To support separate SRS configurations, the following Option 1 is preferred, and it can be supported at least for the usages of “codebook” and “nonCodebook”.

- Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

Carrier aggregation

Proposal 34: Support carrier aggregation with a component carrier configured with SBFD operation.

- References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.