3GPP TSG RAN WG1#119		R1-2410199

Orlando, US, November 18th – 22nd, 2024

Source:	Panasonic

Title: 	Discussion on downlink and uplink channels and signals for A-IoT

Agenda Item:		9.4.2.3

Document for:	Discussion

# Introduction

3GPP has studied the ambient IoT (A-IoT) in Rel. 18 in RAN and decided to continue the study in RAN-WGs in Rel. 19 [1, 2]. A-IoT is a new type of device, with low complexity and low-power consumption, to be deployed in 5G NR. In this contribution, we present the physical channel designs and essential signals for enabling A-IoT in 5G networks.

# Discussions

In this section, we present our views on the physical channels and relevant signaling designs.

## R2D control information content and corresponding physical channel

The A-IoT device cannot be remained synchronized with the reader for a long duration. In addition, the A-IoT device may not monitor the R2D transmissions constantly and needs to be triggered beforehand. The reader may also use different transmission rates (e.g., using different chip durations) for the R2D transmissions. Hence, there is a need for a preamble before the start of a R2D transmission, indicating the start of the transmission and synchronization information. In RAN1#118bis, the following agreements were made:

Agreement For the clock-acquisition part of the R2D time acquisition signal, following is captured in the TR 38.769: - Clock-acquisition part is based on OOK without line coding and includes rising/falling edges, including at least two rising or at least two falling edges for the device to determine the OOK chip duration Agreement For the start-indicator part of the R2D time acquisition signal, for providing the start of the R2D transmission, following is captured in the TR 38.769: - Following options have been studied for the start-indicator part of the R2D time acquisition signal: - Option 1: ON-OFF transmission is considered based on energy/edge detection, and multiple alternatives have been studied including - Alt 1: A single ON-OFF transmission, i.e. one high-voltage transmission followed by one low-voltage transmission, where ON and OFF may have same or different durations - Alt 2: A multi-ON-OFF transmission, where different ON and different OFF may have same or different durations and different parts may have same or different duration - Option 2: ON-OFF sequence-based design is considered which consists of a pre-defined sequence for detection of start-indicator part based on digital correlation - For both the options, it is observed that a fixed duration for the start-indicator part can be considered, regardless of the value of M used for PRDCH transmissions. Agreement For the clock-acquisition part of the R2D time acquisition signal for OOK chip duration determination, following options are studied: - Option 1: Duration of the clock-acquisition part is variable for different M values, i.e. the duration becomes shorter with increasing value of M - Option 2: Duration of the clock-acquisition part is constant for different M values based on repetition, i.e. repetition factor is increased with increasing value of M to keep the duration constant - FFS: Whether/what restriction on M values for the clock-acquisition part Note: Other functionalities of clock-acquisition part is a separate discussion

For the reduced power consumption, the detection of the start indication should be feasible by simple analogue circuit. In addition, for the case CW and communication are in the same carrier, it should be distinguishable from CW wave. Then such requirement can be achieved by using a single ON-OFF transmission The differentiation of the start indication from the control/data information can be achieved, for instance, by using a different ON duration compared to the chip duration of line coding (e.g., Manchester coding) employed for control/data information. Note that the duration of ON/OFF start indication is not required to be proportional to the chip duration of control/data information because chip duration is detected by synchronization part after ON/OFF start indication part.

Proposal 1: A single ON/OFF transmission should be used as a start indication before R2D transmission. The ON/OFF start indication should be differentiated from the control/data information, e.g., using a different duration compared to the chip length.

The start indication is followed by a clock-acquisition part, to enable the reader deriving the chip duration, at least used for the control information. This could be achieved by using a known sequence. The duration of ON/OFF sequence could be equal to the duration of employed chip duration of the control, and possibly data information. The chip duration varies depending on the value of M. Using the same duration for ON/OFF sequence, by using repetition, could improve the synchronization accuracy and reduce the missed-detection probability. However, this would reduce the communication efficiency and increase the R2D transmission duration for large value of M. To balance the performance requirement and the communication efficiency, the clock-acquisition sequence should be designed considering the largest supported M value without repetition.

Proposal 2: A clock-acquisition sequence should be designed to meeting the communication requirements considering the largest supported M value without using repetition.

The reader may need to deliver different control information to the A-IoT devices. The possible control information includes system information, configuration of contention-based channel access, the indication of R2D data transmissions, triggering signal for D2R transmissions, and the D2R channel time boundaries. The control information may target a single or group of devices. To simplify the detection of target device(s), the control information should use a separate CRC. In addition, the control signal can be carried alone or along with the data information. To improve communication efficiency, different control information could be carried with a single transmission. This requires having the possibility of sending control information with variable sizes.

Proposal 3: The control information for R2D should be carried alone or along with the data. The control information includes system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R channel time boundaries.

Proposal 4: A separate CRC should be used for control information.

Proposal 5: The variable control information size should be supported.

The use of midamble was discussed for the purpose of synchronization. However, this requires dedicating a unique codeword for the midamble to be distinguished from the message. This could complicate encoding and decoding the message. In addition, the device could keep synchronization if the line coding, such as Manchester or PIE, is utilized.

Proposal 6: The midamble should not be considered for the R2D transmissions.

The A-IoT device also needs to determine the end of a R2D transmission. This could be achieved in two different ways. One solution is that the control information (decoded independently) indicates the size of the following data part. This requires that the A-IoT device be able to decode the control message and determine the size indication, while receiving the message information. Otherwise, the A-IoT device needs to receive signals for a duration corresponding to the maximum message size. The bitfield size for the size indication depends on the maximum message size and the granularity of message size. The other solution is to use a message end indication, e.g., a postamble. The postamble should be a known sequence appearing after the data information. The benefit of using the postamble is that the A-IoT device does not require to decode the control information before receiving the message part. The demerit can be the data information cannot contain the same sequence with postamble. In order to avoid using the same sequence with postamble, some escape sequence may be required, which is a variable payload size depending on the information sequence. To decide which solution to be used, the overhead of size indication and the postamble should be further studied. We prefer no limitation on the control/data information sequence in order to avoid variable payload size using escape sequence.

Proposal 7: The message size could be determined either using a message size indication in the control information or a postamble. The overhead of message size indication and postamble should be further studied.

Proposal 8: The message size should be deterministic by the upper layer information. Variable message size depending on the information sequence using escape sequence should be avoided.

## D2R control information content and corresponding physical channel

The A-IoT device also needs to deliver different types of control information to the reader. Some of the possible control information includes the contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report. Such control information could be carried alone or along with the data information. In this regard, a single transmission may contain only control information, only data information, and a combination of control and data information. For a combined control and data transmissions, a joint CRC could be considered in order to reduce the overhead compared with separate CRCs. This however might require using a postamble to indicate the end of the message. In case separate CRCs are utilized of control and data information, the message size could be derived from the control information, so there is no need to utilize the postamble. Similar to R2D, the CRC design should be discussed after the content of control information is determined.

Proposal 9: The control information for D2R should be carried alone or along with the data information. The control information includes contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report.

Proposal 10: The CRC design of D2R should be discussed after the content of control information is determined.

# Conclusions

In this contribution, we provided our views on physical channels and relevant signals for A-IoT. We made following proposals:

Proposal 1: A single ON/OFF transmission should be used as a start indication before R2D transmission. The ON/OFF start indication should be differentiated from the control/data information, e.g., using a different duration compared to the chip length.

Proposal 2: A clock-acquisition sequence should be designed to meeting the communication requirements considering the largest supported M value without using repetition.

Proposal 3: The control information for R2D should be carried alone or along with the data. The control information includes system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R channel time boundaries.

Proposal 4: A separate CRC should be used for control information.

Proposal 5: The variable control information size should be supported.

Proposal 6: The midamble should not be considered for the R2D transmissions.

Proposal 7: The message size could be determined either using a message size indication in the control information or a postamble. The overhead of message size indication and postamble should be further studied.

Proposal 8: The message size should be deterministic by the upper layer information. Variable message size depending on the information sequence using escape sequence should be avoided.

Proposal 9: The control information for D2R should be carried alone or along with the data information. The control information includes contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report.

Proposal 10: The CRC design of D2R should be discussed after the content of control information is determined.

# Reference

[1] 	TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN,” V18.0.0.

[2] 	RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” RAN#103.

[3]	R1-2407555, “FL summary#3 on downlink and uplink channel/signal aspects.”