3GPP TSG RAN WG1 #119			      R1-2410384

Orlando, US, November 18th – 22nd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	9.2.4

Document for: 	Discussion and Decision

- Introduction

In RAN#105 meeting, Rel-19 WID of ‘NR MIMO Phase 5’ was revised [1]. The detailed objectives for RAN1 are as follows. In this contribution, we discuss the enhancements for asymmetric DL sTRP/UL mTRP scenarios.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

- Discussion

2.1 LS reply

In [2], the following question is asked from RAN4.

RAN4 is discussing the ‘asymmetric DL sTRP/UL mTRP deployment scenario’ objective of the WI for FR2 deployments. In one proposed scenario, the UL TRP(s) have full UL reception functionality but downlink functionality limited to SSB transmission. Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.

We believe it is appropriate to reply to the question from UE perspective, because gNB’s hardware assumption is not a target of 3GPP discussion. Hence, the discussion point is whether UE expects to receive SSB from UL TRP(s) or not.

Rel.19 asymmetric DL sTRP/UL mTRP deployment scenario has the following 3 objectives:

- 1) PL-offset configuration

- 2) Two CL-PC adjustment states for SRS separate from PUSCH.

- 3) Two TA without coresetPoolIndex

For the objective 1), it is clear that UE expects to receive SSB from DL TRP only, because it is the reason why PL-offset configuration is necessary. For the objective 2) and 3), UE may or may not receive SSB from UL TRP. Hence, we propose to reply as follows:

Proposal 1

- Reply to Question1 in RAN4 LS (R1-2409353) as follows:

  - If UE is configured with PL-offset for joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s).

  - Else, UE may expect to receive SSB from UL TRP(s).

2.2 Two closed-loop PC adjustment states for SRS, both separate from PUSCH

DCI format 1_1 to indicate TPC command.

In RAN1#118bis, the following agreement was made.

Agreement Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

Regarding the working assumption, since we already agreed to introduce 1-bit CL-PC indicator in DCI format 2_3, the straightforward way is to reuse the same mechanism for DCI format 1_1. The existing TPC command field in DCI format 1_1 with data is used for TPC command indication for PUCCH, and additional TPC command is necessary to indicate TPC command for one of the two SRS CL-PC adjustment states. Hence, we support to confirm the working assumption.

Proposal 2

- Confirm the following working assumption in RAN1#118bis.

  - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

    - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

  - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

    - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

For Rel.19 two CL-PC adjustment states for SRS both separate from PUSCH, we have agreed to support DCI format 2_3 and DCI format 1_1 to indicate TPC command. Whether both DCI formats can be configured simultaneously to indicate the TPC command should be discussed.

Proposal 3

- Both DCI format 2_3 and DCI format 1_1 can be configured simultaneously in a BWP/CC to indicate TPC command for Rel.19 two CL-PC adjustment states for SRS, both separate from PUSCH.

2.3 PLoffset configurations for PL calculation to UL TRP(s), when the PL-RS is from DL sTRP

PRACH transmission to UL TRP in FR2

In RAN#105, two TA without coresetPoolIndex for asymmetric DL sTRP/UL mTRP deployment scenarios was included in Rel.19 MIMO. To support two TA in FR2 properly, it is necessary to transmit PDCCH order PRACH to UL TRP, and hence, support of PLoffset for PDCCH order PRACH in FR2 becomes essential.

In the previous meeting, some companies claimed that UE cannot decide appropriate tx beam of PDCCH order PRACH because there is no DL RS transmitted from UL TRP. To solve this issue, one option is to specify tx beam of PDCCH order PRACH (e.g. indicate SRS resource as QCL source RS of UL TCI state for PRACH). However, some companies had strong preference to keep tx beam of PDCCH order PRACH unspecified. Since TCI state is indicated for PDCCH order PRACH to indicate PLoffset, we believe UE can use the TCI state as a guidance to determine tx beam of PDCCH order PRACH.

Proposal 4

- Support indicating PLoffset to PDCCH-order PRACH transmission in FR2.

  - Note: Tx beam of PDCCH order PRACH is unspecified. Indicated TCI state for PDCCH order PRACH for the indication of PLoffset can be used as a guidance for UE to determine tx beam of PDCCH order PRACH.

Triggering condition of PHR

In RAN1#118 meeting, the following agreement was made.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

When PLoffset is applied to PUSCH, UE reduces transmission power of PUSCH based on the PLoffset. Hence, PLoffset impacts PHR calculation. The triggering condition of PHR is specified as the following and PHR is triggered when the “pathloss” changes more than phr-Tx-PowerFactorChange dB. If “pathloss” in the current specification is intended for “UL pathloss”, it should be updated to “PL – PLoffset” because actual UL pathloss is “PL – PLoffset” in asymmetric DL sTRP/UL mTRP scenarios. On the other hand, if “pathloss” in the current specification is intended for “DL pathloss”, there is no need to update. Since RAN2 specified the condition of PHR triggering, it is better RAN2 to discuss this issue.

TS38.321 v18.2.0: 5.4.6 Power Headroom Reporting A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

Proposal 5

- For the triggering condition of Type 1 PHR, send LS to RAN2 to discuss whether “the path loss” in the following text should be updated to “PL – PLoffset” or not.

  - Clause 5.4.6 in TS38.321: phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

- Note: If “pathloss” in the current specification is intended for “UL pathloss”, it should be updated to “PL – PLoffset” because it is actual “UL pathloss” in asymmetric DL sTRP/UL mTRP scenarios. On the other hand, if “pathloss” in the current specification is intended for “DL pathloss”, there is no need to update.

2.3 Two TA for single DCI based multi-TRP/panel or single-TRP

In RAN1#118bis, the following agreement was made.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Regarding the first FFS, UE autonomous TA adjustment was discussed in Rel.18 two TA. However, it was not agreed and not specified in any of RAN1 spec. We notice that UE autonomous TA adjustment is captured in TS38.133, however, whether and how to update the TS38.133 can be discussed in RAN4. Hence, we believe no additional discussion is needed in RAN1.

Regarding the second FFS, some companies claimed even if UE supports “UE capability of “Overlapping UL transmission reduction”, it does not make sense to reduce transmission occasion if intra-slot TDM PUSCH type-B repetition is configured, because each transmission occasion is up to 7-symbol. Hence, they claimed it is necessary to preclude intra-slot TDM PUSCH type-B repetition transmission, if two consecutive repetitions associated with different TAGs are overlapped. In Rel.18 two TA for multi-DCI multi-TRP, there was no such spec. restriction that each transmission occasion must be larger than 7-symbol if two consecutive repetitions associated with different TAGs are overlapped. Hence, we don’t understand why we need to restrict that intra-slot TDM PUSCH type-B repetition cannot be supported for two TA in Rel.19.

Proposal 6

- For two TA for the asymmetric DL sTRP/UL mTRP deployment scenarios, do not support the following FFS:

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

  - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

In Rel.18 two TA for mDCI mTRP, both intra-cell and inter-cell scenarios are supported, and PL-RS of PDCCH order PRACH can be associated with serving cell or additional cell. In Rel.17 unified TCI framework for sTRP, ICBM (Inter-Cell Beam Management) is supported, and an indicated TCI state can be associated with serving cell or additional cell.

For the asymmetric DL sTRP/UL mTRP deployment scenario in Rel.19, it was agreed that Rel.17 unified TCI framework for sTRP can be used for ICBM.

Conclusion (RAN1#117) For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

For two TA for the asymmetric DL sTRP/UL mTRP deployment scenario in Rel.19, both intra-cell and inter-cell scenario can be supported for Rel.17 unified TCI framework for sTRP. Since Rel.18 two TA already supports both intra-cell and inter-cell scenarios, no additional specification impact is necessary.

Proposal 7

- Support both intra-cell and inter-cell for two TA for the asymmetric DL sTRP/UL mTRP deployment scenarios.

  - Note: For inter-cell scenario, reuse Rel.18 two TA spec. that PDCCH order PRACH can be associated with serving cell or additional cell.

- Conclusion

In this contribution, we discussed the enhancements for asymmetric DL sTRP/UL mTRP scenarios. Based on the discussion, we made following observations and proposals.

Proposal 1

- Reply to Question1 in RAN4 LS (R1-2409353) as follows:

  - If UE is configured with PL-offset for joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s).

  - Else, UE may expect to receive SSB from UL TRP(s).

Proposal 2

- Confirm the following working assumption in RAN1#118bis.

  - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

    - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

  - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

    - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

Proposal 3

- Both DCI format 2_3 and DCI format 1_1 can be configured simultaneously in a BWP/CC to indicate TPC command for Rel.19 two CL-PC adjustment states for SRS, both separate from PUSCH.

Proposal 4

- Support indicating PLoffset to PDCCH-order PRACH transmission in FR2.

  - Note: Tx beam of PDCCH order PRACH is unspecified. Indicated TCI state for PDCCH order PRACH for the indication of PLoffset can be used as a guidance for UE to determine tx beam of PDCCH order PRACH.

Proposal 5

- For the triggering condition of Type 1 PHR, send LS to RAN2 to discuss whether “the path loss” in the following text should be updated to “PL – PLoffset” or not.

  - Clause 5.4.6 in TS38.321: phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

- Note: If “pathloss” in the current specification is intended for “UL pathloss”, it should be updated to “PL – PLoffset” because it is actual “UL pathloss” in asymmetric DL sTRP/UL mTRP scenarios. On the other hand, if “pathloss” in the current specification is intended for “DL pathloss”, there is no need to update.

Proposal 6

- For two TA for the asymmetric DL sTRP/UL mTRP deployment scenarios, do not support the following FFS:

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

  - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Proposal 7

- Support both intra-cell and inter-cell for two TA for the asymmetric DL sTRP/UL mTRP deployment scenarios.

  - Note: For inter-cell scenario, reuse Rel.18 two TA spec. that PDCCH order PRACH can be associated with serving cell or additional cell.

- Reference

- 3GPP RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105, Sep. 2024.

- 3GPP R1-2409353, “LS on UE RF issues for Rel-19 MIMO enhancement”, RAN1#119, Nov. 2024.