3GPP TSG RAN WG1 #119   		                                                                       R1-2410485 Orlando, USA, November 18th – 22nd, 2024

Agenda item:	9.5.3

Source: 	Qualcomm Incorporated

Title: 	Adaptation of common channel transmissions

Document for:	Discussion/Decision

# Introduction

The contribution provides our views on adaptation of common signal/channel transmissions that was one of the objectives in [1] for NR network energy savings enhancements as follows:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

# Adaptation of SSB in Time Domain

RAN1#118 and RAN1#118b made the following agreement and conclusion related to adaptation of SSB in time domain. In RAN1#118, It was agreed that in the PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported. It was also agreed that SSB adaptation in SCell is supported for the non CD-SSB. In RAN1#118b, it was concluded that there is no consensus on the support of adaptation of SSB for the idle mode UEs in Rel-19.

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

In the current specification for Pcell, CD-SSB is always transmitted and NCD-SSB can be configured to support redcap in the BWP without CD-SSB. If NCD-SSB is transmitted, its periodicity and timing match those of CD-SSB (see NonCellDeginingSSB-r17 from TS38.331). This means that the gNB will be in an active state to transmit the CD-SSB when it is transmitting the NCD-SSB. If at a given time instance the network doesn’t transmit the NCD-SSB in the same time it is transmitting CD-SSB, the energy saving gain will be marginal. Hence, energy saving gain of time domain adaptation of NCD-SSB in Pcell is marginal.

Observation 1: NCD-SSB can be additionally configured for supporting redcap UEs. Furthermore, the NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

Observation 2: Time domain adaptation of NCD-SSB in Pcell provides marginal network energy saving gain.

For the CD-SSB that is not on sync-raster, it is not clear what is the use case of this SSB in the practical deployment.

Observation 3: The use case of having CD-SSB that is not on sync raster in Pcell/Scell is unclear.

It should be noted that Pcell/Scell is from UE perspective. While adapting CD-SSB in SCell of a Rel-19 connected mode UE may be fine for this UE, such adaptation would have negative impact to other UEs that are being in idle/inactive mode or performing initial cell selection. The CD-SSB has associated SIB1 and is transmitted on sync raster in a cell supporting initial access/idle mode operation. It is used for initial cell selection for Idle/Inactive mode UEs where the UE assumes SSBs are being sent every 20ms therefore, adapting CD-SSB to have a periodicity longer than 20ms would have a negative impact on legacy UE’s cell search. Moreover, sending CD-SSBs with associated SIB1 with larger periodicity would impact the initial access latency and idle mode operation.

Observation 4: Adaptation of SSB in time domain for CD-SSB on sync-raster in SCell has a negative impact on idle/inactive UEs (including legacy UEs and R19 UEs) in terms of cell search, initial access latency and idle mode operation.

RAN1#117 agreed on the configuration of on-demand SSB for Scell operation with an FFS on whether more than one on-demand SSB configurations can be configured for the cell to UE. In addition, for time-domain SSB adaptation, it was noted that using Option 2 in which adaptation based on two SSB configurations [where up to two configurations can be active] to realize the agreed Option 1 is not precluded.

Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

In our view, SSB adaptation is similar to on-demand SSB case#2. For an example illustrated in Figure 1, the always-on SSB and OD-SSB transmission correspond to the transmission of SSB with a periodicity of 20ms which can be interpreted as an updated SSB periodicity for the always-on SSB. In this case, ssb-PositionsInBurst and ss-PBCH-BlockPower should be identical between the OD-SSB and always-on SSB. Moreover, the frequency parameter absoluteFrequencySSB provided in DownlinkConfigCommon can be used for both always-on SSB and OD-SSB.

Figure 1: Always-on SSB and OD-SSB on the same frequency when both SSBs are NCD-SSB

This example shows that the agreed Option 1 is similar to the on-demand SSB transmission for SSB transmission case #2 in which the UE is configured with multiple configurations of on-demand SSB burst periodicity (which is related to FFS on whether more than one on-demand SSB configurations can be configured for the cell to UE). Now the time-domain SSB adaptation can be done with the indication of on-demand SSB transmission e.g., the indication not only indicates the availability of SSB transmission but also updates the periodicity of the SSB burst.

It was agreed in RAN1#117 to support MAC CE based signaling to indicate on-demand SSB transmission on the cell. Given the commonality between on-demand SSB and adaptation of SSB burst periodicity, a MAC CE should be sufficient to also indicate the adaptation of SSB burst periodicity. Furthermore, it is desirable to enable a unified MAC-CE framework for on-demand SSB and adaptation of SSB burst periodicity for network and UE implementation efficiency.

Observation 5: Case #2 under agenda 9.5.1 can achieve an adaptation of SSB periodicity for SCell.

Proposal 1: Use Case #2 under agenda 9.5.1 to support adaptation of SSB periodicity for SCell.

# Adaptation of PRACH in Time Domain

In this section, we present our views on the adaptation of PRACH in time domain.

## Configuration of additional PRACH resources

RAN1#117 has made the following agreements related to the configuration of additional PRACH resources:

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

One of the main goals of this work item is to have no/minimal impact to the legacy UE. Therefore, it is important to keep the legacy PRACH resources the same with no impact from the additional PRACH resources. No impact would be achieved by not allowing time and frequency overlap, even partial, between the legacy and additional ROs. This goal can be realized by the proper definition of what an additional PRACH resource is. Since an additional PRACH resource is only complementary to the legacy PRACH resources, the additional PRACH resources should not override or change the definition of the legacy PRACH resources. Therefore, the additional PRACH resources should be defined as the set of PRACH resources defined by the additional PRACH configuration parameter(s) (e.g., for PRACH configuration periodicity or PRACH configuration index) that do not overlap, even partially in time and frequency with any legacy PRACH resources.

Proposal 2: Define the additional PRACH resources as the PRACH resources that are derived based on the additional PRACH configuration parameter(s) and do not overlap in both time and frequency with PRACH resources configured for legacy UEs.

Further to the above discussion the following agreements were made in RAN1#118b.

Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

First, it is important to clarify that the parameter y in Opt1-2a and Opt1-2b serves the same purpose of applying an offset on the frame level. However, the PRACH configuration index table in TS38.211 shows that the parameter y can take values 0 and 1 only for FR1 while taking values of 0, 1 and 2 for FR2. It is important to clarify whether the value of y in Opt1-2a and Opt1-2b is restricted to the existing values in the PRACH configuration index table or it can go beyond these values since other values would extend the range of the possible frame-level offsets.

Proposal 3: RAN1 to clarify the range of values that the parameter y in Opt1-2a and Opt1-2b can take.

It is our view that for Opt1-4, additional timing offset at the frame-level can be achieved by Opt1-2a or Opt1-2b without an overlap with the legacy resources, which are illustrated in the following examples:

- Example 1 (PRACH configuration index for additional ROs is different from the index for legacy ROs): The configuration index for the legacy PRACH resources is 2 while the additional PRACH resources can be achieved by configuring a PRACH configuration index 21 with value of (x, y) updated to (2,0).

Figure 2: Opt1-4 is achieved by Opt 1-2a without overlapping between legacy and additional PRACH resources

- Example 2 (PRACH configuration index for additional ROs is identical to the index for legacy ROs): The configuration index for the legacy PRACH resources is 2 while the additional PRACH resources can be achieved by configuring a PRACH configuration index 2 with value of (x, y) updated to (4, 0).

Figure 3: Opt1-4 is achieved by Opt 1-2b without overlapping between legacy and additional PRACH resources

It should also be noted that Opt 1-2b is just a special case of Opt 1-2a. Hence, adopting Opt 1-2a not only covers configurations that Opt 1-2b can offer but also provides many other configurations for network implementation flexibility.

Observation 6: Configuration of additional PRACH resources based on additional timing offset can be achieved by adapting value of (x, y) or by adapting value of y only.

Observation 7: Opt 1-2a not only covers configurations that Opt 1-2b can offer but also provides many other configurations for network implementation flexibility.

Proposal 4: The additional PRACH resources are configured based on an additional value of (x, y) (i.e., Opt 1-2a).

## Adaptation of additional PRACH resources

The following agreement and working assumptions were made in RAN1#118 and RAN1#118b respectively:

Agreement For DCI-based adaptation for additional PRACH resources, - Select from the following DCI format(s) to carry the adaptation indication. - DCI format 1_0 - DCI format 2_7 - DCI format 2_9 - FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

The paging DCI has a short message indicator with the following possible values from TS 38.212.

Figure 3: Short Message indicator in DCI1_0 with CRC scrambled with P-RNTI

If the short message indicator indicates short message such as the values 10 and 11, the short message has 8 bits that are interpreted as follows (from TS 38.331):

As can be seen, bits 5-8 are not used by the legacy UE. Therefore, they can be used by the network to indicate to the Rel-19 UE the adaptation of the additional PRACH resources.

Proposal 5: For adaptation indication using the paging DCI, reuse bits 5-8 in the short message to indicate the adaptation of the additional PRACH resources.

For a UE that does not support DCI format 2_7, it is required to monitor paging DCI. On the other hand, for a UE supporting DCI format 2_7, it may skip monitoring paging DCI. Therefore, it is reasonable to use both paging DCI and DCI format 2_7 for the adaptation otherwise the UE supporting DCI format 2_7 would have to monitor both PEI and paging DCI to monitor the adaptation indication.

Proposal 6: In addition to paging DCI, consider supporting adaptation based on DCI format 2_7 for UE supporting PEI.

In addition to the above discussion, it is our view that PDCCH order DCI can also be used to indicate the adaptation of additional PRACH resources for the Rel-19 UEs in RRC connected mode. Such indication can be useful to dedicate activation of additional PRACH resources for a specific UE excluding the idle/inactive mode UEs.

Proposal 7: For the adaptation indication of additional PRACH resources, in addition to supporting the adaptation indication via the paging DCI and DCI format 2_7, DCI format 1-0 with C-RNTI for PDCCH-order RACH is also supported to indicate adaptation of PRACH resources for the connected mode UEs.

Further to the above discussion, the following agreement was made in RAN1#118 in which multiple alternatives were made related to how the DCI would carry the adaptation.

Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated. - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available. - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period - FFS: details

Alt 1 in the agreement above would require single bit to indicate the availability of the additional PRACH resources. While Alt 2 requires more bits for the indication of availability of a subset of the PRACH resources, it also provides more flexibility to the network specially that there is a single additional PRACH configuration and enabling the adaptation of a subset of the PRACH resources would provide a greater degree of freedom to the network optimization.

Proposal 8: Support adaptation of PRACH resources by indicating the availability of a subset of the additional PRACH resources.

To determine the time granularity that determines the subset of additional PRACH resources, it is important first to define what the SSB-to-RO mapping cycle, association period and association pattern period mean. In our view, there are two options listed and illustrated with an example (assuming 4 SSB indices) as follows:

- Option 1: Additional PRACH resources don’t have a separate association period

  - SSB-RO mapping is reset in every association period which is defined based on the legacy PRACH resources.

Figure 4 Two options illustrating how to define the association period of the additional ROs

Option 2: Additional PRACH resources have a separate association period

In each option, the definition of the SSB-RO mapping cycle, association period and association pattern period could be different.

Proposal 9: Discuss the definition of the association period for the additional PRACH resources.

Further to the above discussion, in our view, the longer the duration that determines the granularity of the subset of the additional PRACH resources, the more flexibility that the network has but also the longer the bit sequence needed for the indication. However, since the association pattern period may contain multiple association periods which in turn may contain one or more SSB-to-RO mapping cycles, defining the subset across the association pattern period may require a very long sequence of bits for the indication.

Proposal 10: Support DCI adaptation by indicating the availability of a subset of the additional PRACH resources where this set is defined within an SSB-to-RO mapping cycle level or PRACH association period level.

Another question to be asked is once the network indicates an adaptation of the additional PRACH resources, for how long would these resources be active for? To minimize signaling overhead, the availability indication can be valid for a given window of time after which the UEs implicitly assume that the additional PRACH resources are not available anymore.

Proposal 11: The adaptation of additional PRACH resources is timer-based.

## SSB-RO mapping and association period

The following agreements related to the SSB-RO mapping were made in RAN1#117 meeting

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

In RAN1#117, it was agreed that SSB-RO mapping for the additional PRACH resources is going to be separate from the legacy PRACH resources and will follow the current specs order. In this discussion, we show that if an offset is applied to the first SSB index to be mapped to the additional ROs, latency improvements can be achieved.

First, we define the maximum latency as the worst-case delay between a UE arrival (UE ready to perform preamble transmission) and the availability of a PRACH occasion that maps to the required SSB index.

The following figure shows two examples for mapping 4 SSB indices to the legacy PRACH resources in blue and the additional PRACH resources in pink. Both timelines follow the current specs mapping order for the legacy ROs. For the mapping to the additional ROs, the first timeline shows that SSBs mapped to the ROs starting from SSB index-0 and the second timeline shows SSBs mapped to the RO starting from SSB index-2.

Figure 5 SSB offset to the SSB-RO mapping for the additional PRACH resources

As noted in the figure, the worst-case delay/latency for a UE trying to transmit preamble in a RO that maps to SSB-1 is the longest possible time until a RO mapped to SSB-1 exists. Please note that similar latency can be observed for other SSB indices.

In the second line, due to the offset in SSB indices, the mapping is kind of interlaced between the legacy ROs and additional ROs which gives an advantage of reducing the worst-case delay/latency for RO mapped to SSB-1 compared to the worst-case delay/latency in the first timeline. As an example, the worst-case latency in the first timeline as shown is 35 mseconds while the worst-case latency in the second timeline is only 25 mseconds which is a reduction of about 28%.

In order to have a more detailed perspective, one can look at the latency itself (not just the worst-case scenario). Since the latency itself is a variable that depends on the UE arrival, assuming that UE arrival is uniform, and assuming that the UE needs to transmit a preamble in a RO mapped to SSB index-1, the latency distribution of each scenario can be simulated and is shown in the following figure.

Figure 6: CDF of the PRACH latency which is defined as the delay between a UE arrival and availability of an RO

Observation 8: Applying an offset to the SSB indices that are mapped to the additional ROs result in a more uniform distribution of SSBs across all ROs which may, on-average, reduce the PRACH latency defined as the delay between a UE arrival and RO availability as well as reducing the worst-case PRACH transmission latency by about 28%.

Proposal 12: Discuss applying an offset to the SSB indices mapped to the additional PRACH resources.

- FFS: whether the offset value x is part of the configuration or the adaptation of the additional PRACH configuration.

The association period is defined based on the available PRACH occasions and their mapping to SSBs. Upon adaptation of additional PRACH resources, the number of available PRACH occasions may change which may impact the association period. However, since the activation/deactivation is opportunistic, changing the association period relative to the activation/deactivation indication may lead to additional complexities that doesn’t have a strong motivation. Therefore, it is beneficial to keep the association period independent from the activation/deactivation of the additional PRACH resources.

Proposal 13: The association period definition for the R19 UEs is based on SSB-RO mapping in PRACH resources for legacy UEs.

## Considerations on the impact to legacy UE

One of the main objectives of this work item is to minimize the impact on legacy UE. While Rel-19 UE can be indicated with available additional PRACH resources, legacy UEs can only use the legacy PRACH resources.

As the number of UEs transmitting in the same RO increases, the interference among UEs increases even if they use different preambles and the probability of collision also increases. In order to minimize the impact to the legacy UE, when additional PRACH resources are available for transmission, the Rel-19 UE should prioritize the transmission over the additional PRACH resources.

Proposal 14: Rel-19 UE should prioritize the transmission over the additional PRACH resources when they are indicated as available for transmission.

Moreover, if the Rel-19 UE uses the legacy PRACH resources, it may reduce its interference by transmitting with a different Tx power.

Proposal 15: Discuss Rel-19 UE PRACH power control when additional PRACH resources are available for transmission.

# Conclusion

The contribution has discussed our views on adaptation of signal/channel transmissions. In particular, we make the following observations and proposals:

SSB adaptation in time domain

Observation 1: NCD-SSB can be additionally configured for supporting redcap UEs. Furthermore, the NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

Observation 2: Time domain adaptation of NCD-SSB in Pcell provides marginal network energy saving gain.

Observation 3: The use case of having CD-SSB that is not on sync raster in Pcell/Scell is unclear.

Observation 4: Adaptation of SSB in time domain for CD-SSB on sync-raster in SCell has a negative impact on idle/inactive UEs (including legacy UEs and R19 UEs) in terms of cell search, initial access latency and idle mode operation.

Observation 6: Case #2 under agenda 9.5.1 can achieve an adaptation of SSB periodicity for SCell.

Proposal 1: Use Case #2 under agenda 9.5.1 to support adaptation of SSB periodicity for SCell.

PRACH adaptation in time domain

Observation 5: Configuration of additional PRACH resources based on additional timing offset at frame-level can be achieved by additional value(s) of y.

Observation 6: Configuration of additional PRACH resources based on additional timing offset can be achieved by adapting value of (x, y) or by adapting value of y only.

Observation 7: Opt 1-2a not only covers configurations that Opt 1-2b can offer but also provides many other configurations for network implementation flexibility.

Observation 8: Applying an offset to the SSB indices that are mapped to the additional ROs result in a more uniform distribution of SSBs across all ROs which may, on-average, reduce the PRACH latency defined as the delay between a UE arrival and RO availability as well as reducing the worst-case PRACH transmission latency by about 28%.

Proposal 2: Define the additional PRACH resources as the PRACH resources that are derived based on the additional PRACH configuration parameter(s) and do not overlap in both time and frequency with PRACH resources configured for legacy UEs.

Proposal 3: RAN1 to clarify the range of values that the parameter y in Opt1-2a and Opt1-2b can take.

Proposal 4: The additional PRACH resources are configured based on an additional value of (x, y) (i.e., Opt 1-2a).

Proposal 5: For adaptation indication using the paging DCI, reuse bits 5-8 in the short message to indicate the adaptation of the additional PRACH resources.

Proposal 6: In addition to paging DCI, consider supporting adaptation based on DCI format 2_7 for UE supporting PEI.

Proposal 7: For the adaptation indication of additional PRACH resources, in addition to supporting the adaptation indication via the paging DCI and DCI2_7, PDCCH order is also supported to indicate adaptation of PRACH resources for the connected mode UEs.

Proposal 8: Support adaptation of PRACH resources by indicating the availability of a subset of the additional PRACH resources.

Proposal 9: Discuss the definition of the association period for the additional PRACH resources.

Proposal 10: Support DCI adaptation by indicating the availability of a subset of the additional PRACH resources where this set is defined within an SSB-to-RO mapping cycle level or PRACH association period level.

Proposal 11: The adaptation of additional PRACH resources is timer-based.

Proposal 12: Discuss applying an offset to the SSB indices mapped to the additional PRACH resources.

- FFS: whether the offset value x is part of the configuration or the adaptation of the additional PRACH configuration.

Proposal 13: The association period definition for the R19 UEs is based on SSB-RO mapping in PRACH resources for legacy UEs.

Proposal 14: Rel-19 UE should prioritize the transmission over the additional PRACH resources when they are indicated as available for transmission.

Proposal 15: Discuss Rel-19 UE PRACH power control when additional PRACH resources are available for transmission.

# Reference

RP-242354, Enhancements of network energy savings for NR

TS-38.211, 5G; NR; Physical channels and modulation

TS-38.213, 5G; NR; Physical layer procedures for control