3GPP TSG RAN WG1#118			    R1-2406044

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.2.1

Source: 	Moderator (ZTE)

Title: 	Moderator Summary #1 on UE-initiated/event-driven beam management

Document for:	Discussion and Decision

- Introduction

In RAN#102, the Rel-19 WID on NR MIMO phase 5 is approved. In the approved WID, UE-initiated/event-driven beam management is a part of the RAN1 objectives as follows:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

- Plan

Per RAN1#117 outcome, the following issues are prioritized for this meeting:

|  | Issue | Topics |
| --- | --- | --- |
| 1 | Trigger-event detection | Stabilize RS configuration for new beams for Event-2, e.g., down-selection from Option 1~3 for explicit RS configuration, etc. |
| 2 |  | Left-over for RS measurement for current beam, e.g., down-selection from Option-1~ 4 when only one TRS is configured in the indicated TCI, selection of scheme-1 and Scheme-2 (by RRC or an implicit manner), etc. |
| 3 |  | Further evaluate candidate Event 1 and Event7a/7b, and then down-selection or combination from the candidate. |
| 4 | UL signaling content(s) | Details on L1-RSRP report format (Option-3) depending on Event-2, e.g., differential L1-RSRP, candidate values of ‘N’, etc. |
| 5 |  | Additional content(s), e.g., L1-SINR |
| 6 | UL signaling medium/container | Details on 1-bit first PUCCH for Mode-A and Mode-B, e.g., resource configuration (dedicated RRC signaling), multiplexing with other UCIs, etc. |
| 7 |  | Details on Step-2&3 in Mode-A, e.g., mechanism of triggering UEI beam report. |
| 8 |  | Details on Step-2 in Mode-B, e.g., second channel and the related resource pre-configuration. |
| 9 | Other procedure as required | Activation-latency reduction, etc |

Then, based on the outcome of pre-RAN1#118 discussion [1] and the contributions from companies [2]-[39], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – Trigger-event detection

  - Issue 2 – UL signaling content(s)

  - Issue 3 – UL signaling medium/container

  - Issue 4 – Other procedure as required

- Observations and recommended proposals based on the summary of companies’ views

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Apple | Hong He | hhe5@apple.com |
| CATT | Jiayi Yang | yangjiayi@catt.cn |
| CEWiT | Pardh | pardhasarathy.j@cewit.org.in |
| CMCC | Yan LI | liyanwx@chinamobile.com |
| Ericsson | Claes Tidestav | claes.tidestav@ericsson.com |
| ETRI | Cheulsoon Kim | cs.kim@etri.re.kr |
| Fujitsu | David | wangguotong@fujitsu.com |
| FUTUREWEI | Weimin Xiao | weimin.xiao@futurewei.com |
| FUTUREWEI | Zhigang Rong | zrong@futurewei.com |
| Google | Alex Liou | alexliou@google.com |
| Huawei/HiSilicon | Keyvan Zarifi | Keyvan.zarifi@huawei.com |
| Huawei/HiSilicon | Fanbo | asen.fanbo@huawei.com |
| HONOR | Guozeng Zheng | zhengguozeng@honor.com |
| Intel | Gary | gang.xiong@intel.com |
| InterDigital | Jonghyun Park | jonghyun.park@interdigital.com |
| KDDI | Shunsuke Kamiwatari | sh-kamiwatari@kddi.com |
| LG | Hyungtae Kim | ht.kim@lge.com |
| MediaTek | Rebecca Chen | rebecca.chen@mediatek.com |
| Meta | Avik Sengupta | aviksg@meta.com |
| NEC | Peng GUAN | guan_peng@nec.cn |
| NEC | Yukai GAO | gao_yukai@nec.cn |
| NICT | Kenichi Takizawa | takizawa@nict.go.jp |
| Nokia | Mihai Enescu | mihai.enescu@nokia.com |
| Nokia | Youngsoo Yuk | youngsoo.yuk@nokia.com |
| NTT DOCOMO | Mamoru Okumura | Mamoru.okumura.nz@nttdocomo.com |
| OPPO | Li Guo | guoli@oppo.com |
| Panasonic | Khalid Zeineddine | khalid.zeineddine@eu.panasonic.com |
| Qualcomm | Wooseok Nam | wnam@qti.qualcomm.com |
| Ruijie Networks | Ke Zhong | zhongke@ruijie.com.cn |
| Samsung | Dalin Zhu | dalin.zhu@samsung.com |
| Samsung | Sa Zhang | sa.zhang@samsung.com |
| Sharp | Taka | fukui.takahisa@sharp.co.jp |
| Sony | Tingting Fan | Emme.Fan@sony.com |
| Spreadtrum | Yu Yang | yu.yang2@unisoc.com |
| vivo | Rakesh Tamrakar | rakesh@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Yang Zhang | zhang.yang220@zte.com.cn |

- Discussion

## Issue 1 – Trigger-event detection

Table 1-1 Summary for Issue 1

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 1.1 | RS config. for new beam for Event-2 | [117] Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list [117] Agreement Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state FL observation: Option-1 (28): ZTE, OPPO, E///, Qualcomm, ETRI, xiaomi, MediaTek, NEC (MAC-CE), DOCOMO, Nokia, vivo (MAC-CE), Fujitsu, Apple (MAC-CE), Lenovo, CMCC, Futurewei, Meta, Sharp, CATT, CEWiT, Spreadtrum, LG, Huawei/HiSi, KDDI, Panasonic, TCL, RUIJIE NETWORKS, NICT Option-2 (9): Intel, xiaomi, NEC, Lenovo, Meta, CEWiT, Huawei/HiSi, Honor, KDDI, Option-3 (2): Samsung, Google, FL Note: Option-1 seems to be a reasonable compromise, as it is based on legacy. Then, for whether/how to flexible update (e.g., timely tracking neighbouring beams, or only being based on a subset of beams) depending on event-2, we may further study FFS part in Option-1 and Option-2/3 in the next round in RAN1#118. Q1.1: Please share your views on the following compromise proposal per companies’ input. Proposal 1.1: Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - [Introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.] - FFS: The RS in the RS resource set can be updated by MAC-CE - FFS: Option-2 and Option-3 - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. Supported by: (30): ZTE, OPPO, E///, Qualcomm, ETRI, xiaomi, MediaTek, NEC (MAC-CE), DOCOMO, Nokia, vivo (MAC-CE), Fujitsu, Apple (MAC-CE), Lenovo, CMCC, Futurewei, Meta, Sharp, CATT, CEWiT, Spreadtrum, LG, Huawei/HiSi, KDDI, Panasonic, TCL, RUIJIE NETWORKS, NICT, Samsung, Transsion, Sony, Concerned by: Google (Option-3), Intel (Option-2) |
| 1.2.1 | Left-over for current beam for Event-2 —— Scheme-1 and Scheme-2 | [117] Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. FL observation-A: Regarding WA (Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam) - Confirmed by (15): Huawei/HiSi, MediaTek, ZTE, Samsung, E///, vivo, CMCC, Intel, xiaomi, CATT, TCL, Lenovo, RUIJIE NETWORKS, Transsion Holdings, ITRI, - Not confirmed by (1): Google, FL Note: Per companies input, confirming the above WA seems stable. Proposal 1.2.1A: Regarding RS measurement for the current beam for Event 2, for Option-2a, the following working assumption in RAN1#117 is confirmed: - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. Supported by (28): Ericsson, Samsung, Huawei/HiSi, xiaomi, Intel, ZTE, Lenovo, IDC, LG, Fujitsu, Spreadtrum, Qualcomm, vivo, Nokia, MediaTek, CMCC, Sharp, HONOR, Futurewei, Transsion, Apple, Ruijie, KDDI, TCL, OPPO, NTT DOCOMO, ETRI, Sony, Concerned by (2): Google, CATT, Q1.2.1B: Please share your views on selection of scheme-1 and scheme-2, i.e., explicit RRC vs implicit manner. FL observation-B: Regarding selection of scheme-1 and Scheme-2 - Explicit RRC signaling (10): ZTE, Samsung, xiaomi (3b), TCL, Transsion Holdings, Intel, Google, IDC, NEC(?), CATT - Implicit manner (if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled) (22): Huawei/HiSi, MediaTek, Spreadtrum, CMCC, xiaomi, Fujitsu, Panasonic, TCL, ITRI, Ericsson, Lenovo, LG, Spreadtrum, Qualcomm, vivo, CATT, CMCC, Sharp, HONOR, Futurewei, Apple, Ruijie, OPPO, NTT DOCOMO, ETRI, Sony, |
| 1.2.2 | Left-over for current beam for Event-2 —— Introduce a new scheme | [117] Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded. FL observation: - Option-1 (7): Qualcomm, Fujitsu, CATT, Panasonic, Honor, CEWiT, xiaomi, - Option-2 (10): MediaTek/Ericsson (Already supported by Scheme-1 without any additional configuration), Apple, NTT DOCOMO, vivo, Futurewei, CMCC, Fujitsu, Samsung, NEC - Option-3 (4): Huawei/HiSi, NTT DOCOMO, Transsion Holdings, Google, - Option-4 (18): ZTE, Samsung, Nokia, Qualcomm, OPPO, Futurewei, Spreadtrum, Intel, CATT, Lenovo, ETRI, Meta, Sharp, xiaomi, IDC, LG, NEC, Ruijie, FL Note: Companies’ inputs are still diverged, and then some further discussion may be beneficial. Q1.2.2: Please share your views whether there are any critical issues for handling the case that only one TRS is configured in the indicated TCI state, regarding RS measurement for the current beam for Event 2, for Option-2a, besides for Scheme-1 and Scheme-2. - If yes, please share your preference and its advantage over other candidates. |
| 1.3 | Candidate Event 1 & Event 7 | [117] Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality. FL Note: Per companies input, we have the following observation as a summary Event-1:; Usage/motivation: - Soft BFR used as an early warning on the quality degradation of the current beam; - In DRX, it is enough to know that a beam is good enough, and, compared with Event-2, Event-1 will lead to fewer reports, which is more important in DRX - Additionally, ‘Event-1+Event-2’ may handle concerns about power consumption, measurement resource configuration etc.. Observations per evaluation: - No evaluation results. Event-7a (worst) or update (M-th worst quality); Usage/motivation: - Timely update activated TCI-state list with low-latency and report overhead; - If high-quality beam(s) can be pre-activated before indication, the TCI state switch delay caused by UE tracking the first SSB can be eliminated.. Observations per evaluation: - MTK (SLS) [5]: With 4 active TCI states, - Both Event-7a and Event-7c can significantly reduce the number of reporting, compared to using P-CSI reporting for active TCI state list updating. - Using Event-7a with applying larger offset value can achieve the similar triggering frequency and overhead of Event-7c. - ZTE (SLS) [9]: - For event-7a, UEIBR can be sufficiently but not properly triggered, especially when consider the wasting of resource overhead for updating activated TCI state with the worst quality that may not be applied in a long time. - To leverage the tradeoff, threshold value is expected to fulfill this but not work well (i.e., increased from 4dB to 8dB). Instead, as shown in the simulation result, it is more proper and efficient to replace the worst quality by the M-th worst quality (e.g., the 3rd or 5th worst quality), where the value of M can be configured by gNB for availability and flexibility (Left) RSRP difference between the new beams and the worst activated TCI state; (Right) Timer interval between two consecutive UEIBRs. Event-7b (best); Usage/motivation: - ?. Observations per evaluation: - No evaluation results. . Q1.3: Please share your preference or strong concerns on above candidate event(s). - For your preferred events, please describe their potential usages and whether/how to co-exist with the event 2 or other UE initiated procedure (like BFR). - Note: At for now, we can NOT just approve additional event(s) without any guidance/restriction. The framework/picture for each alternative, e.g., detection metrics, report format, report procedure, etc, are definitely needed. FL observation: Per companies input, there is the following observation on above candidate events: - Event-1 (23): Huawei/HiSi, Samsung, Apple, Qualcomm, Ericsson, NTT DOCOMO, Futurewei, Spreadtrum, CMCC, Intel, xiaomi, Fujitsu, Panasonic, Sony, Lenovo, RUIJIE NETWORKS, KDDI, Transsion Holdings, Meta, Sharp, ITRI, Google, NEC, - Event-7a & update (M-th worst) (20): MediaTek (M-th worst), ZTE (M-th worst), Nokia (Combined), Apple, Ericsson, NTT DOCOMO, LG (Updated), Futurewei, IDC (M-th worst), xiaomi, Honor, KDDI, ETRI, Meta, Sharp, ITRI, CEWiT (M-th worst), ASUSTeK, Google, NEC, - Event-7b (9): LG, CATT, Lenovo, Honor, KDDI, ETRI, Meta, Sharp, ASUSTeK - Not support any extra events (2): OPPO, vivo, FL Note: Per companies input, the following proposal may be what we can do for now. That is, let’s focus on Event 1 and 7a (update) as a starting point (possible outcome may be nothing) for RAN1#118, and then, in Masstricht, we may make the final decision after F2F discussion. - Clearly, Event-1 and Event7a (update) may have the same #. of supported companies (far more than Event-7b), and, maybe, equal number of opponents. - Still two companies provided their concerns on any extra events. Proposal 1.3: On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th worst quality. - FFS: candidate value of M, e.g., M=1 only, or M is RRC configured. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality. Supported by: HONOR, Futurewei, Ericsson, Apple, NTT DOCOMO, xiaomi, KDDI, Meta, Google, NEC, Sharp, ITRI, Ruijie, TCL, ETRI, [Huawei/HiSi, Samsung, Qualcomm, Spreadtrum, CMCC, Intel, Fujitsu, Panasonic, Sony, Lenovo, Transsion Holdings,], [MediaTek (M-th worst), ZTE (M-th worst), Nokia (Combined), LG (Updated), IDC (M-th worst), CEWiT (M-th worst), ASUSTeK], Concerned by: FL Note: @ Event-1 only and Event-7a & Update only, please review current situation! Can you live with the above proposal 1.3? Once approved, we will identify final way-forward solution/compromise in the next round. |
| 1.4 | Left-over issue for event evaluation —— To be captured in RAN1 or RAN2 spec | [117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Q1.4: Please share your views on the above FFS: the functionality of event evaluation is captured in RAN1 or RAN2 spec. - Note: From spec draft perspective, we do need to make a final decision on which spec to capture. Usually, the implementation in a UE is fully independent from which RAN1 or RAN2 spec the functionality is captured by. FL observation: Per companies input, there is the following observation on above issues: - RAN1 spec: Samsung, vivo, Futurewei, NEC, Sharp, - RAN2 spec: Huawei/HiSi, ZTE, E///, Panasonic, Spreadtrum, Qualcomm, Nokia, CATT, HONOR, Panasonic, Transsion, Apple, NTT DOCOMO, ETRI, - Both RAN1 and RAN2: Intel, - Up to RAN1 and RAN2 spec editors: MediaTek, - Postponed by: OPPO, |
| 1.5 | Other pending issues | FL Assessment: Per companies input and previous meeting discussion, further progress on the following pending issues may be difficult for now. - Issue 1.A: Whether/how to introduce additional option(s) (e.g., Option-3b/3c) for new beam measurement, besides for explicit manner (Option 3a); - … As a FL recommendation, the corresponding discussion may be postponed till we achieve some progress on their related topics (e.g., stabilizing a baseline framework for new/current beam, introducing additional Events besides for Event-2, additional progress/income from RAN2, etc). If you have any views on moving forward the following issues, please share them accordingly. |

Table 1-2 Company input for Issue 1

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your comment/preference to those proposals and questions on the issue 1.1~1.5, if needed. |
| Ericsson | P1.1: Support P1.2.1A: Support Q1.2.1B: Implicit seems OK. Difficult to see the benefit of an extra RRC parameter, since its value is determined by another RRC parameter Q1.2.2: Fair to continue the discussion. In our understanding, there is no need to add anything as measurements on individual CSI-RS resources is defined in 38.215. We note that both scheme-1 and scheme-2 are agreed, and that it should be possible to configure the UE to perform measurements accordingly. It is thus not OK that the UE performs measurements on SSB if it is configured to perform measurements on CSI-RS, if that is the interpretation of option-4. [Mod]: Tend to agree. Scheme-2/4 proponents may have different understanding on the potential/back-up solution. Before down-selecting or removing some candidates, we may wait for some more input(s) in the first round. Q1.3: Our views are captured. Then the proponents for the events have repeatedly explained the use case, and there is a large majority that see the value of the additional events. Event-1 would be used as an early BFR, and the NW would use it to trigger, e.g., aperiodic beam reports. Event-7a would be used to update the list of activated TCI states, to facilitate beam selection using DCI. Question to FL: what does the note mean? RAN1 always makes agreements without all details are settled. Why would we use a different approach here? It is rather the other way around: it becomes very difficult to [Mod]: Thanks for your clarification. Yeah, directly having the outcome of down-selection seems to be possible as you mentioned, but, to be honest, considering the recent progress for event-2, it is fair to consider the co-existence, together, while doing that. Q1.4: Just to clarify: the comparison of the RSRP measurements will be captured in a RAN1 specification, most likely 38.214. The discussion is then related to the counters, and here we note there are many cases where counters have been implemented in RAN2 specifications, but no case where they have been implemented in RAN1 spec. It is thus better to rely on the RAN2 competence when this is implemented. Of course, there is no performance impact on in which document the functionality is described: the implementation does not depend on where this is captured. |
| Samsung | Issue 1.1: In Option-1, the UE needs to measure all the configured candidate beam RS(s) in the RS set to evaluate Event-2, which could become quite complicated especially when the number of configured candidate beam RS(s) in the RS set is relatively large (e.g., 64). Different from legacy beam measurement, evaluations and/or triggering of Event-2 additionally involves: - For each measurement occasion, comparing between each of the candidate beam RSs and the current beam RS with a threshold - Identifying that at least a same new beam occurs across a number of measurement occasions (within a time window) To resolve the above issue, the network can configure a relatively small number of candidate beam RSs, but in this case, the UE would not be able to timely track the (candidate) beam changes, which would result in performance degradation. If a small number of candidate beam RSs is configured in the set, RRC reconfiguration or MAC CE update would become unavoidable, which is another source of latency and overhead – note that for small to medium configurations (which is the case here), MAC CE update or MAC CE signaling in general would not have significant advantages (if any) over RRC signaling in terms of latency and/or overhead reduction(s). We have similar concerns for Option-2 with MAC CE based activation of a subset of the configured RSs. For Option-3, each configured TCI-State can be associated with candidate beam RS resources at RRC level (an example of the corresponding IE is provided below) – hence no issues of signaling overhead. With Option-3 and along with a TCI state update (hence an update of the current beam RS), the UE can just focus on a smaller set of candidate beam RSs, which would significantly ease the UE’s efforts of measuring and monitoring beam quality(s) of the candidate beam RS(s) for Event-2 evaluation(s). [Mod]: Thanks for your input. Do understand your preference, but we still need to identify a way-forward solution. Your being flexible is highly appreciated. Issue 1.2.1: We support to confirm the WA. For selecting between Scheme-1 and Scheme-2, prefer to introduce a dedicated RRC parameter. [Mod]: Captured! Issue 1.2.2: We are open to Option-2 and Option-4 (slightly prefer). We are also fine that for event(s) evaluations, the UE would not expect to be configured with only one TRS (Type-A/D) in the indicated TCI state. Introducing additional QCL rules or configurations for this purpose seems not strongly motivated. [Mod]: Captured! Issue 1.3: We do not see valid use cases for Event-7a or Event-7b. For Event-7a, even if a new beam that is a threshold value better than the worst activated beam can be identified, there are still a handful number of activated beams (having better beam qualities than the worst activated beam or all of the activated beams except the worst one) that can be used for beam indication; hence, there is no need for the network to update the activated TCI states based on the new beam(s) identified during the evaluations of Event-7a. If the M-th worst value is used instead of the worst value, how to set the value M is also tricky, which would at least depend on the number of activated TCI states – we do not think having the network to set the M-th worse is workable as all the measurement quantities are obtained at the UE side. For Event-7b, the activated beam or TCI state with the best quality in general would correspond to the indicated TCI state, making Event-7b equivalent or close to Event-2. We do not think Event-7a/b would be beneficial in reducing overhead but rather result in “invalid” reports hence more “overhead”. Application scenario(s) for Event-1 has become more clear to us – it targets at preventing BFR from happening, which can be beneficial in terms of latency and overhead reductions. If Event-1 is supported, to minimize potential duplicated efforts in specifying the corresponding UE’s behaviors (as for now, almost all the discussions related to the measurement RS(s), reporting content(s)/format(s) etc. are focused on Event-2), it is preferable to reuse what RAN1 has agreed so far for this agenda item, e.g., the same reporting format, the same steps in both Mode-A and Mode-B for the UCI based UL signaling procedures and etc., for Event-1, though the event triggering condition(s), configuration/determination of measurement RS(s) may need to be customized for Event-1. Issue 1.4: We support to capture the condition(s) of evaluating Event-2 in RAN1 specifications. According to the agreement below, Event-2 evaluation or triggering determination involves counting at least one same new beam a number of times within a time window. This essentially is a beam-level or beam-specific counting process of event instance(s), which is preferably to be captured in RAN1 specifications – note that RAN2 specification does not have precedence of this beam-level or beam-specific counting process, and it is unclear how to signal from lower layers of the UE to their higher layers regarding identifying the same new beam(s). Besides, the below agreement does not read that the Event-2 instance has to be indicated from lower layers to higher layers of the UE. In fact, some essential assumptions or components regarding the beam-level or beam-specific counting process, e.g., whether the time window is a sliding window, condition(s) of resetting the counting, and etc., for evaluating Event-2 are still missing. It would become clearer on how/where to capture the condition(s) after these details are revealed and addressed. [Mod]: Good point. Tend to agree with you that the left-over issues should be handled later, after handling this FFS. [117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. |
| Huawei, HiSilicon | Proposal 1.1: Support. Our first preference is Option 2 but we can support Option 1 as it is simple and aligned with the legacy CSI framework. Proposal 1.2.1A: Support. Two different RS types are not comparable in general due to their different beam widths, number of ports, and transmission power. [Mod]: Captured! Q1.2.1B: Support implicit manner. Since we already have the WA which ensures the same RS type for the current beam and new beam, the RS type of the current beam can be implicitly derived based on the new beam. No need to introduce an RRC parameter. [Mod]: Capture! Q1.2.2: We think there is a critical issue for handling the case that only one TRS is configured in the indicated TCI state and, to resolve this issue, we support Option-3. We think only Option-3 is the functional choice when TRS is the QCL source of the indicated TCI state. Below, we explain the critical issue first and then analyze the four options in RAN1 117 agreement to resolve this issue. Explanation of critical issue for the case that only one TRS is configured in the indicated TCI state: Our understanding is that gNB usually has two sets of beam-widths, that is, wide beams and narrow beams in a typical implementation. Therefore, gNB needs to track the best wide beam and the best narrow beam. To facilitate this, gNB needs to configure two CSI-reportConfig; one for wide beam measurements and the other for narrow beam measurements. For the CSI-reportConfig for wide beam measurements, since SSBs are transmitted using wide beams, gNB configures SSBs as measurement resources for new beams and scheme-2 is used. For the CSI-reportConfig for narrow beam measurements, if the QCL RS in the indicated TCI state is a CSI-RS for beam management, gNB configures a set of CSI-RS for beam management as measurement resources for new beams and scheme-1 is used. However, for narrow beam measurements, if the QCL RS in the indicated TCI state is a TRS, neither of scheme-1 or scheme-2 can be used since 1) both SSB and TRS are usually transmitted via a wide beam; and 2) Even if TRS is transmitted using a narrow beam, TRS is not supported for beam measurement due to its restricted number of port (only one port) and large overhead (2 or 4 times than CSI-RS for beam management). Analyzing 4 Options: To solve above issue, only Option-3 is functional as explained below: For Option-1 if the TRS is QCLed with an SSB, it is not possible to derive a CSI-RS for beam management from the TRS, as the whole QCL chain does not include a CSI-RS for beam management. For Option 2, as discussed above, TRS is usually transmitted via a wide beam and cannot be used for narrow beam measurement, and even if TRS is transmitted with a narrow beam, TRS is not supported for beam measurement due to its restricted number of port (only one port) and large overhead (2 or 4 times than CSI-RS for beam management). For Option 4, scheme-2 is used without any enhancement. In this case, SSB which is QCLed with the TRS in the indicated TCI state is considered as the current beam and is compared to the SSBs that are configured as measurement resources for new beams. The consequence is that gNB cannot use UE initiated beam reporting to obtain the best narrow beam when TRS is the configured QCL RS of the indicated TCI-state. This is however a typical configuration already applied in implementation. Thus, Option-4 makes UEIBR not useful under typical implementation and is not acceptable for us. In Option-3, gNB explicitly configures a measurement resource for the current beam when the QCL source of the indicated TCI state is a TRS. Option-3 is the only option that facilitates narrow beam measurements when the QCL source of the indicated TCI state is a TRS by means of, e.g., configuring a CSI-RS for beam management as the measurement resource for the current beam. [Mod]: @all, please review the above analysis for supporting Option-3. Q1.3: We support Event 1. Use case: Event-1 can provide an early warning on the quality degradation of the current beam to avoid a BFR procedure. Once Event-1 is reported, gNB can trigger/activate some AP/SP beam measurement to find a new beam and prevent the beam failure and the subsequent latency of finding a new beam using a BFR procedure. Further, such report can also help reducing RS overhead as, having Event-1 in place as an early warning for the degradation of the current beam, gNB can configure UE to measure candidate beams with a longer periodicity. Coexistence with Event 2: Event 1 and Event 2 are used to report different info of UE and are compatible to each other. Event-1 is much less complex than Event-2 since there is no new beam to measure. Also, the report overhead of event-1 is much less than that of event-2. Nevertheless, they may be used jointly. For example, when Event 1 and Event 2 are both configured, if the current beam degrades in quality and a better beam exists, UE can report Event 2; while if the current beam degrades in quality and no better beam is available, UE can report Event 1. As another example, RAN1 could further specify that UE ignores event-2 unless event-1 is already occurred (i.e., UE looks for potentially better beams (as in event-2) only if it realizes that the current beam is not good enough (as in event-1)). This can save a lot of measurements/reports associated with event-2. Coexistence with BFR: For BFD/BFR, UE triggers beam failure indication to higher layers if hypothetical PDCCH is below 10% (and, hence, the link is presumed failed). The hypothetical PDCCH is calculated based on specific set of parameters provided in Table 8.1.2.1-1 of 38.133 (if q0 is SSB) or in Table 8.1.3.1-1 of 38.133 (if q0 is CSI-RS). This is different from event-1 where the report is sent simply when the L1-RSRP of current beam is below a configurable threshold. Since BFD message to higher layers is only triggered when hypothetical PDCCH is below 10% (i.e., the link is presumed failed), an immediate subsequent BFR procedure is required to substitute current beam with a new beam. This is not necessarily the case in event-1. NW can flexibly select the threshold for event-1 so the UE sends the event-1 report once the current beam L1-RSRP is below the configurable threshold. This can be used as an early warning to the NW that current beam may not be ideal. Subsequent steps is up to NW. For instance, NW can trigger AP beam measurement to possibly indicate a new beam if necessary. Based on the above explanation, Event 1 can be considered as a soft BFR. Comparing to BFR, it is more efficient as it can report an early “warning” and avoid the serving beam going into beam failure which causes long-time interruption of communication. Q1.4: We think the functionality of event evaluation is better to be captured in RAN2 spec. The evaluation of event 2, i.e., timing window + counting based evaluation, is similar to the evaluation of beam failure, which is captured in RAN2 spec. Introducing a new batch of spec impact in RAN1 seems not necessary. Some may argue that the interaction between layer 1 and layer 2 would cause some latency. However, such kind of latency is negligible compared to other sources of latency in the UE initiated beam reporting procedure. For example, for Mode A, the latency for scheduling the second channel by DCI can be several milliseconds. Further, for both Mode A and Mode B, the periodicity of the first channel can be several milliseconds (as a reference, the maximum SR periodicity for 120 kHz can be as long as 640 slots or 80 milliseconds). Therefore, when the measurements are ready, the UE may need to wait for quite a long time for the first available PUCCH resource just to send the first channel to start the beam reporting procedure. As such, the overall latency of the procedure is much larger than the latency of the interaction between layer 1 and layer 2 of the UE. |
| Xiaomi | Proposal 1.1 In addition to Option -1, also support Option -2 or 3b with activated TCI state as the new beam to reduce the number of measurement resources. Proposal 1.2.1A/Q1.2.1B Ok to proposal 1.2.1A. But for Q1.2.1B, if Option 3b will be supported for new beam configuration, implicit manner can’t work. [Mod]: After double checking this Q1.2.1B, we may only consider the case of option3a (to which Scheme-1 and Scheme-2 belong). If so, can I remove xiaomi from the list of ‘explicit RRC signaling’? Q1.2.2 We support Option 4 at least. But if only Option 4 is supported, the CSI-RS resource can’t be used for beam management, how to obtain the TCI state in which CSI-RS/TRS resource is the QCL source? So, in addition to Option 4, we slightly prefer to support Option 1, but how to determine the CSI-RS for beam management derived from the QCL RS in the indicated TCI state need to be clarified. [Mod]: Captured! Q1.3 Support Event 1 and Event 7a. Event 1 can be configured first, if event 1 is triggered, gNB can configure Event 2. If means that only when the current beam becomes worse than a threshold, UE need to measure the new beam. in this case, the measurement and power consumption can be reduced at UE side. Event 7a and Event 2 can be configured simultaneously. But Event 7a may be triggered before Event 2. With Event 7a, the activated TCI state can be updated before. Thus, it is more likely that the new beam triggers Event 2 is one of the activated TCI states, which can reduce the latency of indicated TCI state switching. |
| Intel | For Proposal 1.1, we still think Option-2 would be a good solution for proper system operation. The measurement RS overhead, measurement latency would be an issue if a large number of beams is configured for new beam measurement for Event 2. In this case, MAC-CE can help reduce the complexity, latency and overhead. [Mod]: Do understand your preference, but we still need to identify a way-forward solution. Your being flexible is highly appreciated. For Proposal 1.2.1A, we support to confirm the working assumption. [Mod]: Captured! For Q1.2.1B, we think explicit RRC configuration for scheme 1 and scheme 2 would work. We do not see the need for implicit approach. [Mod]: Captured! For Q1.2.2, we prefer no further enhancement, i.e., Scheme 2 can be used to handle this case. For Q1.3, we support Event 1, and do not support Event 7a/7b. As mentioned by FL, Event 1 can ensure that the UE detects current beam deterioration but has not found a better candidate beam from the measured beams. This can be jointly supported with Event 2 for performance improvement. Further, event-1 can thus be useful for reducing candidate beam measurement RS overhead. If both Event 1 and event 2 are configured, different PUCCH resources in the first step can be used to differentiate these two events. For Q1.4, our view is that both RAN1 and RAN2 should discuss the specification impact. For instance, L1 instance and periodicity can be discussed in the RAN1 while the procedure for triggering the event and counters/timers resetting can be discussed in the RAN2. We support event triggering in the MAC layer. |
| ZTE | Proposal 1.1: Support. Alt1 is most inline with the legacy signaling design of CSI report Config that aligned with the statement in WID to leverage (as much as possible) legacy CSI measurement and reporting configuration framework. For the update of measurement RS via MAC CE, we think it can be beneficial to performance gain of Event-2. Proposal 1.2.1A: Support to confirm the WA to guarantee the quality metric to be aligned between new beam and current beam. [Mod]: Captured! Q1.2.1B: Support explicit RRC signaling. First, it can be the second safeguard on top of Proposal 1.2.1A to avoid that SSB is configured as measurement RS of new beam but UE wrongly measures CSI-RS as in Scheme-1, or vice versa. In particular, the implicit manner may be inefficient even invalid for cross-CC UEIBR. For instance, if different RS types are configured in different CCs, it cannot be foreseen by gNB that either Scheme-1 or Scheme-2 of current beam is applied in each of multiple CCs where UEIBR is triggered. Q1.2.2: Support Option-4. We are open to hear any views to prove the necessity of this issue as FL mentioned. Besides, if go with Option-2, we’d like to know if it is common understanding that the current specs already supported the UE can measure L1-RSRP of TRS. If so, at least an LS needs to be sent to RAN4 for clarification. [Mod]: Okay, let’s check other companies view about this possible solution! Q1.3: Support Event-7a by updating “the activated TCI state with the RRC configured M-th wort quality” (call it Event-7c for legibility) according to companies SLS results and also the following reasons: - Event-7c is critical to facilitate fast beam switching on top of Event-2 due to if a new beam applied to Event-2 is already in the activated TCI state list updated based on the report triggered by Event-7c, the TCI state switch delay (i.e., Tfirst-SSB and TSSB-proc for tracking the first SSB transmission after MAC CE command as specified in TS 38.133) can be properly reduced. Otherwise, the motivation of solely supporting Event-2 may be quit marginal w.r.t. beam switching latency reduction by UEIBR. - Event-7c can properly avoid frequent but inefficient UEIBR in Event-7a due to the lower quality of the activated TCI state is, the less potential it can be applied for transmission (See our SLS result provided in Table 1 in R1-2406028). - Instead of solely relying on threshold value in Event-7a, it is more efficient and practical to change the comparing object of activated TCI state with the M-th worst quality in Event-7a (See SLS results provided by MTK and ZTE above). Regarding the specific value of M-th, it can be determined by gNB to trade-off the resource overhead of UEIBR and the possibility of activated TCI states to be applied for transmission. [Mod]: Thanks for analysis. Let’s check other companies view on this updated event7. For Event-1, the use case mentioned by companies is still inconclusive on top of Event-2+BFD, and the next-level details are unclear due to the lack of new beams. Besides, as mentioned by FL, the additional work load is indeed worth to be carefully assessed in terms of quality metric (L1-RSRP or L1-SINR), report format (Option-3 of Event-2) and reported procedure (Mode-A/B of Event-2), etc. By comparison, most of them can be smoothly reused/extended to Event-7. Besides, we encourage companies to provided simulation result to evaluate the necessity of Event-1. Q1.3: Analogous to counter and timer of the legacy BFD is defined in RAN2’s spec, we think it is proper to be up to RAN2 to specify event triggering procedure of UEIBR. Compared to the latency caused by awaiting a PDCCH for scheduling in Mode A and that caused by a time gap between the first PUCCH and second UL channel, the latency caused by the interaction between L1 and L2 seems marginal. [Mod]: Okay. But, being captured in RAN2 spec, if approved, does not mean that guidance/recommendation of this triggering procedure from RAN1 is precluded. |
| Lenovo | Q1.1: OK with Proposal 1.1. P1.2.1A: Support. Q1.2.1B: Implicit manner should be sufficient. [Mod]: Captured! Q1.2.2: There is no critical issues for handling the case that only one TRS is configured in the indicated TCI state and in this case the NW can configure the UE to measure the SSB associated with the TRS according to RAN1#117 agreement. Q1.3: Our preference has been well captured. Repeat the motivation provided in our contribution, event-1 is useful to avoid BFR procedure. Event-7b can help the NW to identify another set of beams which can be updated as the activated TCI states. Q1.4: Considering that the beam report will be carried in UCI, which will be captured in RAN1 spec, and the UE shall firstly send a SR or a notification in PUCCH. Similar with the BFRQ, we this the procedure corresponding to this agreement should at least be captured in RAN1 spec. |
| Google | Proposal 1.1: We do not support this proposal. As we mentioned in offline, the new beam measurement should be related to the current beam (i.e., the indicated TCI state). We do not imagine that a new candidate beam would appear on the totally opposite direction of the current beam. In addition, Option-3 can also reduce the amount of measured candidate beam and ease UE complexity. Some companies mentioned signaling overhead issue for Option 3, which we disagree. Even for Option-1, NW needs to configure a multiple of RS resource set(s) for new beam measurement. In addition, signaling overhead of RRC signaling should not be as severe as L1 signaling. [Mod]: Thanks for your sharing views. Captured! Proposal 1.2.1A: We still do not see the critical issue if this WA is not confirmed. In our views, new beam RS set is explicitly configured by NW (regardless of the outcome of Proposal 1.1). If NW prefers current beam and new beam have the same RS type, NW has the freedom to implement it. Then, what is the benefit of confirming this WA? [Mod]: To save UE complexity or avoid over-design/optimization? Q1.2.1B: We prefer explicit manner. [Mod]: Captured! Q1.2.2: TRS itself is not designed for L1-RSRP measurement as HW mentioned. Also, when UE measures TRS, UE should not change its receiving beam. But it is not the case for measuring current beam quality. Option 1 is not clear to us. For example, how to derive the CSI-RS for BM. Assume it is done by the same root SSB. What if there are more than one CSI-RS with the same root SSB, or none? In this regard, we still support Option-3, which is clean and simple. Q1.3: We can also support Event-7a, in addition to Event-1. [Mod]: Captured! Q1.4: Normally, timer/counter related procedure is captured in RAN2 spec. However, we may need to discuss further details before asking RAN2 captures it to avoid frequent LSs back and forth. For example, when to (re-)start the time window and the counter, potential M values? [Mod]: Good point! Before answering that, can we handle this FFS firstly. Otherwise, we may have to skip this discussion. |
| Mod V09 | Captured companies input. The update is marked in red. - For Issue 1.3, let’s wait for some more input. Then, we may need to identify a way-forward solution for accelerating the discussion on this tough issue. |
| IDC | P1.1: We can be flexible with Option-1, if the FFS is updated as follows (to be clear for the dynamic way to add at least the newly activated TCI states by MAC-CE into the set, if not already exist in the set, otherwise NW should only rely on RRC re-configuration for managing the set which doesn’t make sense in terms of Event-2): - FFS: The RS in the RS resource set can be updated by MAC-CE, i.e., QCL RS(s) of activated TCI state(s) by the MAC CE are added into the RS resource set. [Mod]: Thanks for being flexible. But, the newly added part is relevant to implicit manner (which has been FFS while Option-1 explicit one is approved), and herein we focus on explicit manner. But, anyway, we may have an individual discussion for dynamic RS update regardless of explicit or implicit manner. P1.2.1A: Support confirming WA. Q1.2.1B: Support the explicit way to make it clear. Q1.2.2: Support Option-4, which is sufficient and simplifies things, as all the configurations are controllable by NW, so the NW implementation can properly provide intended configurations. Q1.3: We support ZTE’s suggested “Event-7c” (as the activated TCI state with the RRC configured M-th worst quality), basically combining Event-7a and 7b together, which makes sense to aid gNB’s decision on sending the existing TCI-activation MAC-CE, while avoiding ping-pong effects by controlling the parameter of M. [Mod]: Captured! |
| LG | P1.1: Support P1.2.1A: Support Q1.2.1B: Implicit determination is sufficient without extra RRC signaling. Q1.2.2: support Option 4. Q1.3: Event-1 would have the same usage as legacy BFR. Proponents for Event 1 argue to use it as early BFR but current spec supports to configure window/counter/threshold for BFD and those can be configured with small value to reduce latency. On the other hand, Event-7a has limited usage since it can be used to update worst activated beam only, so we support Event 7b or Event-7a with n-th worst beam. @Samsung: There are many cases where activated beam with the best quality is not the same as the indicated TCI state considering MU scheduling, inter-cell interference mitigation, etc. Also, regarding determination M values, gNB determines M value depending on how may activated beams gNB want to update simultaneously. For example, if gNB want to update half of activated beams when their quality is less than new beams, gNB configures M=half. [Mod]: Captured! |
| Fujitsu | P1.1: Support Proposal 1.1. P1.2.1A: Support Proposal 1.2.1A. Q1.2.1B: Implicit manner is preferred. Q1.2.2: We are fine with either Option-1 or Option-2. Option-2 is slightly preferred. For Option-3, additional signaling is required and thus is not preferred. For Option-4, CSI-RS cannot be used for the current beam, which seems to be restrictive. Q1.3: Event-1 is preferred. The usage is for early notification of the degradation of the current beam. Then the gNB can trigger the UE to find new beams timely, e.g., by triggering AP CSI report. Also, we do not see any issue when coexisting with Event-2. [Mod]: Captured! |
| Spreadtrum | Proposal 1.1: Support. For RS configuration of new beam measurement, the gNB can configure multiple appropriate new beam RSs based on the indicated TCI state and the activated TCI states, so it seems that there is no much necessity to introduce additional MAC CE to update new beam RS. Proposal 1.2.1A: Support. Q1.2.1B: Support an implicit manner based on the RS type of the explicitly configured new beam RS, e.g. if the RS for new beam is CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. Q1.2.2: Support Option-4. For the case that only one TRS is configured in the indicated TCI state, the Scheme-2 can be enabled, i.e. an SSB QCL-ed with the TRS in the indicated TCI state is used for current beam measurement. At this time, the RS type of new beam measurement can be configured as SSB. Q1.3: Support Event-1. For Event-1, the UE does not need to monitor the quality of new beams all the time, but rather monitor on demand for new beam measurement overhead reduction. For example, firstly the UE only needs to monitor the current beam quality. When the quality of the current beam is worse than a certain threshold, the UL signaling is transmitted by UE to notify gNB that the current beam has deteriorated. Then SP-/AP-CSI-RS can be activated or triggered by gNB for new beam measurement and report. For Event-7a, when at least one new beam is better than the worst activated TCI state, the activated TCI state can be updated through Event-7a triggered beam report. If Event-2 does not occur at this time, it means that the current beam is still available, and updating the worst activated TCI state is not urgent. If Event-2 occurs at this time, the current beam can be updated based on the Event-2 triggered beam report. Moreover, when multiple new beams are reported in the Event-2 triggered beam report, the gNB can also use these reported new beams to update the activated TCI state. For Event-7b, when at least one new beam is better than the best activated TCI state, Event-2 is also satisfied and the gNB can directly update the indicated TCI state based Event-2 triggered beam report. Therefore, there seems to be no effective scenario or benefit for Event-7a and Event-7b. Q1.4: For the event evaluation, we slightly prefer to discuss counter/timer in RAN2 similar as BFR procedure. [Mod]: Captured! |
| Qualcomm | Proposal 1.1: We support the proposal. We share the same view as FL. Proposal 1.2.1A: We support the proposal. Q1.2.1B: We support the implicit manner. To further elaborate, our view is that, as proposed in our contribution, the current beam RS should be implicitly determined in the same resource set as the new beam(s) – based on the legacy CSI Resource Setting framework, the associated resource set can be configured either as a SSB resource set or as a CSI-RS resource set. Hence, based on the configuration of the associated resource set type, the current beam RS type (Scheme-1 or Scheme-2) can be implicitly determined. Q1.2.2: We support Option-1 because we think Option-1 can add flexibility in the TCI state configuration. However, for simplicity, we also support Option-4. Since there seem to be different understandings on Option-4 across companies, if we clarify our understanding on Option-4: when Scheme-1 has been selected, the UE does not expect that the QCL-TypeD RS in the indicated TCI state is a TRS. On the other hand, if the QCL-TypeD RS in the indicated TCI state is a TRS, the UE should always expect that Scheme-2 has been selected. [Mod]: Got it! Thanks for clarifying your position. Q1.3: we think, in terms of the functionality, Event-7a and Event-7b are redundant to Event-2, or optimized versions of Event-2. With Event-2, to attain the low latency in beam switching, at least all the RSs corresponding to the activated TCI states should be included in the new beam RS set. Then, with multi-beam reporting, the NW can identify which new beams are better than the activated TCI state beams and update the activated TCI state list. Considering the duplicated functionality, we don’t think supporting Event7 op top of Event-2 is necessary. However, we think further enhancement of Event-2 can be discussed to support some features of Event7. For example, the followings can be considered for Event-2: - Whether a negative threshold value can be configured - Whether the number of new beams (rather than “at least one new beam”), whose qualities are a threshold value better than the current beam to trigger the event, can be configurable [Mod]: Let’s check other companies input for modifying option-2. Alternatively, we may consider an individual UE capability of not supporting simultaneous configuration of event-2 and event-7? Q1.4: We think it should be captured in RAN2 specification. In our view, anything related to timer/memory operations should be handled by upper layers. [Mod]: Captured! |
| vivo | P1.1: support P1.2.1A: Support Q1.2.1B: support Implicit determination Q1.2.2: support Option 2. The reason for supporting “TRS as measurement RS of current beam for determining L1-RSRP” is that, the beam width of TRS may not be same as SSB in practical deployments, also mentioned by companies above. In this case scheme-2 is not reasonable. Not supporting TRS as measurement RS because it is single port is not a technical reason. Q1.3: we should focus on completing the functionality based on event-2 first. We don’t see strong motivation for supporting other events, and we don’t know what other issues would unfurl. Another, issue is if other events are supported, maybe we need more than 1 bit for first channel to convey which event has occurred. [Mod]: Do understand your preference! For now, we may have to handle multi-events vs multi-bit one by one. Clearly, we should have a conclusion of whether to introduce extra event(s) firstly. Q1.4: capture in RAN1 spec, the definition of time window in current agreement is not same as timer or counter. |
| Nokia | P1.1: Support P1.2.1A: Support Q1.2.1B: Support FFS The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. For Scheme-1 FFS on TRS Q1.2.2: [Mod]: Something wrong for above replies to Q 1.2.1B/1.2.2? ^ ^ Q1.3: Event-1 is useful to avoid for entering BFR, however similar function can be supported by gNB scheduling (e.g. aperiodic beam reporting). Therefore we see that Event-1 could e deprioritized in Rel-19. Furthermore, Event 7a and 7b could be combined to generalize it for multiple beams or TCI states. We propose FFS: how to combine Event-7a and 7b. Q1.4: MAC (38.321) is responsible for triggering any signals thus the counter or any triggering logic should be captured in RAN2 specifications. The MAC can perform the counting/triggering based on L1 indication i.e. the L1 can evaluate the event criterion. [Mod]: Captured! |
| NEC | Proposal 1.1: We can be fine with the proposal, and we also support the updated FFS from IDC. [Mod]: Thanks for being flexible. Regarding IDC’s FFS, please review above reply. Proposal 1.2.1A / Q1.2.2: We think the proposal is relevant to the outcome of Q1.2.2, which should be jointly considered. For example, if scheme 2 is applied in case of TRS only in indicated TCI state for Q1.2.2. Then for the activated TCI states, if some TCI states with TRS only, and some with CSI-RS, it seems impossible to configure Scheme 1 to monitor narrow current beam, or we may need to configure both SSB and CSI-RS for new beam RS set for scheme 1? Or we need to restrict same type of source RS for the activated TCI states? Activated TCI states; Scheme 1 or Scheme 2; New beam RS set. TRS only in TCI state A and CSI-RS for BM in TCI state B (whether such mixed source RS type for activated TCI states allowed?); Whether scheme 1 can be configured for this case?; If Scheme 1 can be configured, new beam RS set should include both SSB and CSI-RS?. [Mod]: Per your input, may I assume that you prefer explicit manner for forward compatibility. Q1.2.2: Prefer option 2 + option 4, if QCL source for the TRS is CSI-RS, then scheme 1 can still be applied, and if QCL source for the TRS is SSB, it can be decided either to use TRS as measurement RS or SSB (i.e. scheme 2 applied). Actually, TRS comprise two or four CSI-RS resources (with same port), using TRS as measurement RS will not introduce big effort/spec impact, we can just fix one CSI-RS resource (e.g. the first one) in TRS set as the measurement RS for current beam. On the other hand, if current beam information is reported, it could be up to UE to choose which RS (option 1,2,3) is used as the current beam for assessment of event-2 and provide the related information to NW in the UEI beam report. Q1.3: Fine with Event-1 and Event-7a. Q1.4: prefer to handle it in RAN1 to avoid the involvement of higher layer entity and the latency. [Mod]: Captured! |
| MediaTek | Issue 1.1: Support Proposal 1.1 At least legacy framework shall be supported. Issue 1.2.1: Support Proposal 1.2.1A with implicit manner for selecting one of Scheme-1 and Scheme-2. Given that the same RS type should be guaranteed, selection of Scheme-1 and Scheme-2 for the current beam can be simply determined according to the measurement RS type of new beam(s). Additional explicit indication is unnecessary. Issue 1.2.2: TRS-based measurement has been supported. Legacy specification has supported L1-RSRP measurement on a CSI-RS resource, regardless of this CSI-RS resource is used for which purpose (i.e., tracking or BM), as mentioned by Ericsson. Technically, there is no need to preclude TRS as the measurement RS for the current beam or introduce the other scheme to specially handle that. However, legacy specification just allows L1-RSRP computation and reporting on a CSI-RS resource set, due to no use case of L1-RSRP reporting on a single CSI-RS resource only. When it comes to R19 UEI/ED beam reporting, to support reporting of the current beam, L1-RSRP computation and reporting on a single CSI-RS resource is needed anyway, which is irrelevant to that the CSI-RS resource belongs to a set configured with repetition or trs-info. From our view, how to support L1-RSRP computation and reporting on a single CSI-RS resource for the current beam is a common issue to be resolved. [Mod]: Got it! F2F offline may be needed, especially for UE vendors, per input. Issue 1.3: Support Event-7a and do not support Event-1 We support the event which can achieve additional functionality/use cases in addition to Event-2 and/or legacy BFR (i.e., Event-7a). Event-1: Basically, when the current beam becomes non-satisfactory, the UE may or may not find other better beams for switching/recovery. If the UE can find other better beam(s) or qualified beam(s) for recovery, then Event-2 or legacy BFR shall be triggered. If the UE cannot find any better beam(s) or qualified beam(s), that means UE goes into a coverage hole, and there is nothing can do. Hence, Event-1 is redundant when adopting Event-2 and legacy BFR. Event-7a/7b: NW can update active TCI state list according to Event-2 beam report for sure. However, using Event-2 only for active TCI state list updating is equivalent to Event-7b, since the indicated TCI state is usually the best one among the activated TCI state(s). Based on the best activated TCI state or the indicated TCI state, the reporting is rarely triggered and not sensitive enough especially when UE moves fast and/or UE just support up to 2 or 4 activated TCI states. We see the use case of Event-7a in additional to Event-2. Furthermore, if supporting both Event-2 and Event-7a are supported/configured, we expect that report for Event-7a will not be sent when both Event-2 and Event-7a are triggered simultaneously. [Mod]: Good point! Triggered/reported simultaneously or not should be discussed in the next level. Issue 1.4: No need to decide here. It could be handled by RAN1 and RAN2 spec editors. |
| CATT | Proposal 1.1: OK to support. Proposal 1.2.1A: We raise concern regarding this WA. Currently, we almost have the consensus that the new beam RS set is explicitly configured by NW, while the current beam is implicitly indicated. If current beam and new beam have the same RS type, firstly, gNB configures new beams via RRC, then, it must ensure that the current beam must follow the same RS type. However, the RS for current beam is implicitly indicated via indicated TCI state, which is dynamic. If we confirm this WA, the following question is how the gNB to ensure it, which, in our view, will leads to a solution that deteriorates the flexibility of gNB configuration. Besides, the benefits of supporting the WA seem ambiguous. We prefer to not confirm for now after more discussions are performed. [Mod]: Whether this NW flexibility is needed may be questionable! Let’s check other companies’ input. Q1.2.1B: Either is fine to us. Q1.2.2: It is our view that option1 needs further clarification. The solution we prefer is that: If only one TRS is configured as the RS of the indicated TCI, then UE tries to find if there is a CSI-RS that is QCLed with it or not (This means the QCL is not always there). If yes, use that CSI-RS; If not, do not use it. In this case, it is option 4 without the description in the bracket we prefer to. Q1.3: Support Event-7b while it is our view that Event-7a is not essential. Q1.4: We prefer to RAN2 to capture but we can discuss the details before sending anything to RAN2, e.g, an LS. [Mod]: Captured! |
| CMCC | Proposal 1.1: Support. To consider the movement of UE, any RS in the RS resource set is possible to satisfy the condition of Event-2. gNB could not predict and update a suitable subset for UE. Similarly, the RSs for periodic CSI reporting, the RSs for RRM measurement, and the RSs for candidate beam of beam failure recovery are all configured by RRC only without MAC CE update. So, MAC CE is not needed to update the RS in the RS resource set. Proposal 1.2.1A: Support confirming WA. Since CSI-RS and SSB are transmitted with different beamwidth and/or different transmission power, then, it is reasonable to have same RS type for RS measurement for current beam and new beam, and the comparison between current beam and new beam is fair. Q1.2.1B: Support the implicit way. Implicit way is enough to ensure the same RS type for current beam and new beam, we do not see the need to introduce an explicit RRC parameter. When the RSs for new beams are CSI-RS, the QCL RS in the indicated TCI state is used as current beam. When the RSs for new beams are SSB, the SSB which is QCLed with the QCL RS in the indicated TCI state is used as current beam. [Mod]: Captured! Q1.2.2: Support Option-2. For Option-1, in FR1, TRS is QCLed with an SSB, CSI-RS for beam management cannot be derived from the TRS in the indicated TCI state. For Option-3, explicitly configured by RRC is less dynamic than RS derived from indicated TCI state, which may not timely reflect the quality of current beam for UE. For Option-4, only SSB can be used as current beam evaluation in FR1. Since the beam of SSB may be wider than TRS, and the transmission power of SSB may be different from TRS, which may not accurately reflect the quality of current beam for control and data channel. We support Option-2, since TRS can be used for delay estimation, L1-RSRP is a middle value for delay estimation, it does not increase UE complexity to support TRS as measurement RS of current beam for determining L1-RSRP. Q1.3: Support Event-1. For Event-1: Firstly, event-1 does not require configuring and measuring new beam RSs and could reduce the resource overhead. Secondly, Event-1 can be seen as a pre-indication before BFR. Thirdly, regarding the UEI reporting, when the current beam is worse than a certain threshold, UE could report the condition to gNB with 1 bit first PUCCH to request an aperiodic CSI reporting, then gNB could trigger an aperiodic CSI reporting via DCI, which is similar to the procedure of Mode-A. For Event-7a: Firstly, there may be multiple RSs derived from the activated TCI state satisfy the condition, then UE may report multiple times until all the RSs derived from the activated TCI state do not satisfy the condition. This will increase the beam reporting overhead. Secondly, some of the activated TCI state may be not indicated to UE within a certain time, it is not urgent to update the activated TCI state list with UE initiated beam reporting. Legacy periodic beam measurement and reporting is enough for network to update the list of activated TCI states, or the Event-2 could realize the function that when multiple new beams are reported, network could update the activated TCI state list. For Event-7c (update (M-th worst)): Event-7c is better than Event-7a to solve the frequent beam reporting problem. However, when Event-7c is satisfied, how many current beams are reported in the UEI reporting? Whether the M current beams are all reported? [Mod]: Please review LG/MTK’s reply. Thanks. For Event-7b: When Event-7b is satisfied, Event-2 has also happened, since the current beam for Event-2 is one the of RSs derived from the activated TCI state. So, event-2 is enough and more adequately to reflect the quality of current beam for UE. [Mod]: Captured! |
| Sharp | Proposal 1.1: Support the proposal in accordance with the WID. Proposal 1.2.1A: Support to confirm WA. Q1.2.1B: We prefer the implicit manner. It is enough that the UE does not expect different RS types for current beam and new beams. Q1.2.2: In our view, it is unclear whether Option 1/2 works. In Option 1, a CSI-RS cannot be used if a QCL chain does not include the CSI-RS. In Option 2, a TRS is not proper for the beam management due to a single port, the overhead by multiple resources, and so on. We think Option 4 is enough. Q1.3: Our 1st preference is Event 1 and 2nd preference is Event 7a&M-th worst. Event 1 is helpful to prevent from the beam failure and has less standardization effort. In our view, the definition of the current beam, the time window framework, and the report procedures (i.e., Mode-A and Mode-B) except for the report contents in Event 2 can be reused for Event 1. Q1.4: In our view, this feature should be specified in RAN1 spec because both measurement and report are confined in L1. [Mod]: Captured! |
| Mod V21 | Captured companies input. The update is marked in red. - For Issue 1.3, per companies input, one proposed compromise is provided. Please review that and FL notes for clarification. |
| HONOR | Proposal 1.1: Our first preference is Option-2. Option-1 is also acceptable to us. We would like to further study the FFS part of Option-1 to reduce UE measurement on new beams. Proposal 1.2.1A: Confirm the WA. Q1.2.1B: Implicit manner is preferred. Let’s assume Proposal 1.2.1A WA is confirmed, RS type for current beam can be derived from the RS type for new beams. We may need to further study whether additional indication is needed or not when the QCL RS of the indicated TCI state is TRS. Q1.2.2: Support Option 1. When the QCL RS of the indicated TCI state is TRS, network can further indicate whether the current beam is CSI-RS for BM or SSB which is QCLed with the QCL RS in the indicated TCI state. The CSI-RS for BM generally provide more granular beam than SSB. If Option-2 is agreed, does it mean the new beams should also be TRS to ensure the same type of RS (e.g., the same number of ports and time/frequency domain density)? [Mod]: Good point. Recently it is not, but some further study may be needed if going with option-2. Q1.3: Given that the majority of companies supports Even-1, we can accept Event-1 for soft BFR purpose. We have a slight preference to support Even-7a over Even-7b.We can accept current Proposal 1.3 for progress. Q1.4: RAN2 is preferred. It is better to follow the legacy spec in BFR to handle the counter/timer mechanism in RAN2. [Mod]: Captured! |
| FUTUREWEI | Proposal 1.1: We are fine with the proposal in principle. Regarding the FFS part, we are ok to FFS Option-2. However, due to the lack of support on Option-3, we suggest removing Option-3 from FFS such that the group can have a more focused study. [Mod]: Let’s check other companies input. Removing some unpopular options may be possible. Proposal 1.2.1A: We are fine with the proposal. Q1.2.1B: We support implicit manner as it can reduce signaling overhead. Q1.2.2: It seems that all the four options can handle the case with only one TRS configured, but they have different signaling overhead or application complexity. Option-1 introduces an additional scheme where the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state. The drawback for Option-1 is the additional introduction of the new QCL rule for a second CSI-RS. In addition, if Option-1 is supported, the case that CSI-RS for beam management is not configured also needs to be considered, which may make the situation more complicated and other additional new QCL rule may be further required. Therefore, from application complexity perspective, Option-1 is not preferred. Option-2 supports TRS as measurement RS of current beam for determining L1-RSRP. The benefit for Option-2 is no new rule introduced and no additional signaling overhead added, which can be considered for further discussion. Option-3 introduces additional scheme with higher signaling overhead in terms of explicit RRC messages or MAC CE for configuration and/or activation. Option-4 is without further enhancement and scheme-2 (i.e., the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state) is used in this case. In addition, scheme-2 was already agreed in last RAN1 meeting and reusing scheme-2 seems like a better choice from both application complexity and signaling overhead perspectives. Therefore, both Option-2 and Option-4 can be considered for further discussion to handle the case that only one TRS is configured in the indicated TCI state. Q1.3: Event-1 can reduce UE power consumption for beam monitoring and measurement, and Event-7a can be used to update active TCI state list by replacing the TCI state with worst quality. In general, the activated TCI state with the best quality may be used as the “current beam” as defined in agreed Event-2. In that regard, Event-7b and agreed Event-2 may have the same outcome in most of cases/scenarios. Since Event-2 was already agreed, we do not see the need to additionally support Event-7b with almost the same usages or functionality as Event-2. Therefore, we support both Event-1 and Event-7a but not Event-7b. Q1.4: Regarding whether the triggering event determination for Event-2 is captured in RAN1 or RAN2 specification, we prefer to capture it in RAN1 specification for latency and complexity reasons. Even though Beam Failure Detection (BFD) mechanism in current RAN2 specification can be leveraged to capture Event-2, it could introduce more latency and complexity because of the potential interactions between Layer1 and Layer2, such as event instance indications, event instance counting in Layer2, event triggering indications, etc. In addition, from Layer1 CSI beam reporting perspective, it is also unnecessary for Layer2 to be involved. [Mod]: Captured! |
| Panasonic | Proposal 1.1: We support. This follows legacy so complexity should not be an issue. Issue 1.2.1A Support confirming the working assumption. Q1.2.1B: We prefer that the UE implicitly determines the scheme to be used. The RS for current beam measurement is implicitly determined, whereas the RS for the new beam measurement is explicitly configured. Therefore, explicit configuration of schemes 1 and 2 only means more restrictions on the NW on current beam configurations and which degrades performance. Issue 1.2.2 Our understanding is that Option 1 can only be a conditional clause of scheme 1 when only one TRS is configured in the indicated TCI state. So, then we would have “scheme 1 with Option 1”, and “scheme 2”. The phrase “besides for scheme-1 and scheme-2” in the previous agreement is misleading. Issue 1.3 From a workload perspective, it seems reasonable to agree on Event 1 especially that it will share many design principles with Event 2. We have a strong concern about 7a. [Mod]: From the moderator perspective, if each side can NOT open their mind, the outcome may be nothing with high probability! Issue 1.4: This is about where to specify this information and not implementation. It makes sense to add it to RAN2 spec but we are flexible on this matter. [Mod]: Captured! |
| Samsung2 | For new beam measurement RS configuration, we still have concerns on the UE’s complexity of evaluating Event-2 instance(s) when the number of configured new beam RS(s) in the RS resource set is relatively large. We can compromise to have Option-1, but we need to introduce a new UE capability of the maximum number of new beam RS(s) in one RS resource set that they can support for evaluating Event-2 instance(s). [Mod]: Reasonable! Thanks for being flexible. Updated, and let’s check other companies’ views. In addition to the above, we prefer to remove all FFSs from the proposal to have a cleaner solution – anyways the proposal says at least Option-1. Whether/how to support flexible beam update/track can be discussed separately. [Mod]: From moderator perspective, the following part should fine and just make each side a little bit comfortable! But, we may further review companies input (Pls review FW’s comment as above). Proposal 1.1: Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - FFS: The RS in the RS resource set can be updated by MAC-CE - FFS: Option-2 and Option-3 - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. |
| Transsion | Proposal 1.1: Support as it is more inline with the legacy CSI reporting configuration framework. Moreover, it seems unnecessary to update new beam RS by MAC CE. Proposal 1.2.1A: Support to confirm the WA to guarantee the same RS type between new beam and current beam. Q1.2.1B: Support the explicit way to make it clear. Q1.2.2: For Option-1, it seems further clarification is needed. For example, the association between CSI-RS for BM and TRS is needed. For Option-2, TRS is not designed for L1-RSRP measurement as mentioned by other company. For Option-4, CSI-RS cannot be used for the current beam and hence it seems to be restrictive. We support Option-3 as it is clean and simple. Q1.3: Support Event-1. Event-1 can provide early indication of the degradation of the current beam and new beam selection can be triggered by NW. For Event-7a, there may be multiple RSs that satisfy the condition, which require more report overhead. Compared with Event-2, the condition of Event-7b is too restrictive. Q1.4: We prefer to discuss in RAN2 similar as BFR procedure. [Mod]: Captured! |
| Apple | Proposal 1.1: Support. Proposal 1.2.1A: Support. Q1.2.1B: Implicit signal is preferred. Issue 1.2.2: First, in FR2 deployment scenarios, it is widely observed that NW configures TRS as QCL RS, rather than using CSI-RS with ‘repetition’. Consequently, handling the case that TRS is configured in the indicated TCI-state should be carefully examined to ensure the UEIBR feature attractiviveness in commercialization. Additionally, the spec impact is fairly trival as pointed out by other propoenent companies. Third, if we choose Opt.4, align RS type is not feasible when ‘CSI-RS’ is configured as measurement resource, since ‘SSB’ will serve as fallback resource for the current beam. Proposal 1.3: Support. We do not see the necessary to make ‘M’ value configurable and ‘M=1’ is sufficient, as explained in our contribution. But, we can discuss it further for progress. Issue 1.4: Prefer RAN2 spec. Event-based reporting has been supported for L3 handover and implemented in TS 38.331, including measurement report triggering procedure during TTT timer. Based on our assessment, this can be largely reused for the ‘window-based’ UEIBR procedure. Additionally, it is prefereable to consolidate ‘event-based’ beam reporting in one spec for implementation team to reference. We believe that capturing this in RAN2 spec can minimize the spec efforts. [Mod]: Thanks for above analysis and being flexible. Captured! |
| Ruijie | Proposal 1.1: Support. Proposal 1.2.1A: Support. FL observation-B: Support Implicit manner. Q1.2.2: Support Option-4. Proposal 1.3: Support. [Mod]: Captured! |
| Mod V30 | Captured companies input. The update is marked in red. |
| KDDI | Proposal 1.1: We are OK. Our 1st preference is Option-2, and FFS in Option-1 should be considered. Proposal 1.2.1A: Support. Proposal 1.3: We support, and we are OK with “M-th worst quality” for Event-7. [Mod]: Captured! |
| TCL | Proposal 1.1: Support. But we cancer about the UE capability in the first bullet of option-1. It seems unnecessary to introduce the UE capability. Proposal 1.2.1A: Support. Support at least to configure the RS for current and new beam(s) by RRC. Proposal 1.3: Support event-1 and event-7a. Although event-1 do not provide any information about new beams, network can know the worse channel quality of UE quickly, then network maybe can let gNB do beam refinement (P-2 procedure) or let UE do beam refinement (P-3 procedure) to reduce the probability of BFR. Proposal 1.4: Support option-1. [Mod]: Which Option-1? RAN1-spec? |
| OPPO | 1.1: Support, and we might do not need to FFS on Option-2/3 if Option-1 is supported. 1.2.1A: support to confirm WA 1.2.1B: we prefer implicit method. 1.2.2: Option-4 should be supported since it needs the minimum spec impact. 1.3: we have concerns on both Event-1 and Event-7a. Event-1: its purpose seems not align with the WID. WID clearly states that the UEI beam reporting is for 'facilitating fast beam switch'. The beam condition and following up beam reporting proposed in event-1 does not assist any "fast beam switch". Actually, on the contrary, event-1 could even slower down the beam switch. Furthermore, per companies' elaboration, it looks like the necessary condition for event-1 being workable is the system has to configure both event-2 and event-1 at the same time jointly. The event-1 is considered as one 'soft' BFR, thus L1-RSRP cannot be used as the quality metric. In BFR, hypothetical BLER is the quality metric for beam failure detection. Using L1-RSRP in event-1 would result in misalignment: reporting the current beam in event-1 would not prevent any beam failure on current beam. Event-7a: It would cause excessive beam reporting overhead compared with event-2. Compared with event 2, event-7a would introduce much more 'invalid' beam reporting that does not provide any useful information for beam switching. Q1.4: suggest to postpone the discussion to after the design of mode-A/B is concluded, at least after the design of 1st channel in mode-A and mode-B is determined. [Mod]: Captured! |
| NTT DOCOMO | Q.1.1 Support. Proposal 1.2.1A Support. Q.1.2.1B We prefer implicit manner because there would be no benefit on explicit RRC signaling. Q.1.2.2 We should consider two cases as shown in following figure. Case1 is that TRS as QCL RS configured in the indicated TCI state is QCLed with CSI-RS. Case2 is that TRS as QCL RS configured in the indicated TCI state is QCLed with SSB. For case1, Option-1/2/3 could work. On the other hand, if Option-4 is supported, same measurement result would be calculated for the indicated TCI state and certain TCI state which has same root QCL RS as the indicated TCI state (e.g., TCI state#1 and TCI state#3 in the Figure), and NW cannot obtain the difference of measurement result between indicated TCI state and certain TCI state which has same root QCL RS as the indicated TCI state. For case2, Option-2/3 could work. For Option-1, it would not work based on the current agreements since there is no CSI-RS for beam management as QCL RS. For Option-4, it also would not work because of the same reason as case1. Thus, Option-2/3 could be considered as the scheme. We note that, for Option-3, RS configured in the RRC and TRS configured in the TCI state should be one-to-one mapping. Also, Option-3 should be further clarified especially for whether the RS for current beam and RSs for new beam are separately configured or not before down-selection. If those are separately configured, RRC reconfiguration would be needed after beam switch due to change of the RS for current beam. It would not be efficient. If those are configured in one RRC configuration, e.g., if one of the RSs is indicated as current beam, the rest RSs are regarded as new beams by UE, it would be beneficial to do subsequent UEIBR without RRC reconfiguration. Q.1.3 We support Event-1 and Event-7a. We think that if those events are supported, the following enhancement would be needed. For Event-1, further study the configuration of new candidate beams and whether support to skip second UL channel for Mode A/B. For Event-7a, support to reuse the reporting format for Event-2, by adding “the RS derived from the activated TCI state with the worst quality” as the definition of current beam. Q.1.4 We support to capture the functionality in RAN2 spec. because there is no specification related to counter in RAN1 spec. [Mod]: Captured! |
| ETRI | We further address our preference which are not yet captured. Proposal 1.2.1A: Support. Q1.2.1B: We also support implicit way. Proposal 1.3: Support both evens. Q1.4: Capture RAN2 TS. We think that events related texts should be collected in one TS, otherwise BFR, LTM, UEI are separated and not easy to read. [Mod]: Captured! |
| Sony | Q1.1: We are OK with proposal 1.1. Proposal 1.2.1A: Support. Q1.2.1B: the implicit manner should be enough. Q1.3: We prefer to support event-1. Event-1 would be used for earlier indication than BFR when the quality of current beam becomes worse. Event-1 can coexist with Event-2 because use cases are obviously different. Although Event-1 would be similar use case as BFR, it would have benefits in terms of latency and signalling overhead reduction compared to BFR. [Mod]: Captured! |
| Mod Final | Captured companies input. The update is marked in red. |

## Issue 2 – UL signaling content(s)

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 2.1 | L1-RSRP report format | [116] Agreement On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP - FFS: Study and decide whether additional contents can be supported. - FFS: L1-RSRP format, e.g., absolute and/or differential value. - Note: Above does not imply to preclude discussion on L1-RSRP filtering. - The actual reported content depends on the triggering event - Support of one or multiple events will be discussed separately [117] Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case [117] Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. FL Note: - Since the L1-RSRP report format (option-3) is much aligned with legacy report, per the previous offline discussion, it seems workable to take differential L1-RSRP report as a starting point. Then, while the current beam is reported, clearly the related RSRP can NOT be the largest RSRP among reported ones, and then the related differential value seems sufficient. Meanwhile, CRI/SSBRI for current beam may be redundant. - Then, regarding the value of ‘N’, some further study may be needed based on several candidates. Proposal 2.1A: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #N is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - [FFS: Whether/how to additionally report CRI/SSBRI or TCI for the current beam.] - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, 1-bit indication for synchronization state, event ID, or cell ID). Supported by (33): ZTE, OPPO, Intel. QC, ETRI, xiaomi, MediaTek, Samsung, NEC (in principle), Nokia, vivo, IDC, Fujitsu, Google, CMCC, Futurewei, Meta, Sharp, Lenovo, CATT, CEWiT, Spreadtrum, LG (in principle), Huawei/HiSi, KDDI, Apple, Panasonic, Transsion Holdings, NICT, Honor, Ruijie, KDDI, TCL, Sony, Not supported by (1): Ericsson, Proposal 2.1B: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, down-select the following options for the candidate value of ‘N’ in RAN1#118: - Option-1: ‘1, 2, 3, 4’ - Option-2: ‘1, 2, 4’ - Option-3: ‘2, 3, 4’ - If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default. - Option-4: ‘1,2,3,4,5,6,7,8’ - Other candidate values are not precluded. Option-1 (34): ZTE, Intel, QC, ETRI, xiaomi(open to >4), MediaTek, NEC, NTT DOCOMO (only for Event-2), Nokia, vivo, IDC, Fujitsu, Lenovo, Google, CMCC, Futurewei, Meta, Sharp, Lenovo, CATT, CEWiT, Spreadtrum, LG, KDDI, Apple, NEC, HONOR, Panasonic, Transsion, Ruijie, KDDI, TCL, OPPO, Sony, Option-2 (1): OPPO, Option-3 (2): Samsung (open to >4), Huawei/HiSi(?), Option-4 (4): Ericsson, IDC, Qualcomm (open), CATT (open) |
| 2.2 | Additional content(s) —— L1-SINR | [116] Agreement On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP - FFS: Study and decide whether additional contents can be supported. - FFS: L1-RSRP format, e.g., absolute and/or differential value. - Note: Above does not imply to preclude discussion on L1-RSRP filtering. - The actual reported content depends on the triggering event - Support of one or multiple events will be discussed separately [116] Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP. FL note: Per last meeting discussion and companies input, we have the following proposal for L1-SINR study. If still not consensus, we may have to deprioritize this issue or conclude that this feature of L1-SINR based UEIBR is not supported. Proposal 2.2 (from RAN1#116-bis): On UE-initiated/event-driven beam reporting, further study L1-SINR as a measurement quantity on SSB for intra-cell, and periodic CSI-RS for beam management, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR. - FFS: Report format and trigger event, e.g., Event-2 as in L1-RSRP measurement/report - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS for channel measurement - FFS on L1-SINR to identify the best pairs of beams - FFS discuss if Rel-17 group-based beam report (GBBR) is applicable to sTRP with 2 CSI Resource Sets (S=2) - Note-1: Measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - Note-2: ZP/NZP-IMR, if configured, should have the same time-domain behavior as CMR. Supported by: Ericsson, xiaomi, Lenovo, Google, IDC, Fujitsu, Qualcomm, Nokia, NEC, CATT, CMCC, HONOR, Futurewei, Transsion, Apple (open), Ruijie, TCL, NTT DOCOMO, ETRI, Sony, Concerned/deprioritized by: Huawei/Hisi, Intel, ZTE, LGE, vivo, MediaTek, OPPO, |
| 2.3 | Other pending issues | FL Assessment: Per companies input and previous meeting discussion, further progress on the following pending issues may be difficult for now. - Issue 2.A: Whether/how to support semi-persistent CSI-RS and aperiodic CSI-RS (e.g., for event-2 or other events); - Issue 2.B: To introduce event related information (e.g., event ID or CSI reporting configuration ID) as an additional content; - Issue 2.C: To support MPE related information or UE capability value index as an additional content; - … As a FL recommendation, the corresponding discussion may be postponed till we achieve some progress on their related topics (e.g., introducing additional Events besides for Event-2, etc). If you have any views on moving forward the following issues, please share them accordingly. |

Table 2-2 Company input for Issue 2

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your views on proposal 2.1A/B~2.2, if needed. - For Issue 2.3, please review FL assessment, and any suggestions are welcome. |
| Ericsson | P2.1A: Do not support. Note that we have not agreed the value range of the offset: if may be negative. Also, even if we agree that the offset is positive, the quantization of the differential RSRP is 2dB, which means that we cannot encode a smaller difference between best and current beam than 2dB, and this is limiting in our view. Also it would be better to first finalize the content of the report before starting to list exact UCI content. [Mod]: Captured! In this proposal, we have ‘FFS: range and step size of differential L1-RSRP’, and then content that is considered is definitely based on the following agreement. [116] Agreement On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP P2.1B: As explained in our contribution, if we operate with more than 4 active TCI states, there is a need to increase the number of measurements beyond 4. Since the event-driven report is transmitted very rarely, the NW may only have a single report to determine which TCI states to activate. This depends somewhat is Event-7a is agreed or not. P2.2: Would be interesting to understand what the issues would be with L1-SINR. The only issue that was raised is how to define IMR, and this seems to a very small issue. |
| Samsung | Proposal 2.1A: support. This proposal is about the report format. The quantization resolution of the differential RSRP and other design details can be further discussed. [Mod]: Captured! Proposal 2.1B: We are fine with expanding the candidate values of N to value(s) beyond 4, which we think would be beneficial to ensure network’s scheduling flexibility (as pointed out by Ericsson, the event-driven reporting may only occur occasionally). For exactly the same reason, we think having the network to configure N=1 (without knowing how many new beams the UE has identified) is very limiting. We prefer to have the minimum candidate value of N as large as possible – but considering other design aspects including legacy configurations, starting from 2 seems reasonable. In addition, as mentioned by companies, when always reporting the current beam is disabled by RRC, configuring N=1 enforces the UE to always report a new beam (Option-3), which is also somewhat limiting. Hence, we support Option-3 and fine with expanding the candidate value(s) of N to those beyond 4. [Mod]: Captured! Issue 2.2: We are open to discuss whether/how to support L1-SINR. Maybe a first step forward would be to look into potential impacts of measurement resource configuration given what we have now for Event-2 evaluation. Issue 2.3: ok to postpone the discussions. |
| Huawei, HiSilicon | Proposal 2.1A: Generally, support differential RSRP report and inclusion of CRI/SSBRIs in the report. We have the following comments though: - If RRC parameter is enabled, Differential L1-RSRP for current beam can be reported as the last reported parameter without reporting the corresponding CRI/SSBRI as the gNB knows already that the current beam must be reported. This is correctly captured in the last entry of the Table in Proposal 2.1A. However, if RRC parameter is disabled, gNB does not know whether or not the current beam would be reported among the N beams. So, when RRC parameter is disabled, CRI/SSBRI of all beams should be reported including that of the current beam if the current beam is among the N reported beams. This is also correctly captured in the first N entries of the Table. Given that the Table is already completely clear about when the CRI/SSBRI of the current beam should be reported, it seems that there is no need to keep the FFS regarding “Whether CRI/SSBRI or TCI should be additionally reported for the current beam”. - As pointed out in the last FFS of the proposal, additional report content may be necessary to facilitate reporting Events other than Event-2, if agreed, or cross CC reporting. We think this needs to be captured in the Table as well. Based on the above comments, we suggest the following update. Updated Proposal 2.1A: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Other contents are not precluded. . - Differential L1-RSRP #N is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - FFS: Whether CRI/SSBRI or TCI should not be additionally reported for the current beam. - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) for cross-carrier report (e.g., cell id), or when multi-event(s) are approved (e.g., event id or report config id). [Mod]: Captured! Let’s check other companies input. Proposal 2.1B: Support. Prefer option-3. Proposal 2.2: Not support. We think L1-SINR is not needed as the measurement of L1-SINR is too restrictive in current spec. In particular, each CMR (i.e., channel measurement resource) is associated with only one IMR (i.e., interference measurement resource) for L1-SINR measurement. However, due to the flexibility of MU scheduling, the possible interference beam that affects the serving beam is not fixed. Since L1-SINR based on a particular IMR is too restrictive and cannot provide any useful information for MU scheduling, it is not necessary to be supported for UE initiated/event driven beam reporting. [Mod]: Got it. Proponent companies please reviews HW’s concerns. |
| Xiaomi | Proposal 2.1A: Support the proposal. As for the 1st FFS, we don’t think the CRI/SSBRI for current beam is needed. And for the 2nd FFS, why need to report additional indication? Does it mean gNB can’t know such information based on reported RSRP because of the quantization? [Mod]: Thanks for your input. ‘1st FFS’ is in bracket. Then, for 2nd FFS, please review pre-meeting companies input: in short, it is to inform gNB of which beam is stable for a long-duration, i.e., satisfy the condition if M>1. Proposal 2.1B: Support Option-1 and open to discuss whether N larger than 4 can be supported or not. [Mod]: Thanks for being flexible. Proposal 2.2: Support L1-SINR |
| Intel | We support Proposal 2.1A and 2.1B (our preference is Option-1). For Proposal 2.2, we agree with FL assessment that L1-SINR can be deprioritized for UE initiated beam management. [Mod]: Got it. Proponent companies please check Intel’s concerns. For Issue 2.3, we agree with FL assessment that we can postpone the discussions for these issues. |
| ZTE | Proposal 2.1A: Support. It is proper to adopt the legacy report format as much as possible for Option-3. Proposal 2.1B: Support Option-1, which is inline with the legacy report format. Regarding Option-4, our understanding is that it is irrelevant to the number of activated TCI states as mentioned by Ericsson even though we are also the proponent of Event-7, due to this proposal is dedicated to Event-2 as stated in the main bullet. [Mod]: Good point! Let’s focus on Event-2 firstly. Proposal 2.2: Fine to study, we prefer to deprioritize this discussion and focus on other critical issues that we encountered. [Mod]: Got it. Proponent companies please review ZTE’s concerns. Issue 2.3: Tend to agree with FL to postpone these next-level issues. |
| Lenovo | Proposal 2.1A: Support. Proposal 2.1B: Support Option-1. Issue 2.3 2.2: OK to study. |
| Google | Proposal 2.1A: We generally support this proposal. But for each entry in the format table, “if reported” seems to be removed per other company request. But we doubt whether need to remove it, since for beam report table in 38.212, “if reported” is present for each entry. Proposal 2.1B: As we mentioned in offline, Option 3 is ambiguous. If N is not configured and current beam is configured to be reported, what should UE report? Current beam or one new beam or both? Proposal 2.2: We support this proposal. |
| Mod V09 | Captured companies input. The update is marked in red. |
| IDC | P2.1A: Support P2.1B: Support Option-1 and open for Option-4. P2.2: OK for further study. [Mod]: Captured! |
| LG | P2.1A: Regarding FFS, it is not clear to us why CRI of current beam is reported. [Mod]: A list of concerns are received. If views are diverged, the highlighted FFS will be removed next round. P2.1B: Support. P2.2: It should be second priority. We prefer to discuss first L1-RSRP based UEI beam report. |
| Fujitsu | Proposal 2.1A: Support. Proposal 2.1B: Option-1 is preferred. Proposal 2.2: Support. L1-SINR has been supported in the legacy beam reporting. It can be also supported for the event-driven beam reporting. Issue 2.3: Agree with the assessment. [Mod]: Captured! |
| Spreadtrum | Proposal 2.1A: Support. Proposal 2.1B: Fine with Option-1. [Mod]: Captured! |
| Qualcomm | Proposal 2.1A: We generally support the proposal. Proposal 2.1B: We support the proposal. We support Option-1, but we are also open for Optiohn-4. Proposal 2.2: We support the proposal. Leveraging the legacy CSI framework, we think this is a low-hanging fruit. [Mod]: Captured! |
| vivo | P2.1A: Support, however the “Note: Other contents are not precluded” could be misleading, it should be FFS, of course if there is critical information to include should be fine. [Mod]: Personally I fully agree with you. But, for making other comfortable, it is not a big deal! P2.1B: Support Option-1 P2.2: deprioritize [Mod]: Captured! |
| Nokia | P2.1A: Agree that L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP, as in legacy behavior. Other values may be differential. However, differential value for L1-RSRP for current beam may not be sufficiently precise (depending on range and step size of differential L1-RSRP), therefore absolute value for current beam may be preferred. [Mod]: Before updating, please check other companies’ input. @all, please review Nokia’s comment on supporting absolute L1-RSRP for current beam. P2.1B: Support P2.2: Support to further study L1-SINR for UEIBM and the applicability of GBBR |
| NEC | Proposal 2.1A: Support in principle. Proposal 2.1B: fine with option 1 and thanks FL for capturing it already. Proposal 2.2: Support in principle. [Mod]: Captured! |
| MediaTek | Proposal 2.1A: Support with updating on the third FFS - FFS: Additional report content(s) (e.g., reporting configuration ID, 1-bit indication for synchronization state, event ID, or cell ID) for cross-carrier report (e.g., cell id), or when multi-event(s) are approved (e.g., event id or report config id).. [Mod]: Okay, let’s make each side equal happy. Done! Regarding the third FFS, we are open to study any report content in addition to DL-RS ID and L1-RSRP. Considering there are several issues which may need additional content haven’t been finalized, it’s better to have general wording to catch the study of additional report content(s). On the first and the second FFS, we think those are not needed. For the first FFS, NW definitely knows that the RS ID of the current beam, and there has the fixed field dedicated to L1-RSRP value of the current beam, such that indicating CRI/SSBRI for the current beam is redundant. For the second FFS, we understand the intention is to deal with the case that a non-satisfied beam has higher RSRP than a satisfied beam when reporting is triggered, if counter evaluation with a window is applied. However, it can be naturally prevented by UE implementation. As we mentioned before, UE may determine the L1-RSRP by averaging multiple measurements within that window, which is transparent to NW. Under such operation, it is unlikely that the L1-RSRP of non-satisfied beam is higher than the L1-RSRP of the satisfied beam. Alternatively, UE prevents such confusing reporting by not selecting the non-satisfied beam, to always make sure the one with the highest L1-RSRP shall be the satisfied beam. Proposal 2.1B: okay to support Option-1 as the starting point Issue2.2: Prefer to deprioritize it L1-RSRP seems sufficient on supporting Event-2, and we prefer to focus on the other essential issues firstly. [Mod]: Captured! |
| CATT | Proposal 2.1A: We are ok to support the differential reporting as a start point. Proposal 2.1B: We are ok to support N>4 case. It is our view that as long as UEIBM can offer more beam knowledge to gNB, it is beneficial (without introducing much impact). One comment is that we do not support the option(s) that precludes N=1 case. We notice that a few companies argue that N=1 may be troublesome when RRC to indicate whether current beam is always reported is enabled, however, this is not an issue by correct gNB configuration. If that RRC is enabled, gNB just does not configure N=1. But N=1 should always in the candidate pool for gNB to configure. [Mod]: Captured! Proposal 2.2: We are ok to support L1-SINR. Issue 2.3: It is our view that if multiple trigger events are supported, event-ID should be reported. Such discussion should be included in signaling container instead of being postponed. [Mod]: Already highlighted in Proposal 2.1A. |
| CMCC | Proposal 2.1A: We agree with Google that the “if reported” is needed to consider the case when N<4 is configured by RRC, some of the rows are not reported. [Mod]: Different from the related table in current spec, in this proposal, we use ‘N’ (the number of reported RSs) directly. So, ‘if reported’ is not needed. Proposal 2.1B: Support Option-1 at least for Event-2. Proposal 2.2: Support. Legacy periodic beam measurement and reporting and UEI may be used together to facilitate reliable and timely beam management. Currently, both L1-RSRP and L1-SINR are supported for legacy beam measurement and reporting. If the QCL RS of indicated TCI state is derived based on the L1-SINR beam reporting, while UEI is based on L1-RSRP, it is possible that the best beam for L1-RSRP is different from the best beam for L1-SINR, which will cause frequently beam reporting and ping-pang beam switching. [Mod]: Captured. |
| Sharp | Proposal 2.1A: Support. Proposal 2.1B: Support Option 1. Proposal 2.2: We prefer to first discuss the overall design of L1-RSRP based UEIBM. |
| Mod V21 | Captured companies input. The update is marked in red. |
| Huawei, HiSilicon2 | Proposal 2.1A: After further thoughts, we realized that there is a problem with including SSBRI/CRI of the current beam in the report: Current beam can be uniquely identified by its associated SSBRI/CRI only if gNB can include the current beam (ie, the QCL source RS of the indicated TCI state or the SSB QCLed with the QCL source) in the RRC configured RS resource set for the new beam measurement since, at least in the current specifications, the reported RS index is the local resource index within the configured resource set for measurement. However, this may not be possible for gNB since, for instance, gNB does not know the indicated TCI state at the time of RRC configuration of the CSI report. To capture this issue, we suggest the following modification in the proposal: Updated Proposal 2.1A: On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #N is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - [FFS: Whether/how CRI/SSBRI or TCI should be additionally reported for the current beam.] - Note: Current beam can be identified by its associated index only if gNB can include the current beam (ie, the QCL source RS of the indicated TCI state or the SSB QCLed with that QCL source) in the RRC configured RS resource set for the new beam measurement - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, 1-bit indication for synchronization state, event ID, or cell ID) for cross-carrier report (e.g., cell id), or when multi-event(s) are approved (e.g., event id or report config id). [Mod]: This proposal has been stable for a long time, and, in my views, capturing above may be immature. Anyway, we will further study that per FFS. Others are done! Other companies please review HW’s input as above |
| HONOR | Proposal 2.1A: Support. The first FFS in bracket is not needed. Not necessary to add CRI/SSBRS or TCI for current beam in the report since it has already been known by network. Regarding the third FFS, we prefer to remove it for now after we finalize the Q3.2. If the first channel is associated with the report configuration ID, it’s not necessary to include the report configuration ID in the second channel. Including the report configuration ID may increase the network blind detection complexity and potential spec impact on CSI priority rules. Proposal 2.1B: Support Option 1 as legacy. Proposal 2.2: OK to support L1-SINR as a measurement quantity. [Mod]: Captured! |
| FUTUREWEI | Proposal 2.1A: We are fine with the proposal in principle. However, as other companies mentioned, the exact contents of the report still need to keep open before format details of first channel and second channel of Mode A and B decided. Proposal 2.1B: We are fine with the proposal, and at least the legacy number (N) (i.e., 1, 2, 3, 4) of measured RS resources to be reported should be supported. Issue 2.2: We are open to support L1-SINR as in legacy beam measurement report. Issue 2.3: We are fine to postpone the discussions before additional Events are decided. [Mod]: Captured! |
| Panasonic | Proposal 2.1A: For N=1, L1-RSRP can be omitted in some cases since the NW knows that it is higher than the threshold and it is the only one reported. So we would prefer further discussion. [Mod]: Before the above is captured, let’s check other companies input. Currently, there is no similar input considering that this proposal is stable for a long time. Proposal 2.1B: Option 1 seems reasonable to us. [Mod]: Captured! |
| Transsion | Proposal 2.1A: Support. It seems unnecessary to include CRI/SSBRS for current beam in the report. Proposal 2.1B: Support Option 1. Proposal 2.2: We are ok to support L1-SINR. [Mod]: Captured! |
| Apple | Proposal 2.1A: Support. Proposal 2.1B: Support Opt.1 Proposal 2.2: Open to discuss. [Mod]: Captured! |
| Ruijie | Proposal 2.1A: Support. Proposal 2.1B: Support Opt.1 Proposal 2.2: Support. [Mod]: Captured! |
| Mod V30 | Captured companies input. The update is marked in red. |
| KDDI | Proposal 2.1A: We support. Proposal 2.1B: We support Option-1, but we are open for N>4. [Mod]: Captured! |
| TCL | Proposal 2.1A: Support. Proposal 2.1B: Support option-1. Proposal 2.2: OK. [Mod]: Captured! |
| OPPO | 2.1A: support 2.1B: Ok with Option 1. 2.2: no support. We should focus on L1-RSRP to complete the Rel-19 UEI BM first, instead of considering L1-SINR now. [Mod]: Captured! |
| NTT DOCOMO | Proposal 2.1 We are fine in principle. Proposal 2.1B We support Option-1 only for Event-2. Proposal 2.2 Support L1-SINR [Mod]: Captured! |
| ETRI | Our preference is correctly captured in the previous round: proposal 2.1A (support) and 2.1B (option-1). In addition, Proposal 2.2: Support L1-SINR. [Mod]: Captured! |
| Sony | Proposal 2.1A: Support. Proposal 2.1B: support Option 1 Proposal 2.2: support L1-SINR [Mod]: Captured! |
| Mod Final | Captured companies input. The update is marked in red. |

## Issue 3 – UL signaling medium/container

Table 3-1 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.1.1 | Resource allocation on Mode-A and Mode-B —— To confirm WA | [117] Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Q3.1.1: Please share your views on whether/how to confirm the above WA: FL Observation: - Confirmed by (33): Samsung, Nokia, Qualcomm, Ericsson, Spreadtrum, Intel, xiaomi, CATT, Lenovo, TCL, RUIJIE NETWORKS, ASUSTeK, Huawei/HiSi, ZTE, IDC, Fujitsu, Spreadtrum, Qualcomm, Nokia, MediaTek, CATT, CMCC, Sharp, HONOR, Futurewei, Panasonic, Transsion, Apple, OPPO, NTT DOCOMO, ETRI, Sony, - Not confirmed by (1): vivo, |
| 3.1.2 | Resource allocation & UCI type on first PUCCH for Mode-A and Mode-B | [117] Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. c.f. TS 38.331 Section 6.3.2 Radio resource control information elements: FL Note: - Let’s try to stabilize candidates (as an outcome from pre-meeting discussion) as much as possible. Please review companies’ input on pros/cons for each candidate. Proposal 3.1.2: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration: - For Mode-A, down-select one of the following - Alt 1-1 (SR-like, being associated with SchedulingRequestId): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with SchedulingRequestId. - FFS: UCI multiplexing rule, e.g., the same as legacy SR, or priority rule if overlapping with normal SR(s)/LLR. - FFS: whether the one-bit indication should be dropped or not, if overlapping with PUSCH. - Alt 1-2 (new UCI type/RRC confg): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource - The dedicated RRC parameter comprises the following: - periodicityAndOffset - PUCCH-ResourceID - FFS: prohibitTimer - FFS: transMax - For Mode-B, down-select one of the following - Alt 2-1(SR-like, i.e., being associated with SchedulingRequestId): Introduce RRC parameter(s), e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with SchedulingRequestId. - FFS: UCI multiplexing rule, e.g., the same as legacy SR, or priority rule if overlapping with normal SR(s)/LLR. - FFS: whether the one-bit indication should be dropped or not, if overlapping with PUSCH. - Alt 2-2(new UCI type/RRC confg): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource - The dedicated RRC parameter comprises the following: - periodicityAndOffset - PUCCH-ResourceID - FFS: prohibitTimer - FFS: transMax Mode-A - Alt 1-1 (SR-like, i.e., being associated with SR-id) - Supported by (22): ZTE, OPPO, E///, Intel, ETRI, xiaomi, vivo, IDC, Fujitsu, Apple, Lenovo, CMCC, Futurewei, Meta, Sharp, Spreadtrum, NTT DOCOMO, Honor, NICT, Huawei/HiSi, Transsion, TCL, - Alt 1-2 (new UCI type/RRC confg, i.e., not being associated with SR-id) - Supported by (9): Samsung, Qualcomm, NEC, Nokia(?), Google, CEWiT, LG, Panasonic, Sony, Mode-B - Alt 2-1 (SR-like, i.e., being associated with SR-id) - Supported by (9): ZTE (strive for unified design as mode-A), Intel, ETRI, vivo, Fujitsu, CMCC, Futurewei, Spreadtrum, Honor - Alt 2-2 (new UCI type/RRC confg., i.e., not being associated with SR-id) - Supported by (22): OPPO, E///, Qualcomm, xiaomi, Samsung, NEC, Nokia, IDC, Apple, Lenovo, Google, Meta, Sharp, CEWiT, LG, NTT DOCOMO, NICT, Huawei/HiSi, Panasonic, Transsion, TCL, Sony, |
| 3.2 | Details on Step-2&3 in Mode-A —— triggering procedure | [117] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH. - The UL-grant DCI format at least comprises DCI format 0_1/0_2. - FFS: DCI format 0_3 - FFS: How to trigger the UEI beam report by the UL-grant DCI format - FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. - The DL-grant DCI format at least comprises DCI format 1_1/1_2. - FFS: DCI format 1_3 FL Note: Essential issue for Mode-A is to specify the triggering procedure, especially which field in the UL-grant DCI is used for triggering the corresponding UEIBR in the PUSCH of step-3. Then, re-Tx of first/second channel transmission and the related gNB response, if any, will be handled in the next round after this issue is stable. Q3.2: Please share your views on details on step-2&3 in mode-A. For triggering procedure, per companies input, we may have the following two options: - Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - Option-2: Associate an AP-CSI trigger-state with the UEI beam report procedure, and, once the trigger-state is indicated by the CSI request field in DCI format 0_1/0_2, the UEI beam report is delivered by the scheduled PUSCH - Option-3: Assign a codepoint (e.g., configured by RRC) in CSI request field for the UEI beam report procedure, and, once the codepoint is indicated by the DCI format 0_1/0_2, the UEI beam report is delivered by the scheduled PUSCH Note: the difference between Option-2 and Option-3 is whether to reuse RRC framework of associating AP-CSI trigger state and a given CSI report config. Besides, please review your views on the other left-over issues: - #1 Whether/how to support DCI format 0_3, besides for DCI format 0_1/0_2. - #2 Whether/how to trigger the UEI beam report by the DL-grant DCI format. FL Observation: There is following observation for triggering procedure, DCI format 0_3 and enabling DL-grant DCI format to trigger the UEI beam report. - Regarding triggering procedure: - Option-1 (Introducing a new 1-bit field): Samsung, Huawei/HiSi, ZTE, Transsion Holdings, Sharp, NTT DOCOMO, - Option-2 (Reusing AP-CSI trigger state): MediaTek, ZTE, vivo, CMCC, Intel, Sony, xiaomi, Lenovo, Google, IDC, LG, Fujitsu, Spreadtrum, Qualcomm, vivo, NEC, MediaTek, CMCC, HONOR, Panasonic, Transsion, Apple, OPPO, ETRI, Sony, - Option-3 (Assigning a codepoint in CSI request field): Qualcomm, Spreadtrum, NEC, Transsion Holdings, Ericsson, IDC, Spreadtrum, Qualcomm, NEC, Futurewei, Transsion, Apple, OPPO, NTT DOCOMO, - Regarding DCI format 0_3: - Supported by: Ericsson, Spreadtrum, Samsung, Intel, Google, Spreadtrum, NTT DOCOMO, - Not supported by: Huawei/HiSi, Lenovo, CMCC, Panasonic, - Postponed by: ZTE, Fujitsu, HONOR, - Regarding DL-grant DCI format: - Supported by: Samsung, - Not supported by: ZTE, Spreadtrum, Intel, ETRI, Sharp, Lenovo, Fujitsu, Spreadtrum, MediaTek, CMCC, HONOR, Panasonic, NTT DOCOMO, |
| 3.3.1 | Details on Step-2 in Mode-B —— second channel | [117] Outcome of RAN1#117 offline in R1-2405638 Proposal 3.3A (PUSCH+PUCCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 can be either PUSCH or PUCCH. - FFS: Signaling design for PUSCH and PUCCH configuration - Note: For a given UE, only one of the two types of resources can be pre-configured for UEIBM report. - Introduce UE capability signaling to indicate support of PUSCH, PUCCH, or both Supported by: Huawei, ZTE (1st priority), QC, CMCC, Sharp, Samsung (1st priority), Fujitsu (2nd priority), Nokia, Intel, LG, Lenovo, NEC(2nd priority), DCM, Panasonic(2nd priority), Qualcomm (1st priority), xiaomi, CEWiT Concerned by: MTK, OPPO Okay with condition: vivo: only okay for CG-PUSCH Proposal 3.3B (At least PUCCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least PUCCH. - FFS: PUSCH as the second channel Supported by: DCM, CeWiT, Sharp, vivo, Panasonic, TCL, xiaomi, LG (2nd priority), FW, NEC, Qualcomm (2nd priority), Futurewei, Concerned by: ZTE, CATT, Nokia Proposal 3.3C (At least PUSCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least PUSCH. FFS: PUCCH as the second channel Supported by: OPPO, MTK, ZTE (2nd priority), Intel (2nd priority), ETRI, Samsung (2nd priority), vivo (CG-PUSCH), Nokia, Fujitsu, Apple, Google, CMCC, Meta, Sharp, Lenovo, CATT, Huawei/HiSi, Concerned by: FL Note: - Firstly, per companies’ input, majority companies prefer to support both PUSCH and PUCCH as pre-configured resource for second channel in Step-2, but a few companies provided strong concerns due to that the benefit of additionally supporting both rather than using a single one, like PUSCH, is questionable. - Then, per previous offline/online, it seems that no companies provide strong concerns on supporting ‘PUSCH’ firstly, even though, during RAN1#117 online, in some companies’ views, there’s been too big a rush to make the down-selection. - The following proposed compromise seems what we can have for now. Proposal 3.3.1 (at least PUSCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data Supported by (29): OPPO, MTK, ZTE (2nd priority), Intel (2nd priority), ETRI, Samsung (2nd priority), vivo, Nokia, Fujitsu, Apple, Google, CMCC, Meta, Sharp, Lenovo, CATT, Huawei/HiSi, Spreadtrum, Sony, NICT, Ericsson, xiaomi, Intel, IDC, Qualcomm, NEC, HONOR, Futurewei, Transsion, TCL, Panasonic, NTT DOCOMO (2nd choice) - CG-PUSCH: vivo, MediaTek (Type-1), Huawei/HiSi (Type-1), ZTE (Type-1), Samsung, Apple, Intel, Panasonic, Concerned by: Panasonic (PUCCH or PUCCH + PUSCH) |
| 3.3.2 | Details on Step-2 in Mode-B —— association between first and second channel | [116b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - … - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FL Note: From the transmission procedure perspective, which transmission occasion of the pre-configured second UL channel resource(s) is indicated by the first PUCCH for carrying UEI beam report should be specified. Then, re-Tx of first/second channel transmission and the related gNB response, if any, will be handled in the next round after this issue is stable. Q3.3.1 (resource allocation/RRC Configuration): Please share your views on details on resource mapping/configuration between first and second channel in mode-B. Per companies input, we may have the following options: - Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with a UEI beam report configuration. - Option-1A: Same periodicity between first and second channel - Option-1B: No restriction in terms of periodicity. - Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with a UEI beam report configuration. - Option-3 (M-to-one): One or more first PUCCH resource(s) and only one pre-configured resource for second UL channel can be associated with a UEI beam report configuration. - Option-4 (M-to-M): One or more first PUCCH resource(s) and one or more pre-configured resource(s) for second UL channel can be associated with a UEI beam report configuration. Q3.3.2 (Tx occasion determination): Please share your views on details on the reporting procedure for mode-B: - UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the report notification on the first PUCCH channel. - FFS: X is configured by RRC, or fixed (e.g., 0). FL Observation: There is following observation for resource mapping/configuration between first and second channel in Mode-B in Q 3.3.1. - Option-1 (one-to-one): Huawei/HiSi, Ericsson, Lenovo, Fujitsu, NEC, Sharp, Apple, OPPO, NTT DOCOMO, - Option-1A (Same periodicity): Huawei/HiSi, Intel, Spreadtrum, Qualcomm, vivo, MediaTek, CATT, Transsion, Apple - Option-1B (No restriction): ZTE, Samsung, Intel (open), IDC, - Option-2 (one-to-M): Samsung, xiaomi (same periodicity), Lenovo (Optional), Google, IDC, CATT, CMCC, Futurewei, Sony, - Option-3 (M-to-one): Nokia, Fujitsu, - Option-4 (M-to-M): HONOR, Then, the following companies can support the reporting procedure mentioned in Q3.3.2: - Supported by: Samsung, Huawei/HiSi, MediaTek, ZTE (X=0), IDC, CATT(?), xiaomi(?), Lenovo, IDC, LG, Fujitsu, Qualcomm, vivo, CATT, Futurewei (open), Transsion, Apple, NTT DOCOMO, - Not supported by: CMCC, OPPO, |
| 3.4 | Cross-carrier UE initiated beam report | [116b] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following options are supported: … For above procedures, cross-CC beam reporting is supported for both options. - FFS: Details. FL note: Before discussing the report content (i.e., a CC id or any other implicit manner), we may need to discuss whether/how to enable cross-CC beam measurement and report depending on Event-2. Q3.4.1: Please share your understanding on how to enable cross-CC RS measurement (involving both current beam and new beam e.g., from the same CC or not)) on event-2. - For current beam - Option-1: To configure RRC parameter (e.g., carrier/servCellIndex) about which CC the indicated TCI state is from, e.g., carrier/servCellIndex - … - For new beam - Option-1: To configure RRC parameter (e.g., carrier/servCellIndex) about which CC the indicated TCI state/RS resource(s) are from - … FL Observation: There is following observation for Q3.4.1. - For current beam: - Option-1: Huawei/HiSi, Fujitsu (?), Qualcomm (RS id + cell id in TCI state), MediaTek (may be different from a CC of new beam), CATT, ETRI - … - For new beam: - Option-1: Ericsson, Samsung, Huawei/HiSi, xiaomi, ZTE, Lenovo, Google (?), Fujitsu, Qualcomm, MediaTek, CATT, NTT DOCOMO (as legacy), ETRI - … Q3.4.2: Then, please share your view on whether, for respective Mode-A and Mode-B, the cross-CC beam report should be carried in the second UL channel in a same CC as the related UEI report configuration, as usual. FL Observation: There is following observation for Q3.4.2. - Yes: Ericsson(?), Samsung, xiaomi, Lenovo, Google, Fujitsu, Qualcomm, MediaTek, CATT, NTT DOCOMO, ETRI, Sony, - No: ZTE, . |

Table 3-2 Company input for Issue 3

| Company | Input |
| --- | --- |
| Mod V00 | Please input your comment/preference to those proposals and questions on the issue 3.1~3.4, if needed. |
| Ericsson | Q3.1.1: Confirm the WA P3.1.2: OK, will need to read some TDocs to understand the pros and cons. Feels like we need all the RAN1 functionality of an SR for this channel, so why not make it an SR? [Mod]: A little bit confusing after reviewing the above. Would you like to update your preference? Q3.2: This feels somewhat like an RRC configuration issue: option-3 sounds similar to the approach we used for LTM. To us, we solve many problems if we use a legacy AP-CSI triggered report. Also, the report configuration may reuse at least some of the fields in the legacy CSI report. Will need to read some TDocs to understand the benefits of using another approach. P3.3.1: OK [Mod]: Thanks for being flexible. Q3.3.1: Option-1 sounds natural: what would be the benefit of having a more first channels than second channels, and vice versa? What would be the benefit of having different periodicities of first and second channel? [Mod]: TBH, I have the similar questions as you mentioned. Captured! Q3.3.2: This is not needed if we go for option-1 in Q3.3.1, I guess? Then the first and second channels should be configured accordingly. [Mod]: Guess not. Q3.3.1 is relevant to resource allocation, and then Q3.3.2 is relevant to transmission occasion. Update a little bit for avoiding ambiguities. Q3.4.1: As in legacy, the resource configuration contains information about which CC the resources are located on. Q3.4.2: As in legacy, the first and second UL channels are located in the serving cell where the report is configured. The resources can be in another serving cell. |
| Samsung | Issue 3.1.1: We support to confirm the WA. Proposal 3.1.2: instead of listing potential specification impacts (such as UCI multiplexing rules), we would prefer to first look into critical issues (or pros and cons) of using SR or new UCI type for the first PUCCH channel in both Mode-A and Mode-B. For SR, we have identified at least the following issues: - According to the definition of SR captured in TS 38.321, it is very clear that SR is used for requesting UL-SCH resources for new transmission which is for UL data only. However, for Mode-A, the first PUCCH is to request a resource for a second UL channel to carry beam report UCI instead of UL data. For Mode-B, the first PUCCH is for notifying a second UL channel to carry beam report UCI. It is clear that the usage of the UCI in the first PUCCH channel for both Mode-A and Mode-B is not aligned with the existing SR. - If the SR is dropped when overlapping with a PUSCH, the second channel cannot be transmitted; this is different from BFR MAC CE as in this case (i.e., the corresponding LRR is dropped), the BFR MAC CE can be directly transmitted in a PUSCH (for new transmission). - How the SR can be applicable to multi-CC or potential multi-event beam reporting setting is questionable, which needs to be further examined. Can the proponents of SR clarify the above? Especially for Mode-B, the usage of the UCI in the first PUCCH is definitely not aligned with the existing SR. Opponents of new UCI type seem more concerned about specification impacts (which can be minimized by leveraging legacy designs) but not about any critical issues of using the new UCI type (?). [Mod]: @SR proponent, please review the above comments. Issue 3.2: - Regarding triggering procedure, introducing a 1-bit field is more straightforward - Regarding DCI format 0_3: support; there seems no reason not to support it for multi-carrier scenario - We see use cases of using DL-grant DCI format: comparing with only having PUSCH as the second UL channel, additional support of PUCCH for the UEI beam report is beneficial for overhead reduction for PDCCH transmission; note that reduction of signalling overhead is also required by the objective of the WID. If only PUSCH is supported for UEI beam report for Mode A, the network would always need to schedule a PUSCH transmission with or without UL data arriving for a UE when detecting a first PUCCH from the UE. Consider a typical case of DL heavy traffic scenario and there is only DL data arrived for a UE: if only PUSCH is supported for the second UL channel, when the network detects the first PUCCH from the UE, the network needs to transmit PDCCH with a DL DCI format scheduling a PDSCH reception and a UL DCI format to schedule a PUSCH for the UEI beam report. For the same setting, if PUCCH can be supported for the second UL channel, the DL DCI format for scheduling a PDSCH reception can also be used to indicate the UEI beam report transmission in a PUCCH; in this case, there is no need to transmit the UL DCI format to schedule a PUSCH thus the signaling overhead can be reduced. Proposal 3.3.1: We are fine with supporting at least PUSCH first though we think in terms of resource configuration/association, there is no difference between PUCCH and PUSCH. Issue 3.3.2: having different periodicities between the first and second UL resources is beneficial to provide better scheduling flexibility. In this case, only a minimum time gap needs to be configured, which also allows enough processing time for the gNB. Furthermore, in TDD, it is possible that some of the configured second UL resources for beam reporting can collide with DL symbol(s); in this case, the second UL resource(s) that collides with the DL symbol(s) cannot be used for transmitting the beam report. Configuring different periodicities with a minimum time gap along with one-to-many mapping can effectively solve this resource collision issue (one conceptual example is provided below for reference). Q3.4.1: Share the same understanding as Ericsson. The current resource configuration can indicate measurement RS resources for a CC. Q3.4.2: Our understanding is that the cross-CC beam report should be carried in the second UL channel in a same CC as the related UEI report configuration. Do not see clear benefits or usage of otherwise. [Mod]: Captured! |
| Huawei, HiSilicon | Q3.1.1: Support confirming the WA. Proposal 3.1.2: Support the proposal. For Mode-A, we are supportive of Alt 1-1. The first PUCCH has a very similar functionality as the legacy SR as it requests resources for PUSCH and is followed by the NW response (DCI) that indicates the assigned PUSCH resources. For Mode-B, the first channel is only a notification for the second channel and is not followed by the NW response. Therefore, Alt 2-2 seems more appropriate: A new RRC parameter could be introduced that assigns periodic PUCCH resources to the one-bit UCI used for notifying the second channel. In any case, we think it is better to have more discussions about all four alternatives before down-selection. Q3.2: We prefer Option-1 as it is more straightforward. Also, to make Option-2 functional, it seems that additional conditions/restrictions are required. To explain this, let us assume that M UEIBR configuration with indexes 0,…, M-1 are configured and UE needs to provide the report associated with UEIBR configuration with index m. Further, UEIBR configuration with indexes 0,…,m,…K-1 are associated with the trigger state 0001 and UEIBR configuration with indexes K,…, M-1 are associated with the trigger state 0010. Then, there seems to be two problems: 1) unless there is a one to one mapping between the first channel resource (1-bit UCI request for report) and each UEIBR configuration index, gNB has no way to know UE intends to send the report associated with which of M UEIBR configuration with indexes (i.e, should CSI request field be 0001 or 0010); 2) Even if there is a one to one mapping between the first channel resource and each UEIBR configuration and, hence, gNB would know that, in this example, CSI request field should be 0001, all reports associated with UEIBR configuration with indexes 0,…,m,…K-1 would also be triggered by gNB. In this case, UE behavior in regards to the unwantedly triggered reports associated with UEIBR configuration with indexes 0,…,m-1,m+1,…K-1 should be clarified. For Option 3, it seems that the first problem above could be resolved by dedicating only one codepoint of CSI request field to UEIBR. However, the second problem above still persists. In addition, Option 3 seems to have a problem of its own: Since a code point is dedicated to UEIBR, it is not possible to trigger both legacy CSI and UEIBR in the same DCI. Regarding Whether/how to support DCI format 0_3, we think it is not needed since DCI 0_3 is introduced for MBSC (multiple CC scheduling with a single DCI) and the rationale to schedule multiple CC transmission for beam reporting is not clear to us. Additionally, the payload size of DCI format 0_3 is, in general, much larger than that of DCI format 0_1 and 0_2. Regarding whether/how to trigger the UEI beam report by the DL-grant DCI format, in principle, we think it is a good idea to provide more opportunities for UEIBR. However, we think the specification effort (eg, multiplexing the report with HARQ) may not be justifiable to support this additional feature. Nevertheless, we are open to further discuss this. [Mod]: Captured! Proposal 3.3.1: We can support this compromise. Q3.3.1: We support Option 1 which is simpler and use up less resources. We don’t see any strong justification for supporting multiple resources for the first or the second channel in Mode B since there won’t be any positive/negative acknowledgement from the NW side regarding the reception of the first or second channel anyways. As for Option-1A/1B, we prefer Option-1A as it is simpler and we don’t see any particular advantage regarding Option-1B. Q3.3.2: support. Q3.4.1&Q3.4.2: In legacy CSI framework, cross cell beam reporting is conducted with following configuration: A CSI-ReportConfig is configured in the cell (e.g., Cell 1) where beam report is transmitted and a CSI-ResourceConfig is configured in the cell where beam measurement is conducted (e.g., Cell 2). A ServCellIndex field is configured in the CSI-ReportConfig which indicate the cell in which the associated measurement resource is configured. To minimize spec impact of supporting cross cell beam reporting in UEIBR, above framework should be reused. Taking the following figure as an example, we provide some details on how to support cross cell beam reporting in UEIBR: Current beam RS derivation: current beam RS is the QCL RS or SSB QCLed with the QCL RS in the indicated TCI-state of the cell indicated by the ServCellIndex (i.e., Cell 2). New beam RS derivation: new beam RS is configured in the CSI-ResourceConfig of Cell 2. 1st PUCCH: the 1st PUCCH is transmitted in the cell where the CSI-reportConfig (e.g., Cell 1) is configured as usual. [Mod]: Confusing. Do you assume that this report-configuration is only in PSCell? Please review above newly added option. 2nd UL channel: the 2nd UL channel should be transmitted at least in the same cell as 1st PUCCH. But in mode A, where UL scheduling is available, it is also possible to schedule a PUSCH in another cell as the 2nd UL channel. |
| Xiaomi | Q3.1.1 Support to confirm the WA Proposal 3.1.2 Support the proposal. and prefer Alt 1-1 for Mode-A, and Alt 2-2 for Mode-B. As for the FFS in Alt 1-1 and Alt 2-1, we suggest the following update - FFS: UCI multiplexing rule, - With legacy SR, e.g, If one of the SRs is a positive UEIBR, the value of the log2K+1 bits indicates the positive UEIBR. - With other UCI type, e.g., the same as legacy SR [Mod]: Captured with a simplified description. Q3.2 Prefer Option 2. Proposal 3.3.1 We can accept it for progress if majority companies support this proposal. [Mod]: Thanks for being flexible! Q3.3.1 Prefer Option-2 to improve the reliability of the beam report. But with Option-2, the periodicity can also be same for 1st and 2nd UL channel. It means that the 1st UL channel of next M-1 period can be omitted. [Mod]: Captured! Q3.3.2 If there is only one first PUCCH resource and one preconfigured resource for the second UL channel in the same period, and the X is larger enough for gNB to receive the first PUCCH resource, it is straightforward to use the preconfigured resource in the same period. Else, it is better to define X. Q3.4.1 Share same understanding as Ericsson and Samsung that current resource configuration can be reused to indicate the CC information. Q3.4.2 Yes. |
| Intel | For 3.1.1, we agree that working assumption can be confirmed for the first PUCCH for both Mode A and Mode B For Proposal 3.1.2, we are fine with the proposal. Our preference is to consider a unified design for both Mode A and Mode B, i.e., using SR like configuration for the first PUCCH (Alt 1-1 and Alt 2-1). Functionally, either requesting the resource in Mode A or notifying the transmission of second step PUSCH would be same as conventional SR procedure. We do not see the need to introduce a new UCI type, which would complicate the design, especially the UCI multiplexing on PUSCH and PUCCH. [Mod]: As a FL, I will like to echo your comment. Unified design, otherwise, my workload is double! ^ ^ For Q3.2, we prefer Option-2 as existing CSI reporting framework can be largely reused. Further, the same design principle for LTM CSI reporting can be considered, i.e., one trigger state is associated with UE initiated beam reporting. There is no need to introduce one explicit bit indication in the DCI. Option 3 is very similar to Option 2, but RRC configuration seems complicated. We support DCI format 0_3 for UEIBM for Mode A. We do not support DL grant + PUCCH for Mode A (already discussed in Rel-17 URLLC, do not need to repeat the same discussion in MIMO) For Proposal 3.3.1 , we can be fine to move forward for the sake of progress. Our view is CG-PUSCH should be considered as starting point for the second step in Mode B. One issue that may need further discussion is the support of beam report on CG-PUSCH without UL-SCH, which is currently not supported. [Mod]: Thanks for being flexible. If my understanding is correct, w. UL-SCH should be starting point. Anyway, let’s discuss this issue in the next round. For Q3.3.1, we think Option-3 is not needed for single event/single CC case, then we should discuss options 1,2. Option -1 could be basic case and we can further discuss whether same or different periodicities can be considered for the first PUCCH and second uplink channel for Mode B. The starting point can be Option-1A with same periodicity, which can follow the same design principle for 2-step RACH where the configured slot offset is used to associate the MsgA PRACH and MsgA PUSCH. We can further discuss Option 1B, which can provide good flexibility on the configuration. For Q3.3.2, we need to wait the progress on Q3.3.1. if only one to one mapping is supported, only one the first PUCCH is associated with the second uplink channel. We do not have the concept of the first available transmission occasion of the second uplink channel. If the second uplink channel is cancelled due to collision with other channels/signals, UE does not transmit the beam report in the second channel. [Mod]: In my views, they are decoupled. Anyway, let’s review others’ input |
| ZTE | Q 3.1.1: Support to confirm the WA. [Mod]: Captured! Proposal 3.1.2: Support SR-like (e.g., being associated with SR-id) in Mode-A and Mode-B as per the following considerations: - It is reasonable to strive for a unified design of first PUCCH between Mode-A and Mode-B. For clarification, first PUCCH in Mode-A is to request resource of UEIBR that indicated by DCI, while first PUCCH in Mode-B can be seen as to request resource of UEIBR that pre-configured by RRC. In other words, the current definition of SR as specified in TS 38.321 can be evenly applied to Mode-A and Mode-B. - Compared with SR-like approach, introducing a new type of UCI will inevitably lead to terrible spec efforts, i.e., brand-new design of RRC configuration and signaling, UCI multiplexing/dropping rules, etc. Q 3.2: - Regarding triggering procedure, slightly prefer to support Option-1 for simplicity. - Regarding DCI format 0_3, we tend to postpone this discussion till we reach the outcome of cross-CCs UEIBR due to: If go with the legacy rule, AP CSI report can be triggered in the serving cell with the smallest id among serving cells scheduled by DCI format 0_3. As a result, if the first PUCCH resource is shared by report configurations across multiple CCs, the NW cannot identify the exact cell(s) on which the UEIBR is triggered. - Regarding DL-grant DCI format, we do not support due to: (1) dynamically indicating a PUCCH resource for UCI transmission by DL DCI is only specified for HARQ-ACK; (2) the necessity of additionally supporting UEIBR transmission on PUCCH indicated by a DCI formats 1_x is ambiguous. Proposal 3.3.1: Support, and we think Type 1 CG-PUSCH can be considered as the starting point. [Mod]: Captured! Q 3.3.1: Support Option-1B as per our elaboration provided in R1-2406028. We can observe some benefits of configuring a shorter periodicity of the second UL channel than the first PUCCH, e.g., more transmission occasions for UEIBR and smaller report latency. However, for the sake of decoupling of the two channels, we prefer individually configuring the periodicity of the first PUCCH and second UL channel. Q 3.3.2: Regarding the time gap between the first PUCCH and second channel, we are fine to configure its value in principle. However, we believe that the time gap is unnecessary (e.g., it can set to 0) if the second UL channel is CG-PUSCH. In legacy, when a UE determines to transmit UL-SCH on the CG-PUSCH, it does not need to notify the network of the subsequent transmission in advance. With the functionality of MU-MIMO, the network can decode the PUSCH transmissions even when the transmissions from multiple UEs collides. Hence, it is not essential to have a time gap to allow the network to prevent collisions between UEs. Q 3.4.1: We believe that the legacy CSI report configuration can be considered as a start point, where the carrier field indicates which carrier the measurement RSs of new beams are associated with. Whether the current beam can be associated with the carrier field can be further discussed. Q 3.4.2: We support that beam report can be carried in the second UL channel in a CC different from that in which the corresponding UEI report configuration is configured. Since the triggering of UEIBR of different report configurations for a UE can be quite sparse in time domain, most transmission occasions of the second UL channel are actually used if they are configured dedicatedly to a single CC or even a single report configuration. But, if taking report configurations on all the cells into account, the likelihood of at least one beam report is triggered to be carried on the second UL channel can be much higher, improving the uplink resource efficiency. [Mod]: Captured! Furthermore, in Mode A, if the first PUCCH resource is associated with multiple UEIBR report configurations of multiple CCs, network cannot identify the cell(s) associated with the triggered UEIBRs via reception of the first PUCCH, hence the network might schedule a PUSCH on any cell to carry the UEIBRs. [Mod]: By default, we may assume that PUCCH is in PSCell, not relevant to the cell where the report configuration is! |
| Lenovo | Q3.1.1: Support to confirm the Work Assumption. P3.1.2: Support. For-Mode A, SR-like mechanism should be sufficient. For Mode-B, new UCI type is preferred because new timeline between the notification message and the corresponding beam report may needed to be specified. Q3.2: Regarding the triggering procedure, we support option 2. Regarding issue#1: We don’t see the need to further support DCI format 0_3 at this stage. Regarding issue#2: Does it mean to support PUCCH for the beam report in step 3 of Mode-A? And we don’t support DCI format 0_3 and DL grant DCI. P3.3.1: Support. Q3.3.1: We understand Option-1 should be the basic feature. And option-2 can be an option feature. Option-3 seems unnecessary when only event 2 is supported. Q3.3.2: Support that UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the report notification on the first PUCCH channel when one notification resource is associated with M UL resources for beam report, where X is related to UE capability. Q3.4.1: Similar with the legacy beam report, the RS resources for new beam identification can be configured in different serving cells or even in non-serving cells for ICBM. Q3.4.2: Yes. [Mod]: Captured! |
| Google | Proposal 3.1.2: For Mode-A, Alt 1-2 is beneficial over Alt 1-1. Directly reusing or revising SR resource can only convey the idea of whether UE is requesting/notifying UL resource for UEI-BR. When receiving the first PUCCH, NW does not have any clue on potential payload size of the UEI-BR, especially for Mode-A. However, using a PUCCH transmission with one bit UCI can at least convey the range of the payload size of UEI-BR by indicating bit 0 or 1. In addition, the UCI multiplexing rule of the first PUCCH should be different from SR. Introducing a new RRC configuration would be more sensible. Hence, we support Alt 1-2, and support Alt 2-2 for similar reasons. [Mod]: Captured! Q3.2: Option-1 and 3 look incomplete without mentioning which triggered UEI CSI report configuration can be transmitted on the scheduled PUSCH. Option-2 could be OK as starting point for single CC report. For cross-CC report, we may need further discussion, e.g., how to trigger UE to report multiple CC results in a UEI beam report. Also, we are fine with support DCI format 0_3 for Mode-A. Q.3.3.1: We support Option-2. More than one pre-configured resources are needed, since NW may not predict the payload size of an UEI beam report or the number of triggered UEI beam report(s). NW can configure pre-configured UL resource(s) for lower payload size case, and other for higher payload size case. Q3.3.2: Generally fine with the description. But some wordings may be refined. For example, first available transmission occasion should be changed to most recent available transmission occasion, which is clearer. [Mod]: Thanks for suggestion. If my understanding is correct, ‘most recent’ is used for ‘xxx before yyy’, i.e., already occurred! Q3.4.1: In our views, triggering event detection and new beam RS configuration can be per-CC. Q3.4.2: Yes. |
| Mod V09 | Captured companies input. The update is marked in red. |
| IDC | Q3.1.1: Support confirming the WA. P3.1.2: Support, with Alt 1-1 (SR-like) for Mode-A, and Alt 2-2 (new UCI type/RRC config.) for Mode-B. But, we are open for further discussions on Mode-B, if unified design (toward commonly “SR-like” signal) is necessary. Q3.2: We support Option-2 (associating an AP-CSI trigger-state) as baseline, but open for discussing Option-3 (assigning a codepoint) which is functionally equivalent to Option-2. [Mod]: Captured! P3.3.1: Support (regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least PUSCH) Q3.3.1: Support at least Option-2 (one-to-M), and we don’t think any restriction in terms of periodicity is necessary even for Option-1. Periodicity parameter for each first or second channel can be independent, up to NW implementation. Q3.3.2: Support the proposed bullets for Tx occasion determination. “a first available transmission occasion” is the proper wording as FL clarified. [Mod]: Captured! |
| LG | P3.1.2: if SR based approach is used, we need a new mechanism for UCI (i.e., physical layer information) to trigger SR since, according to current RAN 2 spec, logical channel or MAC-CE (i.e., higher layer information) can trigger SR. Q3.2: Support option 2 and also association between AP CSI trigger state with UEI beam report can be done without additional spec impact by using CSI report config ID for UEI beam report. We cannot see the benefit of Option 1 and 3. Q3.3.1: It may depend on other issues. For examples, if multiple events are supported, different event can use different first PUCCH resource and Option-3 is possible to reduce preconfigured resource overhead. Also, if multiple events share the same first PUCCH resource, then multiple preconfigured resource in step 2 can be used to differentiate events, so Option-2 is possible. Q3.3.2: Support to introduce X symbols to reuse a preconfigured resource. [Mod]: Captured! |
| Fujitsu | Q3.1.1: Support confirming the WA. Proposal 3.1.2: Support. From the functionality perspective, SR-like UCI or a new type of UCI is similar. From the Spec impact perspective, SR-like UCI is slightly preferred to reuse the existing procedures with only necessary amendments. Therefore, Alt 1-1 for Mode-A and Alt 2-1 for Mode-B are slightly preferred. Q3.2: For the triggering procedure, Option-2 is slightly preferred. However, this may also depend on whether the gNB can acquire the report configuration ID when receiving the first PUCCH. If not, the trigger state for event-driven beam reporting cannot be associated with any particular report configuration ID. Then Option-2 and Option-3 are basically similar in the functionality. For DCI format 0_3, it is preferred to be postponed. For DL-grant DCI, it is not supported. Proposal 3.3.1: Support. Q3.3.1: Support Option-1 and Option-3. Option-1 is the basic procedure. On top of that, it can be also considered to support Option-3. For Mode-A, the UE can retransmit the first PUCCH until receiving the triggering DCI. However, for Mode-B, the UE cannot perform any retransmission for the first PUCCH before transmitting beam reporting, since there is no response from the gNB. By supporting Option-3 M-to-one mapping, the retransmission for the first PUCCH before transmitting beam reporting can be also supported for Mode-B. Q3.3.2: Support. Q3.4.1: In our view, the general procedure can be as follows. To support cross-CC beam reporting, multiple report configurations (report setting) can be configured for a CC (e.g. CC#1). Each report configuration is associated with a resource configuration (resource setting) which includes measurement RSs for a particular CC (e.g. CC#2). By this way, the measurements results for CC#2 can be reported on CC#1, i.e. supporting cross-CC beam reporting. More specifically, for the new beam, RSs are provided in the resource configuration which is associated with the carrier ID information; For the current beam, RSs are derived from the indicated TCI state. Since a UE knows which carrier an indicated TCI state is applied to, the UE will know RSs for the current beam for each carrier. [Mod]: Please check whether option1 for current beam is okay for you? Q3.4.2: Yes. The cross-CC beam reporting will be transmitted on the CC where its report configuration is configured. [Mod]: Captured! |
| Spreadtrum | Q3.1.1: Support to confirm the WA. Proposal 3.1.2: From the perspective of unified design, we support Alt 1-1 for Mode-A and Alt 2-1 for Mode-B. Regardless of Mode A and Mode B, one-bit step1 information (request or notification) can be similar to legacy SR to reduce overhead of the first PUCCH. Although for Mode-B the purpose of the step1 information is different from that of the legacy SR, but a new RRC parameter can be introduce to indicate an special SR ID which is used for UEI beam report to distinguish the regular scheduling request. Q3.2: Regarding triggering procedure, we are fine with Option-2 and Option-3. Regarding DCI format 0_3, when DCI format 0_3 is used by gNB to schedule one or more PUSCHs, if the trigger event is detected and the request for UL resource is transmitted by UE, the DCI format 0_3 can also be used as the step2 DCI on Mode A. Regarding DL-grant DCI format, there is no need to support DL-grant DCI format in addition to UL DCI format. Proposal 3.3.1: Support. For second channel in step2 for Mode-B, as a unified design, it is enough to pre-configure PUSCH for UE initiated beam report. Especially when multiple UE initiated beam reports are triggered, PUSCH can be used for multiple UEs by MU-MIMO manner. Q.3.3.1: Fine with Option-1 with same periodicity which is simpler solution. Q.3.3.2: OK. [Mod]: Captured! |
| Qualcomm | Q3.1.1: We support confirming the WA. Proposal 3.1.2: We generally support the proposal. However, in our view, neither the meaning of “SR-like” nor the distinction from the “new UCI type” is clear. Although the FL tried to clarify it in the proposal, it is questionable whether the understanding is aligned across companies. For instance, although the resource looks like SR, it can be distinguished from the legacy SR in the functionality, sharing rule, multiplexing rule, etc. Then can we still call it “SR-like”? Before delving into the details, we think clarifying the functional/structural distinction between the SR-like and new UCI type is needed. [Mod]: Good point! Being associated with SR-id or not is truly different. Also highlighted in the title! Q3.2: We support reusing CSI-request field, but at this stage, distinguishing Option-2 and Option-3 may not be necessary, because it should be the next level detail – so we may first decide whether to introduce a new DCI field or reuse the CSI request field. [Mod]: Good idea! Let me think about that. Proposal 3.3.1: For progress, we can support this proposal. [Mod]: Thanks for being flexible! Q3.3.1: We support Option-1A for simplicity. The benefit of Option-2/3 is not clear/convincing to us. Q3.3.2: We are fine with the FL’s view. Q3.4.1: If we reuse the legacy CSI Resource Setting (for new beams) and TCI state (for current beam) configurations, they already indicate the CC indices. Therefore, we don’t see any ambiguity. Q3.4.2: Yes – again, if we reuse the legacy CSI Report Setting, we don’t see any ambiguity. [Mod]: Captured! |
| vivo | Q3.1.1: the discussion on additional events is still ongoing, wait until there is conclusion. Q3.1.2: fine with current proposal, seems we need more discussion on understanding when we say SR-like, need to clarify the details. [Mod]: Please review the reply to QC. Q3.2: support option 2, as mentioned by FL the difference between option 2 and 3 is RRC enabling, which can be discussed. Proposal 3.3.1: support, type-1 CG-PUSCH is fine Q3.3.1: support Option-1A: Same periodicity between first and second channel Q3.3.2: support, X shall not be 0 [Mod]: Captured! |
| Nokia | Q3.1.1: Confirm the WA P3.1.2 and Q3.2: One-bit indication is multiplexed as positive SR in the first PUCCH channel, and new RRC parameter configures SR-like signal with necessary parameters. P3.3.1: OK [Mod]: Captured! |
| NEC | Proposal 3.1.2: Support Alt 1-2 and Alt 2-2, one clarification point for SR-like scheme, if multiplexing or priority rule changed (as we think at least multiplexing and priority for first channel should be different from SR), can this channel still be called SR? in our understanding, it’s actually a new UCI. [Mod]: Good point. Please review replies to QC! Q3.2: Support option 2 and option 3 for the triggering procedure, actually, if first PUCCH channel is associated with one Cell or one event, the CSI request field doesn’t need to distinguish associated cell or event for the beam report carried in the second uplink channel. A common trigger state or codepoint in the CSI request field is sufficient. And we don’t think an extra one-bit new field is needed. Proposal 3.3.1: Our first preference is to support both PUCCH and PUSCH, while we can be fine with the proposal for starting point. [Mod]: Thanks for being flexible! Q3.3.1: Option 1 is preferred. One first PUCCH channel can be configured to be associated with one second uplink channel, and the interval between two channels can be preconfigured. |
| MediaTek | Issue 3.1.1: Support to confirm the working assumption. Issue 3.1.2: Support Proposal 3.1.2 and slightly prefer new UCI type for either Mode A or Mode B. The first PUCCH serving as either a request in Mode A or a notification in Mode B requires the same functionality, we don’t see the need of introducing different designs for the first channel in two modes. Hence, we prefer to have unified design for the first channel in two modes, to either reusing SR or having new UCI type for Mode A and Mode B. Adopting different alternatives is not preferred, which doubles the working effort somehow. If reusing SR as the first channel, we expect the following enhancement including: - SR will be dropped when overlapping with any PUSCH, while the first channel should be piggybacked on a PUSCH if the PUSCH is not for UEI/ED beam reporting. - Triggered SR will be cancelled if receiving available UL-SCH resource to accommodate all pending data, while the triggered UEI/ED beam reporting consisting of the first channel transmission and the second transmission should not be cancelled until the current beam is switched. Because there is no re-transmission for UCI in legacy, such that keeping reporting is considered for handling the missing of the UEI/ED report. If introducing new UCI type as the first channel, it is better to leverage the legacy SR behavior as much as possible to minimize spec impact. [Mod]: Good point! Issue 3.2: Option-2 is preferred because no further enhancement is required to support UEI/ED reporting. Option-2 is the legacy way for triggering AP CSI reporting, and it could be reused for triggering UEI/ED beam reporting without any further enhancement. By Option-2, if the trigger state indicated by CSI request field which associates with the UEI/ED beam reporting configuration(s), the DG-PUSCH scheduled by the DCI format is used for the associated UEI/ED beam reporting. Regarding DL-grant DCI format for Mode A, we don’t support it since it requires more spec impact and have limited use case by relying on the DL traffic. Proposal 3.3.1: Support Proposal 3.3.1 and prefer Type-1 CG-PUSCH resource for the second channel Q3.3.1: Support Option-1 with the same periodicity The benefit of configuring multiple resources(s) for the first channel or the second channel for a reporting configuration is unclear. Moreover, we don’t see the need to have the flexibility on supporting Option-1B. It is reasonable to have the same periodicity for resource of the first channel and resource of the second channel, such that Option-1A is preferred. Q3.3.2: Association between the first channel and the second channel based on the minimum time offset is needed. Even the same periodicity for resource of the first channel and resource of the second channel is assumed, the association based on the minimum time offset is still required because some of resource(s) for the first/second channel may be unavailable in TDD operation. Q3.4.1: To enable cross-CC reporting, there are several issues regarding configuration framework to be discussed as follow. Issue#1: How to achieve cross-CC RS measurement? Current NR already supports cross-CC RS measurement. A CSI reporting configuration in CC#A associate with a measurement resource configuration for CC#B. We think the same framework can be reused for R19 UEI/ED beam reporting. Issue#2: Whether the measurement RS for new beam and the measurement RS for the current beam shall be from the same CC? In our view, the measurement RSs for new beam and the current beam can be from different CCs. Because multiple CCs may share the same measurement pool in one common CC for beam measurement, but multiple CC will have the individual indicated TCI states where QCL Type-A RS are in its own CC but QCL Type-D RSs are in the common CC, as the following illustration. Hence, we expect that each reporting configuration will provide one CC ID for measurement RS(s) of new beam(s) and the other CC ID to indicate this reporting configuration shall refer to the indicated TCI state applied in which CC as the current beam. If the CC ID is not provided, the configured CC is assumed as default. [Mod]: Captured! Q3.4.2: As legacy, the report of a reporting configuration configured in one CC shall be reported in the resource for the second channel in the same CC. |
| CATT | Q3.1.1 We are ok to confirm the WA. Proposal 3.1.2 Support the proposal. As mentioned in issue 3, we prefer to include the event-ID if multiple events are supported. In this case, legacy SR scheme may need further enhancements. We prefer to keep the current proposal and going into details after the issue 1 is resolved. Proposal 3.3.1 Support. Q3.3.1 We are ok with either option1A or option B. Q3.3.2 Support and a proposal can be introduced accordingly: Proposal 3.3.2: On the reporting procedure for mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the report notification on the first PUCCH channel. - FFS: X is configured by RRC, or fixed (e.g., 0). [Mod]: Very nice! Thank you! I will use that indeed in the next round. Q3.4.2 Support. |
| CMCC | Q 3.1.1: Support to confirm the WA. P3.1.2: Support SR-like for both Mode-A and Mode-B. For mode-A, the function of first PUCCH is totally same as SR. A dedicated SR resource can be configured for UEI, and the SchedulingRequestID can be used to distinguish the configuration with other PUCCH-SR. In legacy, SR is dropped when overlapping with a PUSCH. For UEI, the first PUCCH in Mode A may be multiplexed on PUSCH. For Mode-B, the function of one-bit indication in the first PUCCH channel is to notify a second UL channel to carry beam report. A dedicated SR resource could be also used to realize the function. Different from Mode-A, UE transmits the first PUCCH channel once a time, although none of DCI is received afterwards. Q3.2: Support Option-2. The UEI beam report can be configured as a dedicated aperiodic CSI reporting. Once UE sends the first PUCCH to request a resource for a second UL channel to carry beam report, gNB could send a UL-grant DCI to trigger the dedicated aperiodic CSI reporting. 1-bit field in DCI is not needed. Not support DCI format 0_3. Not support DL-grant DCI format. Proposal 3.3.1: Support. Q3.3.1: Support Option-2 (one-to-M). Option-1 is a special case of Option-2 with M=1. To further consider multiple events or cross CC beam reporting, multiple second resources may be pre-configured with different payload size. Periodicity restriction is not needed, it can be left to gNB implementation. Q3.3.2: Not support. If Option-1 is agreed in Q3.3.1, then the gap between first and second channel can be left to network configuration. If Option-2 is agreed in Q3.3.1, what is the definition of “first available transmission occasion”? [Mod]: If a first transmission occasion of second channel after first channel is dropped (e.g., D/U slot pattern), the UE still can find the next available transmission occasion. |
| Sharp | Q3.1.1: We support to confirm the WA. Proposal 3.1.2: We support to reuse the legacy SR as much as possible. However, for Mode-B, we think the prioritization of the first PUCCH is lower than the one of the legacy SR. Therefore, we prefer to introduce new UCI for Mode-B. Q3.2: We support to introduce a new 1-bit field. In our view, Option-2 has more spec impact because of determining an association between a trigger state and UEIBM configuration. Q3.3.1: We support Option 1. In Option 2, some additional field in Step 1 or Step-2 is needed to indicate one of several pre-configured resources. [Mod]: Captured! |
| Mod V21 | Captured companies input. The update is marked in red. |
| HONOR | Q3.1.1: Confirm the WA. Proposal 3.1.2: We agree that SR may be appropriate for Mode A and Mode B may need a new UCI type. However, it’s better to have the unified solution for both modes. We can accept to support new UCI type for both modes if the following issue is considered: It’s critical that the first channel should be associate with event type/report configuration/trigger state. - For Mode A, if network doesn’t know which report configuration (or event) is triggered by first channel, how network know which trigger state to indicate in the DCI CSI request field? - For Mode B, if the event type/report configuration is not notified in the first channel, network may need to blindly detect the second channel? It may also have spec impact in CSI priority rules if leverage legacy CSI reporting framework. Therefore, we prefer to add FFS at least for Alt 1-2 and 2-2: Whether and how the event type/report configuration/trigger state is associated with the first channel. [Mod]: As a starting point, the first channel is associated/configured with the UEIBR report configuration which may further comprises event-type! As a plan, we will clarify that while discussing with RRC list next Thursday, Q3.2: Support Option 2 due to reuse legacy CSI reporting. In addition, we haven’t decided whether the DCI in step 2 can also trigger SP-CSI or not. DCI 0_3 should be further studied after the issues of event types/cross CCs are determined. we are agree with ZTE’s view. DL-grant DCI format to indicate PUCCH may have more spec impact, which is deprioritized by us. Proposal 3.3.1 (at least PUSCH): Support and we are open to the option of PUCCH. Q3.3.1: We would like to add a new option: “Option 4 (M-to-M): One or more first PUCCH resource(s) and one or more pre-configured resources for second UL channel can be associated with a UEI beam report configuration.” Option 4 is similar to MSGA in current spec where the first channel and the second channel is one-to-one mapped. In addition, Option 3 and Option 3 are our preferences. We think it’s really necessary that the first the first channel should be associate with event type/report configuration/trigger state if we want to leverage legacy CSI reporting framework, especially for CPU occupancy and CSI priority rule. [Mod]: Captured! |
| FUTUREWEI | Q3.1.1: We are fine to confirm the WA. Proposal 3.1.2: We are fine with the proposal. We prefer Alt 1-1 for Mode-A, and Alt 2-1 for Mode-B. Q3.2: We prefer Option 3. Proposal 3.3.1: Our preference is to support PUCCH. However, for the sake of progress, we are ok to support PUSCH only. In our view, supporting both PUCCH and PUSCH is unnecessary and should be avoided. [Mod]: Thanks for being flexible! Q3.3.1: We prefer Option-2 with more flexibility, in our understanding, Option-1 is a special case of Option-2. Q3.3.2: We are open to discuss this issue. To guarantee the performance of the reallocation of resources of the second channel by gNB, the time gap between the first PUCCH and the second channel needs to be set large enough. However, the time gap can also be configured with small value or zero if the network wants the lower latency and has sufficient resources to handle this. |
| Panasonic | Q3.1.1 Confirm Working assumption Proposal 3.1.2: We support having a new UCI type for modes A and B. There are multiple reasons, and we state two here. First the SR triggering mechanism does not support the two cases of modes A (requesting resources for non-data) and modes B (notification of transmission). Also the rules of overlapping with PUSCH should be different. Issue 3.2: Regarding the triggering procedure for Mode A, the UE is waiting for an UL DCI, so the easiest thing to do is to check whether there is a match between the requested CSI report by the NW and the initiated BM report. Option 2 is one way to check for this match. If there is a mismatch, UE behavior needs to be specified as well. Moreover, at this stage we do not support using DCI format 0_3 and DL DCIs if we go with option 2. If we go with option 1, the perhaps DL DCI can also be used. Proposal 3.3.1 (at least PUSCH): We do not support this. We support PUCCH or PUCCH + PUSCH. First this proposal is outdated since a lengthy discussion has taken place since RAN1#117. Based on the companies’ input, we would like to add more details to this proposal for example whether CG PUSCH or DG PUSCH (or both) is targeted here so we can get a better idea what is being discussed. The second thing we would like to point out is that CG-PUSCH for example is optional while PUCCH is mandatory. Then in this case Mode B cannot be supported if CG-PUSCH is not supported by UE. Therefore, we propose that the second UL channel can be a pre-configured PUCCH. And based on the legacy UCI multiplexing rule, when the second PUCCH for Mode B overlaps in time with another PUSCH whether CG or DG, it can still be reported on PUSCH. Finally, we do not think that the extra workload regarding supporting UE-initiated beam reporting for mode B on dedicated PUSCH is justifiable since in the end, using PUSCH or using PUCCH both have pros and cons. [Mod]: Captured! Do respect your preference, but please review the situation! |
| Transsion | Q3.1.1:Support to confirm the WA Proposal 3.1.2: Support the proposal. We prefer Alt 1-1 for Mode-A, and Alt 2-2 for Mode-B. Q3.2: Support Option-2 and Option-3 as they can coexist with AP/SP CSI. For DL grant DCI format, it seems unnecessary to support DL grant DCI format in addition to UL DCI format. Proposal 3.3.1: Support. For the type of PUSCH, we support to discuss Type-1 CG-PUSCH. Q3.3.1: Support Option-1 with the same periodicity Q3.3.2: Support [Mod]: Captured! |
| Apple | Q3.1.1: Confirm the WA. We should make progress on Event-2 as much as we can. Proposal 3.1.2: we support Alt.1-1 for Mode-A and Alt.2-2 for mode-B, considering the different function of the first channels. Issue 3.2: Open to both Opt.2 and Opt.3. Based on our understanding, they are quite simliar because Opt.2 also requires assigning a codepoint of ‘CSI request’ field to distingurish beween the PUSCH used for UEIBR reports and other purposes, just like in legacy system. Proposal 3.3A: Not support. This unncessarily complicates the UE implementation. Proposal 3.3B: NOT prefer. Proposal 3.3C: Support. Proposal 3.3.1 (at least PUSCH): Support. Q3.3.1: Opt.1 and Opt.1A. Given the small payload of UEIBR report, we do not see the motivation to configure multiple resources. Q3.3.2: Support the procedure. Specially, to better utilize the resource consideirng the UEIBR report maybe rarely triggered, the gap should be large to allow NW to re-schedule the resource to other UEs if no first PUCCH channel is received. [Mod]: Captured! |
| Mod V30 | Captured companies input. The update is marked in red. |
| TCL | Proposal 3.1.2: For mode-A, support alt 1-1. For mode-B, support alt 2-2. Proposal 3.2: Support. Proposal 3.3: Support. [Mod]: Captured! |
| OPPO | Q3.1.1: support to confirm the WA Q3.2: ok with either Option-2 or 3. 3.3.1: support the proposal. Q3.3.1: support Option1 Q3.3.2: not support. If we determine one-to-one mapping (i.e., Option 1), then the design in Q3.3.2 is not needed at all. [Mod]: However, timeline may still need to be clarified, even if X is meaningless! Q 3.4.1: it should be Option 1 for new beam Q 3.4.2: yes. |
| NTT DOCOMO | Q3.1.1 We support to confirm that. Proposal 3.1.2 We are fine in principle. We prefer Alt1-1 and Alt 2-2. For Alt 2-2, it should reuse SR design basically to avoid large spec impact. Q3.2 For UL-grant DCI format, we prefer Option-1 or 3 to avoid that CSI-AperiodicTriggerStateList is used for another purpose. For DCI format0_3, we support DCI format0_3. For DL-grant DCI format, we do not think that it would be necessary. Proposal3.3.1 We do not support. First, we do not see big difference between PUCCH and PUSCH from performance perspective. Thus, we think that it would be fine to support either PUCCH only or PUSCH only. Second, in our understanding, PUSCH means only CG-PUSCH for Mode B because second UL channel are pre-configured based on previous agreement. Then, in current specification, CG-PUSCH is optional so that Mode B cannot be supported if CG-PUSCH is not supported by UE. That is not promising situation from our perspective. Thus, we prefer to support only PUCCH. [Mod]: Per offline, it seems that the above can be workable for DOCOMO. Highly appreciated. Q3.3.1 We prefer Option-1at least for single CC UEIBR. For other options, we do not see the benefit. Q3.3.2 We think that that time gap should be specified to avoid UL collision. Q3.4.1 For cross CC RS measurement, the current resource configuration can indicate measurement RS resources for a CC. Q3.4.2 We think that cross-CC beam report should be reported on a CC on which UEIBR report configuration is configured as in legacy. [Mod]: Captured! |
| ETRI | Q3.1.1: May not rush but wish to confirm the working assumption. Q3.2: We support the option-2, provided that the UE reports 2nd UL channel conditionally when the DCI triggers CSI reports. Regarding DCI format 0_3, multi-carrier resource assignment has a motivation in FR1 however UEI reporting can be FR2. We are not objecting but we are not convinced to support. Q3.3.1: We think that one resource would be mapped but in case of cross CC reports or multiple events occurrence, multiplexing or prioritization can be performed. We may need more clarifications. Q3.3.2: Support Q3.4.1: Option-1 for both cases. Q3.4.2: Support [Mod]: Captured! |
| Sony | Q3.1.1: Support to confirm the WA. Proposal 3.1.2: First of all, we prefer to have a unified design of first PUCCH. For mode B, alt 2-2 (new UCI type) would be more feasible considering the usage. Then, having a unified design, alt 1-2 (new UCI type) is also preferable for mode A. Q3.2: support Option-2 as we prefer to reuse legacy CSI reporting framework as much as possible. Proposal 3.3.1: Support. Q3.3.1: Support Option 2 (one-to-M). Option 2 would be beneficial for facilitating to carry UEI beam report with different payload size. Q3.4.2: Yes. [Mod]: Captured! |
| Panasonic | Regarding the proposal : Proposal 3.3.1 (at least PUSCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least PUSCH. - FFS: PUCCH as the second channel FFS: Whether the PUSCH can be with UL data Given the majority would like to support PUSCH, is it possible to add that it is CG-PUSCH as a compromise? Proposal 3.3.1 (at least PUSCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least CG-PUSCH. - FFS: PUCCH as the second channel FFS: Whether the PUSCH can be with UL data [Mod]: Thanks for being flexible! It seems workable, |
| Mod Final | Captured companies input. The update is marked in red. |

## Issue 4 – Other procedure(s) as required

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 4.1 | Storing the QCL properties of the SSB (e.g., per-sync) | FL note: Companies views seems remain. If still no consensus, we may have to close this issue till some related progress (e.g., any income from RAN4) are made. Issue 4.1 (Activation-latency reduction): After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - Support: MTK (or introduce 1-bit indication), E///, HONOR, NTT DOCOMO, KDDI, vivo, - Not support: - Deprioritized/Postponed by: Samsung, ZTE, Apple, Intel, Nokia, CATT |

Table 4-2 Company input for Issue 4

| Company | Input |
| --- | --- |
| Mod V00 | - Please input your comment/preference to Issue 4.1, if needed. |
| Ericsson | Would be interesting to understand any technical issue with Issue 4. But again: will read some TDocs |
| Intel | We understand the motivation of issue 4.1 for latency reduction. However, when UE reports the beam information from more than one beams, gNB may select one appropriate beam for subsequent transmission, which may not always be the best beam that is reported by the UE. Our view is that we can deprioritize the discussion for this issue. |
| ZTE | Suggest to focus on the critical issues that we have so far ant not rush into any other brand-new issue without technical reason. |
| Google | Understanding the intention, but how does NW know whether UE really stores the QCL properties? |
| Mod V09 | No update. Please review above comments. |
| vivo | We can discuss this issue without additional signaling, it should be optional capability |
| Nokia | Perhaps an interesting addition but not critical at this moment. Let’s focus first on making the main feature functional, this is at best an optimization which needs careful attention. |
| MediaTek | We would like to clarify that there is no related discussion in RAN4 currently. Why should RAN1 wait for any incoming from RAN4? Moreover, there is no company showing technical concern so far. According to the schedule, there are six meetings left for R19 discission, and we expect minor RAN1 spec impact (even no spec impact in RAN1) on supporting it but some RAN4 spec impact is needed. We don’t understand why we have to close the discussion in RAN1 at this moment or keep postponing that. @Google, there are two alternatives can be considered. One is by attaching 1-bit indicator for each reported beam in the UEI/ED beam reporting to indicate whether the QCL properties of the SSB associated with the reported beam has been stored. The other alternative is that NW always assume that the satisfied beam in the UEI/ED beam reporting is synchronized. This means the UE has been stored the QCL properties of the SSB associated with the satisfied beam before reporting. |
| CATT | It is our view that this can be postponed for now. |
| Mod V21 | Captured companies input. The update is marked in red. Please review above comments. |
| HONOR | Generally, we agree with MTK’s comment. This issue has been postponed for about 2 meetings. We think the issue “Activation-latency reduction” is also critical for reducing overhead and/or latency in this Objective. |
| FUTUREWEI | We think that relevant procedure and UE behavior enhancements are needed to effectively reduce latency of beam application after new beam(s) are reported. In addition to the one list here (i.e., UE storing QCL properties of the SSB related to the reported new beam) which may reduce the activation time, enhancement to apply new beam after confirmation/acknowledgement from NW without RRC configuration or MAC-CE signaling should also be considered. Without these enhancements, the latency from UE initiated beam reporting would still be high. Therefore, we suggest to re-formulate the discussion as the following, with the additions to the 2nd and 3rd columns in red: Beam application latency reduction: Storing the QCL properties of the SSB (e.g., per-sync); Applying new beam after confirmation/acknowledgement Issue 4.1 (Beam application latency reduction Activation-latency reduction): After sending a UE-initiated beam report, consider the following enhancement for beam application latency reduction: - the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - apply new beam after confirmation/acknowledgement from NW without RRC configuration or MAC-CE signaling. [Mod]: Let’s check companies’ views on your update. |
| Apple | This is an interesting idea and does offer some latency benefits. However, it may not be essential for enabling UE-initiated beam report. We prefer to foscus on other open issues to finalize the event designs, such as whether to introduce new event(s), what is the report content etc. If we have time after addressing these essenital topics, we are open to discuss this idea. [Mod]: Captured! |
| Mod V30 | Captured companies input. The update is marked in red. Please review above comments. |
| KDDI | We support. |
| Mod Final | Captured companies input. The update is marked in red. Please review above comments. |

- Proposals for Monday Online Discussion

Proposal 3.3.1 (at least PUSCH): On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least CG-PUSCH.

- FFS: PUCCH as the second channel

- FFS: Whether the PUSCH can be with UL data

Supported by (29): OPPO, MTK, ZTE (2nd priority), Intel (2nd priority), ETRI, Samsung (2nd priority), vivo, Nokia, Fujitsu, Apple, Google, CMCC, Meta, Sharp, Lenovo, CATT, Huawei/HiSi, Spreadtrum, Sony, NICT, Ericsson, xiaomi, Intel, IDC, Qualcomm, NEC, HONOR, Futurewei, Transsion, TCL, Panasonic, NTT DOCOMO (2nd choice)

- CG-PUSCH: vivo, MediaTek (Type-1), Huawei/HiSi (Type-1), ZTE (Type-1), Samsung, Apple, Intel, Panasonic,

Concerned by:

Proposal 1.1:

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

    - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

      - [Introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.]

      - FFS: The RS in the RS resource set can be updated by MAC-CE

  - FFS: Option-2 and Option-3

    - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

      - FFS: If a list size is small, MAC-CE activation is not needed

    - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

Supported by: (30): ZTE, OPPO, E///, Qualcomm, ETRI, xiaomi, MediaTek, NEC (MAC-CE), DOCOMO, Nokia, vivo (MAC-CE), Fujitsu, Apple (MAC-CE), Lenovo, CMCC, Futurewei, Meta, Sharp, CATT, CEWiT, Spreadtrum, LG, Huawei/HiSi, KDDI, Panasonic, TCL, RUIJIE NETWORKS, NICT, Samsung, Transsion, Sony

Concerned by: Google (Option-3), Intel (Option-2)

Proposal 1.2.1A:

Regarding RS measurement for the current beam for Event 2, for Option-2a, the following working assumption in RAN1#117 is confirmed:

- (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

Supported by (28): Ericsson, Samsung, Huawei/HiSi, xiaomi, Intel, ZTE, Lenovo, IDC, LG, Fujitsu, Spreadtrum, Qualcomm, vivo, Nokia, MediaTek, CMCC, Sharp, HONOR, Futurewei, Transsion, Apple, Ruijie, KDDI, TCL, OPPO, NTT DOCOMO, ETRI, Sony,

Concerned by (2): Google, CATT,

Proposal 2.1A:

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

…

Differential L1-RSRP #N

Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC

Note: Other contents are not precluded

.

    - Differential L1-RSRP #N is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1

      - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

      - FFS: range and step size of differential L1-RSRP

    - [FFS: Whether/how to additionally report CRI/SSBRI or TCI for the current beam.]

    - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

    - FFS: Additional report content(s) (e.g., reporting configuration ID, 1-bit indication for synchronization state, event ID, or cell ID).

Supported by (33): ZTE, OPPO, Intel. QC, ETRI, xiaomi, MediaTek, Samsung, NEC (in principle), Nokia, vivo, IDC, Fujitsu, Google, CMCC, Futurewei, Meta, Sharp, Lenovo, CATT, CEWiT, Spreadtrum, LG (in principle), Huawei/HiSi, KDDI, Apple, Panasonic, Transsion Holdings, NICT, Honor, Ruijie, KDDI, TCL, Sony,

Not supported by (1): Ericsson,

Proposal 3.1.1:

Regarding beam report transmission procedure for UE-initiated/event-driven beam reporting, the following working assumption in RAN1#117 is confirmed:

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Confirmed by (33): Samsung, Nokia, Qualcomm, Ericsson, Spreadtrum, Intel, xiaomi, CATT, Lenovo, TCL, RUIJIE NETWORKS, ASUSTeK, Huawei/HiSi, ZTE, IDC, Fujitsu, Spreadtrum, Qualcomm, Nokia, MediaTek, CATT, CMCC, Sharp, HONOR, Futurewei, Panasonic, Transsion, Apple, OPPO, NTT DOCOMO, ETRI, Sony,

Not confirmed by (1): vivo,

- Previous agreements

  - RAN1#117

[117] Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

[117] Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

[117] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows.

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

  - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state.

- Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Enabling one of either Scheme-1 or Scheme-2 is selected by NW.

  - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

- The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

[117] Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

- FFS: Option-3b/3c

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

[117] Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

[117] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

[117] Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

[117] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

[117] Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

- Others are not precluded.

- FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

[117] Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state.

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

  - RAN1#116-bis

[116b] Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

- Regarding RS measurement for the current beam for Event-2, Option-2a is supported:

  - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state.

    - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state.

  - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE.

    - Note: SSB or CSI-RS can be configured

[116b] Agreement

On UE-initiated/event-driven beam reporting, further study the following trigger events:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

[116b] Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

- The selected option shall satisfy Event-2.

  - RAN1#116

[116] Agreement

On UE-initiated/event-driven beam report, at least of following aspects should be included:

- Trigger-event detection for beam reporting by UE

  - UE monitors RS to assess if a beam-reporting trigger condition has been met

  - FFS: Trigger condition for declaring beam-reporting event

- Beam-report transmission by UE

  - Signaling contents in the beam report

  - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission

    - MAC-CE

    - UCI

    - Others are not precluded.

On UE-initiated/event-driven beam report, the following aspects may be included:

- UE requesting UL resource(s) for the beam report

- UE notifying transmission of beam report

- gNB preconfigured resources

Other procedure(s) as required

[116] Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

[116] Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

[116] Agreement

On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP

- FFS: Study and decide whether additional contents can be supported.

- FFS: L1-RSRP format, e.g., absolute and/or differential value.

- Note: Above does not imply to preclude discussion on L1-RSRP filtering.

- The actual reported content depends on the triggering event

  - Support of one or multiple events will be discussed separately

# References

| 1 | R1-2406043 | Offline summary for Rel-19 MIMO UEIBM (9.2.1) pre-RAN1#118 | Moderator (ZTE) |
| --- | --- | --- | --- |
| 2 | R1-2405814 | Enhancements for UE-initiated/event-driven beam management | FUTUREWEI |
| 3 | R1-2405870 | On UE initiated/event-driven beam management | Huawei, HiSilicon |
| 4 | R1-2405875 | On Rel-19 UE/Event-Driven Beam Management | InterDigital, Inc. |
| 5 | R1-2405887 | Enhancements for UE-initiated/event-driven beam management | MediaTek Inc. |
| 6 | R1-2405903 | Discussion on UE-initiated/event-driven beam management | Spreadtrum Communications |
| 7 | R1-2405980 | Discussion on enhancements for UE-initiated/event-driven beam management | CMCC |
| 8 | R1-2406027 | Enhancements for event driven beam management | Intel Corporation |
| 9 | R1-2406028 | Discussion on enhancements for UE-initiated/event-driven beam management | ZTE Corporation, Sanechips |
| 10 | R1-2406177 | Discussion on UE-initiated/event-driven beam management | vivo |
| 11 | R1-2406260 | Discussions on UE-initiated/event-driven beam management | OPPO |
| 12 | R1-2406279 | Enhancements for UE-initiated/event-driven beam management | Xiaomi |
| 13 | R1-2406310 | Discussion on enhancements for UE-initiated/event-driven beam management | Fujitsu |
| 14 | R1-2406363 | On UE-initiated/event-driven beam management | CATT |
| 15 | R1-2406420 | UE-initiated beam management | LG Electronics |
| 16 | R1-2406454 | Enhancements for UE-initiated/Event-Driven Beam Management | Panasonic |
| 17 | R1-2406467 | Enhancements for UE-initiated/event-driven beam management | Sony |
| 18 | R1-2406521 | Enhancements for UE-initiated/event-driven beam managementitiated/event-driven beam management | Lenovo |
| 19 | R1-2406531 | Discussion on UE-initiated/event-driven beam management | TCL |
| 20 | R1-2406543 | Discussion on enhancements for UE-initiated or event-driven beam management | NEC |
| 21 | R1-2406574 | Discussion on enhancements for UE-initiated/event-driven beam management | Hyundai Motor Company |
| 22 | R1-2406576 | Discussion on UE-initiated/event-driven beam management | HONOR |
| 23 | R1-2406595 | Discussions on enhancements for UE-initiated/event-driven beam management | Ruijie Networks Co. Ltd |
| 24 | R1-2406642 | Views on Rel-19 UE-initiated/event-driven beam management | Samsung |
| 25 | R1-2406682 | Discussions on enhancements for UE-initiated/event-driven beam management | KDDI Corporation |
| 26 | R1-2406700 | Enhancements for UE-initiated/event-driven beam management | Transsion Holdings |
| 27 | R1-2406722 | Enhancements for UE-initiated/event-driven beam management | ETRI |
| 28 | R1-2406745 | Enhancements to facilitate UE-initiated/event-driven beam management | Nokia |
| 29 | R1-2406831 | Enhancements for UE-initiated beam management | Apple |
| 30 | R1-2406887 | Enhancements for UE Initiated Beam Management | Meta Ireland |
| 31 | R1-2406925 | Discussion on enhancements for UE-initiated/event-driven beam management | NTT DOCOMO, INC. |
| 32 | R1-2407002 | Enhancements for UE-initiated/event-driven beam management | Sharp |
| 33 | R1-2407024 | UE-initated/event-driven beam management | Qualcomm Incorporated |
| 34 | R1-2407066 | Discussion on UE-initiated/event-driven beam management | ITRI |
| 35 | R1-2407075 | Enhancements for UE-initiated/event-driven beam management | CEWiT |
| 36 | R1-2407111 | Discussion on enhancements for UE-initiated or event-driven beam management | Google |
| 37 | R1-2407122 | Discussion on UE initiated beam report | ASUSTeK |
| 38 | R1-2407143 | Enhancements for UE-initiated/event-driven beam management | NICT |
| 39 | R1-2407145 | Enhancements for UE-initiated beam management | Ericsson |