3GPP TSG RAN WG1 #118			R1-2406846

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.4.2.3

Source: 	Moderator (Apple)

Title:	FL summary#3 on downlink and uplink channel/signal aspects

Document for:	Discussion & Decision

# Introduction

This document provides the feature lead summary on the offline discussions/inputs/proposals for AI 9.4.2.3 on downlink and uplink channel/signal aspects for ambient IoT during RAN1#118.

  - Contact Information

If not already provided, please consider providing your company name, your name and email address to be able to reach for any potential offline discussions/contact regarding AI 9.4.2.3 on downlink and uplink channel/signal aspects for ambient IoT.

| Company | Name | Email |
| --- | --- | --- |
| Apple | Ankit Bhamri | a.bhamri@apple.com |
| CEWiT | Deepak PM | deepakpm@cewit.org.in |
| NTT Docomo | Weiqi Sun | sunwq@docomolabs-beijing.com.cn |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| Qualcomm | Le Liu | leliu@qti.qualcomm.com |
| LG Electronics | Youngdae LEE | youngdae.lee@lge.com |
| Vivo | Zhipeng lin Yuanyuan Wang | zhipeng.lin@vivo.com yuanyuan.wang.txyji@vivo.com |
| Xiaomi | Pengyu Ji Ting Fu | jipengyu1@xiaomi.com futing@xiaomi.com |
| FUTUREWEI | Vip Desai | vipul.desai@futurewei.com |
| Panasonic | Hamidreza Shariatmadari Hidetoshi Suzuki Shotaro Maki Yang Kang | hamidreza.shariatmadari@eu.panasonic.com suzuki.hidetoshi@jp.panasonic.com maki.shotaro@jp.panasonic.com yang.kang@sg.panasonic.com |
| Nokia/NSB | Ganesh Venkatraman | Ganesh.venkatraman@nokia.com |
| Huawei, HiSilicon | Zhe Jin Sarun Selvanesan | jinzhe.jinzhe@huawei.com sarun.selvanesan@huawei.com |
| CATT | Fang-Chen Cheng Ren Da | fcc@catt.cn renda@catt.cn |
| SONY | Martin Beale | martin.beale@sony.com |
| China Telecom | Jing Guo Yi Gu | guojing6@chinatelecom.cn guy6@chinatelecom.cn |
| OPPO | Shichang Zhang | shichangzhang@oppo.com |
| TCL | Rongling Jian Wenwen Huang | rongling.jian@tcl.com wenwen5.huang@tcl.com |
| Honor | Jian Zhang | Zhangjian23@honor.com |
| ZTE, Sanechips | Mengzhu CHEN Youjun HU Yuzhou HU | chen.mengzhu@zte.com.cn hu.yuzhou@zte.com.cn hu.youjun1@zte.com.cn |
| Lenovo | Xin GUO Karthikeyan Ganesan | guoxin9@lenovo.com kganesan@lenovo.com |

  - General Guidelines

Please follow the guidelines for offline discussions and input to FL summary as per R1-2405783:

- No technical discussion and no attachment of documents via email thread on the email reflector

- To avoid ending-up with too long file names and downloading/opening issues, the following naming convention is recommended:

  - Keep the previous company’s name (only the most recent one) in the filename, e.g. Summary-1-v000-Mod, followed by Summary-1-v001-Mod-Company1

# Discussion/Inputs

This discussion for this agenda items is organized as follows:

- Topic 1: R2D channels/signals

  - R2D signals design including preamble/midamble/postamble

  - R2D control information (including TB size)

  - PRDCH design details

- Topic 2: D2R channels/signals

  - D2R signals design including preamble/midamble/postamble

  - D2R control information

  - PDRCH design details

- Topic 3: Proximity determination

- Topic 4: Intermediate UE considerations for topology 2

  - Topic 1: R2D channels/signals

### R2D signals including preamble/midamble/postamble

| Company | Observations/Proposals |
| --- | --- |
| Nokia [2] | - Observation 1: The preamble may consist of two distinct fields, namely, a delimiter field to identify the start of a frame followed by a sync word carrying a known sequence of bits or samples to assist the tag to obtain the timing alignment. - Observation 2: Having a header field to identify the type of payload carried by the current PRDCH should be considered. - Proposal 1: Study the need for a separate header field, which may or may not be a part of preamble, to carry the payload format indication message to utilize the same PRDCH channel for both control and data. - Observation 3: Start delimiter based on OFF pattern only may unnecessarily trigger device to perform preamble correlation when there is radio silence observed. - Proposal 2: Start delimiter should be based on ON/OFF pattern to ensure reliable detection of the start of AIoT session or preamble transmission. - Observation 4: The use of the midamble, while easing the reading task, it complicates the D2R signal generation and introduces both spectral and energy overhead since: 1) the midamble is a known sequence, carrying no data. And 2) the midamble generation consumes additional energy from the AIOT device which is otherwise severely limited from this point of view. - Proposal 3: Study in RAN1 how the NW decides and configures whether to activate/deactivate/reconfigure the D2R midamble type and size. The decision is based at least on the expected level of interference and expected payload of the D2R signal. - Observation 5: The use of midamble is needed to reacquire the timing synchronization for the device having large sampling offset, if the duration of the transmission lasts longer following the preamble. - Observation 6: Even with large sampling offset, midamble may not be needed if the payload length is short, since the initial timing alignment is carried out by the preamble sequence. - Observation 7: Terminating the PRDCH transmission with a postamble may provide two benefits, namely, the variable payload length and to provide timing acquisition before the subsequent transmission of either PDRCH or PRDCH, thus improving the detectability at both reader and the device, respectively. - Proposal 4: Study in AI 9.4.2.2 the design of the R2D postamble that may contain a known stop sequence to indicate the end of the frame. - Observation 11: Due to the limited capabilities of AIoT device, complex measurements based on reference signals, are quite challenging. In the case of device type 2a/2b, the AIoT device may perform simple measurement operations that can be useful for link adaptation. However, it is too early to exclude all types of measurements e.g. power based like RSSI, especially for AIoT device type 2a/2b. - Proposal 8: RAN1 to study whether the signals like preamble, midamble, and postamble can be reused for reference signals and if so, then what kind of measurement metrics can be obtained that can be used to improve the detection performance. |
| Ericsson [3] | - Observation 4: A start-indicator (delimiter) is not needed for A-IoT devices which do not rely on RF energy harvesting. - Observation 5: Option 1 enables the start-indicator to be self-contained whereas Option 2 necessitates an RF charging session prior to the start-indicator. - Proposal 6: Adopt Option 1 as the start-indicator part for A-IoT devices for a harmonized solution for all kinds of devices. - Observation 6: It is not necessary to use line coding in the clock-acquisition signal as a pre-defined OOK sequence with regular ON/OFF transitions can be used as a clock signal. - Proposal 7: Clock-acquisition signal can be based on pre-defined OOK sequences without line coding. - Proposal 8: R2D preamble should be designed to provide requisite timing and frequency synchronization for all A-IoT device types and architectures. - Observation 7: If the purpose of a postamble is to indicate the end of a R2D transmission, there is no need for an explicit indication through a postamble if the length of the subsequent R2D data transmission is configured/indicated by R2D control information. - Proposal 9: RAN1 to study whether a PRDCH postamble can serve as an additional timing acquisition signal prior to a PDRCH transmission depending on the device’s timing capability. - Observation 8: If the time interval between an R2D transmission and the corresponding D2R transmission following it is too long, devices may lose the timing obtained from the R2D timing acquisition signal due to timing drift at the time for the D2R transmission. - Proposal 10: Study periodic and aperiodic synchronization signals for A-IoT devices. |
| Huawei [4] | Observation 6: Using a fixed-length low-voltage signal for the R2D start indicator part, with a preceding high-voltage signal, is adequate for the device to distinguish it from the PRDCH transmission. Observation 7: For R2D start indicator part, the required detection metrics considered are Missed Detection Ratio (MDR) and False Alarm Ratio (FAR). Observation 8: For R2D start indicator part, 33.3μs can meet the MDR and FAR performance metric. Proposal 4: The R2D timing acquisition signal start indicator part is a fixed-length low voltage transmission to enable energy detection. The length of the start indicator is an integer multiple of the chip length. Observation 9: The clock acquisition part requires at least two or more transition edges in the same direction in order to determine the R2D chip length. Observation 10: The impact of CP to the detection of the timing acquisition signal is negligible as long as the start indicator with a low-voltage signal is longer than the CP length. Observation 11: In order to ensure that the timing acquisition signal can be differentiated from the PRDCH and compact at the same time, different sequence patterns can be used for different M values. Observation 12: For R2D time acquisition part, the required detection metric considered is the False Detection Ratio (FDR). Observation 13: For R2D time acquisition part, two transition edges detection in the same direction can meet the FDR performance metric. Proposal 5: The R2D timing acquisition signal clock acquisition part includes at least two transition edges to enable transition edge detection for determining the chip length of each R2D transmission. The duration between adjacent transition edges is an integer multiple of the R2D chip length. Proposal 6: No other parts are required for the R2D timing acquisition signal. Proposal 7: The format of the R2D postamble is a constant voltage sequence which cannot occur in R2D data or preamble and is longer than twice the chip length. - For Manchester line coding, the R2D postamble is assumed to be “111” chips. Observation 14: For R2D postamble, the required detection metrics considered are Missed Detection Ratio (MDR) and False Alarm Ratio (FAR). Observation 15: For R2D postamble, at least 3 high-level voltages such as ‘111’ chips can meet MDR and FAR performance metric. Proposal 9: A midamble or reference signal for R2D is not needed, and hence is not studied further. Observation 16: There is no need to study RRM measurements in Rel-19. Observation 17: Measurements are either infeasible or insufficiently accurate to be defined at the A-IoT device. Proposal 10: R2D measurements are not supported, and not studied further, for Ambient IoT for Rel-19. |
| Spreadtrum [5] | Proposal 1: Two parts should be included in the R2D preamble: - Start-indicator part: a High/low voltage signal structure and the length of high voltage is very different with low voltage to avoid error detection. - Clock-acquisition part: indication of the chip length for the following immediate PRDCH transmission Proposal 2: Postamble is supported for R2D transmission, and the sequence is different from preamble and PRDCH. |
| TCL [6] | Observation 1: For OFF pattern, if no extra signal (e.g., activated signal) reminds device to receive start-indicator, device may not correctly idetify start-indicator. Observation 2: If the start chip is low level, device may not correctly judge the first chip of start-indicator by envelop detection because of the threshold or interference/noise. Observation 3: To reduce the detection complexity at device side, more simple line code or design should be considered for ON/OFF pattern. Proposal 1: To avoid false OFF pattern in start-indicator received by device side, one signal (like activation signal) should precede preamble to remind when device should receive preamble in case of the impact of interference and noise. Proposal 2: For ON/OFF pattern of start-indicator, ON should preced OFF to be as the functionality of activation signal to inform device the R2D signal receiving. Proposal 3: To reduce the detection complexity at device side, simpler line code or waveform design should be considered for ON/OFF pattern. Observation 4: In RFID, the length of R=>T calibration in preamble is [2.5Tari~3Tari] (one tari value or tari length is regulated in the range of 6.25 µs to 25 µs), which is longer than delimeter (star-indicator). Observation 5: Line code can be used in clock-acquisition part for implicit indicated functionality, e.g. chip length and backscattering link frequency (BLF), etc. Proposal 4: For preamble structure, start-indicator part should be preceded clock-acquisition part. Proposal 5: The length of clock-acquisition part should not be smaller than that of start-indicator part. Proposal 6: To meet the sampling requirement for different typs of devices, clock-acquisition part can be designed to different types, e.g., low-rate clock acquisition for device 1 and high-rate clock acquisition for device 2a. Proposal 7: Same line code and chip length should be set in start-indicator part and clock-acquisition part. Proposal x：For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration - FFS: how/when to use the R2D chip duration determined by the clock-acquisition part. Proposal 8: Optional part in preamble can be considered for other functionality, including implicit indication and explicit indication, mute functionality for interference estimation, etc. Observation 6: Different modulation and coding schemes can be used for midamble to be identified at device side more conveniently. Observation 7: The length and number of midamble is decided by R2D TBS or packet size. Proposal 9: Self-clocking using line coding should be postponed to discuss until the line coding types and charging/consuming schemes are decided. Proposal 10: If R2D signal is only one value or small data packet (e.g., Q value for random access), Midamble is not necessary. |
| CMCC [7] | Observation 1: By using the start-indicator part with a duration of OFF pattern, i.e., low voltage, a device can simply, and power efficiently determine that a R2D transmission is starting by energy detection. Proposal 1: For the R2D timing acquisition signal immediately preceding the transmissions of a physical channel, the start-indicator part should always be included. Proposal 2: For the start-indicator part of the R2D time acquisition signal, the following option can be considered: - Option 2: OFF pattern, i.e. low voltage transmission. Proposal 3: For the clock-acquisition part of the R2D time acquisition signal: - It should include integer multiple of adjacent transition edges, based on the duration between two adjacent transition edges, the device can determine the OOK chip duration of the subsequent PRDCH. The length of the clock-acquisition part can be as long as possible while considering the overhead. - Using the clock-acquisition part to provide the OOK chip duration/length of the subsequent PDRCH can also be studied. Observation 2: The pattern of the clock-acquisition part is related to M chips per OFDM symbol. When M is small (e.g., M equals 2 or 4), the clock-acquisition part may across multiple OFDM symbols, and the CP insertion may degrade the timing acquisition performance. Observation 3: The pattern of the clock-acquisition part is related to the CP handling method. When CP handling Method Type 2 is used, inevitable error rising/falling edges may occur. Proposal 4: The pattern design of the clock-acquisition part in the R2D time acquisition signal should consider the CP insertion issue and CP handling method. Proposal 5: For R2D transmission in Ambient IoT, if line code including PIE and/or Manchester is considered in the study, midamble is not needed. Observation 5: For small payload size with only a few bits, the presence of long postamble generates large resource overhead. For large payload size with more bits, postabmble is much flexible. Proposal 6: To determine or derive the end of PRDCH transmissions, consider both options: - Option 1 (as baseline): R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2 (at least in case of small packet size): Based on R2D control information. Proposal 7: For R2D postamble immediately follows the PRDCH to determine the end of transmission, postamble can be a duration of high voltage longer than the continuous OOK ON chips in R2D preamble and data transmissions. |
| Intel [8] | Proposal 1: - For the start-indicator part of the R2D time acquisition signal, Option 2 (OFF pattern) is prioritized for study. - Start-indicator part has a fixed length. Proposal 2: - For the clock-acquisition part of the R2D time acquisition signal, same line coding and OOK chip duration is applied for the clock-acquisition part and subsequent PRDCH transmission; - Clock-acquisition part has a fixed length. FFS: variable length - FFS: the detailed pattern, pending on the decision of whether Manchester or PIE encoding is used as line coding for R2D transmission. |
| Lenovo [9] | Proposal 2: Study a channel structure for R2D transmission which comprises a preamble, a control part, a data part, midamble, postamble and a guard period. Proposal 3: Study a start-indicator part in preamble as a pre-known ON/OFF pattern to devices. Proposal 6: Study a postamble at the end of a R2D transmission for indicating the end of the PRDCH transmission. Proposal 8: Study preamble design containing wide pulse tolerating large timing errors at the beginning followed by narrower pulse duration achieving finer synchronization needed for the R2D reception. Proposal 9: The preamble designed using OOK chips preceding R2D contains multiple part: - The first part of the preamble contains wide pulse duration generated using OOK-4, M=1 at the beginning duration - The second part of the preamble contains narrow pulse duration using OOK-4, M>1 following the first part of the preamble - The third part contains sync word as delimiter with narrower pulse duration. - R2D reception at the device determines the chip duration using the second part of the preamble. |
| China Telecom [10] | Proposal 1: Two potential preamble designs for R2D transmission can be considered: - Type 1: Preamble consists of Start-indicator part + Clock-acquisition part. - Type 2: Preamble consists of Start-indicator part + Clock-acquisition part + R2D Cal part. Proposal 2: Symbol-level time unit alignment related design in RFID can be utilized for preamble design in R2D transmission. Proposal 3: For the Start-indicator part of the R2D time acquisition signal, support to study OFF pattern (i.e. low voltage transmission) as a starting point. Proposal 4: The Clock-acquisition part may consist of several OOK chips with the repetition of a single OOK chip. Proposal 5: The R2D Cal may consist of several symbols, several durations or just the repetition of the Clock-acquisition part, which is optionally needed based on the timing error level. |
| Vivo [12] | Proposal 3. For the start-indicator part of the R2D time acquisition signal in an R2D transmission, the pattern and its duration for either Option 1 or Option 2 should be fixed. Proposal 4. The two options of start-indicator part should be evaluated from the perspective of complexity, power consumption, and use cases. Proposal 5. The pattern (i.e., the sequence of on-off chip) of the clock-acquisition part should be fixed. - If a single fixed pattern is supported, R2D clock-acquisition part only indicates the chip length for the following PRDCH, and the D2R chip length of PDRCH (if any) can be indicated by R2D control information. - If two fixed patterns are supported, RFID design can be considered as baseline, where one pattern only indicates the chip length for PRDCH without triggering the PDRCH transmission (similar as RFID R2T frame sync); the other pattern indicates both PRDCH chip length and PDRCH chip length (similar as RFID R2T preamble). Proposal 6. The clock-acquisition part provides a PRDCH chip length, which applies to both data and control information in the following PRDCH. Proposal 7. The clock-acquisition part can provide a PRDCH chip length, which applies to both data and control information (if defined) in the corresponding PDRCH. Observation 1: The necessity of an additional function (i.e., frequency calibration) of the clock-acquisition part for device 2b should be justified. - Occupied BW may need to be more than [200kHz] for 200ppm @ 900MHz. Proposal 8: Study the design of preamble to reduce impact of CP on chip length estimation and end of preamble determination, e.g., the last OOK chip of an OFDM symbol carrying clock acquisition part of preamble is OOK OFF state. Observation 2: Using R2D Postamble to indicate the end of PRDCH transmission can enable any TBS for PRDCH transmission. However, it is not clear how variable TBS is required for PRDCH for indoor inventory and command use cases. Observation 3: R2D Postamble may not be needed for PRDCH with fixed TBS. Observation 4: R2D Postamble may result in PRDCH decoding failure or additional power consumption due to false or miss detection. Observation 5: Using R2D control information to indicate or derive the end of PRDCH transmission can enable limited number of TBS for PRDCH transmission. There is tradeoff between the overhead of R2D control information and the number of TBS for PRDCH transmission. Observation 6: Using R2D control information to indicate or derive the end of PRDCH transmission is more robust to the ambiguity of the end of transmission. Proposal 9: The end of PRDCH transmission that is determined or derived by the R2D control information should be studied as baseline. - FFS the R2D control information is/are the physical layer and/or higher layer control information |
| Oppo [13] | Observation 6: OFF pattern cannot be used as start-indicator for an A-IoT device with energy storage or using non-RF energy source. Proposal 11: For the start-indicator part of the R2D time acquisition signal, an ON/OFF pattern distinguishable from other R2D transmission should be used. Proposal 12: Further study to indicate OOK chip duration explicitly or implicitly via Clock-acquisition part. |
| Xiaomi [14] | Proposal 4: For the Start-indicator part of the R2D time acquisition signal, OFF pattern, i.e., low voltage transmission (Option 2) should be supported due to lower power consumption. Proposal 5: The R2D Clock-acquisition part of the R2D time acquisition signal contains one or several symbols (e.g., OOK symbols) which represents an integer number of “0” or “1” in which the same line coding scheme (e.g., Manchester or PIE) as R2D data transmission is applied. Proposal 6: D2R Clock-acquisition part may be additionally included in the R2D preamble for the device to compute its D2R transmission frequency and data rate. Proposal 7: R2D postamble is immediately transmitted after any PRDCH transmission. - An End-indicator part (a certain time of contiguous logic high/low electrical level) may be included in a R2D postamble. |
| Fujitsu [15] | Observation 1：Before the start indicator, a continuous signal of the level opposite to the long duration level of the start indicator is necessary for the detection of the start indicator, i.e., - If the start-indicator in R2D is identified by a low voltage transmission, a continuous high-level R2D signal before the start of R2D transmission is needed. For example, the reader transmits high-level when there is no need to transmit PRDCH or to receive PDRCH. - If the start-indicator in R2D is identified by a high/low voltage transmission, a continuous low-level R2D signal before the start of R2D transmission is needed. For example, the reader is mute when there is no need to transmit PRDCH or to receive PDRCH. Observation 2: The readers of ambient IoT cannot always transmit high level signal before the start of R2D transmission in in-band deployment scenario, which may increase the potential interference to coexisting NR/LTE. Proposal 1: The start-indicator part of the R2D time acquisition signal is high/low voltage transmission, - The duration of high voltage transmission is longer than the duration of single chip after line coding of PRDCH. - The voltage level of the end of start-indicator part should be opposite to the voltage level of the first chip in clock acquisition part to distinguish start-indicator part and clock-acquisition part of R2D time acquisition signal. Proposal 2: If the Manchester code is adopted for PRDCH, the clock-acquisition part of the R2D time acquisition signal contains at least two transition edges. - The interval between two adjacent transition edges is the duration of a single OOK chip or two OOK chips. - FFS which pattern is adopted to the design of clock-acquisition part. FFS the number of chips included in clock-acquisition part. |
| CATT [16] | Proposal 3: For A-IoT devices, the carrier wave for energy harvest can take place immediately before the R2D transmission. Proposal 4: R2D preamble should use OFDM based OOK-1 or OOK-4 waveform to transmit the clock-acquisition part in order to the get the finer resolution of time synchronization. Proposal 5: The CP handling for R2D preamble should adopt the Method Type 2 (Alt 2) in RAN1#117 agreement to use the CP for the OOK-4 waveform - Alt 2: Method Type 2 does not retain subcarrier orthogonality and use the CP as part of OOK-4 waveform for generating the preamble. Proposal 6: The start-indicator part (SIP) of R2D preamble should be designed as an ON/OFF pattern with high/low voltage transmission. Proposal 7: The start-indicator part (SIP) of R2D preamble should be a pre-defined sequence with fixed-length low voltage and fixed length high voltage. The sequence of the SIP should be different from the sequence of subsequent PRDCH transmission. Proposal 8: The SIP length would be fixed to L OFDM symbol(s) with fixed value of M being specified regardless the M-value of OOK-4 waveform used for PRDCH. roposal 9: Clock-acquisition part (CAP) of R2D preamble should use the same encoding method and chip duration as the PRDCH transmitted subsequently, such as Manchester coding. Proposal 10: The device can determine the chip duration by detecting the length between the rising and falling edges of the CAP sequence. Observation 1: The sampling error rate will be degraded a lot due to un-even chip interval when the CP is inserted within an OFDM symbol. The device needs to detect at least 16 chips in the CAP, in order to obtain about 5% sample error rate performance. Sampling Error Rate (SER) refers to the number of samples which is mismatched for comparing to the total number of samples in a chip. Proposal 11: The A-IoT synchronization performance by CAP of preamble could be degraded with SER if CP is included as part of OOK-4 waveform. Proposal 12: Clock-acquisition part (CAP) of R2D preamble should at least include 16 chips to obtain the 5% sample error rate performance. Proposal 13: The counting of new transition edge of CP would degrade the synchronization performance and increase the A-IoT device complexity Proposal 14: Except for the SIP and CAP, there is no need to introduce any other part for R2D preamble, and the above preamble design is applicable to devices 1/2a/2b. Proposal 15: There is no need to introduce R2D midamble for A-IoT downlink communication. Proposal 16: R2D postamble should not be introduced to indicate the end of PRDCH transmission. TBS indication should be used to implicitly indicate the packet size and transmission time interval and could be included in the control information of PRDCH. |
| ZTE [17] | - Proposal 5: In addition to indicating the chip duration, the design of preamble should also consider the impact of CP. - Observation 7: Fixed duration of low voltage cannot be used as start-indicator part if EH signal (or high voltage) is not transmitted before it. - Proposal 6: For the start-indicator part of the R2D preamble, Option 1: ON/OFF pattern, i.e. high/low voltage transmission, is proposed. - Proposal 7: The start-indicator part should have a fixed length, unaffected by variations in M values. - Proposal 8: For the start-indicator part of the R2D preamble, fixed sequence(e.g., [1 0] ) with fixed M=1 value can be considered. - Proposal 9: When designing the clock-acquisition part, the following factors should be taken into account: - the length of the clock-acquisition part, - the CP handling. - Observation 8: Using multiple alternating high and low voltages, where each voltage occupies one chip, will not introduce additional transition edges, irrespective of the M value, thereby facilitating better decoding performance. This approach is applicable across different M values, simplifying the decoding processing.. - Observation 9: Clock-acquisition part with fixed number of chips may introduce a situation where an OFDM symbol contains both the clock-acquisition part and the data. Since the data is random, it is possible that the CP introduces new transition edges within this OFDM symbol. - Proposal 10: Clock-acquisition part with a pattern of multiple alternating high and low voltages, consisting an odd number of chips, where each voltage level corresponding to a single chip can be considered. - Proposal 11: To determine or derive the end of PRDCH transmission, Option 1：R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH is proposed. - Proposal 12: For postamble pattern, the following two options can be considered: - Option#1: high voltage transmission(preferred) - Option#2: low voltage transmission - Note: The length of postamble should be greater than two chips. |
| Panasonic [18] | Proposal 1: The R2D transmission should be followed by an OFF signal and a known ON/OFF pattern when the device previous operation can be harvesting the energy. The ON/OFF pattern should be differentiated from the control/data information, e.g., using a unique sequence. Proposal 5: The midamble should not be considered for the R2D transmissions. |
| Sony [20] | Observation 1 – Selecting a preamble from sequences with good auto- and cross-correlation properties, the detected peak can be used both for synchronization and indicator of the start of the transmission, eliminating the need for a separate start-indicator in the preamble. Observation 2 – Suitable design of both R2D signals and its reception can avoid the need for any separate start-indicator for instance to determine OOK rising/falling threshold setting. Proposal 1 – Start indicator can be optional for some AIoT devices. Proposal 2 – Support to include OFF pattern as start indicator for AIoT devices with RF harvesting capabilities. Proposal 3 – Select known sequences with good correlation properties clock-acquisition part of the R2D time acquisition signal. |
| Honor [23] | Proposal 1 The design of the R2D preamble is fixed to use the OOK-1 and/or OOK-4 M=1 transmission pattern, and the chip length is acquired/distinguished through different bits information. |
| LGE [24] | Proposal 9: Consider multiple on/off patterns for the clock-acquisition part of R2D preamble which do not occur during PRDCH transmission. |
| Samsung [25] | Observation 1: In A-IoT system, different from RFID, the CW and the R2D transmissions may occur in different frequencies depending on the device types or the topologies. Moreover, in the case of device 2b, there will be no CW signal provisioned prior to the R2D transmission. Proposal 3: Study the star-indicator part based on Option 1 (ON/OFF pattern i.e., high/low voltage transmission) as a baseline. Proposal 5: For R2D, study a clock acquisition part considering different encoding schemes (i.e., Manchester and PIE). Proposal 7: Study clock acquisition part sequence design, which does not incur a false rising or falling edges due to CP insertion when CP-OFDM is used for OOK signal generation. Proposal 8: For R2D transmission, a presence of the midamble is known by a target device either explicitly or implicitly. - Explicit indication is provided in the header field of PRDCH such as the number of payload segmentations and the payload segment size. - Implicit indication is based on either the PRDCH message type and/or the total payload size indicated in the header field of PRDCH. - For a long R2D transmission, if the transmission needs to be paused in the middle of signaling due to charging, the resumed transmission starts with a midamble. Proposal 11: For both R2D and D2R transmission, study a postamble with a short sequence for end-of-signaling indication purpose. |
| ETRI [26] | Proposal 1: The length of a start-indicator part in the R2D preamble is fixed. Proposal 2: ON/OFF (High/Low) voltage pattern is included in the start-indicator part of the R2D preamble if it is not confirmed that there is always a high-voltage transmission before the start indicator part. Proposal 3: To determine the pattern and length of the clock-acquisition part of R2D preamble, it is necessary to study whether the clock-acquisition part can be used for other functionality and its relationship to the R2D control information on OOK chip duration. |
| Apple [28] | Observation 1: For the start-indicator part of the R2D time acquisition signal, if only low-voltage continuous transmissions is supported, then this may not be sufficient for device to determine the start because there is no specific agreement to have high voltage level before the start-indicator part Proposal 1: For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is considered for further study - FFS: Duration(s) of start-indicator part Proposal 2: For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for the following PRDCH carrying control information and/or data - If it is justified to signal separate chip duration for the control information and the data transmitted on PRDCH, then two parts for the clock-acquisition part with different chip durations can be considered Proposal 3: For R2D, the clock-acquisition part of the R2D time acquisition signal should start with a different voltage level compared to the voltage level of the end of the start-indicator part - If the start-indicator part ends with low-voltage level, then the clock-acquisition part starts with high-voltage level or vice-versa Proposal 4: For R2D time acquisition signal, study variable length to indicate different transmission schemes for PRDCH for signaling what R2D information type is mapped to the PRDCH including data, control, system information. |
| Sharp [29] | Proposal 1: For the start-indicator part of the R2D time acquisition signal, support Option 1 (ON/OFF pattern i.e. high/low voltage transmission). |
| Docomo [30] | Proposal 1: - For the start-indicator part of the R2D time acquisition signal, clarify that option 2 (i.e., OFF pattern) assumes there is high voltage before the OFF duration and after the OFF duration. - For the start-indicator part of the R2D time acquisition signal, OFF pattern of option 2 is falling edge + OFF with defined duration + rising edge. Proposal 2: - At least following aspects should be considered for the design of start-indicator. - Accuracy of detection - Complexity and power consumption of detection Proposal 3: - Study following options for determination of chip duration of PRDCH. - Option1: chip duration of R2D data and chip duration of R2D control are determined from clock acquisition part. - Option2: chip duration of R2D control is determined from clock acquisition part, chip duration of R2D data is indicated in R2D control. - Option3: chip duration of R2D control is fixed, chip duration of R2D data is indicated in R2D control. Proposal 4: - If clock-acquisition part is used to determine chip duration of PRDCH, clarify which of the following alternatives is assumed. - Alt.1. based on edge detection of clock acquisition part, e.g., duration between two edges is duration of a chip of subsequent R2D. - Alt.2. based on sequence of clock acquisition part, e.g., different sequence refers to different chip duration. Proposal 5: - Differentiation between start indicator part, clock acquisition part and PRDCH needs to be considered in the design of R2D preamble. Proposal 6: - CP impact needs to be considered in the design of R2D preamble. Proposal 7: - For the R2D timing acquisition signal, study a preamble with one part, which can be a ON/OFF pattern. The one-part R2D preamble indicates start of R2D transmission and is used for timing calibration/synchronization. Proposal 8: - Study temporarily periodic R2D sync signal without subsequent PHY R2D channel. |
| Qualcomm [31] | Proposal 1: For the R2D timing acquisition signal, - Study the start-indicator part to have a power-ON/OFF portion for AGC/threshold setting and a predefined OOK pattern/sequence (Opt1) for correlation detection, different from that of the remaining part of the preamble. - Study the clock-acquisition part to have a OOK pattern/sequence with uniform OOK chip duration after CP insertion. - FFS: the length of the clock-acquisition part needed for device clock sampling frequency calibration. - FFS: whether to need additional part for frequency synchronization. Proposal 2: For the D2R timing acquisition signal, a pattern/sequence is used to indicate the start of the transmission, AGC setting, transmission timing and channel estimation for coherent detection if supported. |
| China Unicom [32] | Proposal 2: It is suggested that the activation function should be implemented in the preamble, while the devices matching function can be implemented in control information on the PR2DCH. |
| Cewit [33] | Proposal 1: Support common pattern as start indicator for group of devices. Proposal 2: Support following options for indication of pattern as start indicator : - Fixed pattern specified in standards. - Can be configured in system information. |

[Closed] 1st Discussion Round

Start-Indicator part

~24 companies provided their views on the start-indicator part of the R2D preamble. Majority of these companies (~15) support the ON/OFF pattern (high/low voltage transmission). This pattern is favored because it provides a clear and distinguishable signal that enhances the reliability of detecting the start of R2D transmissions, reducing the chances of false or missed detections. On the other hand, a few companies (~7), support the low-voltage only (OFF pattern) approach. This approach is primarily motivated by the need for lower power consumption. However, most of these proponents acknowledge that the inclusion of a preceding high-voltage signal is necessary to ensure reliable detection, thus addressing the potential drawbacks of using a low-voltage signal alone. Therefore, based on this summary, proposal 2.1.1-1 is provided.

(HP)Proposal 2.1.1-1

For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern i.e. high/low voltage transmission is applied

- Fixed length high-voltage is followed by fixed length low-voltage transmission

- FFS: Exact duration of the high and low voltage transmission

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | OK |  |
| TCL | Agree with this proposal. If the start chip is low level, device may not correctly judge the first chip of start-indicator by envelop detection because of the threshold or interference/noise. In addition, same line code with clock acquisition part like Manchester coding could be considered to reduce the detection complexity. |  |
| xiaomi | OK |  |
| FUTUREWEI | ok |  |
| Qualcomm |  | We suggest RAN1 to study two alternatives for ON/OFF pattern. Based on our simulation study, Alt2 with simple correlation detection shows much better performance than Alt1. ##### (HP)Proposal 2.1.1-1a For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern, i.e., high/low voltage transmission is applied Alt1: Fixed length high-voltage is followed by fixed length low-voltage transmission - FFS: Exact duration of the high and low voltage transmission Alt2: Predefined ON/OFF sequence - FFS: sequence design and duration |
| Spreadtrum | OK |  |
| Honor | OK |  |
| MTK | OK |  |
| LG Electronics | OK |  |
| Lenovo |  | Fine with Qualcomm updated proposal. |
| Ericsson | Support in principle. But the fixed length may have some problem. If an RF charging signal is provided with the same high voltage as ON part before R2D start-indicator, a device may not be able to distinguish between RF charging signal and the ON part and therefore cannot determine the length of ON part. To solve the problem, the fixed length of the ON part, i.e., high-voltage one, is actually a minimum length. A fixed length for low-voltage transmission is needed. After the low voltage (OFF) transmission, the device will need to detect a high voltage (ON) signal. It will be good to have add “FFS: whether the high voltage (ON) signal after the low voltage (OFF) signal studied as part of the start indicator or the clock acquisition signal? |  |
| InterDigital | ok |  |
| Samsung | We support in principle start-indicator part to be based on a fixed length ON/OFF pattern. However, the sub-bullet point “Fixed length high-voltage is followed by fixed length low-voltage transmission” is confusing as it is interpreted as 10, i.e., H-L. The exact H/L sequence and the length can be further discussed. Proposed modified proposal: For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern i.e. high/low voltage transmission is applied - FFS: Exact sequence/length of the ON/OFF pattern. - Fixed length high-voltage is followed by fixed length low-voltage transmission - FFS: Exact duration of the high and low voltage transmission |  |
| ZTE, Sanechips | OK |  |
| Moderator (Apple) | Updated proposal provided for 2nd round based on the inputs |  |

Clock-acquisition part

~22 companies provided their views on the clock-acquisition part of the R2D preamble. Multiple aspects related to the clock-acquisition part are discussed. First aspect is the determination of the chip duration for both the data and control in the following PRDCH. Majority of companies agree that the clock-acquisition part should be used to determine the chip duration for both data and control transmissions. Second aspect is on the determination of the chip duration by calculating the duration between the adjacent rising/falling edges. For this purpose, most of the companies propose that at least 2 falling/rising edges should be considered for the patterns design of the clock-acquisition part. Moreover, it is observed that with multiple transitions of falling/rising edges, the chip duration accuracy can be better. Third aspect is related to whether the clock-acquisition part has one fixed pattern and/or duration. Regarding one fixed pattern, it is argued that to keep low-complexity for detecting clock-acquisition part, a fixed pattern is preferable. However, to ensure that different chip durations can be signaled via the clock-acquisition part, the duration for the same fixed pattern can be different, i.e. consider supporting same fixed pattern with different durations. Moreover, the fixed patterns should be designed such that it is unique relative to the following PRDCH transmission. Fourth aspect is related to the CP handling in the clock-acquisition part. In agenda 9.4.2.1, it is agreed to study whether/how CP inserting is handled at the transmitter/receiver side. And this discussion may also impact how the CP inserting may impact the clock-acquisition part’s pattern/design. However, considering that the discussion is still ongoing in the other agenda, and depending on the progress of that discussion, it can be later discussed, whether/how the CP inserting is taken into account for the clock-acquisition part design. Furthermore, it is proposed by some companies to further consider/justify whether there is a need for frequency synchronization and if clock-acquisition part can be utilized for this purpose. Based on the discussion, following proposals are made related to the clock-acquisition part for the R2D preamble.

(HP) Proposal 2.1.1-2

For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for both the data and control information transmitted in the subsequent PRDCH

- FFS: Whether the R2D clock-acquisition part can be used to determine the chip duration of the subsequent PDRCH

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | We are fine with the proposal basically. Regarding the chip duration of sub-sequent PDRCH, we believe probably it cannot be determined according to clock-acquisition part only, considering FDM would be supported for PDRCH and control information is obviously needed. This is different from the determination of PRDCH chip duration. Therefore, we propose following change for clarity. - FFS: Whether the R2D clock-acquisition part can be used as reference to determine the chip duration of the subsequent PDRCH (if any) |  |
| TCL | Okay. In last meeting, it was agreed that clock-acquisition part is used to determine the R2D chip duration. One issue is how/when to use the indicated chip duration for R2D. One option is that the indicated chip duration is used to the subsequent physical channel transmission. It means that the chip duration can be changed for each R2D transmission. Another option is the R2D chip duration is fixed in a certain time. For example, during a inventory round, the R2D chip duration is fixed. |  |
| xiaomi | Yes, and we are also fine with OPPO’s modification considering the FDM operation. |  |
| FUTUREWEI | ok |  |
| Qualcomm |  | It is not clear whether the ‘subsequent PRDCH’ will carry control only, data only or control and data. And it is also not clear whether the control and data if using separate coding and separate CRC will use the same chip rate or not. So we prefer to defer the discussion until we have more clear understanding on R2D control discussion. |
| Spreadtrum | OK with the proposal. |  |
| Honor | We prefer the modification of OPPO because the D2R bandwidth and chip duration could be different with R2D. |  |
| MTK |  | For main bullet, similar view as QC |
| LG Electronics | OK with FL proposal |  |
| Lenovo |  | We share the same view as OPPO. |
| Ericsson |  | We see this is one option, while other options can be considered as well. We suggest the following. For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for both the data and control information transmitted in the subsequent PRDCH. OOK chip duration of data in the subsequent PRDCH can be indicated by control information. |
| InterDigital | ok |  |
| Samsung | We are fine with R2D part. For PDRCH chip duration to be based on PRDCH needs further discussion, as R2D, D2R can be based on different line coding scheme. Proposed revised proposal: For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for both the data and control information transmitted in the subsequent PRDCH FFS: Whether the R2D clock-acquisition part can be used to determine the chip duration of the subsequent PDRCH |  |
| ZTE, Sanechips | This proposal is also related to the following in 9.4.2.2: FL1 Medium Priority Question 5.2.3-1: Whether it is necessary and What is the pros and cons for having different OOK chip duration for R2D control and R2D data transmission? If the OOK chip duration for R2D control and R2D data transmission are same, we are OK with this proposal. Also Ok with OPPO’s modification version. However, if the OOK chip duration for R2D control and R2D data transmission are different, we don’t think the clock-acquisition part should indicate the chip duration for both the data and control. The clock-acquisition part can indicate the chip duration for control and how to indicate the chip duration for data can be FFS. Therefore, the following modification is proposed: For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for both the data and control information transmitted in the subsequent PRDCH - FFS: Whether the R2D clock-acquisition part can be used to determine the chip duration of the subsequent PDRCH - FFS: Whether the R2D clock-acquisition part can be used to determine the chip duration for both the data and control information transmitted in the subsequent PRDCH. |  |
| Moderator (Apple) | Based on the inputs so far, updated proposal provided for 2nd round. Basically, two options are added based on preference from multiple companies |  |

(HP) Proposal 2.1.1-3

For R2D, the clock-acquisition part should include integer multiple of adjacent transition edges and based on the duration between two adjacent transition edges, the device can determine the OOK chip duration of the subsequent PRDCH

- FFS: Exact pattern design(s) and duration(s)

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO |  | We propose not to restrict that the duration of a chip in clock-acquisition part is the same as the chip duration of subsequent PRDCH before RAN1 figure out how many transition edges are needed, and which M value(s) for OOK-4 are to be supported. As it may happen that multiple transition edges are needed all the time even for small M (e.g. M=1) , in the case, the overhead of clock-acquisition part is too high, and chip duration in clock-acquisition part smaller than that in PRDCH should be considered. At this stage we propose: For R2D, the clock-acquisition part should include integer multiple of adjacent transition edges and based on the duration between two adjacent transition edges, the device can determine the OOK chip duration of the subsequent PRDCH based on at least the duration between two adjacent transition edges. - FFS: Exact pattern design(s) and duration(s), including whether the duration between two adjacent transition edges and the OOK chip duration are the same or can be different. |
| TCL |  | We think CP insertion may impact the length and pattern of clock acquisition part. This proposal should be postponed to discuss until the issue of whether CP should be inserted between start of indicator and clock acquisition part or not has been discussed. |
| xiaomi | Y |  |
| FUTUREWEI | ok |  |
| Qualcomm |  | Agree with TCL’s concern. May need to wait for CP insertion discussion. |
| Spreadtrum | OK |  |
| Honor | Y |  |
| LG Electronics |  | Agree to wait for CP insertion discussion. |
| Lenovo |  | Fine with TCL’s suggestion. |
| Ericsson |  | Please check our comment to Proposal 2.1.1-1 on ‘chip duration of the subsequent PRDCH’. |
| InterDigital |  | Ok to wait for CP insertion progress |
| Samsung |  | We don’t quite understand how to interpret “integer multiple of adjacent transition edges and based on the duration between two adjacent transition edges”? |
| ZTE, Sanechips | General OK with this proposal. The CP handling should also considered. Thus, the following modification is proposed: For R2D, except for the CP, the clock-acquisition part should include integer multiple of adjacent transition edges and based on the duration between two adjacent transition edges, the device can determine the OOK chip duration of the subsequent PRDCH - FFS: Exact pattern design(s) and duration(s) |  |
| Moderator (Apple) | Based on the comments, let’s discuss this proposal later after the progress on CP handling in agenda 9.4.2.1 |  |

Others

In previous RAN1 meetings, it was also discussed to further consider any additional parts for R2D time-acquisition signal. Based on the inputs in the submitted contributions, there seems to be no strong views on support of additional part. One additional purpose considers if frequency synchronization is needed or not and whether an additional part would be needed. For the purpose of frequency synchronization, if needed, it could possibly be considered within the design of the clock-acquisition part. Other than this, there is no need for any additional parts for R2D time acquisition signal. Therefore, following proposal is made.

Proposal 2.1.1-4

For R2D, no other parts (other than start-indicator part and clock-acquisition part) are further considered for the R2D timing acquisition signal

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | Agree with this proposal. As shown in figure as below, optional part is inserted between start-indicator and clock-acquisition part. Extra functionality can be considered for optional part, including implicit indication, explicit indication, mute functionality for interference estimation, etc. Figure. Preamble structure with optional part |  |
| FUTUREWEI | ok |  |
| OPPO | OK |  |
| Qualcomm |  | We suggest FFS whether to add additional part for the frequency synchronization. |
| Spreadtrum | Yes |  |
| Honor | OK |  |
| LG Electronics | OK |  |
| Ericsson | Support the proposal. |  |
| ZTE, Sanechips | Agree with this proposal. Additional parts will increase the complexity of preamble. And we don’t see the necessity for introducing other parts. |  |
| Moderator (Apple) | Seems like everyone, but one company is fine with proposal. Please provide further comments, if any, in second round @Qualcomm: On additional functionality for frequency synchronization, clock-acquisition could be potentially considered and based on that, we can consider not to support additional parts. |  |

[Open] 2nd Discussion Round

(HP) Proposal 2.1.1-1A

For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: Exact sequence/length of the ON/OFF pattern.

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| TCL | Okay |  |
| LG Electronics | OK |  |
| Huawei, HiSilicon |  | We do not agree that we need an ON-OFF pattern, and a fixed-length low-voltage signal should be used. This can be easily distinguished and detected by the energy detector of the device from the preceding high-voltage signal which is transmitted by the reader for a short period of time prior to the transmission of the start indicator and PRDCH, such that devices that are monitoring for the PRDCH are aware of an upcoming transmission. The advantage of using this design is that the length of the start indicator part can be reduced, while at the same time the timing acquisition signal can be differentiated by the device from the following PRDCH transmission. |
| Lenovo | Yes |  |
| OPPO | We are wondering whether the proposal also includes the OFF pattern, as in this pattern the OFF chip is preceded by high-voltage, it is also ON->OFF pattern essentially. If it is not the intention, maybe we can clarify that the ON/OFF pattern is transmitted from the beginning of OFDM symbol, as we have agreed that for OFDM waveform R2D transmission is aligned with symbol boundary. |  |
| Spreadtrum | Yes |  |
| Samsung2 | Yes |  |

(HP) Proposal 2.1.1-2A

- For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for at least the L1 R2D control information (if any) transmitted in the subsequent PRDCH

- For determining the chip duration for the data transmitted in the subsequent PRDCH, following two options are studied:

  - Option 1: Clock-acquisition part of the R2D time acquisition signal is used

  - Option 2: R2D control information is used

- FFS: Whether the R2D clock-acquisition part can be used to determine the chip duration of the subsequent PDRCH

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| TCL | Yes |  |
| LG Electronics | Option 2 can be changed to ‘L1’ R2D control information. |  |
| Huawei, HiSilicon |  | We do not agree with the proposal. For the first bullet, it does not make sense to discuss whether the chip duration applies to L1 R2D control information when we have not even agreed whether we need the L1 R2D control information or not. In our understanding, only a single chip duration will be determined using the clock acquisition signal, and this is for the corresponding PRDCH transmission, immaterial of whether is contains control, data or both. This was already agreed in the last meeting, so the 2nd bullet is currently repeating something we already agreed previously. We should really be focusing on the FFS that was captured as part of the previous agreement, which is to determine the pattern design. To this end, what we would like to capture is that the clock acquisition part includes at least two transition edges to enable transition edge detection for determining the chip length of each R2D transmission. The duration between adjacent transition edges is an integer multiple of the R2D chip length. |
| Lenovo | Yes The second option is about having different chip length for L1R2D control and subsequent PDRCH. The question remains whether second clock acquisition signal is needed for subsequent PDRCH or not? Otherwise chip length indicated in L1 R2D used for the subsequent PDRCH |  |
| OPPO | It is true that we have not agreed L1 R2D control information, although we believe at least message type and reader/device ID should be transmitted as L1 R2D control information. But at this stage we suggest the following: - For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for at least the L1 R2D control information (if any) transmitted in the subsequent immediately followed PRDCH transmission. - For determining the chip duration for the data transmitted in the subsequent PRDCH, following two options are studied: - Option 1: Clock-acquisition part of the R2D time acquisition signal is used - Option 2: R2D control information is used - FFS whether chip duration of all subsequent PRDCH transmission is the same or can be different. - FFS: Whether the R2D clock-acquisition part can be used as reference to determine the chip duration of the subsequent PDRCH |  |
| Spreadtrum | Yes |  |
| Samsung2 | Agree in principle. We can further discuss the details. |  |

(HP) Proposal 2.1.1-4

For R2D, no other parts (other than start-indicator part and clock-acquisition part) are further considered for the R2D timing acquisition signal

Note: this doesn’t preclude potentially additional functionalities with the clock-acquisition part

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| LG Electronics | OK |  |
| Lenovo | Yes |  |
| OPPO | OK |  |
| Spreadtrum | Yes |  |
| Samsung2 | Yes |  |

[Closed] 3rd Discussion Round

(HP) Proposal 2.1.1-1C

For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied

- Duration of the ON part can be variable

- Duration of the OFF part is fixed

(HP) Proposal 2.1.1-2B

- For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for at least the immediately following PRDCH

  - FFS: Whether the R2D clock-acquisition part can be used to determine the chip duration of the subsequent PDRCH

### R2D Control information (including TB size)

| Company | Observations/Proposals |
| --- | --- |
| Futurewei [1] | Proposal 1: For Ambient IoT, no scheduling information for a PDRCH transmission is carried in the L1 R2D control information of the PRDCH. Proposal 2: Study a design for the L1 R2D control information that: - Carries the following: device ID; cast type, next decode field, Reserved bits - If device ID can indicate broadcast / groupcast, then cast type is not needed. - Does not carry message type nor D2R parameters. - FFS TBS, and Reader ID Proposal 3: For the L1 R2D control information format, study means of supporting flexible number of bits. |
| Nokia [2] | - Observation 8: The broadcast information containing configuration for channel access, e.g., initial contention-based access, may be beneficial, and may precede any device-specific configuration information. - Proposal 5: RAN1 to study what broadcast information is required for the AIoT device and to identify how it should be included in the PRDCH channel. - Observation 9: The need for having a separate control channel depends on the type of information it must carry and the size of the message. If the content of the control information is small, perhaps a separate control channel may not be efficient. However, if different device types support different set of configurable parameters, it may be worth to study the possibility of having a separate control channel in addition to the data channel. - Observation 10: The PRDCH may carry a wake-up info to select a subset of devices for the inventory process in addition to the modulation and resource allocation fields targeted towards a specific device type. - Proposal 6: RAN1 to study the necessary control information to be contained in PRDCH depending on the AIoT device type, including activation and query commands. Proposal 7: RAN1 should consider a flexible design, i.e., by having control only, data only, or control and data fields all carried by PRDCH itself. |
| Ericsson [3] | - Observation 1: Some R2D control information will be A-IoT device-type agnostic and generally applicable to all device types whereas other may be A-IoT device-type specific. - Observation 2: The R2D control information for A-IoT devices should be designed flexibly to cater to both active and passive device types. - Proposal 1: If an R2D preamble transmission is followed by L1 R2D control information, the chip duration of the following R2D data can be indicated using the L1 R2D control information. - Observation 3: If a certain control information is needed to decode PRDCH data, it needs to be transmitted as L1 R2D control information. Otherwise, it can be transmitted as L1 R2D control information or higher layer data. - Proposal 2: Time domain resource allocation, and chip duration for PRDCH scheduling can be transmitted as L1 R2D control information. - Proposal 3: The payload sizes of the fields in L1 R2D control information need to be fixed, because devices may not be able to do blind detection. - Observation 17: DL message size for command use case can be up to 944 bits assuming a 100-byte DL command. - Observation 18: The maximum TB size will depend on whether messages can be segmented into multiple smaller TBs or not. - Proposal 17: RAN1 to wait for coverage evaluation results before discussing the possible values for TB size. Table 1: Fields for PRDCH scheduling PRDCH control information for PRDCH scheduling; Needs to be signaled?; Signalled via?. Time domain resource allocation (TDRA); Yes; L1 control. Frequency domain resource allocation (FDRA); No Reason: FDMA is not supported for R2D. Same frequency resources are used for the transmission of L1 R2D control and the following data in PRDCH.; N/A. MCS/coding rate; No Reason: FEC is not supported for R2D. OOK chip duration is separately indicated.; N/A. TBS; No Reason: It can be calculated from time domain resource allocation and chip duration.; N/A. Repetitions; No Reason: It is supported for D2R only.; N/A. Device ID and/or device group ID and/or device type and/or cast type; FFS There is no discussion in RAN1 regarding device ID and/or device group ID and/or device type and/or cast type. Reader ID; FFS Motivation of this field is needed.. Chip duration; Yes; L1 control. Table 2: Fields for PDRCH scheduling PRDCH control information for PDRCH scheduling; Needs to be signaled?; Signalled via?. Time domain resource allocation; Yes; L1 control or higher layer data. Frequency domain resource allocation; Yes; L1 control or higher layer data. MCS/coding rate; Yes; L1 control or higher layer data. TBS; No Reason: It can be calculated.; N/A. Repetitions; Yes; L1 control or higher layer data. Chip duration; Yes; L1 control or higher layer data. |
| Huawei[4] | Proposal 1: For R2D transmission, the maximum TBS equals the maximum message size of 1000 bits. - Inform RAN2 of this agreement. Observation 1: Using a higher layer control element to transmit the D2R grant avoids physical layer design, specification, and implementation effort, while at the same time offering flexibility in terms of the size of the grant. Observation 2: Transmitting the D2R grant using a pre-defined set of resources within the PRDCH requires the device to be aware of the location of these resources beforehand, which would require additional PRDCH formats and the device to perform blind detection, while at the same time, reducing the flexibility and resource utilization efficiency. Observation 3: Transmitting the D2R grant using a set of resources within the PRDCH with separate CRC to enhance the reception reliability of the grant is not required since the device will not transmit a D2R transmission if it did not receive the data part of the PRDCH successfully. It may result in breaking the pipelining of the CRC decoding due to the added latency, and increases overhead. Observation 4: Transmitting the D2R grant using the R2D timing acquisition signal would negatively impact the ability of the device to determine the start time and chip length of the R2D transmission, without any clear motivation or justification. Proposal 2: A single CRC for PRDCH transmissions is supported, where the D2R grant is transmitted using a higher layer control element on the data part of the PRDCH. Observation 5: Reader ID is not needed to be included in the D2R grant. Proposal 3: D2R grant is transmitted in the preceding PRDCH, containing time-domain resource allocation, MCS/coding rate, TBS, repetitions, device ID, frequency domain resource allocation, chip duration, and midamble related information. Table 1 R2D Control information classification for PDRCH Information classification; Control information. Mandatory/optional; - Time domain resource allocation For the device, it is necessary to know the time domain duration of the PDRCH transmission and should be included in the D2R grant. - MCS/coding rate As discussed in our paper [5] and agreed previously, D2R FEC is supported. If there are different coverage enhancement levels in the system, MCS is a necessity. - TBS As discussed in our paper [3], TBS is needed for the device. - Repetitions As repetitions are being studied as agreed in the previous meeting, the repetition information is necessary for the control information. - Device ID Device ID is needed when the control information is used for scheduling transmissions for a specific device or a group of devices. - Frequency domain resource allocation As discussed in our paper [3], for FDMA, different frequency point should be allocated to different users which needs to be indicated by the D2R grant. Besides, take device 2b for example, it is necessary to indicate the frequency domain resource for D2R transmission. - Chip duration As discussed in our paper [5], chip length and line code repetition are different for different FDMA configurations when the total time length is the same. Therefore, for a specific device using FDMA, it is necessary to indicate the chip duration information in the D2R grant. - Midamble related information As discussed in our paper in [3], midamble is required for accurate timing acquisition, channel estimation and can help improve the system performance. Hence, midamble-related information such as the midamble pattern and number of midambles in a single transmission should be indicated in the D2R grant.. Unnecessary or can be indicated by other control information; - Reader ID It seems that there is no motivation for the device to know the exact ID of the reader.. |
| Spreadtrum [5] | Proposal 4: At least the following R2D control information for PRDCH are supported: - Time domain resource allocation - Coding rate - Device ID (/RN16) and/or device group ID Proposal 5: At least the following R2D control information for PDRCH are supported: - Time domain resource allocation - Coding rate - Repetition - Device ID (/RN16) and/or device group ID - Frequency domain resource allocation - Chip duration - Midamble related information |
| TCL [6] | Observation 8: R2D control information is needed for some cases, e.g., D2R transmission occassion, D2R modulation pattern for channel/interference estimation and L1 configuration should not be down selected. Proposal 11: For PRDCH, time-domain scheduling should be considered in R2D control information, and multiple different R2D channels with frequency-domain scheduling for FDM(A) should be deprioritized. Proposal 12: Discuss how to indicate the D2R transmission occassion for different types of device based on R2D control information. Proposal 13: Discuss concret contents of R2D control information and identify which control information should be configured by L1 or higher layer. |
| CMCC [7] | Proposal 17: TBS determination for R2D and D2R transmissions should be further studied, considering at least the presence or absence of postamble, frame structure (incl. number of transmission occasions, code block size for each transmission occasions, etc.), and R2D control information. If TBS is defined, the TBS size differs for each indices is not restricted to be equally difference. Proposal 8: For PRDCH, at least the following R2D control information can be studied: R2D control information; Fixed or signaled; Via L1 or PHY header; Via higher layer. TBS (if any); Signaled; Preferred. FFS: Chip duration; Signaled; Preferred. FFS: Early indication; Signaled; Preferred. Proposal 9: For PDRCH, at least the following R2D control information can be studied: R2D control information; Fixed or signaled; Via L1 or PHY header; Via higher layer. Time domain resource allocation; Signaled; Preferred. Frequency domain resource allocation; Signaled; Preferred. Coding scheme and coding rate; Signaled; Preferred. TBS (if any); Signaled; Preferred. Repetition; Signaled; Preferred. Chip duration; Signaled; Preferred. |
| Intel [8] | Proposal 3: - For R2D transmission, the following scheduling information can be considered: - Length of the data packet - Modulation and coding scheme if various data rates are supported - A-IoT device ID or group ID |
| Lenovo [9] | Proposal 4: Study a R2D control part immediately following the preamble of the R2D transmission and having separate encoding and CRC from the R2D data part. Proposal 7: Study data part of a PRDCH carrying scheduling information for the corresponding D2R transmission. Proposal 16: Study simple resource allocation scheme with a sub-channel in frequency domain and a time unit in time domain for ambient IoT indicated by a PRDCH for the corresponding D2R transmission. |
| China Telecom [10] | Proposal 6: For R2D control information, at least the following R2D control information (for PRDCH and/or PDRCH) are studied: - Time and frequency domain resource allocation - MCS/code rate - TBS - Repetitions - Device ID and/or device group ID and/or device type - Cast type - Reader ID |
| Vivo [12] | Proposal 1: Prioritize the discussion on ambles structure, and deprioritize the discussion on scheduling information. |
| Oppo [13] | Observation 1: In contrast to post-amble, it is more reliable and efficient to indicate TBS with control information. Proposal 1: TBS of PRDCH is indicated by control information. Observation 3: There exit broadcast transmission, groupcast transmission and unicast transmission in slotted-ALOHA access procedure. And both unicast and groupcast may be needed for support of command use case. Proposal 3: All broadcast, groupcast and unicast should be studied for R2D transmission. Observation 4: It should enable that a device can identify whether it is a target device of a PRDCH and whether the PRDCH is from the serving reader. Proposal 6: Scheduling information of PDRCH transmission should include at least time/frequency resource for the scheduled PDRCH, further study how to indicate the time/frequency resource. Proposal 7: Complexity and power consumption for R2D control information reception should be minimized, the following design principles should be considered: fixed MCS, no blind detection, minimal control information formats, and minimized FAR. Observation 5: Control information needed for support of inventory use case and command use case are different. Proposal 13: Maximum TBS of 1000 bits should be studied as baseline. Proposal 9: Study necessary control information per use case. Table 1 Control information for inventory use case Control information; PRDCH; PDRCH. Needed or not; L1 or higher layer; Needed or not; L1 or higher layer. Time domain resource; [N] (up to scheduling discussion); -; Y; L1 or higher layer. Frequency domain resource; [Y] (up to multiplexing discussion); L1; Y. MCS/repetition; [Y]; L1; [Y]. TBS; [N] (fixed, up to inventory procedure); -; FFS. Device/group ID; Y; L1 and/or higher layer; Y. Reader ID; [Y]; L1; [Y]. Cast type; [N] (fixed, up to inventory procedure); -; N. Table 2 Control information for command use case Control information; PRDCH; PDRCH. Needed or not; L1 or higher layer; Needed or not; L1 and/or higher layer. Time domain resource; [N] (up to scheduling discussion); -; Y; L1 or higher layer. Frequency domain resource; [Y] (up to multiplexing discussion); L1; Y. MCS/repetition; [Y]; L1; [Y]. TBS; Y; L1; FFS. Device/group ID; Y; L1; Y. Reader ID; Y; L1; Y. Cast type; Y; L1; N. |
| Xiaomi [14] | Proposal 1: For a R2D transmission, the corresponding L1 R2D control information is conveyed by a PHY header. - The payload size of the PHY header is fixed and pre-defined by the specification. Table 1 Fields in R2D control information Fields; Description. D2R time domain resource allocation; Indicating the time domain resources for one or multiple A-IoT devices’ D2R transmission.. D2R frequency domain resource allocation; Indicating the time domain resources for one or multiple A-IoT devices’ D2R transmission.. Repetitions; Indicating the number or types (bit level or TB level) for the follow-up D2R transmission.. The end of PRDCH (optional, depends on whether postamble can be used); For the device to determine or derive the end of PRDCH transmission, which is agreed as an option during last meeting.. Device ID and/or device group ID and/or device type; Indicating the current R2D transmission is target for a specific device or a specific group of devices or a specific type of devices.. Proposal 8: For R2D control information, at least the fields in Table 1 can be studied. |
| Fujitsu [15] | Observation 3: If scheduling-based R2D data transmission is supported, the scheduling information should be treated as control information. Observation 4: Delivering the control information for scheduling PRDCH in PHY layer requires fewer dynamic memories and shorter processing time compared with what required when delivering the control information via higher layer. Proposal 3: - Support PHY layer (L1) control information. - The L1 control information at least includes the scheduling information of PRDCH/PDRCH. Proposal 4: Study both device-specific and common scheduling information for PRDCH and PDRCH in Rel-19 SI. Proposal 5: If L1 control information is transmitted on PRDCH, two options can be considered for control information transmission: - Alt.1: L1 control information and R2D data are carried by separate PRDCHs. - Alt.2: One PRDCH carries both L1 control information and R2D data. Proposal 6: For R2D control information, - At least support: - TDRA information for R2D/D2R transmissions, - Device ID for device-specific scheduling. - Do not support: - TBS of D2R transmission, - Cast type. - Further study: - FDRA for R2D/D2R transmissions, - MCS for R2D/D2R transmissions, - Device group ID. |
| CATT [16] | Proposal 21: A set of TTIs for the devices to select for the transmission time of the PDRCH should be included in the PRDCH control information. Proposal 22: A-IoT channel resource allocation information in frequency domain should be included in control information of PRDCH. Proposal 23: Device identity information should be included in control information of PRDCH. Three kinds of device identity information can be considered as follows, - A-IoT device ID: only for single device indication. - A-IoT device group ID: only for group of devices indication. - A-IoT device type ID: only for PRDCH indicating a special type of device, i.e., sensors. Proposal 24: There is no dynamic indication of the waveform parameters and modulation scheme in the control information from PRDCH. Proposal 25: Command type may be included in the PRDCH control information to indicate different types of R2D command messages. Proposal 26: The indication of TB sizes for both PRDCH transmission and PDRCH transmission should be included in the control information of PRDCH. - TB size for PRDCH transmission is indicated by the length of TTI as a separated control field of PRDCH. The TB size for PDRCH transmission is included in the control information field of PRDCH along with the time and frequency resource allocation. Proposal 27: D2R Repetitions information may be included in the PRDCH control information to indicate the types and related parameters of D2R transmission repetition, if D2R transmission repetition is introduced. Proposal 20: For PRDCH, transport block size is no more than 128 bytes according to the message sizes requirements for SA1 applicable use cases of indoor inventory and indoor command, the higher layer control information and the NAS control information carried by PRDCH. - The candidate transport block sizes for PRDCH are {128, 256, 512, 1024} bits. |
| ZTE [17] | - Observation 1: The R2D control information should include the device ID and/or device group ID, as well as the device type or cast type for R2D and D2R transmission, to enable early termination of signal/channel detection and reduce device power consumption. - Observation 2: The R2D control information should include the repetition times and/or code rate for R2D and D2R transmissions, to help decoding and/or transmitting signaling. - Proposal 1: R2D L1 control information includes the following information: - Device type or device group ID - Repetition times/code rate - Proposal 2: R2D L1 control information and data are encoded independently. - Observation 3: Reader ID can be considered at least in R2D signaling. - Observation 4: For D2R transmission, the TDRA, [FDRA], chip duration should also be indicated in R2D control information. - Proposal 3: The following contents of control information are proposed. Information; Needs to be Signaled?; Via L1 Control; Via higher-layer. Device ID; Y; N; Y. device type or device group ID; Y; Y; N. For R2D transmission. Repetition times/code rate; Y; Y; N. Reader ID; Y; N; Y. For D2R transmission. TDRA; Y; N; Y. FDRA; [Y]; [N]; [Y]. Repetition times; Y; N; Y. code rate; Y; N; Y. chip duration; Y; N; Y. - Proposal 4: CRC attachment for L1 control information is not needed. |
| Panasonic [18] | Proposal 2: The control information for R2D should be carried alone or along with the data. The control information includes system information, synchronization, configuration of contention-based channel access, the indication of R2D data transmissions, and D2R channel time boundaries. Proposal 3: The variable control information size should be supported. Proposal 6: The message size could be determined either using a message size indication in the control information or a postamble. The overhead of message size indication and postamble should be further studied. Proposal 7: The message size should be deterministic by the upper layer information. Variable message size depending on the information sequence using escape sequence should be avoided. |
| Interdigital [19] | Proposal 10: R2D scheduling information can include the following for an AIoT device: - Higher-layer control information (e.g. Device/group ID, session ID) - Number of slots for slotted-ALOHA based access - Payload of the R2D and D2R transmission - Duration of the R2D - Duration or maximum duration of D2R transmission - Frequency information of the scheduled D2R transmission - Number of repetitions for D2R transmission - Coding rate for D2R transmission - Power control command Proposal 3: Ambient IoT specification to support a maximum transport block size of approximately 1000 bits. |
| Continental [21] | Observation 1.1.: To support dynamic TDMA for D2R transmissions by one or more A-IoT devices, it is necessary to indicate the time-domain scheduling information for a specific device or group of devices in an R2D transmission. Observation 1.2.: To support dynamic FDMA for D2R transmissions by A-IoT devices, it is necessary to indicate the carrier frequency to be used by a specific device or group of devices in an R2D transmission. Proposal 1.1.: The following dynamic control information is proposed to be included for R2D transmissions: Information; Needs to be signaled?; Via L1 Control; Via higher layer. Time-domain scheduling information/indication; Y; Y; N. Carrier frequency/Frequency-domain scheduling information; Y; Y; N. Request for information on charging status/time required for device to be charged to certain level; Y; N; Y. |
| NEC [22] | Proposal 1: PRDCH control information indicates the data rate/line code chip duration by clock-acquisition part of the timing acquisition signal. Proposal 2: For the repetition number of PRDCH transmission, both explicit and implicit indication can be considered, and option 1 is slightly preferred: - Option 1: PRDCH control information indicates repetition number of data transmission for PRDCH directly, which is located at front of PRDCH with a fixed repetition numbers for the repetition number indication. - Option 2: Midamble or special sequence located between two repetitions indicates the repetition numbers via its length, type or format, which are associated with repetition numbers. Proposal 4: If channel coding is supported for PDRCH, study the following for indicating the channel coding code rate: - Option 1: code rate for PDRCH is indicated by R2D control information - Option 2: code rate for PDRCH is indicated by D2R control information Proposal 6: An additional offset can be indicated for PDRCH transmission reference to the repetition number of PRDCH transmission if the repetition number of PDRCH transmission is indicated by R2D control information. |
| Honor [23] | Proposal 2 The R2D control can be used to provide the information to schedule the R2D transmission, and at least the following contents can be considered, including: - Early control indication, such as unicast, multicast, broadcast, device type/device ID/device group ID/reader ID, etc. - Time domain resource allocation - The number of information bits - Energy harvesting indication - Repetitions Proposal 3 The R2D control can be used to provide the information to schedule the D2R transmission, and at least the following contents can be considered, including: - Time domain resource allocation - Frequency domain resource allocation - MCS - The number of information bits - Power control - Repetitions Proposal 4 Support adding the control field in the PRDCH, which can at least indicate the data transmission type, data field length, scheduling information, etc. Table 1: Contents of R2D control information Contents of control information; R2D control to schedule R2D data transmission; R2D control to schedule D2R data transmission. Early control indication; Y; N. Time domain resource allocation; Y; Y. Frequency domain resource allocation; N; Y. MCS; N; Y. The number of information bits; Y; Y. Energy harvesting indication; Y; N. Power control; N; Y. Repetitions; Y; Y. |
| LGE [24] | Observation 1: For R2D transmission, if Command ID is included in L1 control information and implicitly indicates a known size of a fixed TB, we do not need TBS indication or postamble for the transmission. Proposal 1: For PRDCH including command with a known fixed TBS, command ID implicitly indicating a known size of a fixed TB is included in L1 control information immediately preceding a TB. In this case, neither TBS indication nor postamble is needed for the PRDCH transmission. Observation 2: For D2R transmission in response to R2D transmission, the reader can assume TBS and length of D2R transmission in some cases, e.g. MSG1 responding to MSG0 in contention-based access. The postamble can be omitted for any known fixed size of D2R transmission to avoid overhead. Proposal 2: For D2R transmission in response to R2D transmission, a known fixed TBS of D2R transmission without postamble based on a command in R2D transmission is supported. Proposal 3: For PRDCH/PDRCH with a variable TBS, RAN1 studies the following options: - Option 1: Postamble follows PRDCH transmission without padding. - Option 2: TBS indication can be included in R2D L1 control information. FFS with/without postamble. - If actual TBS is equal to one of TBS candidate values, TBS indication indicates the TBS value without padding or postamble. - FFS if actual TBS is less than one of TBS candidate values. Proposal 5: A (temporary) device ID can be included in L1 control information for unicast PRDCH transmission. Proposal 6: The cast type i.e. indication to unicast, groupcast or broadcast is not included in R2D control information. Proposal 7: Consider the following tables for R2D control information: Table 1: R2D control information for PRDCH Information; Needs to be Signaled; Via L1 Control; Via higher-layer; Note. Command ID; Mandatory; Yes. TDRA; FFS. MCS; FFS. TBS; Optional; Yes. Repetitions; Optional; Yes (if supported). FDRA; Not needed; the same frequency allocation is applied to both L1 control information and TB.. Device ID; Optional for unicast; Yes. Reader ID; Optional; Yes (e.g. for Proximity Determination or aperiodic system information, if supported); This information can be up to RAN2. Device Group ID; Optional for groupcast; Yes (e.g. for Query/Paging). Device Type; Optional for groupcast; Yes (for Query/Paging). Cast Type; Not needed; Presence of device ID or device group ID can implicitly indicate cast type.. Table 2: R2D control information for PDRCH Information; Needs to be Signaled; Via L1 Control; Via higher-layer; Note. TDRA; Maybe; TD2R in option 2 can be indicated as TDRA. MCS; Optional; FFS; FFS. (Maximum) TBS; Optional; FFS; FFS. Repetitions; Optional (if supported); FFS; FFS. FDRA; Optional for frequency shift; FFS; FFS. Device ID; Optional for unicast; FFS; FFS. Reader ID; Optional; Yes (e.g. for Proximity Determination). Device Group ID; Not needed. Device Type; Optional; FFS; FFS. Cast Type; Not needed. Chip Duration; Optional; FFS; FFS; If chip duration of D2R transmission is different than that of R2D transmission. |
| Samsung [25] | Proposal 12: For R2D control information, at least the following R2D control information (for PRDCH and/or PDRCH) are studied: - Message type - Transmitter ID - Receiver ADDR - Payload size - Modulation type/order - Time and frequency domain resource allocation information - repetition |
| ETRI [26] | Proposal 4: Study on R2D control information considering the type of message to be delivered and the structure of PRDCH. |
| Mediatek [27] | Proposal 1: For R2D control transmission, at least the following information can be studied: - L1 control: time and/or frequency resource allocation for D2R transmissions, MCS/coding rate/BLF for R2D and D2R transmissions, TBS of R2D and/or D2R transmission, repetition. - L1 and/or higher layer control: device ID and/or device group ID and/or device type and/or cast type. - FFS time and/or frequency resource allocation for R2D transmission. |
| Apple [28] | Proposal 5: For signaling R2D control information for PRDCH, consider following table as a baseline: Information; Fixed or Signaled; Via L1 Control; Via Higher-layer. TDRA; Signaled; Yes. TBS (if supported); Signaled; Yes. Repetitions (if supported); Signaled; Yes. FDRA (if FDMA supported); Signaled; Yes. Device ID; Signaled as data. Reader ID; Signaled as data. Device Group ID (if supported); Signaled as data. Device Type (if supported); Signaled as data. Proposal 6: For signaling R2D control information for PDRCH, consider following table as a baseline: Information; Fixed or Signaled; Via L1 Control; Via Higher-layer. TDRA; Signaled; Yes. Coding; Signaled; Yes. TBS (if supported); Signaled; Yes. Repetitions (if supported); Signaled; Yes. FDRA (if FDMA supported); Signaled; Yes. Device ID; Signaled as data. Device Group ID (if supported); Signaled as data. Device Type (if supported); Signaled as data. |
| Sharp [29] | Proposal 2: R2D control information at least includes device ID(s) or device group ID(s), or additionally cast type. Proposal 3: R2D control information and higher layer payload are respectively mapped to two separate blocks of R2D chips (e.g. a first block of R2D chips for R2D control information immediately preceding a second block of R2D chips for higher layer payload). Proposal 4: R2D control information includes the following information about the block of R2D chips for higher layer data: R2D line coding code rate (e.g. 1 / number of R2D chips per bit), total number of R2D chips without repetition (or TBS), R2D repetition type, and number of R2D repetitions. Proposal 5: R2D control information includes the following information about the scheduled D2R transmission(s): D2R chip duration, D2R line coding code rate (e.g. 1 / number of D2R chips per bit), number of D2R chips without repetition (or TBS), D2R repetition type, number of D2R repetitions, and D2R frequency domain resource allocation (e.g. “large frequency-shift” for Device 2a, or RBs allocated for Device 2b). Proposal 6: For a R2D data transmission, the maximum TBS is 1000 bits. |
| Docomo [30] | Proposal 3: - Study following options for determination of chip duration of PRDCH. - Option1: chip duration of R2D data and chip duration of R2D control are determined from clock acquisition part. - Option2: chip duration of R2D control is determined from clock acquisition part, chip duration of R2D data is indicated in R2D control. - Option3: chip duration of R2D control is fixed, chip duration of R2D data is indicated in R2D control. Proposal 10: - Support Table I for whether a R2D control information is needed for receiving PRDCH with R2D data and whether a R2D control information is dynamic for each PRDCH or semi-static. - Table I: R2D control information for receiving PRDCH carrying R2D data. Control information; Needed or not; Dynamic or semi-static, if needed. TDRA (start in time domain); No; N/A. TDRA (duration in time domain); Up to 9.4.2.2; Dynamic. Chip duration; Yes; Dynamic. FDRA; Yes if FDMA is studied; Dynamic. Modulation scheme; No; N/A. Line coding scheme; Yes if a device can support both PIE and Manchester; FFS. Bit duration (bit to chip mapping); No; N/A. Device ID / group ID / cast type; Yes. A single field for device to determine whether it is target device.; Dynamic. Reader ID; No; N/A. TBS; No; N/A. Repetition; Yes if repetition is studied for R2D; FFS. CP handling related; Depends on progress on CP handling method; FFS. Proposal 11: - For dynamic control information, L1 R2D control can be used to schedule PRDCH with data and carry the information. - Study whether a semi-static control information for receiving PRDCH with R2D data is carried in L1 R2D control or higher layer. Proposal 12: - Support Table II for whether a R2D control information is needed for PDRCH transmission and whether a R2D control information is dynamic or semi-static. - Table II: R2D control information for PDRCH transmission Control information; Needed or not; Dynamic or semi-static, if needed. TDRA (start in time domain); Up to 9.4.2.2; Dynamic. TDRA (duration in time domain); Up to 9.4.2.2; Dynamic. FDRA; Yes; Dynamic. Chip duration; Yes; Dynamic. Modulation scheme; Yes if a device can support multiple of OOK/PSK/FSK; FFS. Coding scheme; Yes if a device can support multiple of Manchester/Miller/FM0/no line coding; FFS. Bit duration (bit to chip mapping); No for Manchester/FM0. FFS for Miller/no line coding.; Dynamic. Device ID; Yes; Dynamic. Reader ID; No (we don’t see the motivation); N/A. FEC related; Depends on progress on FEC; FFS. TBS; No; N/A. Repetition; Yes; FFS. CDMA related; If CDMA is studied; Dynamic. Proposal 13: - Study following options of scheduling of D2R transmission. - Option 1: L1 R2D control schedules PRDCH with R2D command. Another L1 R2D control schedules D2R. - Option 2: L1 R2D control schedules PRDCH with R2D command. Higher layer R2D information schedules D2R. The higher layer R2D information scheduling D2R is carried in same PRDCH as R2D command. - Option 3: L1 R2D control schedules PRDCH with R2D command. Same L1 R2D control schedules D2R. |
| Qualcomm [31] | Proposal 3: - The D2R transmission rate (e.g., chip length/duration or square-wave length/duration) is based at least on reader’s instruction, e.g., by R2D control. Proposal 6: RAN1 to study the R2D L1 control to support D2R L1 ACK/NACK response for R2D data detection. Proposal 7: RAN1 to study the R2D L1 control and R2D MAC-CE for scheduling D2R data. Proposal 8: The R2D control can be split into R2D L1 control and R2D MAC-CE to support both R2D data scheduling and D2R response or D2R data scheduling as Table 2. - FFS: How to split the R2D control information for D2R scheduling information Table 2 R2D control for R2D/D2R scheduling Common for R2D and D2R scheduling; R2D control info; Needed or not; L1 control; MAC-CE. Info type/message type; [X] bits; Preferred; No. Device ID/GID/type, cast type; Yes; Short ID or partial ID; Long ID or partial ID. Reader ID; Yes to differentiate the reader; Short ID or partial ID); Long ID or partial ID. For R2D scheduling information; TDRA; No if R2D data follows immediately R2D control; No; No. FDRA; No if same BW as R2D preamble; No; No. Modulation type/order; No if Manchester only; No; No. Coding scheme/rate; No since no FEC; No; No. Repetitions; Yes if configured; Yes; No. Chip length; OOK-M if different for R2D L1 control and R2D data; Yes; No. TBS/duration; Yes if no postamble; Yes; No. For D2R scheduling information; TDRA; Yes for TDM/FDM/CDM devices; FFS; FFS. FDRA; Yes for TDM/FDM/CDM devices; FFS; FFS. Modulation type/order; Yes if ASK, BPSK, or FSK; FFS; FFS. Coding scheme/rate; Yes for FEC coding rate; FFS; FFS. Repetitions; Yes, [common for all devices]; FFS; FFS. Chip length; Common or different for multiple devices?; FFS; FFS. TBS/duration; Predefined or configurable; FFS; FFS. D2R X-amble; Additional pilot or not, number of midambles; FFS; FFS. Proposal 13: For time/freq resource allocation of A-IoT communications - For Topology 1: BS configures R2D/D2R time/freq resources for A-IoT - BS/reader to control dynamic R2D/D2R within the configured time/freq resources. - For Topology 2: BS semi-statically or dynamically configures the time/freq resources per the UE/reader via Uu, at least for inband/guardband operation. - UE/reader to trigger R2D/D2R within the configured time/freq resources - FFS: how to solve collision among UEs/readers for a shared resource Proposal 14: Study whether/how to apply power control for A-IoT devices and UE/reader. - The power control for R2D transmission at UE/reader should be independent from that of legacy UL transmission. |
| Cewit [33] | Proposal 3: Supported following content of the scheduling information for D2R transmission: - Time domain scheduling - Frequency domain scheduling - MCS - TBS - Chip length - Device identity information - A-IoT device ID: only for single device indication. - A-IoT device group ID: for group of devices indication. Proposal 4: For R2D, study how to transmit the control information. |
| Google [34] | Proposal 6: A-IoT study assumes maximum TB size of A-IoT device is 1000 bits. Proposal 2: The number of repetition for PDRCH transmission is indicated in R2D L1 control information |
| IIT-K, IIT-M [35] | Observation 1: Ambient IoT can have a range of information payload sizes. Proposal 1: Ambient IoT devices should support the variable lengths of the payload. Proposal 2: To support the wide range of payload size, the following options should be considered: Option 1: A fixed size of a maximum of X bits of payload should be used. FFS: Values of X e.g. 1000. Option 2: Segmentation of a large message payload based on the threshold message bit length [e.g. 200 bits per segment] Proposal 3: Supported the following configuration parameters for PRDCH and PDRCH transmission as the control information on R2D: - Time domain scheduling - Frequency domain scheduling - MCS - TBS size - Device identity information |

[Closed] 1st Discussion Round

Maximum TB size

One of aspects that have been discussed is the maximum TB size to be considered for R2D transmissions. Based on the inputs, it seems that majority of the companies support a maximum TB size of 1000 bits for R2D transmissions. This is mainly based on the maximum TB size captures in TR 38.848. Based on this, following proposal is made.

(HP) Proposal 2.1.2-1

For R2D, maximum TB size of 1000 bits is considered

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | OK |  |
| TCL | Agree with this proposal. However, there is different maximum TBS for different device types based on memory and processing capability of device. |  |
| xiaomi |  | Maybe related to other WGs discussion. |
| FUTUREWEI | ok |  |
| Spreadtrum | Yes |  |
| Honor | OK |  |
| MTK |  | We are not sure whether we can simply say a max TBS up to 1000 bits can be supported by PHY before we have detailed analysis on some impact factors, e.g., the corresponding coverage evaluation, the device availability duration, (max) available resource, device buffer size, etc. For example, based on our simulation, a degradation of D2R BLER performance is observed for a TBS up to 1000bits under a specific residual SFO assumption if no additional enh (e.g., mid-amble insertion) is considered. Additionally, 10 seconds is needed for transmitting/receiving 1000 bits for a data rate of 0.1 kbps, which also calls for a check on the device availability duration in advance. In addition to max TB size, we think it is also necessary to have a consensus on the min TB size can be supported by PHY, which is also useful for identifying the min capability on the TB delivering from RAN1 perspective. |
| LG Electronics | OK |  |
| Ericsson | Fine. We can put it as a RAN1 assumption. Then, we can further refine this value, if needed, based on RAN2 input. |  |
| InterDigital | ok |  |
| Samsung | We are fine in principle, but the exact number may need further discussion. |  |
| Moderator (Apple) | Based on the discussion during the online sessions, it seems companies still want to study further on value for maximum TBS. Let’s postpone this proposal for now |  |

R2D Scheduling Information

Based on the inputs provided by the companies, the views for R2D scheduling information are summarized in the table below. Please provide inputs, if missing or incorrectly captured here. Based on these inputs, corresponding proposal(s) will be provided

Input  for R2D scheduling information

| R2D Scheduling Information | Indicated? | If indicated, then whether L1 R2D control information or higher-layer signaled (as R2D data in PRDCH) |
| --- | --- | --- |
| TDRA | Yes - Ericsson, Spreadtrum, TCL, China Telecom, Fujitsu, CATT, Continental, Honor, Samsung, Apple, [Docomo] | L1 R2D Control – Ericsson, Fujitsu, Continental, Apple |
|  |  | Higher Layer – |
|  | No – [Oppo], Qualcomm (no starting time) | N/A |
| FDRA | Yes - China Telecom, [Oppo], Continental, [Apple], [Docomo] | L1 R2D Control – [Oppo], Continental, [Docomo] |
|  |  | Higher Layer – [Apple] |
|  | No – Ericsson, TCL, , Honor, LGE | N/A |
| MCS/Coding Rate | Yes - Spreadtrum, Intel, China Telecom, [Oppo], ZTE, Samsung, Sharp, Mediatek | L1 R2D Control – ZTE |
|  |  | Higher Layer – [Oppo], ZTE |
|  | No – Ericsson, CATT, Honor, Apple, Docomo, Qualcomm | N/A |
| TBS | Yes - CMCC, Intel, China Telecom, CATT, Panasonic, Honor, LGE, Samsung, Mediatek, [Apple], Qualcomm | L1 R2D Control – LGE (Optional) |
|  |  | Higher Layer – |
|  | No – Ericsson, Oppo, Fujitsu, Docomo | N/A |
| Chip Duration | Yes - Ericsson, Spreadtrum, CMCC, Docomo, Qualcomm | L1 R2D Control – Docomo, Ericsson |
|  |  | Higher Layer – |
|  | No – | N/A |
| Device ID | Yes - Spreadtrum, Intel, China Telecom, Oppo, Fujitsu, ZTE, Interdigital, LGE, Samsung, Mediatek, Apple, Sharp, Docomo, Qualcomm, FUTUREWEI, Ericsson | L1 R2D Control – Oppo, Docomo, FUTUREWEI, LGE (optional for unicast) , Ericsson |
|  |  | Higher Layer – ZTE, Interdigital |
|  | No – | N/A |
| Reader ID | Yes - [Oppo], ZTE, LGE, [Apple], Qualcomm, [FUTUREWEI] | L1 R2D Control – [Oppo] , [FUTUREWEI] |
|  |  | Higher Layer – ZTE, LGE |
|  | No – Docomo, Ericsson | N/A |
| Repetitions | Yes - Spreadtrum, China Telecom, Oppo, ZTE, NEC, Honor, LGE, Samsung, Mediatek, [Apple], [Docomo], Qualcomm | L1 R2D Control – Oppo, ZTE, LGE (if supported) |
|  |  | Higher Layer – |
|  | No – Ericsson | N/A |
| ….. | FUTUREWEI (next decode for device availability) LGE (Command ID or message type) LGE (Device group ID) | FUTUREWEI (next decode for device availability) |
|  |  | LGE (Command ID or message type) LGE (Device group ID) |
| Device type/device group ID | Yes - ZTE | L1 R2D Control – ZTE |
|  | No – | Higher Layer – |
| Moderator (Apple) | Please further check your positions in the 2nd round. Also some new fields are added based on the inputs from companies |  |

D2R Scheduling Information

Based on the inputs provided by the companies, the views for D2R scheduling information are summarized in the table below. Please provide inputs, if missing or incorrectly captured here. Based on these inputs, corresponding proposal(s) will be provided

Input for D2R scheduling information

| D2R Scheduling Information | Indicated? | If indicated, then whether L1 R2D control information or higher-layer signaled (as R2D data in PRDCH) |
| --- | --- | --- |
| TDRA | Yes - Ericsson, Huawei, Spreadtrum, CMCC, Lenovo, China Telecom, Oppo, Xiaomi, Fujitsu, ZTE, Interdigital, Honor, LGE, Samsung, Apple, [Docomo], Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI | L1 R2D Control – Ericsson, Apple, LGE |
|  |  | Higher Layer – Ericsson, Huawei, ZTE, FUTUREWEI |
|  | No – | N/A |
| FDRA | Yes - Ericsson, Huawei, Spreadtrum, CMCC, Lenovo, China Telecom, Oppo, Xiaomi, ZTE, Interdigital, Honor, LGE, Samsung, Apple, Sharp, Docomo, Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI | L1 R2D Control – Ericsson, Apple, Docomo, LGE |
|  |  | Higher Layer – Ericsson, Huawei, CMCC, ZTE, FUTUREWEI |
|  | No – Ericsson, TCL, , Honor, LGE | N/A |
| MCS/Coding Rate | Yes - Ericsson, Huawei, Spreadtrum, CMCC, China Telecom, Oppo, ZTE, Interdigital, Honor, LGE, Samsung, Apple, Docomo, Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI | L1 R2D Control – Ericsson, Apple, LGE |
|  |  | Higher Layer – Ericsson, Huawei, CMCC, ZTE, FUTUREWEI |
|  | No – | N/A |
| TBS | Yes - Huawei, Spreadtrum, CMCC, China Telecom, Oppo, CATT, Interdigital, Honor, LGE, Samsung, [Apple], Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI | L1 R2D Control – LGE |
|  |  | Higher Layer – Huawei, CMCC, FUTUREWEI |
|  | No – Ericsson, Fujitsu, Docomo | N/A |
| Chip Duration | Yes - Ericsson, Huawei, Spreadtrum, ZTE, LGE, Apple, Sharp, Docomo, Qualcomm, FUTUREWEI | L1 R2D Control – Ericsson, Apple, Docomo, LGE |
|  |  | Higher Layer – Ericsson, Huawei, ZTE, FUTUREWEI |
|  | No – | N/A |
| Device ID | Yes - Huawei, Spreadtrum, CMCC, China Telecom, Oppo, Xiaomi, Fujitsu, CATT, Interdigital, LGE, Samsung, Apple, Docomo, Qualcomm, Cewit, IIT-K, IIT-M, Ericsson, ZTE | L1 R2D Control – Apple, Docomo, LGE (common with R2D scheduling info) , Ericsson |
|  |  | Higher Layer – Huawei, CMCC, Apple, Ericsson |
|  | No – | N/A |
| Reader ID | Yes - China Telecom, LGE, Qualcomm, ZTE | L1 R2D Control – |
|  |  | Higher Layer – LGE |
|  | No – Docomo, Ericsson | N/A |
| Repetitions | Yes - Ericsson, Huawei, Spreadtrum, CMCC, China Telecom, Oppo, Xiaomi, CATT, Interdigital, Honor, LGE, Samsung, Apple, Docomo, Google, Qualcomm, FUTUREWEI, ZTE | L1 R2D Control – Ericsson, Apple, LGE |
|  |  | Higher Layer – Ericsson, Huawei, CMCC, FUTUREWEI |
|  | No – | N/A |
| ….. | FUTUREWEI (next decode for device availability) | FUTUREWEI (next decode for device availability) |
| Moderator (Apple) | Based on the discussion during the online sessions, it seems companies still want to study further on value for maximum TBS. Let’s postpone this proposal for now |  |

[Open] 2nd Discussion Round

Input  for R2D scheduling information

| R2D Scheduling Information | Indicated? | If indicated, then whether L1 R2D control information or higher-layer signaled (as R2D data in PRDCH) |
| --- | --- | --- |
| TDRA | Yes - Ericsson, Spreadtrum, TCL, China Telecom, Fujitsu, CATT, Continental, Honor, Samsung, Apple, [Docomo], OPPO (if TDM supported) | L1 R2D Control – Ericsson, Fujitsu, Continental, Apple, OPPO |
|  |  | Higher Layer – |
|  | No – Qualcomm (no starting time), Huawei, HiSilicon | N/A |
| FDRA | Yes - China Telecom, Oppo, Continental, [Apple], [Docomo] | L1 R2D Control – Oppo, Continental, [Docomo] |
|  |  | Higher Layer – [Apple] |
|  | No – Ericsson, TCL, , Honor, LGE, Huawei, HiSilicon | N/A |
| MCS/Coding Rate | Yes - Spreadtrum, Intel, China Telecom, ZTE, Samsung, Sharp, Mediatek, Lenovo | L1 R2D Control – ZTE |
|  |  | Higher Layer – [Oppo] |
|  | No – Ericsson, CATT, Honor, Apple, Docomo, Qualcomm, Huawei, HiSilicon | N/A |
| TBS | Yes - CMCC, Intel, China Telecom, CATT, Panasonic, Honor, LGE, Samsung, Mediatek, [Apple], Qualcomm, Lenovo, OPPO | L1 R2D Control – LGE (Optional), OPPO |
|  |  | Higher Layer – |
|  | No – Ericsson, Oppo, Fujitsu, Docomo, Huawei, HiSilicon | N/A |
| Chip Duration | Yes - Ericsson, Spreadtrum, CMCC, Docomo, Qualcomm, Lenovo | L1 R2D Control – Docomo, Ericsson |
|  |  | Higher Layer – |
|  | No – Huawei, HiSilicon | N/A |
| Device ID | Yes - Spreadtrum, Intel, China Telecom, Oppo, Fujitsu, ZTE, Interdigital, LGE, Samsung, Mediatek, Apple, Sharp, Docomo, Qualcomm, FUTUREWEI, Ericsson, Lenovo | L1 R2D Control – Oppo, Docomo, FUTUREWEI, LGE (optional for unicast) , Ericsson |
|  |  | Higher Layer – ZTE, Interdigital |
|  | No – Huawei, HiSilicon | N/A |
| Reader ID | Yes - [Oppo], ZTE, LGE, [Apple], Qualcomm, [FUTUREWEI], Lenovo | L1 R2D Control – Oppo , [FUTUREWEI] |
|  |  | Higher Layer – ZTE, LGE |
|  | No – Docomo, Ericsson, Huawei, HiSilicon | N/A |
| Repetitions | Yes - Spreadtrum, China Telecom, Oppo, ZTE, NEC, Honor, LGE, Samsung, Mediatek, [Apple], [Docomo], Qualcomm, Lenovo | L1 R2D Control – Oppo, ZTE, LGE (if supported) |
|  |  | Higher Layer – |
|  | No – Ericsson, Huawei, HiSilicon | N/A |
| Device type/device group ID | Yes - LGE, ZTE | L1 R2D Control – ZTE, LGE (optional for groupcast) |
|  | No – Huawei, HiSilicon | Higher Layer – |
| Device Availability | Yes - Futurewei | L1 R2D Control – |
|  | No – Huawei, HiSilicon | Higher Layer – |
| Command/Message Type | Yes - LGE | L1 R2D Control – LGE (e.g. for Queary/Paging) |
|  | No – Huawei, HiSilicon | Higher Layer – |

Input for D2R scheduling information

| D2R Scheduling Information | Indicated? | If indicated, then whether L1 R2D control information or higher-layer signaled (as R2D data in PRDCH) |
| --- | --- | --- |
| TDRA | Yes - Ericsson, Spreadtrum, CMCC, Lenovo, China Telecom, Oppo, Xiaomi, Fujitsu, ZTE, Interdigital, Honor, LGE, Samsung, Apple, [Docomo], Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI | L1 R2D Control – Ericsson, Apple, LGE |
|  |  | Higher Layer – Ericsson, ZTE, FUTUREWEI |
|  | No – | N/A |
| FDRA | Yes - Ericsson, Huawei, HiSilicon, Spreadtrum, CMCC, Lenovo, China Telecom, Oppo, Xiaomi, ZTE, Interdigital, Honor, LGE, Samsung, Apple, Sharp, Docomo, Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI | L1 R2D Control – Ericsson, Apple, Docomo, LGE |
|  |  | Higher Layer – Ericsson, Huawei, HiSilicon, CMCC, ZTE, FUTUREWEI |
|  | No – Ericsson, TCL, , Honor, LGE | N/A |
| CDM | Yes - ZTE | L1 R2D Control – |
|  |  | Higher Layer – |
| MCS/Coding Rate | Yes - Ericsson, Huawei, HiSilicon, Spreadtrum, CMCC, China Telecom, Oppo, ZTE, Interdigital, Honor, LGE, Samsung, Apple, Docomo, Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI, Lenovo | L1 R2D Control – Ericsson, Apple, LGE |
|  |  | Higher Layer – Ericsson, Huawei, HiSilicon, CMCC, ZTE, FUTUREWEI |
|  | No – | N/A |
| TBS | Yes - Huawei, HiSilicon, Spreadtrum, CMCC, China Telecom, Oppo, CATT, Interdigital, Honor, LGE, Samsung, [Apple], Qualcomm, Cewit, IIT-K, IIT-M, FUTUREWEI, Lenovo | L1 R2D Control – LGE |
|  |  | Higher Layer – Huawei, HiSilicon, CMCC, FUTUREWEI |
|  | No – Ericsson, Fujitsu, Docomo | N/A |
| Chip Duration | Yes - Ericsson, Huawei, HiSilicon, Spreadtrum, ZTE, LGE, Apple, Sharp, Docomo, Qualcomm, FUTUREWEI, Lenovo | L1 R2D Control – Ericsson, Apple, Docomo, LGE |
|  |  | Higher Layer – Ericsson, Huawei, HiSilicon, ZTE, FUTUREWEI |
|  | No – | N/A |
| Device ID | Yes - Huawei, HiSilicon, Spreadtrum, CMCC, China Telecom, Oppo, Xiaomi, Fujitsu, CATT, Interdigital, LGE, Samsung, Apple, Docomo, Qualcomm, Cewit, IIT-K, IIT-M, Ericsson, ZTE, Lenovo | L1 R2D Control – Apple, Docomo, LGE (common with R2D scheduling info) , Ericsson |
|  |  | Higher Layer – Huawei, HiSilicon, CMCC, Apple, Ericsson |
|  | No – | N/A |
| Reader ID | Yes - China Telecom, LGE, Qualcomm, ZTE | L1 R2D Control – |
|  |  | Higher Layer – LGE |
|  | No – Docomo, Ericsson, Huawei, HiSilicon | N/A |
| Repetitions | Yes - Ericsson, Huawei, HiSilicon, Spreadtrum, CMCC, China Telecom, Oppo, Xiaomi, CATT, Interdigital, Honor, LGE, Samsung, Apple, Docomo, Google, Qualcomm, FUTUREWEI, ZTE, Lenovo | L1 R2D Control – Ericsson, Apple, LGE |
|  |  | Higher Layer – Ericsson, Huawei, HiSilicon, CMCC, FUTUREWEI |
|  | No – | N/A |
| Device Availability | Yes - FUTUREWEI | L1 R2D Control – |
|  |  | Higher Layer – |
|  | No – Huawei, HiSilicon | N/A |

[Open] 3rd Discussion Round

(HP) Proposal 2.1.1-1A

For D2R scheduling, at least following information is indicated to the device via R2D:

- Time domain resources

- MCS/coding rate

- Chip duration

- Device ID

- Repetitions

FFS: For each information, whether higher-layer signaling or L1 R2D control signaling is used

(HP) Proposal 2.1.1-2A

For R2D scheduling, at least following information is indicated to the device via R2D:

- Device ID

FFS: For each information, whether higher-layer signaling or L1 R2D control signaling is used

### PRDCH design details

| Company | Observations/Proposals |
| --- | --- |
| Futurewei [1] | Proposal 4: For the format of the PRDCH, if present, any L1 R2D control information is placed at the start of the PRDCH. Proposal 5: For the format of the PRDCH, do not support a CRC solely for L1 R2D control information. Observation 1. For the format of the PRDCH, L1 R2D control information covered by the PRDCH CRC has several performance benefits. Proposal 6: For the format of the PRDCH, L1 R2D control information covered by the PRDCH CRC is preferred. |
| Nokia [2] | - Observation 8: The broadcast information containing configuration for channel access, e.g., initial contention-based access, may be beneficial, and may precede any device-specific configuration information. - Proposal 5: RAN1 to study what broadcast information is required for the AIoT device and to identify how it should be included in the PRDCH channel. - Observation 9: The need for having a separate control channel depends on the type of information it must carry and the size of the message. If the content of the control information is small, perhaps a separate control channel may not be efficient. However, if different device types support different set of configurable parameters, it may be worth to study the possibility of having a separate control channel in addition to the data channel. - Observation 10: The PRDCH may carry a wake-up info to select a subset of devices for the inventory process in addition to the modulation and resource allocation fields targeted towards a specific device type. - Proposal 6: RAN1 to study the necessary control information to be contained in PRDCH depending on the AIoT device type, including activation and query commands. - Proposal 7: RAN1 should consider a flexible design, i.e., by having control only, data only, or control and data fields all carried by PRDCH itself. |
| Ericsson [3] | - Proposal 4: For PRDCH generation, use separate CRC attachment for L1 R2D control and R2D data signals. - Proposal 5: The L1 R2D control/system information can be transmitted using unicast, multicast and broadcast. |
| Huawei [4] | Proposal 2: A single CRC for PRDCH transmissions is supported, where the D2R grant is transmitted using a higher layer control element on the data part of the PRDCH. |
| Spreadtrum [5] | Proposal 3: Separate CRCs are supported for control information and data part in PRDCH. |
| Intel [8] | Proposal 4: - Physical structure of PRDCH may consist of L1 control information and optionally data packet. - Separate CRCs are appended for L1 control information and data packet, respectively. |
| Lenovo [9] | Proposal 1: Study burst-type channel structure for R2D transmission for ambient IoT. Proposal 4: Study a R2D control part immediately following the preamble of the R2D transmission and having separate encoding and CRC from the R2D data part. Proposal 5: Study scrambling for R2D data part. Proposal 7: Study data part of a PRDCH carrying scheduling information for the corresponding D2R transmission. |
| Vivo [12] | Proposal 1: Prioritize the discussion on ambles structure, and deprioritize the discussion on scheduling information. Observation 12: Necessity, feasibility and benefits of supporting a maximum TBS of 1000bits are unclear. Observation 13: For a PRDCH/PDRCH with low data rate and large payload size (e.g., a maximum TBS of 1000bits), following disadvantages are observed: - Excessive long time occupation of channel resources. - The AIoT device may be not able to finish transmission/reception if the duration for the PDRCH/PRDCH exceeds the sustainable time of AIoT device due to limited energy storage. Observation 14: Even if RAN1 would agree to use maximum message size as maximum TBS from specification perspective, it should be up to reader to determine the TB size for PRDCH and PDRCH. It does not necessarily mean a message cannot be delivered in multiple transmissions with multiple TBs with TB size less than the max TB size. Proposal 15: Inform RAN2 that from RAN1 perspective, the TBS for a PRDCH and PDRCH transmission can be smaller than the message size and should be determined by the reader. |
| Oppo [13] | Observation 2: To support the target use cases, such as indoor command, it is necessary to implement different reliability for R2D transmissions. Proposal 2: Repetition or simple FEC should be supported for R2D transmission. Proposal 4: Physical layer control information should be used to indicate R2D data transmission, and the control information is separately encoded and added with separate CRC. Proposal 5: The CRC of control information for indicating PRDCH should be scrambled by reader ID or device/group ID. Proposal 8: Further study whether transmission schemes for DL control channel and DL data channel are always the same or can be different. Proposal 10: Scrambling for PRDCH should be supported. |
| Xiaomi [14] | Proposal 2: For PRDCH, there may be three formats can be studied: - Format 0: L1 control information only; - Format 1: R2D data only; - Format 2: L1 control information+ R2D data. Proposal 3: The first two bits of PRDCH transmission can be used for the PRDCH format indication. |
| Fujitsu [15] | Proposal 5: If L1 control information is transmitted on PRDCH, two options can be considered for control information transmission: - Alt.1: L1 control information and R2D data are carried by separate PRDCHs. - Alt.2: One PRDCH carries both L1 control information and R2D data. |
| CATT [16] | Proposal 17: For ambient IoT devices, the PRDCH should be introduced as a dedicated A-IoT physical channel for R2D transmission, in which at least R2D control information, R2D data transmission, and CRC are embedded. Proposal 18: Single CRC should be applied to some of R2D control information and R2D data. (a) (b) Figure 7: Updated PRDCH generation with scrambling Proposal 19: A scrambling block should be added after the CRC block or before the CRC block for PRDCH generation. - A scrambling block should be added after the CRC block and before the line coding block if the scrambling functionality is provided in physical layer. - A scrambling block should be added before the CRC block to have the scrambling functionality mapping to the security mechanism provided by SA3. Proposal 20: For PRDCH, transport block size is no more than 128 bytes according to the message sizes requirements for SA1 applicable use cases of indoor inventory and indoor command, the higher layer control information and the NAS control information carried by PRDCH. - The candidate transport block sizes for PRDCH are {128, 256, 512, 1024} bits. |
| ZTE [17] | - Observation 5: In RFID, the length of downlink command can reach to thousands of bits, where data segmentation is not used. - Observation 6: The benefit of downlink data segmentation should be studied. |
| Panasonic [18] | Proposal 4: The CRC design of R2D should be discussed after the content of control information is determined. |
| Interdigital [19] | Observation 2: Including CRC for small payload size will result in high overhead without clear benefit. Proposal 5: Support multiplexing R2D control information and R2D data on the same PRDCH. Proposal 6: R2D control information is transmitted at the beginning of PRDCH. Proposal 7: Support single CRC attachment to L1 R2D control information and R2D data when control and data are transmitted in the same PRDCH. |
| LGE [24] | Proposal 8: Consider zero-size higher-layer payload with zero-size CRC for PRDCH e.g. if we need to support transmission of L1 R2D control information without higher-layer payload. In this case, TBS in R2D L1 control information can indicate zero size. That does not mean different PRDCH formats. |
| Samsung [25] | Proposal 13: Study a PRDCH providing a signaling for a group of devices to improve the overall A-IoT system efficiency. Proposal 14: Study the feasibility and the potential gain of frequency hopping for PRDCH/PDRCH transmission. |
| Mediatek [27] | Proposal 2: It should be enabled that the decoding of R2D control is separately from the decoding of R2D data. Proposal 3: Study different PRDCH formats for delivering R2D control and R2D data. |
| Apple [28] | Proposal 7: For PRDCH, study at least following transmission schemes for mapping different types of R2D information: - PRDCH-TM0: Only R2D data is mapped - It is unicast type mode - PRDCH-TM1: Only system information is mapped - It is broadcast type mode - PRDCH-TM2: Only R2D L1 control information is mapped - It can either be broadcast type, groupcast type or unicast mode - PRDCH-TM3: R2D L1 control information and R2D data is mapped - It can be unicast mode - L1 control information part precedes the R2D data part within the PRDCH Proposal 8: For PDRCH generation involving both R2D control information and R2D data information, separate CRC attachment is studied: |
| Docomo [30] | Proposal 14: - Study following structure of R2D transmission. - R2D preamble immediately precedes L1 R2D control, L1 R2D control precedes R2D data. - Study whether a gap is needed between L1 R2D control and R2D data. Proposal 15: - For CRC of PRDCH, study following options. - Option 1: CRC is separately generated for R2D data and L1 R2D control. - Option 2: CRC is generated only for R2D data. - Option 3: CRC is jointly generated for R2D data and L1 R2D control. Proposal 16: - Study whether device behavior can be different for receiving L1 R2D control and receiving R2D data from at least following aspects. - Determination of duration/end in time domain. - Determination of chip duration. - Repetition. - CRC Proposal 17: - Study the max TBS in one PRDCH considering at least the max time duration of one PRDCH, coding scheme and chip duration. |
| Qualcomm [31] | Proposal 4: For R2D data mapping to PRDCH, the processing structure includes repetition and scrambling between CRC attachment and Line code block. PRDCH generation Proposal 9: For R2D control mapping to a physical channel, RAN1 to consider - R2D L1 control is carried in PRDCH, separately from that of R2D data - E.g., different PRDCH formats for R2D data and control - R2D control in MAC-CE is carried in PRDCH, same as that of R2D data if transmitted |
| Google [34] | Proposal 1: For A-IoT study, TB level repetition performed for PDRCH transmission is considered. Proposal 2: The number of repetition for PDRCH transmission is indicated in R2D L1 control information. Proposal 3: Support a PRDCH can be used to transmit R2D data or to transmit both R2D data and R2D L1 control (if R2D L1 control is identified as needed). Proposal 4: Study how an A-IoT device understands whether the data conveyed in a PRDCH is cell-specific or device-specific. |
| IIT-K, IIT-M [35] | Proposal 4: The following PRDCH formats should be studied further for mapping control information and payload on R2D transmission: - PRDCH TF-1: When PRDCH only carries the R2D control information - PRDCH TF-2: When PRDCH only carries the D2R control information - PRDCH TF-3: When PRDCH only carries the payload. - PRDCH TF-4: When PRDCH only carries the R2D control information and the payload. - PRDCH TF-5: When PRDCH only carries the D2R control information and the payload. FFS: The detailed structure of the formats. |
| AsusTek [36] | Proposal 2: For PRDCH generation, if L1 R2D control information (if defined) and R2D data are carried together in single PRDCH, separate CRC attachment to L1 R2D control information and R2D data. |

[Closed] 1st Discussion Round

~24 companies provided their views on PRDCH design. Multiple aspects have been discussed in the contributions. First aspect is related to multiplexing of control and data on the same PRDCH. It is mainly proposed that when control and data are transmitted on the same PRDCH, then the control information is added at the start, followed by data. Main benefit of this is to provide any relevant control information can carries scheduling information for the corresponding data part. Second aspect is related to the support of multiple formats depending on what information is transmitted on the same PRDCH. Basically, for the device to identify, whether the PRDCH carriers only data or data & control, etc., different formats are considered. Third aspect is related to encoding of control and data on the same PRDCH. Mainly two options are considered by companies including a joint/single CRC attachment to data & control or a separate CRC attachment to each data and control. In principle, the choice between a single CRC or separate CRCs involves a trade-off between simplicity and overhead (in the case of a single CRC). Fourth aspect is related to additional blocks including scrambling and repetition. Multiple companies proposed to include scrambling and repetition block for increased reliability/robustness for R2D transmission. However, whether scrambling and/or repetition for R2D transmissions is studied or not is considered under agenda 9.4.2.1. Based on further progress in agenda 9.4.2.1, if additional blocks are agreed, then the PRDCH generation block diagram can be updated. Based on these discussions, following proposals related to PRDCH design are made.

(HP) Proposal 2.1.3-1

For PRDCH generation, study following two alternatives for CRC attachment when both L1 R2D control information (if any) and data are transmitted on the same PRDCH

- Alt.1: Single/joint CRC attachment to L1 R2D control information (if any) and data

- Alt.2: Separate CRC attachment to each of L1 R2D control information (if any) and data

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | Maybe we should clarify that the L1 R2D control information is for PRDCH indication rather than PDRCH scheduling, as the 2 kinds of control information are supposed to be multiplexed with data differently. For L1 R2D control information for PRDCH indication, we support Alt. 2, as PRDCH may be very long in time, in case of unicast or groupcast, it is important for a device to decide whether the PRDCH needs to be received or not based on the control information, such as to save limited energy- of device. |  |
| TCL | We prefer Alt.1 for device 1 and 2a because of the limited detection capability. For device 2b, we prefer Alt.2. |  |
| xiaomi | Maybe it needs to be firstly confirmed that the control information is carried by a PHY header; |  |
| FUTUREWEI | OK. Our preference is Alt. 1 |  |
| Qualcomm |  | Some clarification is needed. In case of Alt1, what’s the difference between L1 control and control in MAC-CE? |
| Spreadtrum | Support this proposal. We prefer Alt2. For example, if the L1 control information carries the group/device ID, the device can determine whether to continue receiving the data part based on the ID information, which can achieve power saving of device. |  |
| Honor | After the L1 R2D control is justified necessary and different from control in MAC-CE, we prefer Alt.2. |  |
| LG Electronics | OK |  |
| Lenovo | OK with the proposal. |  |
| Ericsson | Support and prefer Alt2. |  |
| InterDigital | Ok. Prefer Alt. 1 |  |
| Samsung | Support the proposal with the note that we prefer Alt 2. Alt 2 allows a device to first decode control part to identify whether the transmission is addressed to the device or not so that it can early determine to decode the rest of the data part or not, so that it can save energy and less memory buffer size requirement at the device. |  |
| ZTE, Sanechips | General OK with the proposal. In our opinion, the payload size of L1 control information is small(e.g., less than 14bits), and CRC attachment for L1 control information is not needed. However, further clarification is required for Alt 2 to determine that it encompasses a scheme where L1 control information is transmitted without a CRC. The following modification is proposed: ##### (HP) Proposal 2.1.3-1-rev For PRDCH generation, study following two alternatives for CRC attachment when both L1 R2D control information (if any) and data are transmitted on the same PRDCH - Alt.1: Single/joint CRC attachment procedure to L1 R2D control information (if any) and data - Alt.2: Separate CRC attachment procedure to each of L1 R2D control information (if any) and data - Alt.2a: CRC attachment to data, and no CRC attachment to L1 R2D control information(if any). - Alt 2b. one CRC attachment to L1 R2D control information (if any), and another CRC attachment to data. |  |
| Moderator (Apple) | Seems like companies are generally fine with the proposal. Intention right now is not to down-select but list the options. Based on ZTE’s suggestion, update proposal is provided for 2nd round @Oppo: I am not sure if it is relevant for this discussion whether the R2D L1 control information is for PRDCH or PDRCH @Qualcomm: MAC control should be data from PHY perspective |  |
| Samsung2 | Support ZTE’s modified proposal. |  |

Proposal 2.1.3-2

For PRDCH transmitting both L1 R2D control information and data, any L1 R2D control information is placed at the start of the PRDCH.

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | okay |  |
| xiaomi | OK |  |
| FUTUREWEI | Ok |  |
| OPPO | OK |  |
| Spreadtrum | OK |  |
| Honor | OK |  |
| LG Electronics | OK |  |
| Ericsson | Support |  |
| InterDigital | OK |  |
| Moderator (Apple) | Seems like companies are fine. Based on Huawei’s comment during online sessions, I would like to clarify that this is conditional based on whether there is any L1 R2D control information or not. If there is any L1 R2D control information, then this is needed. Proposal for 2nd round updated to clarify the above |  |

Proposal 2.1.3-3

For R2D transmissions, study the need and feasibility to support different PRDCH formats

- Different PRDCH formats may correspond to whether that PRDCH transmits data only, data and control, etc.

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | Agree with this proposal. |  |
| xiaomi | Agree |  |
| FUTUREWEI |  | For device complexity, we want just one format. “For R2D transmissions, one PRDCH format, with L1 R2D control information and data, is supported” |
| OPPO |  | There are 3 possible cases: 1) Dada only, 2) control only, and 3) control and data. Seems there is no need to differentiate 2) and 3) with different formats, they can be transmitted with same format and use control information to indicate whether there is data or not. For 1), we are not sure it may exist. But if it would be there, different formats are necessary. In a word, we suggest to discuss whether there is 1) firstly. |
| Spreadtrum | OK |  |
| Honor | Agree |  |
| LG Electronics |  | Format indicator seems not needed. We think that L1 control information can be always added and device can determine whether subsequent data size is zero or not based on L1 control information for PRDCH. |
| Ericsson | Support |  |
| ZTE, Sanechips |  | We don’t think different PRDCH formats is needed now. It should be discussed after the control information is determined. |
| Moderator (Apple) | Based on the comments so far, it seems we can postpone this discussion until we have some further progress on the R2D control information part. |  |

[Open] 2nd Discussion Round

(HP) Proposal 2.1.3-1A

For PRDCH generation, study following two alternatives for CRC attachment procedure when both L1 R2D control information (if any) and data are transmitted on the same PRDCH

- Alt.1: Single/joint CRC attachment to L1 R2D control information (if any) and data

- Alt.2: Separate CRC attachment to each of L1 R2D control information (if any) and data

  - Alt.2a: CRC attachment to data, and no CRC attachment to L1 R2D control information(if any).

  - Alt 2b. one CRC attachment to L1 R2D control information (if any), and another CRC attachment to data.

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| TCL | Support |  |
| LG Electronics | OK |  |
| Huawei, HiSilicon |  | The crux of this proposal and previous proposal 2.1.1-2A is whether we have L1 R2D control information or if it is transmitted using higher layer signalling. We need to resolve this first before we can finalize such details. We do not see the need for separate CRCs at all, because the motivation for this is unclear for the following reasons. - Since a D2R transmission is always preceded by an R2D transmission containing the D2R grant, if the control is received successfully by the device, but not the data, the device has nothing to transmit at all. Since it did not receive the R2D data, which could be an inventory query or a command, the device does not have any response to transmit, hence rendering the successfully received D2R grant useless. - The inclusion of the CRC bits for the D2R grant may break the pipeline function in the device’s receive processing due to the added latency. The 1st CRC for the D2R grant needs to have completed its processing before the 2nd CRC for the remaining PRDCH can be processed, putting a strain on the device complete the process without additional latency. |
| Lenovo | Yes |  |
| OPPO | As commented in the previous round, we suggest to clarify that the L1 control information is for R2D scheduling. |  |
| Spreadtrum | Yes |  |
| Samsung2 | Yes |  |

(HP) Proposal 2.1.3-2A

For PRDCH transmitting both L1 R2D control information (if any) and data, any L1 R2D control information (if any) is placed at the start of the PRDCH.

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| TCL | Support |  |
| LG Electronics | OK |  |
| Huawei, HiSilicon |  | Again, here we are going ahead in deciding details of the L1 R2D control information without even agreeing to whether we support it. We need to resolve this before we discuss such proposals. |
| Lenovo | Yes |  |
| OPPO | As above, we suggest to clarify the L1 R2D control is for R2D scheduling. The R2D control information for D2R scheduling should be discussed separately. |  |
| Spreadtrum | Yes |  |
| Samsung2 | For sure. |  |

  - Topic 2: D2R channels/signals design

### D2R signals including preamble/midamble/postamble

| Company | Observations/Proposals |
| --- | --- |
| Nokia [2] | - Observation 12: To determine the start of the uplink frame and to detect and correct the sampling offset introduced by the device transmission, the need for preamble is inevitable in the PDRCH transmission. - Proposal 9: RAN1 to study in AI 9.4.2.2 the structure of the D2R preamble and whether the preamble needs to be split between a delimiter part and a synchronization part. - Observation 13: Depending on the payload sizes, i.e., the size of PDRCH transmission, midamble may be needed to compensate the sampling offset when the payload size is large. However, it should be configurable per device as it depends on the observed sampling offset created by the device transmission. Furthermore, it is unclear whether a D2R midamble is different from a D2R reference signal. - Proposal 10: RAN1 to assess in AI 9.4.2.2 whether both D2R midamble and D2R postamble are needed depending on the AIoT device type. - Observation 14: The D2R transmission may end with a set of samples indicating the termination of the frame. - Proposal 11: Study in AI 9.4.2.2 the design of the D2R postamble which may contain a known stop sequence to indicate the end of the frame. - Observation 17: As the reference signal is not considered for D2R link, the signal like preamble/postamble/midamble should be designed to ensure reliable detection at the reader. - Proposal 15: RAN1 to study the design of D2R preamble/midamble/postamble transmission to utilize for proximity determination and or interference detection by the reader. |
| Ericsson [3] | - Observation 12: D2R preamble length should be designed to strike a balance between detection/estimation performance and overheads due to signaling and transmit power consumption. - Observation 13: The sequence design for D2R preamble will also depend on whether multiple access needs to be supported by leveraging a pool of preamble sequences. - Proposal 15: RAN1 to clarify if D2R preamble needs to support multiple access by designing a set of preamble sequences. - Observation 14: If the D2R preamble is generated by the CW node, both binary and complex sequences can be considered for D2R preamble design. - Observation 15: If the D2R preamble is generated by the A-IoT device, binary sequences can be considered for D2R preamble design. - Proposal 16: RAN1 to decide if D2R preamble is generated by the A-IoT device or the reader/CWT. |
| Huawei [4] | Observation 18: Good correlation properties for the D2R physical signals such as time acquisition signal in preamble, midamble and postamble can provide reasonable timing information for the Reader and allow the Reader perform channel/interference estimation. Proposal 17: For D2R preamble signals, sampling frequency training signal can be considered for coarse SFO estimation. Proposal 18: For D2R preamble signals, sub-chip level timing acquisition should be supported. Proposal 19: For D2R preamble signals, line coding should also be applied. Proposal 20: For D2R preamble, the performance metric should include coarse SFO estimation performance, MDR, and timing error performance. Observation 19: For D2R sampling frequency training part, 64-chip “ON-OFF” signal can achieve 1% SFO estimation error with 98% probability under -2.5 SNR. Observation 20: For D2R time acquisition part, 32-length binary sequence encoded to 64-chip Manchester code can achieve the MDR 0.1%, and residual timing error within [-1/8, 1/8] chip with 99.05% probability under -2.5 SNR. Proposal 21: The D2R preamble includes a sampling frequency training part and a time acquisition part, where the sampling frequency training signal is used for coarse SFO estimation and a time acquisition is used for fine timing acquisition with a binary sequence signals with good correlation properties such as Golay sequences. Observation 21: Edge detection method is sensitive to the noise power and can lead to false edge detection even under SNR 5 dB. Proposal 22: D2R midamble signals reuse the timing acquisition sequence in the preamble. Proposal 23: For D2R postamble, a binary signal with good correlation properties such as Golay sequences and orthogonal to time acquisition signal in preamble should be supported. Proposal 24: For D2R postamble, the performance metric should include MDR and fine SFO estimation performance. Observation 22: For D2R postamble, 32-length binary sequence encoded to 64-chip Manchester code can achieve the MDR 0.1%, and residual SFO less than 0.06% with 99% probability under -2.5 SNR. |
| Spreadtrum [5] | Proposal 6: The D2R preamble design can take the binary sequence-based signal with a fixed pattern as a starting point for discussion. Proposal 7: The D2R midamble should be studied and the sequence can be the same as preamble. Proposal 8: Postamble is supported for D2R transmission, and the sequence is different from preamble and PDRCH. |
| TCL [6] | Observation 11: If good correlation needs to be considered at reader to acquire timing information for D2R preamble, sequence design is needed such binary sequence signals. Observation 12: If TBS is small for D2R transmission, midamble should not be supported. To meet the requirement of channel estimation and interference estimation, midamble may be as reference signal to achieve it if TBS can support midamble design. Proposal 15: For D2R preamble -- The D2R preamble format and sequence should be carefully considered based on simple modulation and sequences, e.g., OOK/BPSK/binary sequences. -- If D2R preamble is supported and generated by external for device 1 and 2a, then fully backscatter is needed during D2R preamble transmission. Proposal 16: For D2R midamble -- D2R midamble for the purpose of SFO tracking, whether support D2R midamble should depend on the D2R message size and the number of repetitions of D2R. -- D2R midamble for the other purpose, e.g., channel estimation or interference estimation is not considered. Proposal 17: For D2R postamble --Limited by device’s capability and TBS, D2R postamble should be at least deprioritied for device 1 and 2a for the indicator of end of PDRCH and other functionality. --The end-indicator of PDRCH may be indicated by R2D control information. Proposal 21: Consider possible reference signal patterns by backscatter modulation, including channel estimation pattern, interference estimation pattern, proximity determination pattern, etc. |
| CMCC [7] | Proposal 10: For D2R, a postamble at the end of PDRCH transmission is considered for timing tracking and fine SFO estimation. Observation 6: In case of low SINR, reader is not feasible to perform rising/falling edge detection even line coding is applied. More complicated operations will be processed at gNB or UE side, which requires precise channel estimation performance. Observation 7: Considering 3 km/h device velocity, the channel coherent time is in the level of hundreds of milliseconds. In case of large D2R packet size or low data rate, the channel estimation performance using only preamble and postamble may not be sufficient for decoding. Proposal 11: For D2R, midamble can be considered in the middle of the D2R transmission at least for channel estimation in case of large packet size or low data rate. Proposal 12: For D2R transmissions in Ambient IoT, preamble/midamble/postamble of different devices can be transmitted in a TDMed way to improve link performance in case of FDM(A). Proposal 13: For D2R, the sequence length of preamble and postamble should be long enough to ensure precise timing synchronization performance. Proposal 14: For D2R, binary sequence-based preamble/midamble/postamble design can be studied considering at least the following criteria: - Low peak sidelobe level (PSL); - High merit factor (MF); - Sequence pairs or sequence sets with large zero cross correlation zone (ZCCZ). |
| Lenovo [9] | Proposal 12: Study a D2R control part immediately following the preamble of the D2R transmission and having separate encoding and CRC from the D2R data part. Proposal 14: Study a postamble at the end of a D2R transmission for indicating the end of the PDRCH transmission for contention-based transmission. |
| China Telecom [10] | Proposal 7: It can reuse the legacy NR sequence or select some fixed sequence for the potential preamble sequence in D2R transmission. - Pre-configuration signalling and device complexity should be considered if reusing the legacy NR sequence. - Evaluation performance on the fixed sequence if any. |
| Vivo [12] | Proposal 10: A unique sequence can be considered for preamble sequence with a certain length for D2R transmission. - Sequence pool with multiple sequences with required correlation properties is not needed. Proposal 11: Different preamble length can be considered for different D2R response time. - If D2R response occurs immediately after the PRDCH, e.g., within a few tens of us, a short preamble can be considered. - If D2R response occurs within a long duration after the PRDCH, e.g., tens of ms, a long preamble can be considered. Observation 8: Other than D2R timing acquisition functionality, no additional functionalities of the preamble are necessary for D2R transmissions from sequence design perspective, although it can be up to network implementation to decide whether preamble can be used for channel estimation. Observation 9: Coherent detection has benefit in noise reduction as it retains the desired signal under accurate channel estimation. While sufficient number of midambles/postamble is necessary especially for D2R with long transmission. Proposal 12: For comparison between non-coherent detection and coherent detection using preamble/midamble/postamble for D2R transmission, capture the following observations for detection performance. - For D2R transmission with short duration, the performance gain obtained from coherent detection is negligible. - For D2R transmission with long duration, - Error floor can be observed even with the insertion of multiple midambles/postamble. - Performance of non-coherent detection may outperform coherent detection in high SNR region. Observation 10: If postamble is used to determine the end of PDRCH transmission, mis-detection and false alarm of postambe detection will lead to PDRCH not decodable. Observation 11: Assuming 1000bits as the maximum TB size for PDRCH, a control field with up to 7 bits is sufficient for TB size indication. - To achieve a required mis-detection/ false alarm rate of postamble detection, the overhead or length of postamble may need to be large, comparable with or even larger than that for control field no matter whether the TBS is large or small. Proposal 13: PDRCH payload size can be explicitly indicated in the scheduling PRDCH, and postamble is not necessary for end indication of PDRCH transmission. |
| Oppo [13] | Proposal 17: D2R preamble should be reader specific, collision-resistance and distinguishable from PDRCH, binary orthogonal sequences can be considered as D2R preamble. Proposal 18: Mid-amble is not considered for D2R transmission. Proposal 19: Post-amble is not supported for PDRCH. |
| Xiaomi [14] | Proposal 14: For the D2R preamble which is immediately transmitted before any PDRCH transmission: - Golay sequence can be used as baseline, other sequence, e.g., m sequence or Gold sequence can be further studied. Proposal 15: D2R postamble can be a complementary Golay sequence of the preamble. - Other sequence, e.g., m sequence or Gold sequence can be further studied. Proposal 16: D2R midamble can be same or truncated sequence of preamble. |
| Xiaomi [15] | Observation 5: For coherent reception of D2R transmissions, - Coherent D2R reception at gNB/intermedia UE side causes little complexity and obtains obvious gain compared to non-coherent reception. - The design of D2R preamble based on sequence will cost more standardization effort. Proposal 7: Coherent reception of D2R transmissions is the basic assumption in Rel.19 SI. |
| CATT [16] | Proposal 28: Clock-acquisition part (CAP) for D2R preamble should use the same encoding method and chip duration as the PDRCH transmitted subsequently, such as Manchester coding. Proposal 29: The reader can obtain chip synchronization by detecting the rising and falling edges of the CAP sequence. Proposal 30: The CAP for D2R preamble can carry some additional information, such as device IDs or device group IDs, different CAP sequences correspond to different devices or device groups, so that the reader can obtain device IDs or device group IDs by detecting CAP sequences. Proposal 31: There is no need to introduce D2R midamble for A-IoT downlink communication. Proposal 32: D2R postamble should not be introduced to indicate the end of PDRCH transmission and TBS indication should be included in the D2R scheduling information of PRDCH. |
| ZTE [17] | - Proposal 17: One or multiple sequences can be studied for the D2R preamble. The design/length of D2R preamble should be determined based on the metrics such as synchronization accuracy, channel measurement results, BLER performance, etc.. Proposal 18: Midamble can use the same sequence as preamble or sequences distinct from the preamble. - Proposal 19: To determine or derive the end of PRDCH transmission, study D2R postamble immediately following the PDRCH. |
| Sony [20] | Proposal 7 – Support transmission of a postamble as an end-delimiter of data transmission. |
| Samsung [25] | Proposal 4: For D2R, a preamble does not include a start indicator part. Proposal 6: For D2R, study a clock acquisition part considering different encoding schemes (i.e., Manchester, FM0, and Miller), while the clock acquisition part may have more than one formats for a given encoding scheme. Proposal 9: For D2R transmission, an insertion of a midamble is either explicitly or implicitly indicated to the device. - Explicit indication is provided in the preceding R2D transmission, e.g., PRDCH providing control information. - Implicit indication is based on either the PDRCH message type and/or the total payload size of PDRCH. - For a long D2R transmission, if the transmission needs to be paused in the middle of signaling due to charging, the resumed transmission starts with a midamble. Proposal 11: For both R2D and D2R transmission, study a postamble with a short sequence for end-of-signaling indication purpose. |
| ETRI [26] | Proposal 5: D2R preamble is a single sequence with a fixed pattern as a starting point for discussion. |
| Mediatek [27] | Proposal 5: Separate preamble (followed by a short header) and data channel for D2R transmissions. |
| Apple [28] | Proposal 9: For D2R time acquisition signal, start indicator part could be considered for the scenario when the transmit reader and receive reader are different Proposal 10: D2R preamble can be further considered for the purpose of channel estimation at the reader - FFS: preamble sequence design details - FFS: if measurement-based proximity determination at the reader is further considered, then study D2R preamble for the purpose of proximity determination as well |
| Docomo [30] | Proposal 9: - Study binary sequence with good auto-correlation property for D2R preamble. |

[Closed] 1st Discussion Round

~20 companies provided their views on the D2R time acquisition signal. Multiple aspects are considered including the sequence design for the preamble, number of sequences and functionality of D2R preamble. On the aspect of sequence design and number of sequences, companies consider the support of multiple sequences to allow for more effective multiple access for D2R transmissions. In addition, since the receiver for D2R transmission can be complex, therefore sequence-based correlation is considered for D2R preamble. Based on this, sequence with good correlation properties is considered. In terms of functionality, in addition to time acquisition, also SFO and channel estimation based on preamble is considered. Other aspect is whether a start indicator part is considered or not for D2R preamble. Based on the input, there seems to be no strong justification to support start-indicator part for D2R preamble. Based on these discussions, following proposals related to D2R signals are made.

(HP) Proposal 2.2.1-1

For D2R transmissions, a start-indicator part is not considered for preamble preceding the PDRCH

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | OK |  |
| TCL | okay |  |
| xiaomi | OK |  |
| Spreadtrum | OK |  |
| Honor | OK |  |
| LG Electronics | OK |  |
| Ericsson | Support |  |
| InterDigital | Ok |  |
| Samsung | Ok. |  |
| Huawei, Silicon |  | We think the D2R preamble also need to provide timing acquisition, so prefer to be more specific on the meaning of start-indicator and minor editorial to align with previous agreement. Update as follows, For each D2R transmissions, a R2D-like start-indicator part is not considered for preamble preceding the PDRCH |
| Moderator (Apple) | Already agreed |  |

(HP) Proposal 2.2.1-1

For D2R transmissions, preamble preceding the PDRCH can be considered also for the purpose of:

- SFO estimation

- Channel estimation

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | OK |  |
| TCL | Agree with this proposal. Except for SFO estimation and channel estimation in proposal purposes, other functions may be considered, e.g., D2R signal measurement based proximity determination. Thus, we suggest this proposal as : For D2R transmissions, preamble preceding the PDRCH can be considered also for the purpose of: - SFO estimation - Channel estimation Other function if needed |  |
| xiaomi | OK |  |
| FUTUREWEI | ok |  |
| Spreadtrum | OK |  |
| Honor | Agree with TCL |  |
| LG Electronics | OK |  |
| Lenovo | OK |  |
| Ericsson | Yes. We suggest adding CFO estimation as another purpose. |  |
| InterDigital | Ok |  |
| Samsung | OK but not sure if we need to list the purposes. It can be simplified as follows. Proposed modified proposal: For D2R transmissions, preamble preceding the PDRCH can be considered also for the purpose of: - SFO estimation - Channel estimation |  |
| Huawei, HiSilicon | OK We understand apart from above two, another is for timing acquisition which already agreed. |  |
| Moderator (Apple) | Seems like companies are fine with proposal @Samsung: It is the intention of the proposal to identify specific functionalities in addition to already agreed time acquisition @Huawei: Correct Proposal updated for 2nd round to include FFS on other functionalities, as suggested by TCL |  |

[Open] 2nd Discussion Round

Proposal 2.2.1-1A

For D2R transmissions, preamble preceding the PDRCH can be considered also for the purpose of:

- SFO estimation

- Channel estimation

FFS: Other functionalities

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| TCL | Support |  |
| LG Electronics | OK |  |
| Lenovo | Yes |  |
| Huawei, HiSilicon | Yes with followings We think it would be better to add if any, similar as TCL suggested. FFS: Other functionalities if any needed |  |
| Spreadtrum | Yes |  |

### D2R Control information

| Company | Observations/Proposals |
| --- | --- |
| Futurewei [1] | Proposal 7: The D2R scheduling information contains the following fields. - Time domain resource allocation; TBS; Modulation / coding scheme; Repetition; Frequency domain resource allocation; [line encoding]; next decode. Proposal 8: update the RAN1#117 agreement For L1 D2R control information (if defined), the following are not considered for further study: - CSI feedback - Autonomous SR - FFS: Whether any other L1 D2R control information is needed or not |
| Nokia [2] | - Observation 16: Due to three different device types considered in this AIoT SI, RAN1 should consider either a separate control channel or reuse the existing PDRCH channel to carry control only, data only, or control and data fields in a flexible way. - Proposal 13: RAN1 should study the flexible channel structure for PDRCH to carry control only, data only or both control and data using the same channel. - Proposal 14: RAN1 to study the necessary information to be contained in PDRCH depending on the AIoT device type, including control and data information. |
| Ericsson [3] | - Observation 9: HARQ operation is not supported but a confirmatory response (e.g., using an ACK) for R2D transmission can be considered. - Observation 10: To assist the reader with scheduling and resource allocation, the device can indicate the estimated number of PDRCH transmissions that can be supported before requiring a recharge and the estimated time taken by A-IoT device to recharge by harvesting energy. - Observation 11: Not all D2R control information needs to be supported by all device types. - Proposal 11: PDRCH D2R control information can include a confirmatory response (e.g., ACK) for the R2D message. - Proposal 12: PDRCH D2R control information can include the number of PDRCH transmissions that the A-IoT device can support before requiring a recharge. - Proposal 13: PDRCH D2R control information can include the estimated time taken by the A-IoT device to recharge. - Proposal 14: D2R control information transmitted by A-IoT devices can be carried by PDRCH. - Observation 16: UL message size for inventory reporting is in the range of 144-408 bits assuming the application-level device ID (EPC-like ID) of 96-256 bits. - Observation 18: The maximum TB size will depend on whether messages can be segmented into multiple smaller TBs or not. - Proposal 17: RAN1 to wait for coverage evaluation results before discussing the possible values for TB size. |
| Huawei [4] | Proposal 15: For D2R transmission, the maximum TBS is equal to the maximum message size of 1000 bits for Ambient IoT. - Inform RAN2 of this agreement. Proposal 16:Remove the FFS on D2R control information, and update the bullet as follows. - FFS Whether/how/what No D2R control information (if defined) is needed transmitted on the PDRCH |
| Spreadtrum [5] | Proposal 9: Support ACK/NACK response of device in the command procedure. |
| TCL [6] | Observation 13: For UL transmission, if RN 16-like local ID or EPC-like device ID need to be sent to reader by PDRCH, then D2R control information is not necessary for PDRCH because of the limited data size. Observation 14: More data information seems need to be transmitted in PDRCH, e.g., RN 8/RN 16/device ID. Proposal 18: Data channel for PDRCH should be at least studied for D2R transmission. Proposal 19: Discussion on the motivation and requirement of D2R control information. |
| CMCC [7] | Observation 8: Applying TA is meaningless for an Ambient IoT device since it is difficult to maintain timing synchronization due to poor SFO performance. Observation 9: The feasibility of CDMA is deteriorated by poor SFO performance. The multiplexing capability of CDMA is quite limited. Observation 10: When studying optimization to improve the efficiency of contention-based access, TDMA and FDMA are more feasible and reliable than CDMA. Proposal 15: For D2R, response for contention resolution during contention-based access procedure (i.e., Msg 1) is transmitted on PDRCH. - A dedicated physical channel to transmit response for contention resolution is not considered for study. Proposal 17: TBS determination for R2D and D2R transmissions should be further studied, considering at least the presence or absence of postamble, frame structure (incl. number of transmission occasions, code block size for each transmission occasions, etc.), and R2D control information. If TBS is defined, the TBS size differs for each indices is not restricted to be equally difference. |
| Intel [8] | Proposal 5: - HARQ-ACK feedback in physical layer is not supported for A-IoT system. |
| Lenovo [9] | Proposal 12: Study a D2R control part immediately following the preamble of the D2R transmission and having separate encoding and CRC from the D2R data part. |
| China Telecom [10] | Proposal 8: For A-IoT D2R, no need to define a specific PUCCH-like channel to transmit control information. Proposal 9: For A-IoT D2R, study whether and how to transmit ACK/NACK from device to reader via PDRCH. |
| Vivo [12] | Observation 7: Support of any AIoT D2R control information depends on the D2R scheduling and timing relationship discussions planned in agenda 9.4.2.2 in RAN1 and device message size/status discussions in RAN2. Observation 12: Necessity, feasibility and benefits of supporting a maximum TBS of 1000bits are unclear. Observation 13: For a PRDCH/PDRCH with low data rate and large payload size (e.g., a maximum TBS of 1000bits), following disadvantages are observed: - Excessive long time occupation of channel resources. - The AIoT device may be not able to finish transmission/reception if the duration for the PDRCH/PRDCH exceeds the sustainable time of AIoT device due to limited energy storage. Observation 14: Even if RAN1 would agree to use maximum message size as maximum TBS from specification perspective, it should be up to reader to determine the TB size for PRDCH and PDRCH. It does not necessarily mean a message cannot be delivered in multiple transmissions with multiple TBs with TB size less than the max TB size. Proposal 15: Inform RAN2 that from RAN1 perspective, the TBS for a PRDCH and PDRCH transmission can be smaller than the message size and should be determined by the reader. |
| Oppo [13] | Proposal 20: Feedback to indicate whether an A-IoT device has successfully execute a command or not should be supported; intermediate node should transfer the feedback to gNB in topology 2; FFS whether the feedback is conveyed by PHY layer signalling or higher layer signalling. |
| Xiaomi [14] | Proposal 9: HARQ-ACK for triggering R2D retransmission is not supported in A-IoT is not supported as L1 D2R control information. Proposal 10: The confirmatory response for the R2D command should be included in higher-layer payload. Proposal 11: There is no need to carry the PDRCH end indicator in the L1 D2R control information. - The reader can acquire the end of PDRCH transmission by D2R postamble immediately following the PDRCH. Proposal 12: There is no need to support L1 D2R control information in A-IoT. |
| CATT [16] | Proposal 37: Response transmitted from device to reader during contention-based access procedure should be as follows: - After receiving an interrogation command, the A-IoT device transmits a random access identifier along with contention resolution information as the response to the reader. - After receiving an acknowledgement of the random access identifier from the reader, the A-IoT device transmits device identity as the response to the reader. Proposal 38: Device identity related control information should be included in uplink information payload of PDRCH. Three kinds of device identity information can be considered as follows, - A-IoT Device ID: indicate the identity of the device. - A-IoT Device group ID: indicate the group of the device belonging to. - A-IoT Device type ID: indicate the type of device, i.e., sensors. Proposal 39: The D2R control information and data in PDRCH include the device function, security, and any higher information studied in RAN2, RAN3, SA2, SA3 and CT1. Proposal 36: For PDRCH, transport block size is no more than 100 bytes according to the message sizes requirements for SA1 applicable use cases of indoor inventory and indoor command. The candidate values of transport block sizes for PDRCH can be {128, 256, 512, 1024} bits. |
| ZTE [17] | - Proposal 13: The necessity of D2R control information in D2R transmission should be clarified firstly. |
| Panasonic [18] | Proposal 8: The control information for D2R should be carried alone or along with the data information. The control information includes contention-based request for channel access, request for data transmission (possibly with the indication of the data size), device type report, and power status report. Proposal 9: The CRC design of D2R should be discussed after the content of control information is determined. |
| Interdigital [19] | Proposal 11: Support AIoT device indicating its Device ID/Temporary ID to the reader. Proposal 12: Support AIoT device acknowledging the reception of signal adjusting timing of activity cycle. Proposal 13: AIoT device can indicate minimum time required before transmission of a response to a reader request. Observation 1: The maximum transport block size for a device can be limited at least by its available stored energy. Proposal 4: The maximum transport block size for D2R transmission is device-specific and depends on at least the available energy of the device. |
| Sony [20] | Proposal 6: Introduction of the control information prior to data transmission is needed to indicate the total size or length of the data transmission. |
| Continental [21] | Proposal 1.2.: The following dynamic control information is proposed to be included in D2R transmissions: Information; Needs to be signaled?; Via L1 control; Via higher layer. Response to request for information including charging status/time required for device to be charged to certain level; Y; N; Y. |
| NEC [22] | Proposal 4: If channel coding is supported for PDRCH, study the following for indicating the channel coding code rate: - Option 1: code rate for PDRCH is indicated by R2D control information - Option 2: code rate for PDRCH is indicated by D2R control information |
| Honor [23] | Proposal 5 The D2R control can be used to provide the information to indicate the D2R transmission, and at least the following contents can be considered, including: - Early control indication for distinguishing information types - MCS - The number of information bits - Repetitions - Transmit/max power - ACK/NACK Proposal 6 Support using control field to map the D2R control information, which is similar to the R2D control information mapping method. Table 2: Contents of D2R control information Contents of control information; D2R control to indicate D2R data transmission. Early control indication; Y (At least to distinguish between control and data). MCS; Y (Selected by the device but notified to the reader). The number of information bits; Y (Selected by the device but notified to the reader). Repetitions; Y (Selected by the device but notified to the reader). Transmit/max power; Y. ACK/NACK; Y. |
| LGE [24] | Proposal 12: Consider the following tables for D2R control information: Table 3: D2R control information for PDRCH Information; Needs to be Signaled; Via L1 Control; Via higher-layer; Note. TDRA; Not needed. MCS; Not needed; This info can be indicated by R2D control information.. TBS; Optional; Yes; This info can be needed in case that reader cannot determine TBS for D2R response. TBS is not needed at least for MSG1. Repetitions; Not needed; This info can be indicated by R2D control information, if supported.. FDRA; Not needed; This info can be indicated by R2D control information. Device ID; Maybe; FFS; FFS; FFS: This info may be needed in case random number is not used in D2R i.e. MSG1 has no device ID.. Reader ID; Optional; Yes (for Proximity Determination); This info can be needed in case of multiple reader environment. Device Group ID; Not needed. Device Type; Maybe; FFS; FFS; FFS: whether this info is needed for RACH. Cast Type; Not needed. Chip Duration; Not needed; Chip duration of L1 control information can be same as that of TB.. Proposal 14: For chip duration of D2R preamble and PDRCH, consider the following options: - Option 1: Chip duration of D2R preamble and PDRCH is implicitly determined by the corresponding R2D transmission. (e.g. D2R chip duration is same as R2D chip duration) - Option 2: Chip duration of D2R preamble and PDRCH is indicated by the corresponding R2D transmission. (e.g. D2R chip duration is indicated by R2D control information) |
| ETRI [26] | Proposal 6: ACK/NACK feedback from device to reader is not considered for D2R control information. |
| Mediatek [27] | Proposal 4: For D2R control transmission, at least the following information can be studied: - ACK/NACK-like information - Device available energy information |
| Apple [28] | Proposal 11: For D2R transmissions, “ACK/NACK” feedback from the device to the reader could be studied, if justified for DT type of traffic - This should not imply support HAQR retransmissions Proposal 12: For potential energy-aware scheduling at the reader, additional feedback from the device to the reader could be considered |
| Sharp [29] | Proposal 7: For a D2R data transmission, the maximum TBS is 1000 bits. Proposal 8: RAN1 to study an appropriate maximum payload size of the PDRCH transmission in response to a PRDCH transmission initiating a contention-based access procedure. |
| Docomo [30] | Proposal 18: - Study the max TBS in one PDRCH considering at least the max time duration of one PDRCH, coding scheme including FEC, coding rate, and chip duration. Proposal 19: - Study ACK/NACK feedback in L1 D2R control information. |
| Qualcomm [31] | Proposal 10: For D2R control, RAN1 to consider at least the functionality of ACK/NACK response, e.g., additional information for the ACK, and additional information for the NACK. Proposal 13: For time/freq resource allocation of A-IoT communications - For Topology 1: BS configures R2D/D2R time/freq resources for A-IoT - BS/reader to control dynamic R2D/D2R within the configured time/freq resources. - For Topology 2: BS semi-statically or dynamically configures the time/freq resources per the UE/reader via Uu, at least for inband/guardband operation. - UE/reader to trigger R2D/D2R within the configured time/freq resources - FFS: how to solve collision among UEs/readers for a shared resource Proposal 14: Study whether/how to apply power control for A-IoT devices and UE/reader. - The power control for R2D transmission at UE/reader should be independent from that of legacy UL transmission. |
| AsusTek | Proposal 1: Study ACK feedback from device to reader. Proposal 3: For PDRCH generation, not define L1 D2R control information and CRC attachment for D2R control information. |

[Closed] 1st Discussion Round

~29 companies provided their views related to D2R control information. One of the key points of discussion is related to whether “ACK” feedback is considered or not for the purpose of confirmation of reception of R2D transmissions. Additionally, some companies consider including power status, device type report, Msg1 response as part of the L1 D2R control. Based on these discussions, following proposal is made

Proposal 2.2.2-1

For D2R, ACK/NACK feedback is considered

- FFS: How the ACK/NACK feedback is multiplexed on PDRCH

Please note that this doesn’t imply the support of HARQ operation

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | okay |  |
| xiaomi |  | We think this should be carried by higher layer payload. |
| FUTUREWEI |  | Do not support ACK/NACK feedback – we want to separate physical layer signal from upper layer signaling |
| OPPO | OK. |  |
| Spreadtrum | OK |  |
| Honor | OK |  |
| LG Electronics | OK |  |
| Ericsson | A wording suggestion is ACK/NACK feedback for PRDCH |  |
| InterDigital | ok |  |
| Huawei, HiSilicon |  | We do not see any specific reason to support L1 ACK/NACK. So first thing is why it is needed and secondly why it should be carried in L1. |
| ZTE, Sanechips |  | We don’t think ACK/NACK feedback is needed. In R2D transmission, a D2R response is always triggered, implicitly conveying ACK information. If the reader fails to detect the D2R response, it can infer that the R2D transmission was not successfully received. Consequently, we do not perceive the necessity for explicit L1 ACK/NACK feedback.下 Moreover, this proposal is also related to the discussion in RAN2, and should be depend on RAN2 conclusion. |
| Qualcomm | Support |  |
| Moderator (Apple) | @Futurewei, Xiaomi, Huawei, ZTE: Motivation for this particularly for the command use-case, where this is no explicit or implicit response from device Whether ACK/NACK feedback is sginaled as L1 D2R control information or a part of the response can be further discussed. Intention of this proposal is to study ACK/NACK feedback Proposal updated for 2nd round summary |  |

[Open] 2nd Discussion Round

Proposal 2.2.2-1A

For D2R, ACK/NACK feedback for PRDCH is studied

- FFS: How the ACK/NACK feedback is multiplexed on PDRCH

Please note that this doesn’t imply the support of HARQ operation

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Support but it should be “for PRDCH”? |  |
| TCL | Support this proposal. Same question like Decomo. Or “for PDRCH” ->”on PDRCH”. In addition, we think ACK/NACK could also be studied in re-transmission scenarios. |  |
| Moderator (Apple) | Yes, it is PRDCH. Sorry for the typo. |  |
| LG Electronics | OK |  |
| Lenovo | OK |  |
| Huawei, HiSilicon |  | Thanks for FL’s clarification. Even for command, currently the followings related to command procedure were agreed from RAN2 - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. So there would be corresponding device to reader data transmission to response the command in step D. The only thing is FFS whether it is optional or not. We still don’t see the need for L1 ACK/NACK. |
| Spreadtrum | Yes |  |
| FUTUREWEI |  | There is no need for an ACK/NACK field for a command in L1 D2R control as the ACK/NACK would be forwarded to upper layer anyways. |
| ZTE, Sanechips |  | Based on the discussion so far, we are still not sure whether the ACK/NACK feedback is needed, following update is suggested. For D2R, study whether/how to support ACK/NACK feedback for PRDCH is studied - FFS: How the ACK/NACK feedback is multiplexed on PDRCH |
| OPPO | Support. In our understanding ACK/NACK here corresponds to Step D agreed in RAN2. |  |

### PDRCH design details

| Company | Observations/Proposals |
| --- | --- |
| Futurewei [1] | Proposal 9: For the PDRCH, support a common design using a CRC. |
| Nokia [2] | - Observation 16: Due to three different device types considered in this AIoT SI, RAN1 should consider either a separate control channel or reuse the existing PDRCH channel to carry control only, data only, or control and data fields in a flexible way. - Proposal 13: RAN1 should study the flexible channel structure for PDRCH to carry control only, data only or both control and data using the same channel. Proposal 14: RAN1 to study the necessary information to be contained in PDRCH depending on the AIoT device type, including control and data information. |
| Ericsson [13] | - Observation 9: HARQ operation is not supported but a confirmatory response (e.g., using an ACK) for R2D transmission can be considered. - Observation 10: To assist the reader with scheduling and resource allocation, the device can indicate the estimated number of PDRCH transmissions that can be supported before requiring a recharge and the estimated time taken by A-IoT device to recharge by harvesting energy. - Observation 11: Not all D2R control information needs to be supported by all device types. - Proposal 11: PDRCH D2R control information can include a confirmatory response (e.g., ACK) for the R2D message. - Proposal 12: PDRCH D2R control information can include the number of PDRCH transmissions that the A-IoT device can support before requiring a recharge. - Proposal 13: PDRCH D2R control information can include the estimated time taken by the A-IoT device to recharge. - Proposal 14: D2R control information transmitted by A-IoT devices can be carried by PDRCH. |
| Huawei [4] | Proposal 13: Remove the sub-bullet of FFS on “Details of response”, and leave the details to RAN2, i.e.: “Response transmitted from device to reader during contention-based access procedure is transmitted on the PDRCH - FFS: Details of response” Proposal 14: Add the exact coding method in sub-bullet of “coding” to support FEC/line coding/repetition, i.e.: - FEC and line coding methods within the coding block as necessities. Repetition methods such as TB repetition are also depicted. |
| TCL [6] | Proposal 20: For the repetition type of block level and bit level type 1, further discuss the repetition is before or after coding or modulation. Add “repetition” into the flow of PDRCH generation. |
| CMCC [7] | Observation 11: Line coding is the encoding scheme, e.g., Miller encoding, FM0 encoding, and Manchester encoding, that map a data bit to a codeword. Observation 12: Square wave generator is for small frequency shift, multiplying the Miller/FM0/Manchester encoded symbols by a square wave at different times the encoded symbol rate, the power spectrum can be shifted to different frequency locations (in the level of hundreds of kHz). Proposal 16: For PDRCH generation at the device, use the following generation blocks for study: - Coding block includes FEC and line coding operations, if any. - Modulation block includes small frequency shift modulation and chip modulation operations, if any. And FFS for SFS modulation schemes |
| Intel [8] | Proposal 6: - The following options can be considered for physical structure of PDRCH - Option 1: PDRCH may consist of control information and optionally data packet. - Option 2: PDRCH may consist of data packet. - Separate CRCs may be appended to the L1 control information and the data packet in PRDCH transmission |
| Lenovo [9] | Proposal 10: Study burst-type channel structure for D2R transmission for ambient IoT. Proposal 11: Study a channel structure for D2R transmission which comprises a preamble, a control part, a data part, midamble, postamble and a guard period. Proposal 12: Study a D2R control part immediately following the preamble of the D2R transmission and having separate encoding and CRC from the D2R data part. Proposal 13: Study scrambling for D2R data part. |
| Semtech [11] | - Proposal 1: RAN1 to study if and how to apply power control for A-IoT devices. - Proposal 2: RAN1 to study if and how to allow for different coding rates for A-IoT device transmissions. |
| Vivo [12] | Proposal 14: Study the channel structure of a D2R transmission consisting of preamble part, payload part and optionally CRC part, as a baseline. |
| Oppo [13] | Proposal 14: Scrambling for PDRCH should be supported. Proposal 15: Proposal 16: Re-transmission for PDRCH should be studied. Proposal 21: PRACH-like channel should be studied for contention resolution in A-IoT communication. |
| Xiaomi [14] | Proposal 13: One or two potential formats can be defined for PDRCH in A-IoT: - If only one format is defined, it is used for both the transmission of random temporary binary stream (RN16 like) and other D2R data (e.g., device ID and the potential subsequent D2R data transmission); - If two formats are defined, the following two formats can be considered: - A short format is used for the transmission of the random temporary binary stream (RN16 like); - A long format is used for the transmission of other D2R data (e.g., device ID and the potential subsequent D2R data transmission). |
| CATT [16] | Proposal 33: For ambient IoT devices, the PDRCH should be introduced for D2R transmission channel, in which at least device ID, D2R information payload and CRC are embedded. (a) (b) Figure 10: Updated PDRCH generation with scrambling Proposal 34: A scrambling block should be added before the line coding block or before the CRC block for PDRCH generation. - A scrambling block should be added before the line coding block if the scrambling functionality is provided in physical layer. - A scrambling block could be added before the CRC block to have the scrambling functionality mapping to the security mechanism provided by SA3. Proposal 35: A resource mapping block should be added after the modulation block for PDRCH generation. Proposal 36: For PDRCH, transport block size is no more than 100 bytes according to the message sizes requirements for SA1 applicable use cases of indoor inventory and indoor command. - The candidate values of transport block sizes for PDRCH can be {128, 256, 512, 1024} bits. |
| ZTE [17] | - Proposal 14: Data segmentation in D2R data transmission can be considered. - Proposal 15: Each segment should be encoded separately with CRC generated for each segment and attached afterward. - Proposal 16: For D2R random access of A-IOT, a separate channel for random access, like PRACH can be studied. |
| Interdigital [19] | Proposal 8: The reader can configure a device with PDRCH repetitions. Proposal 9: AIoT device transmits midamble between PDRCH repetitions. |
| NEC [22] | Proposal 5: If channel coding is supported for PDRCH, scrambling the output bits after channel coding is also supported. |
| LGE [24] | Proposal 10: Random number can be used in contention-based access for collision resolution. Study whether to add CRC to a random number in contention-based access. Proposal 11: For MSG1 transmission, PDRCH carries a random number in L1 control information and zero-size higher-layer payload with zero-size CRC. Proposal 15: For repetition of PRDCH or PDRCH transmission, consider bit/symbol/block repetition of x-ambles to the corresponding transmission. |
| Samsung [25] | Proposal 1: For D2R, PDRCH is the only physical channel. Proposal 2: Different types of information carried in the PRDCH or PDRCH is indicated using a header field of the signal. |
| Mediatek [27] | Observation 1: Different D2R transmission types are necessary for better fit different use cases as in RFID. Proposal 6: Study different D2R transmission types including immediate, delayed and in-process D2R transmission. |
| Apple [28] | Proposal 13: For PDRCH, if “ACK/NACK” feedback is considered, it can be classified as simply D2R data/response and therefore, no different transmission modes/schemes need to be considered for PDRCH - FFS: Any other feedback, if considered, could also be considered as data/response |
| Docomo [30] | Proposal 20: - Study dedicated PHY D2R channel for Msg.1. - Study sequence-based signal with good auto-correlation and cross correlation property for Msg.1. |
| Qualcomm [31] | Proposal 5: For D2R data mapping to PDRCH, the processing structure includes the CRC attachment, FEC coding, scrambling, modulation. - The modulation block includes both PSK and square-wave generation for device 1/2a with backscattering transmission. PDRCH generation Proposal 11: For D2R control mapping to a physical channel, RAN1 to consider - D2R L1 control is carried in PDRCH, separately from that of D2R data - E.g., different PRDCH formats for D2R data and control - D2R control in MAC-CE is carried in PDRCH, same as that of D2R data if transmitted Proposal 12: For D2R msg1 mapping to a physical channel, RAN1 to study - Alt1: D2R msg1 is mapped to PDRCH - D2R preamble is transmitted before PDRCH carrying msg1 - Alt2: D2R msg1 is mapped to a new D2R sequence-based physical channel - D2R preamble is not transmitted before the D2R sequence-based physical channel carrying msg1 |
| Google [34] | Proposal 5: Support a PDRCH can be used to transmit D2R data or to transmit both D2R data and D2R L1 control (if D2R L1 control is identified as needed). |
| IIT-K, IIT-M [35] | Proposal 5: PDRCH should be used to transmit at least the following response information along with the data payload. - Contention-resolution response during contention-based RACH. - ACK/NACK messages for indoor command use-cases. |
| AsusTek [36] | Proposal 3: For PDRCH generation, not define L1 D2R control information and CRC attachment for D2R control information. |

[Closed] 1st Discussion Round

~25 companies provided their view related to PDRCH design. In the contributions, mainly two aspects are discussed. One key aspect is related to PDRCH generation, where additional blocks and/or additional details are proposed. Specifically, most of the proposals are directed towards adding more details to the coding block considering the progress in agenda 9.4.2.1. Another aspect is related to msg1 transmission. Basically, two options are considered by companies including a separate PRACH-like channel or a PDRCH carrying msg1. Majority view is support only single PDRCH channel that can also be used for msg1 transmission. Based on these inputs, following proposals are made:

(HP) Proposal 2.2.3-1

For D2R, only single physical channel, i.e. PDRCH is studied, including transmission of msg1

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| [OPPO] |  | We have not seen the point of rushing to preclude other channel structure for Msg. 1: The study on multiple access is ongoing in 9.4.2.1, whether CDM is feasible or not is unclear. However, according to our evaluation CDM is feasible and beneficial in terms of collision mitigation. Furthermore, it is also unclear so far how serious of the collision of Msg. 1 and whether some enhancement is needed, which may depend on the inventory procedure being discussed in RAN2 and inventory evaluation being conducted in RAN1. Therefore, we suggest keeping it open and revisit it later when sufficient progress was made and more study on CDM was done. |
| TCL | Okay |  |
| xiaomi | OK |  |
| FUTUREWEI | ok |  |
| Spreadtrum | OK |  |
| Honor | OK |  |
| LG Electronics | OK |  |
| Lenovo | OK |  |
| Ericsson | Support |  |
| InterDigital | OK |  |
| Samsung | Support |  |
| Huawei, HiSilicon | Support |  |
| ZTE, Sanechips |  | Agree with OPPO, suggest keeping it open. To efficiently accommodate a multitude of devices within a constrained time frame, utilizing specialized modulation and multiplexing techniques on an additional D2R physical channel, beyond the PDRCH, could prove beneficial. Moreover, CDM is not be ruled out for D2R transmissions, and the introduction of an additional D2R physical channel should similarly remain an option. |
| Qualcomm |  | Since the evaluation for FDM, CDM, TDM is still under discussion in 9.4.2.2, we suggest keeping the options of a separate PRACH-like channel or a PDRCH carrying msg1. |
| Moderator (Apple) | It seems companies that consider additional channel (PRACH-like) is related to potential support of CDM. However, from FL perspective, I would like to clarify that even if we agree to study only single channel, i.e. PDRCH (including transmission of msg1), the design of the channel could potentially include a specific format for msg1 transmission, if justified and needed. Updated proposal for 2nd round is provided |  |

Proposal 2.2.3-2

For PDRCH generation, coding block from the previous agreement (from RAN1#116bis) is updated as follows:

PDRCH generation

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | Agree with this proposal for device 1 and 2a. We do not sure if this procedure of PDRCH generation is suitable for device 2b. |  |
| FUTUREWEI | ok |  |
| OPPO |  | 1. bit-level type 1 repetition is before FEC, block level repetition and bit-level type 2 are after FEC. It seems all of the 3 types are being studied according to the agreement last meeting. 2. we suggest to add the following for any update on PDRCH/PRDCH generation agreement. “Note: Other blocks could be added if agreed” |
| Honor |  | There are two types of repetitions are before FEC. Suggest to update according to the progress in 9.4.2.1. |
| LG Electronics | OK |  |
| Ericsson | Support |  |
| Huawei, HiSilicon | OK |  |
| Qualcomm |  | Agree with Honor that this can be revisited after the progress in 9.4.2.1, which includes not only repetitions but also whether line coding or square-wave generation is used or not. In addition, scrambling for randomization is needed for the bits after FEC and repetitions. It should be considered as well. |
| Moderator (Apple) | @Oppo: Yes, additional blocks can be added, depending on progress in other agendas @Honor: Intention here is to capture at least high-levl blocks based on agreements in other agenda. Specific repetition type once agreed can be further updated as we move along @Qualcomm: See comment to Honor on repetitions. For scrambling, we can add block, once we have an agreement in other agenda. Proposal updated to clarify that additional blocks or updates to exiting blocks can be added later |  |

[Open] 2nd Discussion Round

(HP) Proposal 2.2.3-1

For D2R, only single physical channel, i.e. PDRCH is studied, including transmission of msg1

- FFS: Whether a specific format for PDRCH carrying msg1 is needed

| - Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo |  | We think we can first discuss whether different device behavior/ signal generation procedure is needed for Msg.1 transmission, then we discuss whether it is separate channel or format. We feel we may need to agree on this proposal for now. |
| TCL |  | In some case, msg 1may be included in msg A like NR. Thus, we suggest this proposal may be postponed tp discuss until access procedure has been discussed. |
| LG Electronics | OK |  |
| Huawei, HiSilicon |  | We understand FL’s effort. But the proposal adding FFS sub-bullet seems too specific to us. To our understanding, all of existing RAN1 and RAN2 agreements clearly indicated that PDRCH is the only physical channel to carry messages. We even don’t think need to discuss this. To make progress, we would suggest more general wording as follows: For D2R, only single physical channel, i.e. PDRCH is studied, including transmission of msg1. - FFS: Whether a specific different format for PDRCH carrying msg1 is needed |
| OPPO |  | How to interpret “format”, same generation scheme as the PDRCH for other messages or can be different? |
| Spreadtrum | OK with the modification of Huawei |  |
| Samsung2 | fine |  |

(HP) Proposal 2.2.3-2A

For PDRCH generation, coding block from the previous agreement (from RAN1#116bis) is updated as follows:

PDRCH generation

Note: Additional blocks and/or updates to existing blocks can be further done, depending on progress in agenda 9.4.2.1

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo | Yes |  |
| TCL | Support |  |
| LG Electronics | OK |  |
| Huawei, HiSilicon | OK |  |
| OPPO | Bit level type 1 repetition is performed before FEC. I do not know how to reflect this in the diagram, maybe we can add a note to say the repetition block could be before FEC depending on repetition scheme. Agreement Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times |  |
| Samsung2 | FEC is not agreed yet. It’s for study. |  |

  - Topic 3: Proximity determination

| Company | Observations/Proposals |
| --- | --- |
| Futurewei [1] | Observation 2: Proximity determination is based on a reader monitoring D2R transmissions from an A-IoT device in response to a R2D transmission from a reader. Proposal 10: For proximity determination, if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near |
| Nokia [2] | - Observation 18: To enable the Ambient IoT device discovery, but also determining which nodes are in the proximity of the device, AIoT specific mechanisms for proximity determination need to be defined. - Observation 19: For option 2, the type of reader measurements should be investigated e.g. whether power-based measurement is sufficient, or timing measurement is required. In the latter case, device to reader synchronization, or the lack thereof needs to be considered when defining timing measurements. - Proposal 16: RAN1 to study the reader measurement types for proximity determination, where such measurements may be power-based or timing measurements of the D2R signal. - Proposal 17: RAN1 to study and define ways to identify the conditions that affect the proximity determination outcome. Some of these conditions may be: - Lack of AIOT response due to AIOT device being out of CW node’s range i.e., very large distance CW node – AIOT device or due to the lack of activation i.e., activation R2D signal is of insufficient strength. - Failure to detect AIOT reply due to high levels of CW interference. - Receiving a low power AIOT response due insufficient strength of the CW signal as observed by the AIOT device (due to large distance CW node – AIOT device). |
| Ericsson [3] | - Observation 19: Proximity range is the dividing line between near and far devices. - Proposal 18: Option 2, if the measurements are based on received signal strength, can be used for devices with a minimum 3D D2R distance of 1m so as to use pathloss model in TR 38.901. - Observation 20: For bi-static scenarios of D2T2, a reader’s lack of the knowledge of CW2D distance has a smaller impact on proximity determination for a small proximity range than for a large range. - Observation 21: For Topology 1, due to the height difference between indoor BS and devices, a lack of CW2D distance would cause large proximity determination ambiguity. - Observation 22: For passive devices, proximity ambiguity due to unknown CW2D distance at reader side exists for scenario A1 and B, rather than for scenario A2. - Observation 23: Similar to passive devices, active devices with different transmission power, the difference of transmission power, if unknown at reader, generates a smaller impact for small proximity range in D2T2. However, for D1T1 and a large proximity range in D2T2, lack of information of different transmission power by active devices would lead to a large proximity determination ambiguity. - Observation 24: Option 1 is a combined result of R2D and D2R transmissions. With a controlled R2D transmission power, Option 1 can be used to determine the distance between a device and a transmit reader. |
| Huawei [4] | Proposal 11: Proximity determination where successful communication between the BS/intermediate UE and the device means the device is “near” should be considered as the baseline. Proposal 12: Involvement of power control for R2D/D2R in proximity determination is not yet clear. |
| Spreadtrum [5] | Proposal 10: Proximity determination based on measurements at reader side can be studied, and different measurement thresholds can be introduced for different device types. - The case of transmit reader and receive reader are different should be considered in scenario ‘A1’. |
| TCL [6] | Observation 15: In TS 23.703, proximity determination for two UEs is in terms of geographical location and the radio link feasibility. Observation 16: When multiple devices transmit simultaneously D2R transmission to reader using FDMA, reader needs to check different devices to determine as near based on their device ID. Observation 17: For topology 2, UE reader may report the result of proximity determination to gNB. Observation 18: If gNB or UE reader does not receive D2R signal during one regulated time window, the fixed number of repetition may be considered. Proposal 22: Consider different reception criteria based on different device types and consider how to identify correctly device from multi-devices when using FDMA for PDRCH transmission. Observation 19: For device 1, the EIRP of D2R at device side is decided by CW transmission power, pathloss and modulation loss. For device 2a, the EIRP of D2R at device side is decided by CW transmission power, pathloss, modulation loss and reflection amplifier gain. Observation 20: If binary distance between the reader and the ambient IoT device is used for the calculation, then inventory or command transmission may cover the requirement. Proposal 23: Clarify the mean of proximity determination with relative distance and binary distance. Proposal 24: It is preferred using the relative distance to calculate the distance between reader and device. Proposal 25: Discuss potential methods for RSRP-based binary distance, including inventory commands as measured signal, full-backscattering and modulated reference signal. Observation 21: Other methods of getting relative distance, including TOA/RTT and phase-based techniques, have been proposed in RFID. Proposal 26: Consider suitable method for relative distance, like RTT and phase-based method. |
| CMCC [7] | Observation 13: Considering separate deployment of different device types, the issue that the determined “near” is not associated to the same physical distance for different device types with different transmit power no longer exists. Proposal 18: For proximity determination at reader side, the following solutions can be further studied: - Option 1 (preferred): If reader successfully decode D2R transmission from the device in response to R2D transmission, then device is determined as near. - Option 2: Device is determined to be near the reader based on measurements at the reader side. FFS measurements and potential RAN1 specification impact. |
| Intel [8] | Proposal 7: - For proximity determination, Option 1 is prioritized for A-IoT system design - If reader successfully receives D2R transmission from the device in response to R2D transmission, the device is determined as near. |
| Lenovo [9] | Proposal 19: Study following two types of proximity determination: - Type 1: Proximity determination of a dedicated device(e.g., the reader may indicate the identification of the dedicated device). - Type 2: Proximity determination of any device, and the reader doesn’t care about the identifications of the devices during proximity determination procedure. Proposal 20: Study proximity determination for both monostatic and bistatic topology scenarios. Proposal 21: Study the intermediate node selection procedure with proximity detection for topology 2. |
| Vivo [12] | Proposal 16: For Option 1 proximity determination, if reader receives D2R transmission from a device which has already been identified before or if reader identifies a device that is not yet identified , the device is determined as near. Proposal 17: For Option 1, proximity determination can be supported via CW power control by reader. Observation 15: For Option 2, the following are observed: - if the reader treats the measurements as intermediate results and report only ‘near/far’ determination, there may not be a strong motivation to specify the measurements. - for the estimated distance used for ‘near/far’ determination, whether the expected accuracy and benefit can be achieved is questionable. Proposal 18: Support Option 1 as the scheme for proximity determination. Proposal 19: For proximity determination, prioritize scenario of the same transmit and receive reader with CW inside topology. |
| Oppo [13] | Observation 7: The positioning or ranging mechanism(s) used in current NR system for proximity determination cannot be applied to A-IoT. Proposal 23: In RAN1 discussion it should assume that “near” in proximity determination means the device is geographically close to the reader, i.e., within X meters from the reader. Proposal 24: For Option 1 R2D transmit power should be considered for proximity determination. Observation 8: For Option 1, device reception capability, i.e., with/without reception amplifier, would lead to near determination difference. Proposal 25: For Option 2 D2R transmit power should be considered for proximity determination. Observation 9: Option 2 is applicable to mono-static scenario only. Proposal 26: If proximity is determined based on measurement at reader, whether the proximity determination is a separate procedure should also be discussed. |
| Xiaomi [14] | Proposal 19: RAN1 may need to firstly clarify what is the purpose and using scenario for introducing “proximity determination” in Rel-19. Observation 1: There are some usages can be considered for the “proximity determination”, e.g.: - To facilitate the scheduling performance by such kind of measurement, which can be further studied by RAN1; - To determine the intermediate UE or for devices’ positioning in topology 2, which may be up to other WG’s discussion. Proposal 20: For proximity determination, device is determined to be near the reader based on measurements and pre-defined threshold. |
| CATT [16] | Proposal 40: For proximity determination, both the following two options should be supported to determine whether the device is near the reader or not at reader side: - Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near. - The reception criteria should be CRC check at the reader. - If the CRC check is correct at the reader, the device is determined as near. - Option 2: Device is determined to be near the reader based on measurements. - The received timing information to derive the propagation delay would be used to refine the device position in proximity determination. Proposal 41: For proximity determination, the transmit power of R2D and the receive power D2R could be used to determine the round trip distance. |
| ZTE [17] | - Observation 10: For proximity determination, option 1 is a special case of option 2. - Observation 11: Acquiring a measurement result is more advantageous for the detection and location of an A-IoT device.. - Proposal 20: For proximity determination at the reader, Option 2 can be studied, i.e., Device is determined to be near the reader based on measurements at the reader side. |
| Panasonic [18] | Proposal 10: A simple signal detection reusing contention-based or contention-free access should be considered for the proximity determination. |
| Interdigital [19] | Observation 3: Proximity determination procedure requires successful decoding of PDRCH transmission. Proposal 14: The reader can request the device to transmit a PDRCH to initiate proximity detection procedures. Proposal 15: The reader determines the device proximity using the measurement of the received PDRCH. |
| Sony [20] | Proposal 8: Study proximity to active carrier wave emitter for Ambient IoT device proximity determination purposes. Proposal 9: The following reader-side measurements can be used to facilitate device proximity determination: - Reader received signal power. - Time of flight of the direct carrier wave and the backscattered signal from a tag |
| Continental [21] | Observation 2.1.: Proximity determination based on binary success/failure of D2R (or R2D) transmissions are not reliable and require additional specification of criteria to associate number of successes/failures with proximity of the device. Proposal 2.1.: Proximity determination based on option 2, specifically based on measurements of path loss from D2R transmissions, should be prioritized. |
| Honor [23] | Observation For device 1 and device 2a, reporting backscatter power is not feasible; For device 2b with amplification capability, reporting its transmission power is feasible. Proposal 7 Support to use option 1 for proximity determination: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near. |
| LGE [24] | Proposal 22: Study the following cases for Proximity determination - Case 1: The TX or RX reader (gNB or IN) determines proximity of one or more identified devices - Case 2: The TX or RX reader (gNB or IN) determines proximity of one or more non-identified devices Proposal 23: Study the following issues to determine which option is used for proximity determination by reader: - whether to consider identified devices and non-identified devices for proximity determination - whether to support one-shot reception based determination or average reception quality based determination. - which sequence/signal is used for D2R measurement - whether proximity determination procedure is known to devices e.g. whether to support measurement-specific D2R transmission e.g. without a TB. Proposal 24: When TX reader and RX reader are different, a device determines proximity to the TX reader based on CRC check result by the device, like in option 1. Proposal 25: When TX reader and RX reader are different, Option 2 can be used for determination of proximity between the device and RX reader, i.e. measured result on D2R transmission by RX reader. |
| Samsung [25] | Proposal 16: Given the dependency of Option 2 on the topologies, device types, and implementation-specific parameters, study Option 1 as a baseline for proximity determination. Observation 2: Option 1 for the proximity determination is preferable as it can work with any type of devices in any topology, i.e., no fragmented solutions, and it can reuse to-be-designed random access or command mode protocols. |
| ETRI [26] | Proposal 7: Study the followings for the proximity determination - For Option 1, the reader determines the proximity with only some D2R transmission(s) and [X]% of reception success rate. - For Option 2, the D2R transmission power or CW signal power can be reported to or shared with reader. |
| Mediatek [27] | Proposal 8: Utilize R2D control repetition for proximity determination. |
| Apple [28] | Proposal 14: For Option 1 for proximity determination based on response reception, successful reception at both the device and reader are considered to determine that the device is “near” to the reader - For inventory use-case, reader could determine successful reception at both the ends without explicit “ACK” from the device - For command use-case, explicit “ACK” from the device may be considered Proposal 15: For Option 2 for proximity determination based on measurement, preamble bases signal measurement and comparison with threshold could be considered as the baseline Proposal 16: For option 2 for measurement-based proximity determination, if transmit power is considered, then the measurement considered for determination could be based on the ratio of the received signal strength of the D2R and the corresponding D2R transmit power Proposal 17: For further study for proximity determination, consider possible combination of option 1 and option 2: - If response is received successfully, then device is determined within the coverage range of reader, for example, considered near - Additionally, if needed to communicate only with a subset of devices within the coverage range, for example, to determine very near devices, then measurement-based option is applied Proposal 18: For further study for proximity determination, if device passes only response-based proximity determination at the BS (reader), but not pass the measurement-based proximity determination, then network may configure intermediate UE (topology 2) to serve device, if for that intermediate UE, measurement-based proximity determination is passed Proposal 19: For proximity determination in case when the transmit reader and receive reader are different, study following options: - Option 1: Transmit-reader based proximity determination - Option 2: Receive-reader based proximity determination - Option 3: End-to-end system proximity determination |
| Docomo [30] | Proposal 21: - Clarify motivation and use case of proximity determination first before further study of the options. Proposal 22: - Clarify the agreed option 1 (i.e., If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near.) of proximity determination as following. - If reader successfully receives D2R transmission from the device in response to R2D transmission, then device is determined as near, where “successfully receives” means CRC check is correct at the reader. Proposal 23: - All scenarios A1, A2, B should be considered for study of proximity determination. |
| Qualcomm [31] | Proposal 15: RAN1 to clarify that the proximity determination is to identify whether a device is in a very close range (less than 1m) from the reader. - The ‘near’ determination by the reader should NOT be very different for the devices with different transmit power. Proposal 16: Rel-19 A-IoT study to prioritize the proximity determination only for mono-static scenario with the CW inside topology. Proposal 17: RAN1 to prioritize the power-based measurement at the reader side for proximity determination - The D2R transmission power can be reported by the device to avoid distinct proximity determination for different devices. |
| IIT-K, IIT-M [35] | Proposal 6: RAN 1 should agree on target accuracy requirements for AIoT proximity determination before further studying methods for proximity determination. Observation 2: The proximity determination based on the device’s transmitting power can be left to reader implementation. Proposal 7: Device capability exchange should be studied to support different AIoT device classes. |

[Closed] 1st Discussion Round

~27 companies provided views related to proximity determination methods. Based on the two options agreed in RAN1#117, mainly companies shared their views related to their preference among the two options. Based on the inputs, majority companies consider option 1 as the baseline, i.e. response-based method. Furthermore, many companies clarified that the successful response (on both sides) is considered to determine the device as near. In addition, on the aspect of transmit power, it seems, some further discussion may be needed whether/how the transmit power should be taken into account. Based on these inputs, following proposal is made for proximity determination

(HP) Proposal 2.3-1

For proximity determination, option 1 is considered as the baseline, i.e. if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

- Successful reception/decoding at the reader is considered

- FFS: whether/how the transmit power of the device is considered

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| OPPO | We are fine with considering Option 1 as baseline. However, for Option 1 not only the transmit power of the device impacts the proximity determination but also the R2D transmit power. In another word, the reader can also reduce it transmit power, such as the power of A-IoT paging, to prevent the devices far away from responding. - FFS: whether/how the transmit power of the reader and/or device is considered |  |
| TCL | Agree with this proposal. We suggest adding one FFS for the first bullet to further explain how/when receiving successfully D2R transmission from device. Another suggestion is same as OPPO. For proximity determination, option 1 is considered as the baseline, i.e. if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near - Successful reception/decoding at the reader is considered - How to determine if the D2R signal is successfully received for proximity determination. FFS: whether/how the transmit power of the device and/or reader is considered |  |
| xiaomi |  | It is not quite clear until now what the motivation or using scenario of proximity determination, without this, we think it cannot be performed downselection because there may be some use cases which are applicable for option 1 but some others are applicable for option 2, so we think RAN1 should keep |
| Continental | In our view, it is OK to study successful reception of D2R transmission in response to an R2D transmission as baseline, but it needs to be clarified whether it is restricted to a single successful reception of a D2R transmission. In our view, basing proximity determination on just one D2R reception is not reliable, especially with moving readers (e.g., intermediate UE). Ideally option 2 based on measurements should also be studied, and even for option 1, the criteria in terms of number of successful receptions for proximity determination should be discussed. Therefore, we propose to modify the proposal as follows: For proximity determination, option 1 is considered as the baseline, i.e. based on the reader receiving D2R transmission(s) from the device in response to R2D transmission(s) - Successful reception/decoding at the reader is considered - FFS: relationship between number of successful reception(s) and proximity - FFS: whether/how the transmit power of the device is considered |  |
| FUTUREWEI | ok |  |
| Spreadtrum |  | It should be clarified that whether we need to obtain the rough distance value between device and reader. If yes, it’s hard to determine the rough distance values just based on the successful reception/decoding at the reader. So we slightly prefer option 2 that measurement-based method. For option 2, whether/how the transmit power of the device and CW should be further discussed. The case of transmit reader and receive reader are different should also be further discussed if scenario A1 is observed as one of candidate scenarios in 9.4.1.1 for A-IoT deployment. |
| Honor | Ok with the proposal. |  |
| Panasonic | We are generally fine with the proposal. However, we should not restrict the proximity determination to successful detection/decoding the response message. For instance, the reader may identify the existence of more than one device in proximity by detecting the message collisions (e.g., in RACH procedure). The reader may further trigger devices for individual device detection, if needed. So, we suggest revising the proposal as follows: - Successful reception/decoding at the reader is at least considered. |  |
| LG Electronics | OK |  |
| Ericsson | Support the proposal. We are fine with Oppo’s revision. |  |
| InterDigital |  | Ok to take Option 1 as baseline, but FFS should not focus only on Tx power, i.e. Rx power from device should be left open as possible additional factor. |
| Samsung | Support. Option 2 should be discouraged due to the following reasons. Dependency on the topologies: The Option 2 may work for D1T1-A2, wherein a receiving R and the associated CW node are collocated such that the CW propagates the same round-trip paths, i.e., the CW2D path and the D2R path are identical. However, for D1T1-A1 and for D1T1-B, the signal propagation consists of two different paths of CW2D and D2R, which can be quite dissimilar. As an example, one cannot distinguish between the two cases: - Case A) CW node is next to D, while receiving R is quite distant from D, i.e., D2R distance >> CW2D distance - Case B) Receiving R is next to D, while CW node is quite distant from D, i.e., CW2D distance >> D2R distance For both cases, the measurement results at a reader may be similar, while for Case A, one should conclude that the device is not in proximity and, while for Case B, the conclusion should be the opposite, i.e., the device is in proximity. Dependency on the device type: A device 1 transmits via backscattering without a capability of amplification. A device 2a transmits via backscattering with a capability of amplification. A device 2b transmits via internally generated signal using LO and an amplifier. For device 2a and 2b, there needs to be a separate control signal designed to align the amplification gain or the internally generated signal power between the reader and the device. To begin with, the basic assumption here is that the device has a capability to adjust the amplification gain or the internally generated signal power, which itself may be daunting for A-IoT devices. Dependency on other parameters: The backscattered signal power will be further dependent on the backscattering loss, which can be different from one vendor to another. From a vendor perspective, this backscattering loss is an implementation specific value, which is preferably not to be revealed or reported to the reader. |  |
| ZTE, Sanechips | General OK with this proposal. It needs further clarification whether the response pertains to any R2D transmission or to specific R2D transmissions. The following modification is proposed: For proximity determination, option 1 is considered as the baseline, i.e. if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near - Successful reception/decoding at the reader is considered - FFS: whether/how the transmit power of the device is considered - FFS: the type of R2D transmission |  |
| Qualcomm |  | Based on RANP discussion, there are two definitions of proximity determination: - Defined by the ability of the Ambient IoT device and the reader can carry out a communication session - Defined by the reader to identify whether a device is in a very close range (less than 1m) from the reader. Without clarification, it’s too early to down-select Option 1 and Option 2. |
| Moderator (Apple) | Majority companies seem fine to take option 1 as baseline (with some minor modifications). However, also a few companies would like clarification on proximity determination definition/use-case before down-selecting to one option. Therefore, for 2nd round, 2 related proposals are made |  |

[Closed] 2nd Discussion Round

(HP) Proposal 2.3-0

Proximity determination is defined by reader’s and device’s ability to carry out a communication session

- If the reader is able to communicate with the device, it is determined as near

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| LG Electronics |  | The FL proposal seems too general and so it can be changed to: Proximity determination is used to identify a non-identified device in proximity and/or check if an identified device is still in proximity for communication between a reader and the device. |
| Lenovo | Yes, after the 4 step RACH procedure and transmission of EPC ID, the reader can determine whether the device is near to it or not. The reader can apply any threshold. But to support to both monostatic and bistatic cases the second bullet needs revision - If the reader is able to communicate with the device, it is determined as near to that reader ( that Tx or Rx from the device) |  |
| OPPO |  | Not all devices that can be inventoried by the reader is in proximity in our view, otherwise proximity determination is completely the same as inventory. We support Option 1, but reader should be able to control the range where devices can send response, the range is supposed to be smaller than the coverage of the reader. |
| Samsung2 | Support |  |

(HP) Proposal 2.3-1A

For proximity determination, option 1 is considered as the baseline, i.e. if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

- Successful reception/decoding at the reader is considered

- FFS: whether/how the transmit power of the reader and/or device is considered

- FFS: Other considerations

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | Support |  |
| LG Electronics | OK |  |
| OPPO | OK |  |
| ZTE, Sanechips |  | The following is suggested for other considerations, like using a preamble without D2R data for for proximity determination to reduce resource overhead. For proximity determination, option 1 is considered as the baseline, i.e. if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near - Successful reception/decoding at the reader is considered - FFS: whether/how the transmit power of the reader and/or device is considered - FFS: Other considerations, e.g., D2R frame structure for proximity determination |
| Samsung2 | May not be needed. It’s an implementation issue. |  |

  - Topic 4: Intermediate UE considerations for topology 2

| Company | Observations/Proposals |
| --- | --- |
| Futurewei [1] | Observation 3: When an intermediate UE transmits an R2D transmission, the transmission power of the R2D transmission may impact gNB reception operation. Observation 4: An intermediate UE may be unable to perform Uu operations while performing A-IoT operations. Proposal 11: For Topology 2, support the operation of an intermediate UE where the intermediate UE is configured with a resource pool indicating frequency resources, power levels, and time when the resource pool is active. - FFS: whether/how the gNB can signal when the intermediate UE should perform R2D transmissions - FFS: whether/how the intermediate UE operates in autonomous mode |
| Spreadtrum [5] | Proposal 11: A new DCI format for A-IoT topology 2 can be studied. And at least the following contents should be included. - UL time-frequency domain resource indication - R2D time-frequency domain resource indication - D2R time-frequency domain resource indication - Device’s ID indication Proposal 12: Support UL response from intermediate node to BS to indicate inventory status or A-UL reception. |
| TCL [6] | Observation 9: The resource allocation for R2D/CW/D2R transmission and interference measurement has impact on NR UE transmission for AIoT topology 2. Proposal 14: Consider carefully how/when sends R2D/CW/D2R and measures interference (e.g., harmonic interference for coexistence system) for AIoT topology 2. |
| Lenovo [9] | Proposal 17: Study radio resource allocation mechanism for intermediate node in Topology 2. |
| China Telecom [10] | Proposal 10: Considering intermediate UE for topology 2, study the capability indication related issues, including the indication of whether current cell supports A-IoT and the indication of whether the intermediate UE supports A-IoT. Proposal 11: For intermediate UE for topology 2, study at least network controlled method to enable/disable it serving as an intermediate UE. Proposal 12: For topology 2, DL control information may be needed from gNB to intermediate UE and from intermediate UE to A-IoT devices. A new DCI format could be considered, the following content can be further studied: - Indicate whether the intermediate UE can autonomously communicate with A-IoT devices, e.g., whether the intermediate UE could communicate with A-IoT devices in RRC_ACTIVE or periodically. - Resource/time allocation for R2D or D2R transmission according to the reception status or feedback from the A-IoT devices. |
| Oppo [13] | Proposal 22: A new DCI format should be introduced to schedule the intermediate node to transmit control/data/signal and/or carrier wave to A-IoT devices. FFS whether some change to existing PUCCH is needed to support transferring of the feedback from A-IoT device to intermediate node. |
| Fujitsu [15] | Proposal 8: The gNB configures the intermedia UE via Uu interface. - At least the time-frequency resource allocation for R2D and D2R transmission is configured. Proposal 9: Considering low requirement on latency for A-IoT, feedback information from intermedia UE carried by PDSCH is the baseline in Rel-19 SI. |
| Continental [21] | Observation 3.1.: A UE with A-IoT capability needs to be designated as an intermediate UE for A-IoT by the gNB. Observation 3.2.: It is desirable to avoid frequent requests for resources from the intermediate UE(s) to the gNB for performing R2D or D2R communications with A-IoT devices to avoid signaling overheads. Observation 3.3.: It is desirable to have a logical separation between the resources used for A-IoT communications between intermediate UE(s) and A-IoT device(s) and the resources used for NR DL/UL communications between the intermediate UE(s) and gNB(s). Proposal 3.1.: The gNB transmits an indication to a UE with A-IoT capability to enable it to operate as an intermediate UE. This may be provided via RRC signaling. Proposal 3.2.: A separate resource pool may be allocated by a gNB to one or more intermediate UE(s) to communicate with A-IoT devices. This A-IoT resource pool may be allocated in-band to LTE/NR, in guard-band to LTE/NR or in standalone band(s). - The A-IoT resource pool configuration includes a set of time slots which may be periodically repeated with a specified time interval and a set of subchannels which may be periodically repeated over a specified frequency interval. - The A-IoT resource pool configuration may be provided by the gNB to the intermediate UE(s) via RRC signaling. |
| Honor [23] | Proposal 8 The BS can configure time domain, frequency domain, and CW resources for each intermediate UE through static/semi-static/dynamic methods, for in-band, guard-band, and standalone deployment. Proposal 9 Study sharing CG for different intermediate nodes to report UL data/results to the BS. |
| LGE [24] | Proposal 16: Study the following cases for frequency domain resource allocation: - Multiple channels for a single cell/gNB - Multiple channels for different INs under a single cell/gNB - Multiple channels for different cells/gNBs Proposal 17: Study both contiguous and incontiguous frequency channel deployment. Proposal 18: Study frequency channel coordination among gNBs/INs Proposal 19: Study a time domain resource pool for TX and/or RX of gNB and IN. Proposal 20: Study both dynamic resource allocation and semi-static resource allocation for gNB and IN. Proposal 21: Study the following resource types for R2D and D2R: - TX only resource (used for R2D) - TX/RX resource with or without CW transmission (used for D2R) Proposal 26: Study DCI based resource allocation for R2D resource with or without D2R resources e.g. similar to SL mode 1 Proposal 27: Study IN autonomous resource allocation for R2D resource with or without D2R resources e.g. similar to SL mode 2 Proposal 28: Introduce a new DCI format for dynamic R2D/D2R resource allocation Proposal 29: Agree that UEs functioning as IN should be in RRC_CONNECTED. |
| Samsung [25] | Proposal 15: Study controlling an intermediate UE for transmission or reception between the intermediate UE and the BS and also between the intermediate UE and one or more devices including - CW transmission (for D2T2-A scenarios) - Execution of a command received from a BS to device(s) - Transferring data between a BS and device(s) - Energy harvesting signal transmission, if different than CW or R2D signal. |
| Apple [28] | Proposal 20: For topology 2 with intermediate UE serving as the reader, study at least following aspects in terms of physical channel/signals: - Downlink control information (DCI) from network to the intermediate UE for transmitting R2D to device and/or receiving D2R from the device - Uplink control information (UCI), if any from the intermediate UE to the network Proposal 21: For topology 2 with intermediate UE, study at least following options for the selection of UE for serving as a reader to communicate with ambient IoT device(s): - Option 1: It is up to network implementation - Option 2: Study specification-based methods, e.g. based on proximity of number of ambient IoT devices to the UE |
| Qualcomm [31] | Proposal 13: For time/freq resource allocation of A-IoT communications - For Topology 1: BS configures R2D/D2R time/freq resources for A-IoT - BS/reader to control dynamic R2D/D2R within the configured time/freq resources. - For Topology 2: BS semi-statically or dynamically configures the time/freq resources per the UE/reader via Uu, at least for inband/guardband operation. - UE/reader to trigger R2D/D2R within the configured time/freq resources - FFS: how to solve collision among UEs/readers for a shared resource |
| IIT-K, IIT-M [35] | Proposal 8: RAN1 should clarify whether an intermediate node is allowed to schedule the AIoT D2R transmission or intermediate node performs the transmission and reception on the R2D and D2R link based on the triggers provided by the BS. Proposal 9: BS should allocate the time and frequency resources used by the intermediate node for supporting AIoT operations. Proposal 10: BS should control the transmit power of the intermediate node directed toward the AIoT device and the BS should control the transmit power of the CW node directed toward the AIoT device D2R transmission. Proposal 11: A new DCI format or higher layer signaling to support should be studied to support intermediate node operations. |

[Closed] 1st Discussion Round

14 companies provided their views related to intermediate UE in case of topology 2. One key aspect that all the companies discussed is how/what to signal the intermediate UE to serve as a reader to ambient IoT devices. As it is agreed that the intermediate is under network control, therefore, intermediate UE may need to be informed/signaled with related details for scheduling R2D and/or D2R. Furthermore, a few companies also discuss the need for UCI from intermediate UE to network. Additionally, resource allocation for the intermediate UE is also considered by companies. Based on this, following proposal is made

Proposal 2.4-1

RAN to study at DL/UL L1 control signaling between the network and intermediate UE for topology 2 including at least:

- Resource allocation for the intermediate UE including collision handling, if any

- Need for new DCI format and corresponding details

- UCI related to ambient IoT device(s)

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| TCL | Agree with this proposal. |  |
| Continental | In our view, the nature of the resource allocation to the intermediate UE needs to be clarified first before studying the L1 control signaling. Specifically, is the intermediate UE allowed to perform A-IoT R2D transmissions/D2R receptions autonomously within a resource given by the gNB or does it have to wait for a resource allocation from the gNB for each R2D transmission/D2R reception? In our view the former is preferred, so that the intermediate UE is granted a resource pool and within that resource pool it is free to perform R2D transmissions/D2R receptions with A-IoT devices. |  |
| FUTUREWEI | Ok but details are not needed in study. (i.e. remove “collision handling” in first subbullet and “corresponding details” in second subbullet) |  |
| OPPO | OK |  |
| Spreadtrum | OK |  |
| Honor | OK |  |
| Panasonic | We support the proposal. |  |
| LG Electronics | OK |  |
| Ericsson | We support the study of BS scheduling D2R/R2D resources for intermediate UE. At this moment, we would like to study semi-static scheduling in addition to dynamic scheduling. Is the intention of UCI an SR-like request from intermediate UE? |  |
| InterDigital | Yes. We think there is also need to support measurements/sounding by intermediate UE. |  |
| Huawei, HiSilicon |  | We doubt there is scope in the RAN1 objectives of the SID for an explosive study on the scale of physical layer changes contemplated by the proposal. It has to be much more tightly limited if it is to be achievable in rational time, and we need to understand why it is part of RAN1 work at all, rather than RAN2. Specifically, the RAN1-led objectives begin by saying “For the Ambient IoT DL and UL…”, and there is nothing in this proposal that addresses either of those links. |
| Qualcomm | Ok to study. But it may be better to consider RAN1 study aspects, rather than ‘RAN to study…’. For UCI, we prefer to make it more general to say ‘UCI for AIoT communications’. |  |
| Moderator (Apple) | @Huawei: I don’t think every proposal we discuss has corresponding specific objective in the SID. However, since we agree to support topology 2, that requires some information exchange between network and intermediate UE. Of course, the intention of this proposal is not to agree right away, but rather study the feasibility. Also, just to clarify, this is different from carrier wave. This is purely focusing on intermediate UE for serving ambient IoT devices Proposal updated based on Qualcomm’s suggestion for 2nd round |  |

[Open] 2nd Discussion Round

Proposal 2.4-1A

RAN study aspects related DL/UL L1 control signaling between the network and intermediate UE for topology 2 including at least:

- Resource allocation for the intermediate UE including collision handling, if any

- Need for new DCI format and corresponding details

- UCI related to ambient IoT device(s)

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| Docomo |  | Support to study control signaling related to intermediate UE. But we think the control signaling may not be limited to L1. |
| TCL | Support |  |
| LG Electronics | OK |  |
| Huawei, HiSilicon |  | We would first need to focus on the completion of the A-IoT air interface before we discuss this, since otherwise it is not known what might or not be relevant to another interface. In general, it would be more than enough to simply list out the aspects that are not handled in other WGs, without massive effort to solve everything now: we also need to be mindful of aspects that can be handled by RAN2. Details such as the new DCI and exact control signaling can be treated with lower priority currently. |
| Spreadtrum | Yes |  |
| FUTUREWEI |  | We suggest removing “including collision handling, if any” and “and corresponding details” from the first two bullets. |
| OPPO | Support the proposal, this aspect should be studied as it is clearly indicated in the SID. - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control Typo in main bullet: “RAN”->”RAN1”. |  |

[Open] 3rd Discussion Round

(HP) Proposal 2.4-1B

RAN1 study aspects related DL/UL control signaling between the network and intermediate UE for topology 2 including at least:

- Resource allocation for the intermediate UE including collision handling, if any and corresponding details

- Need for new DCI format and corresponding details

- UCI related to ambient IoT device(s)

# Proposals for offline sessions

  - 1st offline session (Wednesday, August 21)

Proposal 2.1.1-4

For R2D, no other parts (other than start-indicator part and clock-acquisition part) are further considered for the R2D timing acquisition signal

Note: this doesn’t preclude potentially additional functionalities with the clock-acquisition part

Proposal 2.2.1-1B

For D2R transmissions, preamble preceding the PDRCH can be considered also for the purpose of:

- SFO estimation

- Channel estimation

FFS: Other functionalities, if any

Proposal 2.2.3-2B

For PDRCH generation, coding block from the previous agreement (from RAN1#116bis) is updated as follows:

PDRCH generation

Note: Additional blocks and/or updates (including potential change of sequence of blocks) to existing blocks can be further done, depending on progress in agenda 9.4.2.1

Proposal 2.1.1-1A

For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: Exact sequence/length of the ON/OFF pattern.

Proposal 2.1.1-2B

- For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration for at least the L1 R2D control information (if any) transmitted in the subsequent PRDCH for at least the case when PRDCH transmits only data

- For determining the chip duration for the data transmitted in the subsequent PRDCH, following two options are studied:

  - Option 1: Clock-acquisition part of the R2D time acquisition signal is used

  - Option 2: R2D control information is used

- FFS: Whether the R2D clock-acquisition part can be used as a reference to determine the chip duration of the subsequent PDRCH

Proposal 2.3-0A

Proximity determination is defined by reader’s and device’s ability to carry out a communication session

- If the reader is able to communicate with the device, it is determined as near

Proximity determination is defined as reader’s ability to identify a (non-identified) device and/or determine if an identified device is still near for communication between a reader and the device.

Proposal 2.1.3-2B

For PRDCH transmitting both L1 R2D control information (if any) and data, any L1 R2D control information (if any) is placed at the start of the PRDCH.

Note: Need for L1 R2D control information and its content are discussed separately

  - 2nd offline session (Thursday, August 22)

(HP) Updated Proposal 2.1.1-1C

For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: length/pattern of ON/OFF

- FFS: Whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| LG Electronics |  | The benefit of the variable ON part is not clear to us. We prefer to have a fixed length for ON part as well. For this meeting, the first bullet point of the proposal can be removed and the following FFS can be added for the next meetings: - Duration of the ON part can be variable - Duration of the OFF part is fixed - FFS: Duration of the ON part is fixed or variable |
| Huawei, HiSilicon | We would like to bring back the FFS that was discussed in yesterday’s online session. FFS whether the ON duration is included in TD2R_min The reason is that we are still concerned about the latency issue. Having a fixed ON part and an OFF part, where they are both longer than the chip lengths used in the PRDCH transmission such that the device is able to differentiate this, would require a longer latency as compared to a short ON duration with a minimum time duration. Hence, we would like to also study whether the ON duration could be included in the TD2R_min to minimize the latency impact on the longer start indicator. |  |

(HP) Proposal 2.1.1-2B

- For R2D, the clock-acquisition part of the R2D time acquisition signal is at least used to determine the OOK chip duration for the immediately following PRDCH

  - FFS: Whether the R2D clock-acquisition part can also be used as a reference to determine the chip duration of the subsequent PDRCH

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| LG Electronics | OK |  |

(HP) Updated Proposal 2.1.1-1A

For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH:

- Time domain resources

- Frequency domain resources

- MCS

- Chip duration

- ID associated with device

- Repetitions

FFS: other information

FFS: For each information, whether higher-layer signaling or L1 R2D control signaling is used

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| LG Electronics |  | Repetitions seem not always indicated. Or, we can clarify repetition is indicated only for scheduling D2R repetitions. Device ID seems not always indicated e.g. when paging triggers multiple D2R transmissions. Or, we can clarify device ID is indicated only for scheduling D2R transmission from one device |

(HP) Proposal 2.1.1-2A

For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH:

- ID associated with device

- FFS: other information

FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| LG Electronics |  | Device ID seems not always indicated e.g. for non-unicast R2D transmission such as paging/Query. |

(HP) Updated Proposal 2.4-1B

RAN1 study at least following aspects (including their necessity) related to DL/UL signaling between the network and intermediate UE for topology 2 related to ambient IoT device(s):

- Resource allocation for the intermediate UE

- DCI enhancements, if any

- UCI enhancements, if any

| Company | Do you support the proposal in principle? If there is any suggestion for editorial update, please provide here | If you don’t support the proposal and have technical concern on the proposal, please elaborate and suggest alternative proposal |
| --- | --- | --- |
| LG Electronics | OK |  |

# Proposals for online session

  - 1st online session (Monday, August 19)

Proposal 2.1.3-2

For PRDCH transmitting both L1 R2D control information and data, any L1 R2D control information is placed at the start of the PRDCH.

Proposal 2.2.1-1

For D2R transmissions, a start-indicator part is not considered for preamble preceding the PDRCH

Proposal 2.1.2-1A

For R2D, maximum TB size of no more than 1000 bits is considered

Proposal 2.1.1-1A

For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: Exact sequence/length of the ON/OFF pattern.

Proposal 2.1.3-1

For PRDCH generation, study following two alternatives for CRC attachment when both L1 R2D control information (if any) and data are transmitted on the same PRDCH

- Alt.1: Single/joint CRC attachment to L1 R2D control information (if any) and data

- Alt.2: Separate CRC attachment to each of L1 R2D control information (if any) and data

Proposal 2.2.3-2

For PDRCH generation, coding block from the previous agreement (from RAN1#116bis) is updated as follows:

PDRCH generation

Proposal 2.2.1-1

For D2R transmissions, preamble preceding the PDRCH can be considered also for the purpose of:

- SFO estimation

- Channel estimation

- Other functionalities are not precluded

  - 2nd online session (Wednesday, August 21)

Proposal 2.2.1-1B

For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities:

- SFO estimation

- CFO estimation

- Channel estimation

FFS: Other functionalities, if any

Proposal 2.1.1-4B

For R2D, no other parts (other than start-indicator part and clock-acquisition part) are further considered for the R2D timing acquisition signal

- clock-acquisition part is studied also for frequency synchronization

Proposal 2.3-1B

Either take Version1 or Version 2

Version 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission

- then the device is determined as near to the reader based on measurements at the reader side

- FFS: whether/how the transmit power of the reader and/or device is considered

Version 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near

- Note: this implies that there may not be any specification impact

Proposal 2.1.1-1B

For the start-indicator part of the R2D time acquisition signal, a fixed length ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: whether the ON part could be included in start-indicator part or could be included in the gap preceding the start-indicator part when there is no R2D or D2R

Proposal 2.1.3-2B

For PRDCH transmitting both L1 R2D control information (if any) and data, any L1 R2D control information (if any) is placed at the start of the PRDCH.

Note: Need for L1 R2D control information and its content are discussed separately

Proposal 2.1.1-1A

For D2R scheduling, at least following information is indicated to the device via R2D:

- Time domain resources

- MCS/coding rate

- Chip duration

- Device ID

- Repetitions

FFS: For each information, whether higher-layer signaling or L1 R2D control signaling is used

  - 3rd online session (Friday, August 23)

##### Proposal 2.1.1-1D

For the start-indicator part of the R2D time acquisition signal, ON/OFF pattern i.e. high/low voltage transmission is applied

- FFS: length/pattern of ON/OFF

- FFS: Whether/how the start-indicator part is included in TD2R_min or not. To be discussed in 9.4.2.2

##### Proposal 2.1.1-1B

For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH:

- Time domain resources

- Frequency domain resources

- MCS

- Chip duration

- ID associated with device

- Repetitions

FFS: other information

FFS: For each information, whether higher-layer signaling or L1 R2D control signaling is used

##### Proposal 2.1.1-2B

For R2D scheduling, the following information potentially can be explicitly/implicitly indicated to the device via PRDCH:

- ID associated with device

- FFS: other information

FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

##### Proposal 2.4-1C

RAN1 study at least following aspects (including their necessity) related to DL/UL signaling between the network and intermediate UE for topology 2 related to ambient IoT device(s):

- Resource allocation for the intermediate UE

- DCI enhancements, if any

- UCI enhancements, if any

# Contributions in RAN1#118

| [1] [1]s [1] | R1-2405804 | D2R and R2D Channel/Signal Aspects for Ambient IoT | FUTUREWEI |
| --- | --- | --- | --- |
| [2] | R1-2405822 | R2D and D2R channel/signal aspects for Ambient IoT | Nokia |
| [3] | R1-2405828 | Downlink and uplink channel/signal aspects for Ambient IoT | Ericsson |
| [4] | R1-2405854 | Physical channels and signals for Ambient IoT | Huawei, HiSilicon |
| [5] | R1-2405914 | Discussion on downlink and uplink channel/signal aspects for Ambient IoT | Spreadtrum Communications |
| [6] | R1-2405969 | Discussion on downlink and uplink channel/signal aspects for Ambient IoT | TCL |
| [7] | R1-2405991 | Discussion on downlink and uplink channel/signal aspects | CMCC |
| [8] | R1-2406012 | Discussions on physical channel and signals for A-IoT | Intel Corporation |
| [9] | R1-2406072 | Discussion on channel/signal aspects for Ambient IoT | Lenovo |
| [10] | R1-2406093 | Discussion on downlink and uplink channel/signal aspects for Ambient IoT | China Telecom |
| [11] | R1-2406143 | Considerations for downlink and uplink channel/signal aspects | Semtech Neuchatel SA |
| [12] | R1-2406188 | Discussion on Downlink and uplink channel/signal aspects | vivo |
| [13] | R1-2406244 | Discussion on downlink and uplink channel/signal aspects for A-IoT | OPPO |
| [14] | R1-2406290 | Discussion on downlink and uplink channel and signal aspects for Ambient IoT | Xiaomi |
| [15] | R1-2406317 | Discussion on downlink and uplink channel/signal aspects | Fujitsu |
| [16] | R1-2406374 | DL and UL Physical Channels/signals design in support of Ambient IoT devices | CATT |
| [17] | R1-2406407 | Discussion on channel and signal for Ambient IoT | ZTE Corporation, Sanechips |
| [18] | R1-2406429 | Discussion on downlink and uplink channels and signals for A-IoT | Panasonic |
| [19] | R1-2406457 | Downlink and uplink channels aspects of Ambient IoT | InterDigital, Inc. |
| [20] | R1-2406476 | Downlink and uplink channel / signal aspects for Ambient IoT | Sony |
| [21] | R1-2406505 | Considerations on Control Information, Proximity Determination, and Intermediate UE for A-IoT | Continental Automotive |
| [22] | R1-2406559 | Discussion on downlink and uplink channel for ambient IoT | NEC |
| [23] | R1-2406581 | Discussion on downlink and uplink channel/signal aspects for A-IoT | HONOR |
| [24] | R1-2406606 | Downlink and uplink channel/signal aspects for Ambient IoT | LG Electronics |
| [25] | R1-2406656 | Considerations for downlink and uplink channel/signal aspect | Samsung |
| [26] | R1-2406730 | Discussion on downlink and uplink channel/signal aspects for A-IoT | ETRI |
| [27] | R1-2406775 | Downlink and uplink channel/signal aspects | MediaTek Inc. |
| [28] | R1-2406842 | On physical channels/signals and proximity determination for AIoT | Apple |
| [32] | R1-2406880 | Discussion on downlink and uplink channel/signal aspects | Sharp |
| [33] | R1-2406936 | Study on downlink and uplink channel/signal aspects for Ambient IoT | NTT DOCOMO, INC. |
| [34] | R1-2407035 | Downlink and uplink channel/signal aspects | Qualcomm Incorporated |
| [35] | R1-2407071 | Discussion on downlink and uplink channel aspects for A-IoT | China Unicom |
| [36] | R1-2407090 | Discussion on Downlink and Uplink channel/signal aspects | CEWiT |

# Appendix

  - RAN1 Scope & Objectives (Revised SID in RP-240826)

General Scope

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X  is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

- For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

- Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum.

The following objectives are set, within the General Scope:

- Evaluation assumptions

- Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1].

  - Clause 5.3: Applicable maximum distance target values(s)

  - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs

  - Clause 5.8: 2D distribution of devices

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4]

- Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1]

- Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology.

NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.

NOTE: strive to minimize evaluation cases in RAN1.

- Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848.

Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation).

Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

- RAN1-led:

For the Ambient IoT DL and UL:

  - Frame structure, synchronization and timing, random access

  - Numerologies, bandwidths, and multiple access

  - Waveforms and modulations

  - Channel coding

  - Downlink channel/signal aspects

  - Uplink channel/signal aspects

  - Scheduling and timing relationships

  - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation.

For Topology 2, no difference in physical layer design from Topology 1.

  - RAN1 Agreements

### RAN1#116 (Athens, Greece, February 26th – March 1st, 2024)

Agreement

For ambient IoT devices, a dedicated physical broadcast channel for R2D, e.g. PBCH-like, is not considered for study.

Agreement

For ambient IoT devices, at least for R2D data transmission, a physical channel (PR2DCH) is studied,

  - System information (if defined) is transmitted on the PR2DCH

  - FFS Whether/how control information is transmitted on the PR2DCH

- Note: the naming of PR2DCH is used for the sake of the study

Agreement

For ambient IoT devices, at least for D2R data transmission, a physical channel (PDRCH) is studied along with the following,

- Response transmitted from device to reader during contention-based access procedure is transmitted on the PDRCH

  - FFS: Details of response

- FFS Whether/how/what D2R control information (if defined) is transmitted on the PDRCH

- Note: the naming of PDRCH is used for the sake of the study

### RAN1#116bis (Changsha, Hunan Province, China, April 15th – April 19th, 2024)

Agreement

For the R2D timing acquisition signal immediately preceding the transmission of a physical channel, study a preamble with at least two parts which includes a start-indicator part and a clock-acquisition part, where the start-indicator part immediately precedes the clock-acquisition part:

- Start-indicator part provides the start of the R2D transmission

  - FFS: Details of start-indicator part

- Clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission

  - FFS: Details of clock-acquisition part, e.g. structure, encoding, length, etc.

  - FFS: Methods to determine chip duration of the subsequent physical channel transmission

  - FFS: Other functionalities

- Note: the preamble is considered not to be part of a physical channel

- FFS: other part(s) of the preamble, if any

- FFS: whether the above clock acquisition is sufficient for all devices

- FFS: how to make the preamble compact

Agreement

For D2R, a preamble preceding each PDRCH transmission is studied as the baseline at least for the D2R timing acquisition signal:

- Preamble is not part of PDRCH

- FFS: Other functionalities of the preamble

Agreement

For PRDCH generation at the reader, at least following blocks are studied as the baseline:

- CRC bits are appended if there is non-zero length CRC

  - Note: CRC details discussed in agenda item 9.4.2.1

- Line coding block

- OOK-1/OOK-4 modulation with OFDM waveform generation, including resource mapping

  - FFS details

- Note: Other blocks could be added if agreed

PRDCH generation

Agreement

For PDRCH generation at the device, at least following blocks are studied as the baseline:

- CRC bits are appended if there is non-zero length CRC

  - Note: CRC details discussed in agenda item 9.4.2.1

- Coding

  - Exact coding methods within the coding block, e.g. with/without line coding and/or FEC discussed under agenda 9.4.2.1

  - Note: If no line coding is used, there may be an additional block (e.g. square wave generator) before/after modulation block

- Modulation

- Note: Other blocks could be added if agreed

PDRCH generation

Agreement

Reference signals including at least DMRS, PTRS, CSI-RS/TRS, are not further studied for R2D.

Agreement

Reference signals including DMRS, PTRS, SRS, are not further studied for D2R

- Note: This doesn’t preclude the possibility to study preamble, midamble, postamble for different purposes, e.g. channel/interference estimation and/or proximity determination

Agreement

Proximity determination based on device side measurements is not considered.

### RAN1#117 (Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024)

Agreement

For R2D, the only physical channel is PRDCH.

- PRDCH carries any higher-layer payload

- PRDCH carries L1 R2D control information (if defined)

- FFS details of device behaviour(s) for receiving PRDCH

Agreement

For D2R

- PDRCH carries any higher-layer payload

- PDRCH carries L1 D2R control information (if defined)

- Note: PDRCH carries the response agreed at RAN1#116

Agreement

For L1 D2R control information (if defined), the following are not considered for further study:

- CSI feedback

- Autonomous SR

- FFS: Whether any other L1 D2R control information is needed or not

Agreement

Study the following schemes for proximity determination:

- Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

    - FFS: Details on reception criteria (e.g. either successful or not) at reader and device

- Option 2: Device is determined to be near the reader based on measurements at the reader side

    - FFS: Details on measurement methods

- FFS: Whether/how transmit power of R2D and/or D2R is considered for proximity determination

Agreement

For the start-indicator part of the R2D time acquisition signal, study the two options below:

  - Option 1: ON/OFF pattern i.e. high/low voltage transmission

  - Option 2: OFF pattern, i.e. low voltage transmission

Agreement

For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration

FFS: Pattern design to support determination of chip duration

Agreement

Proximity determination based on device side measurements is not considered.

### RAN1#118 (Maastricht, NL, August 19th – 23rd,  2024)

Agreement

For each D2R transmission, no separate part for start-indicator is considered for the preamble preceding the PDRCH.

Agreement

For D2R transmission, preamble preceding the PDRCH is studied also for the potential additional functionalities:

- SFO estimation

- CFO estimation

- Channel estimation

- Interference estimation

Note: this does not preclude studying the above functionalities by using a midamble and/or postamble, if supported

FFS: Other functionalities, if any

Conclusion

Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission

- then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader