3GPP TSG RAN WG1 #118		             			           	R1-2406874

Maastricht, NL, August 19th –23rd, 2024

Agenda Item: 		9.7.1

Source: 		Moderator (AT&T)

Title: 	FL summary #2 on ISAC deployment scenarios

Document for: 	Discussion and Decision

# Introduction

This document provides a summary of the discussion of the ISAC deployment scenarios, which is within the objectives in the Rel-19 recently approved SID (subsequently revised at RAN#103) on the “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR” [1] as per the objectives shown below:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

Additionally, at RAN1#117, some good progress was made and the following agreements were endorsed in the ISAC session report [2] regarding general ISAC channel modeling aspects as well as additional progress on the specific evaluation parameters for the UAV sensing scenarios.

Agreement

For each of the sensing target deployment scenarios using the template agreed in RAN#116-bis, the following principles apply:

- For defining sensing Tx and sensing Rx properties (e.g., cell layout, BS antenna height, and minimum distance), scenario parameter values for the applicable communication scenarios in 38.901 are considered as a starting point. Updates to these evaluation parameter values for ISAC scenarios will consider the following:

  - aerial UEs parameter values as defined in TR 36.777

  - indoor room scenario parameter values defined in TR 38.808

  - automotive scenario parameter values as defined in TR 37.885, 38.859 for Urban grid/Highway

  - Minimum distances between Tx/Rx and target are not defined in the existing communications scenarios and shall be included in the sensing target deployment scenarios.

  - Note: Only deviation from the existing evaluation parameters in the applicable communication scenarios need to be explicitly defined in the ISAC scenario tables.

- For defining sensing target properties, as a baseline

- Evaluation parameter values can be taken from additional TRs, e.g., TR 36.777, 37.885, 38.859, etc.

- Size of sensing targets based on TR 22.837, 37.885 (e.g. for automotive), 38.901 (e.g. for AGV size), etc

Agreement

For ISAC deployment scenarios, carrier frequency, bandwidth, and SCS are not included in the evaluation parameters templates for sensing scenarios, but may be included in the evaluation/calibration phase.

Agreement

For UAV sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note1: this may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. [In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.] |
| Supported sensing modes |  |  | [All 6 sensing modes] |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: Up to 160 km/h [FFS specific velocity(ies) or random distribution] [FFS vertical plane velocity] |
|  |  | 3D distribution | [Uniform between a minimum and maximum height] [Uniform in horizontal domain at a given height] |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | UAV object type(s) [FFS] |
| [Sensing area] |  |  |  |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  | FFS |
| Minimum 3D distance between sensing targets |  |  | FFS |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS |

Agreement

RAN1 agrees to the following revised evaluation parameters values for the UAV sensing target scenarios:

| Parameters | Value |
| --- | --- |
| Sensing transmitters and receivers properties | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note 1: Other Rx/Tx properties (e.g. mobility) can also be taken from the corresponding communication scenario. Note 2: This may include aerial UEs as Rx/Tx that can be selected among locations in the UMi-AV, UMa-AV, RMa-AV communication scenarios. |

## Moderator Tentative Plan

For RAN#118, RAN1 should take into account the above agreements as a starting point and further progress the ISAC deployment scenarios following tentative plan:

- With the agreement of a baseline template at RAN#116-bis and further agreements in RAN#117, RAN1 should discuss and make progress on the following:

  - Solidify high level principles for what needs to be captured in the ISAC deployment scenarios versus the evaluation/calibration phase.

  - Discuss and agree scenario parameters and/or parameter categories for the various sensing targets

  - Specific values/value ranges should be further discussed and resolved

- The goal for completion of detailed of deployment scenarios was initially set for the last meeting (RAN1#117). However, given the dependencies on the progress of the ISAC channel modelling sub-agenda item (9.7.2) this work is extended further in the Study Item.

The following summary is prepared based on the contributions submitted to the current meeting ([3-39]) and leveraging the previously endorsed proposals as a starting point.

Hence, the discussion will focus on the following aspects:

- Address 5GAA LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC

- Resolve square brackets and FFS evaluation parameters for the UAV sensing target scenarios

- Progress for ISAC deployment scenarios tables for the other sensing targets, e.g. automotive, humans, AGV, hazards.

- Discuss and agree evaluation parameters and associated values/ranges for each of the sensing targets/use cases

- Others and next steps

Please note that in this FL summary, a FL proposal may be designated as (H)(M)(L) to indicate its high, medium or low priority for online or offline discussions in this meeting. Nevertheless, we encourage all interested companies to provide feedback on all FL proposals. The FL may revise the priority of the proposals based on inputs from interested companies during the meeting, if it is deemed necessary.

Note that additional proposals are expected to be developed as online and offline discussions progress.

# Contact Information:

See sub-folder in the inbox for AI 9.7 for list of contacts for ISAC topics.

# Topic #1: 5GAA LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC

The related proposals are copied below.

| Company | Views |
| --- | --- |
| 5GAA WG4 [3] | 5GAA wants to share its early findings with 3GPP to be further considered in 3GPP analysis with respect to physical characteristics of sensing targets in ISAC deployment scenarios relevant to automotive scenarios. RAN1#116b established a template to define sensing deployment scenarios. The template includes physical characteristics of sensing targets (e.g., material and size). 5GAA wants to inform 3GPP about physical characteristics of the following sensing targets relevant for automotive scenarios: human outdoor, automotive vehicle and objects creating hazards on roads/railways. |
| OPPO [4] | Proposal 1: RAN1 discusses in AI9.7.2 the following options for handling of a sensing target composited by multiple objects with different sizes/materials and therefore different RCS impacts: - Option-1: Such sensing target is not assumed in Rel-19 ISAC channel model but can be supported in future release. - Option-2: A single sensing target of such kind is treated in Rel-19 ISAC channel model as one sensing target with a single effectively-combined RCS, where the effectively-combined RCS is to be obtained from measurements or left as a placeholder. - Option-3: A single sensing target of such kind is treated in Rel-19 ISAC channel model as multiple co-located sensing targets, where the correlations among these co-located targets for randomness in large/small scale fading generations may need special handling. Proposal 2: At least pothole and precipitation are not considered in RAN1 Rel-19 ISAC channel model study. |
| Ericsson [5] | Proposal 1 RAN1 can study a range of physical size of humans to include both adults and children. Proposal 2 Value range is preferred for other physical parameters, including velocity, which can be determined based on a typical value, such as typical velocity. Proposal 3 Do not consider precipitation as sensing target in Rel-19 ISAC channel modelling SI. |
| Qualcomm [39] | Proposal 4: Support all the objects types and the physical characteristics described in the LS R1-2405964. |

Moderator Discussion:

Based on the LS and associated tdocs, there seems to be some good information with regards to typical physical characteristics of automotive use cases and associated sensing targets. RAN1 can take into consideration the 5GAA details relevant for automotive scenarios: human outdoor, automotive vehicle and objects creating hazards on roads/railways that are within the current study item scope. Based on the associated tdocs there seems to be general consensus to follow the existing SI scope to exclude precipitation as a sensing target.

## Proposal 3-1 (FL1)

Proposal 3-1: (HIGH)

Based on the information in the 5GAA LS (R1-2405964), RAN1 should take into consideration the physical characteristics of automotive-related sensing targets.

- For evaluation parameters of deployment scenarios, consider e.g., typical sizes, velocities.

- For channel modelling, consider various target sizes, shapes and materials, and the associated RCS impacts.

NOTE1: No LS response is necessary based on the requested action(s) in the LS.

NOTE2: Physical characteristics of automotive-related sensing targets as defined in 3GPP TRs are also considered.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI | OK, but | For some parameters such as pedestrian (adult) size/speed, 5GAA LS and 3GPP TR (e.g. 22.837) are showing different values: - 5GAA LS: 0.3m x 0.36m x 1.8m, 3 km/h - TR 22.837: 0.5m x 0.5m x 1.75m, 5km/h We do not hold very strong views on either one. But we suggest following the same logic for all the cases, i.e. when difference appears, we always use the values in 5GAA, or current 3GPP TR. |
| ZTE | No | This proposal is not necessary. Based on the information in the 5GAA LS (R1-2405964), RAN1 should take into consideration the physical characteristics of automotive-related sensing targets that are within existing scope of the ISAC SI. - Size of automotive-related sensing targets in 5GAA LS is same as that defined in 37.885. There is no need to emphasize it again. - This proposal can be merged in tables for each target - The sub-bullets seems contradictory with Proposal 4-1, one is consider target size, the other is “Physical characteristics (e.g., size, material) of the sensing targets should be avoided until consensus is reached on whether/how to model these parameters.” |
| vivo | Yes with comment | We are fine with the 5GAA LS. There is only one thing needed to be emphasized that the typical velocity should not be a fixed value, rather the velocity should be a random variable with a certain distribution, e.g., uniform distribution with 0~3km/h, or 0 or 3km/h with the same or different probability. This is because unlike communication channel, only with a random variable for velocity, from the detection perspective, the hypothesis detection becomes meaningful. |
| Xiaomi | Comment | In 5GAA LS, the velocity configuration which is 16.67 m/s (60 km/h) for all roads is unreasonable at least. In order to model the realistic case, different velocity value should be configurated for different lanes. We suggest to reuse the mobility modeling option-B in 37.885 section 6.1.2. |
| Toyota | Yes |  |
| Panasonic | Comment | We are fine to consider automotive-related sensing targets from 5GAA LS in scope of ISAC because it is one of use case. Regarding velocity configuration, we share similar views with other companies to have variable speed based on a certain distribution, instead of using a fixed speed of 60 km/h as 5GAA LS. |
| InterDigital | Yes |  |
| Qualcomm | comment | We believe that not all companies have the same understanding on “that are within existing scope of the ISAC SI”. From our side, all the objects that are presented in the LS are in scope. The LS also talks about environment objects that create hazards. These also need to be included in the proposal |
| SONY | Yes | We agree to consider the input from 5GAA. However, if there are some deviations (e.g., as pointed by CATT) we need to discuss which one to be used and provide the LS response to 5GAA. |
| Samsung |  | We are fine with this proposal. Although LS consider the automotive relevant physical characteristics for object, all objects’ properties (various target sizes, shapes and materials, and the associated RCS impacts) need to be considered. |
| Lenovo |  | We are fine with taking the recommendations of 5GAA in general, but the target physical properties can be bette determined and modelled in the channel modelling work and then taken from AI 9.7.2 (as currently stated by Proposal 4-1). |
| Ericsson | Yes | A value range for a physical characteristic is preferred than multiple individual numbers. |

## Proposal 3-2 (FL2) – FOR ONLINE DISCUSSION

Proposal 3-2 (MED)

Per the 5GAA LS (R1-2405964), RAN1 should consider the recommendations for the physical characteristics (e.g., sizes, shapes, materials, velocities, etc.) of sensing targets and objects, along with the baseline characteristics defined in 3GPP TRs.

- No LS response from RAN1 to 5GAA is necessary.

- R1-2405964 is proposed to be NOTED.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Comment | For sensing target properties, we have already agreed that the evaluation parameters can be taken from additional TR as baseline. But there are some differences between 5GAA LS and current 3GPP TR for same target, for example, physical characteristics of human: - 5GAA LS: 0.3m x 0.36m x 1.8m, 3 km/h - TR 22.837: 0.5m x 0.5m x 1.75m, 5km/h RAN1 should clarify which parameter should be taken when differences appear. We suggest to consider 3GPP TR with high priority. |
| ZTE |  | We share similar understanding with xiaomi, physical characteristics defined in 3GPP should be taken and that defined in 5GAA can only be optionally considered if necessary . |
| New H3C |  | We share the similar view with ZTE and Xiaomi |

# Topic #2: General principles for ISAC Deployment scenarios

The related proposals are copied below.

| Company | Views |
| --- | --- |
| Spreadtrum [6] | Only certain sensing modes are defined for each of the sensing target scenarios |
| CMCC [7] | Proposal 3: Consider the requirements on detection or tracking of UAVs in Table 3 as baseline design targets in future evaluations. Proposal 6: Study the ISAC channel modeling for both FR1 and FR2. Proposal 7: Both mono-static and bi-static modes can be considered for ISAC channel modeling. |
| Intel [8] | Proposal 1: RAN1 to take as starting point the following additional parameters related to physical environment components and their values from applicable communication scenarios in applicable existing TRs for emulating target use cases: • Room dimensions • Ceiling height/building height • Street width • Effective clutter height • Wall and ceiling material |
| KPN [11] | Proposal 4: No specification needed at this point. Result from companies’ simulations should determine the minimum 3D distance between sensing targets in which the targets are distinguishable from each other. |
| Nokia [14] | Proposal 1: Replace references to “Unintended/Environment objects” in tables capturing evaluation assumptions and elsewhere with “environmental objects (EO), to align terminology across modeling methodology and deployment scenario discussions. Proposal 4: Discussions on evaluation assumptions for physical characteristics of sensing targets should be avoided until consensus is reached on whether/how to model these parameters. Proposal 5: Discussions on evaluation assumptions for unintended/environment objects should be avoided until consensus is reached on whether/how to model these parameters. |
| IDCC [16] | Proposal 1: Define the minimum distance between the Tx/Rx and the target as a function of the Rayleigh distance. Proposal 2: Study whether the minimum distance between the Tx/Rx and the target should be based on the size of the sensing target. Proposal 3: For outdoor deployment, the sensing area is defined as a function of both sensing Tx and sensing Rx locations. Proposal 4: For indoor sensing, the sensing area is defined as either the entire indoor space or the convex hull determined by the locations of the sensing transmitters within the respective deployment scenario. Proposal 5: For both indoor and outdoor scenarios, the sensing area is defined as a function of the minimum distance between the Tx/Rx and the sensing target. Proposal 6: The deployment distribution parameters for sensing targets and unintended/environmental objects should be based on the defined sensing area. Proposal 7: Identify EO (type-1) for each deployment scenario. Proposal 8: Incorporate the micro-Doppler characteristics of the sensing targets and EO type-1 for different deployment scenarios. |
| Xiaomi [18] | Proposal 2: Selecting all TRPs and UEs in corresponding communication scenarios as sensing Tx/Rx can be considered as start point. Proposal 3: RAN1 should consider to limit sensing modes for a specific type of target or ISAC deployment scenario. Proposal 4: Except for the target related parameters agreed at last meeting, target number should be defined additional. Proposal 5: To generate the orientation of sensing target, For vehicles, the orientation aligns to the velocity direction of target For others, the orientation is randomly generated. Proposal 6: Target size should be included in physical characteristics of target at least. |
| CATT [19] | Proposal 1: Take the table of evaluation parameters for UAV sensing scenario as the starting point for other scenarios, and modify parameters accordingly when necessary. Proposal 2: For ISAC channel modeling, only allow reasonable complexity increment compared to communication channel. The following assumption should be avoided, unless clear benefit and necessity are identified: - Time varying target speed; - Target horizontal speed of 0 km/h; - Movement in vertical plane (for UAV target); - Non-uniform distribution. Proposal 3: A clear principle would be helpful for future progress when different characteristics for the same sensing target appear in different TRs or other materials. - E.g., RAN1 may always prioritize those already in RAN1 TR/spec. Proposal 4: Not all the scenarios in the table agreed in RAN1#116 have to be captured in TR 38.901. Whether the ISAC channel model of a specific scenario will be captured in TR 38.901 depends on the matureness of the study at the end of Rel-19. |
| CATT [19] | Proposal 11: For the purpose of evaluation in the future, if down-selection on deployment scenario is needed, consider the prioritized scenario for each use case/sensing target in Table 7. Table 7 Prioritized scenario for each use case for evaluation. Sensing target; Prioritized scenario. UAV; UMa-AV. Humans; Outdoors; UMa. Indoors; One of InF/Indoor-Office. Automotive vehicles; Highway. AGV; InF. Objects creating hazards; Highway. |
| LG [20] | Proposal 1: All 6 sensing modes should be considered for the evaluation of every sensing target in the associated evaluation scenario. Proposal 2: Minimum distance between Tx/Rx and sensing target should be larger than the min. far-field distance of the sensing Tx/Rx from the sensing target. Proposal 3: Only the relevant type of sensing target is deployed in the associated evaluation scenario. Proposal 4: Multiple sensing targets of a same type are deployed in the associated evaluation scenario. Proposal 5: The sensing targets are deployed at random positions in the associated evaluation scenario. |
| Sony [21] | Proposal 1: Prioritize sensing target: UAVs, Automotive vehicles, and Automated guided vehicles (AGVs). The other sensing targets can be developed later (e.g., based on the progress of those selected sensing targets). Proposal 2: Reuse the UE and TRP antenna configuration as in the existing NR deployment scenario. Proposal 3: Down-prioritize/preclude the sensing area parameter in ISAC channel model evaluation table. |
| NVIDIA [22] | Proposal 1: Define a common reference scenario for ray tracing to be used in ISAC evaluation. Proposal 2: Select one the following option to define a common reference scenario for ray tracing to be used in ISAC evaluation: • Option 1: Real-scenario map that is a virtual representation of a real area on earth. • Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory. Proposal 3: Describe the scene geometry and the characteristics of the materials involved in the ISAC deployment scenarios to be used for ray tracing in ISAC evaluation. |
| Panasonic [23] |  |
| KRRI [24] | Proposal 1: Consider all the ISAC use cases and sensing modes defined for ISAC study without prioritization. Proposal 2: Consider both FR1 and FR2 bands for ISAC study. |
| Samsung [25] | Proposal 1 RAN1 consider unintended/environment objects modelling and specify the EO modeling type and the kind of EO for each sensing target Proposal 1 RAN1 does not consider sensing area separately in the channel modeling. |
| Lenovo [26] | Proposal 1: The evaluation scenario should include a description of the sensing area, i.e., the area where the network is expected to sense an object type as a sensing target. Proposal 2: Further discuss the parameters of the unintended/environment objects once consensus is reached in the channel modeling work under AI 9.7.2 regarding the handling of different EO types. |
| MediaTek [27] | Proposal 1: Considering the scope and meeting progress, it is better not to add the sensing area as an evaluation parameter, and the cell layout can be explicitly indicated if needed. Proposal 2: Regarding the supported sensing mode in the evaluation parameter, confirming all six sensing modes are supported. Proposal 3: whether the “unintended/Environment objects” as an evaluation parameter can be decided until the EO modelling discussion in AI 9.7.2 has the clear conclusion. |
| TOYOTA [28] | Proposal 1: RAN1 to define ISAC channel modelling that is flexible enough to support all of the six sensing modes from the SID (i.e., TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Proposal 2: RAN1 to consider the following object types: - Object type 1: UAV - Object type 2: Human, e.g. pedestrian, cyclist, motorcyclist, e-scooter - Object type 3: AGV - Object type 4: Automotive Vehicle, e.g., cars, trucks/bus - Object type 5: Animals - Object type 6: Static obstacles, e.g., tire, wheel, cargo, rock, fallen tree |
| Apple [29] | P1: The deployment scenarios should be defined in an abstract enough manner to cover multiple use cases. On the specific use cases to be modeled: • For UAV targets, at least use case on UAV flight trajectory tracing and use case on sensing for UAV intrusion detection (level 1 and 2) should be selected. • For human indoors and outdoors targets, at least the use cases on intruder detection in and surrounding a smart home and Use case on public safety search and rescue or apprehend should be selected. |
| CAICT [34] | Proposal 5: It is suggested to discuss the parameter settings for channel model calibration/evaluation as quickly as possible, e.g., carrier frequency, bandwidth, SCS, antenna pattern, antenna configuration, and transmit power of sensing transmitter. |
| Ericsson [35] | Proposal 4 Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] is no smaller than the Fraunhofer distance in order to guarantee the sensing target can be assumed in far field of sensing Tx/Rx. Proposal 5 Minimum 3D distance between sensing targets is not needed in ISAC channel model, namely minimum 3D distance between sensing targets = 0. Proposal 6 There is no need to explicitly parameterize sensing area. Proposal 8 Support time-varying mobility for all types of sensing targets. Proposal 9 Value ranges of velocity are more favorable than specific values to support different characteristics of the same type of sensing targets. Proposal 25 For UE-involved sensing modes, study channel modelling for the link between UEs and outdoor sensing targets for UMa, UMi, RMa in 38.901. Proposal 26 Study whether the BS-UE channel modelling for indoor scenarios in 38.901 can be reused for the link between indoor UEs (as sensing Tx/Rx) and indoor sensing targets. |
| NTT DoCoMo [36] | Proposal 1: RAN1 should strive to study the ISAC channel model for all six sensing modes and five use cases which are captured in SID objective. |
| Qualcomm [39] | Proposal 2: For ISAC use cases and sensing targets, RAN1 should consider all six sensing modes without an exception and without an explicit prioritization. Proposal 3: For all the sensing targets considered, both FR1 and FR2 is considered without an exception and without an explicit prioritization. Proposal 4: Support all the objects types and the physical characteristics described in the LS R1-2405964. |

Moderator Discussion:

A number of companies have made proposals on some general guiding principles. However, a majority of the contributing companies are supportive of the initial agreements from RAN1#116 and RAN1#117 to leverage template/baseline UAV sensing scenario for additional sensing target evaluation parameters and deployment scenario table(s) for each sensing target, with a few exceptions.

The moderator proposes that we continue to use the existing template for the other (other than UAV) sensing targets deployment scenarios. Some clarifications on EOs are needed, e.g., it may be that EO evaluation parameters will change depending on the communications scenario chosen. Finally, until we have more clarity on EO modelling, we should not include EO evaluation parameters in the scenario tables at this time.

On sensing modes, although only certain sensing modes may be realistic for each sensing target scenarios, a majority of companies believe that no (de)prioritization or down selection may be necessary for the deployment scenario tables. Hence, it is proposed that all six sensing modes (also aligned with SID) are applicable for each sensing target/use case. In this spirit, this row may be removed from the scenario tables as it is implied that all modes could be supported. The evaluation/calibration phase can articulate the sensing modes leveraged for a given target.

On sensing area, several companies pointed out that this may not be necessary and can be removed as an evaluation parameter in the scenario tables. Sensing area is implied as it can be defined from the communications scenario/cell layout, etc.

## Proposal 4-1 (FL1)

Proposal 4-1: (HIGH)

General principles for all sensing target deployment scenarios should consider the following:

- “Unintended/Environment objects” in all scenario tables should be changed to “Environmental Objects (EO)” to align terminology across modeling methodology and deployment scenario discussions.

- Physical characteristics (e.g., size, shape, material) of the sensing targets as part of the scenarios should be discussed after consensus is reached on whether/how to model these parameters. This also impacts min. 3D distances.

- Evaluation parameters for EOs should not be included in the scenario tables until consensus is reached on whether/how to model these parameters.

- Sensing mode is not included in the scenario tables, but may be included in the evaluation/calibration phase.

- Sensing area can be removed from the scenario parameters as it is relies upon the applicable communications scenario, but may be included as part of calibration/evaluation.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | 1. Yes. 2&3. OK to wait for more progress on channel modeling. 4. OK for the sake of progress. 5. Eventually we need some discussion when evaluating performance of sensing, e.g. how to draw CDF based on statistics from a specific area. We are OK to defer the discussion for now – maybe no need to discuss in this table. |
| ZTE |  | Generally, we do not think this proposal is necessary and it is better to direct discuss each table. For bullet 5: we insist on keeping sensing area for both meeting sensing requirement and simplifying evaluation complexity. Sensing area corresponding to no-fly-zone: for UAV detection/tracking use cases, non-cooperative UAVs may intrude some no-fly zone intentionally or unintentionally which would lead to serious consequence. - Sensing area can be removed from the scenario parameters as it is relies upon the applicable communications scenario. For bullet 1 and bullet 3: we think those two can be merged. For bullet 2: our initial thinking on the impact of target physical characteristics on channel modelling is the RCS. |
| vivo | Yes | We are supportive of the proposal. |
| Xiaomi | Yes |  |
| Toyota | Yes |  |
| Panasonic | Comment | Regarding bullet 2, what does it mean for “should be avoided”? - Does it meant to consider to (1) include it if there is consensus on on how to model physical characteristics (e.g., size, material) of the sensing targets, or (2) to discuss it later? |
| InterDigital |  | We agree with 1, 3, and 5. Regarding point 2, we do not see a need to postpone the discussions, as the sensing targets have already been defined. Their physical properties should remain consistent, regardless of the modelling methods applied. Regarding point 5, the sensing area in the communication scenario is determined by the TRP location (e.g., cell area). However, since different sensing modes can involve additional entities, such as other TRPs and/or UEs, relying on the communication scenario does not accurately define the sensing area. Dropping objects based on an area determined only by TRP location may result in targets not being sensed. Therefore, sensing area should be properly defined and included as a parameter. |
| Huawei, HiSilicon |  | 1. ok. 2/3, despite the size/material of sensing targets or EO, the discussion is going into a circle: modelling discussion sometimes refers to what it is. Scenarios discussion refers to whether it is modelled. Such circling does not help for progress. Scenario should be in general and can be noted as for discussion purpose so whether it is modelled can still be carried out. 4/5. ok |
| Qualcomm |  | We are not sure this proposal would help much and prefer to go directly to the tables shown in the proposals below. |
| SONY |  | Generally OK, except no.2. We think we can still continue the physical characteristics. For example, we can try to get the size information (in bracket [] ). We also need to discuss the incoming LS from 5GAA (which is related to the physical characteristics). |
| Samsung |  | For the 2 (physical characteristics), we need to discuss regardless of channel modeling direction. Because the RCS is related to not only angle and frequency but also physical characteristics (size, material). In order to define the RCS value (e.g., range of value), it should consider, first. Remaining proposals are fine. |
| Lenovo |  | Ok with 1-4. For 5, Description of sensing area (different from the underlying communication scenario) is needed for scenarios including multiple cell areas, or a particular sensing area within a cell (e.g., case of collision avoidance). |
| Ericsson |  | We agree with 4th and 5th bullets. It is true that it is under discussion on whether to model Type-2 EO in 9.7.2, while to distinguish target from unintended objects is one objective of SID. Therefore, we see the need of studying physical parameters of Type-1 EO. In general, physical characteristics of sensing objects are independent from the ways of modelling them. |
| Apple |  | OK |

## Proposal 4-2 (FL2) – FOR ONLINE DISCUSSION

Proposal 4-2: (MED)

General principles for all sensing target deployment scenarios should consider the following:

- “Unintended/Environment objects” in all scenario tables should be changed to “Environmental Objects (EO)” to align terminology across modeling methodology and deployment scenario discussions.

  - Details of evaluation parameters for EOs should not be included in the scenario tables until consensus is reached on whether/how to model these parameters.

- Generally, physical characteristics (e.g., size, shape, material) of sensing targets and EOs are FFS in the scenario tables until consensus is reached on whether/how to model these parameters.

  - NOTE: Some parameters from existing 3GPP TRs can be taken as baseline if there is consensus.

- “Sensing mode” is removed in the scenario tables, but may be included in the evaluation/calibration phase. Per the SI, all sensing modes are possible for the deployment scenarios.

- “Sensing area” may be addressed as part of the sensing target distribution and/or Tx/Rx characteristics.

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We are not ok with this version. How about the following revised version to the group? Proposal 4-2: (MED) General principles for all sensing target deployment scenarios: - Physical characteristics (e.g., size, shape, material) of sensing targets and EOs are listed as reference at this stage for assisting channel modelling discussion. Whether/how to model these parameters will be decided in the agenda of channel modelling discussion . - “Sensing mode” is removed in the scenario tables, but may be included in the evaluation/calibration phase. Per the SI, all sensing modes are possible for the deployment scenarios. - “Sensing area” may be addressed as part of the sensing target distribution and/or Tx/Rx characteristics. |
| Xiaomi | yes |  |
| InterDigital | yes |  |
| ZTE | Yes |  |
| Qualcomm | No | We still don’t agree to change “unintended/environmental” to just environmental. We don’t have EO defined yet, nor it will be clear whether indeed all objects will be categarized as “environmental objects” in the sense that 9.7.2 is attemping to do. Also, the SID talks about “unintended objects”. |
| New H3C | Yes |  |

# Topic #3: Evaluation parameters for UAV sensing target scenarios

The related proposals are copied below.

| Company | Views |
| --- | --- |
| Spreadtrum [6] | Proposal 1: For UAV sensing scenarios, only support gNB mono-static and gNB-gNB bi-static sensing modes. Proposal 2: For UAV sensing scenarios, support the following candidate horizontal velocities for sensing target: - {30, 60, 100, 160} km/h. Proposal 3: For UAV sensing scenarios, support the following vertical plane velocity for sensing target - 0 km/h, at least one non-zero value Proposal 4: For UAV sensing scenarios, support uniform distribution in both horizontal and vertical planes. Proposal 5: For UAV sensing scenarios, at least support two UAV types with (Length x Width x Height) defined as [0.15m * 0.15m * 0.05m] and [3m * 3m * 1m], other UAV types are not precluded. Proposal 6: For UAV sensing scenarios, it is not necessary to define sensing area as one of the evaluation parameters. Proposal 7: For UAV sensing scenarios, minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] can be 35m, and the minimum 3D distance between sensing targets can be 2.5m. |
| CMCC [7] |  |
| Intel [8] |  |
| Tejas [10] |  |
| KPN [11] | Proposal 1: The maximum height of 300 metres that has been discussed so far seems reasonable to detect UAVs that are flying within and above the height regulations. Proposal 2: The underlying terrain configuration, including obstacles, should be taken into account when specifying the 3D distribution of sensing targets, such as the minimal horizontal and vertical distance from/to buildings/obstacles. Proposal 3: Consider a maximum horizontal velocity up to 180 km/h for the UAV sensing target. |
| Tiami [12] |  |
| China Telecom [13] |  |
| Nokia [14] | Proposal 2: Maximum horizontal UAV mobility for calibration of UAV sensing scenarios should be 160 km/hr. Proposal 3: Vertical UAV mobility for calibration of UAV sensing scenarios should not be considered. |
| IDCC [16] | Proposal 9: Specify the sensing target orientation for UAV as uniform bearing angle between [0, 360], fixed down-tilt and slant angles for clarity. Proposal 10: Use Table 1 to update the UMa deployment scenario for UAV sensing. |
| OPPO [17] | Proposal to combine outdoor sensing target tables |
| Xiaomi [18] | Table Ⅰ Evaluation parameters for UAV sensing scenario Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitter and sensing receiver; Rx/Tx locations are the TRPs and UEs locations in the corresponding communication scenario Note 1: Other Rx/Tx properties (e.g. mobility) can also be taken from the corresponding communication scenario. Note 2: This may include aerial UEs as Rx/Tx that can be selected among locations in the UMi-AV, UMa-AV, RMa-AV communication scenarios.. Sensing mode; TRP-TRP bistatic, TRP mono-static, TRP-UE bistatic and UE-TRP bistatic. Sensing target; Target type; UAV. Indoor/outdoor; 100% outdoor. 3D mobility; Uniformly distributed between [20km/h, 160km/h] on horizontal. 3D distribution; Horizontal: uniform on horizontal Vertical: uniformly distributed between 1.5m and 300m. Orientation; Uniform distribution (on horizontal). Physical characteristics; Size: 1.6m x 1.5m x 0.7m. Target number; Case1：1 per TRP Case2：3 per TRP Case3：5 per TRP. Sensing aera; Whole area. Minimum 3D distances; Min BS-target distance; 10m. Min UT-target distance; 1m. Min target-target distance; 10m. |
| CATT [19] |  |
| LG [20] |  |
| Sony [21] |  |
| Samsung [25] |  |
| Lenovo [26] |  |
| MediaTek [27] |  |
| Apple [29] | P3: The following updates should be made to the evaluation parameters for the UAV sensing scenario: - Supported sensing modes: All 6 sensing modes - Sensing Target - 3D mobility - Horizontal velocity: - Option 1: [0km/h], 3km/h, 30km/h, 60km/h, 100km/h, 160 km/h - Option 2: uniform distribution between [0]-160km/h - Vertical velocity: [0km/h], [20km/h, 40km/h] - FFS: time-varying 3D mobility. - 3D distribution - FFS: non-uniform distribution, e.g., delivery route - Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] - Option 1: Fixed distance, e.g., [10m] - Option 2: Distances dependent upon communications scenarios - Option 3: Distances dependent upon sensing mode/service - Minimum 3D distance between sensing targets - Option 1: Fixed distance, e.g., [10m] - Option 2: Distances based on communications scenarios - Option 3: Distances dependent upon sensing mode - Option 4 : Not needed in scenario table |
| AT&T [30] |  |
| CAICT [34] |  |
| Ericsson [35] | Proposal 10 For UAV horizontal distribution, a uniform distribution along the delivery route can be considered. Proposal 11 For UAV vertical distribution, taking off and landing occurs in certain areas close to starting point and destination with a height from 0m to roof level. For other areas along the delivery route, a fixed height value, above roof level, is assumed. Proposal 12 In some cases, the size of a sensing target needs to include the size of an object attached to it, e.g. UAV and package. Proposal 13 The orientation of sensing targets other than UAVs must be consistent with their trajectories. |
| ZTE [37] |  |
| Huawei [38] |  |
| Qualcomm [39] |  |

Moderator Discussion:

## Proposal 5-1 (FL1)

Proposal 5-1:  (HIGH) For UAV sensing target scenarios, the following table is proposed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: Uniform distribution 0km/h - 160km/h Vertical velocity: 0km/h, optional 20km/h, 40km/h Note2: 3D mobility can be horizontal only or vertical only or a combination. |
|  |  | 3D distribution | Uniform in horizontal plane within a cell boundary. Uniform in vertical plane between 1.5 m and 300 m. |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | UAV object type(s) [FFS] |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/ |  |  | Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target |
| Minimum 3D distance between sensing targets |  |  | FFS, based on outcome for AI 9.7.2 |
| [EO, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS, based on outcome for AI 9.7.2 |

Companies can provide comments/inputs on using the proposal above as a starting point in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | Most of the changes are acceptable to us, except: - For 3D mobility, even in 36.777, they use specific set of speeds in Mobility evaluation (see Table A.2.1-1 in 36.777). We should allow the same option for sensing: UE speed; Horizontal speed for terrestrial and aerial UEs; 3 km/h, 30 km/h, 60 km/h, 160 km/h. - For 3D distribution vertical plane, if we strictly follow 36.777 (which is what we are doing), it should be: - Uniformly distributed between 1.5m and 300m - Optionally, fixed height value chosen from {50, 100, 200, 300} m. - Other optional fixed height values are not precluded.. - For min. 3D distance, not sure far field condition should become the prerequisite for sensing. It is still ambiguous and difficult to guide evaluation. |
| ZTE |  | For 3D mobility We propose to consider both fixed velocity and uniform distributed velocity value. On-route flying with fixed velocity is important for commercial UAVs and one of the key use cases of sensing is commercial UAVs’ flight trajectory tracing. Also, fixed velocity setting can be used to evaluate UAV velocity impact on sensing performance. We prefer the following: - Fixed velocity as baseline: {3, 30, 60, 100, 160}km/h - Up to 160km/h with uniform distribution For 3D distribution: not sure what is the cell boundary, prefer to delete boundary and consider two height options from 36.777 Horizontal: Uniform in horizontal plane within a cell boundary. Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m For the last two rows of the table, we prefer to delete “based on outcome for AI 9.7.2”, FFS is sufficient. |
| vivo |  | For the 3D mobility, according to TR36.777, the maximum horizontal velocity can be 160km/h. |
| Xiaomi | Partial yes | 1. For sensing Tx/Rx properties, add one FFS for how to select sensing Tx/Rx among corresponding communication TRP/UE. From our perspective, selecting all TRP/UE as sensing Tx/Rx can be considered as a start point. 2. For 3D mobility, we don’t think the horizontal velocity should include 0km/h. First, we can’t distinguish the UAVs from the environment if the velocity of UAV is 0km/h. Second, some companies propose that the 0km/h UAV can be detected by micro-doppler. However, whether to model micro-doppler is still under discussion in 9.7.2. On the other hand, we don’t think time varying velocity configuration should be supported in this SI. This also makes the 0km/h even more unreasonable. The horizontal velocity can be uniform distributed in [20km/h, 160km/h] and the minimum value should be greater than zero. 3. For min-distance, it’s unclear for the description of min. far-field distance. First, regarding the FR3 channel modeling, the near field definition is still under discussion. On the other hand, the evaluation parameters are defined for specific sensing target/scenarios rather than far/near field. |
| InterDigital |  | We have a similar view as vivo. We propose to change 100km/h to 160km/h for the maximum velocity as defined in the corresponding communication scenario in TR 36.777. |
| Huawei, HiSilicon |  | In general we are fine to further progress on such detailed parameters. |
| Qualcomm |  | We think it is very important to keep the velocity uniform across a large range and not fix a small subset of values. Velocity will be one of the properties that we would need to estimate, and there is a risk that evaluations will be biased to do “classification” amongst the 4 values (not necessarily on purpose), instead of actual estimation. Up to 160km/h with uniform distribution |
| SONY |  | For the 3D mobility, we think we should also use fixed horizontal velocity (instead of only the uniform distribution). |
| Samsung |  | For 3D mobility, the vertical velocity needs to be discussed. If the fixed 0 km/h’s vertical velocity is considered, it precludes that the combination of 3D mobility is not considered. So, could we remove ‘optional’ in here or clarify whether combination of mobility is considered or not? |
| Lenovo |  | Description of sensing area (different from the underlying communication scenario) is needed for scenarios including multiple cell areas, or a particular sensing area within a cell (e.g., case of collision avoidance). Moreover, the Min sensing Tx/Rx distance should be greater than the min far field distance of the Tx/Rx nodes (and not the far field distance of the sensing target). We suggest following modifications: For 3D distribution or Sensing area, adding the option of: Uniform within distance r from a reference. Minimum 3D distances between pairs of Tx/Rx and sensing target: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target |
| Ericsson |  | A question about ‘Horizontal velocity: Uniform distribution 0km/h - 100km/h’, does it imply that the velocity of a target is time-varying? The objective of the SID is to support object tracking. The tracking should allow objects move with a time-variant velocity, which is supported in 7.6.6 Time-varying Doppler shift in 38.901. For 3D distribution, a UAV with delivery task would not be uniformly distributed over the cell and instead we can consider a uniform distribution along UAV delivery route, which is analogous to the trajectory of vehicles in highway scenario. Moreover, for BS-BS sensing, distribution of a UAV may be not limited within a cell. Regarding size of UAV, we suggest considering the size of a UAV alone and the size of a UAV carrying a package. 5GAA considers the height of an E-scooter is the height measured from ground to E-scooter deck plus road user height. We are not sure 9.7.2 will discuss the minimum 3D distance between sensing targets. Channel modelling should assume the minimum distance between two objects as 0. The resolution will be discovered in future sensing algorithm. Lastly, LOS/NLOS is proposed as a field in vehicle table. We can add the field in UAV table as well. |

## Proposal 5-2 (FL2) – FOR ONLINE DISCUSSION

Proposal 5-2:  (HIGH) For UAV sensing target scenarios, the following table is proposed for deployment scenario parameters/values using the agreements from RAN1#117 as a baseline.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. Note1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: Option A: {3, 30, 60, 100, 160} km/h Option B: uniform distribution up to 160km/h Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination. NOTE3: Other mobility parameters are not precluded. |
|  |  | 3D distribution | Horizontal plane: Uniform within a cell. Vertical plane: Uniform between 1.5m and 300m. - Optionally, fixed height value chosen from {50, 100, 200, 300} m. Other optional fixed height values are not precluded. |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | 0.35 x 0.285 x 0.110 m. Material, Other sizes [FFS] |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/ |  |  | Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  |  | FFS |
| [EOs, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | As commented to proposal 4-2. Physical characteristics of targets should be listed as reference at this stage for assisting channel modelling discussion. UAV as sensing target can consider the size of 0.35*0.285*0.110 m as an example. |
| Xiaomi |  | 1. For sensing Tx/Rx properties, add one FFS for how to select sensing Tx/Rx among corresponding communication TRP/UE. From our perspective, selecting all TRP/UE as sensing Tx/Rx can be considered as a start point. 2. For 3D mobility, the minimum velocity value of option-B should be greater than zero, for example [10km/h, 160km/h]. 3. For min-distance, it’s unclear for the description of min. far-field distance. First, regarding the FR3 channel modeling, the near field definition is still under discussion. On the other hand, the evaluation parameters are defined for specific sensing target/scenarios rather than far/near field. |
| ZTE |  | Thanks so much for FL’s update! According offline discussion, sensing area may not occupy a standalone row, but can be addressed as part of the sensing target distribution and/or Tx/Rx characteristics. Therefore, for sensing target distribution row, we suggest the following change: Horizontal plane: Uniform within a sensing area cell. - Sensing area can be either cell layout or one cell (e.g., center cell) of the cell layout - Other sensing area is not precluded |
| Qualcomm |  | We also support ZTE’s changes |
| New H3C |  | OK with ZTE’s modification |

# Topic #4: Evaluation parameters for Automotive sensing target scenarios

The related proposals are copied below.

| Company | Views |
| --- | --- |
| Spreadtrum [6] |  |
| CMCC [7] |  |
| EURECOM [9] |  |
| Tejas [10] |  |
| Tiami [12] |  |
| China Telecom [13] |  |
| Nokia [14] | Table 2: Evaluation parameters for automotive sensing scenarios Parameters; Value. Applicable communication scenarios; Highway, Urban Grid. Sensing transmitters and receivers properties; Rx/Tx Locations including mobility, distribution and height; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario. LOS/NLOS; LOS/NLOS conditions. Sensing target; LOS/NLOS; LOS/NLOS condition. Outdoor/indoor; Outdoor. 3D mobility; Dropping option A, B for highway scenario in TR 37.885, Dropping option A, B for urban grid scenario in TR 37.885. 3D distribution; Dropping option A, B for highway scenario in TR 37.885, Dropping option A, B for urban grid scenario in TR 37.885. Orientation; Determined by UE mobility. Physical characteristics (e.g., size). Number of targets. |
| vivo [15] |  |
| OPPO [17] |  |
| Xiaomi [18] |  |
| CATT [19] |  |
| LG [20] |  |
| Sony [21] |  |
| NVIDIA [22] |  |
| Samsung [25] |  |
| Lenovo [26] |  |
| TOYOTA [28] |  |
| AT&T [30] |  |
| CAICT [34] |  |
| Ericsson [35] | Proposal 1 If the benefit and need of using vehicular UEs as sensing Tx/Rx can be justified, Highway and Urban grid in 37.885 can be considered as applicable communication scenarios for outdoor vehicle sensing scenario. Proposal 2 Sensing scenarios of outdoor vehicles and objects creating hazards on roads/railways can have the same applicable communication scenarios, |
| NTT DoCoMo [36] |  |
| ZTE [37] |  |
| Huawei [38] |  |
| Qualcomm [39] |  |

Moderator Discussion:

## Proposal 6-1 (FL1)

Proposal 6-1:  (HIGH) For Automotive sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid, [UMa, UMi, RMa] |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario |
|  |  | LOS/NLOS conditions |
| Sensing target | LOS/NLOS | LOS/NLOS conditions |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Based on dropping in [37.885] per communication scenario |
|  | 3D distribution | Based on dropping in [37.885] per communication scenario |
|  | Orientation | Lane direction |
|  | Physical characteristics (e.g., size) | FFS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/ |  | Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target |
| Minimum 3D distance between sensing targets |  | FFS, based on outcome for AI 9.7.2 |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | Some suggestions: (1) Applicable communication scenarios: current Highway, Urban Grid are already embedded in UMa/UMi/RMa. We suggest to remove UMa, UMi and RMa. (2) For Sensing transmitters and receivers properties: since urban grid and highway exist, we should add RSU as possible sensing node: UE-type RSU can be selected as the sensing transmitters and/or receivers |
| ZTE |  | For applicable communication scenario: we have the doubts on adding UMi, UMa and RMa here due to the lack of road/lane configuration. But we have a row saying orientation of sensing target is along lane direction. How can we distribute target in the lane without any lane configuration? Highway, Urban Grid, FFS: [UMa, UMi, RMa] |
| vivo | Yes with comments | For the UMi, UMa and RMa scenarios without lanes, the 3D distribution of sensing target can be uniform in the horizontal plane, and the 3D mobility can be uniformly distributed between [FFS] – [FFS] km/h. The velocity can refer to the typical assumption of the communication simulation, for example, in TR38.802, the velocity of outdoor for car UE is 30km/h in UMa and UMi scenarios and 120km/h in RMa scenario. For the 3D distribution, the height of sensing target also needs to be considered, because the 3D distance has to be calculated for channel modeling. The height assumption of vehicles is related to the type of the vehicles and the RCS model outcome in AI 9.7.2. |
| Xiaomi | Partial yes | 1. For sensing Tx/Rx properties, RSU can also be selected as sensing Tx/Rx. 2. For 3D mobility/3D distribution, there is no road configuration in UMa/UMi/RMa scenarios. RAN1 needs to consider how to drop vehicles and configure velocity in those scenarios. |
| Toyota |  | Regarding Applicable communication scenarios, we are not sure whether/how UMa, UMi, and RMa can be considered due to a lack of road/lane configurations in those scenarios. |
| Huawei, HiSilicon |  | Since Uma/UMi/RMa and urban grid are listed as applicable communication scenarios, one question worth discussion is whether the difference is how to drop UE. For example, urban grid is listed since we are thinking to drop UE as potential sensing receiver as the pedestrian UEs along the sidewalk as in urban grid in TR38.885. Another comment we’d like to make is the deployment listed in the table and specifically the ISD could be one option for evaluation and other options can also be considered for evaluation. |
| Qualcomm |  | We think highway and urban grid is enough and there is no need to do UMi, Uma, Rma. We support adding UE-type RSUs as potential sensing nodes also |
| SONY | Yes with comments | Only consider Highway, Urban Grid or at least down-prioritize (UMa, UMi, and RMa) |
| Samsung |  | For sensing Tx/Rx properties, both vehicle UE and pedestrian UE can be considered. |
| Lenovo |  | We suggest following modifications: For 3D distribution or Sensing area, adding the option of: Uniform within distance r from a reference. Minimum 3D distances between pairs of Tx/Rx and sensing target: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target |
| Ericsson |  | We would like to understand the motivation of adding Highway, Urban Grid as applicable communication scenarios. Is it an attempt to use BS and UE modelled in the scenarios for sensing Tx/Rx. What is the difference from using BS and UE in UMa, UMi, RMa as sensing Tx/Rx? |

## Proposal 6-2 (FL2) – FOR ONLINE DISCUSSION

Proposal 6-2:  (HIGH) For Automotive sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid, [UMa, UMi, RMa] |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note: UE-type RSU can be selected as the sensing Tx/Rx. |
|  |  | LOS/NLOS conditions |
| Sensing target | LOS/NLOS | LOS/NLOS conditions |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Based on dropping in [37.885] per communication scenario |
|  | 3D distribution | Based on dropping in [37.885] per communication scenario |
|  | Orientation | Lane direction |
|  | Physical characteristics (e.g., size) | FFS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/ |  | Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | As commented to proposal 4-2. Physical characteristics of targets and EO should be listed as reference at this stage for assisting channel modelling discussion. Vehicle as sensing target can refer to the 5GAA incoming LS in R1-2405964. EO for highway and for urban grid might be different. Wall or building is mainly considered for urban grid as follows for example: Wall: 413m x 230m x 20m |
| Qualcomm |  | We have a comment on the “LOS/NLOS conditions” : It appears in 2 places it seems. Also, what does it mean? That there will always be LOS and NLOS according to the available TRs, e.g. 37.885? We would like to add one additional option that in some cases maybe LOS only conditions is enough: Option 1: LOS/NLOS conditions Option 2: LOS-only condition |

# Topic #5: Evaluation parameters for Humans sensing target scenarios (outdoors/indoors)

The related proposals are copied below.

| Company | Views |
| --- | --- |
| Spreadtrum [6] | Proposal 8: Regarding scenarios for humans indoors, not support Indoor Room, UMi and UMa scenarios. Proposal 9: Regarding scenarios for humans outdoors, support RMa scenario. |
| Intel [8] |  |
| EURECOM [9] | Proposal 1: For the human indoor use case, Indoor Office in TR 38.901 is used for the deployment scenario. Proposal 2: For the human outdoor use case, UMi in TR 38.901 is used for the deployment scenario. |
| Tejas [10] | Proposal-1: The minimum distance between the target-TRP and target-UE can be modelled based on 38.901, considering the sensing target as a UE. Proposal-2: The minimum distance between two targets should be considered based on their dimensions. Proposal-3: We propose that at least two types of EOs should be considered for channel modelling. Type-1 EO (other humans, cyclists, etc.), which has similar characteristics (shape and size) to the sensing target, albeit the mobility model might differ. Proposal-4: The characteristics of Type-2 EOs depend on the terrain and should be modelled based on the measurements. |
| Tiami [12] |  |
| Nokia [14] | Table 3: Evaluation parameters for human sensing scenarios Parameters; Value. Applicable communication scenarios; InF, InO, UMi. Sensing transmitters and receivers properties; Rx/Tx Locations including mobility, distribution and height; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario. LOS/NLOS; LOS/NLOS conditions. Sensing target; LOS/NLOS; LOS/NLOS condition. Outdoor/indoor; InF, InO: Indoor UMi: Indoor/Outdoor. 3D mobility; 3 km/hr.. 3D distribution; Uniform. Orientation; Uniform in horizontal plane FFS on posture and gestures. Physical characteristics (e.g., size). Number of targets. |
| vivo [15] |  |
| IDCC [16] |  |
| OPPO [17] | Combine tables for AGV and human (indoor) |
| Xiaomi [18] |  |
| CATT [19] |  |
| LG [20] |  |
| Samsung [25] |  |

| Lenovo [26] |  |
| --- | --- |
| MediaTek [27] |  |
| Apple [29] | P2: For Human indoors, UMa and UMi should be captured. - The evaluation should discuss the relative locations of the sensing transmitters, receivers and targets and focus on specific cases - Option 1: sensing transmitter outdoors, sensing receiver outdoors only, - Option 2: sensing transmitter outdoors, sensing receiver indoors |
| AT&T [30] |  |
| CAICT [34] |  |
| NTT DoCoMo [36] |  |
| ZTE [37] |  |
| Qualcomm [39] | Proposal 1: Resolve the brackets in the sensing target & scenario mapping table as follows: - For the “Human indoors” sensing target - do not include UMi, UMa as a scenario - For the “Human outdoor” sensing target - Do not include RMa as a scenario |

Moderator Discussion:

## 7.1 Proposal 7-1 (FL1)

Proposal 7-1:  HIGH) For Human sensing target scenarios, (indoor and outdoor), the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, UMa [TR38.901] Urban grid [TR37.885] |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario UE-type RSU can be selected as the sensing transmitters and/or receivers |
|  | Rx/Tx Mobility | Option 1: 0km/h or 3km/h Option 2: Uniform distribution 0km/h - 3km/h | Option 1 : 0km/h or 3km/h Option 2: Uniform distribution 0km/h - 3km/h |
| Sensing target | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | 3km/hr (horizontal) | 3km/hr (horizontal) |
|  | 3D distribution | Uniform | Uniform in horizontal plane within a cell boundary |
|  | Orientation | FFS | FFS |
|  | Physical characteristics (e.g., size) | FFS | FFS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | FFS | FFS |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT, CICTCI |  | The last two rows for ‘Minimum 3D distance between sensing targets’ and ‘Environment Objects, e.g., types, characteristics, mobility, distribution, etc.’ are missing. (though it should be FFS at this time) |
| ZTE |  | For 3D mobility: we prefer to delete 0km/h. our goal in this release is for sensing target detection and tracking, and not sure how to detect human without any velocity. For 3D distribution: we prefer to align the wording with UAV table for human outdoor. We suggest: For human outdoor: Uniform in horizontal plane within a cell |
| vivo | Yes with comments | - For the UE location in indoor scenarios, some stationary UEs, for example UEs on the tables with 0.8m height, can be considered as the sensing TX/RX. - For the 3D mobility of human, the velocity in horizontal can be uniform distribution between 0-[FFS]km/h. For indoor human, the maximum velocity can be 3km/h. For outdoor human, according to the LS of 5GAA, the human can be a pedestrian, cyclist, e-scooter and so on, and thus, the maximum velocity can e larger than 3km/h, for example, 25km/h. - For the 3D distribution, the height of sensing target also needs to be considered, because the 3D distance has to be calculated for channel modeling. The height assumption of human is related to the size of the human and the RCS model outcome in AI 9.7.2. As a possible assumption, the human height can be the height of human body centroid, e.g. 0.75m ~ 1.5m. |
| InterDigital | Yes |  |
| c | Yes in principle | We prefer to have the velocity in horizontal to be uniform distribution between 0-[3] km/ |
| Samsung |  | For human indoor, we wonder whether UMa and UMi is considered or not. To clarify the scope, could we discuss it first? |
| Ericsson |  | For indoor human scenario, we suggest adding UMa and UMi as applicable communication scenarios. We consider outdoor BS and indoor UE modelled in UMa and UMi for indoor human sensing scenario for several reasons. 1. Most indoor traffic in current 4G and 5G networks is served by outdoor BS and to a much lesser extent by indoor BS. This corresponds to the 80% indoor UT ratio in UMi and UMa scenarios in 38.901. Indoor UE and outdoor BS modelled in Uma and Umi can be used for UE-BS sensing modes; indoor UE modelled in UMa and UMi can be used for UE mono/bi-static sensing modes. 2. UMa and UMi are the most important scenarios for communication, so they should definitely be considered for joint communication and sensing. It includes the coexistence between BS-UE communication and several sensing modes, including indoor UE-outdoor BS sensing and indoor UE mono/bi-static sensing. 3. Companies may have concern on O2I penetration loss for the outdoor BS - indoor human link, which exists in communication as well. In addition, adding UMa and UMi as applicable communication scenarios to indoor human sensing scenario can reuse penetration loss model in 38.901 without addition spec impact. 4. Both indoor BS and outdoor BS have been deployed. The choice of choosing indoor BS or outdoor BS as sensing Tx/Rx for indoor human sensing scenario is up to implantation and should not be restricted by channel modelling. |
| Apple |  | Yes |

## 7.2 Proposal 7-2 (FL2) – FOR ONLINE DISCUSSION

Proposal 7-2:  HIGH) For Human sensing target scenarios, (indoor and outdoor), the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for Human sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | UMa, UMi, InH-Office, InF [TR38.901] Indoor room [TR38.808] | UMi, UMa [TR38.901] Urban grid [TR37.885] |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario UE-type RSU can be selected as the sensing transmitters and/or receivers |
|  | Rx/Tx Mobility | 0km/h or 3km/h | Option 1 : {0, 3, [FFS]} km/h Option 2: Uniform distribution 0km/h – [FFS] km/h |
| Sensing target | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1 : 3 km/h Option 2: Uniform distribution 0km/h – 3 km/h | Option 1 : {3, [FFS]} km/h Option 2: Uniform distribution 0km/h – [FFS] km/h |
|  | 3D distribution | Uniform | Uniform in horizontal plane within a cell |
|  | Orientation | FFS | FFS |
|  | Physical characteristics (e.g., size) | FFS | FFS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | FFS | FFS |
| [EOs, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS | FFS |

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #6: Evaluation parameters for AGV sensing target scenarios

The related proposals are copied below.

| Company | Views |
| --- | --- |
| Spreadtrum [6] |  |
| EURECOM [9] |  |
| Tejas [10] |  |
| vivo [15] |  |
| IDCC [16] |  |
| OPPO [17] | Combine tables for AGV and human indoors |
| CATT [19] |  |
| LG [20] |  |
| Sony [21] |  |
| Samsung [25] |  |
| Lenovo [26] |  |
| NTT DoCoMo [36] |  |
| ZTE [37] |  |
| Qualcomm [39] |  |

Moderator Discussion:

## Proposal 8-1 (FL1)

Proposal 8-1:  (MEDIUM) Proposals TBD based on outcome from above scenarios

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# Topic #7: Evaluation parameters for Hazardous objects sensing target scenarios

The related proposals are copied below.

| Company | Views |
| --- | --- |
| Spreadtrum [6] |  |
| EURECOM [9] |  |
| vivo [15] |  |
| Xiaomi [18] | Proposal 1: Limit the 5th sensing target “Objects creating hazards on roads/railways, with a minimum size dependent on frequency” to human and animal in this SI. |
| CATT [19] |  |
| LG [20] |  |
| KRRI [24] |  |
| Samsung [25] |  |
| Lenovo [26] |  |
| TOYOTA [28] |  |
| Ericsson [35] | Sensing scenarios of outdoor vehicles and objects creating hazards on roads/railways can have the same applicable communication scenarios, |
| NTT DoCoMo [36] |  |
| ZTE [37] |  |
| Qualcomm [39] |  |

Moderator Discussion:

## Proposal 9-1 (FL1)

Proposal 9-1:  (MEDIUM) Proposals TBD based on outcome from above scenarios

Companies can provide comments/inputs in the following table:

| Company | Yes/No | Comments |
| --- | --- | --- |

# References

- RP-240799, 	“Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- R1-2405697 	Session notes for 9.7 (Study on channel modelling for Integrated Sensing And Communication for NR), Ad-Hoc Chair (Huawei). RAN1#117.

- R1-2405964	LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC	5GAA, Huawei, Continental Corp.

- R1-2406251	Discussion of 5GAA LS on ISAC	OPPO

- R1-2406903	Discussion on LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC	Ericsson

- R1-2405922	Discussion on ISAC deployment scenarios	Spreadtrum Communications

- R1-2405999	Discussion on ISAC deployment scenarios	CMCC, China Southern Power Grid

- R1-2406009	Deployment scenarios for ISAC channel modeling	Intel Corporation

- R1-2406047	Discussion on ISAC deployment scenarios and requirements	EURECOM

- R1-2406067	Discussion on deployment scenarios for ISAC	Tejas Network Limited

- R1-2406075	Discussion on ISAC deployment scenarios	KPN N.V.

- R1-2406084	Discussion on ISAC deployment scenarios	Tiami Networks

- R1-2406098	Discussion on ISAC deployment scenarios	China Telecom

- R1-2406137	Discussion on ISAC deployment scenarios	Nokia, Nokia Shanghai Bell

- R1-2406196	Views on Rel-19 ISAC deployment scenarios	vivo

- R1-2406207	Discussion on ISAC deployment scenarios	InterDigital, Inc.

- R1-2406249	Discussion on ISAC deployment scenarios	OPPO

- R1-2406298	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

- R1-2406382	Discussion on ISAC deployment scenarios	CATT, CICTCI

- R1-2406450	Discussion on ISAC deployment scenarios	LG Electronics

- R1-2406483	Discussion on ISAC deployment scenarios	Sony

- R1-2406488	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

- R1-2406528	Discussion on ISAC deployment scenarios	Panasonic

- R1-2406529	Discussion on deployment scenarios for ISAC	Hanbat National University

- R1-2406664	Discussion on  ISAC deployment scenarios	Samsung

- R1-2406715	Discussion on ISAC deployment scenarios	Lenovo

- R1-2406767	Discussion on ISAC deployment scenario	MediaTek Inc.

- R1-2406792	Discussion on ISAC deployment scenarios	TOYOTA InfoTechnology Center

- R1-2406856	Discussion on ISAC deployment scenarios	Apple

- R1-2406867	Deployment Scenarios for ISAC Channel Modeling	AT&T, FirstNet

- R1-2406873	FL Summary #1 on ISAC Deployment Scenarios	Moderator (AT&T)

- R1-2406874	FL Summary #2 on ISAC Deployment Scenarios	Moderator (AT&T)

- R1-2406875	FL Summary #3 on ISAC Deployment Scenarios	Moderator (AT&T)

- R1-2406896	Considerations on ISCA deployment scenarios	CAICT

- R1-2406905	Discussion on ISAC Deployment Scenarios	Ericsson

- R1-2406944	Study on deployment scenarios for ISAC channel modelling	NTT DOCOMO, INC.

- R1-2406959	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

- R1-2406974	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

- R1-2407043	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

# Annex: RAN1 Endorsed agreements

RAN1#116 Agreements

Agreement

For progressing ISAC study, the following sensing targets and existing communication scenarios will be considered as a starting point:

Note1: the table below does not imply that the sensing target will be placed at positions defined for UEs and BSs in the scenarios in the right column.

Note2: the table below does not imply that UEs are necessarily placed at positions defined for UEs in the scenarios in the right column.

Note3: the existing communication scenarios are listed with the intent to use the evaluation parameters defined for those scenarios, as a starting point.

| Sensing Targets | scenarios |
| --- | --- |
| UAVs | RMa-AV, UMa-AV, UMi-AV (TR 36.777) |
| Humans indoors | InF, Indoor Office, [Indoor Room (TR 38.808)], [UMi, UMa] |
| Humans outdoors | UMi, UMa, [RMa] |
| Automotive vehicles (at least outdoors) | Highway, Urban grid, UMa, UMi, RMa |
| Automated guided vehicles (e.g. in indoor factories) | InF |
| Objects creating hazards on roads/railways (examples defined in TR 22.837) | Highway, Urban grid, HST |

Agreement

For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions:

- Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver.

- Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter.

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal.

- Background environment: background (clutter and/or environmental objects) that are not the sensing target(s).

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same TRP or UE.

- Bi-static sensing: sensing where the sensing transmitter and sensing receiver are in different TRPs or UEs.

- Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target.

- Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

RAN1#116-bis Agreements

Agreement

RAN1 agrees the following ISAC terminology with minor modifications as follows:

For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions:

- Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver.

- Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter.

- Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal.

- Background environment: background (clutter and/or environmental objects) that are not the sensing target(s).

- Mono-static sensing: sensing where the sensing transmitter and sensing receiver are co-located in the same TRP or UE.

- Bi-static sensing: sensing where the sensing transmitter and sensing receiver are in different TRPs or UEs.

- Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target.

- Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes.

Agreement

Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations. FFS: other possible sensing transmitters and receivers locations.

Agreement

The following table can be used by companies to propose values for each sensing target

- Additional parameters/rows can be added if needed

Table x. Evaluation parameter template for sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  |  |
| Sensing transmitters and receivers properties |  |  |
| Supported sensing modes |  |  |
| Sensing target | Outdoor/indoor |  |
|  | 3D mobility |  |
|  | 3D distribution |  |
|  | Orientation |  |
|  | Physical characteristics (e.g., size) |  |
| [Unintended/Environment objects] | Types |  |
|  | 3D mobility |  |
|  | 3D distribution |  |
|  | Orientation |  |
|  | Physical characteristics (e.g., size) |  |
| [Sensing area] |  |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  |  |

RAN1#117 Agreements

Agreement

For each of the sensing target deployment scenarios using the template agreed in RAN#116-bis, the following principles apply:

- For defining sensing Tx and sensing Rx properties (e.g., cell layout, BS antenna height, and minimum distance), scenario parameter values for the applicable communication scenarios in 38.901 are considered as a starting point. Updates to these evaluation parameter values for ISAC scenarios will consider the following:

  - aerial UEs parameter values as defined in TR 36.777

  - indoor room scenario parameter values defined in TR 38.808

  - automotive scenario parameter values as defined in TR 37.885, 38.859 for Urban grid/Highway

  - Minimum distances between Tx/Rx and target are not defined in the existing communications scenarios and shall be included in the sensing target deployment scenarios.

  - Note: Only deviation from the existing evaluation parameters in the applicable communication scenarios need to be explicitly defined in the ISAC scenario tables.

- For defining sensing target properties, as a baseline

- Evaluation parameter values can be taken from additional TRs, e.g., TR 36.777, 37.885, 38.859, etc.

- Size of sensing targets based on TR 22.837, 37.885 (e.g. for automotive), 38.901 (e.g. for AGV size), etc

Agreement

For ISAC deployment scenarios, carrier frequency, bandwidth, and SCS are not included in the evaluation parameters templates for sensing scenarios, but may be included in the evaluation/calibration phase.

Agreement

For UAV sensing target scenarios, the following table is used as a starting point for deployment scenario parameters/values.

Note: Additional parameters, value/value ranges are not precluded.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Applicable communication scenarios |  |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations |  | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note1: this may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. [In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.] |
| Supported sensing modes |  |  | [All 6 sensing modes] |
| Sensing target |  | Outdoor/indoor | Outdoor |
|  |  | 3D mobility | Horizontal velocity: Up to 160 km/h [FFS specific velocity(ies) or random distribution] [FFS vertical plane velocity] |
|  |  | 3D distribution | [Uniform between a minimum and maximum height] [Uniform in horizontal domain at a given height] |
|  |  | Orientation | Random in horizontal domain |
|  |  | Physical characteristics (e.g., size) | UAV object type(s) [FFS] |
| [Sensing area] |  |  |  |
| Minimum 3D distances between pairs of Tx/Rx and sensing target/[unintended objects] |  |  | FFS |
| Minimum 3D distance between sensing targets |  |  | FFS |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  |  | FFS |

Agreement

RAN1 agrees to the following revised evaluation parameters values for the UAV sensing target scenarios:

| Parameters | Value |
| --- | --- |
| Sensing transmitters and receivers properties | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenario Note 1: Other Rx/Tx properties (e.g. mobility) can also be taken from the corresponding communication scenario. Note 2: This may include aerial UEs as Rx/Tx that can be selected among locations in the UMi-AV, UMa-AV, RMa-AV communication scenarios. |