3GPP TSG RAN WG1 Meeting #118					                R1-2407232

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	8.1

Source:		Moderator (LG Electronics)

Title:	FL Summary #2 for AI 8.1: Co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion and Decision

- Introduction

The summary of contributions submitted to this meeting is as follows:

| Issue# | Description | References |
| --- | --- | --- |
| 1 | Correction on IUC in co-channel coexistence case in TS 38.214 | [1] |

- Discussion

  - [Active] Issue #1: Correction on IUC in co-channel coexistence case in TS 38.214

    - Background

One contribution [1] proposed having a correction on IUC in co-channel coexistence case as shown below.

[ZTE, Sanechips: R1-2406676] CR-Form-v12.2. [DRAFT] CHANGE REQUEST. 38.214; CR; -; rev; -; Current version:; 18.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; x; Radio Access Network; x; Core Network. Title:; Correction on IUC in co-channel coexistence case in TS 38.214. Source to WG:; ZTE, Sanechips. Source to TSG:; RAN WG1. Work item code:; NR_SL_enh2-Core; Date:; 2024-08-09. Category:; F; Release:; Rel-18. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; According the reply LS R1-2403573 on IUC or DRX in co-channel co-existence, SL IUC at NR SL module can be supported and can use NR SL information in NR/LTE V2X co-channel co-existence. And RAN1 does not agree that whether the LTE SL information can also be used for preferred resource set determination. And as discussed in previous meeting (FL summary R1-2405367), companies either think that the LTE SL information should not be used, or it is up to UE implementation to use the LTE SL information for preferred resource set determination. Neither of these understandings are aligned with the current specification. On the other hand, current RAN1 specification in section 8.1.4 and 8.1.4A of 38.214 imply that using of LTE SL information is mandatory for preferred resource set determination in case of dynamic co-channel coexistence, which is not aligned with the reply LS and RAN1 understandings.. Summary of change:; Adding text to show that whether LTE SL information should be used for IUC in co-channel co-existence is up to UE implementation.. Consequences if not approved:; 1. Misalignment between 38.214 specification and RAN1 conclusions on IUC in co-channel co-existence. 2. Make a mandatory UE behaviour which is not agreed.. Clauses affected:; 8.1.4A. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:. ### 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission <Unchanged parts omitted> When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4, it is up to UE implementation to apply the steps using information determined by the E-UTRA radio access in case of dynamic co-channel coexistence of LTE sidelink and NR sidelink, with the above parameters and the following modifications: - Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met: - the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined; - the higher layer parameter sl-Condition1-A-2 is not set to 'Disabled'. <Unchanged parts omitted>

    - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the Draft CR in Section 2.1.1 can be acceptable.

| Company | Yes or not | Comments (including your suggested/modified wording if any) |
| --- | --- | --- |
| OPPO | Yes | Agree to clarify. Otherwise, it seems that using LTE SL info for generating a preferred resource set in IUC is a mandatory behavior according to the current specification, which is never discussed and agreed in RAN1. |
| ZTE, Sanechips | Yes | Without this clarification, in case of dynamic co-channel coexistence of LTE sidelink and NR sidelink, the steps using information determined by the E-UTRA radio access shall be used, which is not aligned with the reply LS R1-2403573 and RAN1 understandings. Regarding the draft proposal 1-1, we believe the following sentence can be added to reflect what such proposal stands for - Option B) There is no consensus in RAN1 to endorse Draft CR#1-1 in Section 4.1.1 of R1-240xxxx (for TS 38.214 clause 8.1.4A), for correction on IUC in co-channel coexistence case, and the Draft CR is not pursued in Rel-18. It’s RAN1’s understanding that for the generation of preferred resource set for IUC co-existence case, UEA shall mandatorily exclude resources based on both NR and LTE information per sensing procedure. |
| CATT/CICTCI | Comment | We understand the current spec is clear and works well. That is, when the LTE SL info is obtained by the UE, using this info in IUC procedure is mandatory behaviour. |
| Qualcomm | comment | RAN 1 did not work on inter-UE coordination for SL coexistence and hence this behavior should not be supported, that is why we preferred the previous version: Preferred text for Sec. 8.1.4A: "When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4, except the steps using information determined by the E-UTRA radio access, with the above parameters and the following modifications:" Is the current version clear enough that this behavior is not specified? Saying, “up to UE implementation” may appear as if it is specified. |
| Nokia | Comment | It might be better to first agree what the desired behavior is; so far it seems there are 3 options: - LTE SL information is taken into account (current spec) - LTE SL information is not taken into account - Up to UE implementation We can accept leaving it up to UE implementation (which seemed the most popular option at the previous meeting), but the proposed spec change is problematic, since the inserted text disrupts the flow and may be confusing; therefore we prefer to insert the “up to UE implementation” text as a separate sentence rather than in the middle of the existing sentence. |

- Collection of Draft proposals

  - Draft proposals for Monday’s online session

    - Issue #1: Correction on IUC in co-channel coexistence case in TS 38.214

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the Draft CR in Section 2.1.1 can be acceptable. Summary on Draft CR in Section 2.1.1: - Yes: - OPPO - ZTE - Nokia: - We can accept leaving it up to UE implementation, but the proposed spec change is problematic, since the inserted text disrupts the flow and may be confusing; therefore, we prefer to insert the “up to UE implementation” text as a separate sentence rather than in the middle of the existing sentence. - Comments: - CATT: - No change to the current specification, i.e., when the LTE SL info is obtained by the UE, using this info in IUC procedure is mandatory behavior. - Qualcomm: - RAN 1 did not work on IUC for SL coexistence and hence this behavior should not be supported. Preferred text for Sec. 8.1.4A: "When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4, except the steps using information determined by the E-UTRA radio access, with the above parameters and the following modifications:"

Draft Proposal 1-1:

- Option A) Draft CR#1-1 in Section 4.1.1 of R1-2407231 is endorsed for TS 38.214.

- Option B)

  - Conclusion:

    - There is no consensus in RAN1 to endorse Draft CR#1-1 in Section 4.1.1 of R1-2407231 (for TS 38.214 clause 8.1.4A), for correction on IUC in co-channel coexistence case, and the Draft CR is not pursued in Rel-18.

      - Note: It’s RAN1’s understanding that for the generation of preferred resource set for IUC in co-channel coexistence case, UE-A shall mandatorily exclude resources based on both NR SL and LTE SL information per sensing procedure.

- Option C)

  - Conclusion:

    - LTE SL information is not taken into account in generating the preference resource set for IUC in co-channel coexistence case.

- Collection of Draft CRs

  - Draft CRs for Monday’s online session

    - Draft CR#1-1 for “Issue #1: Correction on IUC in co-channel coexistence case in TS 38.214”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on IUC in co-channel coexistence case in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According the reply LS R1-2403573 on IUC or DRX in co-channel co-existence, SL IUC at NR SL module can be supported and can use NR SL information in NR/LTE V2X co-channel co-existence. And RAN1 does not agree that whether the LTE SL information can also be used for preferred resource set determination. And as discussed in previous meeting (FL summary R1-2405367), companies either think that the LTE SL information should not be used, or it is up to UE implementation to use the LTE SL information for preferred resource set determination. Neither of these understandings are aligned with the current specification. On the other hand, current RAN1 specification in section 8.1.4 and 8.1.4A of 38.214 imply that using of LTE SL information is mandatory for preferred resource set determination in case of dynamic co-channel coexistence, which is not aligned with the reply LS and RAN1 understandings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding text to show that whether LTE SL information should be used for IUC in co-channel co-existence is up to UE implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Misalignment between 38.214 specification and RAN1 conclusions on IUC in co-channel co-existence. 2. Make a mandatory UE behaviour which is not agreed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

<Unchanged parts omitted>

When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications, except for dynamic co-channel coexistence of LTE sidelink and NR sidelink:

-	Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met:

-	the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined;

-	the higher layer parameter sl-Condition1-A-2 is not set to 'Disabled'.

In case of dynamic co-channel coexistence of LTE sidelink and NR sidelink, it is up to UE implementation whether to apply the steps using information determined by the E-UTRA radio access.

<Unchanged parts omitted>

- References

- R1-2406676	Correction on IUC in co-existence case in TS 38.214	ZTE, Sanechips

- Appendix (outcomes of past meetings)

  - RAN1#118 (August 19th – 23rd, 2024)

Agreement

The TP below is endorsed, with further work on the reason for change.

| Reason for change: |  | According the reply LS R1-2403573 on IUC or DRX in co-channel co-existence, SL IUC at NR SL module can be supported and can use NR SL information in NR/LTE V2X co-channel co-existence. RAN1 does not agree whether the LTE SL information can also be used for preferred resource set determination. And as discussed in previous meeting (FL summary R1-2405367), companies either think that the LTE SL information should not be used, or it is up to UE implementation to use the LTE SL information for preferred resource set determination. Neither of these understandings are aligned with the current specification. On the other hand, current RAN1 specification in section 8.1.4 and 8.1.4A of 38.214 imply that using of LTE SL information is mandatory for preferred resource set determination in case of dynamic co-channel coexistence, which is not aligned with the reply LS and RAN1 understandings. |
| --- | --- | --- |
| Summary of change: |  | Adding text to show that whether LTE SL information should be used for IUC in co-channel co-existence is up to UE implementation. |
| Consequences if not approved: |  | 1. Misalignment between 38.214 specification and RAN1 conclusions on IUC in co-channel co-existence. 2. Make a mandatory UE behaviour which is not agreed. |

8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

<Unchanged parts are omitted>

When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4, it is up to UE implementation whether to use information determined by the E-UTRA radio access in case of dynamic co-channel coexistence of LTE sidelink and NR sidelink, with the above parameters and the following modifications:

-	Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met:

-	the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined;

-	the higher layer parameter sl-Condition1-A-2 is not set to 'Disabled'.

<Unchanged parts are omitted>

Agreement

The CR in R1-2407410 is endorsed.

  - RAN1#116bis (April 15th – 19th, 2024)

Agreement

Final LS R1-2403573 with the Response to RAN2 as follows based on the draft LS in R1-2403572:

Question-1: Is the SL IUC supported in co-channel co-existence?

Answer-1: There has been no special handling for SL IUC with NR/LTE V2X co-channel co-existence in RAN1. From RAN1’s perspective, according to the current specification, SL IUC at NR SL module can be supported and can use NR SL information in NR/LTE V2X co-channel co-existence. Note that RAN1 will not consider any specific optimization for SL IUC with NR/LTE V2X co-channel co-existence.

Question-2: Is the SL DRX supported in co-channel co-existence?

Answer-2: There has been no special handling for SL DRX with NR/LTE V2X co-channel co-existence in RAN1. From RAN1’s perspective, according to the current specification, SL DRX at NR SL module can be supported in NR/LTE V2X co-channel co-existence. Note that RAN1 will not consider any specific optimization for SL DRX with NR/LTE V2X co-channel co-existence.

  - RAN1#116 (February 26th – March 1st, 2024)

Conclusion

For further enhancement of performing stepwise initialization of NR SL candidate resource set in Step 5a, RAN1 does not pursue it in Rel-18.

Conclusion

RAN1 does not pursue further clarification on determining the first overlapping NR SL slot for NR SL transmission of 30kHz SCS with dynamic resource pool sharing.

Agreement

Text Proposal 3 (I) in Section 4.1.2 of R1-2401506 is endorsed for TS 38.214 clause 8.

  - RAN1#115 (November 13th – 17th, 2023)

Conclusion

There is no consensus in RAN1 to endorse Text Proposal 2 (I) in Section 4.1.2 of R1-2312259 (for TS 38.214 clause 8.1.4) for capturing the information shared by the E-UTRA radio access, and the TP is not pursued in Rel-18.

Conclusion

There is no consensus in RAN1 to endorse Text Proposal 3 (I) in Section 4.1.3 of R1-2312259 (for TS 38.214 clause 8.1.4), for the conditions under which NR SL candidate resource exclusion based on LTE SL own transmissions is performed in Step 5LTE2, and the TP is not pursued in Rel-18.

Conclusion

There is no consensus in RAN1 to endorse Text Proposal 5 (I) in Section 4.1.4 of R1-2312259 (for TS 38.214 clause 8.1.4), for performing exclusion of NR SL candidate resource overlapping with LTE PSCCH resources for non-adjacent LTE PSCCH/PSSCH resources in Step 5LTE1, and the TP is not pursued in Rel-18.

Conclusion

There is no consensus in RAN1 to endorse Text Proposal 6 (I) in Section 4.1.5 of R1-2312259 (for TS 38.214 clause 8.1.4), for clarification of LTE SCI format 1 being received by LTE SL module in Step 5LTE3, and the TP is not pursued in Rel-18.

Conclusion

There is no consensus in RAN1 to endorse Text Proposal 12 (I) in Section 4.1.11 of R1-2312259 (for TS 38.214 clause 8.1.4), for further enhancement of performing pre-emption of NR SL resources overlapping with LTE SL resources selected to be used by its own LTE SL module, and the TP is not pursued in Rel-18.

Agreement

Text Proposal 1 (II) in Section 4.2.1 of R1-2312260 is endorsed for TS 38.214 clause 8.1.4.

Conclusion

RAN1 does not pursue specific enhancement on alignment between reserved slots of NR SL and reserved subframes of LTE SL.

Conclusion

RAN1 does not pursue specific enhancement on alignment of SFN/DFN indices between LTE SL and NR SL for dynamic resource pool sharing.

Conclusion

RAN1 does not pursue specific enhancement on further exclusion of NR SL candidate resources of which PSFCH slots overlapping with its own LTE SL resources for dynamic resource pool sharing.

Conclusion

RAN1 does not pursue specific enhancement on Q formula to derive periodic LTE SL resources for dynamic resource pool sharing.

  - RAN1#114bis (October 9th – 13th, 2023)

Agreement

Text Proposal 1 (I) in Section 4.1.1 of R1-2309240 is endorsed for TS 38.214 clause 8.1.4.

Agreement

Text Proposal 3 (II) in Section 4.1.3 of R1-2309240 for TS 38.214 clause 8.1.4 is endorsed with the following change:

- The UE shall perform the procedures in 5LTE1, 5LTE3 and 6LTE based on the information for these LTE subframes which is known to the NR radio access at the latest T msec prior to slot n.

Agreement

Endorse the TP below

- Reason for change:

- The agreement of “The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe” is not specified.

- Summary of change:

- The first symbol of the earlier overlapping NR SL slot and the first symbol of the overlapping LTE SL subframe are aligned in boundary.

- Consequences if not approved:

- It may result in an AGC issue and performance degradation of LTE SL reception for the case when the SCS of NR SL is 30kHz.

---------------- Start of Text Proposal for TS 38.214 ----------------------------- 8. Physical sidelink shared channel related procedures < Unchanged parts are omitted > A UE is not expected to use the last NPRB mod nsubCHsize PRBs in the resource pool. In case of dynamic co-channel coexistence of LTE sidelink and NR sidelink, for NR sidelink transmissions in 30kHz SCS, the UE expects that the start of the first symbol of the earlier overlapping NR SL slot is aligned with the start of the first symbol of the overlapping LTE SL subframe. < Unchanged parts are omitted > ---------------- End of Text Proposal for TS 38.214 ------------------------------

Agreement

Text Proposal 4-2 (I) in Section 4.1.4 of R1-2309240 is endorsed for TS 38.213 clause 16.7.

Agreement

To enable or disable the NR SL resource (re)selection with the dynamic resource pool sharing,

- Add following red changes to the description of sl-NRPSSCH-EUTRA-ThresRSRP-List:

  - Indicates a list of 64 thresholds, and a threshold should be selected based on the priority in the decoded EUTRA SCI and the priority in the NR SCI to be transmitted. A NR SL resource is excluded if it is indicated or reserved by the decoded EUTRA SCI and EUTRA PSSCH RSRP in the associated data resource is above the threshold. If the field is present, the UE shall perform the dynamic co-channel coexistence of LTE sidelink and NR sidelink as specified in TS 38.214; otherwise it shall not perform it.

Agreement

Endorse the TP below:

- Reason for change:

- To align terminology between PHY and MAC specifications

- Summary of change:

- In Step of 5LTE2, “LTE V2X” is replaced with “LTE V2X sidelink” in Step 5LTE2.

- Consequences if not approved:

- No consistency between PHY and MAC specifications.

---------------- Start of Text Proposal for TS 38.214 ----------------------------- ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 < Unchanged parts are omitted > 5LTE2) In case of dynamic co-channel coexistence of LTE sidelink and NR sidelink: The UE shall exclude any candidate single-slot resource Rx,y from the set SA if all the following conditions are met: - the UE has a selected sidelink grant for LTE V2X sidelink according to [19, TS 36.321]. - the selected sidelink grant for LTE V2X sidelink determines the set of LTE resource blocks and LTE subframes which overlaps in time with Rx,y+j×PrsvpTX' for j=0, 1, …, Cresel-1; - the priority value associated with the selected sidelink grant for LTE V2X sidelink is lower than prioTX; It is up to UE implementation whether or not to apply this exclusion step if the priority value associated with selected sidelink grant for LTE V2X sidelink is higher than or equal to prioTX . < Unchanged parts are omitted > ---------------- End of Text Proposal for TS 38.214 ------------------------------

Agreement

Text Proposal 7 (I) in Section 4.1.6 of R1-2309240 is endorsed for TS 38.214 clause 8.1.4.

Agreement

Text Proposal 2 (III) in Section 4.2.1 of R1-2309241 is endorsed for TS 38.214 clause 8.1.4.

  - RAN1#113 (May 22nd – 26th, 2023)

Agreement

The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-T_start)

  - n is the time where NR SL module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214

    - Option 1-2: T_start is 1100ms

- The ending LTE SL subframe is n-T_valid2. Up to UE implementation to additionally use results from subframe(s) later than n-T_valid2.

  - T_valid2 = T + 4 ms

Agreement

In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain:

- For determining the LTE SL periodic reserved resources by other LTE SL UE,

  - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE

- The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

  - The above procedure is applied when the following is met:

    - The SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission and the list of the initial SL RSRP thresholds can be separately (pre)configured

      - If not (pre)configured, the above SL RSRP threshold list is assumed to be set to the minus infinity dBm.

      - Note: the above RSRP threshold will not be boosted up

- Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

Agreement

When different TBs are transmitted on the NR SL slots overlapping with an LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected,

- Option 1: It is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with the LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing

Conclusion

RAN1’s understanding is that for NR SL with dynamic resource pool sharing, overlapping in time domain between LTE SLSS/PSBCH and NR SL resource pool and overlapping in time domain between NR S-SSB and LTE SL resource pool could lead to performance degradation.

  - RAN1#112bis-e (April 17th – 26th, 2023)

Agreement

In NR SL resource (re)selection procedure, for dynamic resource pool sharing, the list of initial SL RSRP thresholds is separately (pre)configured (i.e., Alt 3 in the agreement of RAN1#112 meeting) for the PHY layer of NR SL module to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE.

- FFS: whether a different initial SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH.

Agreement

In NR SL resource (re)selection procedure, for determining the LTE SL periodic reserved resources by other LTE SL UE, the formula of Q in Section 8.1.4 in TS 38.214 is used.

- FFS: whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas

Note: The reservation period and the location of the LTE SL periodic reserved resources are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213.

Agreement

In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe (in which UE has not monitored due to its transmission) in LTE SL module

- For determining the LTE SL resources associated with non-monitored subframe of LTE SL module,

  - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool

    - The same formula of Q for determining the LTE SL periodic reserved resources by other SL UE is used

    - Note: The reservation period and the location of the LTE SL resources repeated Q times in the above procedure are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213

- The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

  - FFS: whether the set SA is initialized to the set of all the candidate single-slot resources excluding NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframes when the amount of candidate single-slot resources is not sufficient as specified in Step 5a in Section 8.1.4 of TS 38.214

  - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in NR SL slots that overlap with LTE SL resource pool where NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframes of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214

  - Note: The NR SL module excludes NR SL candidate resources overlapping with NR SL resources (including NR SL resource reservation period(s) (pre)configured in a NR SL resource pool) associated with non-monitored slot as specified in Section 8.1.4 of TS 38.214 (i.e., NR SL slot in which UE has not monitored due to its own transmission), which is the existing Rel-16/17 NR SL behavior

- Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module

Agreement

In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

- For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission,

  - For determining the above LTE SL selected resources, the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count

- The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

  - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in a NR SL slot where NR SL periodic resources are in the NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214

  - Note: When the PHY layer of NR SL module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

- Alt 1: The above procedure is applied at least when the priority of LTE SL transmission is higher than the priority of NR SL transmission

  - It is up to UE implementation whether or not to apply the above procedure when the priority of LTE SL transmission is not higher than the priority of NR SL transmission

- Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module

Agreement

For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe.

- Note: How to ensure the above condition is up to UE implementation

- FFS on whether same or different frequency allocation may be used in the second overlapping slot

Agreement

When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing

- FFS: whether/how to differently handle the case when different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected.

Agreement

RAN1 does not pursue further enhancements except for the following to handle NR SL candidate resources overlapping with LTE PSCCH for non-adjacent LTE SL resource pool in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18.

- NR SL module uses LTE SL RSRP measurement results (as specified in TS 36.213) shared from LTE SL module to determine excluding NR SL candidate resources overlapping with LTE PSCCH and/or LTE PSSCH resources reserved by other LTE SL UE in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18.

Agreement

In NR SL resource (re)selection procedure, the PHY layer of NR SL module excludes NR SL candidate resources where the corresponding PSFCH transmission occasions overlap with LTE SL reserved resources by other LTE SL UE in time domain,

- Down-selection one of followings:

  - Option 1:

    - When the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission

      - The list of the above initial SL RSRP thresholds is separately (pre)configured

- FFS: whether this initial SL RSRP threshold list can be (pre)configured per subset of PSFCH resources

    - For determining the LTE SL periodic reserved resources by other LTE SL UE,

      - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE

    - The PHY layer of NR SL module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214

      - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE and the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214

    - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

  - Option 2:

    - For determining the LTE SL periodic reserved resources by other LTE SL UE,

      - Reuse the same mechanism as in NR SL resource (re)selection procedure excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE

    - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

      - FFS: whether to apply the above procedure based on the priority of the LTE SL reserved resources and/or the priority of the NR SL transmission

      - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of NR SL periodic resources when the corresponding PSFCH transmission occasions are overlapping with LTE SL reserved resources by other LTE SL UE in time domain according to Step 5 in Section 8.1.4 of TS 38.214.

    - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

Agreement

The NR SL module uses the information from the starting LTE SL subframe to the ending LTE SL subframe in the shared information from the LTE SL module.

- The starting LTE SL subframe is no later than the time (n-T_start)

  - n is the time where NR module triggers its NR SL resource (re)selection procedure as defined in clause 8.1.4 of TS 38.214

  - Down-selection one of followings for T_start:

    - Option 1-1: T_start is T_0 as defined in clause 8.1.4 of TS 38.214

    - Option 1-2: T_start is 1100ms

    - Option 1-3: T_start is up to UE implementation

- The ending LTE SL subframe is no earlier than the time (n-T_valid2)

  - Down-selection one of followings for T_valid2:

    - Option 2-1: T_valid2 is T

    - Option 2-2: T_valid2 <= T+4ms

    - Option 2-3: T_valid2 is up to UE implementation

  - RAN#99 (March 20th – 23rd, 2023)

Agreement

Solution guidance in order to minimize WG efforts to support 15/30kHz SCSs for NR SL with dynamic pool sharing in Rel-18.

- RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz.

  - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer

    - No change to the R16/17 resource allocation procedure in PHY due to this restriction

    - The existing SL slot structure from Rel-16 is unchanged

    - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe

  - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain

    - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered.

  - Mode 2 operation only

Agreement

WGs are tasked to complete Co-Ex objective (#4) by June 2023.

SL CA objective (#1) starts in RAN1 from RAN#100 or earlier if the work for Co-Ex completes before June 2023.

SL CA objective (#1) starts in RAN2/4 from RAN#99.

The work on Co-Ex and SL CA are not prioritized over SL-U and FR2 beam management

  - RAN1#112 (February 27th – March 3rd, 2023)

Agreement

In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE

- Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission

  - For the list of the above initial SL RSRP threshold, down-select one of followings:

    - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool

    - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool

    - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing

    - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing

      - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH

  - For the LTE SL periodic reserved resources by other LTE SL UE,

    - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period

      - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used

  - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214

    - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214

  - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module

  - FFS whether/how LTE SL RSRP is applied

Working assumption

For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation:

- Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH

  - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission

- Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH

  - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH

- Option 3-1: Additional PSFCH periodicity(ies) is introduced

  - Alt 3-1-a: 10 logical slots

  - Alt 3-1-b: 8 logical slots

  - Alt 3-1-c: 5 logical slots

  - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration

- Option 4-1: PSFCH resources and LTE SL resources are TDMed

  - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain

Note: selecting a single option is not precluded

Agreement

In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

- Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

  - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission,

    - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count

  - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214

    - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214

    - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

  - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission

  - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module

- Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

  - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

  - RAN1#111 (November 14th – 18th, 2022)

Agreement

Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms.

Agreement

For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where

- The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer.

  - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information.

  - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214.

  - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

  - RAN1#110bis-e (October 10th – 19th, 2022)

Agreement

For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected:

- Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs

- SL RSRP measurement results

- Resource reservation periods based on decoded SCI and for own LTE SL transmissions

- Priority based on decoded SCI and for own LTE SL transmissions

- Time and frequency location of resources used for own LTE SL transmissions

- Candidate resource set SA or SB

- SL RSSI measurements

- LTE logical subframe related information

- Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE

Agreement

For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission.

- FFS: which layer carries out the resource determination: PHY layer or MAC layer.

Agreement

For dynamic resource pool sharing, where the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, continue studying the following alternatives:

- Alt 1: The LTE SL module provides the NR SL module with the candidate information (excluding at least the candidate resource sets SA or SB)

  - The NR SL module identifies a set of resources based on information shared by the LTE SL module.

    - FFS: how to identify the set of resources

  - The NR SL module excludes these identified resources from its own candidate resource set when performing the resource (re)selection procedure.

  - The exclusion process is performed in the PHY layer.

  - Note: implementation of Alt 1 should not have specification impact to LTE

- Alt 2: The LTE SL module provides the NR SL module with the candidate resource sets SA or SB shared by the LTE SL module

  - The LTE PHY SL module is provided information from the higher layer to generate a candidate resource set SA or SB. The resource set SA or SB is then shared to NR SL module.

  - The NR SL module performs an intersection operation with the candidate resource set received from the LTE SL module and the candidate resource set generated by the NR SL module.

    - FFS: how to handle the case where this results in an insufficient set of resources

  - The intersection operation is performed in the MAC layer.

  - FFS: How to handle NR V2X parameter settings that are not supported by LTE V2X, e.g., periodicities, sub-channel sizes, etc

  - Note: implementation of Alt 2 should not have specification impact to LTE

- In the next meeting strive to decide between the two alternatives

Agreement

For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission.

- T is defined using

  - T ≤ Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence.

    - FFS: Value of Tmax

- FFS: any discussion on the earliest information, if needed

  - RAN1#110 (August 22nd – 26th, 2022)

Working assumption

Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

Conclusion

For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs.

- Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning.

- Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe

- FFS: potential enhancements for synchronization can be further investigated

Agreement

For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints:

- NR SL resource pool is configured with 15 kHz SCS.

  - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools

- For NR PSFCH (if configured), at least the following alternatives are studied:

  - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions.

    - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH.

  - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots.

    - FFS: periodicities of the set.

  - RAN1#109-e (May 9th – 20th, 2022)

Agreement

For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed.

Agreement

For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.

- FFS: UE dropping model

Agreement

For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.

- FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C).

Agreement

For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:

- UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.

- UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}.

Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions.

Agreement

Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

Agreement

For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,

- For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module.

  - FFS details on how the NR SL module uses this information.

  - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.

- FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.

- FFS: Whether/how device type B should be supported.