3GPP TSG RAN WG1 #118		R1-2407520

Maastricht, NL, August 19th – 23rd, 2024

Source:	Moderator (OPPO)

Title:	TPs to capture the outputs of 9.1.3.3

Agenda Item:	9.1

Document for:	Discussion and Decision

- Introduction

Purpose: Just to collect offline comment and to prepare for the CR

Principle: Copy and paste RAN1 agreements/conclusions/observation

- Minor wording change highlighted by Yellow to adapt to the context

# Draft TP

## TP1

--------------------------------------------------------Text omitted --------------------------------------------------------- ### 4.2.1 LCM Flavours The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Legacy 3GPP framework of feature is taken as a starting point. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases. - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Additionally, the following The other options are proposed for model identification type B by some companies during the discussion: - MI-Option 4: Model identification via standardization of reference models (for CSI compression) . MI-Option 4 refers to the inter-vendor collaboration Option 1 of AI-based CSI compression - MI-Option 5: Model identification via model monitoring Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects: - Relationship between model ID and data collection related configuration(s) and/or indication(s) - Information transmitted from NW to UE (if any) - Information transmitted from UE to NW (if any) - The associated procedure - Usage/Applicable use case(s) of MI-Option 1 - Note: whether MI-Option 1 is needed or not is a separate discussion. From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE. In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), are studied, where the applicable models may be a subset of all identified models. Applicable models can be reported by the UE. How to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature is to be studied. Note: it does not preclude any existing solutions. For functionality/model-ID based LCM, once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations. ---------------------------------------------------------Text omitted ---------------------------------------------------------

## TP2

----------------------------------------------------------Text omitted --------------------------------------------------------- ### 4.2.3 Additional conditions For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion. For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied Regarding the associated ID [defined in Section 4.2.1] for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). -----------------------------------------------------------Text omitted ---------------------------------------------------------

## TP3

---------------------------------------------------------Text omitted --------------------------------------------------------- ## 4.3 Collaboration levels In this clause, various levels of collaboration between UE and Network are identified as found pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange for comparison purposes - Various levels of UE/Network collaboration targeting at separate or joint ML operation The following Network-UE collaboration levels are considered as one aspect for defining collaboration levels 1. Level x: No collaboration. 2. Level y: Signalling-based collaboration without model transfer. Note: this level includes cases without model delivery. 3. Level z: Signalling-based collaboration with model transfer. Level x/y boundary is understood such as Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Level y/z boundary is defined based on whether model delivery over the air interface is done in a non-transparent manner to 3GPP signalling. Note: procedures other than model transfer/delivery are decoupled with collaboration Level y-z. Table 4.3-1 introduces different options for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models: Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.. When a model of a known structure at UE (e.g., Case z4) is transferred from the Network, the new model being identified (e.g., via Type B2) has the same structure as a previously identified model at the Network and UE. For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue. From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion. From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information ----------------------------------------------------------Text omitted ---------------------------------------------------------

## TP4

----------------------------------------------------------Text omitted --------------------------------------------------------- # 8 Conclusions The following aspects have been studied for the general framework of AI/ML over air interface for one-sided models and two-sided models: - Various Network-UE Collaboration Levels - Functionality-based LCM and model-ID-based LCM - Functionality/model selection, activation, deactivation, switching, fallback - Functionality identification and model identification - Data collection - Performance monitoring - Various model identification Types and their use cases - Reporting of applicable functionalities/models - Method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE - Model delivery/transfer and analysis of various model delivery/transfer Cases The above studied aspects for General Framework can be considered for developing/specifying AI/ML use cases and common framework (if needed for some aspects) across AI/ML use cases. Rel-19 continues to study different options of model identification and the associated details, and achieve the following conclusions: - From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). - The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work. - The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work. Rel-19 continues to study the details of model delivery/transfer cases and analysis their pros and cons, and achieves the following conclusions - From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). - From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration - From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network - From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. RAN1 is recommending extending the study of the Mmodel identification, and Mmodel transfer/Mmodel delivery based on RAN1 understanding the study is not completed. CSI feedback enhancement: CSI compression sub use case: The performance benefit and potential specification impact were studied for AI/ML based CSI compression sub use case. Evaluation has been performed to assess AI/ML based CSI compression from various aspects, including performance gain over non-AI/ML benchmark, model input/output type, CSI feedback quantization methods, ground-truth CSI format, monitoring, generalization, training collaboration types, etc. Some aspects were studied but not fully investigated, including the options of CQI/RI calculation, the options of rank>1 solution. -----------------------------------------------------------Text omitted ---------------------------------------------------------

## Comments/Inputs

Companies can provide comments/inputs in the following table:

| Company | Comment |
| --- | --- |
| Mod | In general, moderator just copy and paste RAN1 agreements/conclusions/observation. Minor wording change highlighted by Yellow to adapt to the context |
| Mod2 | The agreement made in Thursday’s session is captured in TP4 |

# Summary

## TP1

--------------------------------------------------------Text omitted --------------------------------------------------------- ### 4.2.1 LCM Flavours The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Legacy 3GPP framework of feature is taken as a starting point. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. RAN1 studies the model identification type A with more details related to use cases. RAN1 studies the following options as starting point for model identification type B with more details related to all use cases. - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose Additionally, the following options are proposed for model identification type B by some companies: - MI-Option 4: Model identification via standardization of reference models (for CSI compression) . MI-Option 4 refers to the inter-vendor collaboration Option 1 of AI-based CSI compression - MI-Option 5: Model identification via model monitoring Regarding MI-Option 1 (Model identification with data collection related configuration(s) and/or indication(s)) of model identification type B, RAN1 further study the following aspects: - Relationship between model ID and data collection related configuration(s) and/or indication(s) - Information transmitted from NW to UE (if any) - Information transmitted from UE to NW (if any) - The associated procedure - Usage/Applicable use case(s) of MI-Option 1 - Note: whether MI-Option 1 is needed or not is a separate discussion. From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset In functionality-based LCM, network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signalling (e.g., RRC, MAC-CE, DCI). Models may not be identified at the Network, and UE may perform model-level LCM. Whether and how much awareness/interaction NW should have about model-level LCM requires further study. For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE. In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. After model identification, necessity, mechanisms, for UE to report updates on applicable UE part/UE-side model(s), are studied, where the applicable models may be a subset of all identified models. Applicable models can be reported by the UE. How to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature is to be studied. Note: it does not preclude any existing solutions. For functionality/model-ID based LCM, once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring. Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations. ---------------------------------------------------------Text omitted ---------------------------------------------------------

## TP2

----------------------------------------------------------Text omitted --------------------------------------------------------- ### 4.2.3 Additional conditions For an AI/ML-enabled feature/FG, additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. It does not imply that additional conditions are necessarily specified. Additional conditions can be divided into two categories: NW-side additional conditions and UE-side additional conditions. Note: whether specification impact is needed is a separate discussion. For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied Regarding the associated ID [defined in Section 4.2.1] for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). -----------------------------------------------------------Text omitted ---------------------------------------------------------

## TP3

---------------------------------------------------------Text omitted --------------------------------------------------------- ## 4.3 Collaboration levels In this clause, various levels of collaboration between UE and Network are identified as found pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange for comparison purposes - Various levels of UE/Network collaboration targeting at separate or joint ML operation The following Network-UE collaboration levels are considered as one aspect for defining collaboration levels 1. Level x: No collaboration. 2. Level y: Signalling-based collaboration without model transfer. Note: this level includes cases without model delivery. 3. Level z: Signalling-based collaboration with model transfer. Level x/y boundary is understood such as Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE. (Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.) Level y/z boundary is defined based on whether model delivery over the air interface is done in a non-transparent manner to 3GPP signalling. Note: procedures other than model transfer/delivery are decoupled with collaboration Level y-z. Table 4.3-1 introduces different options for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models: Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.. When a model of a known structure at UE (e.g., Case z4) is transferred from the Network, the new model being identified (e.g., via Type B2) has the same structure as a previously identified model at the Network and UE. For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue. From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion. From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information ----------------------------------------------------------Text omitted ---------------------------------------------------------

## TP4

----------------------------------------------------------Text omitted --------------------------------------------------------- # 8 Conclusions The following aspects have been studied for the general framework of AI/ML over air interface for one-sided models and two-sided models: - Various Network-UE Collaboration Levels - Functionality-based LCM and model-ID-based LCM - Functionality/model selection, activation, deactivation, switching, fallback - Functionality identification and model identification - Data collection - Performance monitoring - Various model identification Types and their use cases - Reporting of applicable functionalities/models - Method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE - Model delivery/transfer and analysis of various model delivery/transfer Cases The above studied aspects for General Framework can be considered for developing/specifying AI/ML use cases and common framework (if needed for some aspects) across AI/ML use cases. Rel-19 continues to study different options of model identification and the associated details, and achieve the following conclusions: - From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). - The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work. - The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work. Rel-19 continues to study the details of model delivery/transfer cases and analysis their pros and cons, and achieves the following conclusions - From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). - From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration - From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network - From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. RAN1 is recommending extending the study of the model identification, and model transfer/model delivery based on RAN1 understanding the study is not completed. CSI feedback enhancement: CSI compression sub use case: The performance benefit and potential specification impact were studied for AI/ML based CSI compression sub use case. Evaluation has been performed to assess AI/ML based CSI compression from various aspects, including performance gain over non-AI/ML benchmark, model input/output type, CSI feedback quantization methods, ground-truth CSI format, monitoring, generalization, training collaboration types, etc. Some aspects were studied but not fully investigated, including the options of CQI/RI calculation, the options of rank>1 solution. -----------------------------------------------------------Text omitted ---------------------------------------------------------