3GPP TSG-RAN WG2 Meeting #128                                                               R2-2409542

Orlando, USA, November 18th– 22nd, 2024

Agenda item:	8.8.6

Source:	vivo

Title:	Further Discussion on PWS Support for NB-IoT

Document for:	Discussion and Decision

- Introduction

The PWS support for NB-IoT was discussed in RAN2#127bis [1], and the following conclusions were made.

Agreements: 1. RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS 2. The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. 3. For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields): - SystemInformationBlockType10-NB for primary ETWS notification; - SystemInformationBlockType11-NB for secondary ETWS notification; - SystemInformationBlockType12-NB for CMAS notification. 4. Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB): - etws-Indication; - cmas-Indication. 5. Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means) 6. Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

Besides, whether to support the PWS acquisition in RRC CONNECTED state was discussed but no conclusion was made.

- Continue the discussion in the next meeting(s) on whether and how to support PWS for NB-IoT NTN UEs in Connected mode

In the contribution, we will discuss the following open issues on supporting PWS for NB-IoT UEs, and give the corresponding TPs.

- FFS whether the PWS indication is added within the Paging-NB message;

- FFS what immediately means, i.e., PWS capable UE acquire the PWS message immediately upon receiving the PWS notification;

- FFS support the PWS acquisition for NB-IoT UEs in RRC CONNECTED;

- UE capability on supporting PWS for NB-IoT UEs;

- Other spec impacts on supporting the PWS for NB-IoT UEs.

- Discussion

  - PWS indication in Paging-NB message

According to 36.331[2], the system information change indication can be carried either in Paging-NB message or in the direct indication message scrambled with P-RNTI without the corresponding Paging-NB message.

Paging-NB ::= SEQUENCE { pagingRecordList-r13 PagingRecordList-NB-r13 OPTIONAL, -- Need ON systemInfoModification-r13 ENUMERATED {true} OPTIONAL, -- Need ON systemInfoModification-eDRX-r13 ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension Paging-NB-v1610-IEs OPTIONAL } Paging-NB-v1610-IEs ::= SEQUENCE { pagingRecordList-v1610 PagingRecordList-NB-v1610 OPTIONAL, -- Need ON nonCriticalExtension SEQUENCE {} OPTIONAL }

### 6.7.5 Direct Indication Information Direct Indication information is transmitted on NPDCCH using P-RNTI but without associated Paging-NB message. Table 6.7.5-1 defines the Direct Indication information, see TS 36.212 [22], clause 6.4.3.3. When bit n is set to 1, the UE shall behave as if the corresponding field is set in the Paging-NB message, see 5.3.2.3. Bit 1 is the least significant bit. Table 6.7.5-1: Direct Indication information Bit; Field in Direct Indication information. 1; systemInfoModification. 2; systemInfoModification-eDRX. 3, 4, 5, 6, 7, 8; Not used, and shall be ignored by UE if received.

Observation 1: The system information change notification for NB-IoT UEs is either indicated via the Paging-NB message or the direct indication information.

In RAN2#127bis, it was agreed to support the PWS indication in direct indication information, and some companies doubt whether we still need to introduce the PWS indication in Paging-NB message too. Before answering the question, let us review why the system information change indication needs to be introduced in both the Paging-NB message and direct indication information.

The following is extracted from 36.212[3]. It can be seen that either the direct indication information or the Paging-NB message can be scheduled by one PDCCH scrambled by P-RNTI, but not both.

#### 6.4.3.3 DCI Format N2 DCI format N2 is used for paging, direct indication, scheduling of one NPDSCH codeword carrying SC-MCCH in one cell, and notifying SC-MCCH change. The following information is transmitted by means of the DCI format N2: - If the format N2 CRC is scrambled by P-RNTI: - Flag for paging/direct indication differentiation – 1 bit, with value 0 for direct indication and value 1 for paging - Else if the format N2 CRC is scrambled by a SC-RNTI: - Information for SC-MCCH change notification – 1 bit as defined in clause 5.8a of [6] - If the format N2 CRC is scrambled by P-RNTI and Flag=0: - Direct Indication information – 8 bits provide direct indication of system information update and other fields, as defined in [6] - Reserved information bits are added until the size is equal to that of format N2 with Flag=1 - If the format N2 CRC is scrambled by P-RNTI and Flag=1, or if the format N2 CRC is scrambled by SC-RNTI: - Resource assignment – 3 bits as defined in clause 16.4.1.3 of [3] - Modulation and coding scheme – 4 bits as defined in clause 16.4.1.5 of [3] - Repetition number – 4 bits as defined in clause 16.4.1.3 of [3] - DCI subframe repetition number – 3 bits as defined in clause 16.6 of [3]

Observation 2: An NPDCCH scrambled with P-RNTI either indicates the direct indication information or schedules Paging-NB message, but not both.

Let us assume a case, i.e., system information change indication needs to be set, and at the same time there is MT service arrival, since there is a system information change notification in the Paging-NB message, then NW can send one Paging-NB message with the system information change notification “true” to provide both the system change notification and the paging for MT service to UE at the same message. This is power saving from both UE and NW perspectives. Otherwise, NW has to send separate PDCCH on different paging occasions, i.e., one for system information change notification, and one for paging UE on the MT service arrival. Besides, this will cause latency on either the system information change indication or the MT service notification. A similar issue is also present for PWS notification.

Observation 3: It is power waste and not efficient if the system information change indication or the PWS notification is only indicated within the direct indication information, especially in case both the system information change indication/PWS notification and the MT service indication need to be sent to UE.

This is different from the design of NR, where the short message indicator is included within the DCI scrambled with P-RNTI. And depending on the indication of the short message indicator, only the paging message or only the short message or both of paging and short message can be scheduled by the same DCI. Thus only adding the system information change indication or the PWS indication within the short message is enough to handle the case where both the system information change indication/PWS notification and the MT service indication arrive at the same time, since NW can send one PDCCH scrambled with P-RNTI including both of the short message and Paging message at the same time.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: - Short Messages Indicator - 2 bits according to Table 7.3.1.2.1-1. - Short Messages - 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - Frequency domain resource assignment - bits. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - is the size of CORESET 0 - Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - TB scaling - 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message, and TRS availability indication if trs-ResourceSetConfig is configured, are carried, this bit field is reserved. - TRS availability indication - 1, 2, 3, 4, 5, or 6 bits, where the number of bits is equal to one plus the highest value of all the indBitID(s) provided by the trs-ResourceSetConfig if configured; 0 bits otherwise. - Reserved bits - (8 - M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 - M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above. ……Skip unrelated part…… Table 7.3.1.2.1-1: Short Message indicator Bit field; Short Message indicator. 00; Reserved. 01; Only scheduling information for Paging, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI. 10; Only short message, and TRS availability indication if trs-ResourceSetConfig is configured, are present in the DCI. 11; Both scheduling information for Paging, TRS availability indication if trs-ResourceSetConfig is configured and short message are present in the DCI.

Observation 4: One DCI scrambled by P-RNTI in NR can schedule both of the paging message and short message at the same time.

Observation 5: The paging design of LTE is not efficient, and it cannot behave like NR, i.e., only add the system information change indication/PWS indication within the short message in NR.

Based on the analysis above,  adding the PWS notification in the Paging-NB message in LTE is power-saving and efficient to deliver the PWS notification/MT service arrival indication to UE, in case the PWS notification and the paging for MT service arrive at the same time. Besides, this is also consistent with the current designn of system information change indication for NB-IoT UE.

Proposal 1: Add the following PWS indication, i.e.,  etws-Indication and cmas-Indication in Paging-NB message.

  - Clarification on what “immediately” means

According to the RRC spec quoted below, the modification period does not apply for PWS, and PWS message acquisition upon receiving the PWS notification is different from the other system information, i.e., UE shall start to acquire the corresponding PWS message immediately upon receiving the notification regardless of the modification period.

5.2.1.3 System information validity and notification of changes Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite assistance information parameters except for discontinuous coverage scenarios and for NB-IoT, other than for AB parameters and satellite assistance information parameters except for discontinuous coverage scenarios) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. For NB-IoT, H-SFN is always provided and the modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0.

5.2.2.4 System information acquisition by the UE The UE shall: 1> apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8; 1> if the procedure is triggered by a system information change notification: 2> if the UE uses an idle DRX cycle longer than the modification period: 3> start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary; 2> else 3> start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received; NOTE 1: The UE continues using the previously received system information until the new system information has been acquired. -------------------------<the unrelated part is omitted>----------------------- 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11; 2> when the UE acquires SystemInformationBlockType1 following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> if schedulingInfoList indicates that SystemInformationBlockType10 is present: 4> if the UE is in CE: 5> start acquiring SystemInformationBlockType10; 4> else 5> start acquiring SystemInformationBlockType10 immediately; 3> if schedulingInfoList indicates that SystemInformationBlockType11 is present: 4> start acquiring SystemInformationBlockType11 immediately; NOTE 2: UEs shall start acquiring SystemInformationBlockType10 and SystemInformationBlockType11 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed. 1> if the UE is CMAS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment; 2> when the UE acquires SystemInformationBlockType1 following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment: 3> if schedulingInfoList indicates that SystemInformationBlockType12 is present: 4> acquire SystemInformationBlockType12; NOTE 3: UEs shall start acquiring SystemInformationBlockType12 as described above even when systemInfoValueTag in SystemInformationBlockType1 has not changed. -------------------------<the unrelated part is omitted>-----------------------

Observation 6: The modification period does not apply to PWS message, i.e., the PWS can be sent immediately after the PWS notification.

Observation 7: Upon receiving the PWS notification, legacy PWS-capable UE shall acquire the corresponding PWS message directly regardless of the modification period.

Considering the urgency of the PWS, similar UE behaviours for LTE UEs/BL UEs/UEs in CE upon receiving the PWS notification should also be taken for NB-IoT UEs upon receiving the PWS notification.

Proposal 2: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (same as the legacy PWS-capable LTE UE).

  - PWS acquisition for NB-IoT UEs in RRC CONNECTED

In RAN2#127bis, some companies propose to make similar enhancements like CE UEs in Rel-16 so as to support the PWS acquisition for NB-IoT UEs in RRC CONNECTED. If we do such enhancements, RAN1 will be impacted, i.e., change the indication of the DCI to support the PWS indication in RRC CONNECTED state.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

According to the WID of the PWS [4], only RAN2 is impacted, that’s because the original intention is to support the PWS for NB-IoT UEs only in RRC IDLE state.

Observation 8: The intention of the WID is to only support the PWS for NB-IoT in RRC IDLE state.

Besides, supporting the PWS acquisition in RRC CONNECTED is not such urgent, since the benefit of supporting the PWS in RRC CONNECTED state is also not clear. Besides, if there is PWS notification, up to NW implementation, NW can release the UE to RRC IDLE, and then UE can acquire the PWS.

Proposal 3: Not support the PWS acquisition for NB-IoT UEs in RRC CONNECTED.

  - UE capability on PWS

In legacy (before Rel-19), the PWS is not supported for NB-IoT, so the ETWS/CMAS capabilities are not applicable for NB-IoT UEs. The following are extracted from TS36.306 [5].

4 UE radio access capability parameters -------------------------<the unrelated part is omitted>----------------------- The following optional features without UE radio access capability parameters specified in clause 6 are applicable in NB-IoT: - RRC Connection Re-establishment for the Control Plane CIoT EPS Optimization (clause 6.7.5) - System Information Block Type 16 (clause 6.8.1) - Enhanced random access power control (clause 6.8.3) - MT-EDT for Control Plane CIoT EPS Optimisation (clause 6.8.10) - MT-EDT for User Plane CIoT EPS Optimisation (clause 6.8.11) - EDT for Control Plane CIoT EPS Optimization (clause 6.8.4) - Enhanced PHR (clause 6.8.6) - Carrier specific NRSRP thresholds for NPRACH resource selection (clause 6.8.15) - Radio Link Failure Report for NB-IoT (clause 6.10.2) - SC-PTM in Idle mode (clause 6.16.1) - Multiple TB scheduling for SC-PTM in Idle mode for NB-IoT (clause 6.16.2) - Relaxed monitoring (clause 6.17.1) - DL channel quality reporting in Msg3 for the anchor carrier (clause 6.17.2) - Serving cell idle mode measurements reporting (clause 6.17.3) - NSSS-Based RRM measurements (clause 6.17.4) - NPBCH-Based RRM measurements (clause 6.17.5) - RRM measurements on non-anchor paging carriers (clause 6.17.6) - NRS presence on non-anchor paging carriers (clause 6.17.7) - DL channel quality reporting in Msg3 for non-anchor carrier (clause 6.17.8) - Assistance information for inter-RAT cell selection to/from NB-IoT (clause 6.17.9) - RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation (clause 6.18.3) - NB-IoT/5GC (clause 6.18.4) - MO-EDT for Control Plane CIoT 5GS Optimisation (clause 6.18.5) - AS RAI (clause 6.18.6) - Cell Reselection Measurements Triggering based on Service Time (clause 6.19.1) - Discontinuous coverage (clause 6.19.2). The optional features without UE radio access capability parameters specified in clause 6 are not applicable in NB-IoT, unless they are listed above.

Now, since we are introducing the support for PWS, it is proposed to support the PWS-related UE capabilities (including ETWS, CMAS, KPAS, and EU-Alert) for NB-IoT UEs, considering the existing PWS feature support quoted below.

6.2 PWS features 6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. 6.2. CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5]. 6.2.3 KPAS It is optional for UE to support KPAS reception as specified in TS 36.331 [5]. The Korean Public Alert System (KPAS) uses the same AS mechanisms as defined for CMAS. Therefore a KPAS-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE. 6.2.4 EU-Alert It is optional for UE to support EU-Alert reception as specified in TS 36.331 [5]. The Europearn Union Warning System EU-Alert uses the same AS mechanisms as defined for CMAS. Therefore a EU-Alert-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE.

Proposal 4: Add the support of PWS capabilities for NB-IoT in TS 36.306.

  - Other spec impacts on supporting PWS

Except for the above issues as we listed, there are also some restrictions on the usage of PWS for NB-IoT UEs in TS36.331 [2]. The following are some examples extracted from the spec.

5.2.1.3 System information validity and notification of changes -------------------------<the unrelated part is omitted>----------------------- In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22]. -------------------------<the unrelated part is omitted>----------------------- ETWS and/or CMAS capable UEs in RRC_CONNECTED, other than BL UEs and UEs in CE, shall attempt to read paging at least once every defaultPagingCycle to check whether ETWS and/or CMAS notification is present or not. -------------------------<the unrelated part is omitted>-----------------------

5.3.2.1 General -------------------------<the unrelated part is omitted>----------------------- The purpose of this procedure is: - to transmit CN initiated paging information to a UE in RRC_IDLE or RRC_INACTIVE and/ or; - to transmit RAN initiated paging information to a UE in RRC_INACTIVE and/or; - to inform UEs in RRC_IDLE, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a system information change and/ or; - to inform UEs in RRC_IDLE other than NB-IoT UEs, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about an ETWS primary notification and/ or ETWS secondary notification and/ or; - to inform UEs in RRC_IDLE other than NB-IoT UEs, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a CMAS notification and/ or; - to inform UEs other than NB-IoT UEs in RRC_IDLE, and other than UEs connected to 5GC about an EAB parameters modification and/ or; - to inform UEs other than NB-IoT UEs in RRC_IDLE, and UEs in RRC_INACTIVE to perform E-UTRAN inter-frequency redistribution procedure. -------------------------<the unrelated part is omitted>-----------------------

To support the PWS for NB-IoT, such restrictions on the usage of PWS for NB-IoT should be removed from the stage 3 spec.

Proposal 5: The restrictions on usage of PWS for NB-IoT should be removed from TS 36.331.

To reflect all the above proposals to support the PWS for NB-IoT, the corresponding TPs on TS 36.300, TS 36.306, and T S36.331 are provided in Annex 1, Annex 2, and Annex 3, respectively. RAN2 can take these TPs as the baseline.

Proposal 6: RAN2 to take the TPs in Annex as the baseline for the support of PWS in NB-IoT.

- Conclusion

In this contribution, we have further discussed the PWS support for IoT-NTN. And we make the following observations and proposals:

PWS indication in Paging-NB message:

Observation 1: The system information change notification for NB-IoT UEs is either indicated via the Paging-NB message or the direct indication information.

Observation 2: An NPDCCH scrambled with P-RNTI either indicates the direct indication information or schedules Paging-NB message, but not both.

Observation 3: It is power waste and not efficient if the system information change indication or the PWS notification is only indicated within the direct indication information, especially in case both the system information change indication/PWS notification and the MT service indication need to be sent to UE.

Observation 4: One DCI scrambled by P-RNTI in NR can schedule both of the paging message and short message at the same time.

Observation 5: The paging design of LTE is not efficient, and it cannot behave like NR, i.e., only add the system information change indication/PWS indication within the short message in NR.

Proposal 1: Add the following PWS indication, i.e.,  etws-Indication and cmas-Indication in Paging-NB message.

Clarification on what “immediately” means:

Observation 6: The modification period does not apply to PWS message, i.e., the PWS can be sent immediately after the PWS notification.

Observation 7: Upon receiving the PWS notification, legacy PWS-capable UE shall acquire the corresponding PWS message directly regardless of the modification period.

Proposal 2: Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (same as the legacy PWS-capable LTE UE).

PWS acquisition for NB-IoT UEs in RRC CONNECTED:

Observation 8: The intention of the WID is to only support the PWS for NB-IoT in RRC IDLE state.

Proposal 3: Not support the PWS acquisition for NB-IoT UEs in RRC CONNECTED.

UE capability on PWS:

Proposal 4: Add the support of PWS capabilities for NB-IoT in TS 36.306.

Other spec impacts on supporting PWS:

Proposal 5: The restrictions on usage of PWS for NB-IoT should be removed from TS 36.331.

Proposal 6: RAN2 to take the TPs in Annex as the baseline for the support of PWS in NB-IoT.

- Reference

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- TS36.212, Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding.

- RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

- TS36.306, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities.

- TS36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2.

Annex 1 TP on TS36.300

## 7.4	System Information

System information is divided into the MasterInformationBlock (MIB) and a number of SystemInformationBlocks (SIBs):

-	MasterInformationBlock defines the most essential physical layer information of the cell required to receive further system information;

-	SystemInformationBlockPos contains positioning assistance data;

-	SystemInformationBlockType1 and SystemInformationBlockType1-BR (for a BL UE or UE in enhanced coverage) contain information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2 contains common and shared channel information;

-	SystemInformationBlockType3 contains cell re-selection information, mainly related to the serving cell;

-	SystemInformationBlockType4 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType5 contains information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters). It can also contain information about E-UTRA and NR idle/inactive measurements;

-	SystemInformationBlockType6 contains information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType7 contains information about GERAN frequencies relevant for cell re-selection (including cell re-selection parameters for each frequency);

-	SystemInformationBlockType8 contains information about CDMA2000 frequencies and CDMA2000 neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SystemInformationBlockType9 contains a home eNB name (HNB name);

-	SystemInformationBlockType10 contains an ETWS primary notification;

-	SystemInformationBlockType11 contains an ETWS secondary notification;

-	SystemInformationBlockType12 contains a CMAS warning notification;

-	SystemInformationBlockType13 contains MBMS-related information;

-	SystemInformationBlockType14 contains information about Extended Access Barring for access control;

-	SystemInformationBlockType15 contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType17 contains information relevant for traffic steering between E-UTRAN and WLAN;

-	SystemInformationBlockType18 contains information related to sidelink communication;

-	SystemInformationBlockType19 contains information related to sidelink discovery;

-	SystemInformationBlockType20 contains information related to SC-PTM;

-	SystemInformationBlockType21 contains information related to V2X sidelink communication;

-	SystemInformationBlockType24 contains information about NR frequencies and NR neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency), which can also be used for NR idle/inactive measurements;

-	SystemInformationBlockType25 contains information about UAC parameters;

-	SystemInformationBlockType26 contains additional information related to V2X sidelink communication;

-	SystemInformationBlockType26a contains information related to NR bands list which can be used for EN-DC operation with the serving cell;

-	SystemInformationBlockType27 contains assistance information for inter-RAT cell selection to NB-IoT;

-	SystemInformationBlockType28 contains information related to NR sidelink communication;

-	SystemInformationBlockType29 contains information related to common resource reservation;

-	SystemInformationBlockType30 contains information related to disaster roaming;

-	SystemInformationBlockType31 contains information required for accessing an NTN cell;

-	SystemInformationBlockType32 contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33 contains assistance information for neighbouring cells in NTN.

System information for NB-IoT is divided into the MasterInformationBlock-NB (MIB-NB) and a number of SystemInformationBlocks-NB (SIBs-NB):

-	MasterInformationBlock-NB defines the most essential information of the cell required to receive further system information;

-	SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information blocks;

-	SystemInformationBlockType2-NB contains common radio resource configuration information;

-	SystemInformationBlockType3-NB contains cell re-selection information for intra-frequency, inter-frequency;

-	SystemInformationBlockType4-NB contains neighboring cell related information relevant for intra-frequency cell re-selection;

-	SystemInformationBlockType5-NB contains neighboring cell related information relevant for inter-frequency cell re-selection;

-	SystemInformationBlockType10-NB contains an ETWS primary notification;

-	SystemInformationBlockType11-NB contains an ETWS secondary notification;

-	SystemInformationBlockType12-NB contains a CMAS warning notification;

-	SystemInformationBlockType14-NB contains information about access barring;

-	SystemInformationBlockType15-NB contains information related to mobility procedures for MBMS reception;

-	SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC);

-	SystemInformationBlockType20-NB contains information related to SC-PTM;

-	SystemInformationBlockType22-NB contains common radio resource configuration information for paging and random access procedure on non-anchor carriers;

-	SystemInformationBlockType23-NB contains common additional radio resource configuration information for random access procedure on anchor and non-anchor carriers;

-	SystemInformationBlockType27-NB contains assistance information for inter-RAT cell selection to E-UTRAN and/or GERAN;

-	SystemInformationBlockType31-NB contains information required for accessing an NTN cell;

-	SystemInformationBlockType32-NB contains assistance information for discontinuous coverage in NTN;

-	SystemInformationBlockType33-NB contains assistance information for neighbouring cells in NTN.

On MBMS-dedicated cell, only system information relevant for receiving MBMS service is broadcasted. MasterInformationBlock-MBMS (MIB-MBMS) and SystemInformationBlockType1-MBMS (SIB1-MBMS) are used instead of MIB and SIB1 respectively:

-	MasterInformationBlock-MBMS defines the most essential physical layer information of the cell required to receive further system information on MBMS-dedicated cell;

-	SystemInformationBlockType1-MBMS contains information relevant for receiving MBMS service and defines the scheduling of other system information blocks on MBMS-dedicated cell;

The MIB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and BR-BCCH, and carried on DL-SCH. Except for BL UEs, UEs in enhanced coverage and NB-IoT UEs, all other SI messages than the MIB which are dynamically carried on DL-SCH, can be identified through the SI-RNTI (System Information RNTI). Both the MIB and SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage) use a fixed schedule with a periodicity of 40 and 80 ms respectively. The scheduling of other SI messages is flexible and indicated by SystemInformationBlockType1 (SystemInformationBlockType1-BR for BL UEs and UEs in enhanced coverage, and SystemInformationBlockType1-NB for NB-IoT). For NB-IoT, the MIB-NB is mapped on the BCCH and carried on BCH while all other SI messages are mapped on the BCCH and carried on DL-SCH. Both the MIB-NB and SystemInformationBlockType1-NB use a fixed schedule with a periodicity of 640 and 2560 ms respectively. The MIB-NB contains all information required to acquire SIB1-NB and SIB1-NB contains all information required to acquire other SI messages.

On MBMS-dedicated cell, the MIB-MBMS and SIB1-MBMS use a fixed schedule with a periodicity of 160 ms. Additionally, SIB1-MBMS may be scheduled in additional non-MBSFN subframes indicated in MIB-MBMS.

For NB-IoT, in TDD mode, the MIB-TDD-NB is transmitted on the same NB-IoT carrier as NPSS/NSSS, SystemInformationBlockType1-NB can be transmitted on NB-IoT carrier other than the MIB-NB, and the SI messages can be transmitted on a NB-IoT carrier other than the MIB-NB. At most two NB-IoT carriers are used to transmit the MIB-NB, SystemInformationBlockType1-NB and the SI messages.

Except for NB-IoT, the eNB may schedule DL-SCH transmissions concerning logical channels other than BCCH or BR-BCCH in the same subframe as used for BCCH or BR-BCCH. The minimum UE capability restricts the BCCH or BR-BCCH mapped to DL-SCH e.g. regarding the maximum rate.

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. For NB-IoT UEs, BL UEs, and UEs in CE, the UE is not required to detect SIB changes when in RRC_CONNECTED, and the network may release the NB-IoT UE, BL UE or UE in CE to RRC_IDLE if it wants the NB-IoT UE, BL UE or UE in CE to acquire changed SIB(s).S

Except for NB-IoT, system information may also be provided to the UE by means of dedicated signalling e.g. upon handover.

### 23.3.1	Earthquake and Tsunami Warning System

ETWS is a public warning system developed to meet the regulatory requirements for warning notifications related to earthquake and/or tsunami events. ETWS warning notifications can either be a primary notification (short notifications delivered within 4 seconds, see TS 22.168 [37]) or secondary notification (providing detailed information). The ETWS primary notification is broadcast in SystemInformationBlockType10/SystemInformationBlockType10-NB while the secondary notification is broadcast in SystemInformationBlockType11/SystemInformationBlockType11-NB.

### 23.3.2	Commercial Mobile Alert System

CMAS is a public warning system developed for the delivery of multiple, concurrent warning notifications (see TS 22.268 [34]). The CMAS warning notifications are short text messages (CMAS alerts). The CMAS warning notifications are broadcast in SystemInformationBlockType12//SystemInformationBlockType12-NB. The E-UTRAN manages the delivery of multiple, concurrent CMAS warning notifications to the UE and is also responsible for handling any updates of CMAS warning notifications.

Annex 2 TP on TS36.331

## 4.4	Functions

The RRC protocol includes the following main functions:

-	Broadcast of system information:

-	Including NAS common information;

-	Information applicable for UEs in RRC_IDLE, e.g. cell (re-)selection parameters, neighbouring cell information and information (also) applicable for UEs in RRC_CONNECTED, e.g. common channel configuration information;

-	Including ETWS notification, CMAS notification;

-	Including positioning assistance data.

-	RRC connection control:

-	Paging;

-	Establishment/ modification/ suspension / resumption / release of RRC connection, including e.g. assignment/ modification of UE identity (C-RNTI), establishment/ modification/ suspension/ resumption/ release of SRB1, SRB1bis, SRB2 and SRB4, access class barring;

-	Initial security activation, i.e. initial configuration of AS integrity protection (SRBs) and AS ciphering (SRBs, DRBs);

-	For RNs, configuration of AS integrity protection for DRBs;

-	RRC connection mobility including e.g. intra-frequency and inter-frequency handover, associated security handling, i.e. key/ algorithm change, specification of RRC context information transferred between network nodes;

NOTE 1:	In NB-IoT, only key change (but no re-keying) at RRC Connection Resumption and RRC context information transfer are applicable.

-	Establishment/ modification/ release of RBs carrying user data (DRBs);

-	Radio configuration control including e.g. assignment/ modification of ARQ configuration, HARQ configuration, DRX configuration;

-	For RNs, RN-specific radio configuration control for the radio interface between RN and E-UTRAN;

-	In case of CA, cell management including e.g. change of PCell, addition/ modification/ release of SCell(s) and addition/modification/release of STAG(s);

-	In case of DC, cell management including e.g. change of PSCell, addition/ modification/ release of SCG cell(s) and addition/modification/release of SCG TAG(s);

-	In case of (NG)EN-DC, transparent transfer of NR RRC messages (e.g. DL: reconfiguration messages used to add or modify the NR SCG configuration or to (re-)configure measurements; configure conditional PSCell change; UL: measurement reports and reconfiguration complete messages) and of configurations of radio bearers using NR PDCP;

-	QoS control including assignment/ modification of semi-persistent scheduling (SPS) configuration information for DL and UL, assignment/ modification of parameters for UL rate control in the UE, i.e. allocation of a priority and a prioritised bit rate (PBR) for each RB (not applicable for NB-IoT);

-	Recovery from radio link failure;

-	In case of LWA, RCLWI and LWIP, WLAN mobility set management including e.g. addition/ modification/ release of WLAN(s) from the WLAN mobility set;

-	Inter-RAT mobility including e.g. security activation, transfer of RRC context information (not applicable for NB-IoT);

-	Measurement configuration and reporting (not applicable for NB-IoT):

-	Establishment/ modification/ release of measurements (e.g. intra-frequency, inter-frequency and inter- RAT measurements);

-	Setup and release of measurement gaps;

-	Measurement reporting;

-	Other functions including e.g. transfer of dedicated NAS information and non-3GPP dedicated information, transfer of UE radio access capability information, support for E-UTRAN sharing (multiple PLMN identities);

-	Generic protocol error handling;

-	Support of self-configuration and self-optimisation (not applicable for NB-IoT);

-	Support of measurement logging and reporting for network performance optimisation, as specified in TS 37.320 [60] (not applicable for NB-IoT).

NOTE 2:	Random access is specified entirely in the MAC including initial transmission power estimation.

#### 5.2.1.3	System information validity and notification of changes

Change of system information (other than for ETWS, CMAS, EAB, UAC, and satellite assistance information parameters except for discontinuous coverage scenarios and for NB-IoT, other than for AB parameters and satellite assistance information parameters except for discontinuous coverage scenarios) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. For NB-IoT, H-SFN is always provided and the modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0.

To enable system information update notification for RRC_IDLE UEs configured to use a DRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. For NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0.

NOTE 1:	If the UE in RRC_IDLE is configured to use extended DRX cycle, e.g., in the order of several minutes or longer, in case the eNB is reset the UE SFN may not be synchronized to the new eNB SFN. The UE is expected to recover, e.g., acquire MIB within a reasonable time, to avoid repeated paging failures.

NOTE 1a:	For the UE in RRC_INACTIVE, the idle mode extended DRX cycle, if configured, is used to compare with the modification period.

When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. During a modification period where ETWS or CMAS transmission is started or stopped, the SI messages carrying the SIBs scheduled in schedulingInfoListExt and/or SI messages carrying the posSIBs scheduled in posSchedulingInfoList may change, so the UE might not be able to successfully receive those SIBs and/or posSIBs in the remainder of the current modification period and next modification period according to the scheduling information received prior to the change. These general principles are illustrated in figure 5.2.1.3-1, in which different colours indicate different system information. Upon receiving a change notification, the UE not configured to use a DRX cycle that is longer than the modification period acquires the new system information immediately from the start of the next modification period. Upon receiving a change notification applicable to eDRX, a UE in RRC_IDLE configured to use a DRX cycle that is longer than the modification period acquires the updated system information immediately from the start of the next eDRX acquisition period. The UE applies the previously acquired system information until the UE acquires the new system information. The possible boundaries of modification for SystemInformationBlockType1-BR are defined by SFN values for which SFN mod 512 = 0 except for notification of ETWS/CMAS for which the eNB may change SystemInformationBlockType1-BR content at any time. For NB-IoT, the possible boundaries of modification for SystemInformationBlockType1-NB are defined by SFN values for which (H-SFN * 1024 + SFN) mod 4096 = 0.

Figure 5.2.1.3-1: Change of system Information

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE is in RRC_CONNECTED or is not configured to use a DRX cycle longer than the modification period in RRC_IDLE, and receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. A UE in RRC_IDLE that is configured to use a DRX cycle longer than the modification period, and receives in an eDRX acquisition period at least one Paging message including the systemInfoModification-eDRX, shall acquire the updated system information at the next eDRX acquisition period boundary. Although the UE may be informed about changes in system information, no further details are provided e.g. regarding which system information will change, except if systemInfoValueTagSI is received by BL UEs or UEs in CE.

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

SystemInformationBlockType1 (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) includes a value tag systemInfoValueTag, that indicates if a change has occurred in the SI messages. UEs may use systemInfoValueTag, e.g. upon return from out of coverage, to verify if the previously stored SI messages are still valid. MasterInformationBlock and RSS (if transmitted, see TS 36.211 [21]) may indicate using systemInfoUnchanged-BR that a change has not occurred in the SIB1-BR and SI messages of the current cell at least over the SI validity time, and the BL UEs or UEs in CE may use the systemInfoUnchanged-BR, e.g. upon return from out of coverage, to verify if the previously stored SIB1-BR and SI messages are still valid. Additionally, for other than BL UEs or UEs in CE or NB-IoT UEs, the UE considers stored system information to be invalid after 3 hours from the moment it was successfully confirmed as valid, unless specified otherwise. BL UE or UE in CE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid, unless the UE is configured by parameter si-ValidityTime to consider stored system information to be invalid 3 hours after validity confirmation. NB-IoT UE considers stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid. If a BL UE, UE in CE or NB-IoT UE in RRC_CONNECTED state considers the stored system information invalid, the UE shall continue using the stored system information while in RRC_CONNECTED state in the serving cell.

For BL UEs or UEs in CE or NB-IoT UEs, the change of specific SI message can additionally be indicated by a SI message specific value tag systemInfoValueTagSI. If systemInfoValueTag included in the SystemInformationBlockType1-BR (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) is different from the one of the stored system information and if systemInfoValueTagSI is included in the SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) for a specific SI message and is different from the stored one, the UE shall consider this specific SI message to be invalid. If only systemInfoValueTag is included and is different from the stored one, the BL UE or UE in CE should consider any stored system information except SystemInformationBlockType10, SystemInformationBlockType11, SystemInformationBlockType12, SystemInformationBlockType14, SystemInformationBlockType25, SystemInformationBlockType31 and SystemInformationBlockType33 to be invalid; the NB-IoT UE should consider any stored system information except SystemInformationBlockType14-NB, SystemInformationBlockType31-NB and SystemInformationBlockType33-NB to be invalid.

On MBMS-dedicated cell and on FeMBMS/Unicast-mixed cell, the change of system information and ETWS/CMAS notification is indicated by using Direct Indication FeMBMS defined in 6.6a. The modification periodicity follows MCCH modification periodicity as defined in 5.8.1.3.

E-UTRAN may not update systemInfoValueTag upon change of some system information e.g. ETWS information, CMAS information, RLOS indication (i.e., rlos-Enabled), regularly changing parameters like time information (SystemInformationBlockType8, SystemInformationBlockType16, hyperSFN-MSB in SystemInformationBlockType1-NB), EAB and AB parameters, UAC parameters, positioning system information blocks, or satellite assistance information. Similarly, E-UTRAN may not include the systemInfoModification within the Paging message upon change of some system information.

NOTE 4:	UE connected to NTN is expected to re-acquire SIB32(-NB) based on its own decision regardless of systemInfoValueTag change.

The UE that is not configured to use a DRX cycle longer than the modification period verifies that stored system information remains valid by either checking systemInfoValueTag in SystemInformationBlockType1 (or MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) after the modification period boundary, or attempting to find the systemInfoModification indication at least modificationPeriodCoeff times during the modification period in case no paging is received, in every modification period. If no paging message is received by the UE during a modification period, the UE may assume that no change of system information will occur at the next modification period boundary. If UE in RRC_CONNECTED, during a modification period, receives one paging message, it may deduce from the presence/ absence of systemInfoModification whether a change of system information other than ETWS information, CMAS information, EAB and UAC parameters will occur in the next modification period or not.

When the RRC_IDLE UE is configured with a DRX cycle that is longer than the modification period, and at least one modification period boundary has passed since the UE last verified validity of stored system information, the UE verifies that stored system information remains valid by checking the systemInfoValueTag before establishing or resuming an RRC connection.

ETWS and/or CMAS capable UEs in RRC_CONNECTED, other than BL UEs and UEs in CE and NB-IoT UEs, shall attempt to read paging at least once every defaultPagingCycle to check whether ETWS and/or CMAS notification is present or not.

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit CN initiated paging information to a UE in RRC_IDLE or RRC_INACTIVE and/ or;

-	to transmit RAN initiated paging information to a UE in RRC_INACTIVE and/or;

-	to inform UEs in RRC_IDLE, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a system information change and/ or;

-	to inform UEs in RRC_IDLE, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about an ETWS primary notification and/ or ETWS secondary notification and/ or;

-	to inform UEs in RRC_IDLE, UEs in RRC_INACTIVE and UEs in RRC_CONNECTED other than NB-IoT UEs, BL UEs and UEs in CE, about a CMAS notification and/ or;

-	to inform UEs other than NB-IoT UEs in RRC_IDLE, and other than UEs connected to 5GC about an EAB parameters modification and/ or;

-	to inform UEs other than NB-IoT UEs in RRC_IDLE, and UEs in RRC_INACTIVE to perform E-UTRAN inter-frequency redistribution procedure.

The paging information of CN initiated paging is provided to upper layers, which in response may initiate RRC connection establishment, e.g. to receive an incoming call.

### 6.7.2	NB-IoT Message definitions

#### –	Paging-NB

The Paging-NB message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: E-UTRAN to UE

Paging-NB message

-- ASN1START

Paging-NB ::=						SEQUENCE {

pagingRecordList-r13				PagingRecordList-NB-r13		OPTIONAL,	-- Need ON

systemInfoModification-r13			ENUMERATED {true}				OPTIONAL,	-- Need ON

systemInfoModification-eDRX-r13		ENUMERATED {true}				OPTIONAL,	-- Need ON

nonCriticalExtension				Paging-NB-v1610-IEs				OPTIONAL

}

Paging-NB-v1610-IEs	::=				SEQUENCE {

pagingRecordList-v1610				PagingRecordList-NB-v1610		OPTIONAL,	-- Need ON

nonCriticalExtension				Paging-NB-v19xy-IEs	OPTIONAL

}

Paging-NB-v19xy-IEs	::=				SEQUENCE {

etws-Indication-r19					ENUMERATED {true}				OPTIONAL,	-- Need ON

cmas-Indication-r19				    ENUMERATED {true}				OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

PagingRecordList-NB-r13 ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-NB-r13

PagingRecordList-NB-v1610 ::=		SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-NB-v1610

PagingRecord-NB-r13 ::=				SEQUENCE {

ue-Identity-r13						PagingUE-Identity,

...

}

PagingRecord-NB-v1610 ::=			SEQUENCE {

mt-EDT-r16							ENUMERATED {true}			OPTIONAL	-- Need ON

}

-- ASN1STOP

Paging-NB field descriptions

cmas-Indication If present: indication of a CMAS notification.

etws-Indication If present: indication of an ETWS primary notification and/or ETWS secondary notification.

mt-EDT Indication of mobile-terminated EDT.

pagingRecordList If E-UTRAN includes pagingRecordList-v1610, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

systemInfoModification If present: indication of a BCCH modification other than for SystemInformationBlockType14-NB (SIB14-NB), SystemInformationBlockType16-NB (SIB16-NB) and SystemInformationBlockType31-NB (SIB31-NB). This indication does not apply to UEs using eDRX cycle longer than the BCCH modification period.

systemInfoModification-eDRX If present: indication of a BCCH modification other than for SystemInformationBlockType14-NB (SIB14-NB), SystemInformationBlockType16-NB (SIB16-NB) and SystemInformationBlockType31-NB (SIB31-NB). This indication applies only to UEs using eDRX cycle longer than the BCCH modification period.

ue-Identity Provides the NAS identity of the UE that is being paged.

#### 6.7.3.1	NB-IoT System information blocks

–	SystemInformationBlockType10-NB

The IE SystemInformationBlockType10 contains an ETWS primary notification.

-- ASN1START

SystemInformationBlockType10-NB-r19 ::= SystemInformationBlockType10

-- ASN1STOP

–	SystemInformationBlockType11-NB

The IE SystemInformationBlockType11 contains an ETWS secondary notification.

-- ASN1START

SystemInformationBlockType11-NB-r19 ::= SystemInformationBlockType11

-- ASN1STOP

–	SystemInformationBlockType12-NB

The IE SystemInformationBlockType12 contains a CMAS notification.

-- ASN1START

SystemInformationBlockType12-NB-r19 ::= SystemInformationBlockType12-r9

-- ASN1STOP

### 6.7.5	Direct Indication Information

Direct Indication information is transmitted on NPDCCH using P-RNTI but without associated Paging-NB message. Table 6.7.5-1 defines the Direct Indication information, see TS 36.212 [22], clause 6.4.3.3.

When bit n is set to 1, the UE shall behave as if the corresponding field is set in the Paging-NB message, see 5.3.2.3. Bit 1 is the least significant bit.

Table 6.7.5-1: Direct Indication information

| Bit | Field in Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | systemInfoModification-eDRX |
| 3 | etws-Indication |
| 4 | cmas-Indication |
| 5, 6, 7, 8 | Not used, and shall be ignored by UE if received |

Annex 3 TP on TS36.306

# 4	UE radio access capability parameters

The following clauses define the UE radio access capability parameters and minimum capabilities for MBMS capable UE. Only parameters for which there is the possibility for UEs to signal different values are considered as UE radio access capability parameters. Therefore, mandatory features without capability parameters that are the same for all UEs are not listed here. Also capabilities which are optional or conditionally mandatory for UEs to implement but do not have UE radio access capability parameter are listed in this specification.

E-UTRAN needs to respect the signalled UE radio access capability parameters when configuring the UE and when scheduling the UE.

All parameters shown in italics are signalled and correspond to a field defined in TS 36.331 [5].

For optional features, the UE radio access capability parameter indicates whether the feature has been implemented and successfully tested. For mandatory features with the UE radio access capability parameter, the parameter indicates whether the feature has been successfully tested.

The mandatory features required to be supported by a UE are the same for all UE categories unless explicitly specified elsewhere in the specifications.

Unless otherwise stated, the requirements on the maximum number of transport block bits are applicable for a TTI length of 1 ms. For other TTI lengths, the requirements shall be scaled according to clause 7.1.7 or 11.1 in TS 36.213 [22] in order to get the corresponding requirement.

The following UE radio access capability parameters specified in clause 4 are applicable in NB-IoT:

-	ue-Category-NB in NB-IoT (clause 4.1C)

-	supportedROHC-Profiles-r13 (clause 4.3.1.1A)

-	maxNumberROHC-ContextSessions-r13 (clause 4.3.1.2A)

-	rlc-UM-r15 (clause 4.3.2.5)

-	multiTone-r13 (clause 4.3.4.55)

-	multiCarrier-r13 (clause 4.3.4.56)

-	twoHARQ-Processes-r14 (clause 4.3.4.62)

-	multiCarrier-NPRACH-r14 (clause 4.3.4.75)

-	multiCarrierPaging-r14 (clause 4.3.4.76)

-	interferenceRandomisation-r14 (clause 4.3.4.80)

-	wakeUpSignal-r15 (clause 4.3.4.113)

-	wakeUpSignalMinGap-eDRX-r15 (clause 4.3.4.114)

-	mixedOperationMode-r15 (clause 4.3.4.115)

-	sr-WithHARQ-ACK-r15 (clause 4.3.4.117)

-	sr-WithoutHARQ-ACK-r15 (clause 4.3.4.118)

-	nprach-Format2-r15 (clause 4.3.4.119)

-	multiCarrierPagingTDD-r15 (clause 4.3.4.134)

-	additionalTransmissionSIB1-r15 (clause 4.3.4.137)

-	npusch-3dot75kHz-SCS-TDD-r15 (clause 4.3.4.177)

-	npusch-MultiTB-r16 (clause 4.3.4.182)

-	npdsch-MultiTB-r16 (clause 4.3.4.183)

-	npusch-MultiTB-Interleaving-r16 (clause 4.3.4.192)

-	npdsch-MultiTB-Interleaving-r16 (clause 4.3.4.193)

-	multiTB-HARQ-AckBundling-r16 (clause 4.3.4.194)

-	groupWakeUpSignal-r16 (clause 4.3.4.195)

-	groupWakeUpSignalAlternation-r16 (clause 4.3.4.196)

-	subframeResourceResvUL-r16 (clause 4.3.4.197)

-	subframeResourceResvDL-r16 (clause 4.3.4.198)

-	slotSymbolResourceResvUL-r16 (clause 4.3.4.199)

-	slotSymbolResourceResvDL-r16 (clause 4.3.4.200)

-	npdsch-16QAM-r17 (clause 4.3.4.222)

-	npusch-16QAM-r17 (clause 4.3.4.223)

-	supportedBandList-r13 (clause 4.3.5.1A)

-	multiNS-Pmax-r13 (clause 4.3.5.16A)

-	powerClassNB-20dBm-r13 (clause 4.3.5.1A.1)

-	powerClassNB-14dBm-r14 (clause 4.3.5.1A.2)

-	dl-ChannelQualityReporting-r16 (clause 4.3.6.37)

-	connModeMeasIntraFreq-r17 (clause 4.3.6.49)

-	connModeMeasInterFreq-r17 (clause 4.3.6.50)

-	accessStratumRelease-r13 (clause 4.3.8.1A)

-	multipleDRB-r13 (clause 4.3.8.5)

-	earlyData-UP-r15 (clause 4.3.8.7)

-	earlySecurityReactivation-r16 (clause 4.3.8.11)

-	coverageBasedPaging-r17 (clause 4.3.8.16)

-	anr-Report-r16 (clause 4.3.12.2)

-	rach-Report-r16 (clause 4.3.12.3)

-	locationInfo-r16 (clause 4.3.12.5)

-	logicalChannelSR-ProhibitTimer (clause 4.3.19.2)

-	dataInactMon-r14 (clause 4.3.19.9)

-	rai-Support-r14 (clause 4.3.19.10)

-	earlyContentionResolution-r14 (clause 4.3.19.14)

-	sr-SPS-BSR-r15 (clause 4.3.19.15)

-	rai-SupportEnh-r16 (clause 4.3.19.22)

-	earlyData-UP-5GC-r16 (clause 4.3.36.9)

-	pur-CP-EPC-r16 (clause 4.3.37.1)

-	pur-UP-EPC-r16 (clause 4.3.37.2)

-	pur-CP-5GC-r16 (clause 4.3.37.3)

-	pur-UP-5GC-r16 (clause 4.3.37.4)

-	pur-CP-L1Ack-r16 (clause 4.3.37.5)

-	pur-NRSRP-Validation-r16 (clause 4.3.37.6)

-	ntn-Connectivity-EPC-r17 (clause 4.3.38.1)

-	ntn-TA-Report-r17 (clause 4.3.38.2)

-	ntn-PUR-TimerDelay-r17 (clause 4.3.38.3)

-	ntn-OffsetTimingEnh-r17 (clause 4.3.38.4)

-	ntn-ScenarioSupport-r17 (clause 4.3.38.5)

-	ntn-SegmentedPrecompensationGaps-r17 (clause 4.3.38.6)

The UE radio access capabilities specified in clause 4 are not applicable in NB-IoT, unless they are listed above.

The following optional features without UE radio access capability parameters specified in clause 6 are applicable in NB-IoT:

-	ETWS (clause 6.2.1);

-	CMAS (clause 6.2.2);

-	KPAS (clause 6.2.3);

-	EU-Alert (clause 6.2.4);

-	RRC Connection Re-establishment for the Control Plane CIoT EPS Optimization (clause 6.7.5)

-	System Information Block Type 16 (clause 6.8.1)

-	Enhanced random access power control (clause 6.8.3)

-	MT-EDT for Control Plane CIoT EPS Optimisation (clause 6.8.10)

-	MT-EDT for User Plane CIoT EPS Optimisation (clause 6.8.11)

-	EDT for Control Plane CIoT EPS Optimization (clause 6.8.4)

-	Enhanced PHR (clause 6.8.6)

-	Carrier specific NRSRP thresholds for NPRACH resource selection (clause 6.8.15)

-	Radio Link Failure Report for NB-IoT (clause 6.10.2)

-	SC-PTM in Idle mode (clause 6.16.1)

-	Multiple TB scheduling for SC-PTM in Idle mode for NB-IoT (clause 6.16.2)

-	Relaxed monitoring (clause 6.17.1)

-	DL channel quality reporting in Msg3 for the anchor carrier (clause 6.17.2)

-	Serving cell idle mode measurements reporting (clause 6.17.3)

-	NSSS-Based RRM measurements (clause 6.17.4)

-	NPBCH-Based RRM measurements (clause 6.17.5)

-	RRM measurements on non-anchor paging carriers (clause 6.17.6)

-	NRS presence on non-anchor paging carriers (clause 6.17.7)

-	DL channel quality reporting in Msg3 for non-anchor carrier (clause 6.17.8)

-	Assistance information for inter-RAT cell selection to/from NB-IoT (clause 6.17.9)

-	RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation (clause 6.18.3)

-	NB-IoT/5GC (clause 6.18.4)

-	MO-EDT for Control Plane CIoT 5GS Optimisation (clause 6.18.5)

-	AS RAI (clause 6.18.6)

-	Cell Reselection Measurements Triggering based on Service Time (clause 6.19.1)

-	Discontinuous coverage (clause 6.19.2).

The optional features without UE radio access capability parameters specified in clause 6 are not applicable in NB-IoT, unless they are listed above.