3GPP TSG-RAN WG2 Meeting #128	R2-2409556

Orlando, USA, Nov 18-22, 2024

Agenda item:	8.7.4.1

Source:	Qualcomm Incorporated

Title:	Discussion on LCP enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we follow up on the agreements made in RAN2#127-bis and discuss additional enhancements needed to support dynamic LCH priority.

# Discussion

At RAN2#127-bis, the following agreements on LCH prioritization were made:

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

This agreement implies that there is no limit on how much data in an LCH may be given a higher priority. All it requires is any amount of priority-adjustable data within the same LCH to enable the adjustment. This absence of restriction can compromise the fairness between LCHs.

For example, consider a configuration in which low bit rate flows (e.g. haptic and control) are mapped to LCH #1 and other high bit rate flows (e.g. video) are mapped to LCH #2. LCH #2 can adjust its priority to the same level as LCH #1 when it has priority-adjustable data.

Congestion is when priority adjustment most likely to occur, because data in low priority LCH can’t get enough UL grants to be scheduled in time. When a UL grant is not large enough, a TB has to be split between the two LCHs. Since in round #1 LCHs with equal priority share available bandwidth proportional to their PBRs, LCH #1 is penalized because of it has a lower PBR than LCH #2. This clearly is not a desired outcome, because LCH #1 should be prioritized over LCH #2 when there is limited UL capacity.

Observation 1.	Priority adjustment without restriction can penalize high-priority logical channels during congestion.

Another example is a scenario in which PDUs in a PDU set arrive in LCH #2 in two separate clusters, with a large time gap between them. Sometime later the first batch of PDUs becomes priority adjustable (remaining time drops below the threshold) but the second batch does not. According to the current agreement, when a UL grant becomes available, the entire PDU set in LCH #2 can be handled with the same priority as LCH #1 throughout the LCP procedure (both round 1 and 2). If Bj at the start of the LCP procedure is larger than the size of the PDU set, then all data from LCH #2 is eligible to participate in at least round 1. Note that these assumptions are not extreme cases. In fact, they can repeat across traffic cycles. When that happens, it implies that LCH #2 consistently competes for available bandwidth with LCH #1, leading to a loss in fairness between the two LCHs.

Observation 2.	Priority adjustment without restriction can lead to loss in fairness between logical channels.

It is clear from the above analysis that some restrictions need to be imposed on priority adjustment. We think a sensible approach is to limit the amount of data that can be adjusted to higher priority. For example, an LCH is allowed to adjust its priority at start of the LCP procedure only if the amount of data whose priority has adjusted within the last T msec is less than a threshold. Of course this is only one of the possible options. Companies can discuss whether there are other better options.

Proposal 1. 	A logical channel with priority-adjustable data is allowed to apply its adjusted priority only if the amount of data transmitted with adjusted priority within the last T msec is less than a configured threshold.

Another approach that can control the extent of damage to fairness is to evaluate the qualification for priority adjustment at start of each round in the LCP procedure. For example, if an LCH has no more priority-adjustable data after round 1, clearly it makes no sense for UE to continue applying the adjusted (higher) priority in round 2.

Evaluating the conditions for priority adjustments at start of round #2 does not create much additional complexity for UE. That is because it does not take much time for UE to check whether any priority-adjustable data is left after round #1. In addition, if there is a change in the priority of any LCHs, adjusting the scheduling order based on the new priority(s) typically would not take much time either, because in most deployments a UE has at most 2 or 3 DRBs.

Proposal 2.	Conditions for priority adjustment are evaluated at start of each round of the LCP procedure.

Next, we discuss the impact of priority adjustment on other MAC procedures. One of the issues is related to status reporting. If an LCH has priority-adjustable data and meets the priority adjustment criteria (as specified in Proposal 1), then the LCH will be handled with its additional priority, instead of the default one. To enable gNB to allocate UL grants more aligned with this UE behavior in the LCP procedure, gNB should be informed based on the adjusted priority than its default priority. Therefore, when UE is sending a BSR or DSR MAC CE, such an LCH should be included in the LCG based on its adjusted priority.

Proposal 3. 	If an LCH meets the priority-adjustment criteria (as specified in Proposal 1) when a DSR or BSR MAC CE is to be sent, it should be included in the LCG according to its adjusted LCH priority.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	Priority adjustment without restriction can penalize high-priority logical channels during congestion.

Observation 2.	Priority adjustment without restriction can lead to loss in fairness between logical channels.

Proposal 1. 	At start of the LCP procedure, a logical channel with priority-adjustable data is allowed to use its additional priority only if the amount of data which was sent with adjusted priority within the last T msec is less than a threshold.

Proposal 2.	Conditions for priority adjustment are evaluated at start of each round of the LCP procedure.

Proposal 3. 	If an LCH meets the priority-adjustment criteria (as specified in Proposal 1) when a DSR or BSR MAC CE is to be sent, it should be included in the LCG according to its adjusted LCH priority.