3GPP TSG-RAN WG2 Meeting #128	                                                                                   R2-2409593

Orlando, USA, Nov.  18th – 22nd , 2024

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on inter-CU LTM [1]. The contribution is organized as the following. In section 2, we discuss the following aspects,

- Security key update related to SA3 LS

- Granularity of R19 Set ID for DC case

- Inter-CU MCG LTM with SCG kept or released

- Inter-CU SCG LTM

- Further co-existence scenarios in DC

Our proposals are summarized in Section 3.

Discussion

Security key update related to SA3 LS

As indicated in the SA3 reply LS [2] on the security key update for inter-CU LTM in standalone, the rekeying method preferred by most companies in SA3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signaling to be used by the UE for key derivation at the next inter-CU LTM cell switch. Also SA3 mentioned that if it is necessary to avoid RRC signaling between cell switches, then SA3 can further analyze other potential methods.

As already indicated in the WID [3], the aim is to avoid the RRC reconfiguration message for the purpose of security update.

Observation 1: As stated in the WID, the RRC configuration message should be avoided for security update in inter-CU subsequent LTM.

Besides, if this solution is employed, each LTM candidate configuration needs to be updated after every inter-CU LTM cell switch execution since the NCC is carried in each LTM candidate configuration (i.e., ltm-CandidateConfig-r18, which is a RRCReconfiguration message). However, this should not be the intention of supporting subsequent LTM.

Observation 2: Providing the NCC value via RRC signaling will necessitate an update of the LTM candidate configuration (i.e., ltm-CandidateConfig-r18) after every inter-CU LTM cell switch execution. This is not in line with the intention for subsequent LTM.

Therefore, it is proposed that,

Proposal 1: RAN2 excludes the solution where the UE is provided with the NCC value via RRC signaling upon every inter-CU LTM cell switch execution. RAN2 sends an LS to SA3 to inform this agreement.

Granularity of R19 Set ID for DC case

During last meeting discussion, it was confirmed how to use the Rel-19 ID jointly with the Rel-18 set ID in the non-DC scenario,

For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

For the DC scenario, some companies mentioned that it may need to configure the Rel-19 ID per RB level to indicate whether to perform PDCP re-establishment, as there may be bear type change upon inter-CU LTM.

However, the Rel-19 set ID is used to indicate the UE whether to perform key update for the corresponding cell group. As for which key to use for each RB, e.g., master key or secondary key, it depends on the keyToUse configured for each RB.

Observation 3: Rel-19 set ID is used to indicate whether the key need to be updated for the corresponding cell group. As for which key to use for each RB, it is indicated by the keyToUse configured for each RB.

Hence it is not needed to configure the Rel-19 set ID per RB level.

Proposal 2a: RAN2 confirms that the Rel-19 set ID is configured per LTM candidate level for DC case.

In SCPAC, the PDCP re-establishment for a RB with changed bear type is triggered by the security key update,

1> else: 2> for each drb-Identity value included in each RadioBearerConfig in the selected subsequent CPAC candidate configuration that is part of the current UE configuration, the UE shall perform the following actions after the end of this procedure: 3> if the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration; or 3> if the bearer is associated with the secondary key (S-KgNB) as indicated by keyToUse in the current UE configuration and a new sk-Counter value has been selected due to the conditional reconfiguration execution for subsequent CPAC: … 4> re-establish the corresponding RLC entity as specified in TS 38.322 [4]; 4> trigger the PDCP entity of the bearer to perform PDCP re-establishment as specified in TS 38.323 [5];

i.e., if the keyToUse for one RB is changed, the UE could assume the security used for the RB should be associated with the key indicated by the new keyToUse. When considering inter-CU LTM, we can follow the similar principle used in SCPAC.

Proposal 2b: For the inter-CU LTM execution in the DC case,

- If the keyToUse of the radio bearer (RB) is changed, PDCP re-establishment is performed.

- If the keyToUse of the radio bearer (RB) is unchanged, UE compares the Rel-19 IDs of the serving cell and the target cell and performs the corresponding L2 reset operation as defined for non-DC.

Inter-CU MCG LTM with SCG kept or released

For the inter-CU MCG LTM, there may be the case where UE in PCell_0 is configured with intra-CU PCell_1 with SCG kept, inter-CU1 PCell_2 with SCG kept as well as inter-CU2 PCell_3 with SCG released, i.e., UE is configured with mixture of intra-CU MCG LTM and inter-CU MCG LTM. The UE trajectory cannot be predicted (for example, PCell_0  PCell_3  PCell_1). As per the Rel-18 principle for intra-CU MCG LTM, the candidate configuration for PCell_1 only includes MCG configuration but not include SCG configuration (i.e., UE only clean the MCG configuration, UE keeps SCG due to optional field with need M). So upon the movement from PCell_3 to PCell_1, the SCG will be released from UE due to the SCG configuration is absent in the PCell_1 candidate configuration, but the intention of candidate PCell_1 is to keep SCG.

To solve this issue, all MCG LTM candidate configurations should include both MCG part configuration and SCG part configuration if DC is configured in the case that at least one inter-CU MCG LTM candidate exists.

Proposal 3: If at least one inter-CU MCG LTM candidate exists, all MCG LTM candidate configurations include both MCG part configuration and SCG part configuration if DC is configured.

If inter-CU MCG LTM candidates with SCG kept and with SCG released are configured simultaneously, it’s possible that the NW firstly triggers LTM cell switch to an LTM candidate with SCG released, and later triggers LTM cell switch to an LTM candidate with SCG kept. But the UE does not have the DC configuration, which results in a SCG addition procedure.

The following solutions were proposed and discussed but not be able to conclude it yet,

- Option 1: The NW ensures that the UE does not switch to an LTM candidate cell with SCG configuration if the UE has released DC prior to the LTM cell switch.

- Option 2: The UE does not apply the SCG configuration (if any) in the target LTM candidate configuration if the NW triggers the LTM cell switch to an LTM candidate cell with SCG configuration but the UE has released DC prior to the LTM cell switch.

- Option 3: The NW can trigger LTM cell switch to any configured LTM candidate cells, regardless of whether the target candidate cell has the SCG configuration and whether the UE is in NR-DC currently. I.e. inter-CU MCG LTM with SCG addition is supported.

Option 1 puts unnecessary restriction on NW configuration. Option 2 allows UE to not follow the NW configuration which is not a regular practice in the 3GPP spec, i.e., apply part of one message. Option 3 can be supported without spec impact.

Therefore, it is suggested to go with option 3. Hence it should support SCG addition for inter-CU MCG LTM execution.

Proposal 4: SCG addition is supported when an inter-CU MCG LTM cell switch is executed.

Inter-CU SCG LTM

On the provision of the configuration for inter-CU SCG LTM, as agreed, the MN sends it to UE via MN RRCReconfiguration. In general, there are two ways to achieve this purpose, as summarized in the table below,

Table 1: Candidate solutions for the configuration of inter-CU SCG LTM

| Candidate solutions | Issues to address |
| --- | --- |
| Option 1: reuse ltm-Config-r18 | - Issue 1: UE needs to know whether a certain candidate configuration in ltm-CandidateToAddModList-r18 received from MCG is for intra-CU MCG LTM or for inter-CU SCG LTM; - Issue 2: When the UE receives the LTM cell switch command from SCG, how to determine the candidate configuration ID included in the command refers to the configuration from MCG or the intra-CU SCG configuration. - Issue 3: How to determine the reference configuration for NW if intra-CU MCG LTM is co-existed with inter-CU SCG LTM; - Issue 4: Need to further clarify whether source SN or MN allocate the candidate configuration ID; - Issue 5: Need to further clarify whether to negotiate the candidate configuration ID with MN, if it is allocated by source SN; |
| Option 2: define new field | - Issue 1: Need to further clarify whether source SN or MN allocate the candidate configuration ID; - Issue 2: Need to further clarify whether to negotiate the candidate configuration ID with MN, if it is allocated by source SN; |

As analyzed above, Option 2 is simpler, i.e., it mainly depends on RAN3 to further clarify the negotiation of candidate configuration ID. Besides, Option 2 is also clearer from ASN.1 point of view. The IE LTM-Config-r18 which includes the reference configuration, candidate configuration could be reused, Therefore, it is proposed that,

Proposal 5: A new field (e.g. ltm-InterCU-SCG-Config-r19 defined by IE LTM-Config-r18) is introduced to provide the configuration for inter-CU SCG LTM.

Based on our Proposal 5, another issue is how to configure the co-existence of inter-CU SCG LTM and intra-CU SCG LTM.

In SCPAC, if the candidate configuration belongs to different SN, even more than one candidate PSCell belong to the same SN, e.g., belong to the source SN for intra-SN SCPAC, the candidate configuration for both intra-SN and inter-SN PSCell(s) can only be MN format. This is because the NW does not know how UE moves between the intra/inter-SN candidate PSCell(s). For example, UE is in PSCell_0 and configured with candidate PSCell_1 (intra-SN) and candidate PSCell_2 (inter-SN). Although the candidate PSCell_1 is an intra-SN candidate cell with source PSCell_0, UE may move along the trajectory PSCell_0  PSCell_2  PSCell_1, therefore the candidate configuration for PSCell_1 should be MN format so that it can provide the MCG configuration such as radio bear.

When considering inter-CU SCG LTM, the similar issue also exists. Therefore, the similar principle should be followed.

Proposal 6: If at least one inter-CU SCG LTM candidate exists, all the SCG LTM candidate configurations including the intra-CU and inter-CU SCG LTM candidate configurations are provided to UE via MN format.

On the generation of candidate and reference configuration, it was agreed that,

Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs.

For the FFS on whether to include MCG part configuration in reference configuration, there is no need to address it as it is already clear in other parts of the agreements, i.e.,” It can be up to the NW implementation whether to include the MCG part.”, as highlighted by green.

Observation 4: No need to address the FFS on whether reference configuration for inter-CU SCG LTM should include SCG part as it is clear in the agreements already.

In an inter-CU SCG LTM execution, both MCG and SCG configurations need to be changed as MCG is also involved. Therefore, when the UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by the UE must contain both MCG and SCG part configurations. How to ensure this can be left to the network implementation.

Proposal 7: It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

Further co-existence scenarios in DC

In Rel-18, intra-CU LTM can be configured for both MCG and SCG, and it’s up to NW implementation to trigger MCG LTM or SCG LTM, but not simultaneously. For inter-CU LTM, it was agreed that,

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

It was questioned whether some additional coordination between the MN and the SN is needed to avoid such simultaneous configuration.

In our understanding, existing network interface interaction is sufficient.

If the MN received an inter-CU SCG LTM request from source SN, the MN can reject it if it has already configured UE with inter-CU MCG LTM.

If UE has been configured with inter-CU SCG LTM, the MN should be aware of this as it was configured via the MN. In such a case, MN shall not initiate the preparation of inter-CU MCG LTM.

Therefore, it can be left to NW implementation to not configure both inter-CU MCG LTM and inter-CU SCG LTM simultaneously.

Proposal 8: No additional coordination between the MN and the SN is needed to avoid configuring both inter-CU MCG LTM and inter-CU SCG LTM simultaneously, i.e., It’s left to NW implementation.

In last meeting, it had been confirmed to support the following co-existence,

RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

Scenario 1 - The co-existence of intra-CU MCG LTM and inter-CU SCG LTM

When UE is configured with both inter-CU SCG LTM and intra-CU MCG LTM, there will be the following potential execution orders.

Case 1-1: Firstly perform intra-CU MCG LTM, and followed by inter-CU SCG LTM

For inter-CU SCG LTM, the complete candidate configuration includes the SCG part and MCG part contained in the MN format RRCReconfiguration,

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

The current PCell1 (after performing the intra-MCG LTM) is different with the PCell0 (before performing the intra-MCG LTM), based on which the NW prepares the inter-CU SCG LTM configuration. Hence, when UE performs inter-CU SCG LTM, UE needs to perform cell switch from PCell1 to the original PCell0. However, PCell change is not the intention of the network upon inter-CU SCG LTM execution.

Observation 5: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs intra-CU MCG LTM and then performs inter-CU SCG LTM, PCell may be unintended changed back to the initial PCell.

Case 1-2: Firstly perform inter-CU SCG LTM, and followed by intra-CU MCG LTM

When UE firstly performs inter-CU SCG LTM, the candidate configuration includes both the MCG configuration and SCG configuration, which means the current MCG configuration will be changed based on the complete candidate configuration. For example, the target SN may re-locate a certain RLC bearers to MN due to load balance. However, the complete configuration for intra-CU MCG LTM is prepared based on the condition that the RLC bearers are located in SN. So the RLC bearer related configuration is not included in the intra-CU MCG LTM configuration. When the UE perform the intra-CU MCG LTM, the radio bearers will be dropped by UE, which is not the intention of the NW.

Observation 6: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs inter-CU SCG LTM, the intra-CU MCG LTM configuration may become invalid.

Scenario 2 - The co-existence of inter-CU MCG LTM and intra-CU SCG LTM

When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, there will be the following potential execution orders.

Case 2-1: Firstly perform inter-CU MCG LTM, and followed by intra-CU SCG LTM

If UE performs inter-CU MCG LTM with SCG released, there is no problem since the release of SCG will also release the intra-CU SCG LTM configuration. If UE performs inter-CU MCG LTM with SCG kept, since the inter-CU MCG LTM may lead to the re-location of RLC bearers, similar issue like case 1-2 also exists.

Observation 7: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs inter-CU MCG LTM with SCG kept, the intra-CU SCG LTM configuration may become invalid.

Case 2-2: Firstly perform intra-CU SCG LTM, and followed by inter-CU MCG LTM

Since the intra-CU SCG LTM does not change SN, there is no problem if UE further performs inter-CU MCG LTM with SCG released. If UE performs inter-CU MCG LTM with SCG kept, similar as case 1-1, it may cause PSCell change upon performing inter-CU MCG LTM, which is not the intention of the NW.

Observation 8: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs intra-CU SCG LTM and then performs inter-CU MCG LTM execution, the PSCell may be unintended changed back to the initial PSCell.

In summary, for both Scenario 1 and Scenario 2, the execution of one type LTM may cause some issue when followed by UE executing the other type LTM. To handle this issue, some companies proposed that UE can automatically release the invalid LTM configuration due to above co-existence. We do not prefer this way, as in Rel-18, the LTM configuration can only be explicitly released by the NW. From our perspective, since the NW controls UE to perform intra or inter-CU MCG/SCG LTM, it can be up to NW implementation to ensure the validation of the LTM configuration, i.e., if above co-existence is configured, the NW can re-configure/release the LTM configuration if it finds the above error cases.

Proposal 9: When LTM execution (either intra-CU or inter-CU) is performed at one CG (either MCG or SCG), It’s up to NW implementation to release or reconfigure the LTM configuration at another CG (either SCG or MCG) to ensure its validity.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: As stated in the WID, the RRC configuration message should be avoided for security update in inter-CU subsequent LTM.

Observation 2: Providing the NCC value via RRC signaling will necessitate an update of the LTM candidate configuration (i.e., ltm-CandidateConfig-r18) after every inter-CU LTM cell switch execution. This is not in line with the intention for subsequent LTM.

Observation 3: Rel-19 set ID is used to indicate whether the key need to be updated for the corresponding cell group. As for which key to use for each RB, it is indicated by the keyToUse configured for each RB.

Observation 4: No need to address the FFS on whether reference configuration for inter-CU SCG LTM should include SCG part as it is clear in the agreements already.

Observation 5: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs intra-CU MCG LTM and then performs inter-CU SCG LTM, PCell may be unintended changed back to the initial PCell.

Observation 6: When UE is configured with both intra-CU MCG LTM and inter-CU SCG LTM, if UE firstly performs inter-CU SCG LTM, the intra-CU MCG LTM configuration may become invalid.

Observation 7: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs inter-CU MCG LTM with SCG kept, the intra-CU SCG LTM configuration may become invalid.

Observation 8: When UE is configured with both inter-CU MCG LTM and intra-CU SCG LTM, if UE firstly performs intra-CU SCG LTM and then performs inter-CU MCG LTM execution, the PSCell may be unintended changed back to the initial PSCell.

Security key update related to SA3 LS

Proposal 1: RAN2 excludes the solution where the UE is provided with the NCC value via RRC signaling upon every inter-CU LTM cell switch execution. RAN2 sends an LS to SA3 to inform this agreement.

Granularity of R19 Set ID for DC case

Proposal 2a: RAN2 confirms that the Rel-19 set ID is configured per LTM candidate level for DC case.

Proposal 2b: For the inter-CU LTM execution in the DC case,

- If the keyToUse of the radio bearer (RB) is changed, PDCP re-establishment is performed.

- If the keyToUse of the radio bearer (RB) is unchanged, UE compares the Rel-19 IDs of the serving cell and the target cell and performs the corresponding L2 reset operation as defined for non-DC.

Inter-CU MCG LTM with SCG kept or released

Proposal 3: If at least one inter-CU MCG LTM candidate exists, all MCG LTM candidate configurations include both MCG part configuration and SCG part configuration if DC is configured.

Proposal 4: SCG addition is supported when an inter-CU MCG LTM cell switch is executed.

Inter-CU SCG LTM

Proposal 5: A new field (e.g. ltm-InterCU-SCG-Config-r19 defined by IE LTM-Config-r18) is introduced to provide the configuration for inter-CU SCG LTM.

Proposal 6: If at least one inter-CU SCG LTM candidate exists, all the SCG LTM candidate configurations including the intra-CU and inter-CU SCG LTM candidate configurations are provided to UE via MN format.

Proposal 7: It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

Further co-existence scenarios in DC

Proposal 8: No additional coordination between the MN and the SN is needed to avoid configuring both inter-CU MCG LTM and inter-CU SCG LTM simultaneously, i.e., It’s left to NW implementation.

Proposal 9: When LTM execution (either intra-CU or inter-CU) is performed at one CG (either MCG or SCG), It’s up to NW implementation to release or reconfigure the LTM configuration at another CG (either SCG or MCG) to ensure its validity.

Reference

[1] Draft_RAN2_127b_Meeting_Report

[2] R2-2409534, Reply LS on security handling for inter-CU LTM in non-DC cases (S3-244316; contact: Samsung)

[3] RP-242356, Revised Work Item: NR mobility enhancements Phase 4