3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409597

Orlando, USA, 18th – 22nd Nov, 2024

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on case 2. The contribution is organized as the following. In section 2, we discuss the following aspects,

- How to allow NES UE to reselect to OD-SIB1 NES cells

- The details of Msg.1 based RA procedure

- Cell selection for R19 NES UE

- SIB1 transmission for emergency case

- OD-SIB1 acquisition for UE in RRC Connected mode

- Content of WUS configuration

Our proposals are summarized in Section 3.

Discussion

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for OD-SIB1.

  - How to allow NES UE to reselect to OD-SIB1 NES cells

In last meeting, the agreement on preventing legacy UE barring is as follows:

RAN2#127bis meeting agreements - Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. - NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

Based on above agreement to access restriction, the NES Cell could be included in the excluded cell list or be given with lower reselection priorities, then the legacy UE will not access to the NES Cell based on the excluded cell list or the reselection priorities.

For the method of introducing reselection priorities to NES Cell, the NES UE could ignore the reselection priorities for NES Cell and apply the new configured or default priorities. But it is not sure NES Cell will be deployed at frequency level or not, it is FFS on whether the reselection priorities on frequency are feasible.

For the method of the NES Cell included in the excluded Cell list in Cell A, based on the RAN1 agreement as below, the NES UE could identify a cell is NES Cell by WUS configuration from Cell and then ignore the NES Cell in excluded list, and try to access this NES Cell if needed. This UE behavior is better than the method of introducing a new separate R19 excluded cell list for NES UE, it avoids to introduce a new excluded list at SIB and reduce the design complexity.

RAN1#118bis Agreement For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Proposal 1: For a Cell in the excluded cell list, the NES UE removes it from the excluded cell list if the cell is a NES Cell (e.g., WUS configuration is provided for the Cell).

  - The details of Msg.1 based RA procedure

About Msg.1 based RA procedure, RAN2 has following agreements.

RAN2#126 meeting agreements As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE. RAN2#127bis meeting agreements In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1. The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

Based on RAN2#126/127bis meeting agreements, as baseline, RAN2 already agreed that UE needs to monitor RAR as network feedback, and upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. In legacy on-demand SIB request procedure, it is UE implementation to request SIB again after the failed RA procedure. However, for on-demand SIB1 request procedure, the SIB1 information is important to UE, UE may be not able to work if SIB1 could not be achieved, so it is suggested to not leave the determination to UE and upper layer will consider it as barred, then UE will perform the cell reselection procedure to find a suitable cell.

Proposal 2: UE considers the cell as barred upon random access procedure failure of OD-SIB1 request.

RAN1#118bis meeting Agreement For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

In another way, after UE receives the RAR successfully, it will monitor the OD-SIB. One remaining issue is how the UE obtain the time information of OD-SIB1 transmission. The latest RAN1 agreements on OD-SIB1 monitoring is above, it seems that the related configuration on OD-SIB monitoring will be included in WUS configuration, but the details are FFS and still under RAN1 discussion, So, it is suggested to wait RAN1 conclusion before we start any discussion on the starting time of PDCCH monitoring occasions for OD-SIB1.

Proposal 3: RAN2 waits RAN1 conclusion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

Besides, because of UE mobility, it is also possible that UE may miss the OD-SIB1 even if the RAR for OD-SIB1 is received successfully. UE behavior for this case should also be defined. Generally, we think the UE needs to consider this NES Cell is barred and perform the cell-reselection immediately.

Proposal 4: R19 NES UE considers the NES Cell as barred if it is failed to acquire the OD-SIB1 finally after a successful RACH procedure.

In last meeting, one case is proposed that, if the UE perform cell selection/reselection to NES Cell and has no WUS configuration or OD-SIB information, it will consider this NES Cell as barred for maximum of 300 seconds. Then UE will select to Cell A and obtain the WUS configuration, will UE ignore this 300s restriction and perform cell reselection to NES Cell right away? In our view, there is no strong reason to trigger UE accessing to NES Cell immediately. The UE could perform the cell reselection as legacy and may reselect to NES cell after the 300s time is expired.

Proposal 5: If a NES cell is barred due to being unable to acquire the SIB1, the legacy access restriction is applied, i.e., UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

  - Cell selection for R19 NES UE

Based on current discussion, R19 NES UE is allowed to perform cell reselection and camp on NES Cell. However, R19 NES UE may power on and perform cell-selection at NES Cell which is transmitting the OD-SIB1 triggered by other UEs or broadcasting periodic SIB1 message, the R19 NES UE behavior should be defined if it could camp on NES Cell. In our view, even it is a corner case and the UE cannot trigger the WUS, but the NES UE could read out the OD-SIB1 triggered by other UEs, so the UE is able to camp on the NES Cell if allowed.

Proposal 6: In cell selection case, R19 NES UE without WUS configuration can camp on NES Cell if it can acquire OD-SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

  - SIB1 transmission for emergency case

According to legacy procedure, if UE is ETWS capable or CMAS capable and receives the etwsAndCmasIndication bit in the Short Message, it will re-acquire the SIB1 immediately. In this case, the network will changed to normal mode other than NES mode, or the network will broadcast the SIB periodically as agreed in last meeting, the NES UE could receive the SIB1 immediately without any delay. Then, there is on issue is explored, it could be considered as network implementation.

Proposal 7: In emergency cases,  it is network implementation to switch to normal mode or broadcast the SIB periodically to make UE re-acquire the SIB1 immediately, there is no further spec impact.

  - OD-SIB1 acquisition for UE in RRC Connected mode

For RRC Connected mode UE in NES cell, it has the requirement to obtain the OD-SIB1 message. For example, the SIB in UE may be invalid for 3 hours expiry, the legacy UE could read the SIB1 by UE implementation since the SIB1 is always broadcasted by serving cell. But for NES UE in connected mode, how to acquire OD-SIB1 by UE needs to be discussed, one way is, the gNB transmits the OD-SIB1 information by legacy dedicatedSIB1-Delivery in RRCReconfiguration to UE when NES UE has a RRC Connection to the network, another way is UE actively request the OD-SIB1 if needed. If the first way is supported, then there is no need to discuss how the UE actively request the OD-SIB1.

Observation 1: Whether UE in RRC Connected mode needs to actively request the OD-SIB1 or not should be clarified.

If the network-based way is not supported, UE actively request the OD-SIB1 is needed. The RRC Connected UE could trigger the WUS to obtain the OD-SIB1, which seems a bit of strange to trigger another RA procedure for OD-SIB1 when UE is in connected mode, but if it is not triggered, some enhancement is needed for RRC connected mode UE to obtain OD-SIB1message, such as by dedicated signaling to request OD-SIB1.

Proposal 8: If UE in RRC Connected mode needs to actively request OD-SIB1 message, down-selection from below two options is needed:

- RRC Connected mode UE in NES cell could trigger the WUS based OD-SIB1procedure.

- RRC Connected mode UE in NES cell will request the SIB1 by dedicated signaling.

  - Content of WUS configuration

Obviously, the WUS configuration needs to include the RA preamble, the RA resource, the parameters used for RA procedure, such as the maximum number of Random Access Preamble transmission preambleTransMax, the parameters on power ramping, the RAR window parameter ra-ResponseWindow and so on, some of them has been discussed and agreed by RAN1#118bis meeting as in below table.

Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

We could wait on RAN1 more progress and then discuss the details in WUS configuration.

Proposal 9: For the WUS configuration parameters of a NES cell, RAN2 waits further RAN1 progress.

Since Cell A/NES Cell may assist more than one neighbor NES Cells, the neighbor NES Cells have better not use the same WUS configuration to avoid the WUS transmission conflict. So, it can be seen that the WUS configuration format in Cell A/NES Cell could be a list including one or more WUS configuration, each WUS configuration is associated to one or more cells.

Proposal 10: The WUS configuration format in Cell A could be a list including one or more WUS configuration, each WUS configuration can be associated to one or more cells.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: Whether UE in RRC Connected mode needs to actively request the OD-SIB1 or not should be clarified.

How to allow NES UE to reselect to OD-SIB1 NES cells

Proposal 1: For a Cell in the excluded cell list, the NES UE removes it from the excluded cell list if the cell is a NES Cell (e.g., WUS configuration is provided for the Cell).

The details of Msg.1 based RA procedure

Proposal 2: UE considers the cell as barred upon random access procedure failure of OD-SIB1 request.

Proposal 3: RAN2 waits RAN1 conclusion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

Proposal 4: R19 NES UE considers the NES Cell as barred if it is failed to acquire the OD-SIB1 finally after a successful RACH procedure.

Proposal 5: If a NES cell is barred due to being unable to acquire the SIB1, the legacy access restriction is applied, i.e., UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

Cell selection for R19 NES UE

Proposal 6: In cell selection case, R19 NES UE without WUS configuration can camp on NES Cell if it can acquire OD-SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

SIB1 transmission for emergency case

Proposal 7: In emergency cases,  it is network implementation to switch to normal mode or broadcast the SIB periodically to make UE re-acquire the SIB1 immediately, there is no further spec impact.

OD-SIB1 acquisition for UE in RRC Connected mode

Proposal 8: If UE in RRC Connected mode needs to actively request OD-SIB1 message, down-selection from below two options is needed:

- RRC Connected mode UE in NES cell could trigger the WUS based OD-SIB1procedure.

- RRC Connected mode UE in NES cell will request the SIB1 by dedicated signaling.

Content of WUS configuration

Proposal 9: For the WUS configuration parameters of a NES cell, RAN2 waits further RAN1 progress.

Proposal 10: The WUS configuration format in Cell A could be a list including one or more WUS configuration, each WUS configuration can be associated to one or more cells.

Reference

[1] Draft_RAN2_127b_Meeting_Report, https://www.3gpp.org/ftp/TSG_RAN/WG2_RL2/TSGR2_127bis/Report