3GPP TSG-RAN WG2 Meeting #128	R2-2409638

# Orlando, Florida, USA, 18 – 22 November 2024

Agenda item:		8.11.3

Source:	Xiaomi

Title:	Other aspects of SBFD

Document for:		Discussion and Decision

Introduction

In RAN2#127bis meeting, following was agreed regarding CLI handling [1]:

For UL resource muting for PUSCH, the configuration of time and frequency location for UL resource muting should be introduced based on R1 agreement.

For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement.

For L1 based UE-to-UE CLI reporting mechanism, the configuration of report quantities should be introduced based on R1 agreement.

In this contribution, we discuss RAN2 aspects of SBFD other than RACH.

Discussion

General working procedure in RAN2

In RAN2#127bis, RAN2 confirms RAN1 agreements regarding the RRC configuration of UL resource muting for PUSCH and L1 based UE-to-UE CLI reporting. SFBD is a RAN1-led WI. Typically before the end of WI completion, RAN1 provides RAN2 with a RRC parameter list for related RRC configuration, as well as UE feature list to define related UE capabilities. RAN2 can then prepare related running CRs to TS 38.331 and 38.306 based on RRC parameter list and UE feature list. Unless RAN2 identifies some issues with RAN1 decision, or wants to influence RAN1 discussion of RRC parameter list and UE feature list, there might be no need for RAN2 to simply confirm what RAN1 has agreed. RAN2 just needs to focus on RAN2 impacts based on RAN1 agreements.

Proposal 1: In general, RAN2 postpones the discussion on details for RRC configuration and UE capability before receiving relevant RRC parameter list and UE feature list from RAN1, at least for the following aspects: SBFD resource indication, UL resource muting for PUSCH, L1 based UE-to-UE CLI measurement and reporting.

Cell reselection

Whether SBFD capable UEs can prioritize SBFD cells over non-SBFD cells during cell reselection was discussed in RAN2#127 meeting without conclusion.

The potential benefit of such prioritization is that UE might have less latency when transiting from IDLE/INACTIVE to CONNECTED as SBFD cells can have more RACH resources compared with non-SBFD cells. But this might not be always the case if quality of SBFD cells is worse compared with non-SBFD cells.

From radio resource efficiency perspective, it is generally preferred that UE reselects to cells with best radio quality if load balancing is not an issue. RAN2 previously agreed some cell reselection prioritization rules either based on service (e.g. MBS) or topology (e.g. HSDN, mobile-IAB, sidelink). But SBFD is not related to either service or topology.

In addition, within one single frequency, SBFD deployment can be cell specific. However, cell specific prioritization might cause additional complexity at UE side, e.g. checking cell level SBFD support during cell reselection.

Considering above discussion, it is proposed to not consider prioritization of SBFD cells / frequencies during cell reselection.

Proposal 2: Prioritization of SBFD cells / frequencies during cell reselection is not considered.

Interference issue

In Rel-19 SBFD, gNB can receive in UL and transmit in DL simultaneously. An example is shown in Figure 1 below. In Case 1, the DL reception may cause interference to the PUSCH reception in network side. In Case 2, neighbor cell’s DL transmission may also cause interference to the PUSCH reception in network side, i.e., CLI. Although the interference issue above could be mitigated to certain extent by implementation as well as CLI handling techniques, the PUSCH transmitted in SBFD symbols still suffers more interference than PUSCH transmitted in non-SBFD symbols. RAN2 needs to discuss whether such PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Figure 1: Interference issue of PUSCH in SBFD symbols

Note that quality difference between SBFD and non-SBFD symbols for RACH was already identified by RAN1, which agreed the following in RAN1#118bis meeting:

For RACH configuration Option 1, support separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols

Proposal 3: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

Conclusion

In this contribution, we discuss RAN2 aspects of SBFD other than RACH, and propose the following:

Proposal 1: In general, RAN2 postpones the discussion on details for RRC configuration and UE capability before receiving relevant RRC parameter list and UE feature list from RAN1, at least for the following aspects: SBFD resource indication, UL resource muting for PUSCH, L1 based UE-to-UE CLI measurement and reporting.

Proposal 2: Prioritization of SBFD cells / frequencies during cell reselection is not considered.

Proposal 3: RAN2 to discuss whether PUSCH quality difference between SBFD and non-SBFD symbols should be addressed or not.

- References

[1] R2-2409212, RAN2 Vice Chairman (CATT), "Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD"