3GPP TSG-RAN WG2 Meeting #128	R2-2409658

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Further discussion on Conditional LTM

Document for:	Discussion, Decision

Introduction

In the last meeting, we discussed and agreed on the basic LTM functions for conditional LTM. However, the detailed procedures for implementing these functions still require further discussion. E.g., how the UE acquires TA and how network indicates the TCI state, etc.

This paper further discussed these procedures and also the scope of conditional LTM.

Discussion

Acquisition of Timing advance and TCI state.

As similar as conditional handover, the LTM CSC MAC-CE should be replaced by UE initiated LTM cell switch procedure, which should be a baseline for conditional LTM.

Proposal 1: For conditional LTM, the LTM CSC MAC-CE is replaced by UE initiated LTM cell switch procedure.

In Rel-18 LTM, the LTM CSC MAC-CE has several information:

- LTM candidate cell ID. To indicate the target cell to UE among the LTM candidate cells.

- TA value. To indicate the TA value used for the UL synchronization toward target cell.

- TCI state ID. To indicate and activate the (DL and/or UL) TCI state for the LTM target cell.

- CFRA related information. To provide CFRA information to UE if RACH-less LTM is not performed.

For conditional LTM, the target cell is decided by UE, so LTM candidate cell ID is not needed. However, how does UE perform LTM cell switch procedure by the absence of these information (TA, TCI state ID, CFRA related information) may still need discussion.

Timing advance and UL grant

The TA value is used for the synchronization of uplink transmission to adjust the timing from UE so that the signalling arrives at the gNB at the correct instance. UE must acquire the TA before sending the first UL message. In legacy RACH procedure, TA is included in the RAR message for both CBRA and CFRA. In RACH-less LTM, network omits the RAR after UE sends a preamble and instead indicates the TA in the LTM CSC MAC-CE to reduce latency and avoid redundant signalling interactions.

In LTM completion stage, CG and DG based first UL transmission is supported in Rel-18 LTM. Either the NW can configure configured grant (CG) via RRC LTM configuration, or provide regular dynamic grant via DCI (DG) after the cell switch.

In the last meeting, both RACH-less and RACH- based Conditional LTM is supported, and both UE based-TA and PDCCH ordered early TA acquisition is supported. For the RACH-less case, CG is supported, and FFS DG case:

- RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

However, it is still unclear how does UE obtain TA by PDCCH ordered early TA acquisition, as the LTM CSC MAC-CE is omitted. The detail will be discussed together in section 2.3.

Observation 1: For conditional LTM, it is unclear how UE obtains TA by PDCCH ordered early TA acquisition by the absence of LTM cell switch command MAC CE.

TCI states acquisition

TCI state is used to indicate the antenna QCL relationship of the target cell.

- In legacy handover, the UE determine the default antenna QCL assumption by RACH procedure, then receives a finer TCI state activation indication after the cell switch.

- In Rel-18 LTM, the TCI states are pre-configured to UE and can be activated before LTM cell switch. UE tracks the QCL RS of this candidate TCI state until receiving the LTM cell switch command MAC CE. If the same TCI state is indicated in LTM CSC, the interruption can be reduced.

In last meeting, RAN2 agreed to support Rel-18 Early candidate TCI State activation/deactivation for conditional LTM.

Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

The Candidate Cell TCI States Activation/Deactivation MAC CE provides a set of candidate TCI states that the UE may use for LTM cell switching. The UE tracks the QCL RS of indicated candidate TCI states until it receives the LTM cell switch command MAC CE, which specifies the precise TCI state ID for the UE.

Observation 2: In Rel-18 LTM, the Candidate Cell TCI States Activation/Deactivation MAC CE provides a set of candidate TCI states, while the LTM cell switch command MAC CE specifies the precise TCI state for target cell.

However, in the case of conditional LTM, UE initiate the LTM cell switch without a network indication. The LTM cell switch command MAC CE is not provided to the UE, and it is unclear how the UE acquires the precise TCI state for LTM cell switch.

Observation 3: For conditional LTM, it is not clear how does UE acquire the TCI state for LTM cell switch by the absence of LTM cell switch command MAC CE.

Cases analysis

Considering the various scenarios for conditional LTM—RACH-based vs. RACH-less, CG vs. DG, and PDCCH-ordered early TA acquisition vs. UE-based TA measurement—the cases can be categorized as follows:

Table 1: Issues encountered by the current conditional LTM framework without enhancements

|  |  | RACH-based C-LTM | RACH-less PDCCH order early TA C-LTM |  | RACH-less UE-based TA C-LTM |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CG based | DG based | CG-based | DG based |
| Early DL sync |  | Candidate Cell TCI States Activation/ Deactivation MAC CE | Candidate Cell TCI States Activation/ Deactivation MAC CE | Candidate Cell TCI States Activation/Deactivation MAC CE | Candidate Cell TCI States Activation/Deactivation MAC CE | Candidate Cell TCI States Activation/Deactivation MAC CE |
| Early UL sync |  |  | PDCCH order | PDCCH order | UE obtains TA by itself | UE obtains TA by itself |
|  |  |  | RACH preamble | RACH preamble |  |  |
| LTM execution (UE initiated without LTM CSC MAC CE) |  | UE selects the target cell and applies the configuration | UE selects the target cell and applies the configuration | UE selects the target cell and applies the configuration | UE selects the target cell and applies the configuration | UE selects the target cell and applies the configuration |
|  |  | RACH preamble (CFRA or CBRA) |  |  |  |  |
|  |  | RAR (UE obtains TA) | (TA is unknown) | (TA is unknown) |  |  |
|  |  | (Contention resolution if needed in CBRA) |  |  |  |  |
| LTM completion | UL grant | (LTM is completed if RA is successfully complete) | UL grant in cg-LTM-Configuration | UE uses selected SSB (indicate in preamble) to monitor DCI for UL grant | If it is CG, no issue for UL grant | If it is DG, UE does not know the TCI state and cannot monitor DCI for UL grant |
|  | TA |  | TA is required to send RRCReconfComplete | TA is required to send RRCReconfComplete |  |  |
| After LTM |  | May receive another TCI state activation indication | TCI state=most recent CG PUSCH | May receive another TCI state activation indication | TCI state=most recent CG PUSCH | May receive another TCI state activation indication |

Sentences in Red requires additional enhancement for conditional LTM, otherwise LTM may not complete successfully.

Based on the above analysis, we can have some more detailed discussion on the issues:

Issue for TCI state acquisition

For RACH-less DG LTM with PDCCH order early TA, the UE may select the initial SSB and informs the network via a RACH preamble during early UL synchronization, to monitor the UL DCI for UL grants in LTM completion. However, this process does not involve network responses and may not be guaranteed by the network.

For RACH-less DG LTM with UE based TA measurement, UE without TCI state is unable to monitor the UL DCI for UL grants, making it difficult to send the RRCReconfigurationComplete message to the network.

Observation 4: Without TCI state indication, RACH-less DG-based conditional LTM may not function effectively with PDCCH-ordered early TA, and cannot function with UE-based TA measurement.

For RACH-based LTM, the UE performs RACH toward target cell and selects the initial SSB. Consequently, the UE may receive another TCI state activation indication after the LTM cell switch. For LTM, this may result in the loss of the benefit of fine beam reporting by L1 MR and an increase to the throughput degradation or even interruption, particularly if CSI-RS-based measurement is used in conditional LTM.

Observation 5: If the UE selects the initial SSB and informs the network via a RACH preamble, UE may receive another TCI state activation indication after the LTM cell switch. The benefit of LTM fine beam reporting by L1 MR may be lost, which may lead to the increasing time of throughput degradation or interruption.

Issue for Timing advance acquisition

In RACH-based LTM, UE perform RACH after LTM cell switch triggering, so TA is obtained via RAR.

For RACH-less LTM:

- If UE based TA measurement is performed, there is no issue with TA acquisition.

- If PDCCH-ordered early TA acquisition, the UE may not know the TA by the absence of LTM cell switch command MAC CE. This will cause UE being unable to send the first UL message to the network.

Observation 6: UE cannot obtain the TA by PDCCH-ordered early TA acquisition in conditional LTM without enhancement. The lack of TA will cause UE being unable to send the first UL message to the network.

Summary and potential solutions

PDCCH-ordered early TA acquisition

Consider PDCCH-ordered early TA acquisition need to be supported, solution is needed to indicate TA to UE. Therefore, reintroducing RAR-like mechanism or a new MAC-CE to indicate the TA value seems a valid choice.

However, for early UL synchronization, UE remains connected to the serving cell while sending RACH preamble to target cell, and it brings extra complexity for UE to receive RAR from target cell while maintaining the connection with serving cell. Therefore, UE need to receive this MAC-CE (or RAR) from the serving cell, and inter-node coordination may be needed.

Even so, the TA value could be outdated if there is a significant gap between early UL sync and LTM execution. UE may still need to perform RACH-based LTM in this case.

Proposal 2: For PDCCH-ordered early TA acquisition, a new MAC-CE (or RAR) is introduced to provide the TA value, which is indicated from serving cell to UE. If the TA is outdated during LTM execution, UE perform RACH-based LTM.

If a new MAC-CE (or RAR) is introduced, it would be beneficial to include the TCI state to inform the UE. This inclusion would help minimize throughput degradation in LTM and support the DG-based RACH-less LTM. By using a single MAC CE, both the TCI state and TA can be indicated to the UE.

Proposal 3: If proposal 2 is agreed, the TCI state can also be included in the new MAC-CE (or RAR).

RACH-based LTM and UE-based TA measurement

However, the above solution (new MAC-CE or RAR) is not compatible with RACH-based LTM and UE-based TA measurement, as the UE does not send a RACH preamble before the LTM cell switch. In these scenarios, there is no issue with TA acquisition, but only a problem with TCI state acquisition remains.

Observation 7: The new MAC-CE (or RAR) solution does not work well with RACH-based LTM and UE-based TA measurement RACH-less LTM. For these scenarios, there is no issue with TA acquisition, only a problem with TCI state acquisition remains.

For RACH-based LTM, the UE selects the initial SSB and sends to the network via RACH preamble. The UE can further use the information provided in early TCI state activation indication to determine the TCI state. After LTM cell switch, the network may provide the UE with another TCI state activation indication MAC-CE if necessary.

Proposal 4: For RACH-based LTM, the UE can use the information of TCI states provided in early TCI state activation indication to select the initial SSB. UE may receive another TCI state activation indication from network after the LTM cell switch.

For UE-based TA measurement with CG, UE sends the first UL message by the configured UL resource. The TCI state can be obtained from the recent CG PUSCH. In this scenario, LTM may function normally without the need for additional enhancements. However, when the network configures multiple UL grants, selecting the TCI state may still have issue, which can be FFS.

For UE-based TA measurement with DG, currently there is no way for UE to inform network its selection for initial SSB, and it is unclear how UE monitor the UL DCI for UL grant. We suggest postponing the discussion in this scenario.
Proposal 5: Postpone the discussion on the support for UE-based TA measurement with DG.

Triggering condition and event triggered L1 MR

In the last meeting, RAN2 agreed that LTM3-like and LTM5-like condition can be used for the triggering condition for conditional LTM:

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM.

However, it is FFS on whether to reuse the CHO conditions for conditional LTM.

In Rel-18, the feature of L1 measurement report is decoupled with LTM, which means LTM can based on L3 measurement and report, at least in FR1. Considering the compatibility benefit of R18 LTM, we see no difference in conditional LTM scenario, and think L3 measurement can also be applicable for conditional LTM.

Proposal 6: L1 measurement feature is decoupled with conditional LTM. The triggering condition of conditional LTM can be based on L3 measurement.

If we support L3 MR for conditional LTM, the triggering condition should also consider CHO conditions. For mobility use case, at least CondEventA3-like and CondEventA5-like conditions can be supported.

- CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

- CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2

Proposal 7: CondEventA3-like and CondEventA5-like conditions can be used for the conditional LTM execution.

Furthermore, event-triggered L1 MR is designed to notify the network only when necessary and has been designed to used a new MAC-CE as the container, This design aligns well with conditional LTM and may assist in informing UE decisions to the network. Therefore, a general proposal is made to integrate event-triggered L1 MR with conditional LTM.

Proposal 8: Event triggered L1 MR can be used for conditional LTM. RAN2 to discuss if it can be used to inform UE decision to network.

The motivation and the scope of conditional LTM

Let’s first review the historical mobility mechanisms to see the benefits they achieved and identify additional benefits that can be gained from conditional LTM:

Conditional Handover

In Rel-16, Conditional Handover was introduced for UE initiated handover for multiple candidate cells. Compared with legacy handover, two main benefits can be found in Conditional Handover:

- Improve Reliability: UE decides to perform handover when condition is met to react quickly to deteriorating conditions to prevent condition degrade to a point where handover might fail. Besides, the handover command is omitted to prevent HOF caused by this signalling failure.

- UE autonomy and flexibility: Allows the UE to proactively decide which candidate cell to perform handover by certain predefined conditions and pre-configured candidate cells configurations. The decision made by UE may be more accurate and reasonable due to more detailed measurement context of its radio environment, which allows UE to detect more changes that might not be included in the measurement report.

Observation 8: Two main benefit can be found in conditional handover compared with legacy handover:

- The reliability is increased by UE’s quick response and omitting signalling from failure.

- The accuracy is increased by UE to proactively decide which candidate cell to perform handover based on its finer measurement context.

Rel-18 LTM

In Rel-18, LTM was introduced for handover based on beam level measurement report. Compare with legacy handover, two main benefits can be found in LTM to optimize the performance for mobility:

- Interruption reduction. The early sync procedure is introduced to allow UE to establish synchronization toward candidate cells before handover execution, which shortens the interruption for each handover process. This is more important in the scenario when cell switch is performed frequently (e.g., FR2, high speed or ping-pong scenarios)

- Latency reduction: LTM decision is made from L1 beam level measurement. This approach allows for a finer and faster report of the radio environment to let network to make a quick decision to get a timely and detailed cell switch indication. MAC-CE is used instead of RRC message to further fasten the transmission of CSC to prevent channel deterioration.

Observation 9: Two main benefit can be found in LTM compared with legacy handover:

- The interruption for each cell switch is reduced to realize higher data throughput.

- The latency is reduced by L1 measurement report by introducing CSC MAC-CE for finer and faster measurement report and cell switch indication.

Rel-19 conditional LTM

The benefits of Conditional Handover are not yet integrated into LTM. As such, Rel-19 aims to incorporate these advantages into Conditional LTM. However, the current LTM synchronization process is network-initiated. To enhance CHO within LTM, several level of improvements can be proposed:

Reliability. LTM currently uses MAC-CE for cell switch commands, which lacks RLC-AM protection, leading to potential signal loss especially at cell edges where channel condition is poor. Therefore, the 1st objective of Rel-19 Conditional LTM is to enable UE initiated LTM cell switch execution without network indication. It can also reduce the latency of signalling interaction when condition is met to react quickly towards deteriorating channel conditions.

UE autonomy: To improve flexibility and accuracy, the UE can be allowed to proactively select a candidate cell based on its detailed measurements context, but it also needs UE initiated early synchronization and then to inform the network. Considering the complexity of different mechanisms used in Conditional LTM, the specific implementation of which mechanism are introduced will be discussed in the following sections.

Considering different expectation, it is recommended that RAN2 first clarify the scope of condition LTM.

Proposal 9: RAN2 to discuss the scope of conditional LTM:

- Option 1: Early synchronization is network-initiated.

Option 1a: Network ensure there is only one candidate cell after early synchronization. UE only decides the LTM execution time to initiate the cell switch procedure.

Option 1b: There may be multiple candidate cells after early synchronization (e.g., for RACH-based LTM and no early UL sync). UE decides the candidate cell and proper time to perform the LTM execution.

- Option 2: The UE can decide the candidate cell to perform early synchronization and informs the network of its decision (FFS how to indicate network in each step). Subsequently, UE decides the LTM execution time and initiate the cell switch procedure.

Conclusion

In this contribution, the following observations and proposals are made:

General principle

Proposal 1: For conditional LTM, the LTM CSC MAC-CE is replaced by UE initiated LTM cell switch procedure.

Issue for TCI state acquisition

Observation 2: In Rel-18 LTM, the Candidate Cell TCI States Activation/Deactivation MAC CE provides a set of candidate TCI states, while the LTM cell switch command MAC CE specifies the precise TCI state for target cell.

Observation 3: For conditional LTM, it is not clear how does UE acquire the TCI state for LTM cell switch by the absence of LTM cell switch command MAC CE.

Observation 4: Without TCI state indication, RACH-less DG-based conditional LTM may not function effectively with PDCCH-ordered early TA, and cannot function with UE-based TA measurement.

Observation 5: If the UE selects the initial SSB and informs the network via a RACH preamble, UE may receive another TCI state activation indication after the LTM cell switch. The benefit of LTM fine beam reporting by L1 MR may be lost, which may lead to the increasing time of throughput degradation or interruption.

Issue for Timing advance acquisition

Observation 1: For conditional LTM, it is unclear how UE obtains TA by PDCCH ordered early TA acquisition by the absence of LTM cell switch command MAC CE.

Observation 6: UE cannot obtain the TA by PDCCH-ordered early TA acquisition in conditional LTM without enhancement. The lack of TA will cause UE being unable to send the first UL message to the network.

Summary and potential solutions

Proposal 2: For PDCCH-ordered early TA acquisition, a new MAC-CE (or RAR) is introduced to provide the TA value, which is indicated from serving cell to UE. If the TA is outdated during LTM execution, UE perform RACH-based LTM.

Proposal 3: If proposal 2 is agreed, the TCI state can also be included in the new MAC-CE (or RAR).

Observation 7: The new MAC-CE (or RAR) solution does not work well with RACH-based LTM and UE-based TA measurement RACH-less LTM. For these scenarios, there is no issue with TA acquisition, only a problem with TCI state acquisition remains.

Proposal 4: For RACH-based LTM, the UE can use the information of TCI states provided in early TCI state activation indication to select the initial SSB. UE may receive another TCI state activation indication from network after the LTM cell switch.

Proposal 5: Postpone the discussion on the support for UE-based TA measurement with DG.

Triggering condition and event triggered L1 MR

Proposal 6: L1 measurement feature is decoupled with conditional LTM. The triggering condition of conditional LTM can be based on L3 measurement.

Proposal 7: CondEventA3-like and CondEventA5-like conditions can be used for the conditional LTM execution.

Proposal 8: Event triggered L1 MR can be used for conditional LTM. RAN2 to discuss if it can be used to inform UE decision to network.

The motivation and the scope of conditional LTM

Observation 8: Two main benefit can be found in conditional handover compared with legacy handover:

- The reliability is increased by UE’s quick response and omitting signalling from failure.

- The accuracy is increased by UE to proactively decide which candidate cell to perform handover based on its finer measurement context.

Observation 9: Two main benefit can be found in LTM compared with legacy handover:

- The interruption for each cell switch is reduced to realize higher data throughput.

- The latency is reduced by L1 measurement report by introducing CSC MAC-CE for finer and faster measurement report and cell switch indication.

Proposal 9: RAN2 to discuss the scope of conditional LTM:

- Option 1: Early synchronization is network-initiated.

Option 1a: Network ensure there is only one candidate cell after early synchronization. UE only decides the LTM execution time to initiate the cell switch procedure.

Option 1b: There may be multiple candidate cells after early synchronization (e.g., for RACH-based LTM and no early UL sync). UE decides the candidate cell and proper time to perform the LTM execution.

- Option 2: The UE can decide the candidate cell to perform early synchronization and informs the network of its decision (FFS how to indicate network in each step). Subsequently, UE decides the LTM execution time and initiate the cell switch procedure.

References

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- RP-242356-Revised-WID-NR-Mob-Enh