3GPP TSG-RAN WG2 Meeting #128											    R2-2409659

Orlando, USA, November 18th – 22th, 2024                           revision of R2-2409206

Source: 	Xiaomi

Title:  	Remaining issues of L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

# Introduction

According to the RAN2 discussion on the L1 event triggered measurement reporting, RAN2 agreed that MAC CE is used to report the L1 measurement result, and the dedicated SR can be configured for the MAC CE report. In this contribution, we provide our understandings on the remaining issues of the L1 event triggered measurement reporting, e.g. how to handle the TTT for event evaluation, how to control the measurement results reported via MAC CE and so on.

# Discussion

  - TTT

In the RAN2#127bis meeting, RAN2 made the following agreement for TTT of the LTM event evaluation:

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

According to the LTM Event(s) (i.e. Event LTM2/3/4/5) agreed by RAN2, the event evaluation during TTT could include: only current beam (for Event LTM2); only candidate beam (for Event LTM4); both current beam and candidate beam (for Event LTM3/5).

According to the discussion in RAN2#127, RAN2 made the following agreements:

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used for LTM event evaluation.

As the current beam could change due the TCI-state activation/deactivation/indication, and the candidate beam fulfilling the event evaluation could also change due TTT, we need to consider how to reset the TTT when the current beam changes or when the candidate beam fulfilling the event evaluation changes.

Figure 1: TTT for candidate beam

Regarding the event evaluation for the candidate beam during the TTT as illustrated in the above figure, we could have the following options:

- Option 1 (Per beam): If the RSRP of a specific beam of a candidate cell fulfils the measurement event during TTT, the UE considers that the measurement event is fulfilled.

- Option 2 (Per cell): If the RSRP of any beam of a candidate cell fulfils the measurement event during TTT, the UE considers that the measurement event is fulfilled.

As the UE could have multiple measurement result of multiple beams from the same cell, the consequence of Option 2 is that the measurement event could be triggered while there is no beam having a stable RSRP fulfilling the measurement event. When the UE reports the latest RSRP of the candidate beam via the measurement report, there could be no RSRP of candidate beam fulfilling the measurement event. As the beam indication (i.e. TCI-state) is mandatory in LTM Cell Switch Command MAC CE, the measurement report could be wasted as the gNB is not able to indicate the TCI-state in LTM Cell Switch Command MAC CE, so as to trigger the LTM cell switch. Thus, we think that Option 1 should be adopted.

Proposal 1: The TTT operation for candidate cell is per beam (i.e. TTT is reset when the candidate beam satisfying the LTM event changes).

TS 38.321: #### 6.1.3.14 TCI States Activation/Deactivation for UE-specific PDSCH MAC CE … - Ti: If there is a TCI state with TCI-StateId i as specified in TS 38.331 [5], this field indicates the activation/deactivation status of the TCI state with TCI-StateId i, otherwise MAC entity shall ignore the Ti field. The Ti field is set to 1 to indicate that the TCI state with TCI-StateId i shall be activated and mapped to the codepoint of the DCI Transmission Configuration Indication field, as specified in TS 38.214 [7]. The Ti field is set to 0 to indicate that the TCI state with TCI-StateId i shall be deactivated and is not mapped to the codepoint of the DCI Transmission Configuration Indication field. The codepoint to which the TCI State is mapped is determined by its ordinal position among all the TCI States with Ti field set to 1, i.e. the first TCI State with Ti field set to 1 shall be mapped to the codepoint value 0, second TCI State with Ti field set to 1 shall be mapped to the codepoint value 1 and so on. The maximum number of activated TCI states is 8. The activated TCI states can be associated with at most one PCI different from the Serving Cell PCI at a time; #### 6.1.3.24 Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE … - TCI state IDi,j: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5], where i is the index of the codepoint of the DCI Transmission configuration indication field as specified in TS 38.212 [9] and TCI state IDi,j denotes the jth TCI state indicated for the ith codepoint in the DCI Transmission Configuration Indication field. The TCI codepoint to which the TCI States are mapped is determined by its ordinal position among all the TCI codepoints with sets of TCI state IDi,j fields, i.e. the first TCI codepoint with TCI state ID0,1 and TCI state ID0,2 shall be mapped to the codepoint value 0, the second TCI codepoint with TCI state ID1,1 and TCI state ID1,2 shall be mapped to the codepoint value 1 and so on. The TCI state IDi,2 is optional based on the indication of the Ci field. The maximum number of activated TCI codepoint is 8 and the maximum number of TCI states mapped to a TCI codepoint is 2.

Figure 2: TTT for current beam

Regarding the event evaluation for the current beam during the TTT, we could have the same Option 1 and Option 2 as we used for the candidate beam, as the activated/indicated TCI state for the current serving cell could be more than 1 according to TS 38.321 as quoted above. On the other hand, the activated/indicated TCI state could also change at any point of time.

- Option 1 (Per beam): If the RSRP of a current beam of SpCell fulfils the measurement event during TTT, the UE considers that the measurement event is fulfilled.

- Option 2 (Per cell): If the RSRP of any current beam of a SpCell fulfils the measurement event during TTT, the UE considers that the measurement event is fulfilled.

As the LTM measurement event is to change the serving cell from the source cell to the target cell, and to find a stable beam in the target cell, we think that we could separate the event evaluation of the current beam from the event evaluation of the candidate beam. For example, for LTM Event-2/5 (i.e. “Event LTM2: Beam of serving cell becomes worse than absolute threshold” and “Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.”), if any current beam is able to work properly, the LTM cell switch should not be triggered. This means that the TTT should be per cell for the event evaluation of the current beam.

Proposal 2: The TTT operation for serving cell is per cell (i.e. TTT is not reset when the current beam satisfying the LTM event changes).

  - Report contents

Regarding the contents reported in the LTM measurement report MAC CE, RAN2 made the following agreements in the RAN2#127bis meeting:

9. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. 10. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration.

Firstly, we think that the measurement result of the candidate beam triggering the measurement report should be reported, so that the LTM Cell Switch Command MAC CE is able to indicate the TCI state of the candidate cell.

Proposal 3: The measurement result of the candidate beam triggering the LTM measurement event is reported.

With regards to other beams of the candidate cell corresponding to the candidate beam triggering the measurement report, we think that the UE should also be able to report the measurement results of other beams, so that the gNB is able to select another beam when the candidate beam triggering the LTM measurement event is congested. However, in order to ensure that the candidate beams reported has a good quality, the gNB can configure an RSRP threshold to control the other candidate beams which are allowed to be reported together with the candidate beam triggering the measurement report. The details on how to ensure the measurement quality of other candidate beams can be discussed further.

Proposal 4: The network can configure N>1 candidate beams to be reported.

Furthermore, as the measurement results are used for a handover to a specific candidate cell corresponding to the candidate beam triggering the LTM measurement event, the other candidate beams reported and the candidate beam triggering the LTM measurement event should be from the same candidate cell.

Proposal 5: The candidate beams reported (including the candidate beam triggering the LTM measurement event and other candidate beams) are from the same candidate cell.

With regards to the L3 cell-level measurement results, we think that the network can provide an extra configuration to control whether the L3 cell-level measurement results of either the SpCell and/or the candidate cell can be reported, so that the gNB can have more information (e.g. cell quality) to determine whether to handover the UE to a candidate cell, e.g. to avoid ping-pong handover.

Proposal 6: The report of the L3 cell-level measurement result of the SpCell is configurable.

Proposal 7: The report of the L3 cell-level measurement result of the candidate cell is configurable.

  - Offset

In the RAN2#126 meeting, RAN2 made the following agreement for the offset configured for LTM event evaluation:

- For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

3GPP TS 38.331: Entering condition of measurement event Event 2: Ms + Hys < Thresh Event 3: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Event 4: Mn + Ofn + Ocn – Hys > Thresh Event 5: Mp + Hys < Thresh1 Mn + Ofn + Ocn – Hys > Thresh2

According to the L3 measurement events based on the cell-level measurement results, one event could have the following offset (as the example of Event A3 given above):

- Frequency/measurement object specific offset (e.g. Ofn/ Ofp)

- Cell specific offset (e.g. Ocn/ Ofp)

According to the RAN2#127 meeting agreement, the cell level measurement result is not be used for the L1 LTM measurement event.

- Beam level measurement result, not cell level measurement result, is used for LTM event evaluation.

Since the gNB is able to control the L1 measurement report for each beam separately by using the beam specific offset and the L1 measurement event is triggered by beam-level measurement result, there is no need to introduce cell specific offset for event evaluation. Thus, the entering/leaving condition for LTM measurement event would be as follows:

Examples of entering/leaving condition of LTM measurement event: LTM Event 2: Entering condition: Ms + Hys < Thresh Leaving condition: Ms – Hys > Thresh LTM Event 3: Entering condition: Mn + Obn – Hys > Mp + Obp + Off Leaving condition: Mn + Obn + Hys < Mp + Obp + Off LTM Event 4: Entering condition: Mn + Obn – Hys > Thresh Leaving condition: Mn + Obn + Hys < Thresh LTM Event 5: Entering condition 1: Mp + Hys < Thresh1 Entering condition 2: Mn + Obn – Hys > Thresh2 Leaving condition 1: Mp – Hys > Thresh1 Leaving condition 2: Mn + Obn + Hys < Thresh2

According to the example given above, the beam specific offset (including Obn of the beam specific offset of the candidate beam and Obp of the beam specific offset of the SpCell) will be used for LTM Event 3/4/5. According to the L3 measurement configuration, the cell specific offset can be configured independently for each cell including both the SpCell and the neighbouring cell(s). Thus, we think that the network can provide the beam specific offset for each beam, including both the serving beam and the candidate beams.

Proposal 8: For LTM event evaluation, cell specific offset is not needed.

## s-Measure

As the LTM procedure is to change the serving cell to another candidate cell, when the quality of the serving SpCell is still good, the UE does not need to perform the measurement for the candidate beam. For the L3 measurement reporting procedure, s-MeasureConfig can be optionally configured to save the UE power, so that the UE only starts measuring the neighbouring cells when the serving cell quality is below a threshold configured by s-MeasureConfig. From our understanding, same mechanism as L3 s-MeasureConfig can be reused for even-triggered L1 measurement report.

Proposal 9: The UE performs the L1 measurement for the candidate beam(s), only when the RSRP of the SpCell is below the threshold configured by RRC, same as the legacy s-MeasureConfig.

## Dedicated SR

In the RAN2#127bis meeting, RAN2 made the following agreements for the SR used for the L1 MR MAC CE:

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In general, we think that the triggering/cancellation/priority of the dedicated SR for the L1 MR MAC CE can reuse the procedure defined for the dedicated SR for BFR MAC CE. This means that the dedicated SR is triggered when a L1 measurement report is triggered. When the L1 measurement report is cancelled (e.g. due to the report/cancellation of the L1 MR MAC CE), the dedicated SR is also cancelled.

38.321: ### 5.4.4 Scheduling Request … NOTE 3: When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

According to TS 38.321 as quoted above, the BFR SR has higher priority than the BSR SR, because the beam failure recovery is more urgent than the data transmission. From our understanding, the transmission of L1 MR MAC CE should also have higher priority, so that the LTM cell switch can be triggered in time. This means that the priority of the SR for MR MAC CE should be higher than that of the SR for BSR.

Proposal 10: The dedicated SR for MR MAC CE has higher priority than the SR for BSR.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: The TTT operation for candidate cell is per beam (i.e. TTT is reset when the candidate beam satisfying the LTM event changes).

Proposal 2: The TTT operation for serving cell is per cell (i.e. TTT is not reset when the current beam satisfying the LTM event changes).

Proposal 3: The measurement result of the candidate beam triggering the LTM measurement event is reported.

Proposal 4: The network can configure N>1 candidate beams to be reported.

Proposal 5: The candidate beams reported (including the candidate beam triggering the LTM measurement event and other candidate beams) are from the same candidate cell.

Proposal 6: The report of the L3 cell-level measurement result of the SpCell is configurable.

Proposal 7: The report of the L3 cell-level measurement result of the candidate cell is configurable.

Proposal 8: For LTM event evaluation, cell specific offset is not needed.

Proposal 9: The UE performs the L1 measurement for the candidate beam(s), only when the RSRP of the SpCell is below the threshold configured by RRC, same as the legacy s-MeasureConfig.

Proposal 10: The dedicated SR for MR MAC CE has higher priority than the SR for BSR.

# Reference

[1] RAN2#125 meeting minutes

[2] RAN2#126 meeting minutes

[3] RAN2#127 meeting minutes

# Annex

## RAN2 meeting agreements

### RAN2#125

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

### RAN2#126

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

### RAN2#127

Agreements on L1 MR event evaluation: - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used for LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. Agreements on L1 measurement reporting: - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

### RAN2#127bis

Agreements on L1 event triggered MR 1. MR can be sent when the leaving condition is met, based on NW configuration. 2. Event triggered periodic MR can be supported, based on NW configuration. 3. For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. 4. For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. 5. For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. 6. The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. 7. TTT operates only based on a timer (like TTT used in L3 event triggered MR). 8. Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). 9. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. 10. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration. 11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

## RAN1 meeting agreements

### RAN1#118

Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study.

### RAN1#118bis

Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells Agreement The following alternatives are further studied: - Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE. - The report is sent to the serving cell and transferred to the candidate/target cell(s) - Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.