3GPP TSG-RAN WG2 Meeting #128											    R2-2409660

Orlando, USA, November 18th – 22th, 2024

Source: 	Xiaomi

Title:  	Discussion on the modelling of the UE-initiated beam report

Agenda Item:	8.12.3

Document for:	Discussion and Decision

# Introduction

According to the RAN1 discussion on the event-driven beam report, RAN1 agreed the following two alternatives (i.e. Mode A and Mode B) for reporting the beam measurement result:

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In this contribution, we provide our analysis on how to model the beam measurement report procedure in PHY and MAC.

# Discussion

## Modelling of event-driven beam report

Figure 1: event-driven beam report procedure

According to the RAN1 agreements given above, after triggering the beam reporting (i.e. the triggering event determination), the UE uses either Mode A or Mode B to send the UCI including the beam measurement results. The UCI transmission/controlling procedure is captured in PHY, without RAN2 specification impacts. However, regarding how to trigger the measurement report (i.e. the triggering event determination), we could have several options to model this function, with some options which could impact the RAN2 specification.

With regards to the triggering event determination, we could split the function into the following steps:

- Step 1 (Reference signal measurement): The UE gets the measurement result by measuring the reference signal of specific beam.

- Step 2 (Event instance evaluation): The beam measurement result is used to evaluate if an event instance (e.g. Event-2) is fulfilled.

- Step 3 (Event determination): By counting the number of event instances within a time window, the UE determines whether a measurement report is triggered.

Figure 2: Options for triggering event determination

From our understanding, we could have the following options (as also illustrated in the above figure) for the modelling of triggering event determination function:

- Option 1: All steps in PHY

- Option 2: Event instance evaluation in PHY; Event determination in MAC

- Option 3: both event instance evaluation and event determination in MAC

Form our understanding, all options for modelling the triggering event determination function are technically feasible. None of the above modelling option is like the LTM event-based measurement report procedure, as the LTM event-based measurement report procedure does not have the Event-x instance evaluation based on the beam measurement result, and does not count the number of Event-x instance(s) in a time window. Option 1 leaves all steps in PHY, without any RAN2 specification impacts in MAC. Option 2 is more like the BFR procedure by using the instance indicated by PHY. However, both Option 2 and Option 3 would require the interaction between PHY and MAC, which may require lots of coordination (e.g. LSs) between RAN2 and RAN1 for clarifying the details on how to evaluate the event instance and how to count the Event-x instance within a time window. As the TU(s) for Rel-19 MIMO is quite limited, we would prefer Option 1 to leave all steps of triggering event determination in PHY.

Proposal: The triggering event determination procedure is performed in PHY.

# Conclusion

According to the above analysis, we have the following proposal:

Proposal: The triggering event determination procedure is performed in PHY.

# Reference

[1] xxxx

# Annex

## RAN1 agreements

### RAN1#116

Agreement

On UE-initiated/event-driven beam report, at least of following aspects should be included:

- Trigger-event detection for beam reporting by UE

  - UE monitors RS to assess if a beam-reporting trigger condition has been met

  - FFS: Trigger condition for declaring beam-reporting event

- Beam-report transmission by UE

  - Signaling contents in the beam report

  - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission

    - MAC-CE

    - UCI

    - Others are not precluded.

On UE-initiated/event-driven beam report, the following aspects may be included:

- UE requesting UL resource(s) for the beam report

- UE notifying transmission of beam report

- gNB preconfigured resources

Other procedure(s) as required

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Agreement

On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP

- FFS: Study and decide whether additional contents can be supported.

- FFS: L1-RSRP format, e.g., absolute and/or differential value.

- Note: Above does not imply to preclude discussion on L1-RSRP filtering.

- The actual reported content depends on the triggering event

  - Support of one or multiple events will be discussed separately

### RAN1#116bis

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported:

- Mode A (dynamically scheduling UCI by gNB):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - FFS: Request format, e.g., SR or a new UCI type.

  - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

  - Step 3: Beam report is transmitted in second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature).

  - No new DCI format is introduced.

- Mode B (UCI in pre-configured resource(s) for second UL channel):

  - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - FFS: Notification format, e.g., SR or a new UCI type.

  - Step 2: UE transmits the beam report in the second UL channel.

    - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both

  - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

FFS: Whether UE receives acknowledge information with response to each step for all modes

For above procedures, cross-CC beam reporting is supported for both modes.

- FFS: Details.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- At least L1-RSRP is supported as quality metrics used for Event-2

  - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient)

  - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event

- Regarding RS measurement for the new beam for Event-2, down-select one or more of the following:

  - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE

  - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s).

- Note-1: ‘New/current beam’ is for discussion purpose.

- Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded.

- Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

Agreement

On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state.

- Regarding RS measurement for the current beam for Event-2, Option-2a is supported:

  - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state.

    - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state.

  - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE.

    - Note: SSB or CSI-RS can be configured

Agreement

On UE-initiated/event-driven beam reporting, further study the following trigger events:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117

- Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

- Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - FFS: Whether the indication of payload size should be provided additionally.

  - FFS: Details on how value of N is determined by the UE

- Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax}

  - The N beam(s) should satisfy the condition of Event-2

  - Nmax is configured by gNB

  - Payload size does not vary as a function of N

  - FFS: Zero-padding can be provided if N is less than Nmax.

- Option-2: Only N=1 beam is reported in the report instance

  - The reported beam should satisfy the condition of Event-2

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

- Other options are not precluded.

- FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC.

- FFS: When current beam is reported, whether the current beam is counted in the N reported beams.

- The selected option shall satisfy Event-2.

### RAN1#117

Agreement

On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported

- Option-3: N ≥ 1 beam(s) are reported in the report instance,

  - At least one of N reported beam(s) should satisfy the condition of Event-2

  - N is configured by gNB

    - FFS: candidate value of ‘N’.

  - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams

- FFS: Option-1/1a/1b/2.

- Above applies at least for the single CC case

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows.

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

  - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state.

- Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

- Enabling one of either Scheme-1 or Scheme-2 is selected by NW.

  - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

  - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

- The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Agreement

Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported

- Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured

- FFS: Option-3b/3c

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

    - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

      - Note: The reported current beam is NOT counted in the N reported beams.

    - When disabled by RRC, N beams are reported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

    - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

      - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

    - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

      - FFS: DCI format 1_3

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

- Others are not precluded.

- FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state.

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

### RAN1#118

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH.

- FFS: PUCCH as the second channel

- FFS: Whether the PUSCH can be with UL data

Agreement

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported

  - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration

  - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set.

  - FFS: The RS in the RS resource set can be updated by MAC-CE

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported.

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

…

Differential L1-RSRP #N

Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC

Note: Other contents are not precluded

.

    - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1

      - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

      - FFS: range and step size of differential L1-RSRP

    - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2.

    - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following

- Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following

- Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

- Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above is at least applied to a single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, select one of the following options

- Option-1: Introduce a new 1-bit field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- Option-2: Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - FFS: DCI format 0_3

- FFS: Whether/how to handle the case that multiple CSI report configuration(s) for the UE-initiated/event-driven beam report are associated with the same first PUCCH resource and/or the same scheduled PUSCH

Conclusion

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4}

- FFS: additional candidate value(s) of {5, 6, 7, 8}

- FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further down-select one of the following for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement

Note: If there is no consensus in RAN1 on one of Options 1/2/3, Option 4 will be taken.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least one or both of the following should be supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection.

- Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

  - Option-1B: No restriction in terms of periodicity.

- Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured.

- Above applies to both cross-CC and same-CC beam report.

- Note: Above is applied to the case that second UL channel is PUSCH.

- FFS: Whether the first and second channels can be from the same/different CC.

Working Assumption

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A and/or Mode-B, further study the following for first PUCCH transmission

- UCI multiplexing/dropping/prioritization rule

- Conditions for the transmission of the first PUCCH.

- Whether the PUCCH resource in the first PUCCH channel can be associated with multiple CSI report configurations for UE-initiated/event-driven beam reporting from one or multiple CC(s).

- Whether/how to re-transmit the first PUCCH channel.

Whether/how to apply prohibit-timer or maximum number of (re)transmission(s) for first PUCCH channel.

### RAN1#118bis

R1-2408318	Moderator Summary #1 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Monday session

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2,

- The differential L1-RSRP is quantized to a 4-bit value with 2 dB step size

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A

- Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2

  - FFS: DCI format 0_3

- FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, resource mapping/configuration between first and second UL channel in Mode-B, at least Option-1 is supported

- Option-1 (one-to-one): Only one periodic PUCCH resource for the first channel and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting.

  - Down-select one of the following in RAN1#118bis

    - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel.

    - Option-1B: No restriction in terms of periodicity.

R1-2408319	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Tuesday session

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for both Mode-A and Mode-B, the first PUCCH and the second PUSCH can be from the same or different CC(s)

- FFS: whether the first PUCCH and the second PUSCH should be from the same PUCCH group

- The first PUCCH and the second PUSCH should be in the same CG

  - FFS: Different CGs

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported:

- Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report.

- Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI.

- Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report

  - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI.

FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report.

Working Assumption

The following working assumption in RAN1#118 is revised in red.

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th Q-th best quality.

  - M Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

R1-2408320	Moderator Summary #3 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Wednesday session

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B,

- The UE expects that there is the same periodicity (in ms) between first PUCCH resource and pre-configured resource for second UL channel.

  - FFS: Whether first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms).

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, there is no RAN1 consensus on the following enhancement for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW.

When the Scheme-1 is used, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

R1-2409212	Moderator Summary #4 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Thursday session

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, confirm the following working assumption

- Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW.

Agreement

Regarding RS measurement for the current beam for Event 2, for Option-2a, the following working assumption in RAN1#117 is confirmed with modification:

- (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

  - Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

Agreement

Regarding RS measurement for the current beam for Event 2, for enabling one of either Scheme-1 or Scheme-2 by NW in Option-2a, the following implicit manner is supported:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

Agreement

Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

Agreement

Regarding the triggering event determination for Event 2, down-select among the following alternatives for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured]

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s)

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS;

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS;

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s):

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}:

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s):

FFS: Whether the periodicity of the current beam RS should be the same as that of the new beam RS(s).

Note: There is the same periodicity for the new beam RS(s).

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- For new beam measurement, in a CSI report configuration, configure legacy RRC parameter carrier that indicates the CC that the RS resource set associated with the CSI reporting configuration can be found

- FFS: Whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement.

- FFS: Whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement.

Conclusion

There is no RAN1 consensus on the following proposal:

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ can further comprise {5, 6, 7, 8}, besides for previously agreed candidate value of {1, 2, 3, 4}.