3GPP TSG-RAN WG2 Meeting #128	R2-2409673

Orlando, U.S.A, Novermber 18th – 22nd, 2024

Source:	CATT

Title:	Further consideration on UL capacity enhancements

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting [1], agreements were made for UL capacity enhancements as follows:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this document, we further discuss the RAN2 objective of UL capacity enhancements in IoT NTN, with our proposals provided.

Discussion

Terminology

In order to facilitate discussion for CB-Msg3 EDT in this document, the following terminologies are used for CB-Msg3 EDT in this document.

- CB-Msg3-EDT Occasion: the time occasion to transmit CB-Msg3. In the time domain, a corresponding UL transmission in N consecutive UL slots, where N = NRepNslots, where NRep is the repetition number which can be determined by the CE level (NRep=2 in Figure 1), and Nslots is the length of basic resource unit, e.g. slot number within one RU in NB-IoT. In frequency domain, the maximum frequency resource number is 4 in Figure 1.

- Replica: a set of consecutive transmissions within a CB-Msg3-EDT occasion according to the CE level. In Each replica in time domain corresponds to the resources for the consecutive transmissions which are used as repetitions (as determined by RSRP for intended CE level).

- CB-Msg3 window: the timer duration in which the UE monitors PDCCH for network response for CB-Msg3 transmission.

- CB-Msg3-RNTI: The RNTI used to scramble PDCCH to schedule network response for CB-Msg3 transmission.

Figure 1: Example for illustration of CB-Msg3-EDT occasion

Resource allocation of CB-Msg3 EDT

RAN2 has agreed that DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW) was supported for CB-Msg3 EDT. In this section, we discuss the how the CB-Msg3 EDT works with DSA.

### 2.1.1	Resource pool configuration per CE level

In RAN2#127bis meeting, it was assumed that CB-Msg3 EDT cell specific PUSCH resources are associated with number of repetitions and RSRP selection threshold to determine the CE level and largest TBS for CB-Msg3 transmission. According to the assumption, it can be seen that the CB-Msg3 PUSCH resources can be configured with association to different CE levels respectively, so that UEs at same/different CE levels selects resources which support same/different times of repetition. From network perspective, a pool of resource can be configured for each CE level, with each resource pool containing the candidate PUSCH resources for the UE to select according to the UE's current CE level decided by RSRP.

Proposal 1: Introduce per CE level resource pools which include the CB-Msg3 EDT cell specific PUSCH resources. UE selects the resource pool associated with the intended CE level, according to the RSRP criterion.

Regarding the specific physical resource structure in the resource pool, it can be configured like PUR, e.g. defining the periodicity and timing offset. However, considering that multiple UEs will perform the transmission with the completion in CB-Msg3 EDT, the density of resource configured for CB-Msg3 EDT may be different from PUR. Anyway, the details are in RAN1 scope.

Proposal 2: The details on physical resource structure of CB-Msg3 EDT in the resource pool (e.g. whether to reuse the existing RU in NB-IoT or design other resource granularity) are left to RAN1.

### 2.1.2	Resource selection

For DSA, the UE needs to select CB-Msg3-EDT Occasions per CE level and the resource for each replica's transmission. UE behavior on how the UE performs the resource selection has not been discussed yet. The resource selection procedure for CB-Msg3 EDT consists of how/when to perform CE level selection, replica number determination and replica resource selection.

- 2.1.2.2	Selection of resources for DSA based collision avoidance

Regarding replica transmission(s) in DSA, it is used to promote the randomization of the transmission occasions in time domain, in order to avoid inter-UE collision and improve the successful transmission rate for CB-Msg3. In this sense, the replica number can be independent from the channel condition, thus not impacted by the CE level. Actually, it highly depends on the channel load which may be more easily estimated from network perspective. Hence, it is straightforward that the network configures the preferred values for determination of replica number, with the following 2 options thus able to be considered:

Option 1: The network indicates the MAX_REPLICA_NUMBER (i.e. the maximum number of replicas for which the UE can select resources for a CB-Msg3 EDT), and the UE selects the Replica number according to a uniform distribution between 1 and the MAX_REPLICA_NUMBER. In this option, different UEs may select different replica number randomly.

Option 2: The network indicates a specific REPLICA_NUNMBER, and the UE selects the Replica number for subsequent replica transmissions for a CB-Msg3 EDT. In this option, different UEs use the same replica number.

RAN2 can down-select 2 options on how UE determines a the number of replicas that can be transmitted for a CB-Msg3 EDT.

Proposal 3: RAN2 down-selects the 2 options for replica number determination for a CB-Msg3 EDT:

- Option 1: The network indicates the MAX_REPLICA_ NUMBER, and the UE selects the replica number according to a uniform distribution between 1 and the MAX_REPLICA_NUMBER (in which case different UEs may select different replica number randomly).

- Option 2: The network indicates a specific REPLICA_ NUMBER, and the UE uses this REPLICA_ NUMBER (in which case different UEs use the same replica number).

- 2.1.2.3	Selection of the resources to meet the CE level (for a given Replica)

After the number of replica is determined per Proposal 3, the issue is how to select the resources for each replica. In the RAN2 assumption agreed in RAN2#127bis meeting "CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission", this "associated number of repetitions" is related to the selected resources for the repetition transmission(s) of a given Replica.

Specifically, since we propose to define the per-CE level resource pools for the CB-msg3 EDT cell specific PUSCH resources as in Proposal 1, the number of repetitions should be associated with each resource pool according to its corresponding CE level. Then, for the transmissions within a Replica, the UE randomly selects a set of resources which support the number repetitions configured for the pool resource pool selected by the UE (for the intended CE level as in Proposal 1).

Proposal 4: Each resource pool is configured with an associated "number of repetition", according to the corresponding CE level.

Proposal 5: For the repetition transmission(s) within a Replica, UE randomly selects a set of resources which support the number of repetitions associated with the resource pool selected by the UE (as per the intended CE level in Proposal 1).

Once replica number is determined, the UE performs transmission based on the replica number. It is questionable whether the CE level can be different for different replicas. If the CE level does not change for different replicas, it can be determined at the CB-Msg3 EDT initiation stage via RSRP evaluation. But when the channel condition changes, the CE level may not be align with the latest channel condition, possibly impacting t resource efficient. On the other hand, if the CE level can be changed for different replicas, the UE has to evaluate the RSRP level before the resource selection for each replica. This is helpful to the resource utilization but with marginal benefits, considering that replica is also one kind of repetition. Besides, this brings complexities to the whole transmission procedure. Considering that the Near-Far Effect is not obvious for NTN, the channel quality change is not significant. So we think resource selection with unchanged CE level for each replica is proposed.

Proposal 6: CE level of each replica does not change, i.e. the UE only evaluate the RSRP at the initial stage to determine the CE level, which is kept the same for all repetitions and all the replica(s) . RAN2 can ask RAN1 for confirmation.

- 2.1.2.4	Resource selection procedure

The CB-Msg3-EDT occasions of each replica can be determined at the beginning of the transmission. The UE can select the CB-Msg3-EDT occasion based on the replica number randomly among the CB-Msg3-EDT occasions. The procedure is shown in Figure 2.

Figure 2: Resource selection for CB-Msg3-EDT procedure

Proposal 7: The resources for CB-Msg3-EDT occasion are selected for all the replicas at one time, when the resource selection is triggered upon CB-Msg3-EDT initiation.

CB-Msg3 transmission

Once the resources have been selected, the UE can perform CB-Msg3 transmission. The UE can deliver the corresponding UL grant, HARQ information to the HARQ entity. Each grant is still corresponding to the transmission occasion of a repetition, similar to legacy UL transmission.

Proposal 8: The UE performs CB-Msg3 transmission based on the legacy UL transmission mechanism (i.e. deliver each grant and HARQ information to the HARQ entity), with each UL grant still referring to the transmission occasion of a repetition.

Network response reception

### 2.4.1	Window after CB-Msg3

Once CB-Msg3 has been transmitted, the UE can monitor PDCCH for network response. Similar to RAR or Msg4, a window/timer, namely CB-Msg3 window, can be started after CB-Msg3 transmission. Some basic principles can be followed. Firstly, the operation of the CB-Msg3 window should cover possible PDCCH for the NW response towards any replica. Secondly, the CB-Msg3 window should be started, taking UE-eNB RTT into account.

Proposal 9a: A window/timer can be started after CB-Msg3 transmission to control UE's PDCCH monitoring for network response.

Proposal 9b: The window/timer should be started, taking UE-eNB-RTT into account.

For multiple replica transmission (i.e. as determined by UE per proposal 5), it is still unclear how to maintain CB-Msg3 window to guarantee that the UE does not miss PDCCH monitoring for response of each replica. Considering that the network does not know which CB-Msg3-EDT occasion is selected for different replica and whether subsequent replica is performed, the following options are provided.

Option 1: The UE starts CB-Msg3 window for the first replica and the CB-Msg3 window length equals to the Replica resource duration plus the CB-Msg3 window for one replica, where the Replica resource duration is calculated by the UE via actual transmission and CB-Msg3 window for one replica is configured by the network.

In this option, it is not friendly to UE power saving, since the UE has to monitor the whole period for all replicas.

Figure 4: CB-Msg3 window running in Option 1

Option 2: CB-Msg3 window is started/restarted for each Replica transmission.

In this option, the UE will not consider the CB-Msg3 transmission has completed when CB-Msg3 window expires for each individual replica transmission, except the last one.

Figure 4a: CB-Msg3 window running in Option 2

Comparing these 2 options, it can be seen that Option 2 is more friendly to power saving, but a bit of complex to maintain window start/restart for different replicas. Besides, the timer interval between each replica may also have impact on the window running. RAN2 can discuss which option is adopted for CB-Msg3 EDT window, and make down selection between them.

Proposal 10: RAN2 down-selects which option is adopted for CB-Msg3-window:

- Option 1: CB-Msg3 window is started towards the first replica transmission, and the CB-Msg3 window length equals to the Replica resource duration plus the CB-Msg3 window length for one replica, where the Replica resource duration is calculated by the UE via actual transmission and CB-Msg3 window length for one replica is indicated by the network.

- Option 2: CB-Msg3 window is started/restarted for each Replica transmission individually, and the UE only monitors PDCCH within the window.

### 2.4.2	RNTI calculation

It has been agreed that the RNTI used at least to schedule Msg4 transmission is derived based on the resources associated to the PUSCH occasion used for contention based CB-Msg3 EDT transmission. Considering that the network does not know how many replicas the UE has selected, the RNTI should be computed for each replica respectively. The UE should monitor PDCCH scrambled by all the RNTIs for different replicas to figure out whether contention resolutions has completed or not.

Proposal 11: CB-Msg3-RNTI should be computed for each Replica in CB-Msg3 EDT respectively.

Contention resolution

In CB-Msg3, if the UE selects the same PUSCH/NPUSCH, there is no way to distiguish them except for contention resolution. It has been agreed that broadcast signalling is used to deliver the CB-Msg3 configuration. In this case, the UE could calcuate CB-Msg3-RNTI based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission. Similar to Msg4 reception, the UE monitors the PDCCH scrambled by the CB-Msg3-RNTI. Then, in order for contenstion resolution, a CB-Msg3 contention resolution MAC CE should be introduced and included in the DL message as a response to the UE. If the CB-Msg3 contention resolution MAC CE is corresponds to the beginning part of the UL message (e.g. first 48 bits), the UE will consider the conention resolution for this CB-Msg3 transmission is successful.

Proposal 12: For CB-Msg3 EDT, the similar mehcanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3.

There is a need to handle the exceptional case that the UE may not receive any feedback from the network after the Msg3 transmission. To this end, the UE just starts PDCCH monitoring for Msg4 scheduling during CB-Msg3 window. If the Msg4 corresponding to its identity is not received within the window, UE considers the CB-Msg3 transmission failure, and perform the failure handling. If the CB-Msg3 window expires, the UE could re-attemp UL transmsision or fallback to RA-EDT or declare CB-Msg3 transmission failure. Since such failure handling may depend on how the UE treats the CB-Msg3-EDT window in Proposal 10, we think the specific failure handling can be further discussed, until CB-Msg3-EDT window handling is determined.

Proposal 13: RAN2 further discusses how the UE handles CB-Msg3 transmission failure (e.g. re-attemp another UL transmission, fallback to RA-EDT, declare CB-Msg3 transmission failure to upper layers, etc.), after concluding the CB-Msg3-EDT window/timer handling.

Considering the CB-Msg3-RNTI is calcuated base don each replica per Proposal 11, the UE has to monitor all the CB-Msg3-RNTIs with the CB-Msg3 window.

Proposal 14: The UE should monitor PDCCH scrambled by all the CB-Msg3-RNTIs for different replicas during the CB-Msg3 window.

- 3.	Conclusion

In this document, we analyse the potential impacts for UL capacity enhancements in IoT NTN, and find the following and proposals:

Proposal 1: Introduce per CE level resource pools which include the CB-Msg3 EDT cell specific PUSCH resources. UE selects the resource pool associated with the intended CE level, according to the RSRP criterion.

Proposal 2: The details on physical resource structure of CB-Msg3 EDT in the resource pool (e.g. whether to reuse the existing RU in NB-IoT or design other resource granularity) are left to RAN1.

Proposal 3: RAN2 down-selects the 2 options for replica number determination for a CB-Msg3 EDT:

- Option 1: The network indicates the MAX_REPLICA_ NUMBER, and the UE selects the replica number according to a uniform distribution between 1 and the MAX_REPLICA_NUMBER (in which case different UEs may select different replica number randomly).

- Option 2: The network indicates a specific REPLICA_ NUMBER, and the UE uses this REPLICA_ NUMBER (in which case different UEs use the same replica number).

Proposal 4: Each resource pool is configured with an associated "number of repetition", according to the corresponding CE level.

Proposal 5: For the repetition transmission(s) within a Replica, UE randomly selects a set of resources which support the number of repetitions associated with the resource pool selected by the UE (as per the intended CE level in Proposal 1).

Proposal 6: CE level of each replica does not change, i.e. the UE only evaluate the RSRP at the initial stage to determine the CE level, which is kept the same for all repetitions and all the replica(s) . RAN2 can ask RAN1 for confirmation.

Proposal 7: The resources for CB-Msg3-EDT occasion are selected for all the replicas at one time, when the resource selection is triggered upon CB-Msg3-EDT initiation.

Proposal 8: The UE performs CB-Msg3 transmission based on the legacy UL transmission mechanism (i.e. deliver each grant and HARQ information to the HARQ entity), with each UL grant still referring to the transmission occasion of a repetition.

Proposal 9a: A window/timer can be started after CB-Msg3 transmission to control UE's PDCCH monitoring for network response.

Proposal 9b: The window/timer should be started, taking UE-eNB-RTT into account.

Proposal 10: RAN2 down-selects which option is adopted for CB-Msg3-window:

- Option 1: CB-Msg3 window is started towards the first replica transmission, and the CB-Msg3 window length equals to the Replica resource duration plus the CB-Msg3 window length for one replica, where the Replica resource duration is calculated by the UE via actual transmission and CB-Msg3 window length for one replica is indicated by the network.

- Option 2: CB-Msg3 window is started/restarted for each Replica transmission individually, and the UE only monitors PDCCH within the window.

Proposal 11: CB-Msg3-RNTI should be computed for each Replica in CB-Msg3 EDT respectively.

Proposal 12: For CB-Msg3 EDT, the similar mehcanism like Msg4 in RACH can be reused for contention resolution, e.g., if the beginning part (e.g. 48 bits) in the DL message matches the corresponding part of the UL message sent by the UE, contention resolution is considered successful for CB-Msg3.

Proposal 13: RAN2 further discusses how the UE handles CB-Msg3 transmission failure (e.g. re-attemp another UL transmission, fallback to RA-EDT, declare CB-Msg3 transmission failure to upper layers, etc.), after concluding the CB-Msg3-EDT window/timer handling.

Proposal 14: The UE should monitor PDCCH scrambled by all the CB-Msg3-RNTIs for different replicas during the CB-Msg3 window.

- 4.	Reference

- RAN2#127bis chairman notes.